3GPP TSG RAN Meeting #108			RP-251243

Prague, Czech Republic, June 9-13, 2025

Source:	Sharp

Title:	Discussion on RAN WG 6G SI

Agenda Item:	16.2

Document for:	Discussion

# Background

The summary of the 6G Workshop [1] includes the Rel-20 6G timeline according to which RAN1 study is expected to start after RAN#108, followed by RAN2/3/4 studies. In this contribution, we present our views and proposals on the RAN WG 6G SI.

The document is organized as follows:

- 6G Architecture

- 6G Use Cases

- PHY Techniques

- Control Plane / L3

- User Plane / L2

# Discussion on RAN WG 6G SI

# 6G Architecture

### 6G Architecture Options and Migration

In 5G NR standardization, multiple NSA options based on MR-DC were introduced in Rel-15. The NSA brought huge complexities from both standardization and development perspectives. 6G standardization should avoid this inefficient approach. Simultaneous operations or seamless services between 5G and 6G may be needed. Possible migration options can be discussed during the 6G study. However, the baseline should be a migration from 5G to a standalone 6G without introducing NSA-like 6G solutions.

Proposal 1: 6G study should focus on 6G standalone as an architecture option and migration model.

### Inter-RAT Spectrum Sharing

Efficient spectrum utilization is critical to support emerging use cases such as immersive communication, massive communication, and ISAC (Integrated Sensing And Communication) in 6G. Inter-RAT spectrum sharing, proven in NR via techniques like DSS (Dynamic Spectrum Sharing), will play a pivotal role in 6G to ensure backward/forward compatibility with legacy LTE/NR and future RATs.

Some new frequency ranges between 7 – 24 GHz are identified for IMT system for at least one ITU region, including 7.025 – 7.125GHz and 10 – 10.5GHz. Besides, 7.125 - 8.4GHz and 14.8 - 15.35GHz are listed as candidate frequency ranges for IMT system by WRC-27 [2]. On the other hand, existing LTE/NR bands for FR1 are extremely important for wide-area coverage although there is very little spectrum remaining in this range. The high frequencies can provide wider bandwidths to support higher data rates although the coverage is limited. A wide-area coverage is crucial for successful deployment of 6G network. Therefore, it is pivotal to deploy 6G in the existing LTE/NR bands. For smooth transition to 6G, spectrum sharing should be supported from 6G Day One.

Because there is very little cell-specific signaling for NR, the spectrum sharing between NR and 6G can be made very efficient with a small overhead. Spectrum sharing between LTE and 6G is not so critical because LTE will have largely been replaced by NR by the time of 6G deployment. We see that most NR specifications of DSS can be reused for spectrum sharing between LTE and 6G.

Proposal 2: Study solutions for Inter-RAT spectrum sharing for 6G Day One.

### Inter-RAT Mobility

A gradual transition to 6G systems will require fallback to older generations, i.e., mobility technologies to and from 6G coverage. At the early stage of 6G deployment, there may be frequent occurences of users moving in and out of 6G coverage. In such a scenario, it is important to minimize communication interruption time. As in NR and LTE, inter-RAT mobility should be supported in 6G with minimal effort by reusing the techniques used for inter-RAT mobility in NR [as the baseline].

Proposal 3: From the RAN perspective, Inter-RAT mobility from/to NR should be supported in 6G with minimal effort while striving to reuse related NR and LTE techniques [as the baseline].

NR allows significantly higher freedom in SSB placement than LTE. For this reason, SMTC and other similar techniques have been introduced. In addition, SSBs could be placed even more flexibly due to the introduction of NES features. For these reasons, supporting Inter-RAT mobility for all SSB placements in NR networks may be somewhat complicated. Therefore, it is desirable to make efforts to support measurement reports for inter-RAT mobility with simple implementation.

Proposal 4: Study measurement reports for inter-RAT mobility to NR with simple implementation.

### Interaction with O-RAN

In the recent 3GPP O-RAN joint workshop, various viewpoints were discussed on motivations and challenges for coordination on 6G standardization. While we believe that deeper alignment between 3GPP and O-RAN will benefit the telecommunication ecosystem, such cooperation must rest on a foundation of mutual transparency. An important point discussed during the workshop was the need for O-RAN to adopt more transparent working practices [3]. Currently, O-RAN’s working documents are largely inaccessible to the broader industry, including many 3GPP members. We believe it is essential for O-RAN to take steps toward opening its processes to align better with the transparent ethos practiced by 3GPP before any systematic coordination can be established between the two SDOs.

Proposal 5: The precondition for systematic coordination with O-RAN on 6G is access to O-RAN working documents.

# 6G Use Cases

### Network Energy Saving Features

In previous generations of the standards, various features have been introduced to support advanced services and applications, especially those requiring higher data rates. Most of these features rely on denser networks, more antennas or larger frequency bandwidths. However, the resulting increase in network energy consumption due to the introduction of these technologies directly impacts the operational expenditure (OpEx) of Mobile Network Operators (MNOs).

To address this issue, Network Energy Saving (NES) features have been introduced in 5G-Advanced (5G-A). Based on the study in Rel-18, many of the adopted features in 5G-A NES has relatively high potential energy-saving gains. For example, in Rel-19 NES, key aspect was how to minimize the transmission of broadcast signals/channels which are periodically transmitted, and how to increase the sleeping opportunity of network, which results in introduction of SSB/SIB1 on-demand transmission and adaptation of common signals have been applied. Such approaches specified for 5G-A should also be considered for 6G (irrespective of change of 6G basic structure).

However, the backward compatibility in 5G framework limits the potential energy-saving gains and may discourage operators from fully deploying NES features. For example, some of the 5G-A NES techniques are applied only to RRC_CONNECTED UEs to accommodate 5G legacy UEs within the same cell. In parallel, to obtain the configuration of on-demand SIB1 transmission with 5G SSB structure, assistance of neighbour cell (Cell A) is required. Therefore, in 6G, to maximize the energy-saving gains, we should study how to apply those techniques across all RRC states and in stand-alone cells. Those include designing the initial access procedure (e.g., SSB structure/MIB contents) with NES perspective in mind. In addition, as the number of antenna elements in 6G is expected to increase significantly, it is important to study how spatial and power adaptation can be effectively applied in this scenario.

Moreover, the coexistence of UE power-saving and NES features introduces complexity, such as the interplay between UE C-DRX and cell DTX/DRX. There exists a trade-off between optimizing UE power consumption and achieving network energy savings. Therefore, for 6G, we propose a joint optimization approach that ensures a balance between UE power saving and NES solutions. Additionally, NES solutions often decline coverage due to limit certain behaviours of UEs or base stations. For example, for spatial/power adaptation, reducing the number of active antenna ports disperses the energy of the transmitted signal and decline coverage. Therefore, strategies to maintain adequate coverage while adopting NES solutions should also be explored.

Proposal 6: Study NES features, focusing on the following aspects:

- Study NES solutions applied for 6G scenarios.

  - Ensure 6G NES solutions are commonly applicable across all RRC states.

  - Ensure 6G NES solutions in stand-alone cell operation (i.e. without Cell A).

- Investigate strategies to balance UE power saving with NES efficiency.

- Investigate strategies to balance coverage with NES efficiency.

### Coverage enhancement

In our view, there are two major directions on coverage enhancements, including NTN scenarios and operations within 7 – 24 GHz.

For NTN scenarios, as discussed in [4], vertical use cases like automotives and maritime demand for smaller mobile VSATs (e.g., 20 x 20 cm). The smaller the size of the antenna of mobile VSATs, the smaller the gain of the antenna gain. Furthermore, as agreed in one of RAN4 sessions for Ku band introduction [5], VSAT’s beam gain reduction at the lower elevation angle to satellite should be considered to define the RF requirements. Therefore, coverage requirements for mobile VSATs are increasingly higher compared to the legacy antenna size of 60 cm equivalent circular aperture.

For operations within 7 – 24 GHz, according to the FSPL (Free Space Path Loss) calculation formula, pathloss will increase 6 dB when frequency is doubled. Therefore, theoretically, pathloss in around 7 GHz will be about 6 dB higher than 3.5 GHz, about 12 dB in around 15 GHz. Thus, to cover the same area as in the existing NR bands around 3.5 GHz, coverage compensation techniques need to be considered.

Therefore, 6G will require greater focus on coverage gaps in noteworthy scenarios that didn’t exist before. We suggest that, especially, NTN scenarios with smaller size mobile VSATs and operations within 7 – 24 GHz are considered for 6G coverage evaluation to design physical channels.

Proposal 7: NTN scenarios with smaller size mobile VSATs and operations within 7 – 24 GHz are considered for 6G coverage evaluation to design physical channels.

### AI/ML

To integrate AI/ML features into the 6G RAN, the AI/ML framework from 6G Day One should adopt future-proof design, i.e., it should be capable of supporting not only an initial set of use cases introduced in 6G Day One but also any future new use cases needed in subsequent 6G Releases.

Regarding the 6G AI/ML use cases, 6G Day One can inherently support the existing 5G-A use cases. Besides them, new use cases with commercial prospects can be further studied in 6G Rel-20. It is desired to use AI/ML prediction capabilities to deliver a better user experience and improve throughput in the dynamic and complex wireless communication environment. One promising use case is to study RS overhead reduction using one-sided model, which could help improve spectral efficiency and improve throughput. Some other new use cases e.g., interference management, Radio Resource Management including mobility could be studied as well for spectral efficiency improvement.

Proposal 8: Study new 6G AI/ML use cases considering one-side models, e.g., RS overhead reduction for 6G Day One.

Regarding the AI/ML framework, the functionality-based LCM which is going to be specified in 3GPP Rel-19 will provide a solid foundation for managing AI/ML models. It supports key functions such as activation, deactivation, fallback operations, and more.

From our perspective, the LCM design in Rel-19 is generic and flexible enough to accommodate new AI/ML use cases beyond those already defined in Rel-19. Therefore, for 6GRel-20, we suggest building on this existing LCM framework and study enhancements tailored to 6G AI/ML scenarios. Potential improvements could include mechanisms for power control and reducing signaling overhead and latency. Overall, the AI/ML framework in 6G can benefit from and extend the groundwork in 5G-Advanced.

On the other hand, normative work in 5G-A Rel-20 will continue for the AI/ML-based CSI compression use case, which utilizes a two-sided model approach. One of the main challenges with the two-sided model lies in enabling inter-vendor collaboration during the LCM procedure. This involves complex processes such as model identification, and the exchange of datasets, model structures, and model parameters.

These issues have already been actively and extensively studied over the two Releases, starting from Rel-18, and this work will carry on in Rel-20. In this context, model-based LCM—which facilitates model identification and pairing—could also be further developed in 5G-A Rel-20.

If a new 6G use case based on a two-sided model is to be introduced from 6G Day One, we recommend deferring to the standardized outcomes of 5G-A Rel-20. Reusing these solutions will help avoid duplication of efforts and reduce unnecessary complexity in the early phase of 6G study.

Proposal 9: Reuse Rel-19 functionality-based AI/ML LCM as baseline for designing 6G AI/ML framework.

Proposal 10: For 6G new use cases, if two-sided model is introduced in 6G Day One, consider the standardized LCM mechanisms for two-sided model developed in 5G-A Rel-20.

### ISAC

We consider the following major directions for ISAC in from 6G Day One.

- OFDM-based physical layer as the baseline

- Prioritization of sensing modes for 6G Day One services

- Protocol enhancements for integrating sensing along with communication services

For ISAC, baseline requirements for new use cases in 6G may be derived from the use cases considered in 5G-A such as UAV tracking. Depending on the outcome of the studies in 5G-A, further use cases should be considered for specifying high-performance sensing services from 6G Day One.

To avoid major changes to the physical layer, we propose to consider OFDM-based waveform as the starting point for ISAC features. Alternative waveforms may be discussed at a later stage, based on an initial study covering possible modifications required for OFDM-based systems to support ISAC.

Proposal 11: Reuse the OFDM-based waveform from 5G-A as the baseline. Study the required enhancements to the OFDM-based waveform and deprioritize alternative waveforms for 6G Day One.

Along with gNB-based mono-static sensing, further sensing modes may also be considered from 6G Day One. Based on the estimated workload, bi-static sensing modes including gNB-gNB, gNB-UE, and UE-gNB modes may be prioritized. To streamline the discussions starting from the initial stages, we propose to deprioritize UE mono-static sensing and UE-UE bi-static sensing, which can be considered once the appropriate use cases have been identified.

Proposal 12: Study gNB mono-static sensing and gNB-gNB, gNB-UE, and UE-gNB bi-static sensing modes.

To deploy ISAC features in tandem with communication services, the required protocol enhancements to consider various degrees of integration between sensing and communication may be considered. In addition, RAN and system architecture enhancements required to initiate and manage sensing services with the necessary performance requirements can be studied. This may, for example, include signalling required between the UE and the network for sensing session setup, configuration, and reporting for the relevant sensing modes.

Proposal 13: Study the required protocol and architecture enhancements for ISAC considering various degrees of integration between sensing and communication.

### NTN

Integrated TN-NTN (Terrestrial Network and Non-Terrestrial Network) were proposed and supported extensively during the recent 6G workshop [1] as a key enabler for achieving pervasive, resilient connectivity. It envisions close coordination between various TN-NTN components in ground, air, and space, with an emphasis on coordination between ground and satellite network infrastructure. A wide range of use cases can be considered – broadband access in unserved and underserved areas, disaster recovery, and so on. This integration is consistent with the 6G vision of delivering consistent user experience.

Proposal 14: Study techniques assuming integrated TN-NTN is supported for 6G Day One.

GNSS-less operation for NTN

To reduce reliance on GNSS, 3GPP is to explore GNSS-less operation for NTN, which finds more use cases in environments where GNSS signals are unavailable or compromised. Alternatives to GNSS include network-based positioning and using satellite ephemeris data for timing and synchronization.

Proposal 15: Study GNSS-less operation in 6G NTN through network-based and ephemeris-assisted techniques.

Seamless TN-NTN mobility

We consider seamless mobility between TN and NTN as a major goal for 6G Day One. This involves addressing differences in delay, Doppler, and coverage. 3GPP is to consider solutions such as predictive handovers and multi-connectivity to maintain session continuity.

In TN-NTN mobility scenarios, whether TN and NTN operate under the same or different PLMNs has architectural and procedural implications. A common PLMN simplifies mobility and policy control However, separate PLMNs may be needed for regulatory or business reasons. 3GPP should consider mechanisms to support both intra- and inter-PLMN mobility including dual registration solutions. Mechanisms should be studied for context transfer and access policies, especially in the case of inter-PLMN mobility.

Proposal 16: Study seamless mobility across TN and NTN in 6G through predictive handover and multi-access session continuity mechanisms.

Store-and-forward and inter-satellite communication

Store-and-forward and inter-satellite communication can be studied for optimizing routing-related latency in NTN. These techniques may reduce latency and reliance on ground stations by allowing data to be cached and relayed between satellites.

Proposal 17: Support store-and-forward and inter-satellite communications in 6G NTN.

TN-NTN spectrum sharing

It is beneficial to study TN-NTN spectrum sharing in 6G in pursuit of optimal spectrum utilization. We should explore techniques such as geolocation-based sharing, time-division access, and interference management to enable spectrum sharing.

Proposal 18: Study spectrum sharing between TN and NTN in 6G. Study coexistence, interference management, and other related enablers.

# PHY Techniques

### Downlink Control and Monitoring

During the discussion of 6G Workshop, it is clear that 6G would face the following new challenges:

- A unified DL control signaling transmission signanling across FR1, FR2, and 7 – 24 GHz.

- More and more DCI formats may be provided to support different functionalities in a wide range of scenarios (e.g., NTN, ISAC). The loading of blind decoding would increase. In addition, more blind decoding attempts would increase the power consumptions in the UE side. However, since energy saving would be a critial topic in 6G communication, RAN WG needs to study whether 2-stage DCI could bring energy-saving gains or cause additional power consumption to the whole 6G communication system.

- Energy saving should be consided from 6G Day One and 6G should study how to keep energy saving even 6G may be requested to support more DCI candidates and DCI formats.

To solve the new challenges, the following directions are proposed for RAN WG’s consideration:

Wake-Up signaling design:

Rel-19 Low-Power WUS design may be considered as a UE capability to enhance DL control signaling delivery. UE can receive a WUS signaling to decide whehter to wake up for the following DL control signaling reception irrespective of RRC states. In addition, WUS design and the assocaited physical resource configuration/allocation should be considered while RAN1 WG starts the discussion of 6G frame structure and waveform design.

Proposal 19: Study Wake-Up signaling for 6G downlink control irrespective of RRC states.

Two-stage DCI

In 6G study item, two-stage DCI mechanism can be studied to solve the issues above. RAN WG is suggested to study how to introudce two-stage DCI mechanism by analyzing the following sub-topics:

- Physical resource allocation, which also includes the assignement reference signaling to support the Two-stage DCI.

- Blind decoding mechanism: With the adoption, RAN WG may need to consider how to laverage the loading of blind decoding on the two-stage DCI mechanism, such as:

  - Blind decoding only on the 1st-stage DCI decoding.

  - Blind decoding on both the 1st-stage DCI decoding and the 2nd-stage DCI decoding.

- How to support cross-carrier scheduling via the Two-stage DCI mechanism.

- Contents in the 1st-stage and the 2nd-stage DCI.

- Latency and overhead evaluation which may be caused by Two-stage DCI mechanism.

- Performance on energy-saving gains brought by Two-stage DCI mechanism.

Proposal 20: Study Two-stage DCI for 6G DL control.

To start the discussion of DL control signaling mechanism, RAN WG is suggested to consider whether some designs in NR (e.g., frame structre , BWP, and CORESET) and further enhancements on 5G-A (e.g., LP-WUS signaling) should be reused as baseline. Then, RAN WG can study how to realize and evaluate 6G DL control mechanism.

Proposal 21: Decide whether to reuse some of the downlink control designs of NR and 5G-A before the discussion of 6G DL control.

### Spectrum aggregation

Our views on spectrum aggregation is summarized below.

Motivation of spectrum aggregation

- There are many discontinuous spectrum fragments in low-band, and we should have a more efficient way to manage them

Benefits of spectrum aggregation

- Reduce always on signals (e.g., cell-defining SSBs) by aggregating multiple spectrum fragments

- Exploit frequency diversity by mapping a single physical channel on multiple spectrum fragments

- Exploit HARQ gain by mapping a retransmission of a single physical channel on a different spectrum fragment to reduce TDD HARQ latency

Proposal 22: Study solutions for aggregating multiple downlink/uplink carriers as a single serving cell.

### MIMO

To compensate for attenuation due to the increased frequency in 7 – 24 GHz, more advanced massive MIMO techniques may be useful with the larger number of antenna elements than those in FR1 [5]. However, there are some potential challenges in radio propagation in 7 – 24 GHz , which is so-called near-field propagation.

The near-filed propagation was studied in Rel-19 and TR38.901 channel model is expected to be updated to include near-field propagation modeling. There are two major approaches studied in Rel-19, which are spherical waves and visibility regions. Figure 1 shows comparison between plane waves and spherical waves. The phase differences between adjacent antenna elements are shown in red. In plane waves which was assumed to design CSI codebooks in 5G NR, the phase differences are the same in which the distance between Tx and Rx is assumed to be large enough for approximation of the phase difference calculation. However, in spherical waves, the phase differences cannot be as simple as in plane waves.

| Plane waves | Spherical waves |
| --- | --- |

Figure 1: Comparison between plane waves and spherical waves

Figure 2 shows concept of visibility region. The left side of the figure presents full visibility, and the right side presents partial visibility in which different UEs sees different part of the Tx antenna. It makes large difference in signal strength between visibility region and non-visibility region.

| Full visibility in far field | Partial visibility in near field |
| --- | --- |

Figure 2: Concept of visibility region

In summary, we propose to study potential enhancements of channel and CSI codebook designs using the updated TR38.901 tailored for 7 – 24 GHz .

Proposal 23: Study potential enhancements of channel and CSI codebook designs using the updated TR38.901 tailored for 7 – 24 GHz.

# Control plane / L3

### Unified Mobility

In NR, multiple mobility mechanisms have been developed over multiple releases—including L3-based traditional handover, CHO, DAPS, LTM, and CLTM—each optimized for specific use cases. However, this fragmented mobility architecture introduces several challenges. First, it increases implementation and testing complexity due to overlapping functionalities and redundant signaling procedures (e.g., both beam-based and cell-based RRM measurements). Second, the lack of coordination among mobility features leads to inefficiencies such as duplicated configurations, suboptimal handover decisions, and increased UE power consumption, especially when early synchronization and beam measurements towards multiple candidate cells are involved. Additionally, limited support for low-latency and low-interruption mobility (e.g., RACH-less HO, minimal L2 reset) is confined to specific scenarios like LTM. These limitations affect both control plane robustness and user plane performance, with potential interruptions and throughput degradation during mobility events. Also, these complexities result in low market adoption.

To overcome complexity and power consumption issues, a unified mobility framework is proposed for 6G. This approach aims to consolidate various mobility features into a single common mobility function with simplified and modular procedures. It can cover both beam-based and cell-based measurements under a unified RRM structure, enabling more efficient signaling and decision-making. Moreover, it enables native support for ultra-low interruption handovers and energy-efficient mobility procedures and enhancing user experience and system resilience. A unified design not only reduces UE burden and improves handover latency, but also avoids excessive feature combinations and streamlines implementation across networks. Therefore, unified mobility is essential to ensure scalability, efficiency, and robustness in future 6G networks.

However, given the implementation complexity of DAPS and the fact that LTM and RACH-less handover can provide comparable benefits in minimizing interruption, DAPS is not considered essential for 6G mobility design. Moreover, to achieve low-latency and low-interruption mobility, native RACH-less HO and minimal-reset handover procedures within the unified mobility framework should be supported.

- Proposal 24: Study a unified mobility procedure that consolidates L3-based traditional handover, CHO, LTM and CLTM into a single configurable framework.

- Proposal 25: 6G study precludes DAPS-based mobility in 6G.

- Proposal 26: Study native RACH-less and minimal-reset handover procedures within the unified mobility framework.

### RRC Signaling Structure Optimization

As NR has a number of functions, RRC configuration has a lot of flexibility and a larger size. For example, CSI-related configuration is very complicated. In 6G, such complicated signaling should be avoided and the signaling structure should focus on essential configuration. We see that 6G study phase should also aim to consider this aspect, and how we improve the signaling framework.

Proposal 27: 6G study should aim to reduce the size of RRC configuration.

### RRC States

As 6G development accelerates, the RRC state model can be under reconsideration to better align with evolving user expectations and network demands. In NR, the introduction of RRC_INACTIVE aimed to reduce signaling overhead while supporting fast transitions between RRC_IDLE and RRC_CONNECTED states. However, its limited commercial deployment has sparked discussions on whether it should be retained, enhanced, or replaced in the context of 6G.

Traditionally, NR defines three RRC states:

- RRC_CONNECTED, where the UE maintains a full connection and frequent communication with the gNB;

- RRC_IDLE, where the UE is reachable via paging but holds no context in the gNB;

- RRC_INACTIVE, introduced since Rel-15 to bridge the two, allowing the UE to suspend its RRC connection and resume it quickly without full re-establishment.

So far, it is observed that the commercial deployments of NR RRC_INACTIVE state are limited primarily due to early reliance on EN-DC (E-UTRA-NR Dual Connectivity) and EPC (Evolved Packet Core), which did not support RRC_INACTIVE well. This led to fragmented specifications and complicated mobility handling. As 6G should be free from EPC legacy constraints, there will be an opportunity to redesign the RRC state machine with clean slates, e.g., only two RRC states are considered in 6G.

- Some solutions to storing UE configurations across state transitions are being considered to achieve the benefits of RRC_INACTIVE.

We propose that RRC_INACTIVE be adopted as the baseline RRC state in 6G, guided by the following principles:

- Preservation of UE configurations across state transitions to reduce redundant signaling overhead.

- Fast resumption from inactive to connected states to support low-latency service requirements.

- Potential unification of RRC_IDLE and RRC_INACTIVE into a single, lightweight state—possibly under a new designation—to streamline the protocol architecture.

Overall, the design of RRC states in 6G should aim to minimize signaling complexity and enhance energy efficiency by taking a holistic and system-level perspective.

Proposal 28: Study whether 6G RRC states can be further enhanced by considering RRC_INACTIVE as the primary RRC state. In addition, migration between RRC_IDLE and RRC_INACTIVE state can also be further studied.

### UE Capability

In NR, UE capability signaling provides wide flexibility which increases the size of UE capability signaling. Due to the large size of the UE capability message, RRC segmentation was inevitably introduced. However, in practice, there are only a few types of deployed UEs. The necessity of flexible signaling is questionable. We may consider UE category as in LTE or a capability group to cover multiple features as potential approaches. In any case, 3GPP requires intensive studies how lite capability signaling can be achieved.

Proposal 29: 6G study should aim to support lite UE capability signaling.

# User plane / L2

### MAC CE Protection

NR security protection supports integrity protection and ciphering in PDCP. NR introduced a number of MAC CEs for dynamic and fast control, but those MAC CEs are not security protected at all. In 6G, we expect such dynamic control mechanisms will become more important, e.g. LTM can be common. In this background, there is a clear demand for MAC CE security protection. Data protection shall be supported, similarly to NR. It would be desirable to study how we have a unified security framework which protects both data and control messages.

Proposal 30: 6G study should aim to support a unified security framework for data and control messages including ciphering and integrity protection of MAC CEs.

### Lean User Plane Protocol Stack

Compared to LTE, NR UP protocol stack is complicated. NR has four sublayers, i.e. SDAP, PDCP, RLC and MAC. Also, there are many similar functions defined independently. For example, both PDCP and RLC have sequence numbering, but they have almost one-to-one correspondence. RLC ARQ could be completely replaced by HARQ. PDCP has multiple compression mechanisms such as ROHC, EHC and UDC. In 6G, the protocol stack can be simplified by removing duplicated functions and unifying similar functions.

Proposal 31: 6G study should aim to simplify User Plane protocols.

# Conclusion

This contribution discussed Sharp’s views on the upcoming 6G SI and made the following proposals:

Proposal 1: 6G study should focus on 6G standalone as an architecture option and migration model.

Proposal 2: Study solutions for Inter-RAT spectrum sharing for 6G Day One.

Proposal 3: From the RAN perspective, Inter-RAT mobility from/to NR should be supported in 6G with minimal effort while striving to reuse related NR and LTE techniques [as the baseline].

Proposal 4: Study measurement reports for inter-RAT mobility to NR with simple implementation.

Proposal 5: The precondition for systematic coordination with O-RAN on 6G is access to O-RAN working documents.

Proposal 6: Study NES features, focusing on the following aspects:

- Study NES solutions applied for 6G scenarios.

  - Ensure 6G NES solutions are commonly applicable across all RRC states.

  - Ensure 6G NES solutions in stand-alone cell operation (i.e. without Cell A).

- Investigate strategies to balance UE power saving with NES efficiency.

- Investigate strategies to balance coverage with NES efficiency.

Proposal 7: NTN scenarios with smaller size mobile VSATs and operations within 7 – 24 GHz are considered for 6G coverage evaluation to design physical channels.

Proposal 8: Study new 6G AI/ML use cases considering one-side models, e.g., RS overhead reduction for 6G Day One.

Proposal 9: Reuse Rel-19 functionality-based AI/ML LCM as baseline for designing 6G AI/ML framework.

Proposal 10: For 6G new use cases, if two-sided model is introduced in 6G Day One, consider the standardized LCM mechanisms for two-sided model developed in 5G-A Rel-20.

Proposal 11: Reuse the OFDM-based waveform from 5G-A as the baseline. Study the required enhancements to the OFDM-based waveform and deprioritize alternative waveforms for 6G Day One.

Proposal 12: Study gNB mono-static sensing and gNB-gNB, gNB-UE, and UE-gNB bi-static sensing modes.

Proposal 13: Study the required protocol and architecture enhancements for ISAC considering various degrees of integration between sensing and communication.

Proposal 14: Study techniques assuming integrated TN-NTN is supported for 6G Day One.

Proposal 15: Study GNSS-less operation in 6G NTN through network-based and ephemeris-assisted techniques.

Proposal 16: Study seamless mobility across TN and NTN in 6G through predictive handover and multi-access session continuity mechanisms.

Proposal 17: Support store-and-forward and inter-satellite communications in 6G NTN.

Proposal 18: Study spectrum sharing between TN and NTN in 6G. Study coexistence, interference management, and other related enablers.

Proposal 19: Study Wake-Up signaling for 6G downlink control irrespective of RRC states.

Proposal 20: Study Two-stage DCI for 6G DL control.

Proposal 21: Decide whether to reuse some of the downlink control designs of NR and 5G-A before the discussion of 6G DL control.

Proposal 22: Study solutions for aggregating multiple downlink/uplink carriers as a single serving cell.

Proposal 23: Study potential enhancements of channel and CSI codebook designs using the updated TR38.901 tailored for 7 – 24 GHz.

- Proposal 24: Study a unified mobility procedure that consolidates L3-based traditional handover, CHO, LTM and CLTM into a single configurable framework.

- Proposal 25: 6G study precludes DAPS-based mobility in 6G.

- Proposal 26: Study native RACH-less and minimal-reset handover procedures within the unified mobility framework.

Proposal 27: 6G study should aim to reduce the size of RRC configuration.

Proposal 28: Study whether 6G RRC states can be further enhanced by considering RRC_INACTIVE as the primary RRC state. In addition, migration between RRC_IDLE and RRC_INACTIVE state can also be further studied.

Proposal 29: 6G study should aim to support lite UE capability signaling.

Proposal 30: 6G study should aim to support a unified security framework for data and control messages including ciphering and integrity protection of MAC CEs.

Proposal 31: 6G study should aim to simplify User Plane protocols.

# References

[1] 6GWS-250238, Chair’s summary of the 3GPP workshop on 6G, TSG SA Chair, TSG RAN Vice Chair, and TSG CT Chair, March 2025.

[2] H. Poddar, T. Yoshimura and A. Ishii "Validation of 3GPP TR 38.901 Indoor Hotspot Path Loss Model Based on Measurements Conducted at 6.75, 16.95, 28, and 73 GHz for 6G and Beyond”, Accepted in 2025 IEEE 101st Vehicular Technology Conference (VTC2025-Spring), Oslo, Norway, June 2025, pp. 1–6.

[3] 3ORW-250035, Chair Summary of the workshop contributions, TSG SA and O-RAN Chairs, April 2025.

[4] RP-242638, Market considerations on Mobile VSAT operating with NGSO/GSO from verticals point view, Airbus, et al.

- [5] R4-2508778, Way Forward for [115][314] NR_NTN_Ku_Band_UE_SAN_RF, CHTTL.

- [6] Next G Alliance Report: 6G technologies, Next G Alliance, June 2022.