3GPP TSG Meeting #108	RP-251285

Prague, EU, 9th to 13th June 2025

Source:	Ericsson

Title:	Discussion on UE RF and BS RF proposals for Rel-20 5G-Advanced

Agenda item:	15.4.2

Document for:	Discussion

# Introduction

At the last RAN plenary meeting (RAN#107 in Incheon) the discussion about RAN4 Rel-20 5G-Advanced work packages started. All the input proposals were collected and categorized into different groups with the aim to further describe justification and objectives at coming meetings, with the intention to define relevant Rel-20 RAN4 work items at the September plenary meeting (RAN#109 in China).

The following work package categorizes have been identified by RAN:

- UE RF centric evolution

- BS RF centric evolution

- UE OTA enhancement

- RRM enhancement

- Demodulation performance evolution

- NTN enhancements

- Cross RF and baseband topics

In this contribution we provide proposals for work packages to be included in Rel-20 5G-Advanced for UE RF centric evolution and BS RF centric evolution. In addition to proposals, also additional information is provided to further stimulate the discussion to establish clear objectives for each work package within Rel-20 timeframe. It is essential to find relevant scope of each work package to be able to fit it within RAN4 Rel-20 Time Unit (TU) budget.

# UE RF centric evolution

During RAN#107, the following list of topics captured in [1] was discussed for UE RF:

- FR1 HPUE enhancement (PC2/PC1.5/PC1) for single carrier and EN-DC/NR-CA.

  - This item receives common interest but needs discussion on the feasibility of some use cases and also need down-selection.

- Power enhancement with relaxed UE TX EVM and/or uplink throughput improvement with advanced BS receiver (AI based receiver is not precluded as the starting point).

  - TBD on objectives should include TX EVM relaxation, uplink throughput improvement, or both.

  - Study phase is needed if it is considered and FFS whether to treat it in UE RF or Demod package.

  - There are still concern on whether TX EVM should be part of Rel-20 package for 5G-Advanced.

- Band/band combination requirements simplification and MSD simplification/optimization.

  - TBD on whether MSD work is for PC3 and/or HPUE as Rel-19 leftover work.

- TX requirements enhancement for U6G.

- Power classes framework

- Optimize MPR/A-MPR

- Requirement enhancement to support coherent UL MIMO

- UL TX switching enhancement

- 256QAM evolution

- SRS enhancement

- SL-U enhancement

- MPR reduction by leveraging asymmetric ACLR

  - High Power UE

During the RAN meeting, in particular the potential objectives on High Power UE (HPUE) enhancement, power domain enhancement and Maximum Sensitivity Degradation (MSD) simplification were discussed.

Considering the need to focus tightly for 5G-Advanced, as discussed in the introduction, it is important to focus on a single or at most a couple of well scoped and defined enhancements that are clear to bring benefit, be implemented and be of relevance in 5G networks. It is important to take into account that there can also be impacts from “cross area” topics.

Regarding the HPUE enhancements, what remains is to identify UE architectures that are relevant for 5G, likely to be implemented within the 5G timescale and are relevant for operator use cases. HPUE scenarios discussed thus far include:

- Non-Handheld Fixed Wireless Access (FWA) UE

  - 4x26 dBm. This architecture would be capable of achieving a power somewhat above PC1 and could be relevant for FWA. One potential drawback with this architecture could be the Maximum Power Reduction (MPR/A-MPR) that could need to be applied for the 26 dBm PAs.

  - 2x29 dBm. Again, this architecture would be capable of achieving a power somewhat above PC1. Less power back-off may be needed.

- Handheld UE

- PC1.5 for FDD has been proposed in both TN and NTN discussions. There does not seem to be a fundamental technical issue, although it should be checked that it is likely to be implemented within the 5G timescale.

- 1x29 dBm. This may be feasible at least for TDD.

In general, if there is a commercial usefulness for these architectures and confirmation that they are realistic to be implemented then it makes sense for the primary focus on the UE RF work to be on completing the HPUE specifications with these architectures.

Proposal 1: For UE RF, consider HPUE as UE RF Rel-20 5G-Advanced work package. Consider the following candidates; further reduce if possible:

  - 4x26 dBm (FWA)

  - 2x29 dBm (FWA)

  - PC1.5 FDD (Handheld)

  - 1x29 dBm (Handheld)

    - Potentially restrict to TDD

As a starting point for WID preparation drafts for justification and objective are provided below:

Justification: Extend support for HPUE to improve UL coverage.

Objective: Extend the support in the UE BS RF specification to support HPUE 4x26 dBm and 2x29 dBm for FWA UE and PC1.5 (based on 2TX) for FDD bands and 29dBm for TDD bands for handheld UE.

  - Power domain enhancements

Regarding power domain enhancements, the proposal is to enable a relaxation of UE EVM by taking into account the possibility of BS compensation of UE transmitter self-interference. This is clearly a technically interesting area to consider within 3GPP. In our view, however, this needs a study phase, considering the following aspects:

- Real UE RF behaviour when the EVM is relaxed. It is one thing to model an algorithm or AI model based on a simple PA model, but it is important to be clear how self-interference is really manifested and the amount of relaxation that is feasible.

- The variation of RF behaviour across UEs; would an algorithm or model work well across different chipsets and UEs?

- Whether there is variation of RF behaviour across RBs and depending on RB allocation.

- The variation of RF behaviour across different PAs/power classes.

- Impact of MIMO, multiple TX.

- Impact of contiguous CA.

Taking these into account, modelling of the BS algorithm or model is needed, which means that in practice, the study is not only RF but also considers algorithmic modelling and/or AI.

Furthermore, the achievable MPR/A-MPR reduction should be considered in the context of real current UE performance, since MPR/A-MPR modelling is currently over-estimating the needed MPR in general.

In our view, a study is needed and is difficult to consider within the scope of 5G-Advanced. However, this kind of approach is interesting and is a candidate for 6G study.

Observation 1: A significant amount of study is needed for the UE power domain enhancements proposal.

Proposal 2: For UE RF, not consider power domain enhancements for 5G-Advanced.

  - Band combination simplification and MSD optimization

Regarding band combination simplification and MSD optimization, considering the considerable work involved in developing band combinations it is important to further optimize the workflow. The MSD simplification for HPUE in Rel-19 has simplified introduction of HPUE band combinations. Further simplifications of MSD are possible, also for PC3 and may be achieved by, for example grouping similar requirements and applying a generic MSD or similar.

The main question with band combination simplification is whether to perform the work as part of 5G or the 6G study. Within 6G, there is a need to consider how to bring in the already specified 5G combinations, what is needed to fit within the 6G framework and how to optimize. It would be preferable to avoid duplicating work in 5G-Advanced and a 6G study.

A potential WF is as follows:

- Keep the structure of the band combination WI and the process for 5G-Advanced into Rel-20.

- Study band combination simplification, including MSD optimization within the 6G concept in order to achieve a unified study on band combinations.

- In case optimizations (e.g. for MSD) are identified for 6G that could be introduced for 5G-A with low effort, then consider introducing those for 5G-Advanced.

Proposal 3: For UE RF further study band combination optimization, in particular MSD optimization as part of the 6G SI. If measures are identified that are straightforward for 5G, consider introducing those for 5G.

Regarding the framework for power classes, the current structure has grown organically during 5G, and in general the structure of power classes considering CA, MIMO and different form factors should be revised. However, to do so this late in 5G may not be possible with reasonable effort. On the other hand, a better means of handling the variation of UE output power, architecture, CA and MIMO configuration behaviours etc. should be high priority for study in 6G, starting as soon as the 6G SI starts.

Proposal 4: For UE RF, not consider power class optimization for 5G-Advanced.

Regarding MPR/A-MPR optimization, the motivation behind this proposal is that the assumptions typically used for deciding MPR and A-MPR are outdated; in fact, UEs have to exceed some of the requirements to meet e.g. EVM requirements for 64QAM. This has serious real-world consequences, since it means for example that certain combinations cannot be configured by the network because the specification allows for a very large backoff. It would make sense to revise these assumptions to match what UEs are capable of, at least for meeting other requirements.

For 5G, it is too late in the lifecycle to revisit MPR and A-MPR requirements. For 6G, it would be useful to study and decide on suitable assumptions in order that the new specifications can have appropriate values. For 5G, however, it could be useful to consider revising A-MPR requirements for a limited number of identified combinations.

Proposal 5: For UE RF and MPR/A-MPR, in 5G-Advanced consider whether there are certain cases where a specific optimization of A-MPR could make sense.

Regarding the proposal for requirement enhancements to support coherent UL MIMO; this would achieve gains and make technical sense. If there would be time available, this could be included, but it is difficult to see a means to prioritize this with the available time units.

# BS RF centric evolution

From the moderator summary [1], the work packages related to BS RF evolution was collected in a list without priority of high-level candidate objectives for BS RF centric evolution and/or enhancement in Rel-20. The list identified in moderator summary is cleaned up with the intention to be clearer to progress the work to establish justification and complete description of objective. The BS RF centric list can be summarized as:

- Improvements of requirement concept for co-location requirements and corresponding test solutions for bands in the upper region of FR1.

- For AAS BS operating in the upper region of FR1 define an additional NR BS type with requirement applicability including more OTA requirements than BS type 1-H.

- Re-evaluate and validate receiver in-band blocking requirement levels for bands in the upper region of FR1.

- Add support for NB-IoT in AAS BS specifications.

- Add support for LP-WUS in TN MSR specifications.

- Specification structure simplifications for OBUE requirements.

The intention is to use the list of candidate objectives as the starting point for 5G-Advanced BS RF centric evolution and/or enhancement in Rel-20, considering the TU splitting between 5G and 6G in RAN4.

It should be noted that all relevant improvements of 5G-Advanced need to be prioritized within Rel-20 to ensure that the transition to 6G will benefit from these improvements in day one.

In following sections specific details related to identified topics are presented. Some objectives are linked together, while other is independent.

  - Improvements of co-location requirement concept

In Rel-19 BS RF evolution work started to study possibilities to evolve the co-location concept originally introduced in specification during Rel-15. Now it is clear that the Rel-15 concept is suitable for NR BS type 1-O AAS BS (and corresponding MSR AAS BS) operating below 3 GHz where the Co-Location Test Antenna (CLTA) is selected according to the description in the conformance test specification. The concept was derived the idea was that the CLTA is selected as commercially available single column passive BS antenna. Now with support of bands up to 7125 MHz and AAS BS are using much larger antenna arrays the requirement concept needs to be improved accordingly to guarantee site multi-vendor BS co-location and test repeatability.

In a previous contribution [4] submitted to RAN4#114-bis technical issues related to the current requirement concept for co-location requirements was listed. Based on the identified technical issues with current concept work have been initiated in Rel-19 BS RF evolution WI with the intension to initiate work to evolve the co-location requirements.

Observation 2: The current concept used for BS type 1-O co-location requirements captured in TS 38.104, clause 4.9 is not practical for conformance testing of product operating above 3.5 GHz. Hence, BS type 1-O is not a feasible option for AAS BS implementations above 3.5 GHz.

It can also be noticed that when BS with large frequency offset is considered, like for the case where a 900 MHz passive antenna is co-located with a 3.5 GHz AAS BS as showed in Figure 3.1-1, there will be a significant different is antenna aperture length.

# Figure 3.1-1: Example situations, co-location between low-band (non-AAS) and mid-band (AAS)

Observation 3: The height difference between to co-located base stations at different bands (900 MHz and 3.5 GHz) is far larger than what is captured in the CTLA description in current specification.

In addition to previous issues additional technical issues related to the relevance of current requirement concept have been identified. These issues come with the background that during last meeting a discussion emerged on the option to let each BS manufacturer design its own set of CLTA(s) per product, to resolve the issue that CLTA(s) according to the specification are not commercially available.

Today the guidance for selection of CLTA is described in the conformance test specification as listed in Table 3.1-1. If now that information is used to design a CLTA some additional technical issues will emerge.

Table 3.1-1: CLTA characteristics (TS 38.141-2)

| Parameter | In-band CLTA | Out-of-band CLTA |
| --- | --- | --- |
| Vertical radiating dimension (h) | Test object vertical radiating length ±30% | Test object vertical radiating length ±30% (Note 2) |
| Horizontal beam width | 65° ± 10° | 65° ± 10° |
| Vertical beam width | N/A | The half-power vertical beam width of the CLTA equals the narrowest declared (D.3) vertical beamwidth ±3° (Note 2) |
| Polarization (Note 3) | Match (Note 4) | Match to in-band (Note 4) |
| Conducted interface return loss | > 10 dB | > 10 dB |
| Note 1: If a multi-column or multi-band antenna is used the column closest to the NR BS shall be selected while other columns are terminated during testing. Note 2: The vertical radiating dimension definition shall be used instead of the vertical beam width definition when the test chamber dimensions limit the use of vertical beam width definition. Otherwise, the vertical beam width definition shall be used. Note 3: For BS type 1-O with dual polarization the CLTA has two conducted interfaces each representing one polarization. Note 4: Matched to the polarization of EUT antenna. |  |  |

Observation 4: It can be noticed that information provided to describe the CLTA in the specification is overlapping and contradicting. The description provides information on both vertical radiating dimension and vertical beamwidth for the out-of-band case. Both conditions cannot be met at the same time.

Observation 5: The CLTA antenna gain is indirectly specified via the horizontal beam width and the vertical radiating dimension, while the antenna efficiency is not specified. This means that large isolation between the test object and CLTA can be created by having a CLTA with poor antenna efficiency.

In the case where the CLTA following the description in TS 38.141-2, clause 4.12 is not commercially available the remaining way-forward is to let the BS manufacturer to build a CLTA antenna according to the technical specification. For this situation where the CLTA needs to be designed we have observed following issues:

- The technical description of the CLTA provides information on how the beamforming characteristics (3-sector coverage in azimuth and height similar as test object gives beamwidth along vertical axis). However, there is no guidance on the antenna efficiency. Hence the isolation created between the test antenna and CLTA can be created by poor antenna efficiency.

- If the BS manufacturer is responsible to design the CLTA, special means to increase the isolation can be used. There will differ among different BS manufacturers. This means that there is no guarantee that different BS manufacturer will use the same CLTA for conformance testing. As a result, the co-location requirement will not guarantee co-location of multiple BS from different vendors at the same site as the co-location performance will be arbitrary and not repeatable, which was the original intention with the co-location requirements.

Observation 6: The CLTA antenna efficiency is not captured by the description in current specification.

Observation 7: Different types of CLTA from different vendors with very different characteristics would risk making the co-location requirements use-less. To guarantee co-location, the CLTA used should be of similar type and have the same characteristics.

Proposal 6: For BS RF in Rel-20 define a work package with clear goal to resolve open issues identified for BS type 1-O co-location requirements for bands in upper region of FR1.

As a starting point for WID preparation drafts for justification and objective are provided below:

Justification: To make BS type 1-O relevant for AAS BS operating in the upper region of FR1, RAN4 need to evolve and improve the requirement concept for which the co-location requirements are based. The requirements should be defined in such a way that multi-vendor BS co-location and tower sharing at a common site can be guaranteed, and conformance tests can be reproduced.

Objective: Evolve the co-location requirement for BS type 1-O for bands in the upper region of FR1 to be based on OTA parameters (such as field strength or power density) instead of a customized test antenna.

  - Introduction of new AAS BS type

This BS type is intended for AAS BS with a large array antenna required to provide coverage operating within bands in the upper region of FR1. The new BS type will include more OTA requirements than BS type 1-H, which would allow for a smooth transition to full OTA and BS type 1-O.

In Rel-15, the concept of NR BS types was established to allow the specification to support different types of base stations for FR1 (non-AAS and AAS) and FR2 (AAS). The requirement applicability for the different base station types is different since requirements are defined at different requirement interfaces referred to as TAB and RIB. The interfaces are visualised in Figure 3.2-1.

# Figure 3.2-1: AAS BS architecture

The currently supported BS types and a potential new additional BS type (here referred to BS type 1-H-enh) are listed in Table 3.2-1.

Table 3.2-1: BS types

| BS type | Requirement interfaces | Note |
| --- | --- | --- |
| 1-C | TAB | FR1 non-AAS BS with few RF ports and passive antenna |
| 1-H | TAB, RIB | FR1 AAS BS with access to RF ports at TAB |
| 1-H-enh | TAB2, RIB | New BS type for FR1 AAS BS with access to RF ports at TAB2 |
| 1-O | RIB | FR1 AAS BS without access to RF ports |
| 2-O | RIB | FR2 AAS BS without access to RF ports |

Considering the coverage capability for AAS BS to be the same for bands in the upper region of FR1 as for mid FR1 bands would lead to larger number of antenna elements needed for AAS BS products in the upper region of FR1 compared with mid FR1 bands. Since the number of antenna elements needed will be significantly larger than the supported number of layer 1 logical ports (also referred to as baseband ports or CSI-RS ports), different types of port expansion techniques will be required to map signals from logical ports to physical antenna elements. Traditionally the port expansion is done by combining elements as vertical sub-arrays. This combination is done using passive combination networks in the Radio Distribution Network (RDN) and often in conjunction with digital port expansion between baseband and radio. For large arrays, the vertical sub-array tends be very large which would restrict the vertical steering range. Therefore, hybrid beamforming schemes are often considered to add steering capabilities in the RDN by adding analogue RF components, such as phase shifters, switched delay lines, etc.

The current BS architecture described in TS 38.104, clause 4.3 do not explicitly describe type of beamforming schemes supported. However, for electrically large arrays it is essential to define the requirement interface such that all emission generated by the electronics is captured. In the case of having active analogue components in the RDN, a requirement interface should be defined after the RDN. With this interface TAB2 all emission generated by the radio electronics will be captured at TAB2 see Figure 3.2-1.

Another aspect to consider is that with very many AAS BS transceiver branches it is essential to define a relevant requirement level per branch. For BS type 1-H, the legacy requirement for the receiver is re-used with the consequence that the receiver for a AAS BS with a large array antenna the system will be over specified, which would increase cost and power consumption without providing benefit for the operator. Therefore, it would be natural to move requirements to the OTA domain for in-band receiver requirements and in-band transmitter requirements. Similar to the transmitter, additional considerations might be needed to ensure that receiver requirements are properly defined, and relevant test framework is in place as we move towards large electrically antennas including the architecture which cover such receiver beam forming and corresponding requirement interface.

In addition, the OTA test lab capabilities in the industry are not mature for AAS BS type 1-O all OTA conformance testing. Therefore, we suggest introducing a new BS type with a requirement set consisting of in-band requirements to be specified as OTA requirement while out of band requirements are defined as conducted requirements.

The general principles for establishing a complete requirement set for the new BS type are listed below:

- All carrier centric transmitter requirements are defined OTA at RIB.

- All receiver requirements are defined OTA at RIB.

- All out-of-band transmitter unwanted emission requirements and receiver spurious emission requirements are defined conducted at TAB2.

- TDD ON/OFF power is defined conducted at TAB2.

Based on the general principles above, a complete requirement set for TS 38.104 have been created in Table 3.2-2.

Table 3.2-2: New BS type requirement applicability mapping

| Requirement | BS type |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 1-C | 1-H | 1-H-enh | 1-O | 2-O |
| BS output power | 6.2 | 6.2 | NA |  |  |
| Output power dynamics | 6.3 | 6.3 | NA |  |  |
| Transmit ON/OFF power | 6.4 | 6.4 | 6.4 |  |  |
| Transmitted signal quality | 6.5 | 6.5 | NA |  |  |
| Occupied bandwidth | 6.6.2 | 6.6.2 | NA |  |  |
| ACLR | 6.6.3 | 6.6.3 | 6.6.3 |  |  |
| Operating band unwanted emissions | 6.6.4 | 6.6.4 | 6.6.4 |  |  |
| Transmitter spurious emissions | 6.6.5 | 6.6.5 | 6.6.5 |  |  |
| Transmitter intermodulation | 6.7 | 6.7 | 6.7 | NA | NA |
| Reference sensitivity level | 7.2 | 7.2 | NA |  |  |
| Dynamic range | 7.3 | 7.3 | NA |  |  |
| In-band selectivity and blocking | 7.4 | 7.4 | NA |  |  |
| Out-of-band blocking | 7.5 | 7.5 | NA |  |  |
| Receiver spurious emissions | 7.6 | 7.6 | 7.6 |  |  |
| Receiver intermodulation | 7.7 | 7.7 | NA |  |  |
| In-channel selectivity | 7.8 | 7.8 | NA |  |  |
| Performance requirements | 8 | 8 | NA |  |  |
| Radiated transmit power |  | 9.2 | 9.2 | 9.2 | 9.2 |
| OTA base station output power |  |  | 9.3 | 9.3 | 9.3 |
| OTA output power dynamics |  |  | 9.4 | 9.4 | 9.4 |
| OTA transmit ON/OFF power |  |  | NA | 9.5 | 9.5 |
| OTA transmitted signal quality |  |  | 9.6 | 9.6 | 9.6 |
| OTA occupied bandwidth |  |  | 9.7.2 | 9.7.2 | 9.7.2 |
| OTA ACLR |  | NA | NA | 9.7.3 | 9.7.3 |
| OTA Operating band unwanted emissions |  |  | NA | 9.7.4 | 9.7.4 |
| OTA transmitter spurious emission |  |  | NA | 9.7.5 | 9.7.5 |
| OTA transmitter intermodulation |  |  | NA | 9.8 | NA |
| OTA spatial emission |  | 9.9 | 9.9 | 9.9 | NA |
| OTA sensitivity |  | 10.2 | 10.2 | 10.2 | NA |
| OTA reference sensitivity level | NA |  | 10.3 | 10.3 | 10.3 |
| OTA dynamic range |  |  | 10.4 | 10.4 | NA |
| OTA in-band selectivity and blocking |  |  | 10.5 | 10.5 | 10.5 |
| OTA out-of-band blocking |  | NA | 10.6 | 10.6 | 10.6 |
| OTA receiver spurious emission |  |  | NA | 10.7 | 10.7 |
| OTA receiver intermodulation |  |  | 10.8 | 10.8 | 10.8 |
| OTA in-channel selectivity |  |  | 10.9 | 10.9 | 10.9 |
| Radiated performance requirements |  |  | 11 | 11 | 11 |

With the given general principles and example requirement set applicability, further details related to the exact mapping required for the applicability table in TS 38.104, subclause 4.6 can be worked out.

Observation 8: Based on high level principles required to define a new BS type for AAS BS there is a direct link to work package related to improvements of co-location requirements. For the new BS type to be practically relevant, work to establish relevant and practically feasible co-location requirements will be needed.

Observation 9: The new BS type (BS type 1-H enh) would require a new requirement interface (TAB2 after the RDN) to be defined.

Proposal 7: For BS RF in Rel-20, include WI to define new BS type with relevant requirement applicability and corresponding requirement interface relevant for AAS BS with large array antenna.

If a new BS type for FR1 AAS BS is defined in addition to BS type 1-H and BS type 1-O, RAN4 need to work out general principles on requirement applicability and for when different BS types are relevant, e.g. defining caps on BS types based on number of ports, number of transceivers and supported beamforming schemes for each AAS BS type, to avoid having a specification with multiple alternatives for a given design.

For the OTA receiver requirement for BS type 1-O, declarations related to the element/sub-array is needed to translate requirement levels from legacy conducted requirement levels to OTA requirements levels. With the introduction of more advanced RDN structures with fast analogue phase shifters combining several sub-arrays in more complex sub-array structures it’s not clear how the declaration defined in current specification should be done. More work is needed to clarify interfaces in relating to the RDN and the antenna array and clearly describe what type of functionality is included in the RDN.

Observation 10: OTA receiver requirements rely on declarations of the element/sub-array characteristics. The description of the RDN needs to be enhanced to support more complex beamforming schemes (e.g. hybrid beamforming) to provide guidance for required declarations.

As a starting point for WID preparation drafts for justification and objective are provided below:

Justification: To properly capture new beam-forming schemes and stimulate the industry to take the step to OTA conformance testing for AAS BS operating in the upper region of FR1, RAN4 need to define a new BS type with more OTA requirements than BS type 1-H.

Objective: Define a new BS type with more OTA requirements than BS type 1-H. As a principle it would be reasonable to define all in-band and receiver requirements as OTA requirements. If needed, update declarations to support complex beamforming schemes (e.g. hybrid beamforming including analogue and digital beamforming).

  - Re-evaluation of blocking requirement levels

The current requirement level of -43 dBm for in-band blocking was originally derived based on simulation models and assumptions originates from 1999. The technical background related to the derivation of in-band receiver blocking requirement can be traced back to UTRA and is captured in TR 25.942.

Now in Rel-20 networks are typically build in different manner as originally assumed for UTRA. For NR, the network grid is denser, UEs are using high output power and AAS BS antennas are significantly larger with dynamic beamforming. It is therefore time to re-evaluate the relevance of the requirement for in-band blocking current stated in 3GPP specification for bands in the higher region of FR1. For this work RAN4 need to establish a new set of simulation assumptions reflecting for NR relevant network deployment situations.

Another difference is also to consider wider bands, where the probability that multiple UE will transmits at the same time is larger than for narrower bands.

As an example, for the case where the in-band blocking power level seen per receiver branch modelled for AAS BS with the same physical antenna area for 2.1 GHz, 3.5 GHz and 7 GHz (different number of elements) the statistical outcome is shifted, as indicated in Figure 3.3-1.

# Figure 3.3-1: Indicative simulation results

The trend that the CDFs are shifted as function on AAS BS array size indicates that having on requirement level for all bands and AAS BS implementations may not be suitable. For AAS BS, the power received per each receiver branch will depend on the number of resources used for the complete array antenna. The power received by a single element/sub-array is always less than the power received by the complete system.

From an OTA requirement perspective, it would also be beneficial to define the requirement based on a statistical analysis in such way that the requirement can be defined based on a EIRL level (or field strength level). Then the requirement would be independent on antenna array configuration and manufacturer declarations of sub-array gain.

Observation 11: Comparing the reference used for deriving the original in-band receiver blocking requirement level to an AAS BS with large array antenna, the power levels differ significantly. This would indicate that in-band blocking requirement levels needs some further re-evaluation for bands in upper region of FR1.

Proposal 8: For BS RF in Rel-20, include WI to re-evaluate relevance of current receiver in-band blocking requirement levels for bands in the upper region of FR1. If needed establish relevant background assumptions for requirement derivation.

As a starting point for WID preparation drafts for justification and objective are provided below:

Justification: For bands in the upper region of FR1 it is reasonable to assume that AAS BS will dominate. Its therefore essential to re-evaluate and validate the in-band blocking requirement level for AAS BS in the upper region of FR1.

Objective: Based on modern models and recent parameter assumption simulate in-band blocking levels for AAS BS to re-evaluate the requirement level. If needed and relevant, consider defining the requirement as an EIRL (or field strength level) to be independent of array configuration and manufacturer declarations.

Regarding the receiver out-of-band blocking requirement, it was derived based on an interferer signal at -15 dBm which was converted to a fixed field strength level for NR BS type 1-O. Considering bands in the upper region of FR1 it would be worth to also consider the relevance of the receiver out-of-band blocking requirement level.

If the TU budget for RAN4 BS RF evolution within Rel-20 5G-Advanced don’t allow for evolution of receiver blocking requirements it is essential to include this work package in the RAN4 6G SI.

# Conclusion

For 5G-Advanced Rel-20 UE RF focus on extending the support for HPUE and for BS RF evolution work packages improving requirement for AAS BS, especially for the bands in the upper region of FR1. With these improvements, 5G will create a solid foundation for 6G.

Analysing the work packages proposed, we observe following:

Observation 1: A significant amount of study is needed for the UE power domain enhancements proposal.

Observation 2: The current concept used for BS type 1-O co-location requirements captured in TS 38.104, clause 4.9 is not practical for conformance testing of product operating above 3.5 GHz. Hence, BS type 1-O is not a feasible option for AAS BS implementations above 3.5 GHz.

Observation 3: The height difference between to co-located base stations at different bands (900 MHz and 3.5 GHz) is far larger than what is captured in the CTLA description in current specification.

Observation 4: It can be noticed that information provided to describe the CLTA in the specification is overlapping and contradicting. The description provides information on both vertical radiating dimension and vertical beamwidth for the out-of-band case. Both conditions cannot be met at the same time.

Observation 5: The CLTA antenna gain is indirectly specified via the horizontal beam width and the vertical radiating dimension, while the antenna efficiency is not specified. This means that large isolation between the test object and CLTA can be created by having a CLTA with poor antenna efficiency.

Observation 6: The CLTA antenna efficiency is not captured by the description in current specification.

Observation 7: Different types of CLTA from different vendors with very different characteristics would risk making the co-location requirements use-less. To guarantee co-location, the CLTA used should be of similar type and have the same characteristics.

Observation 8: Based on high level principles required to define a new BS type for AAS BS there is a direct link to work package related to improvements of co-location requirements. For the new BS type to be practically relevant, work to establish relevant and practically feasible co-location requirements will be needed.

Observation 9: The new BS type (BS type 1-H enh) would require a new requirement interface (TAB2 after the RDN) to be defined.

Observation 10: OTA receiver requirements rely on declarations of the element/sub-array characteristics. The description of the RDN needs to be enhanced to support more complex beamforming schemes (e.g. hybrid beamforming) to provide guidance for required declarations.

Observation 11: Comparing the reference used for deriving the original in-band receiver blocking requirement level to an AAS BS with large array antenna, the power levels differ significantly. This would indicate that in-band blocking requirement levels needs some further re-evaluation for bands in upper region of FR1.

Based on the observations we propose following:

Proposal 1: For UE RF, consider HPUE as UE RF Rel-20 5G-Advanced work package. Consider the following candidates; further reduce if possible:

- 4x26 dBm (FWA)

- 2x29 dBm (FWA)

- PC1.5 FDD (Handheld)

- 1x29 dBm (Handheld)

    - Potentially restrict to TDD.

Proposal 2: For UE RF, not consider power domain enhancements for 5G-Advanced.

Proposal 3: For UE RF further study band combination optimization, in particular MSD optimization as part of the 6G SI. If measures are identified that are straightforward for 5G, consider introducing those for 5G.

Proposal 4: For UE RF, not consider power class optimization for 5G-Advanced.

Proposal 5: For UE RF and MPR/A-MPR, in 5G-Advanced consider whether there are certain cases where a specific optimization of A-MPR could make sense.

Proposal 6: For BS RF in Rel-20 define a work package with clear goal to resolve open issues identified for BS type 1-O co-location requirements for bands in upper region of FR1.

Proposal 7: For BS RF in Rel-20, include WI to define new BS type with relevant requirement applicability and corresponding requirement interface relevant for AAS BS with large array antenna.

Proposal 8: For BS RF in Rel-20, include WI to re-evaluate relevance of current receiver in-band blocking requirement levels for bands in the upper region of FR1. If needed establish relevant background assumptions for requirement derivation.

# References

[1]	RP-250817, “Moderator's summary for RAN4 led REL-20 topics”, RAN4 Chair (Huawei)

[2] 	R4-2402475, “NR BS RF enhancements in Rel-19”, Ericsson

[3]	R4-2319398, “Background and technical challenges with BS RF co-location requirements in relation to development of NCR RF co-location requirements”, Ericsson

[4]	R4-2503725, “Considerations on Rel-20 BS RF package objectives”, Ericsson

[5]	R4-2507058, “Considerations on the Rel-20 5G-A RAN4 package”, Ericsson