3GPP TSG RAN WG1 #121	R1-2505059

# St Julian’s, Malta, May 19 – 23, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.211 | CR | 0576 | rev | - | Current version: | 18.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of LTE-based 5G Broadcast Phase 2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_terr_bcast_Ph2-Core |  |  |  |  |  | Date: |  |  | 2025-06-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduction of LTE-based 5G broadcast phase 2 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding frequency-domain interleaveing for PMCH |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support for LTE-based 5G broadcast phase 2 |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | 38.212, 38.213 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 6.5	Physical multicast channel

The physical multicast channel shall be processed and mapped to resource elements as described in clause 6.3 with the following exceptions:

-	No transmit diversity scheme is specified.

-	Layer mapping and precoding shall be done assuming a single antenna port and the transmission shall use antenna port 4.

-	The PMCH can only be transmitted in the MBSFN region. For PMCH with Δf  = 15 kHz, the index  in the first slot in the MBSFN subframe fulfils l≥lPMCHStart where lPMCHStart is equal to the value given by the higher layer parameter non-MBSFNregionLength [9].

-	The PMCH shall use extended cyclic prefix.

-	The PMCH is not mapped to resource elements used for transmission of MBSFN reference signals.

-	In clause 6.3.1, for Δf = 1.25 kHz and Δf ≈ 0.37 kHz, the scrambling generator shall be initialised at the start of each slot.

-	For Δf=182944Ts≈0.37 kHz the following exception applies to clause 6.3.5:

-	The text "which meet all of the following criteria in the current subframe" shall be replaced by "which meet all of the following criteria in the current slot"

-	The mapping to resource elements k,l on antenna port p not reserved for other purposes shall be in increasing order of first the index k over the assigned physical resource blocks and then the index l.

-	For PMCH symbols belonging to an MBSFN area with NRBPMCH configured, NRBDL shall be replaced by NRBPMCH in clauses 6.3, 6.10.2, and 6.12.

-	If cyclic shifting is configured by the higher-layer parameter XXX,

- 	cyclic shifting according to clause 6.5.1 shall be applied

-	modulation in clause 6.3.2 shall use b0, …, bMbit-1 instead of b0, …, bMbit-1

-	If frequency-domain interleaving is configured by the higher-layer parameter XXX,

-	block interleaving according to clause 6.5.2 shall be applied

-	mapping to resource elements in clause 6.3.5 shall use yp0, …, yp(Msymbap-1) instead of yp0, …, yp(Msymbap-1)

### 6.5.1	Cyclic shift for PMCH

The input to the cyclic shift is the block of bits b0, …, bMbit-1 defined in clause 6.3.1 and the ouput is a block of bits b0, …, bMbit-1.

The block of bits b0, …, bMbit-1 shall be cyclically shifted by Xi bits to form the block b0, …, bMbit-1 according to

bn=b(n-Xi mod Mbit)

where

-	Xi is given by

Xi=r=C-SiC-1Er

Si=iα mod C

-	i∈0, 1, …, N-1 denotes the ith subframe to which the transport block is mapped

-	N is the number of subframes to which the transport block is mapped

-	Er is the number of bits in the rth codeblock within a subframe as defined in TS 36.212 [3]

-	C is the number of codeblocks in the transport block

-	α is given by

-	if the higher-layer parameter XXX is set to alphaOne

α=1

-	if the higher-layer parameter XXX is set to alphaOther

α=CNL

where L is the number of OFDM symbols used for the PMCH transmission

### 6.5.2	Frequency-domain interleaving

The input to the interleaver is the the block of complex-valued symbols yp0, …, yp(Msymbap-1)  defined in clause 6.3.4.1 and the output from the interleaver is a block of complex-valued symbols yp0, …, yp(Msymbap-1).

Interleaving shall be performed as follows:

-	Divide the block of complex-valued symbols yp0, …, yp(Msymbap-1) into L sets such that set l∈0,1,…,L-1 contains the complex-valued symbols to be mapped to OFDM symbol l

-	For each of the sets l=0, 1, …, L-1

-	Set the number of columns K of the block interleaver to

K=Cgcd⁡(L,C)

where

-	L is the number of OFDM symbols used for the PMCH transmission

-	C is the number of codeblocks as defined in clause 5.1.2 of [36.212]

-	gcd⁡(u,v) denotes the greatest common divisor of u and v

-	Set the number of rows of the block interleaver R to the smallest integer fulfilling

Ml≤RK

where

-	Ml is the number of resources elements available for mapping of complex-valued modulation symbols in OFDM symbol l

-	Write the modulation symbols ypi in set l column-wise into the block interleaver in increasing order of i, starting with column 0 and row 0 and in increasing order of first the row number and then the column number. If Ml<RK, append <NULL> elements to fill the size-RK block interleaver.

-	Permute the rows such that row ri is moved to row i where

ri=ri-1+dri-1≤R-d-1idRotherwise
d=RRRK-K+1
r0=0
1≤i≤R-1

-	Cyclically shift the columns of row r with an offset sr given by

sr=(-1)rr+12mod K
r=0,1,…,R-1

-	Read the interleaved modulation symbols  ypi in set l row-wise from the block interlaver in increasing order of i, starting with column 0 and row 0 and in increasing order of first the column number and then the row number, discarding any <NULL> elements.