3GPP TSG-RAN WG1 Meeting # 121	R1-2504967

# St. Julian's, Malta, May 19th – May 23rd 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | 1448 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of LTE-based 5G Broadcast Phase 2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Motorola Mobility |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_terr_bcast_Ph2-Core |  |  |  |  |  | Date: |  |  | 2025-05-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduction of LTE-based 5G Broadcast Phase 2 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Support for LTE-based 5G Broadcast Phase 2 according to RAN1 agreements: - time interleaving for MBMS-dedicated cells with one transport block mapped to multiple non-consecutive subframes - TBS scaling and determination |  |  |  |  |  |  |  |  |
| T |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support of time-interleaving for LTE-based 5G Broadcast Phase 2 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 36.211, TS 36.212 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 11	Physical Multicast Channel (PMCH) related procedures

## 11.1	UE procedure for receiving the PMCH

A UE is not expected to receive PMCH with  (defined in [3]) other than  in an MBSFN subframe with non-zero-size non-MBSFN region.

A UE is not expected to receive PMCH with in an MBSFN subframe with zero-size non-MBSFN region.

A UE is not required to receive the non-MBSFN region in an MBSFN subframe carrying PMCH with  and belonging to an MBSFN area with higher layer parameter pmch-Bandwidth (NRBPMCH) configured.

The UE shall decode the PMCH when configured by higher layers. The UE may assume that an eNB transmission on the PMCH is performed according to Clause 6.5 of [3].

If a UE is configured with higher layer parameter pmch-TimeInterleaving-N (NTimePMCH), higher layer parameter pmch-TimeInterleaving-M (MTimePMCH), and NTimePMCH>1, for PMCH transmitted in a given block of (NTimePMCH∙MTimePMCH) MBSFN subframes indexed 0,1,…NTimePMCH∙MTimePMCH-1 excluding MBSFN subframes for MCCH and MSI, MBSFN subframe indices m+n⋅MTimePMCH with m=0,1,…MTimePMCH-1,  n=0,1,…NTimePMCH-1  are associated with TBm of the PMCH with redundancy version rvidx=n.

The for the PMCH is configured by higher layers. To determine the modulation order () and TBS index () used in the PMCH the UE shall

-	if the UE is configured by higher layers to decode the PMCH based on QPSK, 16QAM, 64QAM, and 256QAM

-	if the PMCH is with and the UE is configured with higher layer parameter timeSeperation with value '4'

-	use and Table 11.1-2 for determining the modulation order () and TBS index ()

-	otherwise,

-	useand Table 7.1.7.1-1A for determining the modulation order () and TBS index ()

-	otherwise,

-	if the PMCH is with and the UE is configured with higher layer parameter timeSeperation with value '4'

-	use and Table 11.1-1 for determining the modulation order () and TBS index ()

-	otherwise,

-	use and Table 7.1.7.1-1 for determining the modulation order () and TBS index ().

The UE shall then follow the procedure in Clause 7.1.7.2.1 to determine the transport block size, assuming is equal to NRBPMCH if PMCH belongs to an MBSFN area with higher layer parameter pmch-Bandwidth configured,  otherwise.

For PMCH with  the UE shall scale the derived transport block size by , then round to the closest valid transport block size in the union of Table 7.1.7.2.1-1 and Table 7.1.7.2.4-1 by including, in the rounding procedure, only the TBS entries in Table 7.1.7.2.1-1 with ITBS≤33B and the entries in 7.1.7.2.4-1 for which the TBS_L1 is present in Table 7.1.7.2.1-1 with ITBS≤33B if the UE is configured by higher layers to decode the PMCH based on QPSK, 16QAM, 64QAM, and 256QAM; and only the TBS entries in Table 7.1.7.2.1-1 with ITBS≤26A and the entries in 7.1.7.2.4-1 for which the TBS_L1 is present in Table 7.1.7.2.1-1 with ITBS≤26A otherwise. In case the scaled TBS has the same distance to two valid transport block sizes, it is rounded to the larger transport block size.

For PMCH and the UE configured with higher layer parameter pmch-TimeInterleaving-N (NTimePMCH) and NTimePMCH>1, the UE shall scale the derived transport block size by α=NTimePMCH, then round to the closest valid transport block size in the union of Table 7.1.7.2.1-1, Table 7.1.7.2.2-1, Table 7.1.7.2.4-1 and Table 7.1.7.2.5-1. In case the scaled TBS has the same distance to two valid transport block sizes, it is rounded to the larger transport block size.

The UE shall set the redundancy version to 0 for the PMCH if the UE is not configured with higher layer parameter pmch-TimeInterleaving-N (NTimePMCH).

A UE may optionally report parameters mbms-MaxBW (T), mbms-ScalingFactor1dot25 (A(1.25) , mbms-ScalingFactor7dot5 (A(7.5), mbms-ScalingFactor2dot5 (A(2.5), and mbms-ScalingFactor0dot37 (A(0.37) to indicate a limitation on baseband capability provided by the following inequality

T≥c=1C(RcBcIc(15kHz +A(7.5BcIc(7.5kHz +A(2.5 BcIc(2.5kHz +A(1.25 BcIc(1.25kHz +A(0.37 BcIc(0.37kHz )

where

-	T is the maximum bandwidth capability of the UE, indicated by mbms-MaxBW:

-	if mbms-MaxBW is set to implicitValue, then

T=roundMaximum number of DL-SCH transport block bits received within a TTIMaximum TBS for 2 layers for the maximum supported modulation scheme× 20MHz

where the numerator and the denominator correspond to the corresponding columns of the maximum indicated ue-CategoryDL in [12].

-	if mbms-MaxBW is set to explicitValue, then T=explicitValue× 40MHz

-	C is the number of serving cells the UE is configured with, or receiving PMCH from.

-	Rc is the number of spatial layers the UE can receive in the c-th serving cell according to MIMO-CapabilityDL.

-	Bc is the bandwidth (in MHz) of the c-th serving cell.

-	Bc=10 if the UE is receiving PMCH belonging to an MBSFN area with higher layer parameter pmch-Bandwidth configured for the c-th serving cell.

-	If the UE is receiving PMCH with 1.25kHz numerology in serving cell c, then

-	Ic(0.37kHz =0,Ic(1.25kHz =1, Ic(2.5kHz =0,Ic(7.5kHz =0,Ic(15kHz =0

-	else, if the UE is receiving PMCH with 7.5kHz numerology in serving cell c, then

-	Ic(0.37kHz =0,Ic(1.25kHz =0, Ic(2.5kHz =0,Ic(7.5kHz =1,Ic(15kHz =0

-	else, if the UE is receiving PMCH with 2.5kHz numerology in serving cell c, then

-	Ic(0.37kHz =0,Ic(1.25kHz =0, Ic(2.5kHz =1,Ic(7.5kHz =0,Ic(15kHz =0

-	else, if the UE is receiving PMCH with 0.37kHz numerology in serving cell c, then

-	Ic(0.37kHz =1,Ic(1.25kHz =0, Ic(2.5kHz =0,Ic(7.5kHz =0,Ic(15kHz =0

-	else,

-	Ic(0.37kHz =0,Ic(1.25kHz =0, Ic(2.5kHz =0,Ic(7.5kHz =0,Ic(15kHz =1

Table 11.1-1: Modulation and TBS index table for PMCH

| MCS Index | Modulation Order | TBS Index |
| --- | --- | --- |
| 0 | 2 | 0 |
| 1 | 2 | 1 |
| 2 | 2 | 2 |
| 3 | 2 | 3 |
| 4 | 2 | 4 |
| 5 | 2 | 5 |
| 6 | 2 | 6 |
| 7 | 2 | 7 |
| 8 | 2 | 8 |
| 9 | 2 | 9 |
| 10 | 2 | 10 |
| 11 | 4 | 10 |
| 12 | 4 | 11 |
| 13 | 4 | 12 |
| 14 | 4 | 13 |
| 15 | 4 | 14 |
| 16 | 4 | 15 |
| 17 | 4 | 16 |
| 18 | 4 | 17 |
| 19 | 4 | 18 |
| 20 | 4 | 19 |
| 21 | 6 | 19 |
| 22 | 6 | 20 |
| 23 | 6 | 21 |
| 24 | 6 | 22 |
| 25 | 6 | 23 |
| 26 | 6 | 24 |
| 27 | 6 | 25 |
| 28 | 6 | 26/26A |
| 29 | 2 | reserved |
| 30 | 4 |  |
| 31 | 6 |  |

Table 11.1-2: Modulation and TBS index table 2 for PMCH

| MCS Index | Modulation Order | TBS Index |
| --- | --- | --- |
| 0 | 2 | 0 |
| 1 | 2 | 2 |
| 2 | 2 | 4 |
| 3 | 2 | 6 |
| 4 | 2 | 8 |
| 5 | 2 | 10 |
| 6 | 4 | 11 |
| 7 | 4 | 12 |
| 8 | 4 | 13 |
| 9 | 4 | 14 |
| 10 | 4 | 15 |
| 11 | 4 | 16 |
| 12 | 4 | 17 |
| 13 | 4 | 18 |
| 14 | 4 | 19 |
| 15 | 6 | 20 |
| 16 | 6 | 21 |
| 17 | 6 | 22 |
| 18 | 6 | 23 |
| 19 | 6 | 24 |
| 20 | 6 | 25 |
| 21 | 8 | 27 |
| 22 | 8 | 28 |
| 23 | 8 | 29 |
| 24 | 8 | 30 |
| 25 | 8 | 31 |
| 26 | 8 | 32 |
| 27 | 8 | 33/33A/33B |
| 28 | 2 | reserved |
| 29 | 4 |  |
| 30 | 6 |  |
| 31 | 8 |  |