3GPP TSG-WG Meeting #121	R1- 2504978

# St. Julian's, Malta, May 19–23, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.215 | CR | 0072 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of Rel-19 AI/ML air-interface for TS38.215 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_AIML_air-Core |  |  |  |  |  | Date: |  |  | May 30, 2025 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Addition of new measurements based on AI/ML air-interface, based on RAN1 agreements from RAN1 #116 to RAN1 #120bis. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Defined new measurements SGCS under Section 5.1, and UL SRS-TDCT and UL SRS-TDCP under section 5.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | AI/ML air-interface measurement is not supported on Release 19 NR. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1, 5.1.1, 5.1.2, 5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--- unchanged text omitted ---

### 5.1.1	SS reference signal received power (SS-RSRP)

| Definition | SS reference signal received power (SS-RSRP) is defined as the linear average over the power contributions (in [W]) of the resource elements that carry secondary synchronization signals. The measurement time resource(s) for SS-RSRP are confined within SS/PBCH Block Measurement Time Configuration (SMTC) window duration. If SS-RSRP is used for L1-RSRP as configured by reporting configurations as defined in TS 38.214 [6], the measurement time resources(s) restriction by SMTC window duration is not applicable. For SS-RSRP determination demodulation reference signals for physical broadcast channel (PBCH) and, if indicated by higher layers, CSI reference signals in addition to secondary synchronization signals may be used. SS-RSRP using demodulation reference signal for PBCH or CSI reference signal shall be measured by linear averaging over the power contributions of the resource elements that carry corresponding reference signals taking into account power scaling for the reference signals as defined in TS 38.213 [5]. If SS-RSRP is not used for L1-RSRP, the additional use of CSI reference signals for SS-RSRP determination is not applicable. SS-RSRP shall be measured only among the reference signals corresponding to SS/PBCH blocks with the same SS/PBCH block index and the same physical-layer cell identity. If SS-RSRP is not used for L1-RSRP and higher-layers indicate certain SS/PBCH blocks for performing SS-RSRP measurements, then SS-RSRP is measured only from the indicated set of SS/PBCH block(s). For frequency range 1, the reference point for the SS-RSRP shall be the antenna connector of the UE. For frequency range 2, SS-RSRP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. For frequency range 1 and 2, if receiver diversity is in use by the UE, the reported SS-RSRP value shall not be lower than the corresponding SS-RSRP of any of the individual receiver branches. |
| --- | --- |
| Applicable for | If SS-RSRP is used for L1-RSRP or predicted RSRP, RRC_CONNECTED intra-frequency. Otherwise, RRC_IDLE intra-frequency, RRC_IDLE inter-frequency, RRC_INACTIVE intra-frequency, RRC_INACTIVE inter-frequency, RRC_CONNECTED intra-frequency, RRC_CONNECTED inter-frequency |

NOTE 1:	The number of resource elements within the measurement period that are used by the UE to determine SS-RSRP is left up to the UE implementation with the limitation that corresponding measurement accuracy requirements have to be fulfilled.

NOTE 2:	The power per resource element is determined from the energy received during the useful part of the symbol, excluding the CP.

### 5.1.2	CSI reference signal received power (CSI-RSRP)

| Definition | CSI reference signal received power (CSI-RSRP), is defined as the linear average over the power contributions (in [W]) of the resource elements of the antenna port(s) that carry CSI reference signals configured for RSRP measurements within the considered measurement frequency bandwidth in the configured CSI-RS occasions. For CSI-RSRP determination CSI reference signals transmitted on antenna port 3000 according to TS 38.211 [3] shall be used. If CSI-RSRP is used for L1-RSRP, CSI reference signals transmitted on antenna ports 3000, 3001 can be used for CSI-RSRP determination. For intra-frequency CSI-RSRP measurements, if the measurement gap is not configured, UE is not expected to measure the CSI-RS resource(s) outside of the active downlink bandwidth part. For frequency range 1, the reference point for the CSI-RSRP shall be the antenna connector of the UE. For frequency range 2, CSI-RSRP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. For frequency range 1 and 2, if receiver diversity is in use by the UE, the reported CSI-RSRP value shall not be lower than the corresponding CSI-RSRP of any of the individual receiver branches. |
| --- | --- |
| Applicable for | If CSI-RSRP is used for L1-RSRP or predicted RSRP, RRC_CONNECTED intra-frequency. Otherwise, RRC_CONNECTED intra-frequency, RRC_CONNECTED inter-frequency |

NOTE 1:	The number of resource elements within the considered measurement frequency bandwidth and within the measurement period that are used by the UE to determine CSI-RSRP is left up to the UE implementation with the limitation that corresponding measurement accuracy requirements have to be fulfilled.

NOTE 2:	The power per resource element is determined from the energy received during the useful part of the symbol, excluding the CP.

--- unchanged text omitted ---

### 5.1.55	Squared Generalized Cosine Similarity (SGCS)

| Definition | Square Generalized Cosine Similarity (SGCS) value is defined in Clause 5.2.1.4.6 of [6, TS38.214]. For frequency range 1 and 2, if receiver diversity is in use by the UE, the reported SGCS value shall be no lower than the minimum and no higher than the maximum measured values across the receiver branches. For frequency range 1, the reference point for the SGCS shall be the antenna connector of the UE. For frequency range 2, SGCS shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. |
| --- | --- |
| Applicable for | RRC_CONNECTED |

--- unchanged text omitted ---

### 5.2.9	UL SRS time domain channel timing (UL SRS-TDCT)

| Definition | UL SRS time domain channel timing (UL SRS-TDCT) is defined as Nt values of timing information of the estimated channel response in time domain, Tn, corresponding to the resource elements carrying sounding reference signals (SRS), the Nt values are derived from Nt consecutive time instances with timing granularity 2k∙Tc that start from the first detected path rounded down with timing granularity 2k∙Tc, where Nt and Nt are given by higher layers and Tc is given in Clause 4.1 of [3, TS38.211], and k is the reporting granularity factor given by higher layer parameter timingReportingGranularityFactor. The Nt values of timing information are selected such that they correspond to the time instances with the highest power values among the Nt consecutive time instances where a power value for a time instance is determined as the power of linear average of the estimated channel response at time instance Tn. For frequency range 1 and 2, if receiver diversity is in use by the gNB for power of linear average of the estimated channel response measurement: - The measured received signal power, P1, value for the selected first time instance, T1, shall not be lower than the corresponding received signal power for the selected time instance, T1, of any of the individual receiver branches and the received signal power, Pn, shall be with the same receiver branch(es) as applied for the selected first time instance, T1. The timing information are reported relative to reference time, Tref. The reference time, Tref, is equal to the UL RTOA reference time T0+tSRS, where - T0 is the nominal beginning time of SFN 0 provided by SFN Initialization Time [15, TS 38.455] - tSRS=10nf+nsf×10-3, where nf and nsf are the system frame number and the subframe number of the SRS, respectively. The reference point for UL SRS-TDCT shall be: - for type 1-C base station TS 38.104 [9]: the Rx antenna connector, - for type 1-O or 2-O base station TS 38.104 [9]: the Rx antenna (i.e. the centre location of the radiating region of the Rx antenna), - for type 1-H base station TS 38.104 [9]: the Rx Transceiver Array Boundary connector. |
| --- | --- |

### 5.2.10	UL SRS time domain channel power (UL SRS-TDCP)

| Definition | UL SRS time domain channel power (UL SRS-TDCP) is defined as a number of Nt values of power of the estimated channel response in time domain, Pn, where, for a given n, Pn is defined as the power of the linear average of the estimated channel response at time instance Tn, for n = 1, …, Nt, where time instance Tnis defined in Clause 5.2.9, from resource elements carrying sounding reference signals (SRS).. UL SRS-TDCP measurements may not be reported without reporting of UL SRS-TDCT measurements. The reference point for UL SRS-TDCP shall be: - for type 1-C base station TS 38.104 [9]: the Rx antenna connector, - for type 1-O or 2-O base station TS 38.104 [9]: the Rx antenna (i.e. the centre location of the radiating region of the Rx antenna), - for type 1-H base station TS 38.104 [9]: the Rx Transceiver Array Boundary connector. For frequency range 1 and 2, if receiver diversity is in use by the gNB for UL SRS-TDCP measurements: - The reported received signal power, P1, value for the selected first time instance, T1, shall not be lower than the corresponding received signal power for the selected time instance, T1, of any of the individual receiver branches and the received signal power, Pn, shall be provided for the same receiver branch(es) as applied for the selected first time instance, T1. |
| --- | --- |

--- unchanged text omitted ---