3GPP TSG RAN Meeting #108	RP-251625

Prague, Czech Republic, June 9-13, 2025

Agenda Item:	16.1

Source: 	CATT, CICTCI

Title: 	Inputs on TP of 6G Scenarios and Requirements

Document for:	Discussion and Decision

Introduction

In RAN#107, 6G RAN SID on 6G scenarios and requirements were approved in [1]. It was concluded to have one external TR 39.xxx capturing the outcome of this study item.

In this contribution, Inputs on TP of 6G Scenarios and Requirements are provided.

References

[1] RP-250810	revised SID: Study on 6G scenarios and requirements

............

Text Proposal starts

# 4.	Deployment scenarios

High-level descriptions on deployment scenarios including carrier frequency, aggregated system bandwidth, network layout / ISD, BS / UE antenna elements, UE distribution / speed and service profile are proposed in this TR. It is assumed that more detailed attributes and simulation parameters, for example, the channel model, BS / UE Tx power, number of antenna ports, etc. should be defined in WGs study item.

## 4.1	Indoor hotspot

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Around 4 GHz or Around 7 GHz or Around 30 GHz |
| Aggregated system bandwidth NOTE1 | Around 7GHz: Up to 400MHz (DL+UL) Around 4GHz:Up to 200MHz(DL+UL) Around 30GHz:1GHz (DL+UL) |
| Layout | Single layer: - Indoor floor (Open office) |
| ISD | 20m (Equivalent to 12TRxPs per 120m x 50m) |
| BS antenna elements NOTE2 | Around 30GHz: Up to 1024/2048 Tx and Rx antenna elements Around 7GHz: Up to 1024 Tx and Rx antenna elements Around 4GHz: Up to 256 Tx and Rx antenna elements |
| UE antenna elements NOTE2 | Around 30GHz or Around 70GHz: Up to 32 Tx and Rx antenna elements Around 4GHz: Up to 4 Tx and Rx antenna elements Around 7GHz: Up to 8 Tx and Rx antenna elements |
| User distribution and UE speed | 100% Indoor, 3km/h, For communication, 10 users per TRxP; For positioning (if this deployment is applied), uniformly distributed over the horizontal evaluation area, which is at least the convex hull of the horizontal BS deployment. |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. For certain KPIs, full buffer traffic is desirable to enable comparison with IMT-2020 values. |

NOTE1:	The aggregated system bandwidth is the total bandwidth typically assumed to derive the values for some KPIs such as area traffic capacity and user experienced data rate. It is allowed to simulate a smaller bandwidth than the aggregated system bandwidth and transform the results to a larger bandwidth. The transformation method should then be described, including the modelling of power limitations. The aggregated system bandwidth is simulation bandwidth for positioning.

NOTE2:	The maximum number of antenna elements is a working assumption. 3GPP needs to strive to meet the target with typical antenna configurations.

## 4.2	Indoor Factory

The following deployment scenarios and parameters are for both positioning and sensing.

Table 4.x: Common scenario parameters applicable for Indoor Factory (for positioning and sensing)

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | For positioning: Around 4 GHz For sensing: Around 30GHz |
| Simulation bandwidth NOTE1 | Around 4GHz: Up to 100MHz/200MHz Around 30GHz: Up to 400MHz |
| Layout | InF-SH: The floor has a surface of 300 m × 150 m and 18 BSs/sites which are placed in D=50 meter spacing as shown in the following figure. |
| ISD | 50m |
| BS antenna elements NOTE2 | Around 4GHz: Up to 256 Tx and Rx antenna elements; Around 30GHz: Up to 1024/2048 Tx and Rx antenna elements |
| UE antenna elements NOTE2 | Around 4GHz: Up to 4 Tx and Rx antenna elements Around 30GHz: Up to 32 Tx and Rx antenna elements |
| User distribution and UE speed | 100% indoor, 3 km/h Uniformly distributed over the horizontal evaluation area, which is at least the convex hull of the horizontal BS deployment. |
| Sensing target distribution and speed | For human as sensing target: 100% indoor, 3 km/h Uniformly distributed over the horizontal evaluation area, which is at least the convex hull of the horizontal BS deployment |

NOTE1:	For both positioning and sensing.

NOTE2:	The maximum number of antenna elements is a working assumption. 3GPP needs to strive to meet the target with typical antenna configurations.

## 4.3	Dense Urban

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Around 4 GHz or Around 7 GHz (single layers) |
| Aggregated system bandwidth NOTE1 | Around 7GHz: Up to 400MHz (DL+UL) Around 4GHz:Up to 200MHz(DL+UL) Around 30GHz:1GHz (DL+UL) |
| Layout | Single layer: - Macro layer: Hex. Grid |
| ISD | Macro layer: 200m |
| BS antenna elements NOTE2 | Around 30GHz: Up to 1024/2048 Tx and Rx antenna elements Around 7GHz: Up to 1024 Tx and Rx antenna elements Around 4GHz: Up to 256 Tx and Rx antenna elements |
| UE antenna elements NOTE2 | Around 30GHz or Around 70GHz: Up to 32 Tx and Rx antenna elements Around 4GHz: Up to 4 Tx and Rx antenna elements for handhold equipment, Up to 8 Tx and Rx antenna elements for FWA Around 7GHz: Up to 8 Tx and Rx antenna elements |
| User distribution and UE speed | Uniform/macro TRxP, 10 users per TRxP 80% indoor (3km/h), 20% outdoor (30km/h) |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. For certain KPIs, full buffer traffic is desirable to enable comparison with IMT-2020 values. |

NOTE1:	The aggregated system bandwidth is the total bandwidth typically assumed to derive the values for some KPIs such as area traffic capacity and user experienced data rate. It is allowed to simulate a smaller bandwidth than the aggregated system bandwidth and transform the results to a larger bandwidth. The transformation method should then be described, including the modelling of power limitations.

NOTE2:	The maximum number of antenna elements is a working assumption. 3GPP needs to strive to meet the target with typical antenna configurations.

## 4.4	Urban Macro

The following deployment scenarios and parameters are for sensing.

Table 4.x: Common scenario parameters applicable for Urban macro (for sensing)

| Attributes | Values or assumptions (for sensing) |
| --- | --- |
| Carrier Frequency | Around 7 GHz |
| Simulation bandwidth | Around 7GHz: Up to 100MHz/200MHz |
| Layout | Single layer: - Hex. Grid |
| ISD | 500m |
| BS antenna elements NOTE1 | Around 7GHz: Up to 1024 Tx and Rx antenna elements |
| Sensing target distribution and mobility | For UAV as sensing target (100% outdoor): -Horizontal distribution: 2 targets uniformly distributed within center cell -Vertical distribution: Fixed height value chosen from {25, 50, 100, 200, 300} -Horizontal mobility: 30 km/h with random direction. -Vertical mobility: 0 km/h |

NOTE1:	The maximum number of antenna elements is a working assumption. 3GPP needs to strive to meet the target with typical antenna configurations.

…..

## 4.x	Satellite Access

This deployment scenario is defined to allow for the provision of services with satellite system, targeting to provide ubiquitous connection integrated with terrestrial network. The supported services via the Satellite system are not limited to just data and voice, but also for others such as machine type communications, broadcast and other delay tolerant services. New services including positioning service and other services are also considered.

Some of its attributes are listed in Table 4.x.

Table 4.x: Examples for Satellite Deployment

| Attributes | Deployment-1 | Deployment-2 | Deployment-3 |
| --- | --- | --- | --- |
| Carrier Frequency NOTE1 | <6GHz GHz for both DL and UL | Around 10 GHz for DL and UL | Around 20 GHz for DL Around 30 GHz for UL |
| Duplexing | FDD/TDD/HD-FDD | FDD/HD-FDD | FDD/HD-FDD |
| Satellite architecture | Bent-pipe, NOTE2 On-Board processing | Bent-pipe, On-Board Processing | Bent-pipe, On-Board Processing |
| Typical satellite system positioning in the 6G architecture | Access network | Access/Backhaul network | Access/Backhaul network |
| System Bandwidth (DL + UL) | Up to 2*10 MHz | Up to 2*200 MHz | Up to 2*400 MHz |
| Satellite Orbit | GEO, MEO,LEO | LEO, MEO, GEO | LEO, MEO, GEO |
| UE Distribution | 100% Outdoors | 100% Outdoors | 100% Outdoors |
| UE Mobility | Fixed, Portable, Mobile NOTE3 | Fixed, Portable, Mobile | Fixed, Portable, Mobile |

NOTE1:	The carrier frequencies noted here are for evaluation purpose only, satellites are deployed in wide range of frequency bands including L band (1-2GHz), S band (2-4GHz), C band (3.4-6.725 GHz), Ku band (10.7-14.8 GHz) and Ka band (17.3-21.2 GHz, 27.0-31.0 GHz). Here the around xGHz is used to denote the carrier frequency close to xGHz rather than used as band proxy to denote a band range.

NOTE2:	Bent pipe refers to the architecure where the satellite transponders are transparent—only amplify and change frequency but preserve the waveform. On Board Processing satellite transponders incorperate regeneration — including modulating and coding the waveform.

NOTE3:	Mobile consitutes of both hand-helds and other moving platform receivers such as automobiles, ships, planes etc. Currently the hand-helds are limited to L and S bands but the research is ongoing to support higher bands.

# 5.	Requirements and key technical principles

Editor note: aspects related to IMT-2030 TPRs will be included in this section. Those would wait the completion or stability of Report ITU-R M.[IMT-2020. TECH PERF REQ] and Report ITU-R M.[IMT-2020.EVAL]. The followings are not included in above two reports:

## 5. A	Network energy efficiency

Network energy efficiency shall be considered as a basic principle in the 6G design.

In case of quantitative requirement, the following shall be applied:

- Network energy efficiency is to support radio access network energy saving in relation to the traffic capacity provided.

- The network energy saving is the relative energy savings between the selected case(s) and a reference case. The selected case(s) is the case of energy saving technology to be evaluated. The reference case is the case with specific parameter set of that the energy saving technology to be evaluated is not used.

Evaluation methodology is based on analysis or system level evaluation. The following models are considered as starting point for the discussion in RAN WGs.

The E(energy consumption) could be defined as average energy consumption during the observation time.

E=i=0nEiTtotal (in energy unit/time unit)

Ei (in energy unit) is the energy computation category at network or device for transmission and/or reception in loaded and unloaded cases; As an illustration, it is assumed at least three energy consumption categories:

Where,

Eon 	is the energy consumption for active transmission and/or reception at the network or device during the observation/evaluation time.

Eoff 	is the energy consumption at network or device when there is no transmission and/or reception.

Etransition 	is the energy consumption at network or device during transition between on and off.

Ttotal (in time unit)	is the total observation/evaluation time.

## 5. B Positioning

6G should enable, and improve if suitable, state-of-art positioning techniques, such as RAT-dependent(Cell-ID, E-Cell ID, DL-TDOA, UL-TDOA, Multi-RTT, UL-AoA, DL-AoD etc.) based on the signals, measurements, and positioning methods from 5G NR, and RAT- independent (GNSS, Bluetooth, WLAN, Terrestrial Beacon Systems (TBS), sensors, etc.).

The 6G positioning shall exploit high bandwidth, massive antenna systems, network architecture/ functionalities (e.g. heterogeneous networks, broadcast, MBMS) and deployment of massive number of devices. 6G NR positioning shall support indoors and outdoors use cases. 6G NR RAT-dependent positioning shall support unified reference signal design for both positioning and communication, and exchange of positioning measurements/result and communication measurements between LMF and RAN. 6G NR positioning shall support RAT-dependent and RAT-independent integrity.

6G NR shall support regulatory positioning requirements.

6G NR design targets for commercial positioning use cases include:

- Performance Flexibility: Support for diverse accuracy levels, latency thresholds, and device categories.

- Standard-Compliant Metrics: Alignment with accuracy and latency requirements defined in 3GPP TR 22.862 for:

- Unmanned Aerial Vehicle (UAV) positioning

- Immersive communication

- UE-assisted bistatic sensing

- Compliance with location and positioning services as specified in 3GPP TR 22.870, referencing TS 22.261 and TS 22.071.

- Operational Efficiency:

- Reduced network complexity

- Lower device costs

- Minimized device power consumption

- Optimized Signaling: Efficient data transmission over air interfaces and within network architectures.

- Methodological Integration: Support for hybrid positioning techniques combining multiple technologies.

- Scalability: Accommodation of large-scale device deployments.

- Robustness:

- High security standards

- Exceptional service availability

- Mobility Support: Compatibility with UE speeds as specified in 3GPP TR 22.862.

## 5. C Sensing

For sensing, 6G shall establish a unified framework from network architecture and air interface perspective, to support various sensing use cases, without significantly compromising communication performance. Object detection and tracking is considered as baseline use case.

Both mono-static sensing mode and bi-static sensing mode should be supported, including: TRP mono-static, UE mono-static, TRP-TRP bi-static, UE-TRP bi-static, TRP-UE bi-static.

Cooperative sensing at TRP side (e.g. Inter-TRP cooperation, inter-frequency cooperation, etc.) shall be supported, to provide competitive sensing performance than RAN-external (radar, etc.) sensing techniques.

In case of quantitative KPI, the following sensing quantitative KPI can be used:

-	detection probability:  [95% @5% false alarm]

-	localization accuracy:  [m level @90% confidence level]

-	velocity accuracy：[m/s level @90% confidence level]

## 5. D Requirements for architecture and migration of Next Generation Radio Access Technologies

The RAN design for the 6G Radio Access Technologies shall be designed to fulfil the following requirements and some other requirements would be discussed in WGs, e.g. CU/DU split:

-    The RAN architecture shall support standalone deployment from Day 1

-    The RAN architecture shall support service continuity between 5G and 6G

-    The RAN architecture shall support both satellite access and terrestrial access

Note: The migration solution requires coordination with SA

-	The RAN architecture shall support connectivity through multiple transmission points, either co-located or non-co-located.

-	The RAN architecture shall allow for deployment flexibility e.g. to host relevant RAN, CN and application functions close together at the edges of the network, when needed, e.g. to enable context aware service delivery, low latency services, etc...

-	The RAN architecture shall allow deployments using Network Function Virtualization.

-	The RAN architecture shall allow for the RAN and the CN to evolve independently.

-	The RAN architecture shall support sharing of the RAN between multiple operators.

-	The design of the RAN architecture shall allow the deployment of new services rapidly and efficiently.

-	The design of the RAN architecture shall allow the support of 3GPP defined service classes (e.g. interactive, background, streaming and conversational).

-	The design of the RAN architecture shall enable lower CAPEX/OPEX with respect to current networks to achieve the same level of services.

-	RAN-CN interfaces and interface between NG-RAN nodes shall be open for multi-vendor interoperability

……

The end of Text proposals