3GPP TSG RAN meeting #108                                                 RP-251736

Prague, Czech Republic, June 9-13, 2025

Title: 	ZTE's views on candidate Demod/RRM topics for Rel-20

Source: 	ZTE Corporation,Sanechips

Agenda item:	15.4.3

Document for:	Discussion

Introduction

In the RAN#107 meeting, there was some initial discussion on Rel-20 RAN4 package and the discussion outcome is summarized in moderator’s summary [1]. In this contribution, we would like to share our views on Rel-20 RAN4 package from Demod and RRM perspective.

Discussion

- 2.1. RRM enhancement

- 2.1.1 ZTE’s view

The high level views on the companies’ Rel-20 UE RRM proposals so far are summarized in the following table.

| Numbering | Objective | ZTE’s Preference |
| --- | --- | --- |
| 1 | RLM/BFD relaxation for RedCap UE - Consider to reuse Rel-17 solution as baseline - Consider both 1Rx and 2Rx. | Middle priority |
| 2 | Fast SCell activation - Consider the reduction of the delay of measurement and/or procedure - FFS on the detailed solutions - Whether to have this item depends on the detailed proposals of solution - One unified solution is expected | Middle priority |
| 3 | Uplink only SCell (NES SSB-less SCell enhancement) | High priority |
| 4 | FR2 multi-Tx and multi-Rx | Neutral |
| 5 | Enhanced RRM requirements for NES | Neutral |
| 6 | Enhanced RedCap with LTM | Neutral |
| 7 | R18 leftover on LTM RRM | Neutral |
| 8 | Interruption at NR SRS antenna port switching | Neutral |
| 9 | Parallel measurement | Neutral |
| 10 | Inter-frequency measurement enhancement for LTM | Neutral |
| 11 | On-demand deactivated SCell measurement | Neutral |
| 12 | A-TRS based RRM enhancement | Neutral |
| 13 | Dynamic RTD/TTD status update | High priority |
| 14 | AL/ML beam prediction testing | Neutral |
| 15 | Handover interruption minimization | Neutral |
| 16 | Interruption-free DCI operation | Neutral |
| 17 | Extend OD-SSB to PSCell in NR-DC | Middle priority |

- 2.1.2 Justification

Objective 3: Scell with uplink only transmission has been discussed in the previous release, lots of merits and applicable use cases have been identified and well discussed e.g. network power saving without DL transmission, addressing outdoor interference problems due to BS transmission, uplink only spectrum transmission etc. However, due to the workload and limited TU restriction, RRM requirement for Scell with uplink only transmission has not been specified yet until Rel-19. In order to enable this feature with full set of requirements in RAN4, it’s proposed to define the corresponding RRM requirement for it in Rel-20.

- 2.1.3 WID objective

3)Define the RRM requirement for Scell with UL only transmission including inter-band and intra-band CA scenario[RAN4].

  - Potential RAN1/2 impact,if any, could be triggered by LS.

- 2.1.4 Proposed TU budget

NOTE: the proposed TU budget for RRM enhancement in Rel-20 is based on the two objectives to be agreed at the end.

- 2.2. Demod enhancement

- 2.2.1 ZTE’s view

The high level views on the companies’ proposals on Rel-20 demod enhancement so far are summarized in the following table.

| Numbering | Objective | ZTE’s Preference |
| --- | --- | --- |
| 1 | SCM based requirements and/or further study of SCM - FFS whether new requirements need be specified in Rel-20 depending on Rel-19 SI progress, and check the progress by September. - [At least for SU-MIMO requirement is expected in Rel-20]. | Middle priority |
| 2 | R-ML receiver for 8Rx SU-MIMO scenario - Focus on demodulation requirements rather than CSI reporting - FFS on whether the new UE capability is needed. | Middle priority |
| 3 | CRI Reporting Requirements | Middle priority. CRI is reporting in the commercial network without RAN4 requirement. This might be done by IoDT testing between BS and chipset vendors from our understanding. |
| 4 | BS MMSE-IRC - For async TDD scenario and/or SBFD, and/or hetnet scenario with co-channel interference | High priority |
| 6 | 6-layer performance requirements for handheld UE | Middle priority It might be difficult to move forward based on the Rel-19 discussion. |
| 7 | UE IC receiver | Middle priority |
| 8 [Rel-19 left over] | 6Rx MMSE-IRC with inter-cell and with intra-cell inter-user interference | High priority |

- 2.2.2 Justification

Objective 4:

In Rel-19, the BS demodulation single carrier performance requirements for UL co-channel interference with MMSE-IRC receiver was defined. In practical network deployment, DL to UL interference becomes an increasingly significant scenario that cannot be ignored. In Rel-20, it is proposed to investigate the advanced receiver to address the co-channel CLI in the HetNet scenario.

Objective 8:

6Rx is one of important features for the latest flagship smartphone on the market which is also within Rel-19 UE RF enhancement, however during the RAN4#115 meeting, RAN4 agreed to define the 6Rx performance requirement without considering interference which is not aligned with 2/4/8Rx and also not aligned with reality without inter-cell and intra-cell inter-user interference, therefore it’s proposed to define the corresponding requirement for 6Rx with MMSE-IRC in Rel-20.

- 2.2.3 WID objective

4) Define the BS MMSE_IRC requirement with BS2BS CLI interference considered [RAN4].

  - For async TDD scenario and/or SBFD, and/or hetnet scenario with co-channel interference

- Define the following requirements for 6Rx TDD UEs with MMSE-IRC receiver [RAN4]:

  - Reuse the existing interference model for 2/4/8 Rx UE

  - With both inter-cell and with intra-cell inter-user interference considered in Rel-20.

- 2.2.4 Proposed TU budget

NOTE: the proposed TU budget for demod enhancement in Rel-20 is based on the two objectives to be agreed at the end.

Conclusions

In this contribution, we shared some further views on Rel-20 RAN4 package based on RAN#107 moderator’s summary and Rel-19 leftover issues.

References

- Moderator's summary for RAN4 led REL-20 topics [RP-250817]