3GPP TSG RAN Meeting #108						RP-250872

Prague, Czech Republic, June 9-13, 2025

Agenda Item:		16.2

Source:		T-Mobile USA, Nokia, Ericsson

Title:				Our views on 6G RAN WG Study item

Document for:		Discussion and decision

### 1	Introduction

In this contribution we present our views on 6G RAN WG study item, its justifications and what are important aspects when developing objectives for the 6G RAN WG study item.

The section 2 proposes text for the justification section of the 6G RAN WG SID describing why new 6G radio access technology is needed and what should be focus areas of the development of new 6G radio access technology. The section 3 then proposes the corresponding study item objectives.

### 2	Proposal for justifications

Improving network spectrum efficiency has always been the top priority for operators and when developing new radio access technologies in 3GPP. 6G radio is no exception. While enhanced Mobile Broadband (eMBB) continues to be an important service provided by operators, new applications/services, such as XR or AI traffic, have significantly different traffic profiles and performance criteria, and needs to be treated as primary services as well when new 6G radio access technology is developed.

From 2G to 5G, the advancement of radio technology and air interface not only provided new network capabilities that enable new services, but also steadily made progresses in better utilizing carrier's limited spectrum resources. Massive MIMO and higher order modulation schemes are good examples. In a conventional network where eMBB services are the predominant traffic carried by the network, maximizing bit/Hz and using it as the main driver for spectrum efficiency makes sense. However, with more new services, such as IoT and XR, being introduced into the wireless network, it is becoming more and more challenging for the network to manage the radio resources which are shared by different services. This shouldn't come as a surprise considering that the current 5G specification is designed for eMBB, i.e. smartphone use cases which primarily are voice, SMS and best effort data applications (video streaming, Web browsing, mapping etc). For successful development of new 6G radio access technology it is important to consider how 6G radio can efficiently serve different new services like XR, AI traffic and IoT together with eMBB on the same spectrum, instead of just serving one type of a service efficiently and maximizing the absolute peak spectral efficiency for best effort full buffer traffic.

The introduction of ChatGPT in late 2022 marked a paradigm shift in the perception, accessibility and power of AI. ChatGPT made sophisticated AI accessible to billions of people via natural conversation. AI is now part of everyday life from students using it for learning to professionals coding, writing, and analysing with AI assistance and AI creative tools that are disrupting industries from software to marketing. AI redefines what and how humans work especially in knowledge-based domains. 6G is expected to make AI natively integrated into the mobile network fabric, enabling new AI applications that are real-time, distributed and mobile native – transforming society through new forms of intelligent interaction, automation and augmentation, all wirelessly and contextually. However, in the existing 5G system, RAN does not have direct exposure to the application performance, neither has it a mechanism that could enforce the specific QoS performance for an application to meet the specific application QoE requirement, not to mention in a real time manner.  For AI applications, other than the regular communication resources, AI computation resources need to be identified and allocated also. To solve this problem in 6G RAN needs to be more adaptive to the applications.

The introduction of new 6G technologies such as XR, AI and sensing will require the network to be even more capable of supporting diversified use case scenarios. In the current 5G system, 5G QoS Identifier (5QI) is used to define the specific quality profile for a service by the core network. Different 5QI value correspondent to a set of service level thresholds such as throughput, latency, error rate etc. 5QI index is passed to the RAN in which the 5QI value, along with other RAN domain parameters, is used by the MAC scheduler to calculate the MAC scheduling priority for each data flow. Such mechanism works for a system carrying data applications with less stringent throughput and latency requirement, such as web browsing and non-real time video streaming, however, RAN is not optimal in supporting support applications, such as XR, which require both stringent throughput and latency adjustments in real time unless the RAN is over dimensioned to ensure the quality of service for such applications. This makes it very challenge for operators to launch such new services without compromising the overall system spectrum efficiency and legacy eMBB service performance. 3GPP made the efforts to address this issue but in commercial networks, operators ran into challenges from device ecosystem players who normally prevents the OS from sending the application information/APP IDs to the AS layer for various reasons. Operators have very limited visibility and control on the massive and diversified applications that are going through the network and are not able to effectively meet all applications’ performance targets while maintaining healthy network spectrum and operation efficiency. This also justifies that in 6G RAN needs to be more adaptive to the applications and 6G needs to be designed to support service aware RAN and address the issue on the basic protocol levels.

Fixed Wireless Access (FWA) has experienced significant growth in recent years, driven by technological advancements. Wireless operators, particularly those in North America and India, have recognized the increased business potential for in-home high-speed internet in areas with limited access to traditional broadband infrastructures such as cable or fiber. The global deployment of 5G has provided unique opportunities for operators to open new revenue streams by offering in-home broadband services to their customers. The rising demand in both residential and commercial sectors for faster, reliable, and high-speed connectivity is propelling the 5G fixed wireless access market. However, the existing network, designed primarily for eMBB services, may not be able to meet the demand of FWA devices, which have different mobility profiles and usage patterns. With the increased demand and projected growth of FWA in both enterprise and consumer areas, solutions that can further improve spectrum efficiency and economies of scale for FWA are essential to ensure the long-term success of the business model. Mobile operators seek to enhance FWA's functionality by optimizing systems, to address the unique needs of FWA users. By revisiting key elements like mobility management, control channel overhead, and other PHY layer enhancements, network capacity could be significantly increased to meet the higher bandwidth demands of fixed users. The U.S. 5G fixed wireless access market size was estimated at USD 8.35 billion in 2023 and is projected to be worth around USD 343.18 billion by 2034, growing at a compound annual growth rate (CAGR) of 40.18% from 2024 to 2034, according to Precedence Research. With FWA becoming the primary traffic in operators’ networks, it is critical that on Day One of 6G, FWA as a primary service offered by operators can meet customer demand with a sustainable and profitable business model. The current 5G specification, protocol and architecture were designed for eMBB, not for FWA. One-pipe-fits-all type approach may not work well for eMBB and FWA. While eMBB continues to be an important service provided by operators, FWA, with significant more traffic volume than eMBB, needs to be treated as primary service in new 6G radio access technology.

6G is expected to be as a platform also for innovative new services like sensing-as-a-service. Integrated Sensing and Communications (ISAC) will allow mobile networks to concurrently support high-quality wireless communication and accurate sensing functions. TR 22.837 has already recognized numerous application domains for sensing, including automotive, UAVs, smart city infrastructure, and environmental monitoring. Sensing capabilities embedded in the radio protocol and architecture can unlock new revenue streams and improve network efficiency.

Voice services have been a part of wireless telecom since its earliest days of commercial operation. It could be said to be the ‘killer app’ of all the services that have been offered. We know that humans are verbal creatures and while add on services such as texting, email, etc. have become a big part of our lives, voice is still the way we are able to quickly communicate with other people. Basic voice service will continue to be a part of any mobile telecom offering for the foreseeable future. Humanity is not diverting from verbal communication so no matter what new features a next generation transport service may offer, voice has to be included.  How voice will be offered in the next generation is worth a serious discussion. One expects that there will be more voice services, possible bundled with OTT application as well as carrier grade voice over the main air interface. That carrier grade service along with all its regulatory encumbrances needs to be a part of the basic feature set in any new generation of telecom technology. It is critical that on the day the Next Generation Launches (6G) that carrier grade, standalone voice be a part of that release. That also will require E911/location, LI, other voice regulatory requirements, and lastly ID security be part of that feature. In order to evolve voice communications from VoLTE and VoNR, which is currently the de facto standard for voice communications and voice roaming, it is imperative that any new technology for 6G fully support VoIP on 6G from Day 1. That voice offering must include all of the regulatory needed capabilities in order for a 6G system to be deployed without having callers required to fallback to previous generations of technology for a simple voice call.

In 5G NR, the widest FR1 channel bandwidth is 100 MHz. Because of this limit, operators with more than 100 MHz of contiguous spectrum need to use carrier aggregation to cover such allocations. The RAN level study on 6G Scenarios and requirements defines that wider channel bandwidth, which is at least 200MHz, needs to developed for 6G at least for FR1 frequency bands above 2 GHz and around 7 GHz. One benefit of wider channel bandwidths in FR1 is higher spectral efficiency than CA. Another benefit of wider channel bandwidths in FR1 is potentially Lower MPR and A-MPR than UL CA due to contiguous resource block allocations. Due to the lack of new spectrum in most FR1 frequency bands, there is strong interest in multi-radio spectrum sharing (MRSS), to allow operators to use the same spectrum for 5G and 6G. Since some operators have > 100 MHz in FR1 bands that are used for 5G, it is important to maintain compatibility between the new, wider channel bandwidths for 6G, and 5G carrier aggregation of channel bandwidths up to 100 MHz, including the use of the same subcarrier spacing.

By integrating data-driven improvements and enabling AI-based solutions in the new 6G radio access technology, 6G radio and system performance can be continuously improved and adapted to diverse scenarios, performance objectives, new services and use cases.	In Release 18, 19 and 20 3GPP RAN intended to build a generic framework for data collection and for life cycle management using AI Mobility, AI Beam Management, AI CSI compression/prediction and Location as example use cases. Time pressure resulted in no stage 3 development for two sided models, very limited management capabilities for an operator to manage trained models, and no standards-based solution for UE sided data collection. To meet the operators’ data collection and transfer needs in 6G, it is important that 6G will address the following aspects:

1. Data Security and Integrity: Data collected by the network must be secured, with full guarantees of data integrity and confidentiality during transfer.

2. User Privacy and Consent: The system must respect user privacy, ensuring anonymity and compliance with user consent requirements for data collection.

3. Mobile Network Operator (MNO) Control Over Data Transfer: MNOs must have full control over the data collection process, including initiating, terminating, and managing the transfer of data to the server for UE-side data collection. This control should not require a Service Level Agreement (SLA) to manage the transfer process.

4. Visibility and Transparency: MNOs must have full visibility over the standardized data, ensuring monitoring and management of the data collection process.

5. Future-proof and Extensible Design: The design should be flexible and scalable to adapt to future 6G AI use cases and evolving technology.

Additionally, the following Life-Cycle-Management (LCM) requirements for model transfer and delivery must be met to ensure a secure, efficient, and manageable AI framework in 6G:

1. Differentiation of Model Transfer Traffic: Traffic for model transfer and delivery should be differentiated from regular user traffic. The priority for model transfer should be configurable by the network, allowing for proper prioritization without impacting user traffic.

2. Security of Model Transfer: Model transfer must be done in a secure and network-aware manner, ensuring that untrusted or malicious models are prevented from being downloaded onto the network or UE.

3. Initiation of Model Transfer: The UE or the 6G base station should be able to initiate model transfer or delivery, providing flexibility and control over the process.

4. Addressability of Models: Trained models must be addressable so that the UE can request specific models to be transferred or delivered, enabling flexibility in model management. The network should be able to enable or disable specific trained models on demand, based on operational needs.

5. Network Control of Model Delivery: The network must retain full control over when and how models are transferred or delivered to the UE. Sufficient monitoring and management information should be available for each trained model, ensuring that MNOs can assess and manage the performance of models effectively.

Therefore, this study aims to develop a 6G radio access technology addressing the above-mentioned needs and expectations for 6G and meet a broad range of use cases and requirements defined in the 6G Scenarios and Requirements TRxx.yyy.

### 3	Proposal for objectives

The study aims to develop a 6G radio access technology to meet a broad range of use cases and requirements and provide improvements for existing and new services as defined by the RAN requirements study.

Detailed objectives of the study item are:

- Target a single technical framework for 6G radio access technology addressing usage scenarios, requirements, evolving services and deployment scenarios defined in TRxx.yyy including

  - eMBB

  - Fixed Wireless Access

  - XR

  - AI applications and traffic

  - Massive IoT

  - Sensing etc

To meet these requirements, the RAN WG will study the following technical aspects for developing physical layer, radio protocol layer, radio access network and UE and BS requirements for 6G new radio access technology:

- Support of wider channel bandwidths (CBW) for the FR1 frequency bands above 2 GHz and new frequency bands around 7 GHz.

  - Physical layer aspects related to the wider channel bandwidths like

    - FFT size

    - Uplink and downlink impacts

    - CBWs including channel bandwidths at least up to 200 MHz for the FR1 frequency bands above 2 GHz and new frequency bands around 7 GHz.

  - RF aspects like

    - Maximum transmission bandwidths (RBs)

    - Raster impacts

    - Minimum guardbands

    - Maximum Power Reduction (MPR)

    - Spectrum Emission Mask

- Multi-RAT Spectrum Sharing (MRSS) between 5G and 6G on the same spectrum including the following aspects

  - MRSS between 5G CA and 6G wider CBW

  - The use of the same SCS for 6G as was used in the same bands for 5G

  - Configurations that allow for MRSS with NR-CA

  - UL

- Solutions for enhanced cell edge and uplink performance.

  - DL/UL decoupling

  - In-band signalling

  - UL scheduling latency improvement

- Enablers for scheduler efficiency

  - Number of simultaneous users

  - Sufficiently small minimum scheduling unit

  - Service specific based on real time application attributes

  - Interference management/coordination schemes that are required to support Service Aware RAN

- Fixed Wireless Access (FWA) specific standard development for significant spectral efficiency gain

  - Air interface/protocol/Mobility

  - Link adaptation/RRM

  - FWA device enhancement e.g. high power UEs, beam forming etc

- Modular radio access protocol with service-based access control

  - Modular radio access and NAS for Service Aware RAN

  - RAN signalling and protocols designed to support Service Aware RAN deployment

  - Service specific RRM “layers” in the RAN domain for each type of services

  - Potential RAN/Core architecture modifications that enables Service Aware RAN

- Support of 6G voice with emergency services like emergency location services, public warning systems (PWS) and lawful interception (LI)

  - Voice coverage; ensure similar 6G coverage to that of data coverage

  - Efficiently support a carrier grade voice over IP call in the RAN e.g. by defining the necessary minimal set of QoE capabilities

  - Differentiation of IMS signalling carried in the user plane to ensure IMS signalling receives almost the same degree of protection/priority as control plane signalling.

  - Voice Quality Enhancements: Evaluate mechanisms such as 5QI-aware scheduling and optimized low-latency processing to ensure high-quality voice services

  - Assess methodologies for improved and precise device identification and network synchronization e.g. Enhanced Cellular ID

  - Explore advanced positioning techniques to augment network-based location accuracy and support emerging location-dependent services e.g. OTDOA

- Security and privacy aspects

- AI native air interface for reducing control and reference signal overhead and enhancing radio performance by enabling air interface i.e. UE and network side AI to continually learn from data and adapt to changing conditions

  - Study and select AI air interface uses e.g. on the areas of advanced receivers, constellation shaping, link adaptation and channel estimation

  - Enablers for AI native 6G air interface while meeting MNO requirements to allow AI-based implementations alongside to conventional implementation approaches, including aspects like

    - Methods and procedures for UE and Network side data collection

      - Identify a robust set of key performance indicators (KPIs) needed to support many different use cases ..

    - LCM procedures for wide and varied use cases

    - Scalability of AI Solutions: Address the challenge of scaling AI solutions across large, heterogeneous, and geographically distributed RAN infrastructures.

    - Data Availability and Quality: Study the challenges of gathering high-quality labeled data for training AI models and the potential use of synthetic data.

    - Explainability of AI Models in RAN: Study the explainability of AI models used in RAN to ensure that operators can understand, trust, and manage AI-driven decisions.

    - AI and Regulatory Issues: Explore regulatory challenges surrounding AI-based automation in RAN, such as ensuring compliance with privacy laws, cybersecurity policies, and fair use of spectrum.

    - Study Agentic AI including use cases for usage of Agentic AI to determine RAN impact

- Radio Access Network aspects for supporting new Services, such as AI applications while ensuring that operators can offer the services at a commercially scalable level and operators can provide new 6G AI services without compromising spectrum efficiency and profitability

  - Potential RAN/Core architecture modifications to enable RAN support for real-time AI applications, which may include impact from AI task identification & triggering, GPU resource discovery & inventory management, cross domain coordination among RIC/SMO, MEC platform and UPF.

  - RAN signalling and protocols that are specifically designed to support RAN for AI application deployment, which may include impact from GPU aware scheduling & allocation, monitoring, adaptation and termination

  - Vertical protocol stack modifications that enable in-band real time communication between the application and radio layers.

  - Interference management/coordination schemes that are required to support RAN for AI applications

- Security and privacy aspects

- Feasibility and potential 3GPP requirements for integrating sensing functionality into 6G systems including the following aspects:

  - Analyse use cases from TR 22.837 and beyond to assess relevance for global operators.

  - Identify scenarios where ISAC is technically and commercially feasible in the near vs. long term.

  - Study sensing modes TRP-TRP bistatic, TRP monostatic, TRP-UE bistatic, UE-TRP bistatic, UE-UE bistatic, and UE monostatic.

    - Study if combining sensing nodes can improve accuracy.

  - Study and define the metrics used to quantify sensing performance, including:

    - Detection Range (maximum and minimum distances at which the system can reliably detect and classify targets)

    - Resolution and Discrimination Sensitivity (the ability to resolve closely spaced objects and to distinguish between similar target classes)

    - Classification Accuracy (the precision and recall of the ML classification model in correctly identifying object types)

    - Telemetry Accuracy (the precision of target tracking data, including location, speed and trajectory)

  - Evaluate the reuse of the existing OFDMA waveform for combined 6G communications and sensing, without ruling out other possibilities such as a dedicated waveform that can co-exist with the communications system.

  - Define processing methods such as 2D FFT for delay-Doppler estimation, and study the integration of AI/ML techniques for identifying and classifying targets

  - Analyse pulse frequency/bandwidth tradeoffs for velocity and range resolution; assess suitability of low, mid (e.g., 2.5 GHz), and high bands.  Assess sensing performance in different frequency bands including the existing and new frequency bands

  - Study the impact on communications performance by introducing sensing.  Investigate the tradeoffs between system capacity and ISAC performance such as Detection Range, etc.

  - Study architectural support for ISAC including IQ data processing at DU/baseband level, edge aggregation, and data fusion for distributed sensing.

  - Study potential new network functions for ISAC, especially for edge-based sensing analysis and multi-source signal fusion

- Study and identify the technical solutions and requirement aspects to enable the 6G radio access technology to meet the objectives and ensure that the 6G network will meet both enhanced performance demands and regulatory mandates.

- Provide performance evaluation of the technologies identified for the 6G radio access technology and analysis of the expected specification work

### 4	Conclusions

In this contribution we propose justifications and objectives for the 6G RAN WG study item, which is planned to be approved in 3GPP RAN#108.

The section 2 proposes justifications for the 6G RAN WG study item and Section 3 proposes objectives for the 6G RAN WG study item.