3GPP TSG RAN Meeting #108	RP-250874
Prague, Czech Republic, June 9-13, 2025

Agenda Item:	15.1.2

Source:	Ericsson

Title:	Discussion on Rel-20 NR MIMO Phase 6

Document for:	Discussion

# 1	Introduction and outcome of RAN#107

Last plenary, MIMO was discussed, and the moderator observation and recommendation was as follows [1]:

MIMO has been part of every NR release with at least 2~2.5 TUs per release. While the support for another MIMO enhancement in Release 20 was clearly high, there were multiple companies who voiced that there was no commercial need for it. Also, clear majority of companies were in favor of a compact scope.

Moderator assumes the following for the purpose of TU planning:

- For Rel-20 5G-Adv MIMO, the scope should be limited and well justified by commercial needs

- [1] TU in RAN1 and [0.5] TU in RAN2 for Release 20 5G-Adv MIMO

Moreover, the detailed list of proposals as summarized by the moderator, can be found in the Appendix, Section 6, of this document.

# 2	Summary of our proposal

The moderator categorized the proposals as follows (see details in Appendix, Section 6).

- Overall, the proposals on Rel-20 MIMO can be categorized into the following three areas

  - Uplink enhancement (items #1, #2, #3, #4, #5)

  - Downlink CSI enhancement (items #6, #7, #8)

  - Multi-beam enhancement (items #9, #10)

  - DMRS enhancement (item #11)

To fit into the [1] TU, this list of 11 items needs to be significantly shortened and there is no room for explorative items or items with no current commercial interest. Instead, the enhancements should target features for which there are urgent problems to be solve and that gives a demonstratable gain. Also, items that require a lengthy study part should also be a better candidate for 6G SI.

We provide our views on these items in the below table and in the next section we give a more detailed motivation for the interested reader.

| Item | Commercial relevance and urgency | Assessment from 1 (lowest) to 5 (most important) |
| --- | --- | --- |
| #1 UL Enh: UL frequency-selective (open-loop) precoder cycling | No gain found in our extensive evaluations | 1 |
| #2 UL Enh: 2Tx full-coherent UL codebook with 8/16PSK alphabets | Gain is disputed | 2 |
| #3 UL Enh: Cross-slot SRS, SRS frequency hopping for partial sounding | The cross slot SRS solves an issue in the field for DDDSU patterns with antenna switching for massive MIMO reciprocity. The solution enables efficient massive MIMO operation for some operators with few UL symbols in the S slot. | 5 for cross slot 2 for partial |
| #4 UL Enh: Asymmetric DL sTRP/UL mTRP beam management enhancements | Some commercial interest but in our view has low priority | 3 |
| #5 UL Enh: UL frequency-selective (closed-loop) TPMI-based precoding | No commercial interest and a large topic, more suitable for 6G SI | 2 |
| #6 DL CSI: Early SRS/CSI/CSI-RS triggering for UEs in IDLE/INACTIVE to CONNECTED; and SCell activation in CONNECTED | The early CSI report is very important for rapid utilization of massive MIMO for Scell after activation. Currently there is a lag of 10s of milliseconds before massive MIMO can be used on an SCell which is too late in most cases | 5 |
| #7 DL CSI: CSI-RS overhead reduction for larger # ports | No commercial interest since there is no UE support yet for the very large number of CSI-RS ports specified in Rel.19. | 1 |
| #8 DL CSI: Sub-band scalar IMR measurement report (targeting reciprocity-based operation) | The adressed problem is relevant but after discussing with many companies during the spring, we belive a thorough study is needed in 6G SI (and where a solution may be applicable for 5G-A as well). | 3 |
| #9 Multi-beam: UEI/ED beam switching (left over from Rel-19 MIMO) | No commercial interest. We are not aware of any vendors that implement this so this has low priority. | 1 |
| #10 Multi-beam: Dynamic QCL update via MAC CE for periodic TRS | Low commercial interest. Although the problem is relevant for FR2, it is not a top priority. | 3 |
| #11 Enhanced DMRS port allocation | No commercial interest since there is no UE support yet for the very large number of CSI-RS ports specified in Rel.19. On the DMRS ports, we don’t see any demand or use case when 24 ports would be needed, we have not seen a shortage in DMRS ports as an issue in our networks. In addition, we haven’t found the claimed gain in our evaluations of the proposal. | 1 |

We have not observed any market demand or reported problems/issues that requires multi-beam enhancements or DMRS enhancements. Hence, our target is to use the single MIMO TU in a balanced manner for the UL and DL CSI enhancements as follows:

For Rel.20 NR MIMO, support at most one DL CSI enhancement and one UL enhancement

Based on our assessment of the proposal on the table, we propose the following two items:

In NR MIMO Rel.20 Support only Item #3 UL Enh: Cross-slot SRS, SRS frequency hopping for partial sounding and Item #6 DL CSI: Early SRS/CSI/CSI-RS triggering for UEs in IDLE/INACTIVE to CONNECTED; and SCell activation in CONNECTED

These objectives are based on a TU allocation as follows:

For NR MIMO Rel.20, allocate [1] TU in RAN1, [0.5] TU in RAN2 and [0.5-0.75] TU in RAN4

# 3	Detailed discussion of Item #1 to #11

Here we provide our detailed analysis and view on the eleven identified items from RAN#107:

## 3.1 Item #1, UL frequency-selective (open-loop) precoder cycling

This feature has quite good support but the evidence among the proponents that this would provide a benefit is weak. Our simulation results in Section 2.3.2 of R1-2500539 doesn’t show any performance benefits, the feature only provides performance degradation at 3 km/h and at 60 km/h, at best, a comparable performance to Rel.15 wideband closed loop precoding. Almost none of the proponents have not yet provided any own simulation evidence. In addition, it has several other drawbacks which are listed below.

- Does not provide any gains in simulations (R1-2500539 and partly copied below)

- Requires significant gNB implementation work since a new per-PRG channel estimator in gNB needs to be implemented

- The use of a large PRG limits scheduling, since allocations must be large enough to cycle precoders and FDM’d UEs should align with PRG boundaries, while a small PRG can degrade channel estimation.

- It’s a scheme that doesn’t work with DFT-s-OFDM waveform

- It is unclear what is included in the proposal. How many Tx? Is it a cycling of precoders from a codebook or are the used precoders UE implementation? What mechanism shall be used at gNB to perform link adaptation, i.e., is a new SRS type needed?  What is the PRG bandwidth? How does introduction of a PRG in uplink impact the interference covariance estimate at the gNB receiver?

For example, the choice of a precoders to cycle was discussed already in LTE, for the PDCCH design for 4 Tx base stations and the use of FSTD technique was chosen due to its attractive features. It uses the precoders [1 0] and [0 1], per subcarrier, hence a simple Tx antenna hopping scheme, to obtain diversity. The choice of the precoder [1 0] and [0 1] has these benefits:

- Wideband DMRS channel estimation can still be used, thereby avoiding the channel estimation losses found in per-PRG precoder cycling and thus avoiding large impact to gNB implementation complexity.

- It can be used for DFT-S-OFDM without harming single carrier properties

- It is compatible with non-coherent UL MIMO transmission, which is the most commercially relevant UL MIMO transmission scheme at present.

- Full power transmission is achieved automatically, since all transmit antennas are used.

Support open loop transmit diversity if and only if gNB wideband channel estimation can be retained, DFT-S-OFDM can be supported, coherence among transmit chains is not required and if there is a clear demonstrated performance benefit.

Figure 1 and Figure 2 present the link-level UL throughput comparison between non-coherent Rel.15 CL wideband precoding, OL precoding cycling, and FSTD. Detailed simulation assumptions are given in Error! Reference source not found. in the Appendix. The analysis considers scenarios where the UE moves at speeds of 3 km/h and 120 km/h, transmitting with a maximum rank of 1. The UE is equipped with 2 full-power non-coherent transmit chains, while the gNB has 4 receive chains. For each figure, the evaluation is done for SRS periodicity of 10ms and 80ms. Accordingly,

- When the UE moves at a pedestrian speed of 3 km/h, OL precoding cycling and FSTD experience a performance loss over a wide SNR range compared to CL wideband precoding (i.e. Rel.15 specification).

  - Specifically, at SNRs of -5, 0, 10 dB, losses are observed for OL precoding cycling and FSTD as -19, -16,-10% and -20, -14,-9%, respectively, for SRS periodicity of 10ms, and -16, -13,-7% and -16, -11,-7%, respectively, for SRS periodicity of 80ms, respectively. This performance gap arises because, in relatively static channels, non-coherent CL precoding can select the optimal transmit chain, whereas OL precoding randomly selects transmit chains per subband, and FSTD blindly transmits on the first transmit chain on odd subcarriers and the second transmit chain on even subcarriers.

- In contrast, at a vehicular UE speed of 120 km/h, OL precoding cycling and FSTD perform almost comparably to CL wideband precoding, regardless of the SRS periodicity. The rapid channel variations at this speed provides a diversity gain for OL precoding cycling and FSTD, enhancing their performance. Specifically, at SNRs of -5,  0, 10 dB, gains are observed for OL precoding cycling and FSTD as 16, 4, 4% and 11, 6, 2%, respectively, for SRS periodicity of 10ms, and 16, 8, 6% and 12, 9, 4%, respectively, for SRS periodicity of 80ms.

Figure 1	  UL throughout versus SNR, comparing non-coherent CL wideband precoding with OL precoding cycling and FSTD for 10ms (left figure) and 80ms (right figure) SRS periodicity with 2 transmit chains and Rel-16 Mode 0 power-scaling. Here, the UEs transmit with max rank 1 and is moving with pedestrian speed, i.e., 3 km/h.

Figure 2 UL throughout versus SNR, comparing non-coherent CL wideband precoding with OL precoding cycling and FSTD for 10ms (left figure) and 80ms (right figure) SRS periodicity with 2 transmit chains and Rel-16 Mode 0 power-scaling. Here, the UEs transmit with max rank 1 and is moving with vehicular speed, i.e., 120 km/h.

FSTD does have the drawback that it has less gain than other schemes at higher code rates. However, our expectation is that the principal benefit of open loop TxD in the scope of NR Rel-20 is that the UE can transmit at full power by combining the power on the antennas in a specified way in scenarios where closed loop MIMO fails, since we do not see performance gains as shown above. Therefore, if it is decided that it should be specified, any design for open loop TxD should focus on minimizing impact to UE and network complexity while maintaining reasonable performance, good PAPR for DFT-S-OFDM, and full power transmission.

Compared to non-coherent closed loop wideband precoding (Rel.15), and when the maximum rank is fixed to 1, precoder cycling and FSTD with two active Tx chains has a loss at pedestrian speeds over a wide range of SNRs and the comparable performance at 120 kmph.

The SRS periodicity does not seem to affect the relative gains of (frequency selective) precoder cycling, FSTD and (wideband) closed loop precoding.

Given their lack of gain over wideband closed loop precoding under conditions favorable to open loop TxD, a primary benefit of open loop TxD schemes seems to be full power transmission.

## 3.2 Item #2 2Tx full-coherent UL codebook with 8/16PSK alphabets

This enhancement aims to take advantage of unused DCI codepoints for TMPI indication to extend the QPSK precoder alphabet for 2Tx UEs to 8-PSK. We have not evaluated to verify the claimed benefits, especially under the rather large allowed phase error between antenna branches from RAN4 requirements. We are open to support this very limited scope feature if some benefits can be shown, although we must question the urgent commercial need for this feature.

## 3.3 Item #3 Cross-slot SRS, SRS frequency hopping for partial sounding

We support cross-slot span of an SRS resource, there is an urgent and commercial demand for this as some operators have problems utilizing the S slots for SRS as there is only two SRS symbols available. With this enhancement (which basically consists of removing a restriction in spec that an SRS resource can only map to one slot), the first symbol or few symbols of the subsequent slot can be used for SRS, which allows the UE to maintain phase coherence is it is a continuous transmission.

We support cross slot transmission of an SRS resource as it addresses an urgent commercial need and it has a very small 3GPP TU footprint.

## 3.4 Item #4 Asymmetric DL sTRP/UL mTRP beam management enhancements

In our understanding, there are two issues addressed in this topic:

- Issue1:  PDCCH order PRACH to UL TRP is supported in R18 (i.e. apply PL-offset to PDCCH order PRACH) for TA acquisition, but transmit beam is up to UE implementation. It is our understanding that this is to specify “tx beam of PDCCH order PRACH is the indicated TCI state”. Then, gNB can configure SRS resource as QCL source of the indicated TCI state by gNB implementation

- Issue2: In case of SRS beam sweeping (i.e. SRS BM without TCI state configuration), gNB cannot determine which Rx beam to be used, because gNB has no idea which tx beam is used for each SRS tx occasion. (this is general issue of SRS beam sweeping). The solution is to introduce repetition to SRS, similar as for CSI-RS. Then, gNB can determine appropriate rx beam when gNB measures multiple SRS.

Although the issues as clear, we don’t see a significant commercial demand for this feature in the near term. If only 1 TU is allocated to MIMO, then this item is not our first-hand choice.

## 3.5 Item #5 UL frequency-selective (closed-loop) TPMI-based precoding

We will not support this enhancement, mainly because two major drawbacks:

- A very large impact to DCI payload, as TPMIs for every subband need to be indicated to the UE. The payload grows with the PUSCH scheduling bandwidth.

- There is a tremendous impact to gNB receiver and channel estimator since PRG is introduced in UL, the benefits of wideband channel estimation is lost, and it is uncertain whether and how (which requirements are needed on UE power and coherency, the codebook etc) to recover from this loss

We have carried out new simulations (See Section 2.3.2 in R1-2500539 and partly copied below) and uplink frequency selective precoding with MMSE-based channel estimation underperforms compared to wideband precoding with (wideband RS based) time-filtering-based channel estimation. This implies a need for exploration and improvement of channel estimation algorithms tailored for frequency selective precoding.

A proper study on DCI redesign vs performance trade-off is needed, and this is a big study suitable as a 6G SI topic. Significant studies are needed to reach an acceptable and workable scheme, on UE RF limitations such as maximum Tx chain power, coherence, or PAPR.

It seems this feature requires coherent codebook UEs, thus addressing a currently very small market segment. A proper study that also considers uplink interference suppressions and its relation to PRG bandwidth is needed.

To summarize, we believe this is a very large topic that spans multiple aspects, DCI, channel estimation, precoder codebook, PAPR etc. We don’t think there is enough TU for 5G-A MIMO Phase 6, instead suggests that it can be thoroughly explored in a 6G SI.

Hence, we do not support closed loop (TPMI indication based) frequency selective precoding for PUSCH in 5G-A.

Here follows some more theoretical background and evaluation results for closed loop subband precoders for PUSCH:

The state-of-the-art MMSE CHEST algorithm can be optimized for subband processing, where channel estimation is performed per subband. However, this approach reduces processing gain due to the limited number of subcarriers within each subband, restricting its ability to effectively suppress noise. While increasing the subband size can enhance performance, it comes at the expense of higher computational complexity, proportional to the number of subcarriers.

Moreover, MMSE CHEST assumes white noise, characterized by uncorrelated and uniform power across frequencies. Consequently, it struggles to handle colored noise, which exhibits non-uniform power spectral density and frequency correlations. This limitation becomes particularly significant in system-level scenarios, where interference from neighboring cells or users often appears as colored noise.

In the following, MMSE CHEST is compared to a time-filtering (TF) based CHEST, which is optimized for wideband precoding. By leveraging time-domain processing, TF-based CHEST effectively eliminates both noise and interference. However, TF-based filtering requires further enhancements to support FS precoding effectively, warranting additional studies.

Figure 3	 Mean and cell-edge (5-th percentile) UL throughout versus served traffic, comparing performance of wideband and FS precoding with MMSE CHEST to the performance of wideband precoding with TF-based CHEST. The channel bandwidth is 256 PRBs with subband size of 4 PRBs for FS precoding.

From Figure 3, it is evident that the performance of wideband and FS precoding with MMSE CHEST degrades compared to wideband precoding with TF-based CHEST as the traffic load increases. Specifically, FS precoding with MMSE CHEST experiences a performance loss of 10% in mean throughput and 34% in cell-edge throughput at 50% traffic load, relative to wideband precoding with TF-based CHEST.

This superior performance of TF-based CHEST is attributed to its ability to effectively suppress noise and interference, particularly at the cell edge. Consequently, the significant advantage of wideband precoding with TF-based CHEST over FS precoding with MMSE CHEST underscores the importance of continued study into CHEST algorithms optimized for FS precoding.

## 3.6 Item #6 Early SRS/CSI/CSI-RS triggering for UEs in IDLE/INACTIVE to CONNECTED; and SCell activation in CONNECTED

It has been an issue within carrier aggregation that activation of SCells takes time and due to the bursty small packet nature of typical eMBB traffic, the activation and use of SCells must be very fast in order to make use of the SCell. Typically, the scheduler uses the PCell to start transmitting the packets and many times it even completes the delivery before the SCell is ready to be used.

Now, the activation time has been reduced in Release 17 and we believe that the next step of this enhancement is to include fast CSI acquisition so that the activated SCell can also utilize MIMO transmissions directly. This would make carrier aggregation very useful also for smaller and bursty traffic, which is very common in reality.

To Ericsson, the early CSI reporting measured on an early CSI-RS for SCells is important but also similar enhancements for IDLE mode UEs are also important. To support higher MCS and MIMO, it can also be discussed whether this should include an early TRS trigger, to allow the UE to perform fine sync.

Another enhancement for the same purpose is the early SRS configuration and triggering, to be used for the case where there is a paired UL carrier to the DL carrier (which is not the typical case for all DL carriers in TDD since a UE supports more DL carriers than UL carriers).

We support Early CSI-RS, SRS and possibly also early aperiodic TRS resource triggering and early CSI reporting to enable MIMO as fast as possible after SCell activation and after IDLE CONNECTED and INACTIVE CONNECTED state change.

Figure 4 shows the reduction in number of slots needed to receive an FTP packet of size of 25 kilobytes with FTP model 1 for the following schemes:

- Early CSI Feedback:  case where single port CSI feedback with early CQI reporting which is taken into account by gNB for scheduling PDSCH

- Initial MCS = 0:  case where there is no early CSI and the gNB uses the most conservative MCS initially for scheduling PDSCH

- Initial MCS = 26:  case where there is no early CSI and the gNB uses the most aggressive MCS initially for scheduling PDSCH

Among the 2 last schemes, ‘Initial MCS=0’ is the most practical as the gNB will likely schedule PDSCH with an initial MCS that is most conservative in the absence of early CSI.  As can be seen from Figure 4, early CSI feedback can significantly reduce the number of slots needed to receive a packet when compared to other schemes where early CSI is not available.

Figure 4. Reduction in slots needed to receive a packet with early CSI

Figure 5 shows performance comparison between early CSI with different number of ports.  Here, PDSCH scheduling is according to RI/PMI/CQI reported as part of the respective early CSI reports.  As can be seen from the figure, notable performance improvement can be achieved when increasing the number of ports in the early CSI.

Figure 5. Performance comparison of early CSI with different number of ports

## 3.7 Item #7 CSI-RS overhead reduction for larger # ports

In Rel.19, up to 128 CSI-RS ports is supported and it’s getting near the pain point where CSI-RS overhead is becoming a real issue. There is a proposal to use frequency domain CSI-RS overhead reduction techniques which are straight forward, simple by extending the density values to lower than ½ that is the minimum supported today. This seems like a small specification change and should easily be adopted. However, we have not seen any indication that UEs will support 128 ports in the near feature, so even if this enhancement is relevant, there is no urgency to have it in Rel.20.

There is also a proposal to consider spatial domain overhead reduction, for example transmitting a subset of configured CSI-RS ports and let the UE use advanced receiver processing to recover of absent ports. This may require larger studies and specification impact and also the need for new receiver algorithms in the UE. We are not fully negative to this but considering the needed workload and the need for study phase, it is in our view better suited for 6G SI.

## 3.8 Item #8 Sub-band scalar IMR measurement report (targeting reciprocity-based operation)

The motivation for this enhancement is to make MU-MIMO operation using channel reciprocity more efficient and simplify operation, remove the need for estimation and replace this with a measurement. It is a request from our product side.

Today, the channel for co-scheduled UEs is obtained using reciprocity from the SRS, but the interference plus noise (IpN) level at the UE is initially unknown to the gNB. To compute the MU-MIMO precoders for each co-scheduled UE, the IpN, or even better, the spatial interference covariance matrix, is needed in the precoder calculation and to estimate the rank, MCS selection.

This forces the gNB to configure another full CSI report, such as a Type I CSI report, to obtain a CQI, which then can be processed to estimate the IpN as experienced for each UE.

We think reporting of IpN as well are reporting spatial covariance is interesting but after some discussions with other companies, we believe this area of enhancements are better suited for 6G SI as they need more studies.

## 3.9 Item #9 UEI/ED beam switching (left over from Rel-19 MIMO)

We are ok to consider leftovers from Rel.19 if they address critical commercial needs. We don’t see this enhancement is crucial to consider for Rel.20.

## 3.10 Item #10 Dynamic QCL update via MAC CE for periodic TRS

The QCL source for the PDCCH/PDSCH reception is the TRS, and since a TRS always is associated with an SSB via QCL association (‘QCL-Type-C’ and ‘QCL-TypeD’ where applicable), it is preferred to configure a TRP at FR2 with a one-to-one mapping between an SSB and a TRS (i.e., for each transmitted SSB, a TRS is transmitted in the same beam as the SSB).

However, for TRPs at mmWave frequencies with many beams and carriers, it is not possible to configure the UE with one TRS per SSB per carrier due to that UEs support a limited number of TRSs across all configured carriers (indicated by maxConfiguredResourceSetsAllCC in 38.306).

This means that when a UE moves to a new SSB beam which is not associated with an TRS (for one or more of the DL carriers), the TRP needs to update the QCL of a TRSs such that it is associated with the new SSB. The QCL update of the TRS requires RRC signaling, which introduces additional latency and overhead signaling in the system.

To mitigate this issue, it would be beneficial to introduce a more dynamic QCL source indication for TRSs, such as to support introduction of MAC CE based switching for QCL source of TRS.

In practice, this would imply that we allow that a semi-persistent TRS is used as QCL source for PDCCH/PDSCH DMRS. The UE would use this semi-persistent TRS also for radio link monitoring and beam failure detection. As the QCL source for semi-persistent CSI-RS can be updated using MAC CE, RRC reconfigurations are avoided.

However, based on the 1 TU limit for the MIMO WI, we don’t see this item is sufficiently important when compared to others.

## 3.11 Item #11 Enhanced DMRS port allocation

The CSI-RS enhancement proposal is similar to Item #7, and we don’t see an urgent commercial need to do further enhancements on 128 port CSI-RS as we are not aware of any UE vendor currently implementing support for such large resources. This is more suitable for 6G SI.

For DMRS enhancements, even in FWA may increase the likelihood and utilization of MU-MIMO, we don’t see the urgent need to support more than 24 ports. On the modification to DCI to minimize OCC multiplexing, we performed some evaluations on the proponents’ proposal.

Since this is a MIMO simulation, the CDL channel model  should be used (and not TDL is in the proponent’s evaluation). From our evaluations, we anyway used the same as the proponent (TDL-C)  we do not see any benefits for the proposal and when we used CDL-C the benefits were even smaller. See Figure 6 and 7 below.

Figure 6 DMRS Type 1 simulation in TDL-C 300 ns

Figure 7 DMRS Type 1 simulation for CDL-C 300 ns

We observe that the DMRS proposal:

- is only for large delay spread, higher order modulations and typically a large number of layers (double DMRS symbol is typically assumed configured in RRC)

- only applies in cases where current DCI is not prioritizing to maximize CDM group usage

- makes more extensive use of TD OCC (to avoid FD OCC since we are still limited to the #CDM groups per type that we have)  using/assuming double symbol DMRS

- the spec is only limited in SU-MIMO. In MU-MIMO, each UE would have a separate DCI and we can handle these aspects in the scheduler

The application when this enhancement can be used is rather limited and we prefer to avoid these types of optimizations this late in the NR design and rather focus on getting 6G specs more flexible in these aspects (not have as severe limitations in DCI as in NR).

The simulation assumptions were as follows where the proponents assumptions are given in boldface.

| Parameter | Value |
| --- | --- |
| Direction | UL |
| UE antenna | 2 x-pol |
| PUSCH layers | Fixed to 3 |
| PRB allocation | 52 PRB |
| SCS | 30 kHz |
| Channel | TDL-C CDL-C |
| RMS delay spread | 100e-9 300e-9 |
| BS antennas [RxCxP] | [2x2x2] |
| Chest | Practical |
| MCS | 23 |
| #HARQ | 3 |
| Carrier frequency | 2 GHz |
| UE speed | 3 km/h |
| Double DMRS | On/Off |
| Number of CDM groups without data | 2 |

# 4	Concluding proposals

Based on the discussion and motivation in the previous sections we propose the following:

Proposal 1	For Rel.20 NR MIMO, support at most one DL CSI enhancement and one UL enhancement

Proposal 2	In NR MIMO Rel.20 Support only Item #3 UL Enh: Cross-slot SRS, SRS frequency hopping for partial sounding and Item #6 DL CSI: Early SRS/CSI/CSI-RS triggering for UEs in IDLE/INACTIVE to CONNECTED; and SCell activation in CONNECTED

Proposal 3	For NR MIMO Rel.20, allocate [1] TU in RAN1, [0.5] TU in RAN2 and [0.5-0.75] TU in RAN4

# 5	Reference

RP-250802

# 6	Appendix: Moderator summary from RAN#107

Wide range of proposals were proposed in the contributions in agenda item 15.2.2. The following table captures some of the proposed objectives with good level of support (at least 5 supporting companies).

|  | Proposed objectives and supporting companies | Motivation and additional details |
| --- | --- | --- |
| 1 | UL Enh: UL frequency-selective (open-loop) precoder cycling Support: Ericsson (conditional), Google, IDC, Nokia, Huawei/HiSi (conditional), Nokia, DOCOMO, Samsung, Spreadtrum, vivo, Xiaomi, ZTE Against: LG, OPPO | Open-loop diversity via precoder cycling across UL PRGs, targeting 2Tx PUSCH transmission, based on legacy UL 2Tx codebook. Proponents argue, from simulation results, performance gain especially at high speed from transmit diversity gain. Opponents argue that it causes channel estimation issue (arguing for non-coherent precoders only), interference fluctuation (for small PRG size), and significant impact on implementations (gNB/UE). There are also results showing lack of performance gain. |
| 2 | UL Enh: 2Tx full-coherent UL codebook with 8/16PSK alphabets Support: CEWiT, IDC, DOCOMO, Qualcomm, Samsung, vivo, ZTE Against: Ericsson, MediaTek, Nokia, OPPO | Extending the legacy UL 2Tx codebook (QPSK-based) by adding additional precoders with higher resolution. The number of additional precoders may or may not require extension to the legacy TPMI design. Proponents argue, from simulation results, significant performance gain. Opponents argue no urgency since the current UL deployment is based on non-coherent 2Tx precoders – requesting clarification on the urgent need. |
| 3 | UL Enh: Cross-slot SRS, SRS frequency hopping for partial sounding Support: Ericsson, Huawei/HiSi, IDC, Intel, Nokia, DOCOMO, Samsung, OPPO, vivo, ZTE | Proponents argue that cross-slot SRS overcomes the inefficiency of, e.g. configuring 2 uplink symbols in one S slot, increased sounding latency in using 2 S slots (especially for antenna switching), or utilizing 2 U slots. SRS frequency hopping for partial sounding increases robustness against interference for partial sounding by allowing interpolation, At least one company opines that cross-slot SRS will introduce tighter phase coherence requirement, which needs to be discussed and addressed. Opponent argues the urgent commercial needs are unclear. |
| 4 | UL Enh: Asymmetric DL sTRP/UL mTRP beam management enhancements Support: Ericsson, IDC, Intel, Nokia (conditional), DOCOMO, OPPO, Samsung, vivo, ZTE Against: MediaTek(conditional) | PDCCH-order-triggered PRACH beam indication, and same Tx beam constraint across SRS occasions (per given SRS resource(s)) for beam sweeping, targeting FR2 Proponents argue these components are needed yet couldn’t be completed in Rel-19 due to scope issues. Opponents argue that the need is unclear. |
| 5 | UL Enh: UL frequency-selective (closed-loop) TPMI-based precoding Support: CATT, CEWiT, Huawei/HiSi, Lenovo, NEC, Nokia, DOCOMO, Samsung, Spreadtrum, ZTE Against: Ericsson, IDC, Intel, LG, MediaTek, OPPO | “Sub-band” precoding for UL based on legacy UL codebooks, targeting 4/8Tx transmission. Proponents argue based on potential performance gain which is beneficial for FWA/CPE scenarios. Opponents argue that this would result in channel estimation issue, large overhead (small # TPMI fields may diminish gain), significant impact on implementations (gNB/UE). There are also results showing lack of performance gain. |
| 6 | DL CSI: Early SRS/CSI/CSI-RS triggering for UEs in IDLE/INACTIVE to CONNECTED; and SCell activation in CONNECTED Support: CATT, Ericsson, IDC, MediaTek, NEC, Nokia, DOCOMO, OPPO, Samsung, Verizon, vivo, Xiaomi, ZTE | Early triggering for IDLE/INACTIVE CONNECTED is done via RACH MSG2/MSG4, with capability and configuration alignment via MSG1/MSG3 and SI, respectively. Early triggering for SCell activation in CONNECTED is done concurrently/in parallel with the activation message. Proponents argue that this avoids large performance loss (e.g. up to 80-90% in DL throughput for SRS-based TDD) during the transient time caused by stale CSI – detrimental in the current commercial deployments. |
| 7 | DL CSI: CSI-RS overhead reduction for larger # ports Support: CEWiT, Ericsson (conditional), IDC, Intel, MediaTek (conditional), DOCOMO, OPPO, Samsung, Tejas, vivo, ZTE, Tejas Networks, CEWiT, Reliance Jio, WiSIG Networks, IISC Bangalore, IIT Madras, IIT Hyderabad, IIT Kanpur, IIT Bhilai Against: CATT, Nokia | Additional support for 1/4 and 1/8 RE/RB/port for 48, 64, 128 ports (frequency-domain); and possibly sub-sampling in port-domain, without impacting legacy rate matching and CSI-RS resource aggregation. Proponents argue for energy saving benefit (e.g. for 7-8GHz hybrid BF). Good performance can be obtained by using advanced measurement algorithms at the UE side. Opponents argue that this would require AI/ML, especially for port-domain sub-sampling, which would be a topic for 6G. |
| 8 | DL CSI: Sub-band scalar IMR measurement report (targeting reciprocity-based operation) Support: Ericsson, IDC, Nokia, DOCOMO, Samsung, Verizon, vivo Against: MediaTek, ZTE | Sub-band (including wideband) scalar interference-plus-noise (IpN) measurement and report with larger dynamic range and higher resolution than the current 4-bit CQI. Rel-18/19 standalone aperiodic report on PUSCH can be reused to minimize spec impact. Proponents argue that interference measurement is tied with CQI and SU-MIMO assumption in legacy – introducing inaccuracy and latency (due to the use of open-loop link adaptation) for MU-MIMO. The benefit comes from higher accuracy and absence of SU-MIMO assumption in the legacy CSI framework. Opponents argue that how to use this report together with CSI (including channel measurement) is unclear. |
| 9 | Multi-beam: UEI/ED beam switching (left over from Rel-19 MIMO) Support: ASUSTek, Apple, Ericsson, Futurewei, IDC, LG, KDDI, Lenovo, DOCOMO, Spreadtrum, Transsion, vivo Against: CATT, Intel, Nokia, MediaTek, OPPO, Samsung, ZTE | Beam switching based on Rel-19 UEI/ED beam reporting using gNB confirmation (i.e. NW-controlled). This is currently being discussed in Rel-19 MIMO. Proponents argue from beam application latency reduction perspective. Opponents argue that FR2 deployment is limited and no urgency and/or the latency reduction benefit compared to UEI/ED beam reporting is unclear. Some prefer to wait until Rel-19 MIMO is complete, else for 6G. The applicability of this topic is contingent on the conclusion of Rel-19 MIMO which makes it difficult. |
| 10 | Multi-beam: Dynamic QCL update via MAC CE for periodic TRS Support: Apple, Ericsson, IDC, Verizon, vivo Against: Huawei/HiSi, Nokia, DOCOMO, Samsung, ZTE | Support for MAC CE based QCL update for common signals especially periodic TRS Proponents argue that this is to avoid frequent RRC reconfiguration (legacy spec uses RRC to update QCL for common signals). Opponents argue that since periodic TRS is cell-specific (UE-common), there is no need for updating this in a UE-specific manner. This topic has been discussed a few times and is controversial. |
| 11 | Enhanced DMRS port allocation Support: Tejas Networks, CEWiT, Reliance Jio, WiSIG Networks, IISC Bangalore, IIT Madras, IIT Hyderabad, IIT Kanpur, IIT Bhilai | Enhanced Multiplexing requirements due to Enhanced CSI ports to 128 or greater. To support a higher number of transmit-receive points (TRP) in mTRP (multiple-TRP) cases. - Introduce DMRS enhancement to support beyond 24 orthogonal DMRS ports Performance enhancements in high MCS, high delay spread to cater to FWA, CPE use cases. Improvement in the performance for deployments using higher SCS (larger impact of delay spread). Enhance mTRP deployment to manage uneven layer performance from issues like time misalignment. Address DMRS port allocation for increased DMRS ports. - Enhance number of DM-RS ports by increasing the OCC length. |

RP-250647 indicated general support for enhancements on UL capacity/coverage enhancements and DL CSI acquisition enhancements.

Based on the contributions submitted to agenda item 15.2.2, the following observations are made.

- Overall, the proposals on Rel-20 MIMO can be categorized into the following three areas

  - Uplink enhancement (items #1, #2, #3, #4, #5)

  - Downlink CSI enhancement (items #6, #7, #8)

  - Multi-beam enhancement (items #9, #10)

  - DMRS enhancement (item #11)

- Good level of support on uplink enhancement, downlink CSI enhancement, and DMRS enhancement

  - However, compared to other proposals, item #5 (UL frequency-selective closed-loop precoding) has relatively large number (6) of companies with negative views

- Compared to uplink enhancement, downlink CSI enhancement, and DMRS enhancement, the level of support on multi-beam enhancement was lower or had significant number of companies who had negative views.