3GPP TSG RAN Meeting #108	RP-250875
Prague, Czech Republic, June 9-13, 2025

Agenda Item:	15.1.1

Source:	Ericsson

Title:	Discussion on Rel-20 AI/ML for PHY

Document for:	Discussion, Decision

# 1	Introduction

The Rel.20 AI/ML for NR PHY is discussed:

- RAN2 leftovers and Rel.19 finalization

- RAN3 impacts

- RAN1/RAN2/RAN4 impact on CSI compresison

# 2	Discussion

## 2.1	Rel.19 leftovers related to RAN2 work

While the discussion around the Rel.19 work item is approaching the end in RAN2 (just one meeting left), it is possible that some of the functionalities necessary to enable an efficient AI/ML ecosystem in RAN will not be included in the Rel.19 work.

For example, related to UE- and NW-side data collection, it is not clear yet whether dynamic activation/deactivation of data collection will be supported in the Rel.19 framework. That dynamic functionality will be supported for the inference, but the discussion is still open for the data collection. This feature turns out to be useful to reduce the redundancy of collected data whenever the network or the UE does not any longer need to perform data collection, thereby reducing the UE power consumption.

Similarly, still on the topic of data collection, other mechanisms e.g. based on L1 events, can be introduced to trigger the data collection. In Rel.19, only L3-event driven data collection will be supported, however considering the use cases at hand (i.e. beam management, CSI predictions), the introduction of L1-event driven data collection (based on L1 beam level measurements) might be considered in order to reduce the overhead of RRC reconfigurations to setup/release the data collection whenever needed/not needed.

While the exact details of the RAN2 leftovers to be included in the Rel.20 WID can be further discuss during plenary #109 based on RAN2 Rel.19 outcome, we believe that it is important at this point to agree to allocate time units in the Rel.20 WI to RAN2 leftovers. With TU in place, the exact content of the RAN2 objective can be discussed later.

Include RAN2 leftovers related to beam management, as an objective of the Rel.20 WID on AI/ML Air Interface. The content of the RAN2 leftovers to be discussed during next RAN plenary after RAN2#131 (e.g.  enhanced procedures for the configuration and activation of UE- and NW-side data collection, etc.)

## 2.2 On RAN3 impacts to support AIML

RAN3 was an involved WG in the Rel.19 WID on AI/ML for PHY.

However, with respect to AI/ML supported positioning use case 3A, namely NG-RAN node assisted positioning with a gNB-side model, RAN3 will not complete the work needed to support the use case in full.

Indeed, RAN3 has triggered an LS to SA2 indicating a number of solutions in support for use case 3a. SA2 has confirmed that solutions where the RAN selects UEs for which training data is needed and should be generated can only be specified in Rel. 20.

It is worth noting that the solution for training data collection in use case 3a, which will be available in Rel.19, is based on collection of such data from UEs that are being positioned due to a request from an external client. If the RAN is involved in such positioning process, the RAN may receive UE location information for these UEs, which enables to construct some training data.

However, this technique is not sufficient to gather training data needed in the required amounts because:

- In most of the cases, a UE positioning process is based on the LMF receiving GNSS coordination directly from the UE. The RAN is not involved in this process and therefore the RAN cannot collect training data for these positioning events.

- The RAN may need training data for UEs with specific capabilities and in specific radio conditions. It is unlikely that such UEs are the subject of positioning sessions triggered by an external client, or at the very least, such case would be a coincidence. Building reliable training data set cannot be left to coincidence.

For this reason, the work on NG-RAN node assisted positioning with a gNB-side model should be continued in RAN3 also in Rel.20, to introduce solutions where the RAN can proactively select UEs for which training data is collected.

Additionally, the framework defined in RAN2 in Rel.19 may have impact in RAN3. For example, RAN2 agreed that the data collected by the UE can be retained by the UE at handover, and eventually transmitted to a gNB different from the gNB that configured the data collection. A solution for example to transfer back the retrieved data to the gNB that configured the data collection might need to be developed in RAN3.

Further work might also be needed in RAN3 to develop a framework to enable a more efficient OAM-centric data collection. According to RAN2, the OAM-centric data collection can be supported via the classical framework of management-based MDT (and in particular on immediate MDT). Enhancements in the interface between the OAM and the gNB could be discussed in RAN3 to enable a more efficient configuration of the UE for NW-side data collection, in order for example to allow the RAN to collect data from the UE in certain conditions.

Include RAN3-related objectives in the Rel.20 WID on AI/ML Air Interface:

  - For the work on NG-RAN node assisted positioning with a gNB-side model, introduce solutions where the RAN can proactively select UEs for which training data is collected

  - Support the necessary inter-node procedures to transfer the data retrieved from the UE from one gNB to the gNB that configured the UE for NW-side data collection

  - Necessary enhancements to enable more efficient UE configurations for OAM-centric data collection.

## 2.3	On CSI compression

Here we first discuss the status of the outcome of the CSI compression study in RAN1 and RAN4:

### 2.3.1 RAN1 parts

RAN1 has concluded the performance-complexity trade-off evaluation study and for Use case 0, the Rel-19 study showed improved performance-complexity trade-off comparing to Rel-18 study, due to the use of more optimized AI/ML model structures and the use of different inputs. For Use case 2, Use case 3 and localized model does not necessarily result in better performance-complexity trade-off than Use Case 0 but may have higher complexity. RAN1 has concluded that Use case 0 is recommended and Case 2 and 3 can be further considered.

Complexity wise, beside AI complexity, Use case 2 requires additional complexity at both UE and NW side to mitigate the impact of UCI loss and handle rank adaptation. This use case also adds extra complexity and cost at the gNB side to store and keep track of the “historic CSI information” per UE. Hence, the performance gain vs complexity/overhead for Use case 2 cannot be justified. Use case 3 requires additional complexity for performance monitoring and error cause analysis.

Moreover, the current RAN4 study focuses only on use case 0. Considering the large effort needed for studying the performance testing feasibility for use case 0, it is unlikely that RAN4 will find time to study the performance testing feasibility for use case 2 and use case 3 in Rel-19.

Hence, to keep the RAN1 and RAN4 workload manageable, we propose to only consider Case 0 in a potential normative phase.

Only CSI compression Use Case 0 is specified in a potential Rel-20 normative phase

RAN1 has identified potential specification impacts for remaining inter-vendor training collaboration options

- Reference model/structure specification (for sub-option 3a-1 and Direction C)

- Format and contents of parameter/dataset/information exchange (for sub-option 3a-1, 4-1)

On the inter-vendor collaboration options, our observation is that sub-option 4-1, 3a-1 and Direction C are all fully functional as stand-alone options, hence multiple options for the same feature should not be specified.

For direction C, RAN1 has made conclusions that the fully specified reference model is equivalent to the test/reference model for RAN4 setting performance requirements, and if specified, the reference/test model will be specified in RAN4. Considering that RAN4 is still study the feasibility of specifying a model, it must be discussed how a WID can be drafted in the plenary in case direction C is selected, before RAN4 has concluded on their study.

For direction A Option 3a-1 and Option 4-1, an LS was sent from RAN1 to RAN2 on signalling feasibility of dataset/model-parameter exchange Error! Reference source not found.. Based on the Reply LS from RAN2 Error! Reference source not found., for OTA based signalling solutions, RAN2 has no consensus on feasibility; for non-OTA based solutions, RAN2 identified a few candidate solutions, and from RAN2 assumes that they can be supported within Rel-19 existing architecture framework. RAN2 has send an LS asking RAN3/SA2/SA5 to confirm RAN2 assumption on non-OTA solutions and awaiting the response.

Hence, for Direction A, the feasibility of standardized signaling for dataset/model-parameter exchange from NW-side to UE-side is not confirmed yet by other WGs.

There is no consensus in RAN2 for Direction A OTA based solutions

There is no consensus in SA5 for Direction A non-OTA based solutions

Hence we propose

Discuss in RAN plenary how to proceed with normative work if Direction A is chosen, as there is no consensus in 3GPP for neither OTA nor non-OTA solutions

In addition, for Option 4-1 and Option 3a-1 there are two different flavours for achieving the same type/purpose of inter vendor training collaboration, i.e., both options belong to direction A, which is based on NW-side first training with information exchange from NW-side to UE-side to assist UE-side offline engineering for model training/testing. Several companies’ evaluation results showed that Option 3a-1 with {Target CSI} sharing and Option 4-1 achieve similar model performance, however, Option 3a-1 requires significantly higher specification efforts comparing to Option 4-1.

In addition, Option 3a-1 has two sub-options, with and without CSI target sharing which we think is needed for UE-side offline encoder training, encoder quantization optimization, performance verification/testing before UE-side deploying the trained/optimized encoder to the devices. To avoid double specification for the same functionality, only one of these three options (3a-1 w/sharing, 3a-1 wo/sharing and 4-1) should be specified if Direction A is necessary for the normative phase, and our preference is Option 4-1.

If Direction A is necessary for the normative phase, then only Option 4-1 should be specified

In addition, for both direction C and option 3a-1, the question on how to represent an AI model/model-structure in 3GPP specification has not been studied at all and the responsibility of reference model and/or test model structures between RAN1 and RAN4 is not clear, it needs to be clarified in the WI objectives. See also the discussion in Section 2.3.2 below.

If Direction C or sub-option 3a-1 is selected for normative work, then, RAN1/RAN4 responsibility on ref/test model structure study/specification needs to be clarified in WID and it needs to be discussed how to proceed given that RAN4 has not yet concluded on the feasibility to specify models.

From RAN1 conclusions, the RAN1 specification impacts common to all inter-vendor training collaboration options is model pairing, capability reporting, inference aspects, training data collection and performance monitoring (If needed).

The system/link/eventual KPI (e.g., throughput, SNR, ACK-NACK, BLER) based monitoring method can be used by the NW to detect potential AI/ML CSI-compression feature/functionality performance degradation, thereby, triggering model LCM actions (e.g., model fallback) or/and performing further error cause analysis (e.g., whether the system/link performance degradation is caused by the AI/ML CSI-compression functionality or other issues). The system/link/eventual KPI based monitoring methods has low complexity and no additional signaling overhead is incurred for monitoring data collection. A large benefit is that such monitoring on the NW side can take into account MU-MIMO performance as well.

Hence, the NW can perform frequent monitoring of system/link/eventual KPIs and use it as a first step for detecting potential AI/ML functionality failure.

Eventual KPI based monitoring has low complexity, low overhead, and can capture network MU-MIMO performance. The NW can perform frequent monitoring of eventual KPIs via NW implementation-based solutions and use it as a first step for detecting potential AI/ML feature/functionality failure.

For NW-side intermediate KPIs based monitoring based on the ground-truth CSI (referred to as monitoring Case 1 in performance monitoring evaluation studies for this use case), to derive the intermediate KPIs, the NW needs to collect the ground-truth CSI and the encoder output. The NW feeds the reported encoder output into its decoder model and generates the reconstructed CSI samples and then the NW compares the reconstructed CSI samples with the associated ground-truth CSI samples to calculate the intermediate KPIs.

In addition, when detecting/predicting potential AI/ML functionality failure based on the observation of eventual KPI degradation and/or the observation of intermediate KPI degradation, the NW performs further error cause analysis. To understand the cause of the failure and potentially resolve it. The cause of failure can be either caused by:

- Data drift, i.e. the field channel conditions are not reflected well in the training data set.

- UE-side issue: e.g., the actual CSI construction model does not pair well with the NW decoder due to variation in the latent space representation caused by the offline engineering (e.g. in case of option3a-1), compiling issues (e.g. in case of 3b), or mismatch in the training objectives (e.g. in case of option 4)

- NW-side issue, i.e. the jointly actual decoder and the nominal encoder (in form of dataset/parameters/model) is poorly trained and not performing well in the field.

RAN1#120 has made the following conclusion regarding identification of root cause for performance issues:

For NW-first training, in inter-vendor training collaboration Direction A and C, identification of root cause for performance issues can be achieved at least by the following

- Data distribution mismatch that is identified by either NW side or UE side

- Via target CSI and CSI feedback pair report, e.g., with the understanding that NW side runs inference using the NW side reference CSI generation part and compare with the inference using UE side CSI generation part.

- Note that this conclusion does not preclude other methods.

- Note that triggering of root cause detection may be after detecting performance degradation

If CSI compression is specified, it is necessary to specify UE reporting high resolution target CSI and CSI feedback pair to enable NW-side intermediate KPIs based performance monitoring and performance degradation error cause detection for two-sided CSI-compression use case.

### 2.3.2 RAN4 parts

RAN4 has studied two options for developing a test decoder. For one, the test decoder is specified directly whereas for the second option, a dataset and reference encoder is standardized, which enables TE vendors to create reproducible, interoperable test decoders. In both cases, the information to be captured in 3GPP is similar. The group has successfully managed to exchange and align datasets and models and shown that the approach may be possible.

The study has not as yet addressed a number of key issues:

- The training dataset for the test model for the study was based on a RAN1 UMa assumption. For a real standardization, in our view the test model should be well enough trained that it could be expected to provide reasonable performance in real scenarios. The reason is that the test model will define a latent space and provide a basic interoperability. If the test model would not be trained with a reasonable training set, there would be little point in performing performance testing. Further discussion is needed on reasonable assumptions for the training dataset.

- There would also be a need for an effort to align and form a mixed training set between companies; this has been observed during the study and would be critical for a work item.

- The model structure for the test decoder needs to be agreed, and assumptions for the structure for the encoder(s). During the SI, a rather arbitrary assumption was made on the model structure. For a real test decoder, the structure should be realistic, ensuring expected good performance of such a decoder in real environments and without being over-complex. Furthermore, the assumptions on complexity of the encoder should be realistic.

- Representation of the model in 3GPP specifications. For the study, ONNX has been used for exchanging models. This has been a useful exercise in exchanging and representing models between companies in a generic format. However, ONNX is open source and subject to change, and is not suitable for 3GPP specifications, considering the needs for accessibility over a long period of time, change management etc. A means of representing models and parameters in a managed way in 3GPP specifications is needed.

These aspects needs to be considered if a WID is drafted, and although RAN4 has performed useful study of model and dataset alignment in the SI, considerable further work is needed in a work item and the TU allocated needs to be reflect this.

From RAN#107, the moderator summary contains the following assumption for the purpose of TU planning:

- Include CSI compression for two-sided model as part of Release 20 5G-Adv

  - Final decision on whether to support CSI compressions for two-sided model as part of Release 20 5G-Adv will be made considering the outcome of Release 19 AI/ML air interface study item

  - If final decision is to not include CSI compression, TU for Release 20 5G-Adv AI/ML air interface will be revised accordingly

- [2] TUs in RAN1 and [2] TUs in RAN2 for Release 20 5G-Adv AI/ML air interface

Our proposal for RAN4 TU, if CSI compression is supported for normative phase, is as follows:

If CSI compression is included in Rel.20 normative work , assume [1.5] TU in RAN4 if “test decoder” is standardized in RAN1, [2.5] TU in RAN4 if test decoder standardized in RAN4

# 3	Proposal

In the previous sections we made the following observations:

Observation 1	There is no consensus in RAN2 for Direction A OTA based solutions

Observation 2	There is no consensus in SA5 for Direction A non-OTA based solutions

Observation 3	Eventual KPI based monitoring has low complexity, low overhead, and can capture network MU-MIMO performance. The NW can perform frequent monitoring of eventual KPIs via NW implementation-based solutions and use it as a first step for detecting potential AI/ML feature/functionality failure.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Include RAN2 leftovers related to beam management, as an objective of the Rel.20 WID on AI/ML Air Interface. The content of the RAN2 leftovers to be discussed during next RAN plenary after RAN2#131 (e.g.  enhanced procedures for the configuration and activation of UE- and NW-side data collection, etc.)

Proposal 2	Include RAN3-related objectives in the Rel.20 WID on AI/ML Air Interface:

a.	For the work on NG-RAN node assisted positioning with a gNB-side model, introduce solutions where the RAN can proactively select UEs for which training data is collected

b.	Support the necessary inter-node procedures to transfer the data retrieved from the UE from one gNB to the gNB that configured the UE for NW-side data collection

c.	Necessary enhancements to enable more efficient UE configurations for OAM-centric data collection.

Proposal 3	Only CSI compression Use Case 0 is specified in a potential Rel-20 normative phase

Proposal 4	Discuss in RAN plenary how to proceed with normative work if Direction A is chosen, as there is no consensus in 3GPP for neither OTA nor non-OTA solutions

Proposal 5	If Direction A is necessary for the normative phase, then only Option 4-1 should be specified

Proposal 6	If Direction C or sub-option 3a-1 is selected for normative work, then, RAN1/RAN4 responsibility on ref/test model structure study/specification needs to be clarified in WID and it needs to be discussed how to proceed given that RAN4 has not yet concluded on the feasibility to specify models.

Proposal 7	If CSI compression is specified, it is necessary to specify UE reporting high resolution target CSI and CSI feedback pair to enable NW-side intermediate KPIs based performance monitoring and performance degradation error cause detection for two-sided CSI-compression use case.

Proposal 8	If CSI compression is included in Rel.20 normative work , assume [1.5] TU in RAN4 if “test decoder” is standardized in RAN1, [2.5] TU in RAN4 if test decoder standardized in RAN4