3GPP TSG RAN Meeting #108 	RP-250876

Prague, Czech Republic, June 9-13, 2025

Title:	Candidate RAN4 led NTN topics for Rel-20 5G Advanced

Source: 	Thales, Eutelsat Group, Aalyria, SES, Sharp, Airbus, ST Engineering iDirect, Intelsat, ESA, Bosch, Gatehouse Satcom, Novamint, Fraunhofer IIS, Fraunhofer HHI, Leonardo, Hispasat, Echostar

Type:	discussion

Document for:	discussion

Agenda Item:	15.4.1 - REL-20 proposals led by RAN4/ General

# Introduction

This document proposes to define a number of RAN4 led activities for NR-NTN radio interface as part of Rel-20 5G Advanced work plan.

# Candidate RAN4 led NR-NTN activities for Rel-20 5G Advanced

## Context

At RAN#107, in RP-250817 “Moderator's summary for RAN4 led REL-20 topics”, a list of enhancements relevant for NTN have been proposed:

- NTN HPUE

  - Study whether the high power than PC1 for IoT-NTN and/or NR-NTN, and what high power classes should be considered.

  - FFS whether the proposed high power class can be applied to handheld + FWA or only FWA.

  - Need differentiate NR-NTN and IoT-NTN in terms of power class to be discussed.

- Enhancements for Ku band NTN, including

  - Intra-SAN intra-band CA in Ku band

  - Fixed and other mobile types VSAT (e.g., with small antenna size or LEO only) RF [and RRM] requirements for single carrier

    - For small antenna size, HD-FDD would be needed.

  - Smaller channel bandwidth support (e.g. 3, 5 MHz) and other Rel.19 leftovers

- NTN CA including intra-band and inter-band UL/DL CA for NR-NTN

- NTN mobile VSAT

- Testing method for VSAT [Moved to UE OTA enhancement section]

- NTN power domain enhancements

- MSR for in-band IoT-NTN in NR

- Irregular CBW 7MHz for NR-NTN

- Mobility between NR-TN/LTE TN and NR-NTN in the RRC_CONNECTED state.

  - Need check whether RAN2 package includes it.

- Repeater for NTN

NOTE: there are overlapping objectives above and FFS on how to merge them.

- Enhance test method [UE OTA enhancement]

- FFS on RedCap NTN

- Radiated test method should be developed for mobile VSAT on Ku and Ka band

In line with the above, some of the identified topics are further detailed in terms of justification and potential objectives.

## Proposed RAN4 led NR-NTN topics for Rel-20 5G advanced

### Intra SAN Carrier aggregation (at both SAN and UE level)

#### Justification

When NR support for Non-Terrestrial Networks (NTN) was introduced in Release 17, support for Carrier Aggregation (CA) was not included in the work scope.

As a result of that, this feature is not available for new NR NTN planned for the near future. As we are nearing deployment of new constellations, the criticality of this features is becoming more and more apparent.

Furthermore, the lack of Carrier Aggregation support for NTN creates a significant feature gap and disparity with the current NR terrestrial solution, and may thus potentially result in a disparity with some satellite systems being planned or deployed.

There are also a number of other technical aspects that strongly justify the introduction of CA.  For example, initial channel allocations for NTN may in certain regions be constrained by existing/legacy services, whereby channels may become available in non-contiguous areas of spectrum.

#### Objectives

The Core Part objectives are as follows:

- Specify the introduction of Carrier Aggregation (CA) for NR NTN on DL and UL for all NR NTN bands (below 7.125 GHz and above 10 GHz)

  - Focus on the intra-SAN scenario, where component carriers are co-located in the same Satellite Access Node (SAN) and supported by the same payload.

  - Support for intra-band contiguous and non-contiguous CA

  - Support for inter-band CA

- Specify the necessary RF and RRM requirements

#### Working groups

Lead WG: RAN4 (addressing both RF/OTA, Demod/RRM)

Contributing WG: RAN1, RAN2

### Support of ultra-small mobile VSAT

#### Justification

This activity aims at defining the requirements for the support of a new “ultra-small Mobile VSAT” (equivalent aperture < 20 x 20 cm) operating with NGSO constellation (and possibly MEO, GEO/GSO) in above 10 GHz bands allocated to satellite services (i.e. for both Ka and Ku-band), as requested by the verticals in [1].

Note 1: Mobile VSAT are referred in ITU-R as Earth Station in Motion (ESIM) which is defined as “an Earth station that is placed on a moving platform, such as a ship at sea, a moving train, or an aircraft in flight” [2]

Such ultra-small Mobile VSAT is intended to be installed on different moving platforms, including aircraft, ships, and different types of land vehicles, with stringent size, weight and power constraints to support data-rich and reliable connections. They are expected to implement flat panel antenna able to generate electronically steered beams to track the orbiting satellites.

Two mobile VSAT may be considered

- FDD mode: which would require at least 1 Rx and 1 Tx. In case there are 2 Rx, the second one can be used either for connectivity with the serving cell (1Rx receiving useful data) and measurement purposes of neighbour cell (1 Rx measuring).

- HD-FDD mode: requiring 1 Rx and 1 Tx

The maximum speed shall be up to 1500 km/h (maximum relative speed with respect to the ground for commercial aircraft) as explained per [1]. The connectivity shall cover at least LEO@600km and LEO@1200km NGSO types.

Note 2: The Release 18 NTN work items scope for operation above 10 GHz, does not support Mobile VSATs (ESIMs) connected to NGSO Satellites in 3GPP previous releases [3]. However, since the WRC-2023, such mobile VSAT can operate with NGSO in all portions of Ka band and Ku band.

Related references:

- [1] RP-242638, Market considerations on Mobile VSAT operating with NGSO/GSO from verticals point view, Source: Airbus, Thales et all.

- [2] Satellite issues: Earth Station in Motion (ESIM) https://www.itu.int/en/mediacentre/backgrounders/Pages/Earth-stations-in-motion-satellite-issues.aspx

- [3] 3GPP TS 38.101-5 v18.7.0 NR; User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements.

#### Objectives

##### Core part

The objectives of core part for the introduction of an ultra-small Mobile VSAT type (equivalent aperture < 20 x 20 cm) as requested in [1] UEs for NR_NTN (Non-Terrestrial Networks) operating with NGSO in satellite service allocated bands in above 10 GHz:

- Specify RF requirements for the ultra-small Mobile VSAT for NR NTN in all Ka-bands (e.g. n512, n511, n510) and all Ku-bands (e.g. n5xx, n2xx) allocated to NTN satellite services

  - Necessary mobile VSAT Tx/Rx requirements (if any)

    - to define VSAT Type 6 for the ultra-small Mobile VSAT communicating with LEO (and possibly MEO, GEO/GSO), in both Ku and Ka-band, based on the RF characteristics of Fixed VSAT Type 3 and Mobile VSAT Type 5 (See table 9.2.1.0-1 in TS 38.101-5)

      - 2 duplex modes may be considered: FDD or HD-FDD mode

    - Note 1: The minimum performance values from existing VSAT Type 3 and 5 are including but are not limited to:

        - Minimum peak EIRP

        - Minimum OTA Rx sensitivity

        - Minimum Pointing/Tracking accuracy

  - Note 2: Other mobile VSAT UE types (if any) and their respective characteristics to be discussed in RAN4

- Specify RRM Requirements as appropriate

  - Ultra-small Mobile VSAT configuration assumption with 1Tx & 1Rx (e.g. for land/maritime vehicle applications) as well as 1Tx & 2Rx (e.g. for aeronautical applications)

  - Timing error requirements enhancements in section 7.1C of TS 38.133

##### Performance part

Upgrade performance part for ultra-small Mobile VSAT as necessary.

#### Working groups

- Lead WG: RAN4

### Testing methodology for VSAT

#### Justification

When RAN approved the WI NR_NTN_enh for VSATs operating above 10 GHz there was an implicit and obvious consequence on test methods since the size of VSATs is much larger than the size of the FR2 handsets used to develop the original FR2 NR radiated test systems whose quiet zone was only 15 cm (now 40 cm). Also, the FR2-NTN frequency range has a lower limit of 17.3 GHz, well below the 24.25 GHz of existing approved FR2 radiated test methods. When RAN approved the WI Introduction of Ku bands for NR NTN in June 2024, the lower frequency dropped further to 10.7 GHz.

In addition to the frequency range and VSAT device size issues, the introduction of Ka band added the issue of polarization. Current radiated test systems are all based on cross polarized antennas, but Ka band operates with circular polarization. Unfortunately, there is no obvious way to convert existing test systems to use circular polarization without reverting to using a vector network analyser approach which would mean the loss of the L3 signalling connectivity normally provided by a base station emulator, which lacks the phase control to generate stable circular polarization.

The need to address the large and growing gap in VSAT testability is further evidenced by a Feb 2025 industry press release which announced the first successful connection between a Ku band VSAT using an NR air interface and a LEO constellation. This indicates the demand for 3GPP-based NTN standards since current commercial developments are actually ahead of 3GPP rather than being a couple of years behind the standards There has already been a 9-month delay with no progress since these testability issues were first formally raised at RAN #104. If 3GPP does not act very soon, then the test infrastructure to complete the work that 3GPP started for NTN above 10 GHz will not be in place when it is needed.

The key current and future VSAT testability issues are:

- VSAT device sizes exceed by far what is currently testable in existing 3GPP FR2 test systems

- The introduction of Ka band with its 17.3 GHz lower limit falls well below the range of existing FR2 test systems of 24.25 GHz.

- The introduction of Ku band (10.7 and 14.5 GHz) further challenges the operating range of existing test methods.

- The need to support circular polarization adds a significant challenge to existing test systems which may require regressing to using non-signalling methods.

The Revised SID in RP-260640 is the proposal for how to progress these issues by extending the scope of the existing FS Study on NR FR2 OTA (Over the Air) testing Phase 3 [RAN4 SI: FS_NR_FR2_OTA_Ph3].

#### Objectives

The objectives of the study are:

- Investigate the applicability of current 3GPP test methods established in TR 38.810 and TR 38.871 for UE testing for the following example VSAT device types (from 38.101-5 v18.5.0 Table 9.2.1.0-1:

  - Mobile VSAT UE type 4 and 5 (mechanical and electronic steering antenna)

- Investigate how to support both linear and circular polarization measurements in uplink and downlink

- Confirm that an L3 signalling approach to VSAT testing is to be used

- Investigate the required size of the quiet zone based on industry needs. A preliminary assumption for quiet zone is 60 cm, larger volumes will be needed for some antenna types

- Test methodologies are studied for the following frequency ranges:

  - Ka bands (i.e., existing bands n510, n511, n512)

  - Ku bands with DL from 10.7 to 12.75 GHz and UL from 13.75 – 14.5 GHz)

Study ETSI Harmonised Standard for access to radio spectrum ETSI EN 303 980 and any other applicable specifications in other regions

#### Working groups

- Lead WG: RAN4

### Mobility between NR-TN and NR-NTN in the RRC_CONNECTED state

#### Justification

Rel-18 specified mobility enhancements between NR-TN and NR-NTN, but so far only the performance part is specified for FR1 and FR2 mobility in RRC_CONNECTED state for NR-SAN to NR-SAN, but not between NR-TN to NR-SAN.

As requested by 5GAA and individual companies since Rel-19, connected mode mobility is important for the user experience of combined TN/NTN scenarios where the frequent HO without interruptions are essential.

#### Objectives

##### Core part

General assumptions:

- GSO (Geo Synchronous Orbit) and NGSO (Non-Geo Synchronous Orbit).

- NGSO includes Low Earth Orbit (LEO) and Medium Earth Orbit (MEO).

- Earth fixed tracking area. Quasi-Earth fixed & Earth moving cells for NGSO

Specify necessary enhancements to support NR TN to NR NTN handover in RRC connected mode.

This work considers existing methods from NR TN as well as outcome of Rel-18 NR NTN WI outcome as baseline for NTN-TN mobility.

- Specify NTN-TN measurement/mobility and service continuity enhancements [RAN4]

##### Performance part

The performance part objectives are applicable to the NR-NTN deployment in FR1 and FR2 bands.

- Specify RRM performance requirements and test cases [RAN4]

- Specify UE demodulation and CSI reporting requirements [RAN4]

- Specify satellite access node demodulation requirements [RAN4]

- Specify satellite access node conformance tests [RAN4]

#### Working groups

- Lead WG: RAN4

### Support of Multi-Standard Radio (MSR) for in band operation of NB-IoT-NTN in NR

#### Justification

For an efficient support of in-band operation, the support of Multi-Standard Radio (MSR) Base Station might be needed.

#### Objectives

Create TS 37.181 (NTN equivalent of TS 37.141 NR, E-UTRA, UTRA and GSM/EDGE; Multi-Standard Radio (MSR) Base Station (BS) conformance testing) and TS 37.108 (NTN equivalent of TS 37.104 NR, E-UTRA, UTRA and GSM/EDGE; Multi-Standard Radio (MSR) Base Station (BS) radio transmission and reception).

#### Working groups

Lead WG: RAN4 RRM

Contributing WG : RAN1

### Missing Core Requirements for additional NR NTN FR1 Channel BWs

#### Justification

When NR NTN was first specified in Release 17, certain channel bandwidths, namely CHBW above 20 MHz, were not specified, primarily due to a concern regarding lack of time during Release 17, and due to the mistaken assumption that NTN operators would not have access to blocks of spectrum larger than 20 MHz.

The latter statement, in particular, is not true, since:

- Some NTN operators in S-band have access to the full 30 MHz of band n256 in certain regions, and some potential allocations of additional S-band elsewhere in the world (e.g. South/Central America) are leading to up to 35 MHz of total available spectrum, which in some instances may be allocated to a single operator.

- In Release 19, there was an operator demand from several operators to introduce new L-band definitions to combine the Extended L-band (n253) with the Standard L-band (n255), giving rise to bands with a total combined DL of up to 41 MHz (i.e. n251, n250).  Some NTN operators have access to the full 41 MHz of L-band spectrum in certain regions and are wishing to utilize it.

- In some instances, operators may opt to pool the spectrum beyond the allocation of any given single operator in a given geographical area, by operating a common shared network, thus improving flexibility and spectral efficiency.

Lastly, even when an operator may have access to interrupted blocks of spectrum across the full band (e.g. 41 MHz), the deployment of a larger channel encompassing the full band would allow an operator to flexibly schedule users across the block, even in sub-blocks less than the minimum 3 MHz required for a single standalone channel to be deployed.  This also helps improved spectrum efficiency by minimizing guard bands and giving more flexibility in the dynamic allocation of resources.

Later on in Rel-17 and early Rel-18, in order to comply with the submission requirements for ITU-R IMT-2020 Satellite Self-Evaluation, support for 30 MHz channel BW was introduced, but with a reduced and incomplete set of requirements, such that some companies requested that a NOTE be added, clarifying that further requirements were needed in order to make the 30 MHz CHBW deployable.

Similarly, now that the new n251 and n250 bands have been introduced in Rel-19, the lack of Core requirements for all CHBW from 25 MHz and up has created significant issues in the WI procedures.  During the Ku band Work Item definition, great pushback was received in introducing CHBW above 20 MHz for Ku band in its FR1 configuration, however, CHBW up to at least 50 MHz are critical for Ku band operators.

It is thus very urgent to complete the Core requirements for the missing CHBWs, namely 25, 30, 35, 40, 45 and 50 MHz, wherever they are missing, to allow in particular UE implementations to be completed and be able to support such operator deployments.

#### Objectives

The objectives of the core part are as follows:

- Specify the missing core UE and SAN for the following FR1 Channel BWs: 25 MHz, 30 MHz, 35 MHz, 40 MHz, 45 MHz, 50 MHz

  - Specify RF, RRM, Performance/Demodulation and Conformance requirements as needed

  - Take the work in NR_NTN_Ku_bands into account, to avoid any duplication.

#### Working groups

Lead WG: RAN4

### Very High Power UE

#### Justification

Increasing the transmit power of UE has significant benefits on extending uplink coverage area and improving the experience of cell edge users.

In Rel-19, as part of HPUE_NR_FR1_FDD_R18WID, NR-NTN capable UE operating in NTN bands with Tx power of 29 dBm and 31 dBm are defined

Enhanced Tx power would extend the coverage, and therefore, it is proposed to define UE with Power Class 0.5 (i.e. up to 33 and 34 dBm) in FR1 NR-NTN bands (i.e. n256, n255, n254, n253) in order to gain additional 2 or 3 dB on the uplink.

#### Objectives

The objectives of the core part are as follows:

- Specify the band specific RF requirements for power class 0.5 NR-NTN FDD bands in FR1 including

  - UE maximum output power and Tx power tolerance

  - A-MPR requirements if needed

  - sensitivity degradation requirements if needed

Note 1: Both 1Tx and 2Tx UE architecture may be considered in this WI.

#### Working groups

Lead WG: RAN4

# Conclusion and Way forward

Proposal 1: RAN to discuss at next plenary candidate RAN4 led NR-NTN topics as part of the Rel-20 5G Advanced work plan in decreasing order of priority:

- Intra SAN Carrier aggregation (at both SAN and UE level)

- Support of ultra-small Mobile VSAT: definition of type 6 and/or additional type in above 10 GHz band Testing methodology for VSAT

- Mobility between NR-TN and NR-NTN in the RRC_CONNECTED state

- SAN Multi-Standard Radio (MSR) radio transmission and reception.

- Missing Core Requirements for additional NR NTN FR1 Channel BWs

- Very High Power UE

END