3GPP TSG RAN Meeting #108 Meeting 	RP-250877

Prague, Czech Republic, June 9-13, 2025

Title:	Non-Terrestrial Networks (NTN) for NR Phase 4

Source: 	Thales, ESA, Eutelsat group, Gatehouse Satcom, Hispasat, Viasat, Inmarsat, Intelsat, Iridium, Novamint, Sateliot, SES, TNO, Bosch, ST Engineering iDirect, ETRI, Airbus, Fraunhofer IIS, Fraunhofer HHI, Leonardo

Type:	discussion

Document for:	discussion

Agenda Item:	15.1.5	Non-Terrestrial Networks (NTN) for NR Phase 4

# Introduction

This document proposes a scope for a RAN1 led WID on NR-NTN as part of Rel-20 5G Advanced work plan.

# Discussion

## Context

At RAN#107, RP-250812 “Summary for RAN Release 20 5G-Adv”, includes in slide 14, a specific conclusion for GNSS resilient operation

- Study Item with checkpoint to determine whether we have normative work and the scope based on results of study item phase.

- Study should consider Rel-16 as baseline for discussion.  Study should focus on NR NTN and be backward compatible with legacy UEs.  Aim to minimize physical layer procedures impact to 5GA and avoid physical layer channel/signal changes.

- Objective to be narrowed down for RAN#108

- No additional NR NTN enhancements with RAN1 impact can be considered at this point given limited 5GA TUs

Additional topic to be included in objective.

- Specify necessary enhancements to support E-UTRA TN to NR NTN handover in RRC connected mode

- TU estimate RAN2 (0.25 TU), RAN4 (0.5)  (+ RAN1 [0.5] as per slide 10)

- Deadline to complete the work (first half of release)

In line with the above, the scope of a Rel-20 RAN1 led NR-NTN activity is proposed below.

## Proposed scope for a Rel-20 RAN1 led WID on NR-NTN

### Justification

In Release 17, a work item was carried out to define solutions enabling New Radio and NG-RAN to support Non-Terrestrial Networks (NTN). Subsequently, Release 18 and 19 introduced enhancements for NR NTN.

As part of Release 20, a new work item is proposed to define further enhancements for NG-RAN based Non-Terrestrial Networks in order to achieve and maintain successful uplink time and frequency synchronization in NTN based access for NR-NTN capable UEs of all types including handheld terminals and mobile VSAT (*) devices under a potential occurrence of GNSS jamming (leading to denial of service) and/or spoofing and/or other effects including unfavourable geometry/ scintillation /space weather beyond the control of the satellite network operator (leading to incorrect location calculation/position fix at UE side and leading to an out of sync) (**). Moreover, a terminal, or a group of terminals attempting to (re-)connect over access channels using the spoofed GNSS position could also cause interference and thereby a degradation of service for all other users in the cell.

(*) It is assumed that for a stationary device (e.g., fixed VSAT, static IoT device), the UE position can be pre-configured.

(**) 5G NR-NTN specifications in Releases 17, 18 and 19 (presently in progress) assume that the UE uses its GNSS coordinates i.e., UE position - see TS 38.300 Section 16.14.2.2, together with the ephemeris of the serving satellite and additional information about the feeder link delay, to pre-compensate the time and frequency shift.

### Objectives

General assumptions

- Consider Low Earth Orbit (LEO) @ 600 km and GSO with at least 1 satellite in visibility.

- Consider NTN operating in Frequency Range 1 and in above 10 GHz frequency bands.

- Consider all types of user equipment (including Mobile VSAT on board aircrafts, ships and vehicles).

- Consider both transparent and regenerative satellites.

The objectives of the work item are the following:

Study and specify, if needed, GNSS resilient NR-NTN operation [RAN1, RAN2, RAN4]

- Consider two approaches:

  - 1) UE Position assisted access: The UE acquires its own location based on information provided by the NTN, and uses the position to access the system. The information may include but is not limited to:

    - 1.a) Reference signals (e.g. PRS) allowing the UE to compute its position without GNSS.

    - 1.b) Assistance data allowing the UE to compute its position in conditions where GNSS availability is degraded.

  - 2) Coarse UE Position access: The UE can access the network using a position with relaxed accuracy. This may include but not limited to: Enhanced close loop timing adjustments, close loop frequency corrections, Relaxed RRM requirements.

- Assess impact on initial access procedure, including potential implications if any on RAN4 RRM specifications.

- Assess impact on connected mode procedures (e.g. related to enhancement(s) to closed loops for timing / frequency corrections).

- Check end of Q2/2026, the conclusions of the study phase and trigger a normative phase based on results of the study item phase

- Notes for this objective:

  - GNSS may not be available or may be less accurate (target relaxed assumed location accuracy compared with that implied by TS 38.133 Rel-19 Section 7.1C - TBD) during a period of time or in some geographical areas

  - NR_NTN capable UE is expected to support 15 kHz, 30 kHz and 120 kHz SCS.

  - NR NTN capable UE can have either directive antenna or omni-directional antenna. The proposed solution/s shall work with both types of antennas.

  - Aim to minimize physical layer procedures impact to 5GA and avoid physical layer channel/signal changes.

  - Study should focus on NR_NTN. However, study phase outcomes may be replicated on IoT_NTN as well.

  - Aim at reusing study outcomes for 6G

### Working groups

- Leading WG : RAN1

- Contributing WG : RAN2 and RAN4

### TU estimate

RAN1 [0.5], RAN2 [0.25], RAN4 [0.5]

### Impacted existing TS/TR

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 38.211 | NR; Physical channels and modulation TBD | RAN#114 | Core part |
| TS 38.213 | NR; Physical layer procedures for control TBD | RAN#114 | Core part |
| TS 38.214 | NR; Physical layer procedures for data TBD | RAN#114 | Core part |
| TS 38.300 | NR; Overall description; Stage-2: TBD | RAN#114 | Core part |
| TS 38.304 | NR; User Equipment (UE) procedures in idle mode and in RRC Inactive state TBD | RAN#114 | Core part |
| TS 38.305 | NG Radio Access Network (NG-RAN); Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN TBD | RAN#114 | Core part |
| TS 38.306 | NR; User Equipment (UE) radio access capabilities TBD | RAN#114 | Core part |
| TS 38.321 | NR; Medium Access Control (MAC) protocol specification TBD | RAN#114 | Core part |
| TS 38.455 | NG-RAN; NR Positioning Protocol A (NRPPa) TBD | RAN#114 | Core part |
| TS 38.331 | NR; Radio Resource Control (RRC); Protocol specification TBD | RAN#114 | Core part |
| TS 38.133 | NR; Requirements for support of radio resource management TBD | RAN#114 | Core part |
| TS 38.133 | NR; Requirements for support of radio resource management TBD | RAN#116 | Perf part |

### Related Work Items and dependencies

| Other related Work/Study Items (if any) |  |  |  |
| --- | --- | --- | --- |
| Acronym | Unique ID | Title | Nature of relationship |
| 800099 | NR_NTN_solutions | Study on solutions for NR to support non-terrestrial networks (NTN) | Leverage initial studies on UL synchronization procedures |
| 860046 | NR_NTN_solutions | Solutions for NR to support non-terrestrial networks (NTN) | Leverage the basic UL synchronization procedures |
| 941006 | NR_NTN_enh | NR NTN (Non-Terrestrial Networks) enhancements |  |
| 1020097 | NR_NTN_Ph3 | Non-Terrestrial Networks (NTN) for NR Phase 3 | - |

# Conclusion and Way forward

Proposal 1: RAN to discuss at next plenary about RAN1 led NR-NTN ph4 work item as part of the Rel-20 5G Advanced work plan considering the Proposed scope in clause 2.2 of RP-250877.

END