3GPP TSG RAN#108 Meeting 	RP-250880

Prague, Czech Republic, June 09-13th, 2025

Title:	Practical NTN related deployment scenarios

Source: 	Thales, Iridium, Echostar, TNO, ESA, Sateliot, Viasat, Inmarsat, Novamint, Airbus, SES, Gatehouse Satcom, Eutelsat Group, Fraunhofer IIS, Fraunhofer HHI

Type:	discussion

Document for:	discussion

Agenda Item:	16.1 RAN led 6G Study on 6G Scenarios and Requirements [RAN led SI: FS_6G_RAN_Scen_Req]

# Introduction

This document proposes Non-Terrestrial Network (NTN)-related typical and practical deployment scenarios along with initial requirements for a 6G Radio design harmonized for both TN and NTN and ready for integrated NTN/TN scenarios since Rel-21.

# Background and general recommendations

## 5G NTN related deployment scenarios

For NTN in the context of 5G, the following reference scenarios have been considered for respectively the definition of the NR-NTN and IoT-NTN radio access ([1], [2]).

- GEO (geo-stationary orbit) based non-terrestrial access network with Earth-fixed cell coverage: provided by beam(s) continuously covering the same geographical areas all the time.

- LEO (low earth orbit) based non-terrestrial access network at 1200 km and 600 km orbits with:

  - Quasi-Earth-fixed cell coverage provided by beam(s) generated by a satellite covering one geographic area for a limited period and a different geographic area during another period.

  - Earth-moving cell coverage: provided by beam(s) whose coverage area slides over the Earth surface.

- MEO (medium earth orbit) based non-terrestrial access network generating fixed beams whose footprints move with the satellite (altitude 10000 km).

As part of 6G, it is strongly recommended to also support:

- Emerging deployment scenarios in the Very Low Earth Orbit (VLEO) at altitude of 300 km.

- GSO (Geo Synchronous Orbit) based deployment scenarios since it corresponds to end of life GEO based deployment scenarios.

- LEO and MEO based deployment scenarios with updated parameters in order to align with the increased performance requirements of NTN in the context of 6G compared to 5G.

(LEO and MEO both appertain to the non-geostationary orbit (NGSO) orbit type. NGSO refers to a type of orbit used by satellites in which the satellite is not stationary relative to the surface of the Earth.)

## NTN in 6G versus in 5G

While in the context of 5G, the support of NTN was added once the basic design principles of the New Radio (NR) access were already defined, it is strongly recommended to define and specify the basic enablers to support NTN as part of the 6G radio access definition right from the initial 6G specification in release 21 (see 6GWS-250027). This approach will allow for:

- Achieving ubiquitous connectivity and availability of services in remote and sparsely populated areas.

- Improving network resiliency, especially in situations where disasters disrupt terrestrial networks or damage undersea communication cables.

By integrating the fundamental NTN enablers in the initial stages of 6G development, users will experience several benefits, including seamless connectivity regardless of geographic location. This will empower individuals, communities, and industries in the most isolated regions, ensuring that they are not left behind in the digital revolution. Moreover, enhanced network resiliency will provide robust and reliable communication channels during emergencies, significantly reducing downtime and maintaining critical infrastructure and services. This harmonized design approach for both TN and NTN components will pave the way for a resilient, versatile, and inclusive communication network for future generations.

The intent is therefore to achieve a 6G Radio design harmonized for both TN and NTN and ready for deployment scenarios integrating NTN and TN components since Rel-21.

# Practical NTN deployment scenarios for 6G

## Orbits

Below are some considerations for practical NTN scenarios for 6G:

- VLEO based scenario at minimum altitude of 300 km: Satellites operating at this altitude benefit from reduced latency due to their proximity to the surface of Earth, which can enhance communication and observation capabilities. At this altitude, the Round Trip Delay becomes closer to the one of terrestrial extended cells. This reduced latency would be particularly beneficial in supporting Time Division Duplexing (TDD) and Half-Duplex Frequency Division Duplexing (HD-FDD) modes, enabling more efficient and reliable real-time data transmission. Further, this closer proximity to Earth can offer more accurate ground metrics, and potentially lower cost for certain types of satellite operations. However, deployment at a VLEO orbit would require a very high number of satellites to generate overlapping foot print and hence service continuity in the targeted coverage. This is now made possible with the ever decreased launch cost. Typically, a maximum satellite beam footprint diameter of [250] km or lower should be supported.

- Medium and High LEO (M-LEO and H-LEO, respectively) based scenarios (respectively 600 and 1200 km altitude): While these scenarios are already supported in the context of 5G, the maximum beam footprint can be reduced in the context of 6G since the targeted service rate is likely higher than NB-IoT-NTN capability and hence require satellites able to achieve high antenna gain and hence generate smaller beams. Typically, a maximum satellite beam footprint diameter of [250] km or lower should be supported.

- MEO based scenarios (10000 km altitude): In line with the above, typically, a maximum satellite beam footprint diameter of 250 km or lower should be supported.

- GSO based scenarios: GSO differs from GEO by a freedom to orbit around the 0° latitude. This allows to extend the satellite lifetime by optimizing the necessary fuel for station keeping.

The table below provides the list of reference practical scenarios (single orbit) to be supported by the 6G radio access with the related parameters.

Table 1: NTN reference orbit scenarios for 6G

| Scenarios | Very LEO based NTN | Medium LEO based NTN | High LEO based NTN | MEO based NTN | GSO based NTN |
| --- | --- | --- | --- | --- | --- |
| Orbit type | circular around the earth | circular around the earth | circular around the earth | circular around the earth | station keeping around a nominally fixed position in terms of elevation/azimuth with respect to a given earth point (NOTE 6) |
| Altitude | 300 km (NOTE 1) | 600 km | 1200 km | 8000 km | 35,786 km |
| Earth-fixed beams | Yes (Satellite generated steerable beams, NOTE 2) |  |  |  |  |
| Max beam footprint size (edge to edge) regardless of the elevation angle (NOTE 3) | [<250] km | [<250] km | [<250] km | [<250] km | [<250] km |
| Min Elevation angle for both sat-gateway and User Equipment | 10° for service link and 10° for feeder link |  |  |  |  |
| Max distance between satellite and User Equipment at min elevation angle | 1160 km | 1 932 km | 3 131 km | TBD km | 40 581 km |
| Max Round Trip Delay (propagation delay only NOTE 4) | 15.48 ms | 25.77 ms | 41.77 ms | 180 ms | 541.46 ms |
| Max Doppler shift (earth fixed user equipment) (NOTE 5) | 25.35ppm | 24.85ppm | 23.85ppm | TBD ppm | 0.93ppm |
| Max Doppler shift variation (earth fixed user equipment) (NOTE 4) | TBD ppm/s | 0.27 ppm/s | 0.13 ppm/s | TBD ppm/s | 0.000 045 ppm/s |
| NOTE 1: VLEO is characterized by altitudes ranging from approximately 180 to 600 kilometers above Earth. However, the lower the altitude, the higher the number of satellites is required to ensure service continuity and the higher the atmospheric drag which would need constant propulsion and re-boosting and hence reduced lifetime. NOTE 2: Each satellite has the capability to steer beams towards fixed points on earth using beamforming techniques. This is applicable for a period of time corresponding to the visibility time of the satellite. NOTE 3: This beam size refers to the Nadir pointing of the satellite. NOTE 4: Assuming transparent payload with service and feeder links and return NOTE 5: Max Doppler shift and Max Doppler shift variation in the absence of any device pre-compensation of satellite Doppler shift on the service link. NOTE 6: In case of GSO, the inclination may be non-zero. This means the satellite’s ground track may trace a figure-eight pattern (analemma) as seen from Earth. |  |  |  |  |  |

Proposal 1: The 6G radio interface/access shall be defined to support all the practical NTN deployment scenarios described mainly in terms of orbits,  and related service link characteristics in the table above.

## Constellation

Typical sizing of constellation according to the orbit:

- GSO constellation may be based on one to three satellites depending on the targeted coverage.

- NGSO constellations providing real time services are sized to ensure at least one satellite in visibility of all UEs (above the min elevation angle) in the targeted coverage

- NGSO constellations providing non real time services such as messaging shall be made of at least one satellite. A given UE can connect to one of the serving satellite(s) from time to time depending on the satellite(s) revisit time.

Additionally, the 6G radio access should also support multi orbit space segment combining at least a GSO with an NGSO based scenario, where NGSO may cover VLEO, medium/high LEO and/or MEO.

## Payloads and feeder links

The following payload types along with 6G functional mapping shall be supported:

Table 2: 6G NTN payload types with respect to 6G system architecture

| Payload type | 3GPP function on board | Feeder link type (NOTE 1) | Other capabilities |
| --- | --- | --- | --- |
| Transparent | Equivalent to RF repeater | 6G radio interface | - |
| Semi transparent | Radio Unit (RU) | Fronthauling of RU-Base station interface | - |
| Regenerative embarking a full base station (BS) | BS | Backhauling of NG interface | - Support of Store and forward - And/or Support of Inter Satellite Links |
| Regenerative embarking part of a base station (Functional BS split) | Radio Unit and a Distributed unit | Base station internal interface (F1 like) | Inter Satellite transport links can be used |
| NOTE1: It refers to the radio link between satellite and a gateway. |  |  |  |

Proposal 2: The 6G radio interface/access shall be defined to support all the NTN payload types in the table above

## Inter-Satellite Link support

Regenerative satellite payloads enable routable front or backhaul traffic which can exploit Intersatellite Links (ISLs) within the underlying transport network.. ISLs can be deployed between two satellites of the same constellation (intra-/inter- orbital plane) or between two satellites of different constellations/orbits (e.g., between LEO and GSO satellites). ISL will allow to provide service in areas where there are no NTN-gateways and provide coverage of remote areas like the oceans. Inter-satellite links may operate in RF frequency or optical bands. The route taken between the GW and the satellite may thus change, adding or removing inter-satellite links to reach the GW.

Note that there are different types of inter satellite links:

- Between satellite of the same orbit which can further be distinguished between

  - Intra orbital plane ISL: between (adjacent) satellites of the same orbital plane

  - Inter orbital plane ISL: between satellites of (adjacent) orbital planes

- Between satellites of different orbits (e.g., GSO and NGSO, or LEO and MEO)

## Targeted User Equipment and usage conditions

6G access network shall be able to serve a wide range of devices (User Equipment - UE). We shall distinguish the following ones in two main categories:

- Smartphone and IoT devices with omnidirectional antenna which should be addressed with satellite networks operating in satellite service allocated frequency bands up to ~7 GHz.

- Vehicle or building mounted devices with directional antenna operating in all satellite service allocated frequency bands. Vehicle encompass cars, buses, railway trains, vessels and aeronautic platforms.

The 6G access network shall be defined to be able to serve devices with the following worst-case performances:

Table 3: NTN capable 6G UE types

| UE types | Min antenna gain | Min Transmit Power | Max Noise figure | UE Ground speed | Coverage |
| --- | --- | --- | --- | --- | --- |
| IoT and smartphones | -5 dB | 20 dBm | 7 dB | 300 km/h (on board high speed trains) | Outdoor, Light indoor (NOTE1) |
| Vehicle and building mounted | Equivalent to the one of 20 cm aperture antenna (FR2-NTN), 0.3 dBic (FR1-NTN) [3], [4] | Equivalent to 1 Watt (FR2-NTN) | [4] dB (FR2-NTN), 7 dB (FR1-NTN) | 1500 km/h (aircraft) | Outdoor only |
| NOTE1: Light indoor requires Maximum Coupling Loss comparable to 3GPP defined NB-IoT. It may lead to service rate degradation. |  |  |  |  |  |

Proposal 3: The 6G radio interface/access shall be defined to support all the NTN capable UE types in the table above.

## Duplex mode

6G should support both duplexing methods — FDD (Frequency Division Duplexing), and TDD (Time Division Duplexing) as well as Half Duplex Frequency Division Duplexing (HD-FDD) mode.

The target Payload types are:

- Full-duplex FDD Satellite Payload:

  - Full duplex networks should support a mix of full and half duplex UEs

- Half-duplex FDD Satellite Payload: For both base station and terminal, a duplexer is not needed.

The targeted UE types are:

•	Smartphone and IoT devices

•	Fixed and Mobile VSAT

•	Full and Half Duplex FDD Mode

•	GNSS receiver capability is not required at UE side for physical layer operation.

The UE will have to support the frequency band and duplex mode of the satellite network.

Table 4: Duplex mode supported by NTN capable 6G UE

| NTN capable UE type | Paired frequency band | Unpaired frequency band |
| --- | --- | --- |
| IoT and smartphones | FDD or HD-FDD | TDD |
| Vehicle and building mounted | FDD or HD-FDD | TDD |

Proposal 4: The 6G radio interface/access shall be defined to support all the duplex modes at UE and network level in the table above.

## Service links

In all cases, the service link (which refers to the radio link between satellite and User Equipment) will provide the 3GPP defined NTN capable 6G radio interface.

It operates in satellite service allocated frequency bands which can be paired (Downlink and Uplink in separate bands) or unpaired (Downlink and Uplink in the same band).

While paired bands can apply to all orbits, unpaired band is restricted to the Low Earth Orbits (VLEO, MLEO and HLEO).

# Practical NTN – TN Interworking scenarios

Interworking of NTN and TN is needed for ubiquitous coverage, service continuity and/or resiliency purposes. This requires for 6G to support mobility and simultaneous multi connectivity between NTN and TN components.

## NTN-TN Mobility

The purpose is to ensure a global service continuity in both idle and connected modes. NTN is expected to provide coverage in overlapping areas with TN but also in areas beyond TN coverage.

While mobility from TN to NTN is mostly triggered by the unavailability of the TN service, the mobility from NTN to TN may be determined by other criteria.

In both cases, the UE shall be able to camp on a TN cell while being able to monitor NTN cells and vice versa.

The related requirements on the UE differ on the type of targeted devices:

Table 5: NTN capable 6G UE possible architecture characteristics

| NTN capable UE types | Modem | Antenna |
| --- | --- | --- |
| IoT and smartphones | Same for both NTN and TN | Same for both NTN and TN |
| Vehicle mounted | Same or distinct respectively both NTN and TN | Two for respectively NTN and TN (FR2, FR1 broadband); Same for both NTN and TN for narrowband IoT services in FR1. |
| Building mounted | Same or distinct respectively both NTN and TN | Two for respectively NTN and TN |

Proposal 5: The 6G radio interface/access shall be defined to support TN/NTN mobility considering the different UE architecture described in the table above

## NTN-TN Dual connectivity

Dual Connectivity or Multi-connectivity between NTN and TN can be used for Coverage and Quality of Service enhancement. Also, it can ensure seamless mobility, exploiting Heterogeneous Network (HetNet) environment, connection to a large NTN macro cell for mobility anchor and connection to a local TN small cell for data transfer in optimal radio conditions. Thereby, multi-connectivity can provide a very fluent (no added data latency during handovers) and robust mobility (connection failures minimized). NTN-TN Multi-connectivity also provides increased throughput especially in case of VLEO and LEO; using multiple radio resources can obviously improve data rates experienced by users.

Observation 1: NTN-TN Dual connectivity may contribute to provide seamless mobility by eliminating handover interruption delays and errors, and optimizes capacity, coverage and mobility for devices connected in a heterogeneous NTN-TN network

Given that the NTN may feature a different bandwidth compared to the one offered by TN, only part of the traffic (most critical one) may be duplicated via the NTN connectivity.

Hence, the traffic is partially duplicated between the two NTN and TN connectivity.

Proposal 6: The 6G radio interface/access shall be defined to support TN/NTN multi connectivity with simultaneous traffic flow in both legs.

# Positioning Navigation and Timing

The provision of robust, high-accuracy positioning is driven by regulatory requirements, demand for ultra-precise location services, need for resilient alternatives to GNSS, and the expansion of service areas. NTN is a key enabler of ubiquity, resilience, and wide area coverage representing a significant opportunity to enhance not only global connectivity but also positioning services especially in remote, underserved, and maritime areas where traditional terrestrial networks are not feasible or cost-effective. Complementing TN with NTN is crucial not only for connectivity but also for positioning services to maximize synergies between the two.

Proposal 7: The 6G radio interface/access shall be defined to provide high-accuracy and resilient positioning without GNSS service.

# Conclusion and Way forward

From the above, the followings are proposed:

Proposal 1: The 6G radio interface/access shall be defined to support all the practical NTN deployment scenarios described in terms of orbits, and related service link characteristics in the table within clause 3.1 of RP-250880)

Proposal 2: The 6G radio interface/access shall be defined to support all the NTN payload types in the table within clause 3.3 of RP-250880)

Proposal 3: The 6G radio interface/access shall be defined to support all the NTN capable UE types in the table within clause 3.5 of RP-250880)

Proposal 4: The 6G radio interface/access shall be defined to support all the duplex mode at UE and network level in the table within clause 3.5 of RP-250880)

Proposal 5: The 6G radio interface/access shall be defined to support TN/NTN mobility considering the different UE architecture described in the table within clause 4.1 of RP-250880)

Observation 1: NTN-TN Dual connectivity may contribute to provide seamless mobility by eliminating handover interruption delays and errors, and optimizes capacity, coverage and mobility for devices connected in a heterogeneous NTN-TN network

Proposal 6: The 6G radio interface/access shall be defined to support TN/NTN multi connectivity with simultaneous traffic flow in both legs.

Proposal 7: The 6G radio interface/access shall be defined to provide high-accuracy and resilient positioning without GNSS service.

# References

[1]	3GPP TR 38.821 “Solutions for NR to support Non-Terrestrial Networks (NTN)”

[2]	3GPP TR 36.763 “Study on Narrow-Band Internet of Things (NB-IoT) / enhanced Machine Type Communication (eMTC) support for Non-Terrestrial Networks (NTN)”

[3]	RP-232733 “LS on Automotive Requirements for Non-Terrestrial Network (NTN) Enhancements in Rel-19”, Contact Person: Khaled Hassan/Bosch, Maxime Flament/5GAA

[4]	https://5gaa.org/maximising-the-benefit-of-future-satellite-communications-for-automotive/

END