3GPP TSG RAN Meeting #108	RP-250887
Prague, Czech Republic, June 9-13, 2025

Agenda item:	16.2

Source:	IITH, WiSig Networks

Title:	Large Scale MIMO considerations for 6G

Document for:	Discussion

# 1	Introduction and Discussion

Use-case: Single-radio, single-baseband “702.x” type interface

- Architecture – One radio unit (RU) connected to a single baseband unit (BBU) delivers full 360° azimuth-and-elevation coverage, enabling true multi-sector service with a single protocol stack.

702.A type (passive 4 TR / 8 TR) deployments

- A single BBU can feed multiple fixed antennas oriented toward different sectors.

- The 5G NR codebook, however, does not convey sector-dependent amplitude variations in channel-state information (CSI) feedback.

- Although SRS-based sounding could capture this information, most 4 TR/8 TR 702.A radios lack TDD-reciprocity calibration for SRS.

- Consequently, zero-forcing (ZF) precoding across sectors using explicit CSI is not supported today.

- Gap: the specifications need an explicit CSI feedback mechanism—e.g., a time-domain CSI report—that includes both phase and amplitude, making 702.A multi-sector deployments more efficient.

702.B type (massive-MIMO) deployments

- Typical implementations use two or three sectorized antenna panels to achieve 360° coverage, which creates inter-sector interference.

- This can be avoided by designing a single RU with a continuous 360° antenna array whose ports are fully calibrated for TDD reciprocity, allowing beamforming in any direction.

- While retrofitting existing sites may be impractical, new (green-field) installations can adopt this integrated, full-circle antenna architecture.

# 2	Proposals

Proposal 1 – Explicit CSI Feedback

Add a UE reporting mode that provides full, time-domain explicit CSI—including both amplitude and phase—for every antenna port/sector.

Proposal 2 – 360° Single-RU Massive MIMO

Initiate a study item on a single-RU, single-baseband massive-MIMO architecture capable of calibrated 360-degree coverage, eliminating inter-sector interference.