3GPP TSG-RAN WG Meeting #107	                                          RP-250897

Prague, Czech Republic, 9 – 12, Jun, 2025

Source: 	Xiaomi

Title:  	Views on R20 AI/ML for Mobility

Agenda Item:	15.2.1

Document for:	Discussion and Decision

Introduction

AI/ML for mobility is studied by RAN2 in R19. In this contribution, we discuss the scope of R20 AI/ML for mobility.

Discussion

The R19 SI includes following use cases,

- AI/ML based RRM measurement and event prediction, - Cell-level measurement prediction including intra and inter-frequency (UE sided and NW sided model) [RAN2] - Inter-cell Beam-level measurement prediction for L3 Mobility (UE sided and NW sided model) [RAN2] - HO failure/RLF prediction (UE sided model) [RAN2] - Measurement events prediction (UE sided model) [RAN2]

Regarding RRM prediction, RAN2 has done lots of simulation results on prediction accuracy. The simulation results showed the prediction can be close to the ground truth in some cases. The RSRP difference can be very small. Based on the simulation results, RAN2 has captured that AI can provide gain compared to Non-AI from different dimensions. Therefore, RRM prediction should be included in the R20 WI. Furthermore, the RRM prediction is categorized into following cases,

| Case number | Evaluation scenario combination | target study goal |
| --- | --- | --- |
| 1 | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal |
| 2 | FR1 to FR1 intra-frequency temporal domain case B | 1st goal |
| 3 | FR1 to FR1 inter-frequency (frequency domain) | 1st goal |
| 4 | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal |
| 5 | FR2 to FR2 intra-frequency temporal domain case B | 1st goal |
| 6 | FR2 to FR2 intra-frequency spatial domain | 1st goal |

RAN2 simulation focused on temporal and frequency domain prediction i.e. case 2, 3, 4. Companies didn’t have enough evaluation on the prediction performance of spatial domain prediction and L3 beam prediction, due to lack of simulation results.

In RAN2#129, RAN2 has agreed that the temporal and frequency prediction will be considered as higher priority as following.

- For now, temporal and frequency domain prediction will be considered with higher priority for detailed study (if needed).

In RAN2#130, Although RAN2 agreed to study the spec impact of spatial domain prediction, the scenario of intra-frequency spatial domain prediction (in cell dimension) is not considered by the UE-side model in 5G as following,

- The scenario of intra-frequency spatial domain prediction (in cell dimension) is not considered by the UE-side model in 5G. RAN2 assumes that a network-sided model requires no specification impacts. Intra-frequency spatial domain prediction (in cell dimension) involves measuring one or more cells as input to the model to derive L3 filtered cell-level measurements for the same time instance of another cell(s). - The specification impact of intra-frequency spatial domain prediction in beam dimension will be studied by RAN2 for both UE-side and network-side model.

Because the feasibility of inter-cell spatial domain prediction is not concluded. Therefore, the RRM prediction on temporal and frequency domain should be prioritized over spatial domain and L3 beam prediction. RRM prediction on spatial domain and L3 beam prediction shall only limited to intra-cell prediction.

Proposal 1: RRM prediction on temporal and frequency domain should be prioritized over spatial domain and L3 beam prediction in R20 WI.

Proposal 2: RRM prediction on spatial domain and L3 beam prediction is limited to intra-cell prediction.

Event prediction can be done in direct or indirect way. RAN2 has agreed to prioritize indirect prediction and the related simulation assumptions. Based on the simulation results, RAN2 concluded that the prediction result is accurate under certain scenarios. In addition, the event prediction can improve the mobility performance or save UE power by reducing the measurement.

Agreements on Measurement event case A - Most of the results show that the F1 score for prediction of measurement event is very good. - For handover decision option 3 and 2 AI/ML performs better than baseline (legacy) in terms of HO failure rate. Agreements on Measurement event case B - Measurement event case B can have very good F1 score depending on filtering approach/PW - F1 score is good for the PW window for case B (i.e. low PW) - F1 score decreases with an increasing MRRT value - We will focus on finding cases that bring benefits to the system rather than trying to compare - With indirect measurement event prediction based on temporal domain Case B (MRRT=50%), the AI-based HO has a minor/no system-level performance (i.e., HOF rate and HO number) decrease compared with the legacy HO mechanism.

Therefore, event prediction should be included in R20 WI.

Proposal 3: Event prediction should be included in the R20 WI.

Three sub use cases are supported for RRM prediction according to TR [1] as following,

Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated

Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s)

Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s)

For L3 beam level measurement prediction, following three sub-cases are supported according to TR [1],

Sub-use case 4: L1 filtered beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 beam-level measurement result is generated

Sub-use case 5: L3 beam-level measurement result(s) is predicted based on actual L3 beam-level measurement result(s)

Sub-use case 6: L3 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s)

The inference input of sub use case 1, 3, 4 and 6 are L1 beam measurement results. Note the L1 beam for RRM is configured by CSI-RS-ResourceConfigMobility, which is different from the beam for beam management. Current CSI report framework doesn’t support reporting L1 beam measurement result for RRM purpose. Furthermore, RRM prediction may require neighbor cell’s L1 beam measurement result as model input. Current CSI report framework only support reporting serving cell’s L1 beam measurement result.

It’s up to UE how to measure the RRM L1 beam within the required latency, i.e. by what periodicity filtering. NW side model may require L1 beam measurement with fixed periodicity as input. It’s new requirement for UE measurement, which is impossible to apply from UE point of view. Furthermore, it’s up to UE how to perform the L1 filtering. UEs may apply different L1 filtering method. NW is not aware which L1 filtering is applied by UE. The inference accuracy may be poor without such information.

As discussed above, new report signalling is required to report L1 beam measurement result. New requirement on UE measurement is needed to align with the model input interval. UE may also need to report how L1 filtering is done. The spec impact is huge to support L1 beam as model input, i.e. sub-case 1 or 3, for NW sided model.

Observation 1: Huge spec impact is foreseen to support L1 beam as model input for NW sided model, i.e. sub case 1, 3, 4 and 6, including

- new signalling to report L1 beam measurement result of serving and neighbour cell for RRM, as current L1 beam for RRM is configured by CSI-RS-ResourceConfigMobility, rather than using CSI framework

- new requirement on UE measurement with potential RAN4 impact

- new signalling to report how L1 filtering is done, which is now completely up to UE implementation

Therefore, we propose to only support sub-case 2, i.e. L3 cell level measurement results as model input, is supported for NW sided model in RRM prediction.

Proposal 4: L1 beam measurement result as model input is not supported for NW sided model in RRM prediction and L3 beam prediction.

As agreed in RAN2, RLF predication has been deprioritized in Rel-19.

- Agreements on priority

- RLF prediction will not be studied in Rel-19

However, the event prediction discussed in SI phase is based on RSRP which may not reflect the situation of HOF and RLF very well since the HOF and RLF evaluation should be performed based on SINR. The RSRP based event prediction may not be able to avoid potential HOF and RLF. To our understanding, HOF and RLF is a key performance KPI in mobility, which would result in bad network performance, worse user experience. HOF and RLF prediction are useful to reduce the failure and interruption time, which can be reused in 6G. RAN2 should continue the study on SINR based failure prediction.

Proposal 5: Further study on SINR based failure prediction can be considered in R20.

The work is mainly on RAN2. RAN4 should also be involved to specify the necessary RRM/test requirement. RAN3 may be involved to introduce inter-node signalling if needed.

Conclusions

According to the discussion above, we have the following proposals:

Proposal 1: RRM prediction on temporal and frequency domain should be prioritized over spatial domain and L3 beam prediction in R20 WI.

Proposal 2: RRM prediction on spatial domain and L3 beam prediction is limited to intra-cell prediction.

Proposal 3: Event prediction should be included in the R20 WI.

Observation 1: Huge spec impact is foreseen to support L1 beam as model input for NW sided model, i.e. sub case 1, 3, 4 and 6, including

- new signalling to report L1 beam measurement result of serving and neighbour cell for RRM, as current L1 beam for RRM is configured by CSI-RS-ResourceConfigMobility, rather than using CSI framework

- new requirement on UE measurement with potential RAN4 impact

- new signalling to report how L1 filtering is done, which is now completely up to UE implementation

Proposal 4: L1 beam measurement result as model input is not supported for NW sided model in RRM prediction and L3 beam prediction.

Proposal 5: Further study on SINR based failure prediction should be considered in R20.

Reference

[1] TR 38.744