3GPP TSG-RAN Meeting #108	RP-250904

# Prague, Czech Republic, 9th – 13th June 2025

Agenda item:			15.4.2

Source:	Nokia

Title:	BS RF topics for 5G-Advanced Rel-20

Document for:	Discussion

# Introduction

During RAN#107, initial discussion took place on RAN4 led items for Rel-20 [1]. This document addresses Nokia’s views on NR BS RF topics for Rel-20.

# Discussion

During RAN#107, the following NR BS RF objectives were proposed and discussed for Rel-20:

- Enhancement of co-location and co-existence requirements

- Additional hybrid requirement sets for BS type 1-H/large antenna array

- OTA requirements enhancement

- NB-IoT to AAS BS specifications

- LP-WUS in TN MSR spec

- OBUE requirements structure simplification

Below are Nokia’s views on the objectives above; it should be noted LP-WUS objective has been removed from the conclusion of the summary and is not considered below.

- Enhancement of co-location and co-existence requirements

  - This is a high priority topic. While there is ongoing work in Rel-19, there is expectation it will further continue in Rel-20. The Rel-19 WI is studying Co-Location Test Antenna (CLTA) alternatives and BS-to-BS isolation values, which can be used as a background for the continuation in Rel-20. The OTA co-location requirement was designed for BS up to 3 GHz and needs to be re-evaluated for higher frequencies that will most likely be AAS with large antenna panels. Co-location test antennas are not available off-the-shelf for higher frequencies, which makes testing challenging, as expensive custom-made co-location test antennas would be needed. This could cause a situation where testing credibility might be compromised. Even if test antennas were available, multiple co-location test antennas would be needed to cover all the frequency ranges for co-location requirements, which makes testing not only impractical but also very expensive. A new solution to cover co-location testing for higher frequencies and large antenna panels must be found.

  - Possible objectives to be considered:

    - Solution on how to test co-location for higher frequencies and larger antenna panels

    - Simplify testing to reduce the need for multiple test antennas and power amplifiers

    - Simplify the test requirements with reasonable expected BS-to-BS isolation for higher frequencies.

- OTA requirements enhancement

  - This is a high priority topic. At least in-band and out-of-band blocking levels (and step sizes if not concluded in Rel-19 WI) should be re-evaluated, taking into account technical background assumptions (UE power, BS antennas, etc.) as well as test time and complexity.

- Additional hybrid requirement sets for BS type 1-H/large antenna array.

  - The justification for this is not clear since there is a possibility already to perform full OTA testing under 1-O. The motivation for creating another BS type (e.g. 1-H enhanced) should be clarified first, and if only selected requirements are proposed to be tested OTA, clear criteria should be identified for the selection of these requirements.

- NB-IoT AAS BS specifications

  - While NB-IoT in-band operation (the only mode supported in NR) can be further considered, it is not clear if there is a market need for guard-band and stand-alone NB-IoT AAS BS. Operators’ input should be sought.

- OBUE requirements structure simplification

  - Further simplifications can always be considered but the proposed objective is not the highest priority topic and can be done under maintenance if seen necessary.

# Conclusion

This document addresses Nokia’s views on NR BS RF topics for Rel-20.

We identify the following two top priority items for Rel-20 NR BS RF enhancement:

- Enhancement of co-location and co-existence requirements, and

- OTA requirements enhancement for in-band and out-of-band blocking levels.

NB-IoT AAS BS specifications for in-band operation only could be considered if there is strong operator demand.

It should be noted there will be very limited time allocation for BS RF 5G specific topics due to 6G Study Item as well as other RAN WG led items and RAN4 led items impacting RF, RRM and demod (including spectrum Work Items).

# Reference

[1] RP-250817; Moderator’s summary for RAN4 led REL-20 topics; RAN4 Chair; RAN#107