3GPP TSG RAN Meeting #108		RP-250906
Prague, Czech Republic, 9th – 13th June, 2025

Agenda item:	15.4.3

Source:	Nokia

Title:	Demod topics for 5G-Advanced Rel-20

Document for:	Discussion

# 1	Introduction

The Rel-19 study item FS_NR_demod_SCM has been progressing over the last RAN4 meetings and has produced initial results in RAN4#115. Notably the down selection to a single candidate and good initial alignment results for the CDL based spatial channel model (SCM) [1], is providing a sound basis to introduce a limited number of optional SCM based requirements for Rel-20.

There is now good alignment across a significant cluster of contributors, when looking at the summary sub-set of alignment results for NR SCM [2] in a scatter plot (and full curves in [3]):

Figure 1: Scatter plot of delivered PDSCH SNR operating point results of 4-layer cases for the down-selected 38.753 CDL channel model candidate (previously known as “truncated and quantized CDL - A4”).
1	PDSCH 4x4:4 rnd 30% TPUT, 2	PDSCH 4x4:4 rnd 70% TPUT;
3	PMI 8x4:4 T1f 70% TPUT, 4	PMI 8x4:4 T1f 90% TPUT.
Note: “rnd” = randomly chosen TypeI codebook precoder;  “T1f” = following TypeI codebook PMI reported by DUT.

The Rel-19 study has agreed to remove FR2 from the scope, and to include MU-MIMO aspects only for information by interested contributors in the appendix of the report TR 38.753.

In this paper we discuss the scope of the continuation of the SCM work in Rel-20.

# 2	Discussion

Following the demonstrated alignment potential and TE implementation feasibility of the CDL based SCM candidate from FS_NR_demod_SCM, as well as the demonstrated need for SCMs in two-codeword PDSCH, eTypeII PMI, and MU advanced receiver PDSCH use cases, we see a need for a first limited set of optional normative requirements for SU-MIMO in Rel-20, based on the Rel-19 CDL aligned SCM. 	
The study is expected to achieve alignment for two codewords.

Observation: The CDL based SCM candidate from FS_NR_demod_SCM has been demonstrated to be alignable among many contributors and was judged implementable from a test equipment point of view. 
It is undisputed that this covers typical spatial channel effects expected in deployment and is aligned with RAN1 assumptions for feature development.

The Rel-19 study has needed to down-scope the MU-MIMO aspects, to be only for information. However, this is one of the use cases observed to be ill covered in legacy channel models, especially in combination with advanced receivers and eTypeII codebooks.	
As such it is necessary to continue the work on MU-MIMO and advanced receivers, as well as “eTypeII follow PMI” alignment.

Observation: MU-MIMO, advanced receiver, and some eTypeII use cases have been down-scoped in the Rel-19 study.

Given the expected constraints on 5GA TUs in RAN4, the content of the 5GA Demod WI should be kept to a minimum and topics should be foundational for 6G work in the future. The SCM work is a top priority for many network operators, and having SCM requirements is foundational to deliver confirmed feasibility for 6G.	
Priority should be given to the SCM topic; hence, no other RAN4-led demod-focused WIs or topic should be covered in Rel-20.

Observation: The SCM topic is foundational for 6G and TUs are expected to be heavily limited for 5GA.

It is our understanding that the introduction of SCM based requirements does not necessitate the development of new receivers; rather it provides assurance and lower bounds for the performance of existing (and future) receivers in typical MIMO environments that were blind spots in legacy channel models.	
Making requirements optional in 5GA can be a way forward to make it clear that the requirements should not be seen as implicitly requiring algorithmic updates at this point.

Observation: The introduction of SCM based requirements does not necessitate the development of new receivers; rather it provides assurance and lower bounds for the performance of existing (and future) receivers in typical MIMO environments that were blind spots in legacy channel models. 	Hence the requirements can be optional in 5GA.

There is precedent in RAN4 demod with “optional UE requirements”, aka “UE declarations”, aka “UE capabilities without signalling” that are only known to the test equipment but not to the network in deployment, e.g. the advanced receiver and simplified 8Rx receiver features defined in TS 38.306. A similar approach can be used to make optional requirements for “UEs declaring to meet the SCM-based requirements”.

# 3	Conclusion

Following the above discussion and background from FS_NR_demod_SCM, we propose that Rel-20 demod work should have the following scope:

- Introduce a first limited set of optional normative requirements for SU-MIMO, based on the TR 38.753 CDL based SCM.

- Include a study objective for continuing work on MU-MIMO and advanced receivers.

Where SCM is relevant for other use cases, the work should be done in the respective WIs.

We do not foresee there being sufficient time available in Rel-20 for any other 5G-A demod items.

# References

- [1] R4-2508776, Way Forward for [115][321] NR_SCM, Moderator (Nokia), RAN4#115.

- [2] R4-2508621, Simulation summary for alignment for NR SCM, Apple, Ericsson, RAN4#115.

- [3] R4-2507823, Results collection table for NR SCM, BT plc, RAN4#115.

The only remaining CDL-based SCM candidate is agreed to be 38.901 CDLC (UMa delay spread) link-level tables based, with defined truncation procedure to 12 clusters and 2ns delay quantization. The randomness reduction framework from TR 38.827 is also included.