3GPP TSG RAN Meeting #108		RP-250907
Prague, Czech Republic, 9th – 13th June, 2025

Agenda item:	15.4.3

Source:	Nokia

Title:	RedCap evolution in 5G-Advanced Rel-20

Document for:	Discussion

# 1	Introduction

Already during Rel-19 a TEI related to introduction of 3MHz channel bandwidth for (e)RedCap devices was proposed and discussed. However, due to scope limitation, and considering it would only be possible to cover 2Rx RedCap devices, it was agreed not to pursue the topic within Rel-19 but instead propose a Rel-20 WI addressing a broader solution covering both 2Rx and 1Rx RedCap devices.

In this paper we discuss the complete set of possible objectives for a (e)RedCap WI in Rel-20.

# 2	Discussion

### 2.1.1	(e)RedCap devices operating in bands supporting 3 MHz (15 PRB) channel bandwidth

There are requests from several companies, with large support from additional companies, to enable (e)RedCap devices to operate in bands supporting 3 MHz (15 PRB) channel bandwidth. One reason for this support is due to a need to migrate to NR from earlier solutions.

Observation 1: There is large support from companies to enable (e)RedCap devices to operate in bands supporting 3 MHz (15 PRB) channel bandwidth.

During Rel-19, CRs were provided enabling 2Rx RedCap devices operating in bands supporting 3 MHz (15 PRB) channel bandwidth. However, support of 1Rx RedCap devices was seen as an essential part of such solution to ensure enhanced UE power savings. Based on this the Rel-19 solution and CRs were not agreed. For Rel-20, RAN4 could work on such a WI.

Proposal 1: Create a Rel-20 WI enabling (e)RedCap devices to operate in bands supporting 3 MHz (15 PRB) channel bandwidth.

Under discussion is now a Rel-20 WI which would enable both 1Rx and 2Rx (e)RedCap devices to operate in bands supporting 3 MHz (15 PRB) channel bandwidth.

Proposal 2: Both 1Rx and 2Rx (e)RedCap devices should be supported in the WI.

Our understanding of such work is that the devices under consideration will re-use the existing (e)RedCap capabilities. Hence, a change would only be needed for some (e)RedCap devices which would additionally indicate support of operating in bands supporting less than 5 MHz FR1 CBW. Such an approach would limit the scope and workload.

Proposal 3: Re-use the existing (e)RedCap capabilities.

However, if other enhancements are seen necessary this will increase the workload of the WI. This would then need to be accounted when allocating TUs, especially if such additional enhancements would involve other WGs.

The bands to be considered in the Rel-20 WI should be limited to the same bands defined in the Rel-18 WI.

Proposal 4: Rel-20 WI should be limited to including the same bands as were defined in the Rel-18 <5MHz WI.

RAN4 has discussed the WI earlier with following proposed WI objectives:

- Introduce support of 3MHz channel bandwidth (15 PRB) for eRedCap devices in FR1 with 15KHz SCS [for both TN and NTN]

- Example bands: n106, n85, n26, n28

- Introduce 3 MHz channel bandwidth support for n106 for 1Rx and 2Rx (e)RedCap devices.

- Utilize existing (e)RedCap capabilities defined in Rel-17, Rel-18 and Rel-19 unless otherwise specified.

- Define the necessary RF and RRM Core and Performance requirements for UE and BS.

We believe this would be a good starting point for the possible WI Objectives. Based on the discussion in this paper, it may be necessary to clarify or make some limitations to the WI scope. We would for example prefer to limit the scope within RAN4 and add following line:

- Strive at no impact on RAN1 and RAN2 except possible capability signalling.

At least we see it important that the work must be focused on enabling (e)RedCap devices to operate in bands supporting 3 MHz (15 PRB) channel bandwidth using Rel-18 solution without further enhancements in Rel-20. Reasoning is to limit the scope and workload ensuring WI approval.

### 2.1.2	RLM/BFD relaxation for 1Rx and 2Rx RedCap UE

Considering the fact that UE power saving for RedCap devices is of importance, we believe that enabling RLM and BFD relaxation for RedCap devices is beneficial and important.

If Rel-20 will include this feature, RAN4 must consider both 1Rx and 2Rx RedCap devices. Additionally, a solution for RedCap devices must use the RLM and BFD relaxation which was defined for non-RedCap devices in Rel-17. Such assumption will ensure that the Rel-20 work load related to such WI can be limited. We assume that both 2Rx and 1Rx can be based directly on Rel-17 work while some simulations may be needed for the 1Rx Redcap device.

Due to the synergies related to the two RedCap proposals in RAN4, we suggest defining one single WI including objectives related to RedCap, covering both proposals:

- RLM/BFD relaxation for RedCap UE

- 3MHz channel bandwidth for (e)Redcap devices.

Proposal 5: Create one single RedCap WI including objectives for RLM/BFD relaxation and 3MHz channel bandwidth for (e)Redcap devices.

# 4	Conclusion

In this paper we take a look at the proposals related to (e)Redcap work in Rel-20. Based on the discussion we propose:

Proposal 1: Create a Rel-20 WI enabling (e)RedCap devices to operate in bands supporting 3 MHz (15 PRB) channel bandwidth.

Proposal 2: Both 1Rx and 2Rx (e)RedCap devices should be supported in the WI.

Proposal 3: Re-use the existing (e)RedCap capabilities.

Proposal 4: Rel-20 WI should be limited to including the same bands as were defined in the Rel-18 <5MHz WI.

Proposal 5: Create one single RedCap WI including objectives for RLM/BFD relaxation and 3MHz channel bandwidth for (e)Redcap devices.

Proposal 6: Objectives of a common WI would be:

- Introduce support of 3MHz channel bandwidth (15 PRB) for eRedCap devices in FR1 with 15KHz SCS [for both TN and NTN]

- Example bands: n106, n85, n26, n28

- Introduce 3 MHz channel bandwidth support for n106 for 1Rx and 2Rx (e)RedCap devices.

- Utilize existing (e)RedCap capabilities defined in Rel-17, Rel-18 and Rel-19 unless otherwise specified.

- Define the necessary RF and RRM Core and Performance requirements for UE and BS.

- Define RLM/BFD relaxation requirements for 1Rx and 2Rx RedCap UE

# References

- [1] RP-250817, Moderator's summary for RAN4 led REL-20 topics, RAN4 Chair (Huawei)

- [2] RP-250xxx, NR Radio Resource Management (RRM) Phase 5, Rapporteur (Apple, CATT)