3GPP TSG-RAN WG4 Meeting #115	R4-2508729

# St. Julians, Malta, 19th – 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.174 | CR | 0133 | rev | 1 | Current version: | 16.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.174 with clarification for channel bandwidths below 10 MHz |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB-Core |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduction of clarification that for IAB channel bandwidths below 10 MHz are not supported. During work on IAB in Rel-16 there was an assumption to not support channel bandwidths below 10 MHz. This is reflected in all TX and RX requirements where 5 MHz channel bandwidth is removed. Actually, from supported by IAB list of FR1 bands only band n41 uses 5 MHz CBW, h owever for IAB for band n41 5 MHz CBW is not supported. During ongoing Rel-19 work item for 7 MHz channel bandwidth it was agreed to not include 7 MHz to IAB specification as well. Following was agreed in WF R4-2502870: 3-1-4: IAB and NCR Agreement: 7 MHz should not be supported by IAB 7 MHz should not be supported by NCR With this addition it will be clear that IAB doesn’t use CBWs below 10 MHz. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of clarification that CBWs below 10 MHz are not supported. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification will be clear that IAB specifications don’t use CBW below 10 MHz. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.176-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is revision of R4-2507236. |  |  |  |  |  |  |  |  |

******************************* < START OF CHANGE > ***************************************

### 5.3.5	IAB-DU and IAB-MT channel bandwidth per operating band

For IAB-DU, the channel bandwidth for NR bands for FR1 in Table 5.2.1 and for NR bands for FR2 defined in TS38.104 [2] is the same as specified for BS in TS38.104 [2], subclause 5.3.5.

For IAB-MT, the channel bandwidth for NR bands for FR1 in Table 5.2-1 is the same as specified for UE in TS38.101-1[3] in subclause 5.3.5 and for NR bands for FR2 defined in TS38.104[2] is the same as specified for UE in TS38.101-2[4] in subclause 5.3.5.

Note: Channel bandwidths below 10 MHz are not supported in the present version of the specification.

********************************** < END OF CHANGE > ***************************************