3GPP TSG-RAN WG4 Meeting # 115	R4-2506085

# Malta, MT,  19th – 23rd May, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.176-1 | CR | 0070 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_IAB-Perf)CR for 38.176-1, Correction on out-of-band blocking co-location requirement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB-Perf |  |  |  |  |  | Date: |  |  | 2025-05-08 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The terms in tables for out-of-band blocking co-location requirement are incorrect. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) Change “IAB” to “IAB-DU” in title and end of Table 7.5.5.2-1. 2) Remove “for WA IAB-DU” for Wanted signal mean power in Table 7.5.5.2-1. 3) Remove “IAB-DU” for interfering signal mean power in Table 7.5.5.2-1. 4) Change “IAB node” to “IAB-MT” in title of Table 7.5.5.4-1. 5) Remove “IAB node” for interfering signal mean power in Table 7.5.5.4-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The out-of-band blocking co-location requirement would be unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.5.5.2, 7.5.5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

7.5.5.2	Co-location requirements for IAB-DU

This additional blocking requirement may be applied for the protection of NR IAB receivers when GSM, CDMA, UTRA, E-UTRA BS or NR IAB operating in a different frequency band are co-located with a NR IAB. The requirement is applicable to all channel bandwidths supported by the NR IAB.

The requirements in this clause assume a 30 dB coupling loss between interfering transmitter and NR IAB receiver and are based on co-location with base stations or IAB of the same class.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel, with a wanted and an interfering signal coupled to IAB antenna input using the parameters in table 7.5.5.2-1 for all the IAB classes. The reference measurement channel for the wanted signal is identified in tables 7.2.5-1, 7.2.5-2 and 7.2.5-3 for each channel bandwidth and further specified in annex A.1.

For IAB type 1-H blocking requirement for co-location with IAB in other bands is applied for all operating bands for which co-location protection is provided.

Minimum conducted requirement is defined at the TAB connector for IAB type 1-H.

Table 7.5.5.2-1: Blocking performance requirement for NR IAB-DU when co-located with BS/IAB in other frequency bands.

| Frequency range of interfering signal | Wanted signal mean power (dBm) | Interfering signal mean power for WA (dBm) | Interfering signal mean power for MR (dBm) | Interfering signal mean power for LA (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| Frequency range of co-located downlink operating band | PREFSENS +6dB (Note 1) | +16 | +8 | x (Note 2) | CW carrier |
| NOTE 1: PREFSENS depends on the IAB-DU channel bandwidth as specified in TS 38.174 [2], tables 7.2.2-1, 7.2.2-2 and 7.2.2-3. NOTE 2: x = -7 dBm for NR IAB-DU co-located with Pico GSM850 or Pico CDMA850 x = -4 dBm for NR IAB-DU co-located with Pico DCS1800 or Pico PCS1900 x = -6 dBm for NR IAB-DU co-located with UTRA bands or E-UTRA bands or NR bands NOTE 3: The requirement does not apply when the interfering signal falls within any of the supported uplink operating band(s) or in ΔfOOB immediately outside any of the supported uplink operating band(s). |  |  |  |  |  |

<End of Change 1>

<Start of Change 2>

7.5.5.4	Co-location requirements for IAB-MT

This additional blocking requirement may be applied for the protection of IAB-MT receivers when GSM, CDMA, UTRA, E-UTRA, NR BS or IAB-node operating in a different frequency band are co-located with an IAB-node. The requirement is applicable to all IAB-MT channel bandwidths supported by the IAB-node.

The requirements in this clause assume a 30 dB coupling loss between interfering transmitter and IAB-node receiver and are based on co-location with base stations of the same class.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel, with a wanted and an interfering signal coupled to IAB type 1-H TAB connector input using the parameters in table 7.5.5.4-1 for all the IAB classes. The reference measurement channel for the wanted signal is identified in subclause 7.2.1 and subclause 7.2.2 for each IAB-MT channel bandwidth and further specified in annex A.1.

The blocking requirement for co-location with BS or IAB-node in other bands is applied for all operating bands for which co-location protection is provided.

Minimum conducted requirement is defined at the TAB connector for IAB type 1-H.

Table 7.5.5.4-1: Blocking performance requirement for the IAB-MT

| Frequency range of interfering signal | Wanted signal mean power (dBm) | Interfering signal mean power for WA (dBm) | Interfering signal mean power for LA (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- |
| Frequency range of co-located downlink operating band | PREFSENS +6dB (Note 1) | +16 | x (Note 2) | CW carrier |
| NOTE 1: PREFSENS depends on the IAB-MT channel bandwidth as specified in subclause 7.2.1 and subclause 7.2.2. NOTE 2: x = -7 dBm for IAB-MT co-located with Pico GSM850 or Pico CDMA850 x = -4 dBm for IAB-MT co-located with Pico DCS1800 or Pico PCS1900 x = -6 dBm for IAB-MT co-located with UTRA bands or E-UTRA bands or NR bands NOTE 3: The requirement does not apply when the interfering signal falls within any of the supported downlink operating band(s) or in ΔfOOB immediately outside any of the supported downlink operating band(s). |  |  |  |  |

<End of Change 2>