# 3GPP TSG-RAN WG4 Meeting #115	R4-2508371

# Malta, MT, 19th – 23rd May, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 5456 | rev | 1 | Current version: | 18.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on Link Recovery Procedures for NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions-core |  |  |  |  |  | Date: |  |  | 2025-05-05 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The requirements of link Recovery Procedures for NTN need to update because of some invalid table numbers, e.g., Table 8.5C.3.2-2, Table 8.5C.5.2-2, Table 8.5C.6.2-2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the invalid tables, including Table 8.5C.3.2-2, Table 8.5C.5.2-2, Table 8.5C.6.2-2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The RRM requirements of LR for NTN in R18 are incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5C.3.2, 8.5C.5.2, 8.5C.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R4-2505684 |  |  |  |  |  |  |  |  |

<Start of Change 1>

#### 8.5C.3.2	Minimum requirement

UE shall be able to evaluate whether the downlink radio link quality on the CSI-RS resource in set q0 estimated over the last TEvaluate_BFD_CSI-RS period becomes worse than the threshold Qout_LR_CSI-RS within TEvaluate_BFD_CSI-RS period.

The value of TEvaluate_BFD_CSI-RS is defined in table 8.5C.3.2-1 for FR1.

The requirements of TEvaluate_BFD_CSI-RS apply provided that the CSI-RS for BFD is not in a resource set configured with repetition ON. The requirements shall not apply when the CSI-RS resource in the active TCI state of CORESET is the same CSI-RS resource for BFD and the TCI state information of the CSI-RS resource is not given, wherein the TCI state information means QCL Type-D to SSB for L1-RSRP or CSI-RS with repetition ON.

P value for an RLM-RS resource to be measured is defined as

-	Psharing factor * Ntotal / Noutside_MG with Navailable = 0

-	Ntotal / Navailable with Navailable > 0

For a window W of duration max(TL1,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps, and starting at the beginning of any RLM-RS resource occasion:

-	Ntotal is the total number of RLM-RS resource occasions within the window, including those overlapped with measurement gap occasions or SMTC occasions within the window W, and

-	Noutside_MG is the number of RLM-RS resource occasions that are not overlapped with any measurement gap occasion within the window W

-	Navailable is

-	the number of RLM-RS resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion within the window W, if UE does not support parallelMeasurementWithoutRestriction and LEO satellites are measured for intra-frequency measurement, and

-	same as Noutside_MG, otherwise

-	TL1 is periodicity of the target RLM-RS

-	Psharing factor = 3.

NOTE:	The overlap between CSI-RS for BFD and SMTC means that CSI-RS for BFD is within the SMTC window duration.

Longer evaluation period would be expected if the combination of the BFD-RS resource, SMTC occasion and measurement gap configurations does not meet previous conditions.

For an FR1 serving cell, longer evaluation period would be expected during the period Tidentify_CGI when the UE is requested to decode an NR CGI.

The values of MBFD used in table 8.5C.3.2-1 are defined as

-	MBFD = 10, if the CSI-RS resource(s) in set q0 used for BFD is transmitted with Density = 3 and over the bandwidth ≥ 24 PRBs.

The values of PBFD used in table 8.5C.3.2-1 are defined as

For each CSI-RS resource in the set q0 configured for PCell SA

-	PBFD = 1.

Table 8.5C.3.2-1: Evaluation period TEvaluate_BFD_CSI-RS for FR1

| Configuration | TEvaluate_BFD_CSI-RS (ms) |
| --- | --- |
| no DRX | Max(50, Ceil(MBFD ´ P ´ PBFD) ´ TCSI-RS) |
| DRX cycle ≤ 320 ms | Max(50, Ceil(1.5 × MBFD ´ P ´ PBFD) ´ Max(TDRX, TCSI-RS)) |
| DRX cycle > 320 ms | Ceil(MBFD ´ P ´ PBFD) ´ TDRX |
| NOTE: TCSI-RS is the periodicity of CSI-RS resource in the set q0. TDRX is the DRX cycle length. |  |

<End of Change 1>

<Start of Change 2>

#### 8.5C.5.2	Minimum requirement

Upon request the UE shall be able to evaluate whether the L1-RSRP measured on the configured SSB resource in set q1 estimated over the last TEvaluate_CBD_SSB period becomes better than the threshold Qin_LR provided SSB_RP and SSB Ês/Iot are according to annex B.2.19.1 for a corresponding band.

The UE shall monitor the configured SSB resources using the evaluation period in table 8.5C.5.2-1 corresponding to the non-DRX mode, if the configured DRX cycle ≤ 320 ms.

The value of TEvaluate_CBD_SSB is defined in table 8.5C.5.2-1 for FR1.

where,

P value for an RLM-RS resource to be measured is defined as

-	Psharing factor * Ntotal / Noutside_MG with Navailable = 0

-	Ntotal / Navailable with Navailable > 0

For a window W of duration max(TL1,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps, and starting at the beginning of any RLM-RS resource occasion:

-	Ntotal is the total number of RLM-RS resource occasions within the window, including those overlapped with measurement gap occasions or SMTC occasions within the window W, and

-	Noutside_MG is the number of RLM-RS resource occasions that are not overlapped with any measurement gap occasion within the window W

-	Navailable is

-	the number of RLM-RS resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion within the window W, if UE does not support parallelMeasurementWithoutRestriction and LEO satellites are measured for intra-frequency measurement, and

-	same as Noutside_MG, otherwise

-	TL1 is periodicity of the target RLM-RS

-	Psharing factor = 3.

Longer evaluation period would be expected if the combination of the CBD-RS resource, SMTC occasion and measurement gap configurations does not meet previous conditions.

For an FR1 serving cell, longer evaluation period would be expected during the period Tidentify_CGI when the UE is requested to decode an NR CGI.

The values of PCBD used in table 8.5C.5.2-1 are defined as:

For each SSB resource in the set q1 configured for PCell.

-	PCBD = 1.

Table 8.5C.5.2-1: Evaluation period TEvaluate_CBD_SSB for FR1

| Configuration | TEvaluate_CBD_SSB (ms) |
| --- | --- |
| non-DRX, DRX cycle ≤ 320 ms | Max(25, Ceil(3 ´ P ´ PCBD) ´ TSSB) |
| DRX cycle > 320 ms | Ceil(3 ´ P ´ PCBD) ´ TDRX |
| NOTE: TSSB is the periodicity of SSB in the set q1. TDRX is the DRX cycle length. |  |  |

<End of Change 2>

<Start of Change 3>

#### 8.5C.6.2	Minimum requirement

Upon request the UE shall be able to evaluate whether the L1-RSRP measured on the configured CSI-RS resource in set q1 estimated over the last TEvaluate_CBD_CSI-RS period becomes better than the threshold Qin_LR within TEvaluate_CBD_CSI-RS period provided CSI-RS Ês/Iot is according to annex B.2.19.2 for a corresponding band.

The UE shall monitor the configured CSI-RS resources using the evaluation period in tables 8.5C.6.2-1 corresponding to the non-DRX mode, if the configured DRX cycle ≤ 320ms.

The value of TEvaluate_CBD_CSI-RS is defined in table 8.5C.6.2-1 for FR1.

P value for an RLM-RS resource to be measured is defined as:

-	Psharing factor * Ntotal / Noutside_MG with Navailable = 0

-	Ntotal / Navailable with Navailable > 0

For a window W of duration max(TL1,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps, and starting at the beginning of any RLM-RS resource occasion:

-	Ntotal is the total number of RLM-RS resource occasions within the window, including those overlapped with measurement gap occasions or SMTC occasions within the window W, and

-	Noutside_MG is the number of RLM-RS resource occasions that are not overlapped with any measurement gap occasion within the window W

-	Navailable is

-	the number of RLM-RS resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion within the window W, if UE does not support parallelMeasurementWithoutRestriction and LEO satellites are measured for intra-frequency measurement, and

-	same as Noutside_MG, otherwise

-	TL1 is periodicity of the target RLM-RS

-	Psharing factor = 3.

Note:	The overlap between CSI-RS for CBD and SMTC means that CSI-RS for CBD is within the SMTC window duration.

Longer evaluation period would be expected if the combination of the CBD-RS resource, SMTC occasion and measurement gap configurations does not meet previous conditions.

For an FR1 serving cell, longer evaluation period would be expected during the period Tidentify_CGI when the UE is requested to decode an NR CGI.

The values of MCBD used in table 8.5C.6.2-1 are defined as:

-	MCBD = 3, if the CSI-RS resource configured in the set q1 is transmitted with Density = 3 and over the bandwidth ≥ 24 PRBs.

The values of PCBD used in table 8.5C.6.2-1 are defined as:

For each CSI-RS resource in the set q1 configured for SA

-	PCBD = 1.

Table 8.5C.6.2-1: Evaluation period TEvaluate_CBD_CSI-RS for FR1

| Configuration | TEvaluateC_CBD_CSI-RS (ms) |
| --- | --- |
| non-DRX, DRX cycle ≤ 320 ms | Max(25, Ceil(MCBD ´ P ´ PCBD) ´ TCSI-RS) |
| DRX cycle > 320 ms | Ceil(MCBD ´ P ´ PCBD) ´ TDRX |
| NOTE: TCSI-RS is the periodicity of CSI-RS resource in the set q1. TDRX is the DRX cycle length. |  |  |

<End of Change 3>