3GPP TSG-RAN WG4 Meeting #115	R4-2507904

# St. Julian's, Malta, 19 - 23 May 2025									   (Revision of R4-2507715)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.764 | CR | 0007 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TR 36.764: Introduction of IoT-NTN band 252 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | EchoStar, Boost Mobile Network |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_FDD_S_band-Core |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Relevant changes and agreements for introduction of IoT NTN band 252 (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL) |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Regulatory requirement for n252 - Channel bandwidth, asymmetric channel bandwidth, channel raster, sync raster - RF requirements including MOP, Additional spurious emission for UE-to-UE coexistence, NS values and A-MRP, Tx-Rx frequency separation |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Agreements on definition of NTN band B252 are not captured. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5, 8 (new section) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev.0: initial version was based on 18.2.0 Rev.1: revised the based version to 18.3.0 and some text revisions |  |  |  |  |  |  |  |  |

--- Start of changes ---

Unneffected parts ommited

# 5	Common band agnostic aspects

## 5.1	System parameters

### 5.1.1	Operating bands

IoT NTN operation is designed in the operating bands defined in Table 5.1.1-1.

Table 5.1.1-1 E-UTRA operating bands for IoT NTN operation

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 254 | 1610 MHz | - | 1626.5 MHz | 2483.5 MHz | - | 2500 MHz | FDD |
| 253 | 1668 MHz | - | 1675 MHz | 1518 MHz | - | 1525MHz | FDD |
| 252 | 2000 MHz | - | 2020 MHz | 2180 MHz | - | 2200 MHz | FDD |
| NOTE: Satellite bands are numbered in descending order from 256 |  |  |  |  |  |  |  |

### 5.1.2		Channel arrangements

#### 5.1.2.1	Channel arrangement for category M1

Table 5.1.2.1-1: E-UTRA channel numbers

| E-UTRA Operating Band | ΔFRaster (kHz) | Downlink |  |  | Uplink |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | FDL_low (MHz) | NOffs-DL | Range of NDL (First – <Step size> – Last) | FUL_low (MHz) | NOffs-UL | Range of NUL (First – <Step size> – Last) |
| 254 | 100 | 2483.5 | 228571 | 228571 –<1>- 228735 | 1610 | 261339 | 261339 –<1>- 261503 |
| 253 | 100 | 1668 | 228501 | 228501-<1>-228570 | 1518 | 261269 | 261269 -<1>-261338 |
| 252 | 100 | 2180 | 228301 | 228301 –<1>- 228500 | 2000 | 261069 | 261069 –<1>- 261268 |

NOTE:	In addition to the above, the following was agreed during RAN4#108-bis:

-	The need for additional flexibility in channel raster for IoT NTN LTE bands has been identified, especially to accommodate future deployments of NB-IoT NTN and eMTC NTN together with NR NTN around existing services, taking into account possible guard bands for protection of other services, Doppler, and for deployment within the same SAN (i.e. same FFT).  This issue could be further discussed independently of band 253, with more general applicability to all IoT NTN LTE bands.

-	Capture this aspect in the TR.

-	FFS whether to capture a note in the specification

Table 5.1.2.1-2: Default UE TX-RX frequency separation

| E-UTRA Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| 254 | 873.5 MHz |
| 253 | -150 MHz |

NOTE:	As highlighted in RAN4#108-bis, based on operator input, there is a strong desire to consider flexible TX-RX separation for the band itself, and moreover for NTN FR1 bands, provided the frequency separation is sufficiently large.

The following conclusion was reached during RAN4#109:

Conclusion: Stop discussion on this issue in this WI.

NOTE:	The Tx-Rx separation for B252 is defined in clause 8.2.1.

Unneffected parts ommited

--- Next change ---

# 8	FDD band B252 (IoT-NTN FDD S_band)

## 8.1	Regulation

Table 8.1-1 shows the service allocation for the frequency range in the ITU-R Radio Regulations [xx] and Table 8.1-2 shows the the FCC frequency allocation table for the frequency range [xx].  From Table 8.1-1 and Table 8.1-2, there is some deviation in how MSS S-band is allocated in region 2 versus in all other regions. Per ITU, the Region 2 MSS S-Band had been allocated to UL 1980-2025 MHZ and DL 2160-2200 MHz. In the case of North America region (US, Canada, Mexico) the MSS band is allocated to UL 2000 MHz -2020 MHz and DL 2180 MHz -2200 MHz which is not exactly aligned to n256 definition, i.e., UL 1980 MHz – 2010 MHz, DL 2170 MHz – 2200 MHz. The 3GPP NTN  S-band B252 (UL 2000 MHz -2020 MHz and DL 2180 MHz -2200) is defined for consistency across North America and Region 2 as a whole.

Table 8.1-1: Allocation of 2000-2020 MHz and 2180–2200 MHz in the ITU-R Radio Regulations

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 1 980-2 010 FIXED MOBILE MOBILE-SATELLITE (Earth-to-space) 5.351A 5.388 5.389A 5.389B 5.389F |  |  |
| 2 010-2 025 FIXED MOBILE 5.388A 5.388B 5.388 | 2 010-2 025 FIXED MOBILE MOBILE-SATELLITE (Earth-to-space) 5.388 5.389C 5.389E | 2 010-2 025 FIXED MOBILE 5.388A 5.388B 5.388 |
| 2 170-2 200 FIXED MOBILE MOBILE-SATELLITE (space-to-Earth) 5.351A 5.388 5.389A 5.389F |  |  |
| 5.388: The frequency bands 1 885-2 025 MHz and 2 110-2 200 MHz are intended for use, on a worldwide basis, by administrations wishing to implement International Mobile Telecommunications (IMT). Such use does not preclude the use of these frequency bands by other services to which they are allocated. The frequency bands should be made available for IMT in accordance with Resolution 212 (Rev.WRC-15)* (see also Resolution 223 (Rev.WRC-15)*). 5.389A: The use of the bands 1 980-2 010 MHz and 2 170-2 200 MHz by the mobile-satellite service is subject to coordination under No. 9.11A and to the provisions of Resolution 716 (Rev.WRC-2000)** 5.389B: The use of the frequency band 1 980-1 990 MHz by the mobile-satellite service shall not cause harmful interference to or constrain the development of the fixed and mobile services in Argentina, Brazil, Canada, Chile, Ecuador, the United States, Honduras, Jamaica, Mexico, Paraguay, Peru, Suriname, Trinidad and Tobago, Uruguay and Venezuela. 5.389F: In Algeria, Cape Verde, Egypt, Iran (Islamic Republic of), Mali, Syrian Arab Republic and Tunisia, the use of the frequency bands 1 980-2 010 MHz and 2 170-2 200 MHz by the mobile-satellite service shall neither cause harmful interference to the fixed and mobile services, nor hamper the development of those services prior to 1 January 2005, nor shall the former service request protection from the latter services. |  |  |

Table 8.1-2: FCC frequency allocation for 2000-2020 MHz and 2180–2200 MHz

The following are the applicable FCC regulatory requirements:

#### 47 CFR § 25.202

According to paragraph (a)(4)(ii), the following frequencies are available for use by the 2 GHz Mobile-Satellite Service:

- 2000-2020 MHz: User-to-Satellite Link;

- 2180-2200 MHz: Satellite-to-User Link.

According to paragraph (f) on emission limitations, except for SDARS terrestrial repeaters and as provided for in paragraph (i), the mean power of emissions shall be attenuated below the mean output power of the transmitter in accordance with the schedule set forth in paragraphs (f)(1) through (f)(4) of this section:

- In any 4 kHz band, the center frequency of which is removed from the assigned frequency by more than 50 percent up to and including 100 percent of the authorized bandwidth: 25 dB;

(2)  In any 4 kHz band, the center frequency of which is removed from the assigned frequency by more than 100 percent up to and including 250 percent of the authorized bandwidth: 35 dB;

(3)  In any 4 kHz band, the center frequency of which is removed from the assigned frequency by more than 250 percent of the authorized bandwidth: An amount equal to 43 dB plus 10 times the logarithm (to the base 10) of the transmitter power in watts;

(4)  In any event, when an emission outside of the authorized bandwidth causes harmful interference, the Commission may, at its discretion, require greater attenuation than specified in paragraphs (f) (1), (2) and (3) of this section.

#### 47 CFR § 25.216

According to paragraph (e) on limits on emissions from mobile earth stations for protection of aeronautical radionavigation-satellite service, the e.i.r.p density of emissions from mobile earth stations with assigned uplink frequencies between 1990 MHz and 2025 MHz shall not exceed −70 dBW/MHz, averaged over any 2 millisecond active transmission interval, in frequencies between 1559 MHz and 1610 MHz. The e.i.r.p. of discrete emissions of less than 700 Hz bandwidth from such stations between 1559 MHz and 1605 MHz shall not exceed −80 dBW, averaged over any 2 millisecond active transmission interval. The e.i.r.p. of discrete emissions of less than 700 Hz bandwidth from such stations between 1605 MHz and 1610 MHz manufactured more than six months after Federal Register publication of the rule changes adopted in FCC 03-283 shall not exceed −80 dBW, averaged over any 2 millisecond active transmission interval.

It should be noted that for the TN Band 23, which is studied and documented in [xx], the FCC regulation part 25.252 on ATC (Ancillary Terrestrial Component) for satellite service was referred to derive the RF requirements. However, since B252 is defined as an NTN band, ATC related requirements are not applicable for this band.

The following figure 8.1-1 shows an overview of the NR TN bands adjacent to the NTN B252 band.

Figure 8.1-1: Adjacent TN bands to NTN band n252 in North America

As shown in Figure 8.1-1, the band 252 is adjacent to LTE band 25 DL and band 2 DL, and is overlap with band 70 DL and band 66 DL. The band 25 DL and band 2 DL protections in terms of UE-to-UE coexistence perspective is addressed in section 8.2.2.2.1.  The UE-to-UE coexistence between band 252 and band 70 is subject to national/regional regulation. For UE-to-UE coexistence between band 252 and band 66, spurious emission requirement of -50dBm/MHz can be met.

## 8.2	UE requirements

### 8.2.1	Tx-RX  frequency separation

#### 8.2.1.1	Tx-Rx frequency separation for Cat. M1

Table 8.2.1.1-1: Default UE TX-RX frequency separation

| E-UTRA Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| 252 | 180 MHz1 161.4 to 198.6 MHz2 |
| NOTE 1: Default TX-RX separation. NOTE 2: The verification of flexible TX-RX frequency separation within this range is limited to reference sensitivity. Further details are specified in clause 7.3A. |  |

Table 8.2.1.1-2: TX – RX carrier centre frequency separation for REFSENS verification

| E-UTRA Operating Band | Channel bandwidth | TX – RX carrier centre frequency separation for REFSENS verification |
| --- | --- | --- |
| 252 | 1.4 MHz | 161.4 MHz, 198.6 MHz |

#### 8.2.1.2	Tx-Rx frequency separation for Cat. NB1 and NB2

Table 8.2.1.2-1: Default UE TX-RX frequency separation

| E-UTRA Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| 252 | 180 MHz1 160.2 to -199.8 MHz2 |
| NOTE 1: Default Tx-Rx separation. NOTE 2: The verification of flexible TX-RX frequency separation within this range is limited to reference sensitivity. |  |

Table 8.2.1.2.2-2: TX – RX carrier centre frequency separation for REFSENS verification

| E-UTRA Operating Band | Channel bandwidth | TX – RX carrier centre frequency separation for REFSENS verification |
| --- | --- | --- |
| 252 | 0.2 MHz | 160.2 MHz, 199.8 MHz |

### 8.2.2	UE transmitter characteristics

#### 8.2.2.1 	Maximum output power

Table 8.2.2.1 -1: UE Power Class for category M1

| EUTRA band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 252 |  |  | 23 | +/-2 | 20 | +/-2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. |  |  |  |  |  |  |

Table 8.2.2.1-2: UE Power Class for category NB1 and NB2

| EUTRA band | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| 252 | 23 | +/-2 | 20 | +/-2 |

#### 8.2.2.2 	Emission requirements and NS values

##### 8.2.2.2.1	Additional spurious emission requirements to protect GNSS frequency

As described in section 8.1, band 252 shall protect the aeronautical radionavigation-satellite service in the frequency range of 1559 MHz and 1610 MHz. The following additional spurious emission requirement in Table 8.2.2.2.1-1 is defined. NS

Table 8.2.2.2.1-1: Additional spurious emission requirement to protect GNSS frequency (“NS_06N”) for UE category M1

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 1.4 MHz |  |  |
| 1559 ≤ f < 1610 | -50 | 700 Hz | Averaged over any 2 millisecond active transmission interval |
| 1559 ≤ f < 1610 | -40 | 1MHz |  |

Table 8.2.2.2.1-2: Additional spurious emission requirement to protect GNSS frequency (“NS_06N”) for UE category NB1 and NB2

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 200 kHz |  |  |
| 1559 ≤ f < 1610 | -50 | 700 Hz | Averaged over any 2 millisecond active transmission interval |
| 1559 ≤ f < 1610 | -40 | 1MHz |  |

##### 8.2.2.2.2	Additional spurious emission requirements for UE-to-UE coexistence to protect band 2 and band 25

To protect band 2 and band 25, the following two different additional spurious emission requirements are defined, each is associated to an NS value, and a corresponding A-MPR.

  - -40 dBm/MHz (NS_07N)

  - -30 dBm/MHz (NS_08N)

This requirement applies based on coordination between operators and subject to regional/national regulation. From UE perspective, UE applies this requirement based on the received NS value. For implementation flexibility, the requirement for aeronautical radionavigation-satellite service is also included in these NSs.

Table 8.2.2.2.2-1: Additional out-of-band requirements for "NS_07N" for UE category M1

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 1.4 MHz |  |  |
| 1559 ≤ f < 1610 | -50 | 700 Hz | Averaged over any 2 millisecond active transmission interval |
| 1559 ≤ f < 1610 | -40 | 1MHz |  |
| 1930 ≤ f ≤1995 | -40 | 1MHz | Frequency range covers DL band 2 and band 25 |

Table 8.2.2.2.2-2: Additional out-of-band requirements for "NS_08N" for UE category M1

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 1.4 MHz |  |  |
| 1559 ≤ f < 1610 | -50 | 700 Hz | Averaged over any 2 millisecond active transmission interval |
| 1559 ≤ f < 1610 | -40 | 1MHz |  |
| 1930 ≤ f ≤1995 | -30 | 1MHz | Frequency range covers DL band 2 and band 25 |

Table 8.2.2.2.2-3: Additional out-of-band requirements for "NS_07N" for UE category NB1 and NB2

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 200 kHz |  |  |
| 1559 ≤ f < 1610 | -50 | 700 Hz | Averaged over any 2 millisecond active transmission interval |
| 1559 ≤ f < 1610 | -40 | 1MHz |  |
| 1930 ≤ f ≤1995 | -40 | 1MHz | Frequency range covers DL band 2 and band 25 |

Table 8.2.2.2.2-4: Additional out-of-band requirements for "NS_08N" for UE category NB1 and NB2

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 200 kHz |  |  |
| 1559 ≤ f < 1610 | -50 | 700 Hz | Averaged over any 2 millisecond active transmission interval |
| 1559 ≤ f < 1610 | -40 | 1MHz |  |
| 1930 ≤ f ≤1995 | -30 | 1MHz | Frequency range covers DL band 2 and band 25 |

### 8.2.3	UE receiver characteristics

#### 8.2.3.1		Reference Sensitivity

##### 8.2.3.1.1		Reference Sensitivity for Cat.M1

Table 8.2.3.1.1-1: Reference sensitivity for FDD UE category M1 QPSK PREFSENS

| NTN Band | REFSENS (dBm) | Duplex Mode |
| --- | --- | --- |
| 252 | -102.2 | FDD |

Table 8.2.3.1.1-2: Reference sensitivity for HD-FDD UE category M1 QPSK PREFSENS

| NTN Band | REFSENS (dBm) | Duplex Mode |
| --- | --- | --- |
| 252 | -103 | HD-FDD |

##### 8.2.3.1.2		Reference Sensitivity for Cat.NB1, Cat. NB2

Table 8.2.3.1.2-1: Reference sensitivity Cat. NB1/NB2

| NTN Band | REFSENS (dBm) |
| --- | --- |
| 252 | - 108.2 |

#### 8.2.3.2		Blocking requirements

##### 8.2.3.2.1		In-band blocking requirements

###### 8.2.3.2.1.1		In-band blocking requirements for Cat. M1

Table 8.2.3.2.1.1-1: In-band blocking for Cat.M1

| E-UTRA band | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | PInterferer | dBm | -56 | -44 |
|  | FInterferer (offset) | MHz | =-BW/2 – FIoffset,case 1 & =+BW/2 + FIoffset,case 1 | ≤-BW/2 – FIoffset,case 2 & ≥+BW/2 + FIoffset,case 2 |
| 256, 255, 254, 253, 252 | FInterferer | MHz | (NOTE 2) | FDL_low – 15 to FDL_high + 15 |
| NOTE 1: For certain bands, the unwanted modulated interfering signal may not fall inside the UE receive band, but within the first 15 MHz below or above the UE receive band NOTE 2: For each carrier frequency the requirement is valid for two frequencies: a. the carrier frequency -BW/2 - FIoffset, case 1 and b. the carrier frequency +BW/2 + FIoffset, case 1 NOTE 3: FInterferer range values for unwanted modulated interfering signal are interferer center frequencies |  |  |  |  |

##### 8.2.3.2.2	Out-of-band blocking requirements

###### 8.2.3.2.2.1	Out-of-band blocking requirements for Cat.M1

Table 8.2.3.2.2.1-1: Out of-band blocking for category M1 UE

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| 252 | Finterferer (CW) | MHz | -110 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -155 < f – FDL_low ≤ -110 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 155 or FDL_high + 85 ≤ f ≤ 12750 |

###### 8.2.3.2.2.2	Out-of-band blocking requirements for Cat.M1

Table 8.2.3.2.2.2-1: Out-of-band blocking parameters for category NB1 and NB2 UE

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pw | dBm | REFSENS + 6 dB |  |  |
|  | Pinterferer | dBm | -44 | -30 | -151 |
| 252 | Finterferer (CW) | MHz | -110 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -155 < f – FDL_low ≤ -110 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 155 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: For operating bands which downlink band frequency range is between 1475.9 MHz < f < 2690 MHz the power level of the interferer (PInterferer) for Range 3 shall be modified to: -20 dBm for the frequency range which is bounded by FDL_low- 200 MHz of the lowest band that UE supports in frequency range 1475.9 MHz < f < 2690 MHz and FDL_high + 200 MHz of the highest band that UE supports in frequency range 1475.9 MHz < f < 2690 MHz.” |  |  |  |  |  |

### 8.2.4	A-MPR and NS value

A-MPR is band specific requirement which is used to meet the additional requirements indicated by network signalling (NS) value.

For band 252, the following three NSs are defined:

-	NS_06N: for protection of aeronautical radionavigation-satellite service. This requirement is based on FCC rule 47 C.F.R § 25.216 (e)

- 	NS_07N and NS_08N: for protection of band 2 and band 25. This requirement applies based on coordination between operators and subject to regional/national regulation. See section 8.2.2.2 for detail on UE-to-UE coexistence additional spurious requirements. From UE perspective, UE applies this requirement based on the received NS value.

Table 8.2.4-1: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (clause) | NR satellite Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_06N | 8.2.2.2.1 | 252 | 5, 10, 15, 20 |  | N/A |
| NS_07N | 8.2.2.2.2 | 252 | 5, 10, 15, 20 |  | N/A |
| NS_08N | 8.2.2.2.1 | 252 | 5, 10, 15, 20 |  | N/A |
| NOTE 1: The NS_07N and NS_08N is signalled based on coordination between operators and subject to regional/national regulation |  |  |  |  |  |

Table 8.2.4-2: Mapping of network signalling label

| NTN satellite band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n252 | NS_01 | NS_06N | NS_07N | NS_08N |  |  |  |  |

--- End of changes ---