3GPP TSG-RAN4 Meeting #115 	R4-2505631

# Malta, Malta, 19th May 2025 - 23th May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 2752 | rev | - | Current version: | 18.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (HPUE_NR_CADC_SUL_R19) CR for addition and correction of notes in clause 6.2A.1.3 UE MOP for Inter-band CA - TS38.101-1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Limited |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | HPUE_NR_CADC_SUL_R19 |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | A first change is needed as the change corresponding the endorsed Draft CR R4-2418058 (included in the big CR R4-2417550) was not implemented, due to that: - In Table 6.2A.1.3-1, CA_n25A-n66A/n71A/n85A and CA_n66A-n71A/n85A have currently a superscript referring to “NOTE 9: The UE supports either PC3 or PC2 within each NR TDD band”. - But n25, n66, n71, n85 are actually all FDD bands. - As those bands are FDD bands, a new note: “NOTE 10: The UE supports PC3 within each NR FDD band” fixes the issue. Additionally, a second change is needed (not related to CR R4-2418058): - In Table 6.2A.1.3-1, CA_n41A-n85A which is a FDD-TDD CA configuration refers to Note 9 (TDD+TDD) is applied to it. It should be corrected to Note 6 (FDD+TDD). |  |  |  |  |  |  |  |  |
| Summary of change: |  | In Table 6.2A.1.3-1: - Add “2610” and “+2/-32” for CA_n25A-n66A/n71A/n85A and CA_n66A-n71A/n85A. - Add “NOTE 10: The UE supports PC3 within each NR FDD band”. - Change the superscript 9 to 6 for CA_n41A-n85A. - Change “CA_25A-n48A” to “CA_n25A-n48A”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is incorrecly refered to: - Note 6 (FDD+TDD) for CA_n25A-n66A/n71A/n85A and CA_n66A-n71A/n85A which are FDD+FDD. - Note 9 (TDD+TDD) for CA_n41A-n85A which is FDD+TDD |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

<<Start of change>>

#### 6.2A.1.3	UE maximum output power for Inter-band CA

For inter-band downlink carrier aggregation with one uplink carrier assigned to one NR band, the transmitter power requirements in Table 6.2.1-1 apply for power class 3 and other power classes if indicated in clause 5.5A.3.

For inter-band carrier aggregation with two uplink contiguous carrier assigned to one NR band, the transmitter power requirements specified in subclause 6.2A.1.1 apply.

For inter-band carrier aggregation with two uplink non-contiguous carrier assigned to one NR band, the transmitter power requirements specified in subclause 6.2A.1.2 apply. For inter-band uplink carrier aggregation with uplink assigned to two NR bands, UE maximum output power shall be measured over all component carriers from different bands. If each band has separate antenna connectors, maximum output power is defined as the sum of maximum output power from each UE antenna connector. The period of measurement shall be at least one sub frame (1 ms). The maximum output power is specified in Table 6.2A.1.3-1.

Table 6.2A.1.3-1: UE Power Class for uplink inter-band CA (two bands)

| Uplink CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n3A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n1A-n5A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n1A-n7A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n1A-n8A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n1A-n18A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n1A-n20A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n1A-n26A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n1A-n28A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n1A-n40A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n1A-n41A |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n1A-n46A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n1A-n74A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n1A-n77A |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n1A-n78A |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n1A-n78C |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n1A-n79A |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n1A-n102A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n1A-n102B |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n1A-n102C |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n1A-n105A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n2A-n5A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n2A-n7A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n2A-n12A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n2A-n14A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n2A-n30A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n2A-n41A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n2A-n48A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n2A-n66A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n2A-n71A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n2A-n77A |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n2A-n78A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n3A-n5A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n3A-n7A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n3A-n8A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n3A-n18A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n3A-n20A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n3A-n26A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n3A-n28A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n3A-n34A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n3A-n38A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n3A-n40A |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n3A-n41A |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n3A-n41C |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n3A-n74A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n3A-n77A |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n3A-n78A |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n3A-n78C |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n3A-n79A |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n3A-n79C |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n3A-n102A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n3A-n102B |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n3A-n102C |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n3A-n105A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n5A-n7A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n5A-n12A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n5A-n13A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n5A-n14A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n5A-n25A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n5A-n28A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n5A-n30A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n5A-n40A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n5A-n41A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n5A-n48A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n5A-n66A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n5A-n77A |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n5A-n78A |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n5A-n78C |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n5A-n79A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n5A-n105A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n7A-n8A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n7A-n20A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n7A-n25A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n7A-n26A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n7A-n28A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n7A-n40A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n7A-n46A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n7A-n66A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n7A-n71A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n7A-n77A |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n7A-n78A |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n7A-n102A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n7A-n102B |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n7A-n102C |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n7A-n105A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n8A-n20A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n8A-n28A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n8A-n34A |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n8A-n39A |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n8A-n40A |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n8A-n41A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n8A-n41C |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n8A-n77A |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n8A-n78A |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n8A-n78C |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n8A-n79A |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n8A-n79C |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n12A-n25A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n12A-n30A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n12A-n66A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n12A-n77A |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n12A-n78A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n13A-n25A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n13A-n66A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n13A-n77A |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n14A-n30A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n14A-n66A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n14A-n77A |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n18A-n28A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n18A-n40A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n18A-n41A |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n18A-n74A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n18A-n77A |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n18A-n78A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n20A-n28A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n20A-n78A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n24A-n41A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n24A-n48A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n24A-n77A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n25A-n38A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n25A-n41A |  |  | 266 | +2/-32 | 23 | +2/-3 |  |  |
| CA_n25A-n41C |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n25A-n48A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n25A-n66A |  |  | 2610 | +2/-32 | 23 | +2/-3 |  |  |
| CA_n25A-n71A |  |  | 2610 | +2/-32 | 23 | +2/-3 |  |  |
| CA_n25A-n77A |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n25A-n78A |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n25A-n85A |  |  | 2610 | +2/-32 | 23 | +2/-3 |  |  |
| CA_n26A-n28A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n26A-n48A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n26A-n66A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n26A-n70A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n26A-n77A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n26A-n78A |  |  | 266 | +2/-32 | 23 | +2/-3 |  |  |
| CA_n28A-n34A |  |  | 266 | +2/-32 | 23 | +2/-3 |  |  |
| CA_n28A-n39A |  |  | 266 | +2/-32 | 23 | +2/-3 |  |  |
| CA_n28A-n40A |  |  | 266 | +2/-32 | 23 | +2/-3 |  |  |
| CA_n28A-n41A |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n28A-n41C |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n28A-n46A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n28A-n50A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n28A-n74A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n28A-n77A |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n28A-n78A |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n28A-n79A |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n28A-n79C |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n28A-n102A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n28A-n102B |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n28A-n102C |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n34A-n79A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n30A-n66A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n30A-n77A |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n34A-n39A |  |  | 269 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n34A-n40A |  |  | 269 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n34A-n41A |  |  | 269 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n34A-n41C |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n34A-n79A |  |  | 269 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n34A-n79C |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n38A-n66A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n38A-n78A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n39A-n40A |  |  | 269 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n39A-n41A |  |  | 269 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n39A-n41C |  |  | 269 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n39A-n79A |  |  | 269 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n39A-n79C |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n40A-n41A |  |  | 269 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n40A-n41C |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n40A-n77A |  |  | 269 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n40A-n78A |  |  | 269 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n40A-n79A |  |  | 269 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n40A-n79C |  |  | 269 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n40A-n105A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n41A-n48A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n41A-n50A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n41A-n66A |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n41A-n70A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n41A-n71A |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n41A-n74A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n41A-n77A |  |  | 269 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n41A-n78A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n41A-n79A |  |  | 269 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n41A-n79C |  |  | 269 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n41C-n66A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n41C-n71A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n41C-n77A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n41C-n79A |  |  | 269 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n41A-n85A |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n46A-n48A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n46A-n48B |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n46A-n77A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n46A-n78A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n48A-n66A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n48A-n70A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n48A-n71A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n48A-n96A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n48B-n96A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n48A-n96B |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n50A-n78A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n66A-n71A |  |  | 2610 | +2/-32 | 23 | +2/-3 |  |  |
| CA_n66A-n77A |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n66A-n78A |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n66A-n85A |  |  | 2610 | +2/-32 | 23 | +2/-3 |  |  |
| CA_n70A-n71A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n70A-n77A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n70A-n78A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n71A-n77A |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n71A-n78A |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n74A-n77A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n74A-n78A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n77A-n79A |  |  | 269 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n77A-n85A |  |  | 266 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n77A-n102A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n77A-n102B |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n77A-n102C |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n78A-n79A |  |  | 269 | +2/-3 | 23 | +2/-3 |  |  |
| CA_n78A-n92A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n78A-n102A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n78A-n102B |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n78A-n102C |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n78A-n104A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_n78A-n105A |  |  |  |  | 23 | +2/-3 |  |  |
| NOTE 1: Void NOTE 2: An uplink CA configuration in which at least one of the bands has NOTE 3 in Table 6.2.1-1 is allowed to reduce the lower tolerance limit by 1.5 dB when the transmission bandwidths of at least one of the bands is confined within FUL_low and FUL_low + 4 MHz or FUL_high - 4 MHz and FUL_high. NOTE 3: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 4: For inter-band carrier aggregation the maximum power requirement should apply to the total transmitted power over all component carriers (per UE). NOTE 5: Power class 3 is the default power class unless otherwise stated. NOTE 6: The UE supports PC3 within NR FDD band, and supports either PC3 or PC2 within NR TDD band. NOTE 7: The UE that supports a PC2 uplink CA configuration with single carrier for each individual band and a composite of supporting PC3 within an NR TDD or FDD band and PC2 within a second NR TDD band may signal a higherPowerLimit-r17 capability whereby the maximum output power indicated in the table may be exceeded in accordance with sub-clause 6.2A.4.1.3. The power classes referenced are according to the reported ue-PowerClassPerBandPerBC-r17 if indicated or ue-PowerClass otherwise. NOTE 8: The UE that supports a PC3 uplink CA configuration with a composite of supporting PC3 within an NR TDD or FDD band and PC5 within a second NR band listed in Table 6.2F.1-1 may signal a higherPowerLimit-r17 capability whereby the maximum output power indicated in the table may be exceeded in accordance with sub-clause 6.2A.4.1.3. The power classes referenced are according to the reported ue-PowerClassPerBandPerBC-r17 if indicated or ue-PowerClass otherwise. NOTE 9: The UE supports either PC3 or PC2 within each NR TDD band. NOTE 10: The UE supports PC3 within each NR FDD band |  |  |  |  |  |  |  |  |

If a UE supports a different power class than the default UE power class for the band combination listed in Table 6.2A.1.3-1 and the supported power class enables the higher maximum output power than that of the default power class:

–	if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is not absent and the average percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-interBandCA-PC2 as defined in TS 38.331 (The exact evaluation period is no less than one radio frame); or

–	 if 10 log10 ∑ pEMAX,c or PEMAX,CA which defined in clause 6.2A.4.1.3 is 23dBm or lower;

–	shall apply all requirements for the default power class to the supported power class and set the configured transmitted power as specified in clause 6.2A.4.1.3;

–	else;

–	shall apply all requirements for the supported power class and set the configured transmitted power as specified in clause 6.2A.4.1.3 (regardless of the average percentage of uplink symbols if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is absent).

The average percentage of uplink symbols is defined as 50%  ( DutyNR, x /maxDutyNR,x + DutyNR, y /maxDutyNR,y, ). DutyNR, x, DutyNR, y represent the actual percentage of uplink symbols transmitted in the same evaluation period (The exact evaluation period is no less than one radio frame) for NR Band x, NR Band y respectively; maxDutyNR,x, maxDutyNR,y represent the field of UE capability maxUplinkDutyCycle-PC2-FR1 per band as defined in TS 38.331.  For NR Band x or NR Band y,

–	if power class of one or both of the bands within the band combination is power class 2 and the corresponding UE capability maxUplinkDutyCycle-PC2-FR1 is absent;

–	the corresponding maxDutyNR,x or maxDutyNR,y is equal to 50%;

–	else if the band is configured with power class 3;

–	the corresponding maxDutyNR,x or maxDutyNR,y is equal to 100%.

Table 6.2A.1.3-2 Void

<<Unchanged parts of the section skipped>>

<<End of change>>