3GPP TSG RAN #108			RP-250972

Prague, Czech Republic, June 9th – 13th 2025

Title:	Discussion on NR Ambient IoT work scope in Rel-20

Source:	OPPO

Agenda:	15.1.3

# Introduction

During RAN#106 meeting a way forward on Ambient IoT work scope and workplan for Rel-19 and Rel-20 was also endorsed [1], where the endorsed workplan for A-IoT in 5G-A Rel-20 includes the following.

Workplan for Rel-20: - Includes D2T2 - Includes Device 2 - A limited number of additional use cases, deployments, connectivity topologies, devices, traffic types in TR38.848 as a starting point - Note: work on D2T2, device 2 shall follow the Rel-19 study conclusions, and further down-selection of architecture options for topology 2 may be needed. For indoor use cases, deviations from Rel-19 study conclusion shall be well justified and agreed. - Details to be discussed at a later RAN plenary meeting, including the need for study phase. For both Rel-19 and Rel-20, co-ordination with SA2/3 is expected

In RAN#107 meeting, further discussions on the potential work scope for Ambient IoT were carried out. A moderator’s summary for the purpose of TU planning is provided in [2] as followed.

- Device 1 in D2T2 - Device 2b in D1T1 and D2T2 - Study on A-IoT for outdoor scenarios - Further discuss other aspects of Rel-20 A-IoT (e.g. DO-A traffic, unavailability direction 2) in RAN#108 - [3~3.5] TUs in RAN1 and [2] TUs in RAN2 for Release 20 5G-Adv A-IoT

In this contribution, we provide our views and suggestions on the general scope of A-IoT work in Rel-20 that should be considered for approval in this RAN plenary meeting.

# Scope discussion

## Deployment 2 Topology 2 (D2T2) and Device 2b

According to the approved WID of Rel-19 A-IoT [3], the general scope for the normative work includes deployment scenario 1 with topology 1 (specifically D1T1-B in TR 38.769 [4]), Device 1 only (~1 µW peak power consumption), FR1 licensed spectrum in FDD with CW in the UL spectrum, A-IoT BS located indoor, and traffic types DO-DTT, DT, for rUC1 (indoor inventory) and rUC4 (indoor command). Based on the study report in [4], this general scope represents only a subset of what had been studied during the study phase of A-IoT in Rel-19. For the leftover aspects that had been studied in WGs but not yet specified and supported in Rel-19, it was endorsed in [2] the workplan for Rel-20 will include both D2T2 and Device 2 with limited number of additional use cases, deployments, connectivity topologies, devices, and traffic types from TR38.848 [5] (which is a RAN level study TR).

Firstly, according to TR38.769 [4] from the WG-level study, D1T1 and D2T2 encompasses several sub-scenarios based on whether the CW is inside / outside topology or no CW is used in the deployment. For the indoor use cases, D1T1-B (BS indoor, device indoor, CW outside topology) was selected to be supported and specified due to simplicity, quick to market, no intermediate UE involvement (less specification effort) and within only 3 WG meetings in Rel-19. For D1T1-A1 (CW node inside topology 1, ‘CW’ in CW2D and ‘R2’ in D2R are different, ‘CW’ in CW2D and ‘R1’ in R2D are same, ‘R1’ in R2D and ‘R2’ in D2R are different), D1T1-A2 (same reader for R2D and D2R and same node for the reader and CW) and D1T1-C (no CW) could provide further deployment options in Rel-20. Moreover, Device 2a/b provides a larger coverage (D2R communication distance) than Device 1. However, compared to D1T1-B, the necessity and additional benefit(s) from supporting A1/2 could be further studied / identified.

Observation 1: The necessity and additional benefit(s) from supporting D1T1-A1/2 could be further discussed and justified for Rel-20 in RAN.

For the deployment scenarios in D2T2, where the BS is located outdoor and device is indoor (which would require a help from an intermediate UE located in indoor), these will enable the support to new/additional A-IoT indoor use cases in Rel-20 such as 5.2 Medical instruments inventory management and positioning, 5.6 Smart homes, 5.15 Smart laundry, 5.11 Online modification of medical instruments status, and 5.26 Elderly Health Care in TR 38.848 [5]. Typically, the number of A-IoT devices in these use cases are relatively/much smaller than a warehouse, factory and supermarket, it is not affordable and sometimes infeasible (e.g., due to regulations) to deploy a BS indoor especially for them in the premise. The situation would be worse when considering that these devices may be distributed with physical blockages in-between or into different rooms separated by walls. Therefore, they will rely on a UE reader which could be a smartphone connects to a serving base station in order to make an inventory or sending commands to the A-IoT devices that are located indoor. Furthermore, with the use of UE readers, it is possible for the already deployed BS can be reused with only software updates to support of A-IoT services. As such, it is functional necessary and endorsed for the inclusion in workplan for Rel-20 [2].

However, not all deployment scenarios in D2T2 should be supported in Rel-20. For scenarios where a CW is needed for backscattering devices, the complication would be in the CW interference handling for coexistence (to both Uu operation and nearby equipment), the cost of installing a CWN or cost of implementing additional hardware and antennas for CW TX and RX, and/or the support of carrier wave cancellation by a UE reader. In some cases, e.g., medical equipment and premises, the operation is sensitive and critical to any radio interferences. For a smart home use, it is impractical and expensive for smartphone users to purchase and install a CWN and keep in ON at all time. Furthermore, the additional complexity and cost in a smartphone UE to implement carrier wave cancellation seem not justifiable. It would be much simpler and overall cheaper to operate with devices that support active transmission (e.g., Device 2b). Note that CW control mechanism, including resource allocation and power control, is not specified in Rel-19 as the reader is base station and the CWN is outside topology. If scenarios where a CW is needed for backscattering devices is to be supported under D2T2, as the reader is UE, considerable workload are needed in RAN1 and RAN2 to specify such mechanism.

Since the workplan for Rel-20 includes already Device 2, based on Rel-19 study in the WGs, it is clear and a common understanding that device (un)availability direction 2 would be needed to extend the sustainable operating time of a device.

Observation 2: For scenarios where a CW is needed for backscattering devices in D2T2, additional complication and cost would be introduced to implement carrier wave transmission and cancellation for UE type reader, and also considerable workload to specify CW control mechanism.

Proposal 1: Deployment scenario D2T2-C, Device 2b and device (un)availability direction 2 should be the main focus for A-IoT normative work in Rel-20.

## Outdoor use cases (D4T1)

Another popular use case group and deployment studied and identified in during the RAN-level study for A-IoT is the outdoor scenario. In TR 38.848 [5], it includes rUC5 (outdoor inventory), rUC6 (outdoor sensor), rUC7 (outdoor positioning) and rUC8 (outdoor command). For the outdoor use, the coverage should support at least micro-cell or pico-cell based deployment, where the target distance should be between 50m to 500m, which is above the design target for the existing device 1 and 2a. Note that, outdoor use cases have not been evaluated so far in Rel-19. If it needs to be supported by Rel-20 specification, the normative work should be based on what is/are supported in Rel-19 for indoor. That is, no UE-based reader (deployment scenario 4), topology 1 only and Device 2b (the only viable device at this stage for a larger coverage). It is not advisable to consider Device 1 and 2a due to insufficient coverage and the requirement of a carrier wave node, which creates a challenge in the outdoor coexistence with NR Uu operation.

On the other hand, Topology 4 poses an interesting deployment usage where a BS does not need to be involved. This Topology 4 could be used in both indoor and outdoor deployment scenario without the need of large coverage. But additional work is required relating to resource allocation for the UE reader without a BS.

If device 2b cannot satisfy the coverage and/or coexistence requirement of outdoor scenario, a new device type, such as Device C could be considered. For Device C, which supports up to 10mW peak power consumption but device architecture is still unknown, would require a study in both RAN1 and RAN4. Furthermore, for outdoor deployment where a CWN is not used, it is unclear whether device energy harvesting could be based on RF energy source. Considering Device C peak power consumption is much higher than that of Device 2, other energy sources maybe needed and this aspect should be also considered and studied, including the sustainable operating time.

Proposal 2: For outdoor use cases,

- A study phase is at least needed to evaluate whether Device 2b is able to satisfy coverage and coexistence requirement of outdoor scenario, if not, Device C should be considered along with study on the device architecture and other energy sources.

## DO-A traffic

For the traffic type of DO-A, which was briefly studied in Rel-19 in RAN WGs, it can facilitate the transmission of data generated by the device, such as sensor data reporting. To support DO-A a device needs to achieve time/frequency synchronization with reader and also to obtain necessary configuration before the very initial transmission. Considering the workload and TU limitation, this can be included as part of normative work in Rel-20 if Device 2b is sufficient for support of outdoor use cases, i.e., no additional workload for study of Device C.

In our opinion, two different ways are feasible for implementing DO-A traffic. The first one is to employ isolated time-frequency resource for DO-A traffic, and the second one is to reserve part of time-frequency resources triggered by R2D message, e.g., R2D trigger message for DO-A traffic. Although the second approach is somewhat opportunistic, but the standardization impact, especially on RAN1 is limited.

Proposal 3: DO-A traffic type can be included as prat of A-IoT normative work in Rel-20 if outdoor use cases are based on Device 2b only.

## Multi-device CFRA

In the Rel-19, only one-device CFRA procedure is agreed to be included. However, the efficiency of resource usage for such implementation is quite low, considering FDM and TDM have been already agreed for D2R transmission, and simultaneously no parallel service request can be performed by Readers. As a result, we suggest to include multi-device CFRA as part of A-IoT normative work in Rel-20.

Proposal 4: Multi-device CFRA procedure can be included as part of A-IoT normative work in Rel-20.

## Multicast for commanding procedure

In the Rel-19, for sending command towards each A-IoT device, at least one MAC SDU attached with individual AS ID and the related command is needed for each A-IoT device in R2D direction. Considering the large amount of A-IoT device may be involved, the efficiency of such unicast commanding procedure is low. As a matter of fact, many A-IoT devices are attached with the same type of equipment, e.g., sensor, so there is no difference between the command sent to such A-IoT devices. So, specifying a new multicast procedure for commanding procedure is suggested to be included as part of A-IoT normative work in Rel-20.

Proposal 5: Specify a new multicast procedure for command use case can be included as part of A-IoT normative work in Rel-20.

## D2R transmission duplication to improve reliability

In the Rel-19, the efficiency of the A-IoT transmission is quite low, the communication link is frozen before the Reader ACKs the success transmission of D2R message and sends the next Command towards the A-IoT device or NACKS the transmission and let the A-IoT device to re-send the D2R message towards the network. As can be found, the success transmission of the D2R message matters a lot to the transmission efficiency, since it takes time for NACK/re-transmitted command to be transmitted between reader and A-IoT device. So, a feasible enhancement is to let Reader issue more than one D2R grant in one R2D message to allow D2R duplication transmission to improve the success rate of the D2R transmission.

Proposal 6: D2R transmission duplication can be included as part of A-IoT normative work in Rel-20.

## TU estimate

- RAN1:

  - D2T2 + Device 2b + unavailability direction 2  TU = 1.5

  - D4T1  TU = 1.5 (if only Device 2b) ~ [2 (if also Device C)]

  - [DO-A traffic  TU = 0.5]

  - Overall, TU = 3.5

- RAN2:

  - D2T2  TU = 1

  - DO-A traffic and higher-layer enhancements  TU = 1~1.5

  - Overall, TU = 2~2.5

- RAN3:

  - D2T2  TU = 0.5~1

# Conclusions

Overall, our proposal for the scope of A-IoT item in Rel-20 is as followed.

Overall proposal for A-IoT in Rel-20:

- Deployment scenario D2T2-C, Device 2b and device (un)availability direction 2 should be the main focus for A-IoT normative work in Rel-20.

  - The necessity and additional benefit(s) from supporting D1T1-A1/2 could be further discussed and justified for Rel-20 in RAN.

- For outdoor use cases,

  - A study phase is at least needed to evaluate whether Device 2b is able to satisfy coverage and coexistence requirement of outdoor scenario, if not, Device C should be considered along with study on the device architecture and other energy sources.

- DO-A traffic type can be included as prat of A-IoT normative work in Rel-20 if outdoor use case is sorely based on Device 2b only.

- Other higher layer enhancements

  - Multi-device CFRA procedure

  - New multicast procedure for command use case

  - D2R transmission duplication

- TU estimates

  - RAN1:

    - D2T2 + Device 2b + unavailability direction 2  TU = 1.5

    - D4T1  TU = 1.5 (if only Device 2b) ~ [2 (if also Device C)]

    - [DO-A traffic  TU = 0.5]

    - Overall, TU = 3.5

  - RAN2:

    - D2T2  TU = 1

    - DO-A traffic and higher-layer enhancements  TU = 1~1.5

    - Overall, TU = 2~2.5

  - RAN3:

    - D2T2  TU = 0.5~1

# References

[1] RP-243280 “Way forward for Ambient IoT for REL-19 and REL-20”, RAN#106

[2] RP-250802 “Moderator's summary for RAN1 led REL-20 topics”, RAN#107

[3] RP-243326 “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, RAN#106

[4] TR 38.769 “Study on solutions for ambient IoT (Internet of Things)”, V2.0.1 (2024-12)

[5] TR 38.848 “Study on Ambient IoT (Internet of Things) in RAN”, V1.0.0 (2023-09)

[6] RP-242801 “Discussion on Rel-19 Ambient IoT potential work item scope”, OPPO, RAN#106