3GPP TSG RAN Meeting #108				RP-250974

Prague, Czech Republic, June 9-13, 2025

Source:	OPPO

Title:	Discussion on 6G Scenarios and Requirements

Agenda Item:	16.1

Document for:	Discussion

Introduction

The study item on 6G scenarios and requirements was approved in RAN#107. The study (SID in [1]) covers two parts of scope:

- 3GPP inputs to ITU-R IMT-2030 TPR;

- 3GPP requirements for 6G Radio.

In this paper, our views on IMT-2030 TPR and 3GPP requirements for 6G Radio are presented.

Discussions on IMT-2030 (6G) TPR

Candidate sets of TPRs

In the ITU-R Recommendation “Framework and overall objectives of the future development of IMT for 2030 and beyond” [2], the in IMT-2030 TPR includes 9 enhanced capabilities and 6 enhanced capabilities:

Enhanced Capabilities for IMT-2030:

- Peak data rate

- User experience data rate

- Spectrum efficiency

- Area traffic capacity

- Connection density

- Mobility

- Latency

- Reliability

- Security and resilience

New Capabilities for IMT-2030:

- Positioning

- Interoperability

- Sustainability

- AI-related capabilities

- Sensing-related capabilities

- Coverage

We think the 15 capability items can be the baseline for IMT-2030 TPR study. Among them, the 9 enhanced capability items are generally inherited from IMT-2020 (5G) TPR and their definition are clear. However, for some KPI items are defined only for extreme use cases. For example, the latency and reliability in 5G are defined assuming zero payload in packet. But the extreme latency (e.g. 1ms) and reliability (e.g. 99.999%) does not make much sense for service provision in real market. Even for URLLC/IIoT services, a moderate size of packet needs to be transferred.

Hence, to evaluate the performance and the real use experience of a service requiring other KPIs than data rate, a joint KPI combining latency, reliability and capacity should be adopted. For example, in Study on XR (Extended Reality) Evaluations for NR (TR 38.838 [4]), the joint KPI is used to evaluate performance of XR system. For example, 11 XR users/cell can be supported with 30Mbps data rate, 10ms PDB (Packet Delay Budget) and 1% PER (Packet Error Rate). 16 XR users/cell can be supported with 30Mbps data rate, 10ms PDB and 5% PER.

We suggest to adopt the evaluation methodology in TR 38.838 [4] as reference, and define a joint KPI combining data rate, PDB, PER and capacity for typical services, as shown in Table 1.

Table 1: Example of joint KPI requirements for different services

|  | Data rate (Mbps) | PDB (ms) | PER (%) | Capacity (UEs/cell) |
| --- | --- | --- | --- | --- |
| Service 1 |  |  |  |  |
| Service 2 |  |  |  |  |
| Service 3 |  |  |  |  |
| …… |  |  |  |  |

For the first release of 6G, the primarily essential application scenarios are Immersive and Massive communication. For massive communication, the joint KPI can be with low data rate, large PDB, basic PER and large number of UEs/cell. We can also define the joint KPI for HRLLC. But from our perspective, the HRLLC can be supported for 6G Day1 by reusing 5G URLLC standards.

Proposal 1: Consider joint KPI combining data rate, PDB (Packet Delay Budget), PER (Packet Error Rate) and capacity (number of UEs/cell).

- Adopt XR evaluation methodology in TR 38.838 as reference.

- At least used for Immersive and Massive communication application scenarios.

KPI values for the communication TPRs

In the first conference call after RAN#107, companies discussed part of KPI values with summary in [3]

Table 1: Discussion status for IMT-2030 TPR values and OPPO’s view

| ITU IMT-2030 TPR |  | IMT-2020 Value | Values proposed in RAN#107 | The agreements in CC#1 for IMT-2030 TPRs (green highlighted) or Moderator proposal | OPPO’s view |
| --- | --- | --- | --- | --- | --- |
| Peak data rate (Gbit/s) |  | DL: 20 UL: 10 | [1~10x] of IMT-2020 | Alternative 1: Not to define a requirement, e.g. Leave the value for peak data rate for the technology proponent to provide Alternative 2: Define a requirement with reasonable value | - Prefer Alternative 1. It is not a requirement from market, but only a theoretical calculation. And this is an excessive marketing which leads to unfeasible user expectation (in IMT-2020, Peak data rate has been 200x of user experienced data rate). If the intention of this item is to justify the CBW requirement, it is OK to add a CBW item, e.g. 200MHz. - If most of companies support Alternative 2, we suggest to keep the same value as IMT-2030. |
| Peak spectral efficiency (bit/s/Hz) |  | DL: 30 UL: 15 | [1~3x] of IMT-2020 | DL: [1.5-2.5x] of IMT-2020 UL: [1.5-2.5x] of IMT-2020 | DL: 2x of IMT-2020. UL: 1.5x of IMT-2020. |
| User experienced data rate (Mbit/s) |  | DL: 100 UL: 50 | DL: [1.25~10x] of IMT-2020 UL: [1.25~4x] of IMT-2020 2 contributions propose to revisit the evaluation methodology before setting values: - Bandwidth scaling of simulation results for the UL User Experienced Data Rate performance should NOT be used. | For UL: - RAN#107 agreement: For user experienced data rate, further discuss applicable deployment scenarios, and revisit the evaluation methodology before setting values for UL Proposal 1: Use system level simulation for the evaluation of UL user experienced data rate with no bandwidth scaling taking UE maximum output power into account. | Agree not to use bandwidth scaling of simulation results. DL: 2x of IMT-2020 UL: 2x of IMT-2020 |
| 5th percentile user spectral efficiency (bit/s/Hz) |  | Indoor Hotspot-eMBB: - DL: 0.3 - UL: 0.21 Dense Urban - eMBB: - DL: 0.225 - UL: 0.15 Rural-eMBB: - DL: 0.12 - UL: 0.045 | [1.5~5x] of IMT-2020 | Companies are encouraged to provide antenna assumption for different frequency ranges together with the requirements, e.g. 4GHz, 7GHz, 30GHz Applicable deployment scenarios should also be considered. | Prefer 2x of IMT-2020. But agree with 3x of IMT-2020. Antenna assumption for 3x of IMT-2020 in DL: Up to CSI-RS 256T4R@7GHz |
| Average spectral efficiency (bit/s/Hz) |  | Indoor Hotspot - eMBB: - DL: 9; - UL: 6.75 Dense Urban - eMBB: - DL: 7.8; - UL: 5.4 Rural - eMBB: - DL: 3.3; - UL: 1.6 | [1.5~5x] of IMT-2020 | [3x] of IMT-2020 | Prefer 2x of IMT-2020. But agree with 3x of IMT-2020. (feasible with AI/ML-based enhancements). |
| Area traffic capacity (Mbit/s/m²) |  | DL: 10 | [1~10x] of IMT-2020 | [5x] of IMT-2020 | Prefer 3x of IMT-2020. But agree with 5x of IMT-2020. (Needs to be justified.) |
| latency | User plane latency (ms) | eMBB: 4 URLLC: 1 | IC: [2] HRLLC: [1] | TBD | eMBB: 4 HRLLC: 1 (No actual justification that IMT-2020 values cannot support HRLLC/IIoT applications in market. Consider the further KPI improvements after the first-wave commercial deployment.) |
|  | Control plane latency (ms) | 20 | [10] | TBD | 20 (No justification of new requirement from market.) |
| Connection density (devices/km²) |  | 10⁶ | [1~100x] of IMT-2020 | [10x] of IMT-2020 | Agree with 10x of IMT-2020. |
| Reliability |  | - 10⁻⁵ | [1~100x] of IMT-2020 | [1-10-5] | 10⁻⁵ (No actual justification that IMT-2020 values cannot support HRLLC/IIoT applications in market. Consider the further KPI improvements after the first-wave commercial deployment.) |
| Mobility (bit/s/Hz @ km/h) |  | Indoor Hotspot-eMBB: 1.5 @10 km/h Dense Urban- eMBB: 1.12 @30 km/h Rural-eMBB: 0.8 @120 km/h 0.45 @500 km/h | 1x of IMT-2020~ 1000/1200km/h | Keep the velocities defined for IMT-2020. Should new velocity be added? (up to 1000/1200km/h for airplane) Should 50% spectrum efficiency be increased for the velocity defined in IMT-2020? e.g. [1.5x] of IMT-2020 | Support to add velocity of up to 1000/1200km/h for airplane. No need to improve the spectrum efficiency for the velocity defined in IMT-2020. |
| Mobility interruption time (ms) |  | 0 | 0 ms | [0 ms] | 0 ms |
| Bandwidth (MHz) |  | 100 | 200MHz or 400MHz | RAN#107 agreement: set a target value of 400MHz (same definition as IMT-2020) (same definition as IMT-2020) | OK with 400MHz |

It should be noted that these KPIs are defined to evaluate the 6G capability to provide traditional communication services, e.g., eMBB, industrial IoT, low-end IoT. The question is that to what extent the KPIs need to be improved over IMT-2020 values. Considering that 6G targets on a more successful technology on market, the performance improvements should be helpful for the success of the services. In this sense, we can categorize the KPIs into three groups:

- Group 1: The KPIs are not the key to make the 6G communication services successful.

- Group 2: The KPIs are the key to make the 6G communication services successful, but have been met by 5G system. No substantial enhancement is needed.

- Group 3: The KPIs are the key to make the 6G communication services successful, and have not well met by 5G system. Enhancement is needed.

From our perspective, the Group 1 include Peak data rate and Peak spectral efficiency. The two KPIs are only theoretical, and are not directly related to service provision capability, even for Immersive/eMBB services. In IMT-2020 (5G) TPR, DL Peak data rate = 20Gbps, UL Peak data rate = 10Gbps. The data rate is not actually needed by any 5G service. It will not be needed by any 6G service either, including XR or AIaaS application. It may be required by holographic display. But we do not see the need to provide holographic display services in mobile communications system in near future. Another consideration is that IMT-2030 may not have new spectrum with much wider bandwidth than IMT-2020. Only 800MHz spectrum are newly provided to IMT services below 7GHz in WRC-23. There is no need to significantly expand the bandwidth. Hence Peak data rate should not be the focus for 6G improvement. We see no need to increase DL/UL Peak data rate to 50~100/25~50Gbps.

We have similar observation on Peak spectral efficiency. In IMT-2020 (5G) TPR, DL Peak spectral efficiency = 30bit/s/Hz, UL Peak spectral efficiency = 15bit/s/Hz, which has been a very high value. We see no need to further increase them.

Group 2 may include Control plane latency, Reliability and Connection density. Latency and reliability are the essential performance requirements for URLLC/IIoT applications. Connection density is the essential performance requirement for massive IoT. The performance has been supported by 5G/5G-A system, e.g., 1ms air interface latency, 10-5~10-6 reliability and 106 devices/km2 connection density. Compared to the actual commercial deployment of URLLC/IIoT and massive IoT, the 5G system performance is far beyond the market needs. No actual justification that 1ms latency and 10-6 reliability cannot support HRLLC/IIoT applications in market. No actual justification that 106 devices/km2 connection density cannot support massive IoT applications in market. We suggest to firstly deploy the system capabilities in market, and consider the further KPI improvements after the first-wave commercial deployment. We suggest to design 6G low-end IoT in 1st release of 6G. But the motivation is to design a low-end IoT which share the single RAT with 6G eMBB/immersive, not to pursue a higher KPI, e.g., connection density. 6G connection density does not need to be improved over 5G.

Group 3 include User experienced data rate, Average spectral efficiency, 5%th percentile user spectral efficiency and Area traffic capacity. The KPIs are related to actual user experience in eMBB/Immersive services. And the values are below the actual market expectation, at least for 5G smartphone market. Among them the most essential performance is the 5th percentile user spectral efficiency. In 5G system, the difference between user experience in cell center and at cell edge became larger. Smartphone users feel that 5G coverage degrades over 4G coverage. However, the feasibility of applicable coverage enhancement techniques needs to be considered. The preliminary evaluation of 6G average spectrum efficiency and 5%-tile spectrum efficiency in 7GHz band is shown in Figure 1.

Figure 1: Preliminary evaluation of 6G average/5%-tile spectrum efficiency gain @7GHz

2 performance over IMT-2020 can be considered for 5%th percentile user spectral efficiency. 3 performance over IMT-2020 can be achieved with specific MIMO configuration, e.g., CSI-RS ports 256T4R@7GHz. For Average spectral efficiency, 2 performance over IMT-2020 can be considered for 7GHz band. Beyond that, the performance gain should be practically estimated considering the improvement of MIMO and AI for air interface techniques. Considering 6G new spectrum is limited, we should not expect an obvious wider bandwidth to significantly improve User experienced data rate. So, the gain mainly relies on the improvement of spectral efficiency. We suggest to consider 2 performance over 5G for them.

Proposal 2: Consider the KPI values in 5th column of Table 1 for IMT-2030 communication TPR.

KPIs for Integrated Sensing and Communication

Integrated Sensing and Communication is another area requiring new KPI system. One discussion point is which level KPIs are defined for 6G sensing. Generally, three options can be considered.

Option 1: KPIs for basic physical parameters, such as distance, velocity and angle. Similar as defined in TR22.837, accuracy and resolution of distance/velocity/angle estimation and related general requirement, Confidence level, latency, refreshing rate, missed detection under constant false alarm. Option 1 could cover some fundamental use cases but it is still restricted within these fundamental use cases. Some popular use cases recent years cannot be covered.

Option 2: KPIs for both basic and advanced physical parameters, such as accuracy of model parameters. Comparing with option 1, option 2 extends to some advanced parameters to cover some popular use cases recent years, such as motion monitoring. For motion monitoring, accuracy of model parameter, i.e. repetition rate can be defined as KPI.

Option 3: KPIs for service parameters, such as probability of rainfall. Comparing with option 1 and option 2, option 3 means that BS/UE will not only perform signal processing in physical layer but also provide service-oriented data analysis. For each service, service-oriented data analysis is very professional, which is not our advantage currently but it gives us opportunity to extend our business.

Comparing with option 1/2/3, Option 2 is a good tradeoff between extensiveness and complexity. So, option 2 can be a starting point at current stage. As sensing function and application develops, option 3 can be considered in later stage.

Proposal 3: For 6G Integrated Sensing and Communication, KPIs for both basic and advanced physical parameters can be defined.

KPIs for Energy saving

Energy efficiency was in principle defined in IMT-2020 (5G) TPR under capability of “sustainability”, without defining numerical KPIs values. It can be considered to define the KPIs for IMT-2030 (6G) TPR.

One way is to define the comparative KPI from 5G energy consumption. This approach has been employed in 5G enhancement for UE power saving and NW energy saving from Rel-16 to Rel-19. We think this approach can be used as a baseline, i.e., how many percent energy 6G NW and UE can save over 5G NW and UE.

A further way is to define the energy efficiency, e.g., consumed energy per transmitted bit (in Joule/bit). However, it needs to be further studied what service/traffic model is used for the definition. In principle, a high-rate data service has a higher energy efficiency than low-rate service, although the energy consumption of high-rate service is much higher.

Proposal 4: For 6G Energy saving, use power/energy saving percentage over 5G as the baseline KPI.

- Study the feasibility to define energy efficiency formulation.

Other 3GPP 6GR requirements

Besides the study on IMT-2030 TPR which will form the input from 3GPP to ITU-R IMT-2030 TPR, this study item will develop the study on 3GPP 6G requirements which may not be captured by IMT-2030 TPR. Some general aspects will naturally be treated in the study, i.e.

- Identify typical and practical deployment scenarios defined by attributes such as carrier frequency, inter-site distance, user density, maximum mobility speed, and other relevant factors.

- Develop 3GPP requirements for 6G Radio for improvement of existing services and for new services.

- Determine the applicability of legacy services to 6G Radio, and define radio requirements for these, as appropriate.

From our perspective, the follow aspects are also essential for the 3GPP 6G requirement study:

- Scalable 6GR design.

- Capability of 6G Massive Communications

- L1/L2 Security and privacy

- Higher-layer requirements

Scalable 6GR design

In the SID for 6G scenarios and requirements [1], several objectives addressed the requirements of a scalable 6G Radio (6GR) design:

- Ensure appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting

- Energy efficiency and energy saving: both for network and device

- …

- Target scalable and forward compatible design for diverse device types

- …

- Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs

- …

These will be challenging requirements for 6GR because following 6GR design goals seem contradictory:

- In addition to the existing high bandwidth, low latency, and high reliability data transmission capabilities of 5G NR, 6GR also need to expand ubiquitous connectivity and sensing capabilities for physical environments. This will be a mobile communication system that is far more "multifunctional" than 5G NR.

- On the other hand, the industry also expects 6GR to significantly reduce deployment and operational costs, thereby truly achieving widespread adoption in various industries.

- Finally, an important design goal of 6GR is to reduce the energy consumption of the network and terminals, and build a greener communication system.

Traditional 5G NR design cannot simultaneously meet the seemingly contradictory design goal of 6GR. The only feasible solution is to design a modular and scalable system that provides "low-cost, low-energy" basic functions and performance through a basic module, and superimposes several "full functional, high-performance" modules on top of it to meet the needs of different usage scenarios, as shown in Figure 2.

Figure 2: Scalable 6GR design

The common base of the 6GR is the basic function set, which provides the basic functions of the 6GR for all 6G usage scenarios, such as wide area coverage, basic access and control channels, low data rate transmission, and basic QoS (basic latency, reliability performance). At the same time, this basic feature set should be able to serve as an "energy-saving mode" for Immersive Communication (which can be seen as the 6G version of eMBB). Finally, this basic feature set can independently fulfil all requirements of Massive Communication (Low end IoT) usage scenario.

In addition to Massive Communication, the air interfaces for other usage scenarios are modularly scaled up from the basic functionality set to form their own full functionality sets. As the most important usage scenario in 6G, the full functionality set of Immersive Communication (IC) is scaled up from the basic feature set.

This is fundamentally different from 5G NR design. In 5G NR, the mandatory features for eMBB are mandatory for all 5G verticals. Various 5G vertical usage scenarios, including RedCap, are scaled down from eMBB, which cannot provide low-cost optimization for vertical usage scenarios. However, in the design of 6GR, only the mandatory features in basic functionality set is mandatory for all usage scenarios and all device types. The mandatory features in a full functionality set of a usage scenario is mandatory only for this usage scenario and corresponding device types. All usage scenarios (incl. Immersive Communication) are scaled up from the basic functionality set.

Besides Massive Communication and Immersive Communication, other vertical usage scenarios can be scaled up from basic functionality set to form their own full functionality sets, which can be categorized into two types:

- Type 1: The full functionality set of some verticals can be directly scaled up from the basic functionality set, such as 6G Sensing and Ubiquitous Connectivity. It may be difficult for these verticals to share all the air interface design of Immersive Communication due to the significantly different requirements. For example, some PHY signal/channel for 6G sensing system may need to be optimized for sensing performance. Some PHY signal/channel for 6G NTN system may need to be optimized for satellite communication.

- Type 2: The full functionality set of some verticals can be directly scaled up from the basic feature set and Immersive Communication full functionality set, such as 6G HRLLC, 6G AI. They can scaled up from the Immersive Communication full functionality set, as their air interface design has similar requirements to Immersive Communication air interface (e.g. higher data rate). The full functionality set of 6G HRLLC can provide low-latency and high-reliability functionalities beyond the wideband functionalities of Immersive Communication. The full functionality set of 6G AI can provide additional functionalities to optimize the transmission of AI traffics (such as models, datasets, AI semantics) between AI agents beyond the wideband functionalities of Immersive Communication.

Proposal 5: Study requirements of a basic functionality set of 6GR which provides:

- Basic functionalities for all usage scenarios, e.g., wide area coverage, basic access and control channels, low data rate transmission, and basic QoS (basic latency, reliability performance).

- Energy-efficient mode of Immersive communication usage scenario.

- Full functionalities for Massive Communication usage scenario.

Note: The mandatory features in basic functionality set is mandatory for all usage scenarios and all device types.

Proposal 6: Study requirements of full functionality set of each usage scenario, which is scaled up from basic functionality set.

- Note: The mandatory features in a full functionality set of a usage scenario is mandatory only for this usage scenario and corresponding device types.

Capability of 6G Massive Communication

It is essential to define requirements of 6G Massive Communication because it also serves as the baseline of 6G basic functionality set. The basic functionality set provides all the Massive Communication needs, such as wide area coverage, basic access and control channels, low data rate transmission, basic QoS (basic latency, reliability performance). Meanwhile the basic functionality set is also essential to minimize the complexity (including minimizing the adoption of multiple options for the same functionality), and lower the cost and energy consumption for 6G Immersive Communication and other verticals

Hence, meanwhile meeting the requirements of low-end IoT services, another two factors should be considered for defining the requirements of 6G Massive Communication include:

- The upper limit of the 6G Massive Communication functionality set should support low-cost IoT services and "energy-efficient mode" of Immersive Communication. If the capability of Massive Communication is set too high, no satisfactory energy saving gain can be achieved with the basic functionality set.

- The lower limit of the 6G Massive Communication functionality set should enable the unified 6GR design ranging from basic functionality set to Immersive Communication full functionality set. If the lower limit is too low (such as bandwidth or capability), it will be difficult to share the basic design (e.g. waveform, modulation, channel coding, synchronization signal, random access channel, and basic uplink and downlink control channel formats) with Immersive Communication full functionality set. Typical examples are Narrow Band IoT (NB-IoT) and Ambient IoT, which have to adopt completely different air interface designs from LTE and 5G NR.

Based on the above considerations, the definition of 6G Massive Communication can be depicted as in Figure 3. From the lesson of NB-IoT, the lower limit of 6G Massive Communication should be significantly higher than NB-IoT, i.e., with typical data rate higher than 1Mbps and bandwidth higher than 1MHz. On the other hand, considering the typical traffic model of low-end IoT and the requirements of energy saving for Immersive Communication, the upper limit of 6G Massive Communication should be lower than RedCap, i.e., data rate/bandwidth lower than 10Mbps/5MHz. Therefore, a suitable bandwidth for 6G Massive Communication can be 3-5MHz.

Figure 3: Capability of 6G Massive Communication/6GR basic functionality set

Proposal 7: The capability of 6G Massive Communication/6GR basic functionality set is:

- To support wide area coverage, basic access and control channels, low data rate transmission, and basic QoS (basic latency, reliability performance).

- Higher than NB-IoT and lower than RedCap, e.g., with 3-5MHz bandwidth.

L1/L2 Security and privacy

Targeted as a new generation of mobile communication and possibly also an information system, 6G may have certain goals to satisfy the evolving social trends and needs of end users. A few examples are listed:

In 6G, sensing is one of the important use cases. By the nature of sensing, sensing data are usually extracted from sensing signals, e.g., sensing reference signal, at the physical layer. An attacker can extract sensing data from periodically transmitted sensing signals through some physical layer methods. As descripted in 4.3 and 5.2.4 of TS 22.137 [5] sensing data security protection is needed, and privacy consideration is necessary for human related sensing scenarios.

Scenarios that may have privacy risk for sensing signal mainly include human-related scenarios such as Use Case on Environment Object Reconstruction, human respiration rate, and etc. [2]. In the human respiration rate scenario, by eavesdropping micro doppler effect by ISAC caused by chest rise/fall during sleeping, an attacker may be able to predict the health status of the target person.

Furthermore, there may be other potential vulnerabilities on the physical layers that 5G has not yet explored, for example, attackers can spoof or eavesdrop scheduling information or CSI reporting information (e.g., RSRP) over the PUCCH and/or during PUCCH activation/deactivation [3], leading to tracking of UE, waste of UE resources and possibly DoS attacks. Such vulnerabilities are also part of the increase interest in the working group to study on protection at the physical layers.

These above examples are just scratching the surface of what is needed to be analyzed. Combination of the above examples with other additional unprotected parameters may prove more vulnerable to privacy exposure and attacks. Therefore, it is beneficial that RAN can include physical layer security considerations and incorporate security requirements/mechanisms at the physical layer.

Higher Layer Requirement

One key higher-layer related aspect of the SID is to steer the RAN WGs for the migration part.

Define a time plan and steer work as appropriate for the RAN WGs during the 6G WG SI to deliver high-level decisions at least on the following areas:

o	Overall high-level aspects of 5G to 6G migration

For the migration part, the key aspect is whether to proceed with the EN-DC like architecture as we did in Day-1 for 5G system. In our view, the widely spread concern on the EN-DC like architecture is not on the technical feasibility, but rather on the deployment complexity and harmful effect on market segmentation. While the former one normally relies on WG level study, yet the latter one more relies on evaluation from business perspective and thus more relies on plenary level study. From this perspective, it is important that RAN study provides a clear guidance and avoid unnecessary RAN WG level technical work / discussion.

Regarding the time plan, it would be helpful to set a intermediate time point, in order to leave some time to WG work, if any technical issue(s) are identified for the integration of 5G/6G (more on the CN-level integration rather than EN-DC like RAN level integration). E.g., for a UE working simultaneous in 5G and 6G mode, whether any simplification is feasible to reduce the overhead and optimize the power efficiency from AS layer perspective.

Proposal 8: RAN Plenary set a check point (e.g., RAN#112) to transfer the evaluation of 5G to 6G migration mechanism, from RAN Plenary level study to RAN WG level study, and RAN Plenary aims to reach a clear guidance on the 5G to 6G migration mechanism before/upon the checking point, e.g., to avoid EN-DC like RAN level integration while rely on WG to further evaluate the impact (if any) of CN level integration.

Another key higher-layer related aspect of the SID is on UE capability signaling design.

Develop 3GPP requirements for 6G Radio for these practical deployment scenarios to ensure substantial gains in all relevant bands: overall performance, user experience, TCO reduction including at least:

o	Ensure appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting

[…]

o	Target scalable and forward compatible design for diverse device types

As stated in clause 3.2, when designing the UE capability, it is preferred to be based on the different communication types, like massive communication, Immersive Communication and so on. The next step is how to reflect that in the capability signaling design, which is critical not only to avoid excessive UE capability reporting, but also to simplify the chipset/device development.For this issue, the key aspect is to introduce additional dimension into UE capability definition, i.e., the device-type and/or use case. Stage-3 wise, it is similar to add pre-requisite or add conditionally mandatory capabilities. It helps to group device-type (hand-held, FWA) / use case (AI, NTN and etc.) specific features and simplify the mandatory / optional debate to a more concrete scenario. Please note that the legacy 4G UE category definition is not applicable here, since the data rate calculation has been generalized in 5G system, and the device-type dimension can already introduce a data rate restriction / limit (like what we did for 5G Redcap).

Proposal 9: RAN Plenary study the device type / use case / communication type that may affect the UE capability design.

Conclusion

The proposals for IMT-2030 TPR are listed below:

Proposal 1: Consider joint KPI combining data rate, PDB (Packet Delay Budget), PER (Packet Error Rate) and capacity (number of UEs/cell).

- Adopt XR evaluation methodology in TR 38.838 as reference.

- At least used for Immersive and Massive communication application scenarios.

Proposal 2: Consider the KPI values in 5th column of Table 1 for IMT-2030 communication TPR.

Table 1: Discussion status for IMT-2030 TPR values and OPPO’s view

| ITU IMT-2030 TPR |  | IMT-2020 Value | Values proposed in RAN#107 | The agreements in CC#1 for IMT-2030 TPRs (green highlighted) or Moderator proposal | OPPO’s view |
| --- | --- | --- | --- | --- | --- |
| Peak data rate (Gbit/s) |  | DL: 20 UL: 10 | [1~10x] of IMT-2020 | Alternative 1: Not to define a requirement, e.g. Leave the value for peak data rate for the technology proponent to provide Alternative 2: Define a requirement with reasonable value | - Prefer Alternative 1. It is not a requirement from market, but only a theoretical calculation. And this is an excessive marketing which leads to unfeasible user expectation (in IMT-2020, Peak data rate has been 200x of user experienced data rate). If the intention of this item is to justify the CBW requirement, it is OK to add a CBW item, e.g. 200MHz. - If most of companies support Alternative 2, we suggest to keep the same value as IMT-2030. |
| Peak spectral efficiency (bit/s/Hz) |  | DL: 30 UL: 15 | [1~3x] of IMT-2020 | DL: [1.5-2.5x] of IMT-2020 UL: [1.5-2.5x] of IMT-2020 | DL: 2x of IMT-2020. UL: 1.5x of IMT-2020. |
| User experienced data rate (Mbit/s) |  | DL: 100 UL: 50 | DL: [1.25~10x] of IMT-2020 UL: [1.25~4x] of IMT-2020 2 contributions propose to revisit the evaluation methodology before setting values: - Bandwidth scaling of simulation results for the UL User Experienced Data Rate performance should NOT be used. | For UL: - RAN#107 agreement: For user experienced data rate, further discuss applicable deployment scenarios, and revisit the evaluation methodology before setting values for UL Proposal 1: Use system level simulation for the evaluation of UL user experienced data rate with no bandwidth scaling taking UE maximum output power into account. | Agree not to use bandwidth scaling of simulation results. DL: 2x of IMT-2020 UL: 2x of IMT-2020 |
| 5th percentile user spectral efficiency (bit/s/Hz) |  | Indoor Hotspot-eMBB: - DL: 0.3 - UL: 0.21 Dense Urban - eMBB: - DL: 0.225 - UL: 0.15 Rural-eMBB: - DL: 0.12 - UL: 0.045 | [1.5~5x] of IMT-2020 | Companies are encouraged to provide antenna assumption for different frequency ranges together with the requirements, e.g. 4GHz, 7GHz, 30GHz Applicable deployment scenarios should also be considered. | Prefer 2x of IMT-2020. But agree with 3x of IMT-2020. Antenna assumption for 3x of IMT-2020 in DL: Up to CSI-RS 256T4R@7GHz |
| Average spectral efficiency (bit/s/Hz) |  | Indoor Hotspot - eMBB: - DL: 9; - UL: 6.75 Dense Urban - eMBB: - DL: 7.8; - UL: 5.4 Rural - eMBB: - DL: 3.3; - UL: 1.6 | [1.5~5x] of IMT-2020 | [3x] of IMT-2020 | Prefer 2x of IMT-2020. But agree with 3x of IMT-2020. (feasible with AI/ML-based enhancements). |
| Area traffic capacity (Mbit/s/m²) |  | DL: 10 | [1~10x] of IMT-2020 | [5x] of IMT-2020 | Prefer 3x of IMT-2020. But agree with 5x of IMT-2020. (Needs to be justified.) |
| latency | User plane latency (ms) | eMBB: 4 URLLC: 1 | IC: [2] HRLLC: [1] | TBD | eMBB: 4 HRLLC: 1 (No actual justification that IMT-2020 values cannot support HRLLC/IIoT applications in market. Consider the further KPI improvements after the first-wave commercial deployment.) |
|  | Control plane latency (ms) | 20 | [10] | TBD | 20 (No justification of new requirement from market.) |
| Connection density (devices/km²) |  | 10⁶ | [1~100x] of IMT-2020 | [10x] of IMT-2020 | Agree with 10x of IMT-2020. |
| Reliability |  | - 10⁻⁵ | [1~100x] of IMT-2020 | [1-10-5] | 10⁻⁵ (No actual justification that IMT-2020 values cannot support HRLLC/IIoT applications in market. Consider the further KPI improvements after the first-wave commercial deployment.) |
| Mobility (bit/s/Hz @ km/h) |  | Indoor Hotspot-eMBB: 1.5 @10 km/h Dense Urban- eMBB: 1.12 @30 km/h Rural-eMBB: 0.8 @120 km/h 0.45 @500 km/h | 1x of IMT-2020~ 1000/1200km/h | Keep the velocities defined for IMT-2020. Should new velocity be added? (up to 1000/1200km/h for airplane) Should 50% spectrum efficiency be increased for the velocity defined in IMT-2020? e.g. [1.5x] of IMT-2020 | Support to add velocity of up to 1000/1200km/h for airplane. No need to improve the spectrum efficiency for the velocity defined in IMT-2020. |
| Mobility interruption time (ms) |  | 0 | 0 ms | [0 ms] | 0 ms |
| Bandwidth (MHz) |  | 100 | 200MHz or 400MHz | RAN#107 agreement: set a target value of 400MHz (same definition as IMT-2020) (same definition as IMT-2020) | OK with 400MHz |

Proposal 3: For 6G Integrated Sensing and Communication, KPIs for both basic and advanced physical parameters can be defined.

Proposal 4: For 6G Energy saving, use power/energy saving percentage over 5G as the baseline KPI.

- Study the feasibility to define energy efficiency formulation.

The proposals for other 3GPP 6GR requirements are listed below:

Proposal 5: Study requirements of a basic functionality set of 6GR which provides:

- Basic functionalities for all usage scenarios, e.g., wide area coverage, basic access and control channels, low data rate transmission, and basic QoS (basic latency, reliability performance).

- Energy-efficient mode of Immersive communication usage scenario.

- Full functionalities for Massive Communication usage scenario.

Note: The mandatory features in basic functionality set is mandatory for all usage scenarios and all device types.

Proposal 6: Study requirements of full functionality set of each usage scenario, which is scaled up from basic functionality set.

- Note: The mandatory features in a full functionality set of a usage scenario is mandatory only for this usage scenario and corresponding device types.

Proposal 7: The capability of 6G Massive Communication/6GR basic functionality set is:

- To support wide area coverage, basic access and control channels, low data rate transmission, and basic QoS (basic latency, reliability performance).

- Higher than NB-IoT and lower than RedCap, e.g., with 3-5MHz bandwidth.

Proposal 8: RAN Plenary set a check point (e.g., RAN#112) to transfer the evaluation of 5G to 6G migration mechanism, from RAN Plenary level study to RAN WG level study, and RAN Plenary aims to reach a clear guidance on the 5G to 6G migration mechanism before/upon the checking point, e.g., to avoid EN-DC like RAN level integration while rely on WG to further evaluate the impact (if any) of CN level integration.

Proposal 9: RAN Plenary study the device type / use case / communication type that may affect the UE capability design.

References

- RP-250810, Revised SID: Study on 6G Scenarios and requirements, CMCC, Verizon, NTT DOCOMO, INC., Deutsche Telekom, 3GPP TSG RAN Meeting #107, Incheon, Korea, March 12-14, 2025.

- Recommendation ITU-R M.2160-0 (11/2023), Framework and overall objectives of the future development of IMT for 2030 and beyond.

- RT-250024, Moderator's summary for IMT-2030 TPRs in CC#1, Moderator (CMCC), CC#1 for IMT-2030 TPR discussion, April 29, 2025, 14:00-16:00 CEST

- TR 38.838, Study on XR (Extended Reality) Evaluations for NR (Release 17), V17.0.0 (2021-12)

- TS 22.137, Integrated Sensing and Communication (Release 19), V19.1.0 (2024-03)