3GPP TSG RAN#108 Meeting 	RP-251001

Prague, Czech Republic, June 9-13th, 2025

Title:	NTN related views on 6G RAN WG SI

Source: 	Thales, TNO, Novamint, Airbus, ESA, Sateliot, SES, Gatehouse Satcom, Eutelsat

Group, Iridium

Type:	discussion

Document for:	discussion

Agenda Item:	16.2	New RAN WG SI on 6G

# Introduction

This document proposes a number of capabilities for the 6G radio access technology (RAT) to support Non-Terrestrial Networks to be taken into account as part of the 6G RAN WG SI.

# Discussion

In the tables below, are listed candidate capabilities to be studied as part of relevant RAN WG led study items. We distinguish between

- NTN capabilities to be studied in priority as part of the Rel-20 (impacting low layers of the physical layer)

- NTN capabilities to be studied as part of Rel-21 or beyond (software impacts)

Table 1: 6G radio access technology capabilities for the support of NTN to be studied in priority as part of the Rel-20

| Capability | Justification | Requirements | Impacted RAN WGs |
| --- | --- | --- | --- |
| NTN resiliency to GNSS service | Given that the GNSS service may be unavailable or degraded, NTN (GSO and NGSO, including VLEO) should be designed to operate independently of GNSS. | The 6G RAT shall be able to accommodate the extended and variable Doppler and latency associated to NTN deployment scenarios. | RAN1, 2 |
| Satellite PAPR requirements for NTN downlink transmission | PAPR (peak-to-average power ratio) is a very important metric when it comes to operate as efficiently as possible a transmission equipment. In Very High Throughput Satellite (VHTS) payload, it is common to have several carriers amplified simultaneously by the same amplifier. PAPR increases drastically compared to mono-carrier scenario | PAPR challenges/requirements to be overcome in satellite payload designs in particular when OFDM variants are considered | RAN1 |
| Flexible frame structure | Unlike the TN channel, the NTN propagation channel is mainly characterized with low multi path conditions and low delay spread typically measuring in the few tens of nanoseconds | The 6G RAT shall be designed to support a frame structure optimized to the NTN channel. | RAN1 |
| Extended coverage | Provide service in light indoor/in vehicle environment | The 6G RAT shall be able to support a higher Coupling Loss at the expense of the service rate (i.e. same as NB-IoT’s MCL) | RAN1, 2 |
| Flexible duplex mode support at User Equipment level | Most satellite service allocated frequency bands are paired while some are unpaired bands. Half Duplexing at the UE can reduce antenna size. | The 6G RAT shall be designed to operate in FDD, TDD or HD-FDD via NTN. | RAN1, 2 |
| Support of HD-FDD at Network side | Although the duplexing mode is FDD at UE level, HD FDD operation at network side would be beneficial to cope with satellite payload design constraints. | The 6G RAT shall be designed to optionally support HD-FDD Satellite Payload | RAN1, 2 |
| Mitigate specific NTN Propagation impairments | In specific usage conditions (on board helicopter, car in a forest), the UE may experiences regular deep fades of tens of ms duration | The 6G RAT shall be designed to implement time diversity over several time frames | RAN1, 2 |
| Operation at low error rate | Due to the long propagation delay, HARQ causes latency which degrades the QoS | The 6G RAT shall be designed to achieve a 10-6 BLER without HARQ | RAN1, 2 |
| Massive messaging capability | Support a high number of terminals willing to transmit quasi simultaneously low bandwidth services (i.e. messaging) within a beam | The 6G RAT shall be designed to provide an efficient grant free access protocol | RAN1, 2 |
| Extension of satellite foot print | Limited RF power on board may prevent to illuminate all beams generated by the satellite at a given time (ratio can be as low as 1%) | The 6G RAT shall be designed to support an equivalent of the “SSB periodicity” which can be extended up to 320 ms | RAN1, 2 |
| Positioning Navigation and Timing | Integrating GNSS into IoT devices operating over satellite access may significantly impacts battery consumption, especially in battery-powered UEs requiring frequent positioning fixes/updates. Wide range of sectors, e.g. transportation, critical infrastructure (e.g. 6G) and regulatory services (e.g. emergency services) are fully dependent on GNSS to provide PNT services, impacting their resilience. | The 6G RAT shall provide means for the UE to determine its location independently of non-3GPP positioning technologies (e.g. GNSS) | RAN1, 2 |
| Enhanced network verified UE location service | The current (Rel-18 defined) network verified UE location scheme may not be able to determine reliably the UE location with an accuracy of few kms with large beam size | The 6G RAT shall provide means for the network to determine the location of a UE without GNSS service | RAN1, 2 |

Table 2: 6G radio access technology capabilities for the support of NTN to be studied as part of Rel-21 or beyond

| Capability | Justification | Requirements | Impacted RAN WGs |
| --- | --- | --- | --- |
| Quasi seamless TN/NTN mobility in connected mode | The TN coverage may be patchy and NTN can be used to bridge two separate TN coverage areas | The 6G RAT shall be designed to support mobility in connected mode between NTN/TN without any packet loss while addressing the difference in RTD and QoS | RAN2, 3 |
| 6G NTN coexistence with IoT_NTN or NR-NTN in same beam | Need to support a smooth migration path to 6G for space segment designed for IoT-NTN or NR-NTN | It shall be possible to deploy/operate NTN capable 6G RAT and NTN capable 4G/5G RAT in adjacent channels within the same beam | RAN1, 4 |
| Multi connectivity/carrier aggregation | Provide high speed and/or highly reliable broadband service | The 6G RAT shall support multi connectivity/carrier aggregation between two satellite access with different radio link availability: e.g. one operating in high frequency band (> 10 GHz) and another operating in low frequency band (< 7 .125 GHz) | RAN2, 3 |
| RAT energy consumption determination for one-to-many service | When there are several RAT (e.g. TN and NTN), the energy consumption of a one-to-many service may be considered to select a RAT for the delivery of the service | The 6G RAT shall provide means to determine the energy consumption of a one-to-many service | RAN1, 2, 3 |
| Distributed MIMO across several satellites | To be able to increase the throughput for a UE | Support MIMO across multiple TRP on board different satellites | RAN1, 2 |

# Conclusion and Way forward

Proposal 1: RAN to study in priority the following NTN capabilities for the 6G RAT as part of the Rel-20

- NTN resiliency to GNSS service

- PAPR requirements for NTN downlink transmission

- Flexible frame structure

- Extended coverage

- Flexible duplex mode support at UE level

- Support of HD-FDD at Network side

- Mitigate specific NTN Propagation impairments

- Operation at low error rate

- Massive messaging capability

- Extension of satellite footprint

- Positioning Navigation and Timing

- Enhanced network verified UE location service

Proposal 2: RAN to study in priority the following NTN capabilities for the 6G RAT as part of the Rel-21 or beyond

- Quasi seamless TN/NTN mobility in connected mode

- 6G NTN coexistence with IoT-NTN or NR-NTN in same beam

- Multi connectivity/carrier aggregation

- RAT energy consumption determination for one-to-many service

- Distributed MIMO across several satellites

END