3GPP TSG RAN meeting #108	RP-251002

Prague, Czech Republic, 9 – 13 June 2025

## Status Report to TSG

Agenda item:			9.3.2.3

| WI / SI Name | XR (eXtended Reality) for NR Phase 3 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_XR_Ph3 |  |  |  |  |
| Unique ID | 1020098 |  |  |  |  |
| TSG TDoc of latest approved WI/SI description (if any) | RP-250107 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: N/A | Core part: 2025/09 | Performance part: 2026/03 | Testing part: N/A |  |
| Overall Completion level | Study Item: N/A | Core part: 90% | Performance Part: 0% | Testing part: N/A |  |

Source:

| Leading WG |  | RAN2 |
| --- | --- | --- |
| Rapporteur | Name | Benoist Sébire |
|  | Company | Nokia |
|  | Email | benoist.sebire@nokia.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

## 2.1	RAN1

### 2.1.1	Discussions and Agreements

#### 2.1.1.1	RAN1#120bis

XR was not handled in RAN1#120bis.

#### 2.1.1.2	RAN1#121

During RAN1#121 following was agreements were made:

Valid downlink slot for CSI reference resource definition is only determined based the configured measurement gap pattern and not affected by dynamic measurement gap occasion cancellation.

Adopt following TP to draft CR of TS38.213 in Section 10.6:

## 10.6 Indication for cancelation of RRM measurement gaps An XYZ measurement gap cancellation field in a DCI format [5, TS 38.212] provided by a PDCCH reception is associated to an earliest RRM measurement gap occasion that starts at least 3 msec or 5 msec, as determined by a reported UE capability, after the end of the PDCCH reception. The indication by the measurement gap cancellation field is applied to the associated RRM measurement gap occasion on all cells in the applicable range of the measurement gap occasion that include the one or more scheduled cells associated with the DCI format [10, TS 38.133] as described below. A UE can be indicated by a value ‘1’ of an XYZ measurement gap cancellation field in a DCI format [5, TS 38.212] provided by a PDCCH reception to cancel an earliest associated RRM measurement gap occasion where the UE would perform RRM measurements on all cells in the applicable range of the measurement gap occasion that include the one or more scheduled cells associated with the DCI format [10, TS 38.133]. The earliest RRM measurement gap occasion is at least 3 msec or 5 msec, as determined by a reported UE capability, after the end of the PDCCH reception. If a UE indicates “ValueOption 1” for FG 64-1-ABC, the UE ignores a value ‘0’ of the XYZ measurement gap cancellation field in a DCI format. If a UE indicates “ValueOption 2” for FG 64-1-ABC, a value ‘0’ of the XYZ measurement gap cancellation field in a DCI format provided by a PDCCH reception indicates to the UE that the UE behavior for the earliest associated RRM measurement gap occasion after the PDCCH reception where the UE would perform RRM measurements on cells in a same frequency band as the one or more scheduled cells associated with the DCI format is as described in [10, TS 38.133]. When the UE detects more than one DCI formats that include the XYZmeasurement gap cancellation field and are associated with a same earliest associated RRM measurement gap occasion where the UE would perform RRM measurements on cells in a same frequency band as the one or more scheduled cells associated with the more than one DCI formats, the UE applies the indication provided by the XYZmeasurement gap cancellation field of a last DCI format, from the more than one DCI formats, that is in a PDCCH reception that ends at least 3 msec or 5 msec, as determined by a reported UE capability, before the start of that earliest RRM measurement gap occasion.

Note: Above TP is not intended to impact future discussions related to UE features for XR.

Also following conclusions were made in RAN1#121:

-	RAN1 does not discuss the measurement gap occasion cancellation for concurrent measurement gaps further unless triggered by RAN4.

-	UE does not expect different values in ‘Measurement gap cancellation’ fields in DCI formats that are associated with the same measurement gap occasion and are received in same PDCCH monitoring occasion.

### 2.1.2	Main Open issues

No major open issues identified, CRs brought for approval to RAN#108.

## 2.2	RAN2

### 2.2.1	Discussions and Agreements

#### 2.2.1.1	RAN2#129bis

Agreements related to PDCP CR:

1.	In PDCP specifications, use “PDU Set remaining time” to describe the shortest remaining time till discardTimer expiry among the PDCP SDUs belonging to the PDU Set.

2.	Keep the text “The dsr-ReportingThresholds configured for the PDCP entity are ordered in ascending order.” in the RRC specification.

3.	Specify delay-reporting indication in the PDCP specification.

4.	Do not specify explicitly an update of delay-reporting indication in the PDCP. FFS (when discussing the CR) whether some explanation/note can be added to clarify that the UE should ensure the indicated data volume is up to date.

Agreements related to RRC CR:

1.	Introduce RRC configuration to enable/disable the inclusion of non-delay critical data ahead of delay critical data in the buffer size calculation for DSR

2.	The maximum number of entries in the dsr-ReportingThresList is 4.

3.	For the available data rate query MAC CE, confirm on the following:

•	Introduce a prohibit timer for the UL transmission of the data rate query MAC CE

•	Enable/disable the rate query MAC CE by the presence of the prohibit timer in the RRC configuration

•	FFS the details of prohibit timer, e.g. granularity.

Agreements on UE Capabilities:

1.	RAN2 assumes that we define separate UE capabilities for different XR features (as in Rel-18 XR).

2.	RAN2 to confirm that AS UE capability for multi-modality is not needed in Rel-19.

3.	An optional UE capability with signalling (e.g. lcp-PriorityAdjustment-r19) is introduced to indicate the support of dynamic logical channel priority based on delay status of buffered data. No dependency on support of delayStatusReport-r18.

4.	An optional UE capability with signalling (e.g. enhancedDelayStatusReport-r19) is introduced to indicate the support of enhanced delay status report of the buffered data associated with multiple thresholds. FFS A UE supporting this feature shall also indicate support of delayStatusReport-r18.

5.	An optional UE capability with signalling (e.g. autonomousRLC-Retx-r19) is introduced to indicate the support of autonomous RLC retransmission based on delay status. The capability does not have pre-requisites.

6.	An optional UE capability with signalling (e.g. enhancedPolling-r19) is introduced to indicate the support of enhanced polling based on delay status. The capability does not have pre-requisites.

7.	An optional UE capability with signalling (e.g. ul-RateControl-r19) is introduced to indicate the support of UL rate control MAC CE from the gNB to the UE. The capability does not have pre-requisites. FFS whether there is a separate UE capability for UL rate query.

8.	For the above Rel-19 XR UE capabilities defined by RAN2 are per UE, not FDD-TDD DIFF, not FR1-FR2 DIFF.

Agreements on LCP Enhancements:

1.	Working assumption (to be revisited next meeting): No Bj enhancement is introduced.

2.	It will only be considered provided that gains are proven and that we have a common understanding on the change that is needed in specifications

3.	We will check the impact of using additional LCP priority for SR priority determination during intra-UE prioritization in specifications and make a final decision on whether to have it based on that, i.e. it should be simple enough.

4.	It should be explicitly specified which priority, e.g. “default” or additional priority, to use for a LCH in order to determine the priority of the corresponding UL grant, i.e. highest priority used/to be used during LCP.

5.	For shared spectrum case, UE shall apply same rules for the LCH priority being used for determining the priority of a HARQ process as for determining the priority of an UL grant.

6.	Understanding in RAN2 is that we will not address any other NR-U specific issues to support XR.

7.	All UEs falls back to default priority in the 2nd round of the LCP procedure if there is no LCH-priority adjusted data left, i.e. no capability is needed (this reverts previous RAN2 agreement on this).

Agreements on DSR Enhancements:

1.	To avoid the case that there is no delay-reporting data for DSR, at least one configured reporting threshold should be no lower than the DSR triggering threshold. This means we do not have to address “empty DSR” issue, but it is FFS whether we need to capture this restriction in specifications

2.	During DSR data volume calculation the remaining time of retransmitted is not considered, i.e. it is always put either in the smallest configured or smallest reported threshold. FFS which one.

3.	Clarify RAN2#128 agreement as “the UE may also support including non-delay-reporting data ahead of delay-reporting data for buffer size calculation of Rel-19 DSR, based on the capability indication” (the exact terminology to be discussed as part of CR review)

4.	We will try to find a way to describe how the UE determines non-delay-reporting data ahead of delay-reporting data for delay-reporting data volume calculation. The aim is to have consistent UE behaviour to avoid fairness issues, but also consider different UE implementations.

Agreements on RLC Enhancements:

1.	When the t-RxDiscard expires, the expiration of t-RxDiscard triggers an SR. FFS whether this is just usual SR or some changes are needed, or if UE implementation can decide (to be discussed during CR review)

2.	For autonomous retransmission and polling, the remaining time is determined based on discardTimer at PDCP. FFS whether/what additional conditions are needed to prevent too early and/or unnecessary retransmission

3.	Autonomous retransmission is triggered for an RLC SDU (segment) provided that the original RLC SDU has been submitted to lower layers.

4.	Autonomous retransmission is not triggered if the RLC SDU (segment) is already pending for retransmission. FFS specifications impact.

Agreements on XR Rate Control:

1.	Bit rate in Rel-19 UL Rate Control MAC CE is a physical-layer bit rate.

2.	We specify a single table.

3.	As a starting point, we aim to have a table with 8 bits and exponential distribution and no multiplier. This can be subject to further agreements on MAC CE contents/design. Overhead will be considered.

4.	The working assumption is changed to agreements on rate query MAC CE.

5.	From RAN2 point of view per flow indication in MAC CE is preferred, but there are concerns on F1 impact which needs to be verified by RAN3, so RAN2 will go with per flow approach unless R3 has issues with this.

6.	FFS how QFI is indicated, e.g. with DRB ID + QFI ID.

#### 2.2.1.2	RAN2#130

Agreements related running CRs:

MAC

1.	We go with option 1, i.e. Single and multiple entry DSR MAC CE

2.	In MAC CR, FFS whether some changes are needed due to using single/multiple entry DSR MAC CE terminology

UE capabilities

3.	An optional UE capability with signalling (e.g. delayStatusReportNonDelayReportingData-r19) is introduced to indicate the support of including non-delay-reporting data ahead of delay-reporting data in the buffer size calculation for enhanced delay status report. A UE supporting this feature shall also indicate support of enhanced delay status report (enhancedDelayStatusReport-r19). The capability is per UE, not FDD-TDD DIFF, not FR1-FR2 DIFF.

4.	An optional UE capability with signalling (e.g. ul-RateQuery-r19) is introduced to indicate the support of bit rate query message (in UL Rate Control MAC CE) from the UE to the gNB. A UE supporting this feature shall also indicate support of UL rate control MAC CE (ul-RateControl-r19). The capability is per UE, not FDD-TDD DIFF, not FR1-FR2 DIFF.

RLC

5.	Use the term of “discard” for avoiding unnecessary retransmission, e.g. “stopReTxDiscardedSDU”, “t-RxDiscard”.

6.	Other specifications will be updated accordingly, e.g. the corresponding RRC parameter(s), and corresponding description in TS 38.300.

7.	The term “remaining time based retransmission” is used for autonomous retransmission in RLC.

8.	Other specifications will be updated accordingly, e.g. the corresponding RRC parameter(s), and corresponding description in TS 38.300.

Stage-2 (on PDU set handling clarification)

9.	We will update 38.300 in line with SA2 updated specs

Agreements on measurement gap skipping

1.	(RRC-05) UE reports the assistance information of recommended gap cancellation ratio when the configuration is received (and the UE has preference) or when the assistance information changes since the last report.

2.	(RRC-05) It is left to UE implementation to decide whether the recommended gap cancellation ratio changes.

3.	(RRC-06) A prohibit timer is used to limit frequent transmission of the UAI with recommended gap cancellation ratio. FFS the granularity of prohibit timer

4.	Consider RAN4 progress on the granularity for the recommended gap cancellation ratio when drafting the RRC CR

5.	(UE capability-03) RAN2 defines a per UE capability for reporting the ratio of gap occasions. And its prerequisite condition is that it supports enabling TX/RX during measurement gap scheduling restrictions by DCI.

Agreements for scheduling enhancements

1.	[MAC-01] The additional LCP priority is NOT used for SR priority determination in intra-UE prioritization.

2.	(MAC-02) When PSI-based discarding is activated for a DRB, priority adjustment is applied only if there is at least one high importance packet in the buffer has a remaining time smaller than the threshold. (MAC spec is already aligned with this agreement).

3.	(MAC-13) When pdu-SetDiscard is configured, PDU sets should be treated as a whole in the LCP procedure with adjusted priority.

4.	Confirm the working assumption “No Bj enhancement is introduced” as an agreement.

Agreements for DSR enhancements

1.	(PDCP-1, RLC-5) Both PDCP and RLC consider Control PDU and/or retransmitted data into the shortest configured reporting threshold.

2.	(PDCP-1, RLC-5) The value of the remaining time field in the enhanced DSR MAC CE is set to 0, if there are only control PDUs and/or retransmitted data to be reported for the shortest configured reporting threshold of the LCG.

3.	(PDCP-1, RLC-5) An understanding is that there will be no DSR with no data indication (i.e. indicating only volume of C-PDU and/or retransmissions for any LCG)

4.	(RRC-7) If at least one LCG is configured with dsr-ReportingThresList, any LCG configured with a triggering threshold shall be configured with at least one reporting threshold.

5.	New LCID is used for R19 DSR

6.	(MAC-03) For DC case, no further enhancements on DSR due to transmission of DSR MAC CE in the other MAC entity and this DSR MAC CE with the delay information of all the PDCP SDUs associated with the DSR.

7.	(MAC-03) In DC, no enhancements are needed when a pending DSR is cancelled because all its associated PDCP SDUs have been discarded or included in a MAC PDU.

8.	(MAC-03) An understanding is that if MAC PDU is sent in one MAC entity, then the other MAC entity will see that there is no PDCP SDU associated with DSR and will cancel the DSR.

Agreements on RLC timely retransmissions

1.	(RLC-8) We just keep the current specifications for polling triggering, i.e. no need to specify that polling shall only be triggered once per RLC SDU when its remaining time falls below a specified threshold, unless an issue is identified with this

2.	(RLC-9) No additional conditions are needed for the polling enhancement.

3.	(RLC-7) RAN2 agrees that autonomous retransmissions should trigger an increment of the RETX_COUNT.

4.	(PDCP-2) The UE shall trigger remaining-time-based RLC retransmission and polling in PDCP SDU level, no matter pdu-SetDiscard is configured or not. No specification change is needed.

5.	(RRC-3, RRC-4) Autonomous retransmission and polling is triggered only based on discardTimer, i.e. not based on discardTimerForLowImprotance

6.	(RRC-8) The remaining time thresholds (for both autonomous retransmission and polling) are configured per PDCP entity.

Agreements for unnecessary retransmission avoidance in RLC

1.	(RLC-6) There is no specification impact foreseen for RLF triggering due to RLC maximum retransmission.

2.	Working assumption: (RLC-11) No need to address window stalling issue with polling retransmission (TBC next meeting)

a.	Assumption from companies is that this should not happen due to SR triggering from Rx side, but it needs to be checked whether this will always work

3.	(UE capability-2) Define an (optional) per-UE capability with signalling for the Rx-side aspect, where an outdated SDU is abandoned based on a new RLC timer and the abandoned SDUs are positively acknowledged in an RLC status report.

4.	(UE capability-2) Define an (optional) per-UE capability with signalling for the Tx-side aspect, where the Tx side stops transmissions for an outdated SDU based on an indication from the PDCP. FFS A UE supporting this feature shall also indicate the support of Rx-side aspect.

Agreements for XR rate control

1.	(MAC-05) XR rate control MAC CE can comprise multiple QoS flows (which may belong e.g. to different LCHs) unless it becomes very complicated to specify.

2.	(MAC-04) We rule out explicit signalling of PDU session ID + QFI

3.	(MAC-04) We will down-select between, considering the max number of flows we want to be able to indicate and trying to minimize overhead:

a.	Explicit DRB ID + QFI (FFS if QFI is explicit or implicit)

b.	Implicit, e.g. index or mapping

4.	(RRC-1) The granularity of bit rate query prohibit timer is QoS flow.

a.	FFS The value of the prohibit timer is the same for all flows

5.	(MAC-08, MAC-10) Legacy Recommended bit rate query procedure (i.e. triggering, multiplexing, cancellation, prohibit timer) is used as baseline for bit rate query, with the change of logical channel to QoS flow.

6.	(MAC-12) The index 0 in the new bit rate table does not have a special meaning, i.e., indicate 0 bit rate or make it reserved bit.

7.	(MAC-12) The available bit rate query is not supported and the bit rate query from the UE always includes a desired bit rate.

8.	(MAC-09) No optimization is needed to address DC case for XR rate control MAC CE

9.	(MAC-09) We capture in stage-2 that XR rate indication is for application and not for MAC entity.

10.	(MAC-11) RAN2 will not specify DL rate control and DL rate control query

The endorsed baseline CRs (not yet reflecting the agreements of RAN2#130) are:

R2-2503360 Introduction to R19 XR enhancements

R2-2503436 Draft 38.306 CR for Rel-19 XR UE capabilities

R2-2503437 Draft 38.331 CR for Rel-19 XR UE capabilities

R2-2503564 Draft Stage 2 CR for XR

R2-2503620 RLC running CR for R19 XR

R2-2503696 PDCP running CR for R19 XR

R2-2503787 Running RRC CR for R19 XR

### 2.2.2	Main Open Issues

The format of the UL rate control MAC CE.

## 2.3	RAN3

#### 2.3.1	Discussions and Agreements

#### 2.3.1.1	RAN3#127bis

RAN3#127bis started discussing the new objectives on uplink rate control, PDU set based QoS handling enhancement, TTNB and burst size, exposure of available data rate…

For support of DL PDU Set marking without PDU Set QoS, RAN3 agreed 1) introduce DL PDU Set Information Marking Support Indication IE in the QoS Flow Level QoS Parameters IE. This indication is also provided to target gNB over XnAP. 2) reuse existing PDU Set based Handling Indicator IE for gNB to inform SMF.

For support alternative PDU Set QoS, RAN3 agreed 1) Add UL PSDB, DL PSDB, UL PSER and DL PSER in Alternative QoS Parameter Set List IE in NGAP/XnAP/F1AP/E1AP. 2) Add new codepoints “not fulfilled DL” and “not fulfilled UL” in the Notification information/cause IE in NGAP/XnAP/F1AP.

For support of exposure of available data rate, RAN3 agreed 1) Introduce thresholds and direction to trigger NG-RAN monitor available data rate in NGAP/XnAP/F1AP/E1AP. 2) Introduce “available data rate” in TS 38.425 and TS 38.415.  FFS on non-homogeneous deployment.

RAN3 factored in the new agreements from SA2/SA4 on Burst Size, TTNB and MMSID in NG-U/F1-U and Xn-U interfaces.  FFS on specific container/frame to be used.  FFS on impact to TS 38.420/470.

For MMSID, RAN3 agreed to introduce MMSID in NGAP, XnAP, F1AP and E1AP. RAN3 agreed on capturing the usage by the gNB “use this information for multi-modality service.”

For the support of UL rate control, RAN3 agreed with the RAN2 assumption that gNB receives the information on which QoS flow is subject to UL rate control from CN and sent an LS back to RAN2/SA2 in R3-252491. RAN3 also discussed the F1AP impacts, but could not reach agreement on whether the indicator to gNB-DU is per QoS flow or per DRB.

RAN3 agreed following BL CRs:

-	R3-252534 (BL CR to 37.483) Support of XR enhancements

-	R3-252535 (BL CR to 38.300) Support of XR enhancements

-	R3-252536 (BL CR to 38.413) Support of XR enhancements

-	R3-252537 (BL CR to 38.415) Support of XR enhancements

-	R3-252538 (BL CR to 38.423) Support of XR enhancements

-	R3-252539 (BL CR to 38.425) Support of XR enhancements

-	R3-252540 (BL CR to 38.473) Support of XR enhancements

#### 2.3.1.2	RAN3#128

For uplink congestion signalling:

-	RAN3 agreed to introduce a per QoS flow indication in F1AP, similar to NGAP Indication of Bitrate Adaptation IE, to inform the gNB-DU which QoS flow(s) are subject to uplink rate control, and the same indication is transferred to target gNB during Xn handover preparation procedure.  TS 38.300 TP agreed in R3-253925, TS 38.473 TP agreed in R3-253833.

-	RAN3 agreed LS to RAN2 R3-253927 regarding RAN3 decision to support the per QoS flow indication in F1AP.

For Support of exposure of available bitrate:

-	RAN3 agreed LS to SA2 R3-253929, asking whether the gNB need to report the status of Available Bitrate monitoring.

-	RAN3 agreed Stage-3 TPs to fix the Editor’s Note, e.g. define available bitrate and threshold as a 32-bit integer with unit kbps, use value “8” for maxnoofThresholds.  TS 38.413 TP agreed in R3-253928, TS 38.423 TP agreed in R3-253839, TS38.473 TP agreed in R3-253835, TS 37.483 TP agreed in R3-253840.

-	The TS 38.413 TP R3-253928 also includes the update of the semantics text for support of PDU Set AQP, i.e. for the PDU Set Delay Budget Downlink, add such clarification in semantics : This IE is included if the PDU Set Packet Delay Budget IE in the DL PDU Set QoS Information IE is present and is ignored otherwise.

For Burst Size and TTNB:

-	RAN3 agreed to fix the Editor’s Notes in TS 38.415 BL CR and TS 38.425 BL CR. TS 38.415 TP agreed in R3-253837, TS 38.425 TP agreed in R3-253930.

-	RAN3 agreed the forwarding of BSSize and TTNB in DC case. TS 37.340 TP agreed in R3-253836.

RAN3 agreed the MMSID is defined as OCTET STRING (SIZE (1)).

For RLC retransmission enhancements, RAN3 agreed to reuse the existing PDCP discard indication in F1-U to indicate the gNB-DU to stop the transmission/retransmission of a RLC SDU or the segment of a RLC SDU. FFS how to capture it in spec.

#### 2.3.2	Main Open issues

Whether the gNB need to report the status of Available Bitrate monitoring, pending on SA2 feedback.

For RLC retransmission enhancements:

-	For uplink, FFS on whether/how gNB-DU can configure the autonomous retransmission and/or enhanced polling thresholds for the DRB.

-	For downlink, FFS on whether/how enhance Signalling between CU and DU to support timely retransmission.

## 2.4	RAN4

#### 2.4.1	Discussions and Agreements

#### 2.4.1.2	RAN4#114bis

Way forward on RRM requirements for XR_Ph3 agreed in (R4-2504911):

-	For deployment scenarios:

-	Skipping requirements applies for NR-SA

-	L1/L2 triggered mobility measurements and procedures

-	Requirements in clause 9.15 NR inter-frequency L1 measurement with gaps are updated for gap skipping

-	Types of measurement gaps

-	Gaps which shall not be cancelled: MUSIM & Positioning

-	Time offset requirement for measurement gap skipping

-	Not define additional condition for the optional capability of 3 ms

-	Regarding UE assistance information, the following is agreed:

-	Capture the following agreement in the LS to RAN1/2. Meanwhile, it is not precluded to update the number of 1 s if any issue will be identified in the future meeting.

-	The information in UAI: a ratio (R) of gap occasions that is recommended for cancellation during a time period of 1 s.

-	R ∈ {0, 20, 40, 60}%

#### 2.4.1.3	RAN4#115

Way forward on RRM requirements for XR_Ph3 agreed in (R4-2508285):

-	Types of measurement gaps:

-	Type 2 gaps:

-	MG cancelling DCI applies to concurrent MGs and cancels the MG surviving the collision handling rules.

-	The timeline X ms for the MG cancelling the colliding MGs is based on the one that survives collisionhandling rules.

-	DCI skipping cannot be applied if any gap is configured with gapAssociationPRS.

-	NCSG: MG cancelling DCI doesn’t apply to MGs configured via GapConfig-r17 with ncsgInd-r17.

-	Preconfigured measurement gaps: MG cancelling DCI doesn’t apply to MGs configured via GapConfig-r17 with preConfigInd-r17.

-	Rel-18 concurrent gaps: MG cancelling DCI doesn’t apply to Rel-18 concurrent gaps.

-	Regarding UE assistance information, the following is agreed:

-	The UE may provide the UAI related information, for the UAI in the agreement captured in RAN4 #114bis, per MG configuration:

-	per-FR gap if gap is configured with gapConfig with per-FR gaps;

-	per gap configuration if gap is configured with gapConfig-r17.

-	Regarding semi-static signalling schemes, the following is agreed:

-	From RAN4 perspective, no consensus to support semi-static solutions in Rel-19.

-	Regarding skipping indication on frequency domain, the following is agreed:

-	It is RAN4 common understanding that: If UE is configured with per-FR gap, the MG skipping only applies to the same FR and has no impacts on UE behavior on the other FR. If UE is configured with per-UE gap, the MG skipping applies to both FRs.

-	Further discuss whether it has already been specified in RAN1 CR, and whether/how to capture in RAN4 spec.

-	Regarding CR drafting and naming convention, the following is agreed

-	Use “Measurement gap occasion cancelation” to refer to measurement gaps that were skipped or cancelled by DCI;

-	Use Lcancel for number of skipping occasions;

-	Indication of which formula/table to use: For the UE supporting [FFS feature name] provided that the configuration and conditions in clause 9.1.y.4 are met (new);

-	Use RAN2 pseudo code approach for new clause 9.1.y;

-	Regarding UE capabilities, the following is agreed: define optional UE feature for support of UAI with per UE capability, [and no FR1/FR2 differentiation].

#### 2.4.2	Main Open issues

The one RAN4-related objective (related to RRM measurements) remains open. See agreed WF in R4-2508285.