3GPP TSG RAN Meeting #108	RP-251006
Prague, Czech Republic, June 9-13, 2025

Agenda item:	16.1

Source:	Reliance Jio, IIT M, WiSig, IIT H, CEWiT, [Airtel], [Vodafone],[Tejas Netwoks] [HFCL]

Title:	RIS for 6G coverage enhancements, Study Proposal

Document for:	Discussion

# Introduction

In 5G, the use of higher frequency radio waves offers benefits like higher speeds and data capacity but also presents challenges in coverage. These challenges include limited range, increased interference, and the need for denser network infrastructure.

6G technology is expected to utilize a further wider range of frequencies including the sub-terahertz (THz) band apart from mid-band (3.5-6 GHz), and millimetre-wave ranges (30 – 300 GHz) and new frequencies in the 7-15 GHz. Coverage is a critical issue in higher bands due to the absence of Line of sight (LoS) in many situations and as well due to penetration and other path losses that degrades the RSRP and thereby the end user experience.

In all these cases, it is seen that Reconfigurable Intelligent Surfaces (RIS) have proved to enhance the overall RSRP (or SINR) and thereby improving the overall gain in the RF path and thus enhancing the end user experience. In 6G, we recommend an initiation of a detailed study on RIS to assess their overall benefits and how it can be included in the upcoming 6G networks.

# Discussion

Reconfigurable Intelligent Surface (RIS) has been widely studied in both academia and industry in recent years. A series of RIS prototypes have been developed, and various field trials have been done in the recent past, which has clearly demonstrated a notable improvement in both 5G coverage and capacity performance. This indicates the potential for the inclusion of RIS under the 6G SI.

Coverage is a critical issue in the current 5G deployments especially in higher bands (Mid bands and in mm Wave). Additionally, UL enhancement is critical for FWA deployments as well, In India 5G FWA deployments are Indoor and Outdoor CPE, LBR and others. Outdoor to Indoor penetration loss and indoor path loss significantly degrade RSRP. RIS assistance to FWA and outdoor mobility scenario provides a low-cost solution to address issues of coverage.

Based on our recent study / POC – Significant improvements have been observed with RIS for deployed CPE with up to -

- 4x UL throughput gain per subscriber.

- 2x DL throughput gain on RIS-assisted CPEs

- 2x UL coverage improvement on RIS-assisted CPE

- Up to 5dB RSRP gain

RIS also seems to be a better choice than A&F repeater OR Network Controlled Repeater (3GPP NCR) in terms of the following:

- Cost (80-90% cheaper)

- Power (with 88% power reduction) &

- Performance (Better SINR)

## Importance of Study on RIS for 6G -

In India 5G FWA deployments are having a combination of Indoor and Outdoor CPE’s, using various technologies such as LBR and others. Outdoor to Indoor penetration loss and indoor path loss are contributing significantly to the RSRP degradation and thereby reducing the overall end user experience.

Other solutions that are proposed to address the coverage issues are – Integrated Access Backhaul (IAB), RF-repeaters, Network Controlled Repeaters (NR) etc.

Among them, in 3gpp Release -19 –

- NCR (Network Controlled Repeaters) was introduced to provide a high gain and high efficiency solutions while enhancing the overall coverage aspects.

- But the deployment and production cost, including logistics, is still on the higher side to opt for such deployments by any mobile operator.

On Similar lines, based on the study and trials done thus far - RIS assistance to Fixed Wireless Access (FWA) and outdoor mobility scenario provides a very low-cost solution to address issues of coverage.

ETSI [2],[3],[4] has also done initial study on RIS and have drafted multiple GR’s emphasizing on definition, type of RIS, use cases, deployment scenarios, Channel model and channel estimation mechanisms. Likewise RIS is also well studies in relevant SDO’s, academia and industry.

Summary –

- RIS is a very cost effective and easy to deploy options for any mobile operator to offer better user experience for the mobile users that are behind a blockage compared to other options that exist so far.

- RIS can also be very effective for FWA use-cases to improve Indoor coverage.

- Thus, a detailed study needs to be done from the following aspects  –

  - types of RIS, definition RIS and RCU

  - Use cases, deployment scenarios, Possible architecture,

  - Channel estimation and channel modeling based on the RIS definition (if RIS is involved in any Low PHY layer functionalities or it is a mere reflector),

  - Interference study - both Adjacent Channel Interference (ACI) and Co-Channel Interference (CCI) perspectives,

  - Interfaces and its openness.

## Open Interfaces Study on RIS for 6G -

The path from research (both academic and industrial) to inclusion of RIS in 3GPP needs more detailed study. In this journey, we need to make sure that the below-mentioned interfaces are always kept open and standardized from inter-operability perspectives.

Majorly there are 2 interfaces that are relevant in case of RIS (R1 and R2 for reference purpose) –

- R1 - Interface between the Base Station and RCU (RIS Controller Unit) &

- R2 - Interface between the RCU (RIS Controller Unit) & RIS Hardware.

Fig: Block diagram representation of the RIS assisted wireless communication system.

# 2	Conclusion

Proposal 1: To initiate a detailed study on RIS as part of the 6G SI nt scenarios, Possible architecture and its associated interfaces. Channel model and channel estimation shall also be included.

Proposal 2: Ensure that any new interface defined shall be “Open” from an interoperability perspective.

# References

Qingqing Wu; Rui Zhang, "Towards Smart and Reconfigurable Environment: Intelligent Reflecting Surface Aided Wireless Network", IEEE Communications Magazine (Volume: 58, Issue: 1, January 2020)

The ETSI GR RIS 001: "Reconfigurable Intelligent Surfaces (RIS); Use Cases, Deployment Scenarios and Requirements".

ETSI GR RIS 002 (V1.1.1): "Reconfigurable Intelligent Surfaces (RIS); Technological challenges, architecture and impact on standardization

ETSI GR RIS 003 (V1.1.1): "Reconfigurable Intelligent Surfaces (RIS); Communication Models, Channel Models, Channel Estimation and Evaluation Methodology

- ETSI TR 138 901 (V16.1.0): "5G; Study on channel model for frequencies from 0.5 to 100 GHz (3GPP TR 38.901 version 16.1.0 Release 16)".