3GPP TSG RAN Meeting #108	RP-251007
Prague, Czech Republic, June 9-13, 2025

Agenda item:	16.1

Source:	Reliance jio

Title:	6G Waveform Study considerations

Document for:	Discussion

# 1	Introduction

Fifth-generation cellular technology uses the same waveform as the fourth generation, except that orthogonal frequency-division multiplexing (OFDM) is now mandatory both in downlink and uplink. Moreover, discrete Fourier transform-spread-OFDM (DFT-s-OFDM) can be used in the uplink. DFT-s-OFDM is a single carrier (SC) like transmission scheme mainly designed for coverage-limited cases due to its better power efficiency. As part of 5G NR evolution in 3GPP, a study item [2] towards the identification of new waveform beyond 52.6 GHz was undertaken in Release 16. The respective study decided to address only 52.6 GHz - 71 GHz spectrum and further spectrum consideration was left for the future.

However, no waveform change was introduced for the respective 52.6-71 GHz work item, apart from new SCS configurations. NR waveforms CP-OFDM DL with CPOFDM and DFT-s-OFDM UL were retained. Also, in 5G there was no study to address waveforms specific to vertical use cases or new services.

However, the 6G SI for waveforms should not be constrained and shall evaluate other potential new waveforms across FR1, FR2 and FR3.  3GPP now gets an opportunity to not only evaluate new waveforms for different spectrum bands BUT also for new services

# 2	Discussion

6G should serve as a platform for introducing radical technologies that go beyond the incremental improvements. The transition from 4G to 5G, however, has not significantly enhanced spectral efficiency and Multi-User MIMO (MU-MIMO) which was expected to boost spectral density has seen impediments in realizing its potential given the implementation and complexity challenges.

Many of the verticals also render themselves for intrinsically better waveforms than what’s generally developed for eMBB.

Waveform Study

If we look at the requirements for each of the use cases (below non exhaustive list in the table), it’s clear that the scenarios for evaluation has to be carefully curated to encompass both traditional and emerging use cases. From dense urban areas and suburban environments to rural and remote regions, each scenario presents unique challenges in terms of coverage, capacity, latency, energy efficiency, and quality of service.

6G waveform evaluation should consider these diverse scenarios to achieve the best possible performance across a set of waveforms.

| Sr. | Scenario | Description | Impact on waveform | KPIs |
| --- | --- | --- | --- | --- |
| 1 | 6G Mobile Broadband | Users are moving in pedestrian speed and have to do both voice and data together | cell-edge or 5th percentile throughput with ability to support voice of x Kbps and data of y Mbps | Low PAPR waveform, coverage/link budget > 140dB (400-700 MHz) |
| 1 | High Speed Vehicular | Users or devices are in constant motion, such as high-speed trains, vehicles, or drones | Resilient to high Doppler effects and rapidly changing channel conditions | Maximum Doppler tolerance. [Velocity: X m/s] |
| 2 | Sub-THz communication | Utilizing the frequency range in the sub-terahertz (THz) spectrum for wireless communication | Unique propagation characteristics of the terahertz band, such as high path loss and atmospheric absorption | OOBE, Phase noise, timing jitter, Maximum Doppler tolerance, Maximum CFO tolerance, PA efficiency [ Not covered by TS 38.901] |
| 3 | Satellite access (NTN) | Providing connectivity to remote or underserved areas through satellite links | Optimize spectral efficiency, provide robustness against doppler, fading, and ensure seamless integration with ground-based networks | Energy efficiency, Reliability, Interference resilience, Doppler resilience, Coverage, PA efficiency ]TS 38.811 Primarily LoS based channel model] |
| 4 | JCAS | leveraging 6G waveforms for advanced sensing applications like environmental monitoring, industrial automation, healthcare, and smart infrastructure. | Support for simultaneous communication and sensing with low interference | MSE of - Localization (velocity, angle, range) False alarm rate. [Maximum Doppler tolerance, SINR/SE of desired user] [Ray tracing based channel] [Representative cases] |
| 5 | Industrial indoor scenario | Consider two broad class of use cases, one pertaining to high throughput applications such as XR and digital twin, while the other pertaining to low power, low data rate applications such as sensors. | Candidate waveforms that support the coexistence of two broad class of use cases | IIoT: - High Throughput (Evaluation of High QAM case under Phase noise) - Reliability - Latency |

Also, for broadcast, ATSC 3.0 has used a digital waveform based on coded orthogonal frequency-division multiplexing (COFDM) with low-density parity-check (LDPC) forward error correction. This waveform allows for robust transmission and reception and enables interactive services and higher quality video.

A similar argument can be made for V2X and FWA scenarios as well. In summary its important to validate new 6G waveforms for frequency specific goodness but also for its optimal fit for one or more scenarios as listed above.

# 2	Conclusion

Proposal 1: The new RAT SI shall include a comprehensive study of new waveforms, applicable across multiple frequency band sets (FR1, FR2, FR3…).

Proposal 2: The new 6G SI shall include use case specific (Broadcast, NTN, IoT…) waveform study and if necessary be willing to adopt more than one set of waveforms under the collective umbrella of 6G.

# References

TSDSI report on “STUDY OF WAVEFORMS FOR 6G COMMUNICATION SYSTEMS”.

“Study on Requirements for NR beyond 52.6 GHz”, 3GPP TR 38.807