3GPP TSG RAN Meeting #108								RP-251022

Prague, Czech Republic, June 9-13, 2025

Agenda item:	15.1.2

Source: 	Samsung, AT&T, Boost Mobile Network, BT, CATT, China Telecom, China Unicom, ETRI, Fraunhofer HHI, Fraunhofer IIS, Fujitsu, Futurewei, Google, Huawei, HiSilicon, Intel Corporation, Interdigital, KDDI, KT Corporation, Kyocera, LG Electronics, LG Uplus, MediaTek, NEC, NICT, NTT DOCOMO, NTT CORPORATION, OPPO, Orange, Panasonic, Rakuten Mobile, Sharp, SK Telecom, Sony, Spark, TCL, Telia Company, Verizon, Xiaomi, ZTE, Sanechips

Title: 	Proposal for Rel-20 NR MIMO Ph6 Objectives

Document for:	Discussion and Decision

## Introduction

Starting from the down-selected set of topics in the RAN#107 moderator summary [1] and based on the RAN-chair guidance document [2] (where the Rel-20 MIMO Work Item is allocated 1TU), the following objectives are proposed for the Rel-20 NR MIMO Ph6 Work Item.

## Proposed objectives for Rel-20 NR MIMO Ph6: RAN1

Leading WG: RAN1

Other involved WGs: RAN2, RAN4

- On enhancing UL capacity and coverage, specify the following enhancements:

  - For SRS, reusing the legacy port numbering for SRS resource, comb design, SRS sequence, SRS power control, symbol-level dropping rules (when SRS collides with another UL signal), TCI/UL-spatial-relation frameworks, and SRS bandwidth configuration for SRS frequency hopping, with the maximum number of SRS resources per set following the legacy specification:

    - Multiple frequency-domain starting positions for SRS repetition symbols within each SRS frequency hop for RB-level partial frequency sounding

      - Note: On phase continuity, the applicable conditions and requirements from the legacy RAN4 spec for DMRS bundling should be retained as much as possible.

    - Cross-slot SRS between one U slot and one adjacent S slot within a single SRS resource set

      - When used for one SRS with repetition, cross-slot SRS symbol mapping is limited to within one SRS resource, with a common timing advance (TA), a common UL spatial filter, and common transmit power for the SRS resource across the two consecutive slots

  - For asymmetric DL sTRP/UL mTRP, targeting FR2 assuming the Rel-17/18 unified TCI framework:

    - RRC configuration in an SRS resource set for beam management (BM) to indicate whether to apply a same spatial filter and pathloss offset across all the configured SRS resources in the SRS resource set for the case when UL TCI state is not provided to SRS, where more than one SRS resource set for BM can be configured for UL beam sweeping

    - PDCCH-order-triggered PRACH beam indication via the indicated UL TCI state or one of the two indicated UL TCI states for the PRACH transmission to UL TRP without DL RS transmission

- On enhancing DL CSI acquisition, targeting FR1, specify the following enhancements:

  - Early SRS/CSI/CSI-RS triggering for UE transitioning from IDLE/INACTIVE to CONNECTED mode via MSG4 of 4-step RACH, as well as for SCell activation and switching out of SCell dormancy in CONNECTED mode via the legacy signaling mechanisms (based on legacy SCell activation MAC CE, and legacy switching DCI out of SCell dormancy, respectively)

    - Reusing the legacy design, limit the scope only to SRS for antenna switching, aperiodic wideband CSI with Rel-15 Type-I Single-Panel codebook (targeting FDD) and PMI-free report (targeting TDD) via the legacy UCI, as well as CSI-RS for CSI and (only for IDLE/INACTIVE to CONNECTED mode) CSI-RS for tracking

    - For IDLE mode, UE capability for early triggering and resource/reporting configuration are signaled via MSG3 and System Information (SI), respectively

  - For 48, 64, and 128 CSI-RS ports aggregated over multiple CSI-RS resources per legacy specification: CSI-RS density of 1/3, 1/4, 1/6, and 1/8 RE/RB/port while fully reusing legacy CSI-RS RE mapping per RB for CDM group generation (including OCC length, CDM group type, number of CDM groups), without impacting the legacy specification for rate matching on CSI-RS REs

    - Density of 1/3 and 1/6 RE/RB/port are intended only for 48 ports

## References

[1] 3GPP, RP-250802, RAN1 Chair (Samsung), “Moderator’s summary for RAN1-led REL-20 topics”

[2] 3GPP, RP-250040, RAN Vice Chairs, RAN1, RAN2, RAN3, RAN4 Chairs, “Summary for RAN Release 20 5G-Adv”