3GPP TSG RAN Meeting #108					                  RP-251030

Prague, CZ, 9 - 13 June 2025

Agenda item:	15.3.2

Source: 	Samsung

Title: 	Views on scope of 5G-A Rel-20 ISAC for NR

Document for:	Discussion and decision

Introduction

Rel-20 ISAC study has been discussed for several meetings. According to the summary for Rel-20 5G-A in RP-250812 [1], the following guidelines are captured,

The following are discussed and to be further checked in RAN#108: - The discussion on sensing/ISAC in 5GA and 6G are independent. - A set of possible objectives of Sensing/ISAC in Rel-20 are listed below: - Study on gNB mono-static mode and possibly bi-static mode for UAV use case. [RAN3, RAN1] - Study the procedures and signaling support between RAN and CN, and the corresponding sensing measurements. [RAN3, RAN1] - Performance evaluation for gNB sensing. [RAN1] - Note: No new waveform/RS to be specified in RAN1 and no UE impact is expected. - Note: Coordination with SA2 when needed.

In this contribution, we further provide our views on the scope based on the outlines above.

Discussion

As outlined by [1], the first thing to confirm is that the discussion on ISAC in 5GA and 6G are independent.

Last meeting, some companies expressed their concern that the outcome of Rel-20 5GA ISAC study will indicate a tendency to be reused also in 6G ISAC, e.g. the architecture to support 5GA ISAC may have more chance to be adopted also in 6G ISAC.

However, it should be noted that the potential R20 ISAC study is in parallel of 6G fundamental study. Without knowing the 6G basic architecture and interfaces, it is impossible to carry out a reasonable study in R20 that is intended to be compatible with both 5G and 6G architectures. So limiting the scope of R20 ISAC study in 5GA is safer and more pragmatic.

While 6G ISAC is better to be studied in future release when the discussion on 6G architecture becomes stable, and the 6G ISAC study should be independent and more compliant to 6G design.

Proposal 1: Confirm that ISAC in 5GA and 6G are independent. Rel-20 focuses on 5GA ISAC, and 6G ISAC is studied in future release.

The next thing to do is to refine the possible objective for Rel-20 ISAC study as provided in [1].

According to the discussion last meeting, companies seemed to have a common understanding to focus on UAV use cases (which is related to low-altitude economy) because of the strong interest from operators and vendors. In addition, at this late stage of 5G, it is not expected to support all possible ISAC use cases in 5G timeline. Therefore, the scope of 5G-A ISAC highly depends on the use cases to be commercialized urgently. Currently the use cases that are identified urgent are those related to UAV.

Observation 1: The scope of Rel-20 5G-A ISAC study should be minimized, and focus on those use cases that are to be commercialized urgently.

Observation 2: Rel-20 5GA ISAC study should focus on UAV use cases because of strong interest from operators and vendors.

The typical UAV use cases, including UAV trajectory tracking and UAV intrusion detection, are expected to be achieved by gNB mono-static sensing mode, which means one gNB is able to track UAVs and/or detect intrusion all by itself when the UAV/intruder is within the coverage.

Observation 3: The typical use cases are expected to be achieved by gNB mono-static sensing mode.

From CN-RAN signaling perspective, gNB bi-static mode is expected to bring more signalling impact than gNB mono-static mode, which means the additional coordination among Sensing Function (SF), sensing signal transmitting gNB and sensing signal receiving gNB should be considered if we consider gNB bi-static mode.

To be more specific, one initial observation on the coordination is that for gNB mono-static mode, one round of coordination between SF and gNB is enough to configure sensing signal as well as requesting the measurement; while for gNB bi-static mode, separate rounds of coordination between SF and transmitting/receiving gNB is necessary: the coordination on measurement request between SF and receiving gNB has to be done after the coordination on sensing signal configuration between SF and transmitting gNB.

To think it from another aspect, the necessary CN-RAN signalling to be specified for gNB mono-static mode can also be applied to gNB bi-static mode if needed. And it is also another possibility that separate rounds of coordination between SF and gNB is adopted in order to be more future-proof.

Observation 4: From signalling point of view,

- From one aspect, gNB bi-static mode is expected to have more signalling impact over CN-RAN interface than gNB mono-static mode.

- From the other aspect, the necessary CN-RAN signalling to be specified for gNB mono-static mode can also be applied to gNB bi-static mode.

By considering above aspects, for Rel-20 5GA ISAC, it would be preferable to study gNB mono-static mode at the initial stage, and further consider bi-static mode if needed and time allows. Considering it is at a late stage of 5G, the study on gNB bi-static mode should reuse the outcome of gNB mono-static mode as much as possible.

Proposal 2: Focus study on gNB mono-static mode for UAV use cases, and consider potential extension to bi-static mode maximizing the reuse of study on mono-static mode in Rel-20.

Another thing that is needed to be considered in Rel-20 ISAC study is the network architecture. As discussed in previous meetings, there are several architecture alternatives to support sensing functionalities in 3GPP architecture, and different architecture alternatives will also impact the procedures and signalling support between RAN and CN. For example,

Alt 1a: Introducing an independent Sensing Function (SF) in the CN, and the SF connects directly to gNB.

Alt 1b: Introducing an independent SF in the CN, and the SF connects indirectly to gNB via AMF.

Alt 2: Implementing sensing functionalities as part of current LMF in CN, and the current NRPPa is needed to be enhanced.

As a result, before exploring details on procedures and signalling support, the architecture aspect should be studied in advance.

Proposal 3: Study network architecture aspect before studying on procedures and signalling support.

As a summary, after considering above proposals, the tentative objectives of Rel-20 ISAC study is proposed as follows,

- The objectives of Sensing/ISAC in Rel-20 are listed below:

- Focus Study on gNB mono-static mode for UAV use case. Consider potential extension to bi-static mode maximizing the reuse of study on mono-static mode. [RAN3, RAN1]

- Study the procedures and signaling support between RAN and CN, and the corresponding sensing measurements. [RAN3, RAN1]

- Performance evaluation for gNB sensing. [RAN1]

- Study network architecture aspect [RAN3]

- Study the procedures and signaling support between RAN and CN, and the corresponding sensing measurements. [RAN3, RAN1]

- Note: No new waveform/RS to be specified in RAN1 and no UE impact is expected.

- Note: Coordination with SA2 when needed.

- Note: The discussion on sensing/ISAC in 5GA and 6G are independent.

Proposal 4: Agree the objectives of Rel-20 ISAC study after considering above proposals.

Conclusion

In this paper, we provides our views on the scope Rel-20 ISAC study. The following proposals are provided,

Proposal 1: Confirm that ISAC in 5GA and 6G are independent. Rel-20 focuses on 5GA ISAC, and 6G ISAC is studied in future release.

Observation 1: The scope of Rel-20 5G-A ISAC study should be minimized, and focus on those use cases that are to be commercialized urgently.

Observation 2: Rel-20 5GA ISAC study should focus on UAV use cases because of strong interest from operators and vendors.

Observation 3: The typical use cases are expected to be achieved by gNB mono-static sensing mode.

Observation 4: From signalling point of view,

- From one aspect, gNB bi-static mode is expected to have more signalling impact over CN-RAN interface than gNB mono-static mode.

- From the other aspect, the necessary CN-RAN signalling to be specified for gNB mono-static mode can also be applied to gNB bi-static mode.

Proposal 2: Focus study on gNB mono-static mode for UAV use cases, and consider potential extension to bi-static mode maximizing the reuse of study on mono-static mode in Rel-20.

Proposal 3: Study network architecture aspect before studying on procedures and signalling support.

Proposal 4: Agree the objectives of Rel-20 ISAC study after considering above proposals.

The objectives of Sensing/ISAC in Rel-20 are listed below: - Focus study on gNB mono-static mode for UAV use case. Consider potential extension to bi-static mode maximizing the reuse of study on mono-static mode. [RAN3, RAN1] - Performance evaluation for gNB sensing. [RAN1] - Study network architecture aspect [RAN3] - Study the procedures and signaling support between RAN and CN, and the corresponding sensing measurements. [RAN3, RAN1] Note: No new waveform/RS to be specified in RAN1 and no UE impact is expected. Note: Coordination with SA2 when needed. Note: The discussion on sensing/ISAC in 5GA and 6G are independent.

References

[1] RP-250812