3GPP TSG RAN Meeting #108	RP-251039

Prague, Czech Republic, June 9-13, 2025

Agenda Item:	16.1

Source:	Apple

Title:	On IMT-2030 Technical Performance Requirements

Document for:	Discussion

# Introduction

In this contribution we provide our views on the IMT-2030 Technical Performance Requirements (TPRs).

While the paper is about the TPRs, we would like to start by stressing that in our view, 6G main goal is not necessarily to push further theoretical “paper” requirements, but rather:

- Simplification and clean design, and

- New services (e.g. ISAC, AI/ML)

# Discussion

RAN#106 approved the SI [1], to investigate the IMT-2030 Technical Performance Requirements, which are being discussed in ITU-R WP5D.

In RAN#107, based on the moderator’s summary [2], the following proposals have been endorsed:

Proposal 1: Define positioning accuracy as a quantitative TPR Proposal 2: Define Sensing-related capabilities as a TPR. Proposal 3: define AI-related capabilities as qualitative TPR evaluated by inspection. Proposal 4: Define energy efficiency as a TPR. Proposal 5: For user experienced data rate, further discuss applicable deployment scenarios, and revisit the evaluation methodology before setting values for UL Proposal 6: For bandwidth TPR, it is proposed to set a target value of 400MHz (same definition as IMT-2020) - TBD on other TPR values Proposal 7: Define TPR (e.g. joint/composite requirement) related to data rate, latency, reliability, capacity etc. as a candidate IMT-2030 TPR. - Applicable usage scenario: immersive communication

Furthermore, two conference calls have been scheduled between RAN#107 and RAN#108. In the first conference calls the following preliminary agreements have been reached [3] (marked in green):

| ITU IMT-2030 TPR | IMT-2020 Value | Proposed Values | Agreements in CC#1 |
| --- | --- | --- | --- |
| Peak data rate (Gbit/s) | DL: 20 UL: 10 | [1~10x] of IMT-2020 | Alternative 1: Not to define a requirement, e.g. Leave the value for peak data rate for the technology proponent to provide Alternative 2: Define a requirement with reasonable value |
| Peak spectral efficiency (bit/s/Hz) | DL: 30 UL: 15 | [1~3x] of IMT-2020 | DL: [1.5-2.5x] of IMT-2020 UL: [1.5-2.5x] of IMT-2020 |
| User experienced data rate (Mbit/s) | DL: 100 UL: 50 | DL: [1.25~10x] of IMT-2020 UL: [1.25~4x] of IMT-2020 | For UL: - RAN#107 agreement: For user experienced data rate, further discuss applicable deployment scenarios, and revisit the evaluation methodology before setting values for UL - Proposal 1: Use system level simulation for the evaluation of UL user experienced data rate with no bandwidth scaling taking UE maximum output power into account. |
| 5th percentile user spectral efficiency (bit/s/Hz) | Indoor Hotspot-eMBB: - DL: 0.3 - UL: 0.21 Dense Urban - eMBB: - DL: 0.225 - UL: 0.15 Rural-eMBB: - DL: 0.12 - UL: 0.045 | [1.5~5x] of IMT-2020 | Companies are encouraged to provide antenna assumption for different frequency ranges together with the requirements, e.g. 4GHz, 7GHz, 30GHz Applicable deployment scenarios should also be considered. |

Proposal 2-3: Do not define security, interoperability as IMT-2030 TPRs, these can be covered by submission template for IMT-2030. Proposal 2-4: Sustainability is covered by energy efficiency, do not define separate TPR for sustainability.

## Legacy TPRs

We start the discussion with “legacy” TPRs, i.e. these IMT-2030 “inherits” from IMT-2020.

Arguably, the most visible TPRs are the ones related to data rate, i.e. Peak Data Rate, User Experienced Data Rate and Spectrum Efficiency. We observe that in real deployments in the field the gap between what a 3GPP technology can deliver (on paper) and what is actually being used has grown larger and larger. This is an indication that market demand has not caught up with IMT requirements and 3GPP technologies capabilities. We believe that driving the requirements much further will widen the gap and may have negative consequences.

A similar view has been expressed by multiple operators in [4], who propose “For IMT-2030 (aka 6G) realistic and practically reachable TPRs should be defined by ITU from the outset to avoid even higher expectations”. Furthermore, another north American operator reminds us in [5] that IMT requirements are minimum requirements, with the understanding that a technology satisfying IMT requirements can and most likely will exceed them: “Specifically, the ITU needs the minimum technical performance requirements (and corresponding evaluation criteria) which show the baseline capabilities of IMT-2030.”.

We concur with the operators and we also think that it is rather unhealthy for the ecosystem to continue pushing the gap between theoretical requirements and practical deployments.

Observation 1: the gap between what 5G/IMT-2020 can deliver “on paper” and what is actually being deployed is significant; such gap is unhealthy for the ecosystem.

With this in mind, many companies provided the opinion in RAN conference call on TPRs, that perhaps in IMT-2030/6G it would be beneficial not to define a peak data rate requirement, but rather leave this to the technology proponents (e.g. 3GPP) to decide. The following two alternatives have been discussed in the conference call:

- Alternative 1: Not to define a requirement, e.g. leave the value for peak data rate for the technology proponent to provide

- Alternative 2: Define a requirement with reasonable value

Speaking of alternative 2, it is rather unclear how one can possibly define “reasonable value” for IMT-2030 peak data rate, especially considering the fact that even the IMT-2020 values are anything but reasonable.

Proposal 1: 6G/IMT-2030 should address real market needs and therefore should focus on new services and new markets (e.g. Sensing, AI/ML) rather than pushing the theoretical data rates higher (in disconnect with real deployments).

Proposal 2: do not define IMT-2030/6G peak data rate requirement.

On the question of peak spectral efficiency, our view is that there is no need to go above 1.5 times that of IMT-2020. However, before agreeing to a value, we should perhaps agree on the parameters that spectral efficiency is derived from.

One such parameter is the number of antennas. According to [6], the IMT-2020 number of antenna requirements were:

| Parameters | Values |
| --- | --- |
| Number of BS antenna elements | 700 MHz: Up to 64 Tx/Rx 4 GHz / 30 GHz: Up to 256 Tx /Rx 70 GHz: Up to 1 024 Tx/Rx |
| Number of UE antenna elements | 700 MHz / 4 GHz: Up to 8 Tx /Rx 30 GHz: Up to 32 Tx /Rx 70 GHz: Up to 64 Tx /Rx |

Similarly to the peak data rate requirements, these remained theoretical and following the same logic, we think we should wait and let the market catch up with the requirements, and therefore we don’t see the need to increase the number of UE antennas in 700 MHz and 4 GHz. As for 7 GHz, we think the same requirement should apply

Proposal 3: for 700 MHz, 4 GHz and 7 GHz the number of UE antenna elements is up to 8 Tx /Rx.

With this in mind, we provide our view on the “legacy” TPR values in the Table 2 below. We are generally open to a discussion with regards to the actual numbers, but the overall “message” should be clear – 6G/IMT-2030 requirements should not be theoretical but rather address actual market needs.

| TPR | Value |
| --- | --- |
| Peak data rate | – Not to define a requirement, leave the value for peak data rate for the technology proponent to provide |
| Peak spectral efficiency | – Downlink peak spectral efficiency is 45 bit/s/Hz. – Uplink peak spectral efficiency is 22 bit/s/Hz. |
| User experienced data rate | – Downlink user experienced data rate is 300 Mbit/s. – Uplink user experienced data rate is 200 Mbit/s. Note: there are proposals to redefine it, in which case the numbers would obviously change; we are open to redifing this TPR. |
| 5th percentile user spectral efficiency | 1.5 times that of IMT-2020, value depends on the Test Environment |
| Average spectral efficiency | 1.5 times that of IMT-2020, value depends on the Test Environment |
| Energy efficiency | Use power models for network and UE EE TPR, value TBD |
| Area traffic capacity | Area traffic capacity in downlink is 10 Mbit/s/m |
| User plane latency | 1ms |
| Control plane latency | 15ms |
| Connection density | 10 000 000 devices per km2. |
| Reliability | 1-10−6 success probability of transmitting a layer 2 PDU |
| Mobility | The following classes of mobility are defined: – Stationary: 0 km/h – Pedestrian: 0 km/h to 10 km/h – Vehicular: 10 km/h to 120 km/h – High speed vehicular: 120 km/h to 500 km/h. |
| Mobility interruption time | 0 ms |

Table 2: Proposed values for "legacy" TPRs

NOTEs:

- There are proposals to redefine the User experienced data rate, which would obviously impact the numbers (our inputs assume the legacy definition, however we are not opposed to redefine it as we acknowledge the considers that have been raised with the legacy definition).

- Some values, are to be evaluated by inspection and therefore do not depend on a Test Environment (TE), yet to be defined by ITU-R WP5D. Some others, though, specifically those to be evaluated by simulations in a yet-to-be-defined TE, naturally depend on the simulation assumptions in the TE. Therefore, for those TPRs where the value depends on the TE, instead of providing a number (which is not possible without knowing the details of a TE), we provide a relative value (relative to the corresponding IMT-2020 values).

Proposal 4: to include the values provided in Table 2 in the TR and to communicate them to ITU-R WP5D.

## New TPRs

In this section we discuss the new IMT-2030 TPRs, i.e. those not evaluated in IMT-2020. As some of these have already been addressed either in RAN#107 or in the conference calls, here we focus on the remaining ones.

### Positioning

Positioning is a new TPR as far as ITU-R evaluation is concerned, but it’s definitely not new to 3GPP technologies. This makes the discussion easier, as we can refer to the positioning capabilities of 5G,. Going beyond the 5G capabilities for positioning should be justified with clear market use cases and requirements.

Observation 2: IMT-2030 positioning accuracy requirements can be based on 5G positioning requirements; going beyond those should be justified with clear market use cases and requirements.

The bulk of the work on positioning was carried out in 3GPP Release-16 and Release-17. According to the Release-17 TR 38.857 [5], the target positioning accuracy requirements were:

- For commercial use cases

  - Horizontal position accuracy (< 1 m) for 90% of UEs

  - Vertical position accuracy (< 3 m) for 90% of Ues

- For Industrial-IoT use cases

- Horizontal position accuracy (< 0.2 m) for 90% of UEs

- Vertical position accuracy (< 1 m) for 90% of UEs

In Release-18, according to TR 38.859 [6], a number of positioning methods (such as NR carrier phase positioning) have been added, which can significantly improve positioning accuracy beyond the Release-17 numbers. It is important to note, though, that such high accuracy is not intended for commercial (and neither public safety) use cases.

The Release-18 positioning accuracy requirements can be found in TS 22.104 [7]. The most extreme requirements are for the “Inbound logistics for manufacturing (for storage of goods):

- Horizontal accuracy: < 20cm

- Vertical accuracy: < 20cm

Therefore, in our view, the recommended positioning accuracy numbers depend on the Usage Scenario. If they are to be evaluated in a Test Environment (TE) for a HRLLC, it is reasonable to target the numbers achievable with Release-18 functionality. On the other hand, if IMT-2030 positioning is to be evaluated in a TE environment targeting commercial use cases (such as IC), it would be reasonable to target Release-17 commercial use cases numbers (with potentially a modest improvement).

Observation 3: positioning accuracy numbers depend on whether they are to be evaluated in a commercial (e.g. IC) or industrial (e.g. HRRLC) TE.

Proposal 5:  positioning accuracy requirements are based on Release-17 numbers (when evaluated in an IC TE) and on Release-18 numbers when evaluated in a MC or HRLLC TE.

### Sensing-related capabilities

A number of sensing-related capabilities are under discussion in WP5D and the group is yet to down-select them. Therefore, we think it would be beneficial if 3GPP could:

- Provide a recommendation of which capabilities to define, and

- Provide a recommendation on the TPR values for those

While sensing is a new area for IMT (and 3GPP) with potentially wide-ranging capabilities, it is important to select a few representative ones for the purpose of evaluation (which in no way should limit future sensing usages and applications).

With this in mind, out of the multitude of different TPRs which are in the current WP5D draft report [4], we propose to recommend the following:

- Localization accuracy

- Resolution

In 3GPP, we can refer to TS 22.137 [8], which includes ISAC performance requirements. The TS [8] provides a range of numbers for different Sensing Service Categories. In our view, category 3 is the most applicable for IMT evaluation, as IMT requirements are minimum requirements and 3GPP technologies often exceed the minimum IMT requirements. Therefore, we propose the following numbers for ISAC TPRs:

- Localization Accuracy

  - Horizontal: 1m

  - Vertical: 1m

- Range resolution: 1m

Notes:

- Velocity accuracy can also be considered; however it would be good to keep the number of TPRs limited; if velocity accuracy is included, similarly to localization accuracy, service category 3 from TS 22.137 should be used.

- Maximum Sensing Range (which is also being discussed in WP5D) should be part of a TE definition, not a TPR.

- Localization is to be evaluated by simulations, resolution – by analysis.

- The numbers might depend on TEs, which are not yet defined. Therefore, we should be open to revising the numbers (if needed), when TE definitions are available.

Proposal 6: For ISAC, the requirements are: localization accuracy of 1m and range resolution of 1m.

Furthermore, sensing functionality (and the evaluation) would differ depending on:

- Mono-static vs. bi-static vs. multi-static mode of operation, and

- UE-based vs. network-based sensing

On the former, we suggest to focus (for the purpose of evaluation) on mono-static, as the other variants are more dependent on implementation details and harder to evaluate. On the latter, it is important to cover both UE-based and network-based sensing in the evaluation.

Proposal 7: ISAC evaluation should focus on mono-static sensing and should cover both UE-based and network-based sensing (e.g. UE-based can be evaluated in an Indoor TE while network-based evaluated in an Outdoor TE).

# Conclusion and Proposals

Observation 1: the gap between what 5G/IMT-2020 can deliver “on paper” and what is actually being deployed is significant; such gap is unhealthy for the ecosystem.

Observation 2: IMT-2030 positioning accuracy requirements can be based on 5G positioning requirements; going beyond those should be justified with clear market use cases and requirements.

Observation 3: positioning accuracy numbers depend on whether they are to be evaluated in a commercial (e.g. IC) or industrial (e.g. HRRLC) TE.

Proposal 1: 6G/IMT-2030 should address real market needs and therefore should focus on new services and new markets (e.g. Sensing, AI/ML) rather than pushing the theoretical data rates higher (in disconnect with real deployments).

Proposal 2: do not define IMT-2030/6G peak data rate requirement.

Proposal 3: for 700 MHz, 4 GHz and 7 GHz the number of UE antenna elements is up to 8 Tx /Rx.

Proposal 4: to include the values provided in Table 2 in the TR and to communicate them to ITU-R WP5D.

Proposal 5:  positioning accuracy requirements are based on Release-17 numbers (when evaluated in an IC TE) and on Release-18 numbers when evaluated in a MC or HRLLC TE.

Proposal 6: For ISAC, the requirements are: localization accuracy of 1m and range resolution of 1m.

Proposal 7: ISAC evaluation should focus on mono-static sensing and should cover both UE-based and network-based sensing (e.g. UE-based can be evaluated in an Indoor TE while network-based evaluated in an Outdoor TE).

# References

[1]	RP-243327	New SID: Study on 6G Scenarios and requirements

[2]	RP-250819	Moderator's summary for IMT-2030 TPRs

[3]	RT-250024	Moderator's summary for IMT-2030 TPRs in CC#1

[4]	RT-250023	Operator’s view on realistic TPR definition for IMT-2030 / “6G” 	Deutsche Telekom, Telenor, Spark NZ, Odido, BT, Vodafone, Bouygues Telecom, Telia Company, Orange, KPN, TIM

[5]	RT-250020	AT&T input to CC on IMT-2030 TPR discussion, AT&T

[6]	M.2412		Guidelines for evaluation of radio interface technologies for IMT-2020