3GPP TSG-RAN Meeting #108	RP-251041

Prague, Czech Republic, 9 – 13 June 2025

Agenda item:	15.1.1

Source:	NEC

Title: 	AIML for NR air interface enhancements

Document for:	Discussion and Decision

# Introduction

In the last RAN meeting [1], companies showed their views on having enhancements for AI/ML for NR air interface and the following observations were made by the moderator. In this contribution, we provide our further views on AI/ML for NR air interface enhancements for 5G-A in Rel-20.

Based on the Wednesday offline discussions, the following observations have been made: - CSI compression for two-sided model - There was good level of support for specifying CSI compression for two-sided model as part of Rel-20 5G-Adv - However, the final decision to support CSI compression should be dependent on the outcome of the ongoing Release 19 AI/ML air interface study item - New use case / features in Release 20 5G-Adv - There was no consensus on including new use cases / features in Release 20 - New use case / features in Release 20 (18): ZTE, Google, Huawei, NEC, vivo, Tejas NW, Fujitsu, IDC, IIT Madras, SKT, KDDI, Nokia, Orange, IIT Kanpur, DeepSig, KT, Vodafone, T-Mobile - No new use case / features in Release 20 (17): DOCOMO, CMCC, CATT, Samsung, Intel, Lenovo, Panasonic, OPPO, Ericsson, MTK, AT&T, Apple, Kyocera, Xiaomi, Verizon, FAI, Spreadtrum - 2nd priority cases from Rel-19 AI/ML positioning - Clear majority of companies were in negative of including 2nd priority cases from Rel-19 AI/ML positioning in Rel-20 - Include 2nd priority cases from Rel-19 AI/ML positioning as part of Release 20 (3): NEC, CeWiT, Intel - Do not include 2nd priority cases from Rel-19 AI/ML positioning as part of Release 20 (12): Samsung, Google, Spreadtrum, CATT, Huawei, Futurewei, DOCOMO, Xiaomi, Verizon, OPPO, T-Mobile USA, Fujitsu Moderator assumes the following for the purpose of TU planning: - Include CSI compression for two-sided model as part of Release 20 5G-Adv - Final decision on whether to support CSI compressions for two-sided model as part of Release 20 5G-Adv will be made considering the outcome of Release 19 AI/ML air interface study item - If final decision is to not include CSI compression, TU for Release 20 5G-Adv AI/ML air interface will be revised accordingly - No new use case / features in Release 20 5G-Adv AI/ML air interface - No 2nd priority cases from Rel-19 AI/ML positioning in Release 20 5G-Adv AI/ML air interface - Further discuss additional LCM specification support (e.g. data collection and model transfer/delivery) in RAN#108 - [2] TUs in RAN1 and [2] TUs in RAN2 for Release 20 5G-Adv AI/ML air interface

# Discussion

### CSI compression

In addition to the Rel-18 study, Rel-19 further demonstrated the performance gain of cell/site specific models, and CSI compression with time domain aspects, e.g., spatial/temporal/frequency compression, CSI compression plus prediction. The following table summarizes the cases for evaluation of temporal domain aspects.

| Case | Target CSI slot(s) | Whether the UE uses past CSI information | Whether the network uses past CSI information |
| --- | --- | --- | --- |
| 0 | Present slot | No | No |
| 1 | Present slot | Yes | No |
| 2 | Present slot | Yes | Yes |
| 3 | Future slot(s) | Yes | No |
| 4 | Future slot(s) | Yes | Yes |
| 5 | Present slot | No | Yes |

In the RAN1#121 meeting, RAN1 recommended the following with potential RAN1 specification impacts common to all inter-vendor collaboration options.

- Recommend Case 0 - Case 2 and 3 can be considered for further enhancement built upon the specification of case 0.

Meanwhile, Rel-18/19 studies identified and are trying to provide solutions to issues related to inter-vendor training collaboration. And the following conclusions have been reached in the RAN1#121 meeting.

For inter-vendor collaboration, RAN1 concludes that both Direction A and Direction C are feasible. For Direction A, RAN1 concludes that the feasible sub-options are sub-option 4-1 and sub-option 3a-1, and in case of sub-option 3a-1, with and without target CSI sharing from NW side. - Performance: Direction C with standardized reference model (structure + parameters) ensures a minimum performance requirement, while Direction A with standardized reference model structure + parameters (sub-option 3a-1), or standardized dataset exchange (sub-option 4-1) from NW-side to UE-side may achieve better performance. - Direction C addresses inter-vendor collaboration complexity: From RAN1 perspective, Direction C (a.k.a., option 1) eliminates the inter-vendor collaboration complexity if it is feasible to standardize the model. The fully standardized reference model (structure + parameters) should be trained using synthetic data. The model structure and parameters are assumed to be equivalent to the to-be-specified model, if specified, for performance requirements in RAN4. RAN1 concludes that the scalable model structure specification and model parameters specification is feasible, but the conclusion on interoperability and RAN4 testing feasibility is a separate study in RAN4. - For Direction A with sub-option 4-1 and 3a-1, RAN1 concludes that standardized signaling, if feasible and specified, can be used for model parameter / dataset exchange to alleviate / resolve the inter-vendor collaboration complexity. The standardized signaling, if needed, may be over-the-air, or other approaches. For OTA and non-OTA based standardized signaling, as well as the feasibility of standardized signaling, related status have been provided in [R2-2503169]. For sub-option 3a-1, the specified encoder (a.k.a., CSI generation part) structure should be determined using synthetic data, and the scalable encoder structure specification is feasible.

Furthermore, in RAN1, Rel-19 studies on model identification, data collection and model transfer for two-sided model are ongoing.

For CSI compression, Rel-19 further demonstrated the performance gain of cell/site specific models, and CSI compression with time domain aspects, e.g., spatial/temporal/frequency compression, CSI compression plus prediction.

For CSI compression, RAN1 concluded that both Direction A and Direction C are feasible. RAN1 also concluded that sub-option 4-1 and sub-option 3a-1 are feasible.

It is important to specify CSI compression in 5G-Adv Rel-20 as the representative use case of two-sided model. As mentioned above, time domain aspects are essential to achieve the promised performance gain. Therefore, it is necessary to specify the support of Case 2 and Case 3 which include past and/or future CSI. In our understanding, Case 0 can also be regarded as a special case of Case 2.

It is also required to alleviate/resolve issues on inter-vendor training collaboration. Though RAN1 concluded that multiple methods are feasible, there is no need to specify all feasible approaches and down selection is needed. In our understanding, one method can be sufficient for inter-vendor collaboration, e.g., Direction A sub-option 4-1 which is based on dataset exchange. If a further method is needed, we are open to Direction A sub-option 3a-1 too.

Furthermore, the model identification procedure can be based on the chosen approach of inter-vendor collaboration, for example, MI-Option 1/2 (Model identification with data collection related configuration(s) and/or indication(s), or dataset transfer) to be adopted if Direction A sub-option 4-1 is agreed, and a combination of MI-Option 1/2 and MI-Option 3 (Model identification in model transfer from NW to UE) to be adopted if Direction A Option 3a-1 or Direction C is agreed.

The general procedure as in Figure 1 is needed to support CSI compression in Rel-20, which includes RAN1/RAN2 efforts to specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations (including model identification), data collection and dataset/model transfer (e.g., based on open format within 3GPP) for two-sided model.

Figure 1 General procedure to support CSI compression

Specify AI/ML based CSI compression with two-sided model [RAN1/RAN2]

Support spatial/temporal/frequency CSI compression (Case 2 and Case 3), including measurement and report configurations, within CSI framework

Support dataset exchange based on inter-vendor training collaboration Direction A sub-option 4-1

Support LCM including model identification and data collection for two-sided model

### AI/ML for Beam Management

Rel-19 supports AI/ML based beam measurement and report within CSI framework and inference configuration based LCM, but time is not sufficient to reach agreements on how to apply the prediction results. Clearly AI/ML based BM is not completed if both NW and UE cannot directly apply the predicted beam(s), which is the case with current Rel-19 AI/ML based BM. It seems inevitable that the RRC reconfiguration reconfigure the RS corresponding to the predicted beam in Set A as the QCL reference in a new TCI state. And MAC CE activation is still needed to activate this new TCI state. In our understanding, at least the following very basic issues need to be solved to complete the beam management procedures:

- Fast TCI state activation/indication with the predicted beam(s) as QCL (typeD) reference RS of the TCI states without RRC reconfiguration.

- TCI state activation/indication for future time instances.

- Fast beam selection based on prediction reporting without RRC reconfiguration.

With Rel-19 AI/ML based BM, both NW and UE cannot directly apply the predicted beam(s) after model inference. It is inevitable that RRC reconfiguration reconfigure a TCI state with predicted beam as QCL reference.

In addition, Rel-19 AI/ML based BM only works under a very basic assumption: single TRP, intra-cell, and Tx-beam prediction. It is not sufficient for AI/ML based BM to be used in more practical FR2 scenarios with BFR, MTRP, inter-cell and Tx-Rx alignment requirements. The following can be considered to enhance AI/ML based BM.

- Beam failure detection and new beam identification based on spatial/temporal beam prediction can solve the blockage issue more efficiently

- MTRP is a more typical deployment scenario and beam group prediction requires more designs on Set B/Set A pattern

- Inter-cell beam prediction can be done jointly with AI/ML mobility

- Tx-Rx beam alignment is a fundamental and realistic requirement and it needs more designs to enable Rx beam prediction

For AI/ML based beam management, Rel-19 specification only supports a very basic scenario, i.e., intra-cell single TRP Tx beam prediction. If considering common FR2 deployment like BFR, MTRP, inter-cell scenarios and Tx-Rx beam pair prediction, more enhancements are needed.

Considering the limited TUs, at least we need specification enhancement for NW and UE to apply the predicted beam(s) for further communication. Therefore we have the following proposal.

Complete AI/ML based BM to enable NW/UE to apply the predicted beam, e.g., fast TCI state activation/indication with the predicted beam(s) as QCL (typeD) reference RS of the TCI state without RRC reconfiguration.

If time allows, we have the following proposal for enhancing AI/ML based BM for more practical use.

Consider AI/ML based BM in BFR, MTRP, inter-cell scenarios and Tx-Rx beam pair prediction for further enhancements if time allows.

### Groupcast/Broadcast based LCM

Considering it is the very beginning of introducing AI/ML into the radio interface, it is hard to provide personalized AI/ML configuration for different individual UEs, e.g., AI/ML functionality/model itself and/or association between network additional condition and AI/ML functionality/model. Instead, groups of UE may share the same AI/ML configuration. Therefore, from resource usage efficiency consideration, broadcast/groupcast based AI/ML configuration provision is more appropriate, which can be further discussed in Rel-20.

It is suggested to consider broadcast/groupcast based AI/ML configuration provision in Rel-20.

### AI/ML related report procedure optimization

For UE-sided AI/ML model，it is necessary to provide the inference result and/or performance monitoring information (if configured) from UE to the network. In Rel-19 AI/ML for beam management, AI/ML related report are supported by reusing legacy report framework with necessary LCM specific parameters inclusion.

To enable dynamic fallback from AI/ML mode to non-AI/ML mode (i.e., legacy mode) in 5G-A, both legacy reports according to traditional measurement procedure and AI/ML-related reports can be independently configured for a specific UE in Rel-19. However, because AI/ML-related report procedure optimization, such as considering the interrelation with the legacy report, was not discussed in Rel-19 due to time constraints, independent AI/ML related report may result in unnecessary uplink resource usage, especially when specific functionality is inapplicable at UE side while inference report is still required. To address this issue, we propose further discussion of AI/ML-related report procedure optimization in Rel-20.

It is recommended to discuss AI/ML-related report procedure optimization further in Rel-20.

### Other use cases

The Rel-19 WI objectives for AI/ML based positioning accuracy enhancement were revised at RAN #107 as follows [4].

- Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any - Investigate and specify the necessary signalling of necessary measurement enhancements (if any) - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases

Compared with the previous WID, the revised WID removed 2nd priority cases. However, Case 2a and Case 2b demonstrate advantages in DL positioning, and there is a lack of convincing technical reasons to deprioritize and remove them in Rel-20 5G-A. Therefore, we maintain that specifying these cases in Rel-20 5G-A remains meaningful. Additionally, the leftover of 1st priority cases may continue to be specified in Rel-20.

For AI/ML based positioning, if time allows, specify 2nd priority cases in addition to the leftover of 1st priority cases in Rel-20 5G-A [RAN1/RAN2/RAN3].

To complete above objectives of CSI compression, BM, positioning, ~2TUs in RAN1 and ~2TU in RAN2 are needed. To avoid overlaps between 5G-Adv and 6G, it is proposed to have no other new use case study in Rel-20 for 5G-Adv.

No new use case study other than CSI compression, AI/ML based BM and AI/ML based positioning in Rel-20 for 5G-Adv.

# Conclusion

In this contribution, we provided our views on Rel-20 AI/ML for air interface Phase2, and we have the following observations and proposals:

Observation 1:	For CSI compression, Rel-19 further demonstrated the performance gain of cell/site specific models, and CSI compression with time domain aspects, e.g., spatial/temporal/frequency compression, CSI compression plus prediction.

Observation 2:	For CSI compression, RAN1 concluded that both Direction A and Direction C are feasible. RAN1 also concluded that sub-option 4-1 and sub-option 3a-1 are feasible.

Observation 3:	With Rel-19 AI/ML based BM, both NW and UE cannot directly apply the predicted beam(s) after model inference. RRC reconfiguration is inevitable to reconfigure a TCI state with predicted beam as QCL reference.

Observation 4:	For AI/ML based beam management, Rel-19 specification only supports a very basic scenario, i.e., intra-cell single TRP Tx beam prediction. If considering common FR2 deployment like BFR, MTRP, inter-cell scenarios and Tx-Rx beam pair prediction, more enhancements are needed.

Proposal 1:	Specify AI/ML based CSI compression with two-sided model [RAN1/RAN2]

−	Support spatial/temporal/frequency CSI compression (Case 2 and Case 3), including measurement and report configurations, within CSI framework

−	Support dataset exchange based on inter-vendor training collaboration Direction A sub-option 4-1

−	Support LCM including model identification and data collection for two-sided model

Proposal 2:	Complete AI/ML based BM to enable NW/UE to apply the predicted beam, e.g., fast TCI state activation/indication with the predicted beam(s) as QCL (typeD) reference RS of the TCI state without RRC reconfiguration.

Proposal 3:	Consider AI/ML based BM in BFR, MTRP, inter-cell scenarios and Tx-Rx beam pair prediction for further enhancements if time allows.

Proposal 4:	It is suggested to consider broadcast/groupcast based AI/ML configuration provision in Rel-20.

Proposal 5:	It is recommended to discuss AI/ML-related report procedure optimization further in Rel-20.

Proposal 6:	For AI/ML based positioning, if time allows, specify 2nd priority cases in addition to the leftover of 1st priority cases in Rel-20 5G-A [RAN1/RAN2/RAN3].

Proposal 7:	No new use case study other than CSI compression, AI/ML based BM and AI/ML based positioning in Rel-20 for 5G-Adv.

In addition, model transfer, dataset transfer, and data collection for two-sided model may require collaboration with other TSGs (e.g., SA2, SA5), which can be triggered when necessary.

# References

- RP-250802, Moderator's summary for RAN1 led REL-20 topics, 3GPP TSG RAN #107, Incheon, Korea, March 12th – 14th, 2025

- RP-250118, NEC, AIML for air interface Phase2, 3GPP TSG RAN #107, Incheon, Korea, March 12th – 14th, 2025

- TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

- RP-250792, Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface