3GPP TSG-RAN Meeting #108	RP-251042

Prague, Czech Republic, 9 – 13 June 2025

Agenda item:		15.1.3

Title:		View on enhancements for Ambient IoT in NR

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

In RAN#107, discussion on the scope of Ambient IoT was endorsed in RP-250802[1] with the following summary.

Moderator assumes the following for the purpose of TU planning: - Device 1 in D2T2 - Device 2b in D1T1 and D2T2 - Study on A-IoT for outdoor scenarios - Further discuss other aspects of Rel-20 A-IoT (e.g. DO-A traffic, unavailability direction 2) in RAN#108 - [3~3.5] TUs in RAN1 and [2] TUs in RAN2 for Release 20 5G-Adv A-IoT

In this paper, details of the scope of Ambient IoT are discussed and expectations are given.

# 2. Discussion

2.1	Device 1 in D2T2

The structure of Topology 2 is shown by Fig.1, and details of Topology 2 were discussed in Rel-19 A-IoT study phase with outputs in TR 38.769[2].

Figure 1: Topology 2 structure [2]

Due to limited time, Topology 2 is excluded from Rel-19 A-IoT WI, and is expected in workplan of Rel-20 A-IoT according to the endorsed workplan in RP-250802[1]. One benefit of Topology 2 is extended coverage through the help of intermediate node. By moving around carrying the intermediate node, the requirement of high-density indoor base-station can be avoided, and the application scenario can be extended to cost-sensitive end users, for example, convenience store. From RAN1’s perspective, Rel-19 A-IoT has already established R2D and D2R channel; such channels can be reused in Rel-20 A-IoT, unless necessary changes are identified.

Proposal 1: For device 1 in D2T2, support reuse of physical layer aspects of Rel-19 A-IoT with limited changes [RAN1].

2.2	Device 2b in D1T1/D2T2

Similar to Topology 2, device 2b is excluded from Rel-19 A-IoT WI due to time limitations, and is expected in workplan of Rel-20 A-IoT according to the discussion in RP-250802[1]. Device 2b has the ability to generate independent signals without requirement of carrier wave. Hence, device 2b is expected to have larger coverage/usage than device 1, for example, outdoor usage. Since device 2b is expected to be utilized in both D1T1 and D2T2, a common design at the beginning would save overall workload.

Proposal 2: For device 2b in both D1T1 and D2T2, support a common design for device 2b [RAN1].

2.3	Necessity of outdoor scenarios

In TR 38.848[3], the outdoor use cases include 4 general categories, i.e., rUC5(outdoor inventory), rUC6(outdoor sensors), rUC7(outdoor positioning), rUC8(outdoor command). Since Rel-19 A-IoT targets indoor inventory and indoor command, rUC5 and rUC8 would be enhancements for those inventory and command services. For example, when only indoor inventory is equipped, inventory has to be done per warehouse, and those goods in transportation may be regarded as “missing”. With both indoor and outdoor inventory, those goods can be inventoried regardless of their indoor or outdoor location. For rUC6, outdoor sensors have new applications in forest fire monitoring, dairy farming, etc. In summary, outdoor scenarios include many potential commercial applications beyond indoor scenarios.

Proposal 3: For outdoor scenarios, support coverage evaluation of device 2b to satisfy the requirements of outdoor scenarios [RAN1].

2.4	Higher layer aspect

2.4.1	Topology 2 architecture

In TR 38.769 [2], there are 3 solutions for Topology 2 architecture.

To support Topology 2, the following solutions are to be studied for conveying A-IoT upper layer information: -RRC based solution. With this solution, A-IoT CN applies A-IoT upper layer information explicitly over XXAP signaling. A-IoT upper layer information is then relayed explicitly to/from the A-IoT-enabled UE via NR Uu RRC. -NAS based solution. With this solution, there is no explicit termination of A-IoT upper layer information at A-IoT-enabled gNB. A-IoT upper layer information is transmitted over A-IoT enabled UE's NAS. -UP based solution. With this solution, there is no explicit termination of A-IoT upper layer information at A-IoT-enabled gNB. A-IoT upper layer information is transmitted as A-IoT-enabled UE's user plane data. NOTE 1: The protocol stack for each solution option does not illustrate A-IoT CN internal architecture and how A-IoT upper layer information is transported, if any.

Among the three solutions, the RRC-based solution offers A-IoT upper layer information awareness to the RAN node, which is advantageous for more precise reader control. Given the need for down-selection from the aforementioned three architectures during Rel-20, the RRC-based solution should be supported.

Proposal 4: RRC-based solution should be supported for Topology 2 [RAN2, RAN3].

To support topology 2, the impacts on the intermediate UE must be taken into account. This includes inventory and command data transmission, the authorization of the UE reader, resource allocation, resource validation, and RAN-CN interface design. These issues also encompass several key points, such as the RRC states in which the intermediate UE can operate, as well as the mechanisms for activating or terminating the reader function or A-IoT resources. All these aspects require collaborative efforts from both RAN2 and RAN3.

2.4.2	Fundamental Signaling Requirements

The following fundamental messages must be carefully designed:

- Inventory Request/Response: For initiating and acknowledging inventory procedures.

- Inventory Report: For transmitting collected inventory data.

- Command Request/Response: For executing control commands.

- Resource Allocation Procedures: For managing resources over both the CN-RAN interface and the Uu interface between the UE reader and A-IoT-enabled gNB.

- Assistance Information Transmission: For exchanging necessary information to enable the A-IoT service.

Additionally, the information contained within these messages must be defined to ensure interoperability and efficiency.

Proposal 5: Support signaling over both the CN-RAN interface and the Uu interface between the UE reader and A-IoT-enabled gNB [RAN3, RAN2].

2.4.3	Intermediate UE RRC states

RRC states for UE reader to perform A-IoT operations are expected to be determined during the Rel-20 WI phase. Besides RRC-CONNECTED, which is the baseline, supporting the RRC-INACTIVE state is also crucial for power saving, as no Uu signaling exchange is expected during the inventory procedure for A-IoT devices.

Proposal 6: The UE reader's operation in the RRC-CONNECTED and RRC-INACTIVE state should be supported for Topology 2 [RAN2, RAN3].

2.5	Other aspects

2.5.1	Do-A traffic

In TR 38.769 [2], DO-A service is assessed as follows.

## 6.10 DO-A assessment The study included an assessment of whether the harmonized air interface design can address the DO-A use case, only to identify which part(s) of the harmonized air interface design is/are not sufficient for the DO-A use case. From the RAN1 perspective, at least the following aspect of the air interface for DO-DTT and DT traffic types is not sufficient for the DO-A traffic type: For DO-DTT and DT traffic types, the D2R resource(s) for D2R transmission is/are indicated in a R2D transmission, but this is not applicable at least for the first D2R transmission for DO-A traffic. From RAN2 perspective, the DO-A traffic type/use case cannot be supported with the current design in the study item. It is assessed that, from RAN2 perspective, at least the A-IoT paging is an aspect/part of the current design which is not sufficient for the DO-A use case.

Rel-19 AIoT design targets for DO-DTT and DT traffic types and is not suitable for DO-A traffic both from RAN1 and RAN2 perspective. To equip Rel-20 A-IoT with DO-A service, efforts should be made by at least RAN2 and RAN1 working groups.

Moreover, DO-A service is crucial for a wide area of applications. One benefit of DO-A service is low traffic delay. For example, fire alarm application does not need frequent reporting from sensors, but when hazards happen, those sensors are expected to have quick feedback to reader, which could not be fulfilled by Rel-19 A-IoT design.

Proposal 7: DO-A support is important for commercial usage and should be in scope of Rel-20 A-IoT [RAN2, RAN1].

2.5.2	Device (un)availability direction 2

In Rel-19 SI, device (un)availability has been discussed several times. However, due to time constraints, a quick summary with two directions for study is agreed to conclude the discussion in Rel-19. According to TR 38.769 [2], direction 1 is that a reader does not provide information to a device regarding when the device may become available/unavailable, while direction 2 is that a reader can provide information to a device based on which the device may become available/unavailable.

From our perspective, although direction 1 has minor spec impact, it potentially consumes more time and needs more procedures to achieve shared understanding between the reader and the device. When the number of devices goes larger, the latency and workload at both reader side and device side would be much longer and heavier, and the A-IoT system tends to be a low efficiency system as well. On the contrary, with explicit indication from reader, the behaviour of the device is relatively determined, and reader can be aware of that from multiple devices simultaneously. This would definitely facilitate a fast and common understanding among the reader and multiple devices. As a consequence, the workload of system due to uncertainty of device behavior under direction 1 can be reduced, the efficiency of system can be guaranteed as well.

Proposal 8: For Rel-20 A-IoT, support device (un)availability direction 2 with explicit indication from reader [RAN1, RAN2].

2.5.3	Carrier wave control

In Rel-19 A-IoT WI, control of carrier wave was not discussed, since R2D transmission is on FDD DL band, and carrier wave (CW) along with backscattered D2R transmission are deployed on FDD UL band. However, for Topology 2, R2D transmission from the intermediate node and CW transmission could be both on UL band with the possibility of interference with each other.

Proposal 9: Support of carrier wave control if both R2D and carrier wave transmission are on UL band [RAN1].

2.6	Suggested objectives

Based on the discussions above, we consider the following objectives are well justified and fit the expected TUs.

Proposal 10: Approve the following objectives as Rel-20 Ambient IoT enhancement (underlined parts are differences from the summary [1]).

- Device 1 in D2T2

  - reuse of physical layer aspects of Rel-19 A-IoT with limited changes [RAN1]

- Device 2b in D1T1 and D2T2

  - support a common design for device 2b in D1T1 and D2T2[RAN1]

- Study on A-IoT for outdoor scenarios

  - coverage evaluation of device 2b to satisfy the requirements of outdoor scenarios [RAN1]

- Topology 2 architecture

  - support RRC-based solution, including signaling over both the CN-RAN interface and the Uu interface between the UE reader and A-IoT-enabled gNB [RAN2, RAN3]

  - support the UE reader's operation in the RRC-CONNECTED and RRC-INACTIVE state [RAN2, RAN3]

- Other aspects:

  - support DO-A traffic, unavailability direction 2 [RAN1, RAN2]

  - support carrier wave control if both R2D and carrier wave transmission are on UL band [RAN1]

- [3~3.5] TUs in RAN1, [2] TUs in RAN2 and [1] TU in RAN3 for Release 20 5G-Adv A-IoT

# 3. Conclusion

In this contribution, we shared our further considerations on Rel-20 A-IoT and the following proposals were drawn from the analysis.

Proposal 1: For device 1 in D2T2, support reuse of PRDCH and PDRCH of Rel-19 A-IoT with limited changes [RAN1].

Proposal 2: For device 2b in both D1T1 and D2T2, support a common design of PRDCH and PDRCH for device 2b [RAN1].

Proposal 3: For outdoor scenarios, support coverage evaluation of device 2b to satisfy the requirements of outdoor scenarios [RAN1].

Proposal 4: RRC-based solution should be supported for Topology 2 [RAN2, RAN3].

Proposal 5: Support signaling over both the CN-RAN interface and the Uu interface between the UE reader and A-IoT-enabled gNB [RAN3, RAN2].

Proposal 6: The UE reader's operation in the RRC-CONNECTED and RRC-INACTIVE state should be supported for Topology 2 [RAN2, RAN3].

Proposal 7: DO-A support is important for commercial usage and should be in scope of Rel-20 A-IoT [RAN2, RAN1].

Proposal 8: For Rel-20 A-IoT, support device (un)availability direction 2 with explicit indication from reader [RAN1, RAN2].

Proposal 9: Support of carrier wave control if both R2D and carrier wave transmission are on UL band [RAN1].

Proposal 10: Approve the following objectives as Rel-20 Ambient IoT enhancement (underlined parts are differences from the summary [1]).

- Device 1 in D2T2

  - reuse of PRDCH and PDRCH of Rel-19 A-IoT with limited changes [RAN1]

- Device 2b in D1T1 and D2T2

  - support a common design of PRDCH and PDRCH for device 2b [RAN1]

- Study on A-IoT for outdoor scenarios

  - coverage evaluation of device 2b to satisfy the requirements of outdoor scenarios [RAN1]

- Topology 2 architecture

  - support RRC-based solution, including signaling over both the CN-RAN interface and the Uu interface between the UE reader and A-IoT-enabled gNB [RAN2, RAN3]

  - support the UE reader's operation in the RRC-CONNECTED and RRC-INACTIVE state [RAN2, RAN3]

- Other aspects:

  - support DO-A traffic, unavailability direction 2 [RAN1, RAN2]

  - support carrier wave control if both R2D and carrier wave transmission are on UL band [RAN1]

- [3~3.5] TUs in RAN1, [2] TUs in RAN2 and [1] TU in RAN3 for Release 20 5G-Adv A-IoT

# References

[1] RAN1-led: RP-250802, Moderator's summary for RAN1 led REL-20 topics, RAN1 Chair (Samsung)

[2] TR 38.769, Study on solutions for ambient IoT (Internet of Things).

[3] TR 38.848, Study on Ambient IoT (Internet of Things) in RAN.