3GPP TSG-RAN Meeting #108	RP-251043

Prague, Czech Republic, 9 – 13 June 2025

Agenda Item:		15.2.1

Source:			NEC

Title:			Views on Rel-20 AI/ML Mobility

Document for:		Discussion and decision

# Introduction

In Rel-19, the potential benefits and gains of AI/ML aided mobility for network triggered L3-based handover was extensively studied and evaluated for NR. The study focuses on mobility enhancement in RRC_CONNECTED mode over air interface by following existing mobility framework, i.e., handover decision is always made in network side. Mobility use cases in the study focus on standalone NR PCell change. The following use cases were studied and evaluated:

- AI/ML based RRM measurement prediction

- AI/ML based measurement event prediction

- AI/ML based RLF prediction

In the last RAN#107, the potential scope of the WI is summarized [1].

Conclusions on AI/Mobility Only the following objectives are considered as baseline: - Specify support for RRM measurement prediction for both UE and network sided models [RAN2]: - Specify signalling and protocol aspect to enable reporting inference outcome of UE sided model for RRM measurement prediction - Specify signalling and protocol aspect to enable reporting information from UE for inference operation of network sided model for RRM measurement prediction - At least the use cases studied so far in RAN2 SI are supported: Temporal domain prediction and Frequency domain prediction. FFS Spatial domain pending RAN2 Study progress. FFS Cell level vs beam level prediction NOTE: network sided models can be concluded in RAN#108 after RAN2 discussions - Specify support for measurement event prediction for UE sided models [RAN2]: - Specify signalling and protocol aspect to enable reporting based on inference outcome of UE sided model for measurement event prediction - For both RRM measurement prediction and measurement event prediction [RAN2]: - Specify signalling and protocol to facilitate functionality-based LCM procedures - To be based on the LCM framework for Rel-19 AI/ML for NR air interface WI as much as possible - For both RRM measurement prediction and measurement event prediction: [RAN4] - Specify core requirements identified in study item phase - Specify LCM-related requirements, if any TU estimation from moderator: 1.5 TU for RAN2, [0.5-1] TU for RAN4 (performance part to be confirmed with WG), RAN3 (no impact to RAN3 at this point) RLF/HOF, CHO and LTM enhancements are not considered in the initial Rel-20 AI/ML mobility WI objective

This paper provides our views on the work scope of Rel-20 AI/ML Mobility according to the outcome of the Rel-19 study on AI/ML aided mobility captured in TR38.744 [2].

# Discussion

## 2.1 AI/ML based RRM measurement prediction

During the study, AI/ML based RRM measurement prediction was discussed for both UE-side and NW-side model. The study for RRM measurement prediction was carried out in terms of temporal domain, frequency domain and spatial domain.

In intra-frequency temporal domain case A, continuous measurement results in PW (Prediction Window) are predicted by continuous historical measurement result(s) in OW (Observation Window). Then OW and PW slide forward with either sampling period(s) or measurement period(s), where measurement result(s) are actually measured before sliding. In intra-frequency temporal domain case B, measurement results in PW are predicted by historical measurement result(s) in OW. Then OW and PW slide forward with either sampling period(s) or measurement period(s) and measurement result(s) in previous PW is/are skipped during window sliding. The examples of case A and case B are depicted in Figure-1 below.

Figure-1: Temporal domain case A and case B for RRM measurement prediction

During the study, both beam-level RRM measurement prediction and cell-level RRM measurement prediction were studied.

There are three sub-use cases considered during the study for beam-level RRM measurement prediction:

- Sub-use case 4: L1 filtered beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 beam-level measurement result is generated

- Sub-use case 5: L3 beam-level measurement result(s) is predicted based on actual L3 beam-level measurement result(s)

- Sub-use case 6: L3 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s)

For above sub-use case 4,5,6 (in the TR 38.744), it is better to be applied for Intra-frequency intra-cell temporal domain Case A.

There are three sub-use cases considered for cell-level RRM measurement prediction during the study:

- Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated

- Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s)

- Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s)

For above sub-use case 1,2,3 (in the TR 38.744) for cell-level RRM measurement prediction, it can be applied for the following scenarios:.

- Intra-frequency intra-cell temporal domain Case A;

- Intra-frequency intra-cell temporal domain Case B;

- Intra-frequency intra-cell spatial domain (as depicted in Figure-2);

- Frequency domain (as depicted in Figure-2).

Figure-2: Intra-cell spatial domain and Inter-cell spatial domain prediction

Based on the simulation results submitted in RAN2 in Rel-19, it is observed that:

For FR2 intra-frequency temporal domain case A, AI algorithm can outperform sample and hold in terms of prediction accuracy. The gain improves with increase of UE speed and PW length within a certain window length.

For FR1 intra-frequency temporal domain case B, AI algorithm can outperform sample and hold in terms of predication accuracy. The gain is higher with increase of UE speed and MRRT.

FR1 inter-frequency predictions in co-located scenario, AI algorithm with cluster approach shows better performance compared to pathloss offset-based algorithm. But AI algorithm with single cell approach achieves limited gain compared to pathloss offset based algorithm without the help of neighbour cell measurement results.

Therefore, based on the observation, we think these 3 cases can be prioritized in Rel-20:

Proposal-1: Rel-20 WI should focus on intra-frequency temporal domain Case A and Case B and also frequency domain with cluster approach for RRM measurement prediction.

In Rel-19 SI, RAN2 identified cell-level and beam-level prediction, and the goal is to improve the handover performance and reduce measurement overhead. Furthermore, for L3 beam-level prediction, it is used to facilitate the random access procedure, e.g., predict which beam/SSB resource that the UE should select for RA. In our understanding, it is further enhancement. Therefore in the beginning phase we should focus on the basic handover framework, i.e., cell-level prediction due to heavy workload in Rel-20. If time allows, we can further consider L3 beam-level prediction as low priority.

Proposal-2: Rel-20 WI should focus on Cell-level RRM measurement prediction.

## 2.2 AI/ML based measurement event prediction

The existing handover mechanism is triggered based on reported measurement event, i.e., it is kind of a reactive scheme. However, with the development of the network, such kind of reactive handover mechanism may not be optimal considering high mobility, high density and future services, e.g., XR. Such a reactive scheme may result in unintended events, e.g., HOF, RLF, Ping-Pong, throughput loss etc.

With the aid of AIML, a proactive scheme can be achieved for mobility. Proactive mobility may be based on the prediction of the occurrence of a certain measurement event based on AIML. As shown in Figure 3, with the prediction of the fulfilment of the A3 event, there may be less delay for the handover decision made at the network and accordingly the handover performance may be improved in terms of handover preparation in advance.

Figure-3: Example of prediction of measurement event by AIML

During the study in Rel-19, both direct prediction and indirect prediction were studied for AI/ML based measurement event prediction based on UE-side model. Indirect prediction is reusing AI outputs from RRM measurement prediction to determine the occurrence of measurement events, while for direct prediction, measurement event is predicted directly by an AI/ML model.

According to current observation, the AIML model based measurement event predictions give a possibility to reduce the measurement effort of the UE without impacting the handover performance. The handover performance gain was also observed with measurement event prediction (e.g. A3 event) comparing to non-AI case based on the discussion in RAN2. Hence, the support of measurement event prediction should be specified for Rel-20.

Proposal-3: Specify AI/ML based event prediction for UE-side model, to predict events occurrence time instance.

According to the evaluation results from simulations in Rel-19 AI mobility, the usage of AI/ML is beneficial for L3 handover. Taking advantage of both AI/ML based RRM measurements prediction and measurement event prediction, the enhancement of L3 handover mechanism should be specified in Rel-20. For other scenarios (e.g., CHO, LTM) should not be considered at least in the initial WI objective.

Proposal-4: Confirm that the target scope is L3 handover mechanism based on AI/ML based RRM measurements prediction and measurement event prediction.

## 2.3 Data collection and LCM

During the study of Rel-19 AI mobility, there were studies on the data collection for the network side model for AI mobility. As for AI physical layer study, collecting information from the UE side is beneficial for the operation of network side models, which includes both model training and model inference.

In addition, the general aspects of LCM mechanism including performance monitoring should also be specified in terms of signaling and protocol support.

Proposal-5: Specify the signalling support to enable the Data collection and LCM for AI/ML based mobility.

## 2.4 AI/ML based RLF prediction

Radio link failure (RLF) can be a problem in 5G networks because it can reduce communication reliability and increase latency.  RLF prediction system in Radio Access Networks (RANs) is critical for ensuring seamless communication and meeting the stringent requirements of high data rates, low latency, and improved reliability in 5G networks.

In NR, T310 timer is used to govern the process of RLF detection and declaration. The T310 timer gives the UE a chance to re-sync. If the UE can't re-sync within the T310 timer, the UE declares a RLF. Hence the prediction of the occurrence of T310 start and T310 expiration can help to determine if there is RLF ahead. As shown in the example of Figure-4, the left side shows an example of the prediction of the RLF with T310 expiration and the right side shows an example of the prediction of the RLF with T310 stop before expiration.

Figure-4: Example of RLF prediction by AIML

Even though the study in RAN2 didn’t give too much attention to RLF prediction, we see the value of the RLF prediction to improve the system performance from UE side. Considering heavy workload for this topic in Rel-20 WI, it would be better to have a check-point at later stage of the release to see if we can consider RLF prediction as well.

Proposal-6: RAN consider to set a check-point for RLF prediction at later stage of the Rel-20.

The proposal 6 can be reflected like this:

Conclusions on AI/Mobility .. RLF is not considered in the initial Rel-20 AI/ML mobility WI objective - Set a Check Point in RAN#10x [TBD] for RLF prediction. CHO and LTM enhancements are not considered in the initial Rel-20 AI/ML mobility WI objective

# Proposals

In this contribution, we shared our further considerations on the scope of Rel-20 AI/ML for Mobility and the following proposals were drawn from the analysis.

Proposal-1: Rel-20 WI Prioritizes intra-frequency temporal domain Case A and Case B and also frequency domain with cluster approach for RRM measurement prediction.

Proposal-2: Rel-20 WI should focus on Cell-level RRM measurement prediction.

Proposal-3: Specify AI/ML based event prediction for UE-side model, to predict events occurrence time instance.

Proposal-4: The enhancement of L3 handover mechanism based on AI/ML based RRM measurements prediction and measurement event prediction should be specified in Rel-20.

Proposal-5: Specify the signalling support to enable the Data collection and LCM for AI/ML based mobility.

Proposal-6: Rel-20 continues to consider AI/ML based RLF prediction for UE-side model (as low priority).

# Reference

[1] RP-250803, Moderator's summary for RAN2 led REL-20 topics, RAN2 Chair (InterDigital)

[2] TR38.744 v0.0.8

[3] RP-242393, SID on AIML for mobility in NR

[4] RP‑250121, Views on AIML for Mobility, NEC