3GPP TSG-RAN Meeting #108	RP-251045

Prague, Czech Republic, 9 – 13 June 2025

Agenda item:		15.2.3	NR mobility enhancements Phase 5

Title:		View on NR Mobility enhancement

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

In RAN#107, discussion on the scope of mobility enhancement was endorsed in RP-250803[1] with the following summary.

The following objectives are considered as baseline for Rel-20 - Specify LTM SCell improvements for further reducing interruption delays - SCell activation - Specify the necessary configuration and UE procedures for NW triggering of LTM SCell activation with LTM cell switch [RAN2, RAN3]. - Evaluate and specify as necessary the needed RAN4 requirements for this [RAN4]. NOTE: Reuse the candidate cell LTM measurement and reporting procedures where possible for the SCell improvements NOTE: This objective is NOT applicable to non-co-located CA and is NOT applicable to C-LTM NOTE: This objective is applicable to MCG and SCG - Dynamic L1 measurement and reporting configuration change - Specify the necessary configuration and procedures for MAC CE based configuration selection among prior provided RRC configurations of UE L1 measurement and reporting LTM configuration [RAN2] - Evaluate and specify as necessary the needed RAN4 requirements for this [RAN4]. ???? NOTE 1: This sub-objective is applicable to C-LTM as well. NOTE 2: For this objective, we strive to avoid RAN1 impact (including reporting) NOTE 3: RAN1/RAN3/RAN4(?) involvement is based on LS if necessary - No further RAN2 objectives are considered for Rel-20

In this paper, details of the scope of mobility enhancement are discussed and expectations are given.

# 2. Discussion

2.1	LTM SCell improvements

Currently, the SCell activation/deactivation is only supported by the indication in the RRCReconfiguration associated with the target candidate cell, which is not very dynamic. And activating SCells after completion of the LTM cell switch would cause delay of usage of CA, and decreases data throughput and user experience. Therefore, the Rel-20 LTM SCell enhancement targets dynamic and early activation of SCells during LTM cell switch procedure.

In Rel-18 mobility WI, intra-CU intra-DU and inter-DU MCG/SCG LTM cell switch are supported, and in Rel-19 mobility, inter-CU MCG LTM cell switch and inter-CU SCG LTM cell switch is supported. We don’t see a reason to exclude any of the above scenarios for now, and propose to consider dynamic SCell activation for all the scenarios in Rel-18 and Rel-19 mobility WIs

Proposal 1. Dynamic LTM SCell activation is supported for following scenarios:

- Intra-CU Intra-DU MCG/SCG LTM cell switch

- Intra-CU Inter-DU MCG/SCG LTM cell switch

- Inter-CU MCG/SCG LTM cell switch

In existing SCell activation/deactivation, the network can configure some SCells in advance, and when the radio condition of a SCell is good enough, the SCell can be activated by MAC CE. When the radio condition is bad, the SCell can be deactivated. In this sense, the measurement result from the UE is a very import factor for the network to determine whether and when to activate/deactivate the SCell. For the measurement of LTM SCells, the note in the current scope indicates that we should reuse the candidate cell LTM measurement and reporting procedures where possible for the SCell improvements, which in our understanding is to avoid further extending the existing measurement of LTM cells, by reusing it as much as possible. And since the triggering of LTM cell switch can be based on L1 measurement and also L3 measurement of the candidate cells, here, for dynamic SCell activation, we should also take both L1 measurement and L3 measurement into consideration for SCell activation.

Proposal 2: For LTM SCell dynamic activation, both L1 and L3 measurement associated with the LTM SCells should be taken into consideration.

2.2	Dynamic LTM L1 measurement and reporting configuration change

Currently, the UE is configured with a set of LTM CSI resource configurations, and a set of CSI report configurations associated with the LTM CSI resource configurations which can be periodic, semipersistent or event triggered. The UE performs measurement based on the LTM CSI resource configurations and performs measurement reporting based on the LTM CSI report configurations. This objective, in our understanding, may imply two parts, i.e., dynamic activation/deactivation of L1 measurement, and dynamic activation/deactivation of L1 measurement reporting. The dynamic L1 measurement activation/deactivation may have some impact on measurement reporting, for example, if one L1 measurement is deactivated, then the L1 measurement reporting associated with the L1 measurement cannot be performed; for another example, once the L1 measurement configuration is activated, there are some warm up periods before reporting valid measurement results in general. Considering this complexity and the potential RAN4 impact, we think at least dynamic measurement report configuration change is supported in this WI, and whether to support dynamic measurement configuration change can be discussed during the WI phase.

Proposal 3: At least dynamic LTM L1 measurement report configuration change is supported in this WI, and whether to support dynamic LTM L1 measurement configuration change can be discussed during the WI phase.

We also notice that in the current scope that the scenario is not clear for this objective. Since the LTM cell switch can be supported for both MCG and SCG, which means that LTM L1 measurement and reporting can be configured for the UE in both MCG and SCG, therefore, the dynamic LTM L1 measurement and reporting configuration should cover both MCG and SCG case.

Proposal 4: Dynamic LTM L1 measurement and reporting configuration change is applicable to both MCG and SCG.

2.3	Suggested objective

Based on the discussions above, we consider the following objectives can be fit to the expected TU in the last meeting.

Proposal 5: Approve the following objectives as Rel-20 NR mobility enhancement for 5G-Adv (underlined parts are differences from the summary [1]).

- Support SCell activation

  - Specify the necessary configuration and UE procedures for NW triggering of LTM SCell activation with LTM cell switch [RAN2, RAN3].

  - Evaluate and specify as necessary the needed RAN4 requirements for this [RAN4].

NOTE 1: Reuse the candidate cell LTM measurement and reporting procedures where possible for the SCell improvements, and both L1 and L3 based measurements can be considered

NOTE 2: This sub-objective is NOT applicable to non-co-located CA and is NOT applicable to C-LTM

NOTE 3: This sub-objective is applicable to MCG and SCG, and applicable to intra-CU MCG/SCG LTM and inter-CU MCG/SCG LTM as well

- Support Dynamic L1 measurement and reporting configuration change

  - Specify the necessary configuration and procedures for MAC CE based configuration selection among prior provided RRC configurations of UE L1 measurement and reporting LTM configuration [RAN2]

    - At least a dynamic change of reporting configuration is supported.

  - Evaluate and specify as necessary the needed RAN4 requirements for this [RAN4].

NOTE 1: This sub-objective is applicable to MCG and SCG, and applicable to C-LTM as well.

NOTE 2: For this sub-objective, we strive to avoid RAN1 impact (including reporting)

NOTE 3: RAN1/RAN3/RAN4 involvement is based on LS if necessary

# 3. Conclusion

In this contribution, we shared our further considerations on Rel-20 mobility and have the following proposals:

For LTM SCell improvements:

Proposal 1. Dynamic LTM SCell activation is supported for following scenarios:

- Intra-CU Intra-DU MCG/SCG LTM cell switch

- Intra-CU Inter-DU MCG/SCG LTM cell switch

- Inter-CU MCG/SCG LTM cell switch

Proposal 2: For dynamic LTM SCell activation, both L1 and L3 measurement associated with the LTM SCells should be taken into consideration.

For dynamic LTM L1 measurement and reporting configuration change:

Proposal 3: At least dynamic LTM L1 measurement report configuration change is supported in this WI, and whether to support dynamic LTM L1 measurement configuration change can be discussed during the WI phase.

Proposal 4: Dynamic LTM L1 measurement and reporting configuration change is applicable to both MCG and SCG.

Proposal 5: Approve the following objectives as Rel-20 NR mobility enhancement for 5G-Adv (underlined parts are differences from the summary [1]).

- Support SCell activation

  - Specify the necessary configuration and UE procedures for NW triggering of LTM SCell activation with LTM cell switch [RAN2, RAN3].

  - Evaluate and specify as necessary the needed RAN4 requirements for this [RAN4].

NOTE 1: Reuse the candidate cell LTM measurement and reporting procedures where possible for the SCell improvements, and both L1 and L3 based measurements can be considered

NOTE 2: This sub-objective is NOT applicable to non-co-located CA and is NOT applicable to C-LTM

NOTE 3: This sub-objective is applicable to MCG and SCG, and applicable to intra-CU MCG/SCG LTM and inter-CU MCG/SCG LTM as well

- Support Dynamic L1 measurement and reporting configuration change

  - Specify the necessary configuration and procedures for MAC CE based configuration selection among prior provided RRC configurations of UE L1 measurement and reporting LTM configuration [RAN2]

    - At least a dynamic change of reporting configuration is supported.

  - Evaluate and specify as necessary the needed RAN4 requirements for this [RAN4].

NOTE 1: This sub-objective is applicable to MCG and SCG, and applicable to C-LTM as well.

NOTE 2: For this sub-objective, we strive to avoid RAN1 impact (including reporting)

NOTE 3: RAN1/RAN3/RAN4 involvement is based on LS if necessary

# References

[1] RAN2-led: RP-250803, Moderator's summary for RAN2 led REL-20 topics, RAN2 Chair (InterDigital)