3GPP TSG RAN Meeting #108	RP-251052
Prague, Czech Republic, June 9-13, 2025

Agenda Item:	15.3.2

Source:	Ericsson

Title:	On ISAC for NR

Document for:	Discussion

# 1	Introduction and outcome of RAN#107

At RAN #107, the status of discussions for Rel-20 ISAC (sensing) as captured in the final summary is as follows [1].

The following are discussed and to be further checked in RAN#108:

- The discussion on sensing/ISAC in 5GA and 6G are independent.

- A set of possible objectives of Sensing/ISAC in Rel-20 are listed below:

  - Study on gNB mono-static mode and possibly bi-static mode for UAV use case. [RAN3, RAN1]

  - Study the procedures and signaling support between RAN and CN, and the corresponding sensing measurements. [RAN3, RAN1]

  - Performance evaluation for gNB sensing. [RAN1]

  - NOTE: No new waveform/RS to be specified in RAN1 and no UE impact is expected.

  - NOTE: Coordination with SA2 when needed.

We would like to provide our view on some of the above, further building on our previous discussion in [2].

# 2	Discussion

“The discussions on sensing/ISAC in 5G-A and 6G are independent.”

As previously observed [2], the bar for possible changes or additions to NG-RAN architecture and functionality at this point is very high due to the very late stage in its lifecycle. Furthermore, once a solution for 5G-A is specified, a certain natural “inertia” may cause the same solution to be simply ported over to 6G, but such a solution may be suboptimal for the new RAT and architecture. So, this principle should be maintained.

ISAC discussions for 5G-A and 6G should continue to be independent, i.e. agreements taken in 5G-A discussions should not automatically set the baseline for 6G discussions.

Of course, if any obvious commonalities arise during discussion, it should be possible to explore their reuse toward 6G where applicable, subject to further verification. One such example might be the functional design of the application protocol between RAN and core network sensing functions: it might be possible to reuse at least parts of e.g. transmission configuration and measurement configuration functionality.

If any obvious commonalities arise during discussion, it should be possible to explore their reuse toward 6G where applicable, subject to further verification.

- “Study on gNB mono-static mode and possibly bi-static mode for UAV use case. [RAN3, RAN1]”

While there seems to be consensus that mono-static sensing should be included, bi-static sensing seemed to have somewhat less support at RAN #107 [3].

Monostatic sensing includes the same TRP both transmitting the sensing signal and receiving echoes from the target. Some consider it as less complex than bi- or multistatic sensing, which involve one transmitting TRP and one or more receiving TRPs. However, we observe the following.

- While in principle it is true that monostatic and bi-static sensing might justify different channel models, current RAN1 agreements seem to already allow monostatic sensing with different antenna panels. While the actual distance might be up to implementation (minimum 10 m separation was mentioned in RAN1), the radio links should have the same LOS probability to qualify as monostatic from a channel model perspective. Hence, with these conditions the “delta” between monostatic and bi-static sensing does not seem too high.

- From the point of view of application protocol design, the functionality required to support monostatic and multistatic sensing is the same, and it is likely to require dedicated procedures for transmission configuration and measurement configuration. But it seems logical that the application protocol design should be the same for monostatic and multistatic sensing. In monostatic sensing, transmission and measurement configuration procedures will be initiated toward the same TRP, while in multistatic sensing they will be initiated toward different TRPs.

- Echoes from the targets will be received by the receiving TRP in the downlink frequency (similar scenario as e.g. D2D or V2X, but with much higher received power). This will result in more demanding receiver capabilities, completely new with respect to legacy 5G RF RX chains, especially for monostatic sensing where TX and RX TRP is the same. Hence, to support monostatic sensing, currently deployed RF hardware is extremely likely to require swapping, potentially incurring significant additional costs for an operator. With bistatic sensing, the situation seems to be less critical, due to the separation distance between TX and RX TRPs.

- It does not seem necessary to study potential synchronization aspects for bistatic sensing: synchronization aspects (e.g. RAN synchronization, positioning signals synchronization etc.) are typically considered out of 3GPP scope and are not specified. Synchronization for ISAC is no different.

Hence, we observe

It seems artificial to restrict the 5G-A sensing work to only receive the sensing signal from a target by using a single TRP (monostatic case), since multi-TRP reception (multistatic case) will have better performance and sensing reliability

Therefore, we propose:

Bi-static sensing should remain part of the Rel-20 ISAC study (removing “possibly” if agreeable).

- “Study the procedures and signaling support between RAN and CN, and the corresponding sensing measurements. [RAN3, RAN1]
“Performance evaluation for gNB sensing. [RAN1]
“NOTE: No new waveform/RS to be specified in RAN1 and no UE impact is expected.
“NOTE: Coordination with SA2 when needed.”

- Given the high amounts of work in RAN1 and considering that no new waveform/RS or no UE impacts are expected in 5G-A ISAC, we propose to maintain the responsibility in RAN3, to be agreed as the leading WG for the ISAC SI.

Maintain RAN3 as the leading WG for the ISAC SI.

According to current discussion status, no new waveform or RS is to be specified in RAN1 for ISAC; nothing is captured with respect to whether new measurements should be defined. It may be beneficial to further discuss whether ISAC support may be possible without defining new measurements for the gNB. If such an option is feasible (pending RAN1 check), this may bring further simplification to the SI scope.

Further discuss at RAN #108 whether it may be possible to support ISAC without defining new measurements for the gNB.

As mentioned above and as previously observed [2], there is very little room for possible additions to NG-RAN architecture and functionality at this stage, because we are quite late in its lifecycle. For this reason, it seems appropriate to limit the scope of the ISAC SI (and WI) to monolithic gNB only (i.e. split gNB aspects shall not be considered).ISAC SI (and WI) shall only consider the case of monolithic gNB (i.e. split gNB aspects are out of scope).

# 3	Conclusions, Proposals

Based on the discussion in the previous sections we propose the following:

- ISAC discussions for 5G-A and 6G should continue to be independent, i.e. agreements taken in 5G-A discussions should not automatically set the baseline for 6G discussions.

- If any obvious commonalities arise during discussion, it should be possible to explore their reuse toward 6G where applicable, subject to further verification.

- Bi-static sensing should remain part of the Rel-20 ISAC study (removing “possibly” if agreeable).

- Maintain RAN3 as the leading WG for the ISAC SI.

- Further discuss at RAN #108 whether it may be possible to support ISAC without defining new measurements for the gNB.

- ISAC SI (and WI) shall only consider the case of monolithic gNB (i.e. split gNB aspects are out of scope).

# 4	Reference

RP-250812	Summary for RAN Release 20 5G-Adv, RAN Vice Chairs, RAN1, RAN2, RAN3, RAN4 Chairs.

RP-250532	Discussion on Release 20 5G Advanced ISAC, Ericsson.

RP-250787	Moderator’s summary for RAN3 led REL-20 topics, RAN3 Chair (ZTE Corporation).

Monostatic sensing seems challenging in case of full-duplex, especially in massive MIMO setups; it would complicate self-interference cancellation and would limit sensing range with respect to the multi-static case.