3GPP TSG-RAN5 Meeting #107 	R5-252951

# St Julian's, Malta, 19th May 2025 – 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 3995 | rev | - | Current version: | 18.6.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to NR-NTN Timing Advance test 14.3.2.1 including TT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Korea |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TT analysis for NR-NTN test case 14.3.2.1 has become available. This CR implements this TT analysis in the test. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Implemented TT analysis in test 14.3.2.1 - Updated editor’s notes |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case would remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.3.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of Changes >>

#### 14.3.2.1	NR SA FR1 Timing Advance adjustment accuracy for Satellite Access

Editor's Note:

-	Message contents are FFS

14.3.2.1.1	Test purpose

The purpose of the test is to verify UE Timing Advance adjustment delay and accuracy requirement defined in clause 14.3.2.0.

14.3.2.1.2	Test applicability

This test applies to all types of NR UE from Release 17 and onwards supporting satellite access.

14.3.2.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clauses 14.3.2.0.1 and 14.3.2.0.2.

The normative reference for this requirement is 38.133 [6] clause A.14.3.2.1

14.3.2.1.4	Test Description

In all test cases, single cell served by SAN is used. Each test consists of two successive time periods, with time duration of T1 and T2 respectively. In each time period, timing advance commands are sent to the UE and Sounding Reference Signals (SRS), as specified in table 14.3.2.1.5-2, are sent from the UE and received by the test equipment. By measuring the reception of the SRS, the transmit timing, and hence the timing advance adjustment accuracy, can be measured.

The UE shall be provided with the valid information about the SAN serving cell before the test. During time period T1, the test equipment shall send one message with a Timing Advance Command MAC Control Element, as specified in Clause 6.1.3.4 in TS 38.321 [12]. The Timing Advance Command value shall be set to 31, which according to Clause 4.2 in TS 38.213 [8] results in zero adjustment of the Timing Advance. In this way, a reference value for the timing advance used by the UE is established.

During time period T2, the test equipment shall send a sequence of messages with Timing Advance Command MAC Control Elements, with Timing Advance Command value specified in Table 14.3.2.1.4.1-3. This value shall result in changes of the timing advance used by the UE, and the accuracy of the change shall then be measured, using the SRS sent from the UE.

The UE shall adjust its uplink timing at slot n+k+1+2µ∙Koffset for a timing advance command received in slot n. This delay must be taken into account when measuring the timing advance adjustment accuracy, via the SRS sent from the UE.

14.3.2.1.4.1	Initial Conditions

This test shall be tested in one of the configurations defined in Table 14.3.2.1.4.1-1.

Table 14.3.2.1.4.1-1: Supported test configurations for NR SA FR1 Timing Advance adjustment accuracy for Satellite Access

| Configuration | Description |
| --- | --- |
| 14.3.2.1-1 | GSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 14.3.2.1-2 | NGSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| Note: If UE supports both NGSO and GSO, the test case Config 1 can be skipped if the UE passes test case Config 2. |  |

Configure the test equipment and the DUT according to the parameters in Table 14.3.2.1.4.1-2

Table 14.3.2.1.4.1-2: Initial conditions for NR SA FR1 Timing Advance adjustment accuracy for Satellite Access

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.12-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 14.5.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |

1.	Message contents are defined in clause 14.3.2.1.4.3.

2.	Cell 1 is the NR PCell with the power level set according to clauses C.1.2 and C.1.3 for this test. Cell 1 is a satellite access cell.

3.	The initial test environment conditions are setup according to section 14.0.5.

Table 14.3.2.1.4.1-3: General test parameters for NR SA FR1 Timing Advance adjustment accuracy for Satellite Access

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF channel number |  | 1 |  |
| Initial DL BWP |  | DLBWP.0.1 | As specified in Table A.8.1-1 |
| Dedicated DL BWP |  | DLBWP.1.1 | As specified in Table A.8.1-2 |
| Initial UL BWP |  | ULBWP.0.1 | As specified in Table A.8.2-1 |
| Dedicated UL BWP |  | ULBWP.1.1 | As specified in Table A.8.2-2 |
| Timing Advance Command (TA) value during T1 |  | 31 | NTA_new = NTA_old for the purpose of establishing a reference value from which the timing advance adjustment accuracy can be measured during T2 |
| Timing Advance Command (TA) value during T2 |  | 39 | For 15 kHz SCS NTA_new = NTA_old + 8192*Tc (based on equation in clause 4.2 of TS 38.213 [8]) |
| T1 | s | 5 |  |
| T2 | s | 5 |  |

14.3.2.1.4.2	Test procedure

The UE Time Alignment Timer, described in Clause 5.2 in TS 38.321 [12], shall be configured so that it does not expire in the duration of the test.

1.	Ensure the UE is in [state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [6] clause 4.5]. Cell 1 is the active cell.

2.	Set up NR Cell according to parameters given in Table 14.3.2.1.5-1.

3.	The SS shall transmit an RRCReconfiguration message configuring the UE with periodic SRS transmissions according to clause 14.3.2.1.4.3.

4.	The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5.	During time period T1, the test equipment shall send one message with a Timing Advance Command MAC Control Element with value set to 31. This results in zero adjustment of the Timing Advance, and therefore establishes a reference value for the timing advance used by the UE. T2 starts once T1 expires.

6.	During time period T2, the test equipment shall send Timing Advance command MAC Control Elements, with a Timing Advance value of 39 as specified in table 14.3.2.1.4.1-3. This value shall result in changes of the timing advance used by the UE, and the accuracy of the change shall then be measured, using the SRS sent from the UE.

7.	The UE shall adjust its uplink timing at slot n+ k+1+2µ ∙Koffset for a timing advance command received in slot n. This delay must be taken into account when measuring the timing advance adjustment accuracy, via the SRS sent from the UE.

8.	The result from the SRS and adjustment of the timing advance in step 7) is used to measure whether the UE adjusts the timing of its transmission with a relative accuracy better than or equal to value specified in Table 14.3.2.1.5-3 to the signalled timing advance value compared to the timing of preceding uplink transmission. If the UE adjusts its timing within the requirement, the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

9.	The SS shall transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

10.	After the RRC connection is released, the SS shall switch OFF and then ON the UE to proceed with the next iteration.

11.	Repeat step 1-10 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

14.3.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 14.3.2.1.4.3-0: Common Exception messages for NR SA FR1 Timing Advance adjustment accuracy for Satellite Access

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | FFS |

Table 14.3.2.1.4.3-1: SRS-Config for NR SA FR1 Timing Advance adjustment accuracy for Satellite Access

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-182 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SRS-Config ::= SEQUENCE { |  |  |  |
| srs-ResourceSetToAddModList SEQUENCE (SIZE(0..maxNrofSRS-ResourceSets)) OF SEQUENCE { | 1 entry |  |  |
| SRS-ResourceSet[1] SEQUENCE { |  | entry 1 |  |
| resourceType CHOICE { |  |  |  |
| periodic SEQUENCE { |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| srs-ResourceToAddModList SEQUENCE (SIZE(1..maxNrofSRS-Resources)) OF SEQUENCE { | 1 entry |  |  |
| SRS-Resource[1] SEQUENCE { |  | entry 1 |  |
| freqHopping SEQUENCE { |  |  |  |
| c-SRS | 12 |  |  |
| } |  |  |  |
| groupOrSequenceHopping | neither |  |  |
| resourceType CHOICE { |  |  |  |
| periodic SEQUENCE { |  |  |  |
| periodicityAndOffset-p CHOICE { |  |  |  |
| sl5 | 2 | Once every 5 slots |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

14.3.2.1.5	Test Requirements

Tables 14.3.2.1.5-1 and 14.3.2.1.5-2 define the primary level settings including test tolerances for NR SA FR1 Timing Advance adjustment accuracy for Satellite Access.

Table 14.3.2.1.5-1: Cell Specific Test Parameters for NR SA FR1 Timing Advance adjustment accuracy for Satellite Access

| Parameter |  |  | Unit | Test1 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 |  | T2 |
| Duplex mode |  | Config 1,2 |  | FDD |  |  |
| Satellite information |  | Config 1 |  | SSC.1 |  |  |
|  |  | Config 2 |  | SSC.2 |  |  |
| BWchannel |  | Config 1,2 | MHz | 10: NRB,c = 52 |  |  |
| BWP BW |  | Config 1,2 | MHz | 10: NRB,c = 52 |  |  |
| DRX Cycle |  |  | ms | Not Applicable |  |  |
| PDSCH Reference measurement channel |  | Config 1,2 |  | SR.1.1 FDD |  |  |
| RMSI CORESET Reference Channel |  | Config 1,2 |  | CR.1.1 FDD |  |  |
| Dedicated CORESET Reference Channel |  | Config 1,2 |  | CCR.1.1 FDD |  |  |
| TRS configuration |  | Config 1,2 |  | TRS.1.1 FDD |  |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |  |
| SMTC configuration |  | Config 1,2 |  | SMTC.1 FR1 |  |  |
| SSB configuration |  | Config 1,2 |  | SSB.1 FR1 |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | -98 |  |  |
|  |  |  | dB | 3.3 |  |  |
|  |  |  | dB | 3.3 |  |  |
| IoNote3 | Config 1,2 |  | dBm/ 9.36MHz | -65.08 |  |  |
| Propagation condition |  |  | - | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

Table 14.3.2.1.5-2: SRS Configuration for NR SA FR1 Timing Advance adjustment accuracy for Satellite Access

| Field |  | Value | Comment |
| --- | --- | --- | --- |
| c-SRS | Config 1,2 | 12 | Frequency hopping is disabled |
| b-SRS |  | 0 |  |
| b-hop |  | 0 |  |
| freqDomainPosition |  | 0 | Frequency domain position of SRS |
| freqDomainShift |  | 0 |  |
| groupOrSequenceHopping |  | neither | No group or sequence hopping |
| SRS-PeriodicityAndOffset |  | sl5=2 for SCS 15kHz | Once every 5 slots |
| pathlossReferenceRS |  | ssb-Index=0 | SSB #0 is used for SRS path loss estimation |
| usage |  | Codebook | Codebook based UL transmission |
| startPosition |  | 0 | resourceMapping setting. SRS on last symbol of slot, and 1symbols for SRS without repetition. |
| nrofSymbols |  | n1 |  |
| repetitionFactor |  | n1 |  |
| combOffset-n2 |  | 0 | transmissionComb setting |
| cyclicShift-n2 |  | 0 |  |
| nrofSRS-Ports |  | port1 | Number of antenna ports used for SRS transmission |
| Note: For further information see clause 6.3.2 in TS 38.331 [2]. |  |  |  |

The UE shall apply the signalled Timing Advance value to the transmission timing at the designated activation time i.e. k+1+2µ∙Koffset slots after the reception of the timing advance command, where k=5.

The Timing Advance adjustment accuracy shall be within the limits specified in Table 14.3.2.1.5-3.

Table 14.3.2.1.5-3: UE Timing Advance adjustment accuracy for NR SA FR1 Timing Advance adjustment accuracy for Satellite Access

| Sub Carrier Spacing, SCS kHz | 15 |
| --- | --- |
| UE Timing Advance adjustment accuracy | ±344 Tc |

The rate of correct Timing Advance adjustments observed during repeated tests shall be at least 90%.

<< End of Changes >>