3GPP TSG-RAN WG5 Meeting #107	R5-253612

# St. Julian’s, Malta, 19th May 2025 – 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-5 | CR | 0095 | rev | 1 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of reference sensitivity test cases with variable Tx-Rx spacing for NR NTN bands |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | EchoStar |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | RAN2 specifications allow configuration UL and DL center frequency independently. Normally RAN4 defines minimum requirements with a single default Tx-Rx separation. Satellite industry has a strong need to use different Tx-Rx spacing in different deployments. Therefore, RAN4 specified variable Tx-Rx spacing for NTN bands and added a requirement to validate those different Tx-Rx spacing for reference sensitivity test case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updates clauses 5 and 7 per updates made in the core specifications. Adds new clause/test case for reference sensitivity test cases for variable Tx-Rx spacing to support conformance of variable Tx-Rx spacing for NR NTN UEs |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformance of variable Tx-Rx spacing cannot be completed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.4, 7.1, 7.3.2.1, 7.3.2.3, New clause 7.3.2_1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: i) Updated the test case title; ii) added editor’s note indicating test frequencies and TP analysis specifications need to be updated;ii) updated clause number based on feedback; iv) Revised by referencing the aspects baseline test case for default Tx-Rx separation that remain unchanged; v) updated test purpose in 7.3.2.1 as requested; vi) Added Annex F under Editor’s Note. |  |  |  |  |  |  |  |  |

### 5.4.4	TX–RX frequency separation

The default TX channel (carrier centre frequency) to RX channel (carrier centre frequency) separation for operating bands is specified in Table 5.4.4-1 for FR1-NTN.

Table 5.4.4-1: UE TX-RX frequency separation (FR1-NTN)

| NTN Satellite Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| n256 | 190 MHz1 165 to 215 MHz2 |
| n255 | -101.5 MHz1 -72.5 to -130.5 MHz2 |
| n254 | 862 – 885 MHz |
| NOTE 1: Default Tx-Rx separation NOTE 2: The verification of flexible Tx-Rx frequency separation within this range is limited to reference sensitivity. Further details are specified in clause 7.3.2 |  |

<<NEXT CHANGE >>

## 7.1	General

Unless otherwise stated the receiver characteristics are specified at the antenna connector(s) of the UE. For UE(s) with an integral antenna only, a reference antenna(s) with a gain of 0 dBi is assumed for each antenna port(s). UE with an integral antenna(s) may be taken into account by converting these power levels into field strength requirements, assuming a 0 dBi gain antenna. For UEs with more than one receiver antenna connector, identical interfering signals shall be applied to each receiver antenna port if more than one of these is used (diversity).

The levels of the test signal applied to each of the antenna connectors shall be as defined in the respective clauses below.

With the exception of clause 7.3, the requirements shall be verified with the network signalling value NS_01 configured in Table  6.2.3.3.1-1.

All requirements in this clause are applicable to devices supporting GSO and/or NGSO satellites.

All the parameters in clause 7 are defined using the UL reference measurement channels specified in Annex A.2.2, the DL reference measurement channels specified in Annex A.3.2 and using the set-up specified in Annex C.3.1.

All Rx requirements are verified with default Tx-Rx separation specified in Table 5.4.4-1. Additional Tx-Rx frequency separations specified in clause 7.3.2 do not apply to other requirements.

<<NEXT CHANGE >>

### 7.3.2	Reference sensitivity power level

#### 7.3.2.1	Test purpose

The test purpose is to verify the ability of the UE to receive data with a given average throughput for a specified reference measurement channel, under conditions of low signal level, ideal propagation and no added noise for default Tx-Rx frequency separation.

#### 7.3.2.2	Test applicability

The requirements of this test apply to all types of NR Power Class 3 UE release 17 and forward that support satellite access operation.

#### 7.3.2.3	Minimum conformance requirements

The throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annex A3.2, with parameters specified in Table 7.3.2.3-1.

Table 7.3.2.3-1: Two antenna port reference sensitivity QPSK PREFSENS for FDD bands

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Operating Band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| n256 | 15 | -99.5 | -96.3 | -94.5 | -93.3 |
|  | 30 |  | -96.6 | -94.6 | -93.5 |
|  | 60 |  | -97.0 | -94.9 | -93.7 |
| n255 | 15 | -100.0 | -96.8 | -95.0 | -93.8 |
|  | 30 |  | -97.1 | -95.1 | -94.0 |
|  | 60 |  | -97.5 | -95.4 | -94.2 |
| n254 | 15 | -99.5 | -96.3 | -94.5 |  |
|  | 30 |  | -96.6 | -94.6 |  |
|  | 60 |  | -97.0 | -94.9 |  |
| NOTE：The transmitter shall be set to PUMAX as defined in clause 6.2.4. |  |  |  |  |  |

The reference receiver sensitivity (REFSENS) requirement specified in Table 7.3.2.3-1 shall be met with uplink transmission bandwidth less than or equal to that specified in Table 7.3.2.3-2 and with default Tx-Rx carrier center frequency separation except for cases specified in Table 7.3.2.3-3.

Table 7.3.2.3-2: Uplink configuration for reference sensitivity

| Operating band / SCS (kHz) / Channel bandwidth (MHz) / Duplex mode |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS | 5 | 10 | 15 | 20 | Duplex Mode |
|  | 15 | 25 | 50 | 75 | 100 |  |
| n256 | 30 |  | 24 | 36 | 50 | FDD |
|  | 60 |  | 10 | 18 | 24 |  |
|  | 15 | 25 | 50 | 75 | [75] |  |
| n255 | 30 |  | 24 | 36 | [36] | FDD |
|  | 60 |  | 10 | 18 | [18] |  |
| n254 | 15 | 25 | 50 | 75 |  | FDD |
|  | 30 |  | 24 | 36 |  |  |
|  | 60 |  | 10 | 18 |  |  |
| NOTE: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth in Table 5.3.2-1. |  |  |  |  |  |  |

Table 7.3.2.3-3: TX – RX carrier centre frequency separation for REFSENS verification

| Operating Band | Channel bandwidth | TX – RX carrier centre frequency separation for REFSENS verification |
| --- | --- | --- |
| n256 | 5 MHz | 165 MHz, 215 MHz |
|  | 20 MHz | 180 MHz, 200 MHz |
| n255 | 5 MHz | -72.5 MHz, -130.5 MHz |
|  | 20 MHz | -87.5 MHz, -115.5 MHz |

The minimum requirements specified in Table 7.3.2.3-1 shall be verified with the network signalling value NS_01 configured in Table 6.2.3.1-1.

The normative reference for this requirement is TS 38.101-5 [11] clause 7.3.2.

<<NEXT CHANGE >>

### 7.3.2_1	Reference sensitivity power level for flexible Tx-Rx frequency separation

Editor’s Note: This test is incomplete. The following aspects are not yet determined:

- TP analysis needs to be captured in TR 38.905

- Test frequencies have to be specified for this test case

- Clause reference for the test frequencies is FFS

- Annex F needs to be updated and is FFS

#### 7.3.2_1.1	Test purpose

The test purpose is to verify the ability of the UE to receive data with a given average throughput for a specified reference measurement channel, under conditions of low signal level, ideal propagation and no added noise for Tx-Rx frequency separation other than the default value.

#### 7.3.2_1.2	Test applicability

Same as in clause 7.3.2.2.

#### 7.3.2_1.3	Minimum conformance requirements

Same as in clause 7.3.2.3.

#### 7.3.2_1.4	Test description

##### 7.3.2_1.4.1	Initial conditions

Same as in Clause 7.3.2.4.1 with test configuration as listed in Tables 7.3.2_1.4.1-1, 7.3.2.4.2-1 and 7.3.2.4.3-1.

Table 7.3.2_1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [12] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [12] subclause FFS |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [12] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest |  |
| Test Parameters |  |  |  |  |
| Test ID | Downlink Configuration |  | Uplink Configuration |  |
|  | Modulation | RB allocation | Modulation | RB allocation |
| 1 | CP-OFDM QPSK | Full RB (NOTE 1) | DFT-s-OFDM QPSK | REFSENS (NOTE 2) |
| NOTE 1: Full RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2. NOTE 2: REFSENS refers to Table 7.3.2.4.1-3 which defines uplink RB configuration and start RB location for each SCS, channel BW and NR band. |  |  |  |  |

##### 7.3.2_1.4.2	Test procedure.

Same as in 7.3.2.4.2.

##### 7.3.2_1.4.3	Message contents

Same as in 7.3.2.4.3.

#### 7.3.2_1.5	Test requirement

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.3.2 with reference receive power level specified in Tables 7.3.2.5-1 and parameters specified Tables 7.3.2_1.4.1-1, Tables 7.3.2.4.1-2 and Tables 7.3.2.4.1-3.

The Test Tolerances in Table 7.3.2.5-2 shall be applied.

<<END OF CHANGES >>