3GPP TSG-RAN5 Meeting #107	R5-251889

# Malta, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 1144 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | FR2 MU - Measurement grids updates for PC6 in Annex M |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_HST_FR2-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Measurement grids definitions in Annex M require certain updates for PC6. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Defined TRP Grids for Spurious Emissions for PC6 in annex M.4.5. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specification will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | M.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

## M.4.5	TRP Grids for Spurious Emissions

The worst antenna array assumptions for the MU simulations are outlined in Tables M.4.5-1 and M.4.5-2 for PC1, PC3, PC5 and PC6 with the antenna configurations per power class listed in Table M.4.5-2c.

Table M.4.5-1: Single Antenna Element Radiation Pattern for spurious emission measurements for PC1, PC3, and PC5

| Antenna element horizontal radiation pattern | , Am =25 dB |
| --- | --- |
| Horizontal half-power beam width of single element | 90o |
| Antenna element vertical radiation pattern | , SLAv =25 dB |
| Vertical half-power beam width of single array element | 90o |
| Array element radiation pattern |  |
| Element gain without antenna losses | GE,max = 5 dBi |

Table M.4.5-1a: Void

Table M.4.5-2: Composite Antenna Array Radiation Pattern for spurious emission measurements for PC1, PC3, PC5 and PC6

| Composite array radiation pattern in dB | the super position vector is given by: the weighting is given by: |
| --- | --- |
| Antenna array configuration (Row×Column) | M x N |
| Horizontal radiating element spacing, dh/λ | 1 |
| Vertical radiating element spacing, dv/λ | 1 |

Table M.4.5-2a: Void

Table M.4.5-2c: Antenna Configuration Assumptions for Different Power Classes

| Power Class | M | N |
| --- | --- | --- |
| PC1 | 12 | 12 |
| PC3 | 8 | 2 |
| PC3 (Alternate) | 4 | 2 |
| PC5 | 12 | 12 |
| PC5 (Alternate) | 6 | 6 |
| PC6 | 6 | 6 |
| Note: The alternate grids are based on an optional vendor declaration, see Table A.4.3.9-10 in [11] for PC3 and Table A.4.3.9-10a in [11] for PC5. |  |  |

The fine TRP measurement grid selection for spurious emissions is up to test system implementation but shall meet the criteria shown in Table M.4.5-3 for PC1, PC3, and PC5.

Table M.4.5-3: Fine TRP measurement grid requirement for spurious emission measurements

| Power Class | Antenna Assumption | Grid Type | Standard Deviation of MU Element ‘Influence of TRP Measurement’ | Systematic error due to TRP calculation/quadrature | Number of unique grid points |
| --- | --- | --- | --- | --- | --- |
| PC1 | 12x12 | Constant Density | 0.23 | 0dB | 1600 |
|  |  | Constant-Step Size – sin(q) | 0.21 | 0dB | 2522 (Dq=Df=5°) |
|  |  | Constant-Step Size – CC | 0.21 | 0dB | 2522 (Dq=Df=5°) |
| PC3 | 8x2 | Constant Density | 0.29 | 0dB | 450 |
|  |  | Constant-Step Size – sin(q) | 0.29 | 0dB | 614 (Dq=Df=10°) |
|  |  | Constant-Step Size – CC | 0.28 | 0dB | 614 (Dq=Df=10°) |
|  | 4x2 (alternate) | Constant Density | 0.30 | 0dB | 125 |
|  |  | Constant-Step Size – sin(q) | 0.31 | 0dB | 182 (Dq=Df=18°) |
|  |  | Constant-Step Size – CC | 0.28 | 0dB | 182 (Dq=Df=18°) |
| PC5 | 12x12 | Constant Density | 0.23 | 0dB | 1600 |
|  |  | Constant-Step Size – sin(q) | 0.21 | 0dB | 2522 (Dq=Df=5°) |
|  |  | Constant-Step Size – CC | 0.21 | 0dB | 2522 (Dq=Df=5°) |
|  | 6x6 (alternate) | Constant Density | 0.25 | 0dB | 400 |
|  |  | Constant-Step Size – sin(q) | 0.25 | 0dB | 614 (Dq=Df=10°) |
|  |  | Constant-Step Size – CC | 0.23 | 0dB | 614 (Dq=Df=10°) |
| PC6 | 6x6 | Constant Density | 0.25 | 0dB | 400 |
|  |  | Constant-Step Size – sin(q) | 0.25 | 0dB | 614 (Dq=Df=10°) |
|  |  | Constant-Step Size – CC | 0.23 | 0dB | 614 (Dq=Df=10°) |
| Note: The alternate grids are based on an optional vendor declaration, see Table A.4.3.9-10 in [11] for PC3 and Table A.4.3.9-10a in [11] for PC5. |  |  |  |  |  |

Table M.4.5-3a: Void

For spurious emissions, TRP measurements with measurement antennas displaced up to 10o from the focal point (based on electrical switching) in an IFF (based on CATR) test system, alternate TRP approaches for constant-step size grids are allowed for the coarse and fine grids:

-	interpolation to the non-offset system coordinate system that allows the use of Clenshaw-Curtis or classical sin() quadratures

-	use of the advanced Jacobian matrix quadrature approach that uses triangulations of the sphere

## <<< END OF CHANGES >>>