3GPP TSG-RAN5 Meeting #107	R5-252694

# Malta, Malta, May.19th – 23rd 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 3925 | rev | - | Current version: | 18.6.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to ATG SS-RSRQ accuracy test cases 19.6.2.X with TT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ATG-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - To update following ATG SS-RSRQ accuracy test cases based on TT analysis results: - 19.6.2.1, 19.6.2.2.1, 19.6.2.2.2; |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Editor’s notes are removed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work plan is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 19.6.2.1, 19.6.2.2.1, 19.6.2.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TR 38.903 CR 1014 TS 38.533 CR 3929 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | RRM TT |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

19.6.2.1	NR SA FR1 SS-RSRQ absolute measurement accuracy for ATG

19.6.2.1.1	Test purpose

The purpose of this test is to verify that the intra-frequency SS-RSRQ measurement accuracy is within the specified limits for all bands.

19.6.2.1.2	Test applicability

This test applies to all types of NR ATG UE from Release 18 onwards.

19.6.2.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 19.6.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.19.6.2.1.

19.6.2.1.4	Test description

19.6.2.1.4.1	Initial conditions

Same as the initial conditions given in clause 6.7.2.1.4.1 with following exceptions:

-	During the test, UE positioning and UE speed are set by AT command. UE speed is 0km/h, UE specific positioning is emulated by test system.

-	During the test, the specific gNB reference location is emulated by test system.

-	Table 6.7.2.1.4.1-1 is replaced by Table 19.6.2.1.4.1-1;

-	Table 6.7.2.1.4.1-2 is replaced by Table 19.6.2.1.4.1-2;

Table 19.6.2.1.4.1-1: Supported test configurations

| Test Case ID | Description |
| --- | --- |
| 19.6.2.1-1 | NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD |
| 19.6.2.1-2 | NR: 15 kHz SSB SCS, 10MHz bandwidth, TDD |
| 19.6.2.1-3 | NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 19.6.2.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.17-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 19.6.2.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | - Without the LTE link |  |  |

19.6.2.1.4.2	Test procedure

Same as the test procedure given in clause 6.7.2.1.4.2 with following exceptions:

-	Table 6.7.2.1.5-1 is replaced by Table 19.6.2.1.5-1;

-	Table 6.7.2.1.5-2 is replaced by Table 19.6.2.1.5-2;

19.6.2.1.4.3	Message contents

Same as the message contents given in clause 6.7.2.1.4.3.

19.6.2.1.5	Test requirement

Same as the test requirements given in clause 6.7.2.1.5 with following exceptions:

-	Table 6.7.2.1.5-1 is replaced by Table 19.6.2.1.5-1;

Table 19.6.2.1.5-1: NR Cell specific test parameters (Same as Table 6.7.2.1.5-1 with following exceptions)

| Parameter | Test configuration | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| Propagation Condition | Config 1,2 | - | AWGN+220Hz |  | AWGN+220Hz |  | AWGN+220Hz |  |
|  | Config 3 | - | AWGN+500Hz |  | AWGN+500Hz |  | AWGN+500Hz |  |

19.6.2.2	Inter-frequency SS-RSRQ measurements for ATG

19.6.2.2.1	NR SA FR1-FR1 SS-RSRQ absolute measurement accuracy for ATG

19.6.2.2.1.1	Test purpose

The purpose of this test is to verify that the inter-frequency SS-RSRQ absolute measurement accuracy is within the specified limits for all bands.

19.6.2.2.1.2	Test applicability

This test applies to all types of NR ATG UE from Release 18 onwards.

19.6.2.2.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 19.6.1.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.19.6.2.2.

19.6.2.2.1.4	Test description

19.6.2.2.1.4.1	Initial conditions

Same as the initial conditions given in clause 6.7.2.2.1.4.1 with following exceptions:

-	During the test, UE positioning and UE speed are set by AT command. UE speed is 0km/h, UE specific positioning is emulated by test system.

-	During the test, the specific gNB reference location is emulated by test system.

-	Table 6.7.2.2.1.4.1-1 is replaced by Table 19.6.2.2.1.4.1-1.

-	Table 6.7.2.2.1.4.1-2 is replaced by Table 19.6.2.2.1.4.1-2.

Table 19.6.2.2.1.4.1-1: Supported test configurations

| Test Case ID | Description |
| --- | --- |
| 19.6.2.2.1-1 | NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD |
| 19.6.2.2.1-2 | NR: 15 kHz SSB SCS, 10MHz bandwidth, TDD |
| 19.6.2.2.1-3 | NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 19.6.2.2.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.17-1 and TS 38.508-1 [14] sclause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 19.6.2.2.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | - Without the LTE link |  |  |

19.6.2.2.1.4.2	Test procedure

Same as the test procedure given in clause 6.7.2.1.1.4.2 with following exceptions:

-	Table 6.7.2.1.1.5-1 is replaced by Table 19.6.2.2.1.5-1.

-	Table 6.7.2.1.1.5-2 is replaced by Table 6.7.2.2.1.5-2.

19.6.2.2.1.4.3	Message contents

Same as the message contents given in clause 6.7.2.2.1.4.3.

19.6.2.2.1.5	Test requirement

Same as the test requirements given in clause 6.7.2.2.1.5 with following exceptions:

-	Table 6.7.2.2.1.5-1 is replaced by Table 19.6.2.2.1.5-1.

Table 19.6.2.2.1.5-1: NR Cell specific test parameters (Same as Table 6.7.2.2.1.5-1 with following exceptions)

| Parameter | Test configuration | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| Propagation Condition | Config 1,2 | - | AWGN+220Hz |  | AWGN+220Hz |  | AWGN+220Hz |  |
|  | Config 3 | - | AWGN+500Hz |  | AWGN+500Hz |  | AWGN+500Hz |  |

19.6.2.2.2	NR SA FR1-FR1 SS-RSRQ relative measurement accuracy for ATG

19.6.2.2.2.1	Test purpose

The purpose of this test is to verify that the inter-frequency SS-RSRP relative measurement accuracy is within the specified limits for all bands.

19.6.2.2.2.2	Test applicability

This test applies to all types of NR ATG UE from Release 18 onwards.

19.6.2.2.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 19.6.1.0.4.

The normative reference for this requirement is TS 38.133 [6] clause A.19.6.1.2.

19.6.2.2.2.4	Test description

19.6.2.2.2.4.1	Initial conditions

Same as the initial conditions given in clause 6.7.2.2.2.4.1 with following exceptions:

-	During the test, UE positioning and UE speed are set by AT command. UE speed is 0km/h, UE specific positioning is emulated by test system.

-	During the test, the specific gNB reference location is emulated by test system.

-	Table 6.7.2.2.2.4.1-1 is replaced by Table 19.6.2.2.2.4.1-1.

-	Table 6.7.2.2.2.4.1-2 is replaced by Table 19.6.2.2.2.4.1-2.

Table 19.6.2.2.2.4.1-1: Supported test configurations

| Test Case ID | Description |
| --- | --- |
| 19.6.2.2.2-1 | NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD |
| 19.6.2.2.2-2 | NR: 15 kHz SSB SCS, 10MHz bandwidth, TDD |
| 19.6.2.2.2-3 | NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 19.6.2.2.2.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.17-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 19.6.2.2.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | - Without the LTE link |  |  |

19.6.2.2.2.4.2	Test procedure

Same as the test procedure given in clause 6.7.1.1.2.4.2 with following exceptions:

-	Table 6.7.2.2.2.5-1 is replaced by Table 19.6.2.2.2.5-1.

19.6.2.2.2.4.3	Message contents

Same as the message contents given in clause 6.7.2.2.1.4.3.

19.6.2.2.2.5	Test requirement

Same as the test requirements given in clause 6.7.2.2.2.5 with following exceptions:

-	Table 6.7.2.2.2.5-1 is replaced by Table 19.6.2.2.1.5-1.

<End of Change 1>