# 3GPP RAN WG5 Meeting #107							R5-252817

# Malta, MT, 19th – 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.551 | CR | 0053 | rev |  | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | On FR2 MIMO OTA requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple (UK) Limited |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_OTA_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2025-04-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Clause proper indexation,text clarification and FFS removal |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 7.1.2 was missing, thus this caluse was re-indexed. Test Purpose clause was clarified withnthe text " The purpose of this test is to ensure that the UE meets the MASC minimum performance requirements." Therefore "FFS" was removed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification scope won't be acurate, including faulty indexation on clause 7 an unecessary "FFS" |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< START OF CHANGE >>>

# 7	FR2 MIMO OTA requirements

## 7.1	General

### 7.1.1       MIMO Average Spherical Coverage (MASC)

The MIMO Average Spherical Coverage (MASC) is the Figure of Merit of FR2 MIMO OTA requirement. FR2 MIMO OTA is measured with 36 constant-density points within the 3D sphere. The MASC is determined by the averaging of the best 18 sensitivity values for power class 3 UE. The averaging shall be done in linear scale for the MASC result according to the formula:

MASC70=10log18110P70,110+110P70,210+⋅⋅⋅+110P70,1810

Such that {P70,1, …, P70,18} are the best 18 sensitivity values from all the 36 constant density measurement points, as defined in [7] Annex B.2.3.

The MASC is determined by the averaging of the best 5 sensitivity values for power class 1 UE. The averaging shall be done in linear scale for the MASC result according to the formula:

MASC70=10log5110P70,110+110P70,210+⋅⋅⋅+110P70,510

Such that {P70,1, …, P70,5} are the best 5 sensitivity values from all the 36 constant density measurement points, as defined in [7] Annex B.2.3.

The MASC shall be measured at the mid channel as specified in TS 38.508-1 subclause 4.3.1 [4]. The MASC shall be lower than the requirements specified in Clause 7.2.

For FR2 MIMO OTA, PRS-EPRE-MAX, i.e., the maximum downlink RS-EPRE supported by the test system, is defined as -79.1dBm/120kHz.

For power class 3 UE, if the number of test points where the UE can meet 70% maximum throughput outage even under maximum downlink power condition (i.e., -79.1dBm/120kHz) is less than 18, then UE fails the test.  For power class 1 UE, if the number of test points where the UE can meet 70% maximum throughput outage even under maximum downlink power condition (i.e., -79.1dBm/120kHz) is less than 5, then UE fails the test.

Other criteria for FR2 are FFS.

7.1.1.1	Test Purpose

The purpose of this test is to ensure that the UE meets the MASC minimum performance requirements.

7.1.1.2	Test Applicability

This test case applies to all types of NR UE release 18 and forward. See clause 4.2 for additional applicability.

7.1.1.3	Minimum Conformance Requirements

Editor's Note: MASC values for bands n257/n258/n260 are pending in RAN4 spec TS38.151.

FR2 MASC minimum performance requirements for power class 3 NR handheld UEs in free space and the primary mechanical mode for averaging of the best 18 sensitivity values for 70% DL throughput with the corresponding measurement configurations (i.e., channel model and gNB configuration) specified in Annex I.1 and Annex E.2 are defined in Table 7.1.1.3-1.

Table 7.1.1.3-1: FR2 MASC minimum performance requirements for NR handheld UEs in free space and the primary mechanical mode

| NR bands | Bandwidth [MHz] | MIMO layer | Channel model | Reference channel | MASC70 [dBm/120kHz] |
| --- | --- | --- | --- | --- | --- |
| n257 | 100 | 2x2 | FR2 UMi CDL-C | R.PDSCH.5-2.2 TDD | […] |
| n258 | 100 | 2x2 | FR2 UMi CDL-C | R.PDSCH.5-2.2 TDD | […] |
| n260 | 100 | 2x2 | FR2 UMi CDL-C | R.PDSCH.5-2.2 TDD | […] |
| n261 | 100 | 2x2 | FR2 UMi CDL-C | R.PDSCH.5-2.2 TDD | -100.0 |

7.1.2     Test Description

7.1.2.1	Initial Conditions

Before throughput testing, the initial conditions shall be confirmed to reach the correct measurement state for each test case.

1.	Ensure environmental requirements of Annex E are met.

2.	Configure the test system according to Annex E.2 and Annex I.1 for the applicable test case.

3.	Verify the implementation of the channel model as specified in Annex I.3.

4.	Position the UE in the chamber according to Annex G.3.

5.	Power on the UE.

6.	Set up the connection.

Note:	For step 3, the verification of the channel model implementation is usually performed once for each channel model and for each validation frequency listed in Table I.3.1-1 as part of the laboratory accreditation process, and will remain valid as long as the setup and instruments remain unchanged. Otherwise the channel model validation may need to be performed prior to starting each throughput test.

7.1.2.2	Test Procedure

For throughput testing, the following steps shall be followed in order to evaluate FR2 MIMO OTA performance of the DUT:

1.	Position the DUT in the default P0 alignment option (Orientation 1), as defined in Clause G.3.

2.	Measure MIMO OTA throughput, the maximum downlink power is TBD. MIMO OTA throughput is the minimum downlink signal power resulting in a pre-defined throughput value (70%) of the maximum theoretical throughput. The downlink signal power step size shall be no more than 0.5 dB when RF power level is near the NR MIMO sensitivity level.

3.	Rotate the UE to the next test point. Table 7.1.2. 2-1 lists 36 evenly spaced test points determined using the charged particle approach and with test point #1 centred at (0,0).

4.	Repeat the test from step 2 for each specified test point. If the re-positioning concept is applied, the device needs to be positioned in P0 Orientation 2 (either option 1 or option 2), see Clause G.3.

5.	The postprocessing method and the performance metric are defined in Clause 7.1.

Note:	For step 2 of throughput testing, the measurement is not needed to start from maximum downlink power each time. To save testing time, the starting downlink power can be set as a proper value (lower than maximum downlink power supported by test system) as long as the throughput curve can reach at least 70% of the maximum theoretical throughput.

Table 7.1.2.2-1: Evenly spaced FR2 test points with a constant density

| Test Point Number | Theta [deg] | Phi [deg] |
| --- | --- | --- |
| 1 | 0.0 | 0.0 |
| 2 | 33.5 | 139.7 |
| 3 | 33.9 | 49.7 |
| 4 | 35.5 | -142.9 |
| 5 | 35.5 | -76.9 |
| 6 | 37.6 | -17.2 |
| 7 | 52.3 | 94.7 |
| 8 | 56.9 | 175.7 |
| 9 | 62.5 | 20.4 |
| 10 | 63.7 | -99.8 |
| 11 | 67.1 | -55.0 |
| 12 | 69.3 | -139.5 |
| 13 | 69.5 | 130.1 |
| 14 | 70.3 | 60.8 |
| 15 | 72.1 | -16.2 |
| 16 | 88.7 | -167.5 |
| 17 | 88.7 | 98.5 |
| 18 | 89.3 | 157.0 |
| 19 | 93.9 | -78.9 |
| 20 | 94.6 | 31.6 |
| 21 | 95.3 | -115.6 |
| 22 | 99.6 | -38.3 |
| 23 | 103.8 | -1.1 |
| 24 | 104.4 | 66.3 |
| 25 | 110.1 | 127.5 |
| 26 | 115.1 | -145.6 |
| 27 | 120.8 | 171.9 |
| 28 | 125.3 | -60.7 |
| 29 | 128.2 | -104.1 |
| 30 | 128.8 | 91.3 |
| 31 | 129.9 | 35.8 |
| 32 | 136.0 | -13.4 |
| 33 | 145.8 | 138.1 |
| 34 | 150.2 | -153.3 |
| 35 | 160.6 | -67.4 |
| 36 | 161.7 | 59.1 |

7.1.2.3	Test Requirements

Editor's Note: MASC values for bands n257/n258/n260 are pending in RAN4 spec TS38.151.

The test requirements are shown in Table 7.1.2.3-1.

Table 7.1.2.3-1: FR2 MASC test requirements for NR handheld UEs in free space and the primary mechanical mode

| NR bands | Bandwidth [MHz] | MIMO layer | Channel model | Reference channel | MASC70 [dBm/120kHz] |
| --- | --- | --- | --- | --- | --- |
| n257 | 100 | 2x2 | FR2 UMi CDL-C | R.PDSCH.5-2.2 TDD | […] +TT |
| n258 | 100 | 2x2 | FR2 UMi CDL-C | R.PDSCH.5-2.2 TDD | […] +TT |
| n260 | 100 | 2x2 | FR2 UMi CDL-C | R.PDSCH.5-2.2 TDD | […] +TT |
| n261 | 100 | 2x2 | FR2 UMi CDL-C | R.PDSCH.5-2.2 TDD | -100.0 + TT |

<<< END OF CHANGE >>>