# 3GPP RAN WG5 Meeting #107							R5-253608

# Malta, MT, 19th – 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.551 | CR | 0052 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | On FR2 MIMO OTA MU |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple (UK) Limited |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_OTA_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2025-04-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Deffinition of FR2 MIMO OTA MU budget |  |  |  |  |  |  |  |  |
| Summary of change: |  | This change defines the FR2 MIMO OTA MU budget and its terms |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification won't be complete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, Annex H |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-252816r1: Includes revisions on clauses 2, H1, H2.2, H2.3, H2.7, H2.8, and H2.11 |  |  |  |  |  |  |  |  |

<<< END OF CHANGE #1 >>>

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[3]	3GPP TS 38.101-3: "NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios".

[4]	3GPP TS 38.508-1: "5GS; User Equipment (UE) conformance specification; Part 1: Common test environment".

[5]	3GPP TR 38.901: "Study on channel model for frequencies from 0.5 to 100 GHz".

[6]	3GPP TS 38.101-4: "NR; User Equipment (UE) radio transmission and reception; Part 4: Performance requirements".

[7]	3GPP TS 38.151: "NR; User Equipment (UE) Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) performance requirements".

[8]	3GPP TS 38.561: "UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies for FR1 (NR SA and EN-DC); User Equipment (UE) conformance specification".

[9]	IEEE Std 149: "IEEE Standard Test Procedures for Antennas", IEEE.

[10]	3GPP TR 38.827: “Study on radiated metrics and test methodology for the verification of multi-antenna reception performance of NR User Equipment (UE)”.

[11]	3GPP TS 38.101-2: "NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone".

[12]	3GPP TS 38.508-2: "5GS; User Equipment (UE) conformance specification; Part 2: Common Implementation Conformance Statement (ICS) proforma".

[13]	3GPP TS 38.521-2: "User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone".

[14]	3GPP TR 38.903: " NR; Derivation of test tolerances and measurement uncertainty for User Equipment (UE) conformance test cases"

<<< END OF CHANGE #1 >>>

<<< START OF CHANGE #2>>>

Annex H (informative):
Estimation of FR2 measurement uncertainty

## H.1	MU Budget for FR2 3D-MPAC system

This clause defines the measurement uncertainty (MU) for FR2 3D-MPAC system, as shown in Table H.1-1.

Table H.1-1: Preliminary measurement uncertainty budget for FR2 3D-MPAC system

| UID | Description of uncertainty contribution | Example value (26.5GHz≤f≤29.5GHz) | Example value (37GHz ≤f≤40GHz) | Distribution of the probability | Std Uncertainty (26.5GHz≤f≤29.5GHz) [dB] | Std Uncertainty (37GHz ≤f≤40GHz) [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |
| 1 | Mismatch for measurement process | 1.30 | TBD | Actual | 1.30 | TBD |
| 2 | Measure distance uncertainty | 0.00 | TBD | Rectangular | 0.00 | TBD |
| 3 | Quality of quiet zone | 0.6 | TBD | Actual | 0.6 | TBD |
| 4 | Base Station simulator | 2.9 | TBD | Normal | 1.45 | TBD |
| 5 | Channel Emulator -absolute value -stability -linearity | 1.5dB 0.5dB 0.4dB | TBD | Actual (normal- power; rect-stability) | 0.84 | TBD |
| 6 | Amplifier uncertainties | 2.10 | TBD | Normal | 1.05 | TBD |
| 7 | Random uncertainty | 0.50 | TBD | Normal | 0.25 | TBD |
| 8 | Throughput measurement: output level step resolution | 0.25 | TBD | Rectangular | 0.14 | TBD |
| 9 | Signal flatness | 0.00 | 0.00 | Normal | 0.00 | TBD |
| Stage 1: Calibration measurement |  |  |  |  |  |  |
| 10 | Mismatch for calibration process - loopback cable path - system input path - reference antenna | 0.00 | TBD | U-Shaped | 0.00 | TBD |
| 11 | Reference antenna positioning misalignment | 0.01 | TBD | Rectangular | 0.01 | TBD |
| 12 | Quality of quiet zone | 0.60 | TBD | Actual | 0.60 | TBD |
| 13 | Total uncertainty of the Network Analyzer | 0.73 | TBD | Normal | 0.37 | TBD |
| 14 | Uncertainty of an absolute gain of the calibration antenna | 0.60 | TBD | Normal | 0.30 | TBD |
| 15 | Offset of the Phase Center of the Reference Antenna | 0.00 | TBD | Rectangular | 0.00 | TBD |
| Total Expanded Uncertainty, U, with 95% Confidence Interval |  |  |  |  | 5.05 | TBD |

The detailed descriptions of each measurement uncertainty contributor are defined in in clause H.2.

# H2	Measurement error contribution descriptions for 3D-MPAC

## H.2.1	Mismatch for measurement process

This term comes from the mismatch between the system input cables connecting to the base station simulator output port. For more information, see clause A.4.2.1 of 3GPP TS 38.561 [8].

## H.2.2	Measurement distance uncertainty

The cause of this uncertainty contributor is due to the reduction of distance between the measurement antenna and the DUT. A minimum range length of 0.75m is required for testing. This MU can be considered 0dB as the range length is primarily dependent on the PSP.

## H.2.3	Quality of quiet zone

The quality of the quiet zone procedure characterizes the quiet zone performance of the anechoic chamber, specifically the effect of reflections within the anechoic chamber including any positioners and support structures, see Annex O of 3GPP TS 38.521-2 [13].

The ripple test shall be repeated when the RF/propagation conditions inside the chamber have changed, e.g., the chamber has been disassembled and reassembled, portions of the absorber been replaced, measurement antennas/probes been replaced, positioning system been replaced, etc.

## H.2.4	Base Station simulator

gNB emulator is used to drive a signal to the channel emulator and then to the device under test. Generally there occurs uncertainty contribution from absolute level accuracy, non-linearity and frequency characteristic of the gNB emulator.

For practical reasons, in a case that a VNA is used as a calibration equipment, gNB emulator is connected to the system after the calibration measurement is performed by the VNA. Hence, the uncertainty on the absolute level of gNB emulator (transmitter device) cannot be assumed as systematic. This uncertainty will be determined from the manufacturer's datasheet and the distribution used shall match that provided in the datasheet which are commonly quoting MUs/accuracies with a "95 % confidence level" and/or a "coverage factor of 2". In the absence of a declared distribution in the datasheet, the rectangular distribution should be used. Furthermore, the uncertainty of the non-linearity is included in the absolute level uncertainty.

## H.2.5	Channel Emulator

The channel emulator is also working as a signal source in the NR MIMO OTA system, therefore there occurs uncertainty contribution from absolute level accuracy, non-linearity, frequency characteristic and stability of the channel emulator. These uncertainty contributions shall be taken from the manufacturer's data sheet.

## H.2.6	Amplifier uncertainties

Any components in the setup can potentially introduce measurement uncertainty. It is then needed to determine the uncertainty contributors associated with the use of such components. For the case of external amplifiers, the following uncertainties should be considered but the applicability is contingent to the measurement implementation and calibration procedure:

-	Stability:

-	An uncertainty contribution comes from the output level stability of the amplifier. Even if the amplifier is part of the system for both measurement and calibration, the uncertainty due to the stability shall be considered. This uncertainty can be either measured or determined by the manufacturers' data sheet for the operating conditions in which the system will be required to operate.

-	Linearity:

-	An uncertainty contribution comes from the linearity of the amplifier since in most cases calibration and measurements are performed at two different input/output power levels. This uncertainty can be either measured or determined by the manufacturers' data sheet.

-	Noise Figure:

-	When the signal goes into an amplifier, noise is added so that the SNR at the output is reduced with regard to the SNR of the signal at the input. This added noise introduces error on the signal which affects the Error Rate of the receiver thus the EVM (Error Vector Magnitude). An uncertainty can be calculated through the following formula:

-	Where SNR is the signal to noise ratio in dB at the signal level used during the sensitivity measurement.

-	Mismatch:

-	If the external amplifier is used for both stages, measurement and calibration the uncertainty contribution associated with it can be considered systematic and constant -> 0dB. If it is not the case, the mismatch uncertainty at its input and output shall be either measured or determined by the method described in 3GPP TR 38.901 [5].

-	Gain:

-	If the external amplifier is used for both stages, measurement and calibration the uncertainty contribution associated with it can be considered systematic and constant -> 0 dB. If it is not the case, this uncertainty shall be considered.

## H.2.7	Random uncertainty

This contribution is used to account for all the unknown, unquantifiable, etc. uncertainties associated with the measurements, see Annex B.2.2.9 of 3GPP TR 38.903 [14].

Random uncertainty MU contributions are normally distributed. The random uncertainty term, by definition, cannot be measured, or even isolated completely. A fixed value of 0.5 dB is suggested for MASC measurements to include a digital error rate uncertainty and is aligned with the random uncertainty for TRS for NR FR1 (3GPP TS 38.561 [8]).

## H.2.8	Throughput measurement: output level step resolution

When output power of the communication tester is swept to reach the throughput target that defines the sensitivity threshold, the final power step resolution represents an asymmetric uncertainty contribution that can be corrected since this uncertainty is device and test system independent. The lab shall correct the MASC results by -0.5 output level step resolution of the final power step search and note the correction in the test report.

## H.2.9	Signal flatness

For wireless technologies with wide channel bandwidths, the test system might not have a flat frequency response across the entire channel. While the range calibration corrects for any variation of frequency response as a function of the centre frequency of the channel, the broadband delivered to the test zone will be a function of the entire channel bandwidth as opposed to just the centre frequency. For more information, see clause A.4.2.14 of 3GPP TS 38.561 [8].

## H.2.10	Mismatch for calibration process

During calibration stage, there will be impendence mismatch between the various RF cables and components used within the system. Standing waves are created by the reflections between any two components and uncertainty in the signal level will be generated.  In general, three mismatch for calibration process should be considered:

-	Loopback cable path: This item comes from the mismatch between the reference cable and the loopback cable during the loopback cable measurement step.

-	System input path: This item comes from the mismatch between the loopback cable and the system input cable (generally the output cable after BS simulator). The reflectivity of the source output port is measured at the end of the loopback cable connecting to the system input cable.

-	Reference antenna: This item comes from the mismatch between the VNA input port and the reference antenna. The reflectivity of the VNA input port is measured at the end of the reference cable connecting to the reference antenna.

For more information, see clause A.4.2.1 of 3GPP TS 38.561 [8].

## H.2.11	Reference antenna positioning misalignment

This contribution originates from reference antenna alignment and pointing error.

## H.2.12	Total Uncertainty of the Network Analyzer

This contribution originates from all uncertainties involved transmission magnitude measurement (including drift and frequency flatness) with a network analyser. will be determined from the manufacturer's datasheet which is now commonly quoting MUs/accuracies with a "95 % confidence level" and/or a "coverage factor of 2"and the distribution used shall match that provided in the datasheet. In the absence of a declared distribution in the datasheet, the rectangular distribution should be used.

## H.2.13	Uncertainty of an absolute gain of the calibration antenna

The calibration antenna only appears in calibration phase (Stage 1). Therefore, the gain uncertainty has to be taken into account.

This uncertainty shall come from a calibration report (which is now commonly quoting MUs/accuracies with a "95 % confidence level" and/or a "coverage factor of 2") with traceability to a National Metrology Institute with measurement uncertainty budgets generated following the guidelines outlined in internationally accepted standards. In the absence of a declared distribution in the present document, the rectangular distribution should be used.

## H.2.14	Offset of the Phase Center of the Reference Antenna

During range reference measurement, if a directional antenna is used, the uncertainty in the accuracy of positioning its phase center on the axis of rotation will directly generate an uncertainty in this part of the measurement. In practical measurement, sleeve dipoles and loops are used for FR1 calibration, then the uncertainty of this element should be 0 dB, since the phase center offset is negligible.

<<< END OF CHANGE #2 >>>