3GPP TSG-RAN5 Meeting #107	R5-252126

# Malta, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-3 | CR | 3089 | rev | - | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update Annex F for IoT NTN enh TCs based on MUTT analysis |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Mediatek India Technology Pvt. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_enh_plus_CT1-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The Annex F currently do not contain content of 13.1.1.4 and 13.1.1.6. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update Annex F based on MUTT analysis of 13.1.1.4 and 13.1.1.6. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Related information remain missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex F.1.2, F.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | Y |  | Test specifications |  |  |  | TS/TR ... CR 0468, 0469 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< START OF CHANGES 1>>>

Annex F (normative):
Measurement uncertainties and Test Tolerances

# F.0	General

The requirements of this clause apply to all applicable tests in the present document.

# F.1	Acceptable uncertainty of Test System (normative)

F.1.0	General

See 3GPP TS 36.521-1 [10] Annex F1.

## F.1.1	Measurement of test environments

See 3GPP TS 36.521-1 [10] Annex F1.1.

## F.1.2	Measurement of RRM requirements

Table F.1.2-1: Maximum Test System Uncertainty for RRM Requirements

| Subclause | Maximum Test System Uncertainty1 | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 4.2.1 E-UTRA FDD - FDD cell re-selection intra frequency | Noc ±1.0 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB averaged over BWConfig | NOTE: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 4.2.2 E-UTRA TDD - TDD cell re-selection intra frequency | Same as 4.2.1 |  |
| 4.2.3 E-UTRA FDD - FDD cell re-selection inter frequency | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.3 dB averaged over BWConfig | NOTE: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |
| 4.2.4 E-UTRA FDD - TDD cell re-selection inter frequency | Same as 4.2.3 |  |
| 4.2.5 E-UTRA TDD - FDD cell re-selection inter frequency | Same as 4.2.3 |  |
| 4.2.6 E-UTRA TDD - TDD cell re-selection inter frequency | Same as 4.2.3 |  |
| 4.2.7 E-UTRA FDD Inter frequency re-selection in the existence of non-allowed CSG cell | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig Ês3 / Noc1 ±0.3 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.3 dB averaged over BWConfig | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês3 / Noc1 is the ratio of cell 3 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |
| 4.2.8 E-UTRA TDD Inter frequency re-selection in the existence of non-allowed CSG cell | Same as 4.2.7 |  |
| 4.2.9 E-UTRAN FDD - FDD intra frequency cell re-selection case for 5MHz bandwidth | Same as 4.2.1 |  |
| 4.2.10 E-UTRAN FDD - FDD reselection using an increased number of carriers | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.3 dB averaged over BWConfig Noc3 ±1.0 dB averaged over BWConfig Ês3 / Noc3 ±0.3 dB averaged over BWConfig Noc4 ±1.0 dB averaged over BWConfig Ês4 / Noc4 ±0.3 dB averaged over BWConfig | Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Noc3 is the AWGN on frequency 3 Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Noc4 is the AWGN on frequency 4 Ês4 / Noc4 is the ratio of cell 4 signal / AWGN |
| 4.2.11 E-UTRAN TDD - TDD reselection using an increased number of carriers | Same as 4.2.10 | Same as 4.2.10 |
| 4.2.12 E-UTRAN FDD - FDD Intra frequency case for Cat-M1 UE in normal coverage | Same as 4.2.1 |  |
| 4.2.13 E-UTRAN HD - FDD Intra frequency case for Cat-M1 UE in normal coverage | Same as 4.2.1 |  |
| 4.2.14 E-UTRA TDD - TDD intra frequency case for Cat-M1 UE in normal coverage | Same as 4.2.1 |  |
| 4.2.15 E-UTRAN FDD - FDD Intra frequency case for Cat-M1 UE in enhanced coverage | Same as 4.2.1 |  |
| 4.2.16 E-UTRAN HD - FDD Intra frequency case for Cat-M1 UE in enhanced coverage | Same as 4.2.1 |  |
| 4.2.17 E-UTRAN TDD - TDD Intra frequency case for Cat-M1 UE in enhanced coverage | Same as 4.2.1 |  |
| 4.2.18 HD-FDD Intra frequency cell reselection for Category NB1 UE in In-Band Mode under Normal Coverage | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig Êsn1 / Noc1 ±0.3 dB averaged over BWConfig Êsn2 / Noc1 ±0.3 dB averaged over BWConfig | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of eCell 1 signal / AWGN Êsn1 / Noc1 is the ratio of nCell1 signal / AWGN Êsn2 / Noc1 is the ratio of nCell2 signal / AWGN |
| 4.2.19 HD - FDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig Ês2 / Noc1 ±0.3 dB averaged over BWConfig Êsn1 / Noc1 ±0.3 dB averaged over BWConfig Êsn2 / Noc1 ±0.3 dB averaged over BWConfig | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of Cell 1 signal / AWGN Ês2 / Noc1 is the ratio of Cell 2 signal / AWGN Êsn1 / Noc1 is the ratio of nCell1 signal / AWGN Êsn2 / Noc1 is the ratio of nCell2 signal / AWGN |
| 4.2.20 E-UTRAN FDD - FDD Intra frequency case for UE Category 1bis | Noc ±1.0 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB averaged over BWConfig | NOTE: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 4.2.21 E-UTRAN TDD - TDD Intra frequency case for UE Category 1bis | Same as 4.2.20 | Same as 4.2.20 |
| 4.2.22 E-UTRAN FDD - FDD cell re-selection intra frequency case for UE configured with highSpeedEnhancedMeasFlag | Same as 4.2.1 |  |
| 4.2.23 E-UTRAN TDD - TDD cell re-selection intra frequency case for UE configured with highSpeedEnhancedMeasFlag | Same as 4.2.1 |  |
| 4.2.24 HD - FDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig Êsn1 / Noc1 ±0.3 dB averaged over BWConfig Êsn2 / Noc1 ±0.3 dB averaged over BWConfig | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of Cell 1 signal / AWGN Êsn1 / Noc1 is the ratio of nCell1 signal / AWGN Êsn2 / Noc1 is the ratio of nCell2 signal / AWGN |
| 4.2.28 E-UTRAN FDD - FDD Inter frequency case for CE UE in enhanced coverage | Same as 4.2.3 | Same as 4.2.3 |
| 4.2.29 E-UTRAN HD - FDD Inter frequency case for CE UE in enhanced coverage | Same as 4.2.3 | Same as 4.2.3 |
| 4.2.30 E-UTRAN TDD Inter frequency case for CE UE in enhanced coverage | Same as 4.2.3 | Same as 4.2.3 |
| 4.2.35 E-UTRAN TDD - TDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage | Same as 4.2.18 | Same as 4.2.18 |
| 4.2.36 E-UTRAN TDD - TDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage | Same as 4.2.19 | Same as 4.2.19 |
| 4.2.37 E-UTRAN TDD - TDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage | Same as 4.2.24 | Same as 4.2.24 |
| 4.2.38 HD - FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with serving cell RRM measurement relaxation | Same as 4.2.18 | Same as 4.2.18 |
| 4.3.1.1 E-UTRA FDD - UTRAN FDD cell reselection: UTRA FDD is of higher priority | E-UTRA cell Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig UTRA cell Ioc ±0.7 dB Ior / Ioc ±0.3 dB CPICH Ec / Ior ±0.1 dB | Notes: Noc is the AWGN on cell 1 (E-UTRA )frequency Ês / Noc is the ratio of cell 1 signal / AWGN Ioc is the AWGN on cell 2 (UTRA) frequency Ior / Ioc is the ratio of cell 2 signal / AWGN CPICH Ec / Ior is the fraction of cell 2 power assigned to the CPICH Physical channel |
| 4.3.1.2 E-UTRAN FDD - UTRAN FDD cell re-selection: UTRA FDD is of lower priority | E-UTRA cell Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig UTRA cell Ioc ±0.7 dB Ior / Ioc ±0.3 dB CPICH Ec / Ior ±0.1 dB | Notes: Noc is the AWGN on cell 1 (E-UTRA )frequency Ês / Noc is the ratio of cell 1 signal / AWGN Ioc is the AWGN on cell 2 (UTRA) frequency Ior / Ioc is the ratio of cell 2 signal / AWGN CPICH Ec / Ior is the fraction of cell 2 power assigned to the CPICH Physical channel |
| 4.3.1.3 EUTRA FDD-UTRA FDD cell reselection in fading propagation conditions: UTRA FDD is of lower priority | E-UTRA cell Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.6 dB averaged over BWConfig UTRA cell Ioc ±0.7 dB Ior / Ioc ±0.3 dB CPICH Ec / Ior ±0.1 dB | Notes: Noc is the AWGN on cell 1 (E-UTRA )frequency Ês / Noc is the ratio of cell 1 signal / AWGN Ês / Noc uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB Ioc is the AWGN on cell 2 (UTRA) frequency Ior / Ioc is the ratio of cell 2 signal / AWGN CPICH Ec / Ior is the fraction of cell 2 power assigned to the CPICH Physical channel |
| 4.3.1.4 E-UTRAN FDD - UTRAN FDD cell re-selection: UTRA FDD is of lower priority for 5MHz bandwidth | Same as 4.3.1.2 |  |
| 4.3.1.5 Idle mode FDD to UTRA FDD interRAT reselection | E-UTRA cell Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig UTRA cells Ioc ±0.7 dB Ior / Ioc ±0.3 dB CPICH Ec / Ior ±0.1 dB | Notes: E-UTRA Cell 1: Noc is the AWGN on cell 1 (E-UTRA )frequency Ês / Noc is the ratio of cell 1 signal / AWGN UTRA Cells 2 and 3: Ioc is the AWGN on cell 2 and cell 3 (UTRA) frequency Ior / Ioc is the ratio of cell 2 and cell 3 signal / AWGN CPICH Ec / Ior is the fraction of cell 2 and cell 3 power assigned to the CPICH Physical channel |
| 4.3.2 E-UTRA FDD - UTRAN TDD cell re-selection | E-UTRA cell Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig UTRA cell Ioc ±0.7 dB Îor / Ioc ±0.3 dB PCCPCH Ec/Ior ±0.1 dB DwPCH_Ec/Ior ±0.1 dB | NOTE: Noc is the AWGN on cell 1 frequency Ês / Noc is the ratio of cell 1 signal / AWGN Ioc is the AWGN on cell 2 frequency Îor / Ioc is the ratio of cell 2 signal / AWGN PCCPCH Ec / Ior is the fraction of cell 2 power assigned to the PCCPCH Physical channel DwPCH_Ec/Ior is the fraction of cell 2 power assigned to the DwPCH channel |
| 4.3.2A E-UTRA FDD to UTRA TDD cell re-selection for IncMon | E-UTRA cell Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig UTRA cells Ioc ±0.7 dB Îor / Ioc ±0.3 dB PCCPCH Ec/Ior ±0.1 dB DwPCH_Ec/Ior ±0.1 dB | NOTE: Noc is the AWGN on cell 1 frequency Ês / Noc is the ratio of cell 1 signal / AWGN Ioc is the AWGN on cell 2 and cell 3 frequency Îor / Ioc is the ratio of cell 2 and cell 3 signal / AWGN PCCPCH Ec / Ior is the fraction of cell 2 and cell 3 power assigned to the PCCPCH Physical channel DwPCH_Ec/Ior is the fraction of cell 2 and cell 3 power assigned to the DwPCH channel |
| 4.3.3 E-UTRAN TDD - UTRAN FDD cell re-selection: UTRA FDD is of lower priority | Same as 4.3.1.2 |  |
| 4.3.3A Idle mode TDD to UTRA FDD interRAT reselection | Same as 4.3.1.5 | Same as 4.3.1.5 |
| 4.3.4.1 E-UTRA TDD - UTRAN TDD cell re-selection : UTRA is of higher priority | E-UTRA cell Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig UTRA cell Ioc ±0.7 dB Îor / Ioc ±0.3 dB PCCPCH Ec/Ior ±0.1 dB DwPCH_Ec/Ior ±0.1 dB | NOTE: Noc is the AWGN on cell 1 frequency Ês / Noc is the ratio of cell 1 signal / AWGN Ioc is the AWGN on cell 2 frequency Îor / Ioc is the ratio of cell 2 signal / AWGN PCCPCH Ec / Ior is the fraction of cell 2 power assigned to the PCCPCH Physical channel DwPCH_Ec/Ior is the fraction of cell 2 power assigned to the DwPCH channel |
| 4.3.4.2 E-UTRA TDD - UTRAN TDD cell re-selection : UTRA is of lower priority | Same as 4.3.2 |  |
| 4.3.4.3 EUTRA TDD-UTRA TDD cell reselection in fading propagation conditions: UTRA TDD is of lower priority | E-UTRA cell Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.6 dB averaged over BWConfig UTRA cell Ioc ±0.7 dB Îor / Ioc ±0.3 dB PCCPCH Ec/Ior ±0.1 dB DwPCH_Ec/Ior ±0.1 dB | NOTE: Noc is the AWGN on cell 1 (E-UTRA) frequency Ês / Noc is the ratio of cell 1 signal / AWGN Each Ês / Noc uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB Ioc is the AWGN on cell 2 (UTRA) frequency Îor / Ioc is the ratio of cell 2 signal / AWGN PCCPCH Ec / Ior is the fraction of cell 2 power assigned to the PCCPCH Physical channel DwPCH_Ec/Ior is the fraction of cell 2 power assigned to the DwPCH channel |
| 4.3.4.4 E-UTRA TDD to UTRA TDD cell re-selection for IncMon | Same as 4.3.2 | Same as 4.3.2A |
| 4.4.1 E-UTRAN FDD - GSM cell re-selection | E-UTRA cell Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig GSM cell Signal level ±0.7 dB | Notes: Noc is the AWGN on cell 1 (E-UTRA )frequency Ês / Noc is the ratio of cell 1 signal / AWGN Cell 2 (GSM) has only the wanted signal, without AWGN |
| 4.4.2 E-UTRAN TDD - GSM cell re-selection | Same as 4.4.1 |  |
| 4.5.1.1 RRC IDLE / E-UTRAN to HRPD Cell re-selection / E-UTRAN FDD - HRPD cell re-selection: HRPD is of lower priority | E-UTRA cell Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig HRPD cell Ioc ±2.0 dB Îor / Ioc ±0.7 dB | NOTE: Noc is the AWGN on cell 1 frequency Ês / Noc is the ratio of cell 1 signal / AWGN Ioc is the AWGN on cell 2 (HRPD) frequency Ior / Ioc is the ratio of cell 2 signal / AWGN |
| 4.5.2.1 E-UTRAN TDD - HRPD Cell Reselection: HRPD is of Lower Priority | Same as 4.5.1.1 | Same as 4.5.1.1 |
| 5.1.1 E-UTRAN FDD-FDD Handover intra frequency case | Noc ±1.0 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB averaged over BWConfig | NOTE: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 5.1.2 E-UTRAN TDD-TDD Handover intra frequency case | Same as 5.1.1 | Same as 5.1.1 |
| 5.1.3 E-UTRAN FDD-FDD Handover inter frequency case | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.3 dB averaged over BWConfig | NOTE: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |
| 5.1.3_2 E-UTRAN FDD - FDD Inter frequency handover for UE Category 1bis | Same as 5.1.3 | Same as 5.1.3 |
| 5.1.4 E-UTRAN TDD-TDD Handover inter frequency case | Same as 5.1.3 | Same as 5.1.3 |
| 5.1.4_2 E-UTRAN TDD - TDD Inter frequency handover for UE Category 1bis | Same as 5.1.3 | Same as 5.1.3 |
| 5.1.5 E-UTRAN FDD-FDD inter-frequency Handover with unknown target cell | Same as 5.1.1 | Same as 5.1.1 |
| 5.1.5_2 E-UTRAN FDD - FDD Inter frequency handover: unknown target cell for UE Category 1bis | Same as 5.1.1 | Same as 5.1.1 |
| 5.1.6 E-UTRAN TDD-TDD inter-frequency Handover with unknown target cell | Same as 5.1.1 | Same as 5.1.1 |
| 5.1.6_2 E-UTRAN TDD - TDD Inter frequency handover; unknown Target Cell for UE Category 1bis | Same as 5.1.1 | Same as 5.1.1 |
| 5.1.7 E-UTRAN FDD-TDD Handover inter frequency case | Same as 5.1.3 | Same as 5.1.3 |
| 5.1.7_2 E-UTRAN FDD - TDD Inter frequency handover for UE Category 1bis | Same as 5.1.3 | Same as 5.1.3 |
| 5.1.8 E-UTRAN TDD-FDD Handover inter frequency case | Same as 5.1.3 | Same as 5.1.3 |
| 5.1.8_2 E-UTRAN TDD - FDD Inter frequency handover for UE Category 1bis | Same as 5.1.3 | Same as 5.1.3 |
| 5.1.9 E-UTRAN FDD-FDD Intra frequency handover for 5MHz bandwidth | Same as 5.1.1 | Same as 5.1.1 |
| 5.1.10 E-UTRAN FDD-FDD Handover intra frequency handover for UE category 0 | Same as 5.1.1 | Same as 5.1.1 |
| 5.1.11 E-UTRAN HD-FDD Handover intra frequency handover for UE category 0 | Same as 5.1.1 | Same as 5.1.1 |
| 5.1.12 E-UTRAN TDD-TDD Handover intra frequency handover for UE category 0 | Same as 5.1.1 | Same as 5.1.1 |
| 5.1.13 E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA | Same as 5.1.1 | Same as 5.1.1 |
| 5.1.14 E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA | Same as 5.1.1 | Same as 5.1.1 |
| 5.1.15 E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeA | Same as 5.1.1 | Same as 5.1.1 |
| 5.1.16 E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB | Same as 5.1.1 | Same as 5.1.1 |
| 5.1.17 E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB | Same as 5.1.1 | Same as 5.1.1 |
| 5.1.18 E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeB | Same as 5.1.1 | Same as 5.1.1 |
| 5.1.19 E-UTRAN FDD - FDD Intra frequency handover for UE Category 1bis | Noc ±1.0 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB averaged over BWConfig | NOTE: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 5.1.20 E-UTRAN TDD - TDD Intra frequency handover for UE Category 1bis | Same as 5.1.19 | Same as 5.1.19 |
| 5.1.28 E-UTRAN HD-FDD inter frequency handover for CE UEs in CEModeA | Same as 5.1.3 | Same as 5.1.3 |
| 5.1.34 E-UTRAN HD-FDD intra frequency handover for CE UEs in CEModeA without SFN acquisition | Same as 5.1.1 | Same as 5.1.1 |
| 5.1.41 E-UTRAN FDD – FDD Intra-band Inter-frequency sync DAPS handover | Same as 5.1.3 | Same as 5.1.3 |
| 5.1.42 E-UTRAN FDD – FDD Intra-band Inter-frequency async DAPS handover | Same as 5.1.3 | Same as 5.1.3 |
| 5.1.43 E-UTRAN FDD – FDD Inter-band Inter-frequency sync DAPS handover | Same as 5.1.3 | Same as 5.1.3 |
| 5.1.44 E-UTRAN FDD – FDD Inter-band Inter-frequency async DAPS handover | Same as 5.1.3 | Same as 5.1.3 |
| 5.1.45 E-UTRAN FDD - FDD Intra frequency DAPS handover | Same as 5.1.1 | Same as 5.1.1 |
| 5.1.46 E-UTRAN TDD - TDD Intra frequency DAPS handover | Same as 5.1.1 | Same as 5.1.1 |
| 5.1.47 E-UTRAN FDD - FDD Intra frequency conditional handover | Same as 5.1.1 | Same as 5.1.1 |
| 5.1.48 E-UTRAN TDD - TDD Intra frequency conditional handover | Same as 5.1.1 | Same as 5.1.1 |
| 5.1.49 E-UTRAN FDD - FDD Inter frequency conditional handover | Same as 5.1.3 | Same as 5.1.3 |
| 5.1.50 E-UTRAN TDD - TDD Inter frequency conditional handover | Same as 5.1.3 | Same as 5.1.3 |
| 5.1.51 E-UTRAN FDD - TDD Inter frequency conditional handover | Same as 5.1.3 | Same as 5.1.3 |
| 5.1.52 E-UTRAN TDD - FDD Inter frequency conditional handover | Same as 5.1.3 | Same as 5.1.3 |
| 5.1.53 E-UTRAN TDD – TDD Intra-band Inter-frequency sync DAPS handover | Same as 5.1.3 | Same as 5.1.3 |
| 5.1.54 E-UTRAN TDD – TDD Inter-band Inter-frequency sync DAPS handover | Same as 5.1.3 | Same as 5.1.3 |
| 5.1.55 E-UTRAN FDD - TDD inter-band inter-frequency synchronous DAPS handover | Same as 5.1.3 | Same as 5.1.3 |
| 5.1.56 E-UTRAN TDD - FDD inter-band inter-frequency synchronous DAPS handover | Same as 5.1.3 | Same as 5.1.3 |
| 5.1.57 E-UTRAN FDD – TDD Inter-band Inter-frequency async DAPS handover | Same as 5.1.3 | Same as 5.1.3 |
| 5.1.58 E-UTRAN TDD – FDD Inter-band Inter-frequency async DAPS handover | Same as 5.1.3 | Same as 5.1.3 |
| 5.2.1 E-UTRAN FDD - UTRAN FDD handover | E-UTRA cell Noc ±1.0dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig UTRA cell Ioc ±0.7 dB Ior / Ioc ±0.3 dB CPICH Ec/Ior ±0.1 dB | Notes: Noc is the AWGN on cell 1 (E-UTRA )frequency Ês / Noc is the ratio of cell 1 signal / AWGN Ioc is the AWGN on cell 2 (UTRA) frequency Ior / Ioc is the ratio of cell 2 signal / AWGN CPICH Ec / Ior is the fraction of cell 2 power assigned to the CPICH Physical channel |
| 5.2.2 E-UTRAN TDD - UTRAN FDD handover | Same as 5.2.1 | Same as 5.2.1 |
| 5.2.3 E-UTRAN FDD - GSM handover | E-UTRA Cell Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig GSM cell Signal level ±0.7 dB | NOTE: Noc is the AWGN on cell 1 frequency Ês / Noc is the ratio of cell 1 signal / AWGN Cell 2 (GSM) has only the wanted signal, without AWGN |
| 5.2.4 E-UTRA TDD - UTRA TDD handover | E-UTRA Cell: Noc ± 1.0 dB averaged over BWConfig Ês / Noc ± 0.3 dB averaged over BWConfig UTRA cell Ioc ± 0.7 dB Îor / Ioc ± 0.3 dB P-CCPCH_Ec / Ior ± 0.1 dB DwPCH_Ec / Ior ± 0.1 dB | NOTE: Noc is the AWGN on cell 1 frequency Ês / Noc is the ratio of cell 1 signal / AWGN Ioc is the AWGN on cell 2 frequency Îor / Ioc is the ratio of cell 2 signal / AWGN P-CCPCH_Ec / Ior is the fraction of cell 2 power assigned to the P-CCPCH physical channel. DwPCH_Ec / Ior is the fraction of cell 2 power assigned to the DwPCH channel |
| 5.2.5 E-UTRA FDD - UTRA TDD handover | Same as 5.2.4 | Same as 5.2.4 |
| 5.2.6 E-UTRA TDD - GSM handover | Same as 5.2.3 | Same as 5.2.3 |
| 5.2.7 E-UTRAN FDD - UTRAN FDD handover: unknown target cell | E-UTRA cell Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig UTRA cell Ioc ±0.7 dB Ior/Ioc ±0.3 dB CPICH Ec/Ior ±0.1 dB | NOTE: Noc is the AWGN on cell 1 frequency Ês / Noc is the ratio of cell 1 signal / AWGN Ioc is the AWGN on Cell 2 (UTRA) frequency Ior/Ioc is the ratio of Cell 2 signal/AWGN CPICH Ec/Ior is the fraction on Cell 2 power assigned to the CPICH physical channel |
| 5.2.8 E-UTRAN FDD - GSM handover: unknown target cell | Same as 5.2.3 | Same as 5.2.3 |
| 5.2.9 E-UTRAN TDD - GSM handover: unknown target cell | Same as 5.2.3 | Same as 5.2.3 |
| 5.2.10 E-UTRAN TDD - UTRAN TDD HO test: unknown target cell | E-UTRA cell Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig UTRA TDD cell Ioc ±0.7 dB Ior/Ioc ±0.3 dB PCCPCH_Ec /Ior ±0.1 dB DwPCH_Ec /Ior ±0.1 dB | NOTE: Noc is the AWGN on cell 1 frequency Ês / Noc is the ratio of cell 1 signal / AWGN Ioc is the AWGN on Cell 2 (UTRA TDD) frequency Ior/Ioc is the ratio of Cell 2 signal/AWGN PCCPCH_Ec /Ior is the fraction of Cell 2 power assigned to the PCCPCH physical channel DwPCH_Ec /Ior is the fraction of Cell 2 power assigned to the DwPCH physical channel |
| 5.2.11 E-UTRAN FDD - UTRAN FDD Handover for 5MHz Bandwidth | Same as 5.2.1 | Same as 5.2.1 |
| 5.3.1 RRC CONNECTED / Handover from E-UTRAN to non-3GPP RATs / E-UTRAN FDD - HRPD handover | E-UTRA cell Noc ±0.7 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig HRPD cell Ioc ±2.0 dB Îor / Ioc ±0.7 dB | NOTE: Noc is the AWGN on cell 1 frequency Ês / Noc is the ratio of cell 1 signal / AWGN Ioc is the AWGN on cell 2 (HRPD) frequency Ior / Ioc is the ratio of cell 2 signal / AWGN |
| 5.3.5 E-UTRAN TDD - HRPD handover | Same as 5.3.1 | Same as 5.3.1 |
| 6.1.1 E-UTRAN FDD Intra-frequency RRC Re-establishment | Same as 5.1.1 | Same as 5.1.1 |
| 6.1.1_2 E-UTRAN FDD Intra-frequency RRC Re-establishment for UE Category 1bis | Same as 6.1.1 | Same as 6.1.1 |
| 6.1.2 E-UTRAN FDD Inter-frequency RRC Re-establishment | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.3 dB averaged over BWConfig | NOTE: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |
| 6.1.2_2 E-UTRAN FDD Inter-frequency RRC Re-establishment for UE Category 1bis | Same as 6.1.2 | Same as 6.1.2 |
| 6.1.3 E-UTRAN TDD Intra-frequency RRC Re-establishment | Same as 6.1.1 | Same as 6.1.1 |
| 6.1.3_2 E-UTRAN TDD Intra-frequency RRC Re-establishment for UE Category 1bis | Same as 6.1.1 | Same as 6.1.1 |
| 6.1.4 E-UTRAN TDD Inter-frequency RRC Re-establishment | Same as 6.1.3 | Same as 6.1.3 |
| 6.1.4_2 E-UTRAN TDD Inter-frequency RRC Re-establishment for UE Category 1bis | Same as 6.1.2 | Same as 6.1.2 |
| 6.1.5 E-UTRAN FDD Intra-frequency RRC Re-establishment for 5MHz Bandwidth | Same as 6.1.1 | Same as 6.1.1 |
| 6.1.6 E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for UE category 0 | Noc ±1.0 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB averaged over BWConfig | NOTE: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 6.1.7 E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for UE category 0 | Same as 6.1.6 | Same as 6.1.6 |
| 6.1.8 E-UTRAN TDD Intra-frequency RRC Re-establishment for UE category 0 | Same as 6.1.6 | Same as 6.1.6 |
| 6.1.9 E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA | Same as 6.1.1 | Same as 6.1.1 |
| 6.1.10 E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA | Same as 6.1.1 | Same as 6.1.1 |
| 6.1.11 E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA | Same as 6.1.1 | Same as 6.1.1 |
| 6.1.12 E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB | Same as 6.1.1 | Same as 6.1.1 |
| 6.1.13 E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB | Same as 6.1.1 | Same as 6.1.1 |
| 6.1.14 E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB | Same as 6.1.1 | Same as 6.1.1 |
| 6.1.15 HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in In-Band mode under enhanced coverage | Noc ±1.0 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig Êsn1 / Noc ±0.3 dB averaged over BWConfig Êsn2 / Noc ±0.3 dB averaged over BWConfig | NOTE: Ês1 / Noc is the ratio of eCell 1 signal / AWGN Êsn1 / Noc is the ratio of nCell 1 signal / AWGN Êsn2 / Noc is the ratio of nCell 2 signal / AWGN |
| 6.1.16 HD-FDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage | Same as 6.1.15 | Same as 6.1.15 |
| 6.1.18 E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for CE UE in CEModeA | Same as 6.1.2 | Same as 6.1.2 |
| 6.1.23 E-UTRAN TDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage | Same as 6.1.15 | Same as 6.1.15 |
| 6.1.24 E-UTRAN TDD - TDD Intra-frequency RRC Re-establishment for UE category NB1 in In-Band mode under enhanced coverage | Same as 6.1.15 | Same as 6.1.15 |
| 6.2.1 E-UTRAN FDD - Contention Based Random Access Test | Test 1 and Test 2: Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig Uplink absolute power measurement ±1.0 dB Uplink relative power measurement ±0.7 dB ±3Ts Uplink signal transmit timing relative to downlink | NOTE: Ês1 / Noc1 is the ratio of cell 1 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 6.2.1_2 E-UTRAN FDD - Contention Based Random Access Test for UE Category 1bis | Same as 6.2.1 | Same as 6.2.1 |
| 6.2.2 E-UTRAN FDD - Non Contention Based Random Access Test | Same as 6.2.1 | Same as 6.2.1 |
| 6.2.2_2 E-UTRAN FDD - Non Contention Based Random Access Test for UE Category 1bis | Same as 6.2.1 | Same as 6.2.1 |
| 6.2.3 E-UTRAN TDD - Contention Based Random Access Test | Same as 6.2.1 | Same as 6.2.1 |
| 6.2.3_2 E-UTRAN TDD - Contention Based Random Access Test for UE Category 1bis | Same as 6.2.1 | Same as 6.2.1 |
| 6.2.4 E-UTRAN TDD - Non Contention Based Random Access Test | Same as 6.2.1 | Same as 6.2.1 |
| 6.2.4_2 E-UTRAN TDD - Non Contention Based Random Access Test for UE Category 1bis | Same as 6.2.1 | Same as 6.2.1 |
| 6.2.5 E-UTRAN FDD - Contention Based Random Access Test for 5MHz bandwidth | Same as 6.2.1 | Same as 6.2.1 |
| 6.2.6 E-UTRAN FDD - Non-contention Based Random Access Test for 5MHz bandwidth | Same as 6.2.1 | Same as 6.2.1 |
| 6.2.7 E-UTRAN FDD - Non-Contention Based Random Access Test for SCell in sTAG | Test 1, Test 2 and Test 3 Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.3 dB averaged over BWConfig Uplink absolute power measurement ±1.0 dB Uplink relative power measurement ±0.7 dB ±3Ts Uplink signal transmit timing relative to downlink | NOTE: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 6.2.8 E-UTRAN TDD - Non-Contention Based Random Access Test for SCell in sTAG | Same as 6.2.7 | Same as 6.2.7 |
| 6.2.9 3DL/3UL TDD CA Non-Contention Based Random Access Test for 2 SCells | Test 1, Test 2 and Test 3 Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.3 dB averaged over BWConfig Noc3 ±1.0 dB averaged over BWConfig Ês3 / Noc2 ±0.3 dB averaged over BWConfig Uplink absolute power measurement ±1.0 dB Uplink relative power measurement ±0.7 dB ±3Ts Uplink signal transmit timing relative to downlink | NOTE: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Noc3 is the AWGN on cell 3 frequency Ês3 / Noc3 is the ratio of cell 3 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 6.2.10 E-UTRAN FDD - Contention based random access test for Cat-M1 UEs in normal coverage | Test 1 and Test 2: Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig Uplink absolute power measurement ±1.0 dB Uplink relative power measurement ±0.7 dB ±3Ts Uplink signal transmit timing relative to downlink | NOTE: Ês1 / Noc1 is the ratio of cell 1 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 6.2.11 E-UTRAN HD-FDD - Contention based random access test for Cat-M1 UEs in normal coverage | Same as 6.2.10 | Same as 6.2.10 |
| 6.2.12 E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage | Same as 6.2.10 | Same as 6.2.10 |
| 6.2.13 E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage | Test 1 and Test 2: Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig Uplink absolute power measurement ±1.0 dB Uplink relative power measurement ±0.7 dB ±3Ts Uplink signal transmit timing relative to downlink | NOTE: Ês1 / Noc1 is the ratio of cell 1 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 6.2.14 E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage | Same as 6.2.13 | Same as 6.2.13 |
| 6.2.15 E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage | Same as 6.2.13 | Same as 6.2.13 |
| 6.2.16 Contention Based Random Access Test for UE category NB1 UEs In-band mode in normal coverage | Noc ±1.0 dB averaged over NB-IoT Cell BWConfig Ês / Noc ±0.3 dB averaged over NB-IoT Cell BWConfig Uplink absolute power measurement ±1.0 dB Uplink relative power measurement ±0.7 dB ±3Ts Uplink signal transmit timing relative to downlink | NOTE: Ês / Noc is the ratio of Ncell 1 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 6.2.17 Contention Based Random Access Test for UE category NB1 UEs In-band mode in Enhanced Coverage | Same as 6.2.16 | Same as 6.2.16 |
| 6.2.18 Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs In-band mode in Enhanced Coverage | Same as 6.2.16 | Same as 6.2.16 |
| 6.2.19 TDD Contention Based Random Access Test for UE category NB1 UEs In-band mode in normal coverage | Same as 6.2.16 | Same as 6.2.16 |
| 6.2.20 TDD Contention Based Random Access Test for UE category NB1 UEs In-band mode in enhanced coverage | Same as 6.2.16 | Same as 6.2.16 |
| 6.2.21 TDD Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs In-band mode in Enhanced Coverage | Same as 6.2.17 | Same as 6.2.17 |
| 6.3.1 Redirection from E-UTRAN FDD to UTRAN FDD | E-UTRA cell Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig UTRA cell Ioc ±0.7 dB Ior/Ioc ±0.3 dB CPICH Ec/Ior ±0.1 dB SCH Ec/Ior ±0.1 dB | NOTE: Noc is the AWGN on cell 1 frequency Ês / Noc is the ratio of cell 1 signal / AWGN Ioc is the AWGN on Cell 2 (UTRA) frequency Ior/Ioc is the ratio of Cell 2 signal/AWGN CPICH Ec/Ior and SCH_Ec/Ior are the fractions of Cell 2 power assigned to the CPICH and SCH physical channels |
| 6.3.1_2 Redirection from E-UTRAN FDD to UTRAN FDD for UE Category 1bis | Same as 6.3.1 | Same as 6.3.1 |
| 6.3.2 Redirection from E-UTRAN TDD to UTRAN FDD | Same as 6.3.1 | Same as 6.3.1 |
| 6.3.2_2 Redirection from E-UTRAN TDD to UTRAN FDD for UE Category 1bis | Same as 6.3.1 | Same as 6.3.1 |
| 6.3.3 Redirection from E-UTRAN FDD to GERAN when System Information is provided | E-UTRA Cell Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig GSM cell Signal level ±0.7 dB | NOTE: Noc is the AWGN on cell 1 frequency Ês / Noc is the ratio of cell 1 signal / AWGN Cell 2 (GSM) has only the wanted signal, without AWGN |
| 6.3.3_2 Redirection from E-UTRAN FDD to GERAN when System Information is provided for UE Category 1bis | Same as 6.3.3 | Same as 6.3.3 |
| 6.3.4 Redirection from E-UTRAN TDD to GERAN when System Information is provided | Same as 6.3.3 | Same as 6.3.3 |
| 6.3.4_2 Redirection from E-UTRAN TDD to GERAN when System Information is provided for UE Category 1bis | Same as 6.3.3 | Same as 6.3.3 |
| 6.3.5 E-UTRA TDD RRC connection release redirection to UTRA TDD | E-UTRA cell Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig UTRA cell Ioc ±0.7 dB Ior/Ioc ±0.3 dB PCCPCH_Ec/Ior ±0.1 dB DwPCH_Ec/Ior ±0.1 dB | NOTE: Noc is the AWGN on cell 1 frequency Ês / Noc is the ratio of cell 1 signal / AWGN Ioc is the AWGN on cell 2 frequency Îor / Ioc is the ratio of cell 2 signal / AWGN PCCPCH Ec / Ior is the fraction of cell 2 power assigned to the PCCPCH Physical channel DwPCH_Ec/Ior is the fraction of cell 2 power assigned to the DwPCH channel |
| 6.3.5_2 E-UTRA TDD RRC connection release redirection to UTRA TDD for UE Category 1bis | Same as 6.3.5 | Same as 6.3.5 |
| 6.3.6 E-UTRA FDD RRC connection release redirection to UTRA TDD | Same as 6.3.5 | Same as 6.3.5 |
| 6.3.6_2 E-UTRA FDD RRC connection release redirection to UTRA TDD for UE Category 1bis | Same as 6.3.5 | Same as 6.3.5 |
| 6.3.7 E-UTRA TDD RRC connection release redirection to UTRA TDD without SI provided | Same as 6.3.5 | Same as 6.3.5 |
| 6.3.7_2 E-UTRA TDD RRC connection release redirection to UTRA TDD without SI provided for UE Category 1bis | Same as 6.3.5 | Same as 6.3.5 |
| 6.3.8 E-UTRA FDD RRC connection release redirection to UTRA TDD without SI provided | Same as 6.3.5 | Same as 6.3.5 |
| 6.3.8_2 E-UTRA FDD RRC connection release redirection to UTRA TDD without SI provided for UE Category 1bis | Same as 6.3.5 | Same as 6.3.5 |
| 6.3.9 Redirection from E-UTRAN FDD to UTRAN FDD without System Information | Same as 6.3.1 | Same as 6.3.1 |
| 6.3.9_2 Redirection from E-UTRAN FDD to UTRAN FDD without System Information for UE Category 1bis | Same as 6.3.1 | Same as 6.3.1 |
| 6.3.10 Redirection from E-UTRAN FDD to GERAN when System Information is not provided | Same as 6.3.3 | Same as 6.3.3 |
| 6.3.10_2 Redirection from E-UTRAN FDD to GERAN when System Information is not provided for UE Category 1bis | Same as 6.3.3 | Same as 6.3.3 |
| 6.3.11 Redirection from E-UTRAN TDD to GERAN when System Information is not provided | Same as 6.3.3 | Same as 6.3.3 |
| 6.3.11_2 Redirection from E-UTRAN TDD to GERAN when System Information is not provided for UE Category 1bis | Same as 6.3.3 | Same as 6.3.3 |
| 6.3.12 E-UTRAN TDD RRC connection release redirection to UTRAN FDD without SI provided | Same as 6.3.1 | Same as 6.3.1 |
| 6.3.12_2 E-UTRAN TDD RRC connection release redirection to UTRAN FDD without SI provided for UE Category 1bis | Same as 6.3.1 | Same as 6.3.1 |
| 7.1.1 E-UTRAN FDD - UE Transmit Timing Accuracy | Noc ±3.0 dB averaged over BWConfig Ês / Noc ±0.3 dB ±3Ts Uplink signal transmit timing relative to downlink ±0.5Ts relative during UE timing adjustment | NOTE: Ês / Noc is the ratio of cell 1 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 7.1.1_2 E-UTRAN FDD - UE Transmit Timing Accuracy for UE category 1bis | Same as 7.1.1 | Same as 7.1.1 |
| 7.1.2 E-UTRAN TDD - UE Transmit Timing Accuracy | Same as 7.1.1 | Same as 7.1.1 |
| 7.1.2_2 E-UTRAN TDD - UE Transmit Timing Accuracy for UE category 1bis | Same as 7.1.1 | Same as 7.1.1 |
| 7.1.3 E-UTRAN FDD - UE Transmit Timing Accuracy Tests for SCell | Noc1 ±3.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB Noc2 ±3.0 dB averaged over BWConfig Ês2 / Noc2 ±0.3 dB ±3TS Uplink signal transmit timing relative to downlink ±0.5TS relative during UE timing adjustment | NOTE: Ês / Noc is the ratio of cell signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 7.1.3_1 E-UTRAN FDD - UE Transmit Timing Accuracy Tests for SCell (Release 12 and forward) | Same as 7.1.3 | Same as 7.1.3 |
| 7.1.4 E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell | Same as 7.1.3 | Same as 7.1.3 |
| 7.1.4_1 E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell (Release 12 and forward) | Same as 7.1.3 | Same as 7.1.3 |
| 7.1.4A E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell for 20 MHz +10 MHz bandwidth | Same as 7.1.3 | Same as 7.1.3 |
| 7.1.5 E-UTRAN FDD - UE Transmit Timing Accuracy Tests for 5MHz Bandwidth | Same as 7.1.1 | Same as 7.1.1 |
| 7.1.6 E-UTRAN FDD - UE Transmit Timing Accuracy Tests for SCell in sTAG | Noc1 ±3.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB Noc2 ±3.0 dB averaged over BWConfig Ês2 / Noc2 ±0.3 dB ±3Ts Uplink signal transmit timing relative to downlink ±0.5Ts relative during UE timing adjustment | NOTE: Ês / Noc is the ratio of cell signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 7.1.7 E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG | Same as 7.1.6 | Same as 7.1.6 |
| 7.1.7A E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +20MHz bandwidth | Same as 7.1.6 | Same as 7.1.6 |
| 7.1.7B E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +10MHz bandwidth | Same as 7.1.6 | Same as 7.1.6 |
| 7.1.10 E-UTRAN FDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA | Same as 7.1.1 | Same as 7.1.1 |
| 7.1.11 E-UTRAN HD-FDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA | Same as 7.1.1 | Same as 7.1.1 |
| 7.1.12 E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA | Same as 7.1.1 | Same as 7.1.1 |
| 7.1.13 3DL/3UL TDD CA UE Transmit Timing Accuracy Tests for 2 Scells | Noc1 ±3.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB Noc2 ±3.0 dB averaged over BWConfig Ês2 / Noc2 ±0.3 dB Noc3 ±3.0 dB averaged over BWConfig Ês3 / Noc3 ±0.3 dB ±3Ts Uplink signal transmit timing relative to downlink ±0.5Ts relative during UE timing adjustment | NOTE: Ês / Noc is the ratio of cell signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 7.1.14 E-UTRAN FDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB | Same as 7.1.1 | Same as 7.1.1 |
| 7.1.15 E-UTRAN HD-FDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB | Same as 7.1.1 | Same as 7.1.1 |
| 7.1.16 E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB | Same as 7.1.1 | Same as 7.1.1 |
| 7.1.17 E-UTRAN HD-FDD - UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage | Noc ±3.0 dB averaged over BWConfig Ês / Noc ±0.3 dB ±3Ts Uplink signal transmit timing relative to downlink ±0.5Ts relative during UE timing adjustment | NOTE: Ês / Noc is the ratio of Ncell 1 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 7.1.18 E-UTRAN HD-FDD - UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under enhanced coverage | Noc ±3.0 dB averaged over BWConfig Ês / Noc ±0.3 dB ±3Ts Uplink signal transmit timing relative to downlink | NOTE: Ês / Noc is the ratio of Ncell 1 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 7.1.21 E-UTRAN FDD - UE Transmit Timing Accuracy Tests for CE UE in CEModeA | Same as 7.1.1 | Same as 7.1.1 |
| 7.1.22 E-UTRAN HD-FDD - UE Transmit Timing Accuracy Tests for CE UE in CEModeA | Same as 7.1.1 | Same as 7.1.1 |
| 7.1.23 E-UTRAN TDD - UE Transmit Timing Accuracy Tests for CE UE in CEModeA | Same as 7.1.1 | Same as 7.1.1 |
| 7.1.24 E-UTRAN FDD - UE Transmit Timing Accuracy Tests for CE UE in CEModeB | Same as 7.1.1 | Same as 7.1.1 |
| 7.1.27 E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage | Same as 7.1.17 | Same as 7.1.17 |
| 7.1.28 E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Category NB1 UE In-band mode under enhanced coverage | Same as 7.1.18 | Same as 7.1.18 |
| 7.2.1 E-UTRAN FDD - UE Timing Advance Adjustment Accuracy | Noc1 ±3.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB Timing Advance Adjustment: ±0.5Ts | NOTE: Ês1 / Noc1 is the ratio of cell 1 signal / AWGN The timing unit TS = 1/(15000 * 2048 ) seconds, as defined in 3GPP TS.36.211 [9] |
| 7.2.1_2 E-UTRAN FDD - UE Timing Advance Adjustment Accuracy for UE category 1bis | Same as 7.2.1 | Same as 7.2.1 |
| 7.2.2 E-UTRAN TDD - UE Timing Advance Adjustment Accuracy | Same as 7.2.1 | Same as 7.2.1 |
| 7.2.2_2 E-UTRAN TDD - UE Timing Advance Adjustment Accuracy for UE category 1bis | Same as 7.2.1 | Same as 7.2.1 |
| 7.2.3 E-UTRAN FDD - UE Timing Advance Adjustment Accuracy Test for 5MHz Bandwidth | Same as 7.2.1 | Same as 7.2.1 |
| 7.2.4 E-UTRAN FDD - UE Timing Advance Adjustment Accuracy Test for SCell in sTAG | Noc1 ±3.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB Noc2 ±3.0 dB averaged over BWConfig Ês2 / Noc2 ±0.3 dB Timing Advance Adjustment: ±0.5Ts | NOTE: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN The timing unit TS = 1/(15000 * 2048 ) seconds, as defined in 3GPP TS.36.211 [9] |
| 7.2.5 E-UTRAN TDD - UE Timing Advance Adjustment Accuracy Test for SCell in sTAG | Same as 7.2.4 | Same as 7.2.4 |
| 7.2.5A E-UTRAN TDD - UE Timing Advance Adjustment Accuracy Test for SCell in sTAG for 20MHz +20MHz bandwidth | Same as 7.2.1 | Same as 7.2.1 |
| 7.2.5B E-UTRAN TDD - UE Timing Advance Adjustment Accuracy Test for SCell in sTAG for 20MHz +10MHz bandwidth | Same as 7.2.1 | Same as 7.2.1 |
| 7.2.6 E-UTRAN FDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA | Same as 7.2.1 | Same as 7.2.1 |
| 7.2.7 E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA | Same as 7.2.1 | Same as 7.2.1 |
| 7.2.8 E-UTRAN TDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA | Same as 7.2.1 | Same as 7.2.1 |
| 7.2.9 HD-FDD UE Timing Advance Adjustment Accuracy Test for Category NB1 UE in Standalone Mode under Enhance Coverage | Same as 7.2.1 | Same as 7.2.1 |
| 7.2.10 E-UTRAN FDD UE Timing Advance Adjustment Accuracy Test in CEModeB | Same as 7.2.1 | Same as 7.2.1 |
| 7.2.11 E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test in CEModeB | Same as 7.2.1 | Same as 7.2.1 |
| 7.2.12 E-UTRAN TDD UE Timing Advance Adjustment Accuracy Test in CEModeB | Same as 7.2.1 | Same as 7.2.1 |
| 7.2.13 E-UTRAN FDD - UE Timing Advance Adjustment Accuracy Test for sTTI and Short Processing Time | Same as 7.2.1 | Same as 7.2.1 |
| 7.2.14 E-UTRAN TDD - UE Timing Advance Adjustment Accuracy Test for sTTI and Short Processing Time | Same as 7.2.1 | Same as 7.2.1 |
| 7.2.15 E-UTRAN TDD - TDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Enhanced Coverage | Same as 7.2.1 | Same as 7.2.1 |
| 7.3.1 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync | ± 0.6dB (Subtest 1&2, AWGN conditions) ± 0.8dB (Subtest 3, Fading conditions, single antenna transmission) ± 0.9dB (Subtest 4, Fading conditions, two antenna transmission) | Subtests 1 & 2: Overall system uncertainty for AWGN condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB Subtests 3: Overall system uncertainty for fading condition comprises three quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Average signal-to-noise ratio uncertainty 2 + Signal-to-noise ratio variation2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Signal-to-noise ratio variation ±0.5 dB Fading profile power uncertainty ±0.5 dB for single Tx Subtest 4: Same calculations as for subtest 3 but with Fading profile uncertainty of ±0.7 for two Tx. |
| 7.3.1_1 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync with 4 Rx antenna ports | Same as 7.3.1 | Same as 7.3.1 |
| 7.3.2 E-UTRAN FDD Radio Link Monitoring Test for In-sync | ± 0.8dB (Subtest 1, Fading conditions, single antenna transmission) ± 0.9dB (Subtest 2, Fading conditions, two antenna transmission) | Subtest 1: See 7.3.1 subtest 3 Subtest 2: See 7.3.1 subtest 4 |
| 7.3.2_1 E-UTRAN FDD Radio Link Monitoring Test for In-sync with 4 Rx antenna ports | Same as 7.3.2 | Same as 7.3.2 |
| 7.3.3 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync | Same as 7.3.1 | Same as 7.3.1 |
| 7.3.3_1 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync with 4 Rx antenna ports | Same as 7.3.1 | Same as 7.3.1 |
| 7.3.4 E-UTRAN TDD Radio Link Monitoring Test for In-sync | Same as 7.3.2 | Same as 7.3.2 |
| 7.3.4_1 E-UTRAN TDD Radio Link Monitoring Test for In-sync with 4 Rx antenna ports | Same as 7.3.4 | Same as 7.3.4 |
| 7.3.5 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync in DRX | ±0.9dB (Subtest 1, Fading conditions, two antenna transmission) ± 0.6dB (Subtest 2, AWGN conditions) | Subtest 1: See 7.3.1, subtest 4 Subtest 2: See 7.3.1, subtest 1 |
| 7.3.5 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync in DRX with 4 Rx antenna ports | Same as 7.3.5 | Same as 7.3.5 |
| 7.3.6 E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX | ± 0.6dB (AWGN conditions) | See 7.3.1, subtest 1 |
| 7.3.6_1 E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX with 4 Rx antenna ports | Same as 7.3.6 | Same as 7.3.6 |
| 7.3.7 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX | ±0.9dB (Subtest 1, Fading conditions, two antenna transmission) ± 0.6dB (Subtest 2, AWGN conditions) | Subtest 1: See 7.3.1, subtest 4 Subtest 2: See 7.3.1, subtest 1 |
| 7.3.7_1 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX with 4 Rx antenna ports | Same as 7.3.7 | Same as 7.3.7 |
| 7.3.8 E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX | ± 0.6dB (AWGN conditions) | See 7.3.1, subtest 1 |
| 7.3.8_1 E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX with 4 Rx antenna ports | Same as 7.3.8 | Same as 7.3.8 |
| 7.3.9 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with Non MBSFN ABS (eICIC) | Cell 1: Ês1/Noc ± 0.9 dB Cell 2: Ês2/Noc ± 0.9 dB Ês2/ Ês1 ± 1.1 dB | System uncertainties comprise a number of quantities which are selected according to their applicability in the equations below. Uncertainties are assumed to be uncorrelated, so can be root sum squared. For Ês/Noc uncertainty AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. For Ês2/Ês1 uncertainty AWGN flatness effect has been excluded, because it would cancel out for the ratio Es2/Es1. Ês1/Noc or Ês2/Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Ês2/Ês1 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + Cell 2 SNR uncertainty 2 + Cell 2 Fading profile power uncertainty 2) Component uncertainties: Cell 1 SNR uncertainty (Es1 / Noc ratio before fading) ±0.3 dB Cell 2 SNR uncertainty (Es2 / Noc ratio before fading) ±0.3 dB Fading profile power uncertainty ±0.7 dB for 2 Tx AWGN flatness and signal flatness ±2.0 dB |
| 7.3.10 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with Non MBSFN ABS (eICIC) | Same as 7.3.9 | Same as 7.3.9 |
| 7.3.11 E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with Non MBSFN ABS (eICIC) | Cell 1: Ês1/Noc ± 0.9 dB Cell 2: Ês2/Noc ± 0.9 dB Ês2/ Ês1 ± 1.1 dB | System uncertainties comprise a number of quantities which are selected according to their applicability in the equations below. Uncertainties are assumed to be uncorrelated, so can be root sum squared. For Ês/Noc uncertainty AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. For Ês2/Ês1 uncertainty AWGN flatness effect has been excluded, because it would cancel out for the ratio Es2/Es1. Ês1/Noc or Ês2/Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Ês2/Ês1 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + Cell 2 SNR uncertainty 2 + Cell 2 Fading profile power uncertainty 2) Component uncertainties: Cell 1 SNR uncertainty (Es1 / Noc ratio before fading) ±0.3 dB Cell 2 SNR uncertainty (Es2 / Noc ratio before fading) ±0.3 dB Fading profile power uncertainty ±0.7 dB for 2 Tx AWGN flatness and signal flatness ±2.0 dB |
| 7.3.12 E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with Non MBSFN ABS (eICIC) | Same as 7.3.11 | Same as 7.3.11 |
| 7.3.13 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS (eICIC) | Same as 7.3.9 | Same as 7.3.9 |
| 7.3.14 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS (eICIC) | Same as 7.3.9 | Same as 7.3.9 |
| 7.3.15 E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS (eICIC) | Same as 7.3.11 | Same as 7.3.11 |
| 7.3.16 E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS (eICIC) | Same as 7.3.11 | Same as 7.3.11 |
| 7.3.17 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS assistance information and Non MBSFN ABS (feICIC) | Cell 1: Ês1/Noc ± 0.9 dB Cell 2: Ês2/Noc ± 0.9 dB Cell 3: Ês3/Noc ± 0.9 dB Ês2/ Ês1 ± 1.1 dB Ês3/ Ês1 ± 1.1 dB | System uncertainties comprise a number of quantities which are selected according to their applicability in the equations below. Uncertainties are assumed to be uncorrelated, so can be root sum squared. For Ês/Noc uncertainty AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. For Ês2/Ês1 and Ês3/Ês1 uncertainty AWGN flatness effect has been excluded, because it would cancel out for the ratio Esx/Es1. Ês1/Noc, Ês2/Noc or Ês3/Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Ês2/Ês1 or Ês3/Ês1 uncertainty = SQRT (Cell 1 SNR uncertainty 2 + Cell 1 Fading profile power uncertainty 2 + Cell 2 SNR uncertainty 2 + Cell 2 Fading profile power uncertainty 2) Component uncertainties: Cell 1 SNR uncertainty (Es1 / Noc ratio before fading) ±0.3 dB Cell 2 SNR uncertainty (Es2 / Noc ratio before fading) ±0.3 dB Cell 3 SNR uncertainty (Es3 / Noc ratio before fading) ±0.3 dB Fading profile power uncertainty ±0.7 dB for 2 Tx AWGN flatness and signal flatness ±2.0 dB |
| 7.3.18 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS assistance information and Non MBSFN ABS (feICIC) | Same as 7.3.17 | Same as 7.3.17 |
| 7.3.19 E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with CRS assistance information and Non MBSFN ABS (feICIC) | Same as 7.3.17 | Same as 7.3.17 |
| 7.3.20 E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with CRS assistance information and Non MBSFN ABS (feICIC) | Same as 7.3.17 | Same as 7.3.17 |
| 7.3.21 E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with CRS assistance information and MBSFN ABS (feICIC) | Same as 7.3.17 | Same as 7.3.17 |
| 7.3.22 E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with CRS assistance information and MBSFN ABS (feICIC) | Same as 7.3.17 | Same as 7.3.17 |
| 7.3.23 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync for 5MHz Bandwidth | ± 0.9dB (Subtest 4, Fading conditions, two antenna transmission) | See 7.3.1, subtest 4 |
| 7.3.23_1 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync for 5MHz Bandwidth with 4 Rx antenna ports | Same as 7.3.23 | Same as 7.3.23 |
| 7.3.24 E-UTRAN FDD Radio Link Monitoring Test for In-sync for 5MHz Bandwidth | ± 0.9dB (Subtest 2, Fading conditions, two antenna transmission) | See 7.3.2, subtest 2 |
| 7.3.24_1 E-UTRAN FDD Radio Link Monitoring Test for In-sync for 5MHz Bandwidth with 4 Rx antenna ports | Same as 7.3.24 | Same as 7.3.24 |
| 7.3.25 E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX for 5MHz Bandwidth | ±0.6dB (AWGN conditions) | See 7.3.1, subtest 1 |
| 7.3.25_1 E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX for 5MHz Bandwidth with 4 Rx antenna ports | Same as 7.3.25 | Same as 7.3.25 |
| 7.3.26 E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for UE category 0 | ± 0.9dB (Fading conditions, two antenna transmission) | Overall system uncertainty for fading condition comprises three quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Average signal-to-noise ratio uncertainty 2 + Signal-to-noise ratio variation2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Signal-to-noise ratio variation ±0.5 dB Fading profile power uncertainty ±0.7 dB for two Tx |
| 7.3.26_2 E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for UE Category 1bis | Same as 7.3.26 | Same as 7.3.26 |
| 7.3.27 E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync for UE category 0 | Same as 7.3.26 | Same as 7.3.26 |
| 7.3.27_2 E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync for UE Category 1bis | Same as 7.3.26 | Same as 7.3.26 |
| 7.3.28 E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0 | ± 0.6dB (AWGN conditions) | Overall system uncertainty for AWGN condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |
| 7.3.28_2 E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE Category 1bis | Same as 7.3.28 | Same as 7.3.28 |
| 7.3.29 E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE category 0 | Same as 7.3.28 | Same as 7.3.28 |
| 7.3.29_2 E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category 1bis | Same as 7.3.28 | Same as 7.3.28 |
| 7.3.30 E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for UE category 0 | Same as 7.3.26 | Same as 7.3.26 |
| 7.3.31 E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync for UE category 0 | Same as 7.3.26 | [Same as 7.3.26 |
| 7.3.32 E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0 | Same as 7.3.28 | Same as 7.3.28 |
| 7.3.33 E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE category 0 | Same as 7.3.28 | Same as 7.3.28 |
| 7.3.34 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for UE category 0 | Same as 7.3.26 | Same as 7.3.26 |
| 7.3.34_2 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for UE Category 1bis | Same as 7.3.26 | Same as 7.3.26 |
| 7.3.35 E-UTRAN TDD Radio Link Monitoring Test for In-sync for UE category 0 | Same as 7.3.26 | Same as 7.3.26 |
| 7.3.35_2 E-UTRAN TDD Radio Link Monitoring Test for In-sync for UE Category 1bis | Same as 7.3.26 | Same as 7.3.26 |
| 7.3.36 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0 | Same as 7.3.28 | Same as 7.3.28 |
| 7.3.36_2 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE Category 1bis | Same as 7.3.28 | Same as 7.3.28 |
| 7.3.37 E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE category 0 | Same as 7.3.28 | Same as 7.3.28 |
| 7.3.37_2 E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE Category 1bis | Same as 7.3.28 | Same as 7.3.28 |
| 7.3.38 E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC | ± 0.9dB (Fading conditions, two antenna transmission) | Overall system uncertainty for fading condition comprises three quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Average signal-to-noise ratio uncertainty 2 + Signal-to-noise ratio variation2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Signal-to-noise ratio variation ±0.5 dB Fading profile power uncertainty ±0.7 dB for two Tx |
| 7.3.38_1 E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with 4 Rx antenna ports | Same as 7.3.38 | Same as 7.3.38 |
| 7.3.39 E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in asynchronous DC | Same as 7.3.38 | Same as 7.3.38 |
| 7.3.39_1 E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in asynchronous DC with 4 Rx antenna ports | Same as 7.3.39 | Same as 7.3.39 |
| 7.3.40 E-UTRAN TDD-TDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC | Same as 7.3.38 | Same as 7.3.38 |
| 7.3.41 E-UTRAN FDD-FDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity | ± 0.6dB (AWGN conditions) | Overall system uncertainty for AWGN condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |
| 7.3.42 E-UTRAN FDD-FDD DC Radio Link Monitoring Test for In-sync in DRX in asynchronous DC | Same as 7.3.41 | Same as 7.3.41 |
| 7.3.43 E-UTRAN TDD-TDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity | Same as 7.3.41 | Same as 7.3.41 |
| 7.3.44 E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in FDD | Same as 7.3.38 | Same as 7.3.38 |
| 7.3.45 E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in TDD | Same as 7.3.38 | Same as 7.3.38 |
| 7.3.46 E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in FDD | Same as 7.3.41 | Same as 7.3.41 |
| 7.3.47 E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in TDD | Same as 7.3.41 | Same as 7.3.41 |
| 7.3.48 E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A | ± 0.9dB (Fading conditions, two antenna transmission) | Overall system uncertainty for fading condition comprises three quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Average signal-to-noise ratio uncertainty 2 + Signal-to-noise ratio variation2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Signal-to-noise ratio variation ±0.5 dB Fading profile power uncertainty ±0.7 dB for two Tx |
| 7.3.49 E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A | ± 0.9dB (Fading conditions, two antenna transmission) | Overall system uncertainty for fading condition comprises three quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Average signal-to-noise ratio uncertainty 2 + Signal-to-noise ratio variation2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Signal-to-noise ratio variation ±0.5 dB Fading profile power uncertainty ±0.7 dB for two Tx |
| 7.3.50: E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-Sync in DRX for UE Category M1 Configured in CEMode A | Same as 7.3.28 | Same as 7.3.28 |
| 7.3.51 E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE Category M1 Configured in CEMode A | Same as 7.3.28 | Same as 7.3.28 |
| 7.3.52 E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A | ± 0.9dB (Fading conditions, two antenna transmission) | Overall system uncertainty for fading condition comprises three quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Average signal-to-noise ratio uncertainty 2 + Signal-to-noise ratio variation2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Signal-to-noise ratio variation ±0.5 dB Fading profile power uncertainty ±0.7 dB for one Tx |
| 7.3.53 E-UTRAN HD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A | ± 0.9dB (Fading conditions, two antenna transmission) | Overall system uncertainty for fading condition comprises three quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Average signal-to-noise ratio uncertainty 2 + Signal-to-noise ratio variation2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Signal-to-noise ratio variation ±0.5 dB Fading profile power uncertainty ±0.7 dB for one Tx |
| 7.3.54: E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE Category M1 Configured in CEMode A | Same as 7.3.28 | Same as 7.3.28 |
| 7.3.55: E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A | Same as 7.3.28 | Same as 7.3.28 |
| 7.3.56:E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A | Same as 7.3.48 | Same as 7.3.48 |
| 7.3.57:E-UTRAN TDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A | Same as 7.3.48 | Same as 7.3.48 |
| 7.3.58:E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A | Same as 7.3.28 | Same as 7.3.28 |
| 7.3.59:E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A | Same as 7.3.28 | Same as 7.3.28 |
| 7.3.60 HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage | Same as 7.3.28 | Same as 7.3.28 |
| 7.3.61 HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage | Same as 7.3.28 | Same as 7.3.28 |
| 7.3.62 HD-FDD Radio Link Monitoring Test for In-sync with DRX for Category NB1 In-Band mode in Enhanced Coverage | Same as 7.3.28 | Same as 7.3.28 |
| 7.3.63 HD-FDD Radio Link Monitoring Test for In-sync with DRX for Category NB1 In-Band mode in Normal Coverage | Same as 7.3.28 | Same as 7.3.28 |
| 7.3.64 HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage | ± 0.6dB (AWGN conditions) | Overall system uncertainty for AWGN condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |
| 7.3.65 HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage | ± 0.6dB (AWGN conditions) | Overall system uncertainty for AWGN condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |
| 7.3.66 HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage | Same as 7.3.64 | Same as 7.3.64 |
| 7.3.67 HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage | Same as 7.3.64 | Same as 7.3.64 |
| 7.3.69 E-UTRAN HD-FDD Early Out-of-sync reporting Test for CE UE in CEMode A | Same as 7.3.52 | Same as 7.3.52 |
| 7.3.88 TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage | Same as 7.3.28 | Same as 7.3.28 |
| 7.3.89 TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage | Same as 7.3.28 | Same as 7.3.28 |
| 7.3.90 TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Normal Coverage | Same as 7.3.28 | Same as 7.3.28 |
| 7.3.91 TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage | Same as 7.3.28 | Same as 7.3.28 |
| 7.3.92 TDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage | Same as 7.3.64 | Same as 7.3.64 |
| 7.3.93 TDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage | Same as 7.3.65 | Same as 7.3.65 |
| 7.3.94 TDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage | Same as 7.3.64 | Same as 7.3.64 |
| 7.3.95 TDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage | Same as 7.3.64 | Same as 7.3.64 |
| 7.4.1 E-UTRAN FDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC | ± 0.6dB (AWGN conditions) | Overall system uncertainty for AWGN condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |
| 7.4.2 E-UTRAN TDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC | Same as 7.4.1 | Same as 7.4.1 |
| 7.4.3 E-UTRAN FDD-FDD Interruption at transitions between active and non-active during DRX in asynchronous dual connectivity | Same as 7.4.1 | Same as 7.4.1 |
| 7.5.1 E-UTRAN FDD - UE ProSe Direct Discovery Transmission Timing Accuracy Test | Noc ±3.0 dB averaged over BWConfig Ês / Noc ±0.3 dB ±3Ts Uplink signal transmit timing relative to downlink | NOTE: Ês / Noc is the ratio of cell 1 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 7.5.4 E-UTRAN FDD - UE ProSe Direct Communication Transmission Timing Accuracy Test | Noc ±3.0 dB averaged over BWConfig Ês / Noc ±0.3 dB ±3Ts Uplink signal transmit timing relative to downlink | NOTE: Ês / Noc is the ratio of cell 1 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 7.6.1 E-UTRAN FDD-TDD CA interruption at SRS carrier based switching | ± 0.6dB (AWGN conditions) | Overall system uncertainty for AWGN condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |
| 7.6.2 E-UTRAN TDD-TDD CA interruption at SRS carrier based switching | Same as 7.6.1 | Same as 7.6.1 |
| 8.1.1 E-UTRAN FDD-FDD intra frequency event triggered reporting under fading propagation conditions in asynchronous cells | Noc ±1.0 dB averaged over BWConfig Ês1 / Noc ±0.6 dB averaged over BWConfig Ês2 / Noc ±0.6 dB averaged over BWConfig | NOTE: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN Ês / Noc uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB |
| 8.1.2 E-UTRAN FDD-FDD intra frequency event triggered reporting under fading propagation conditions in synchronous cells | Same as 8.1.1 | Same as 8.1.1 |
| 8.1.3 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX | Same as 8.1.1 | Same as 8.1.1 |
| 8.1.5 E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps | Noc ±1.0 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB averaged over BWConfig | NOTE: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 8.1.6 E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX | Same as 8.1.5 | Same as 8.1.5 |
| 8.1.7 UE Measurement Procedures / E-UTRAN FDD-FDD Intra-frequency event-triggered reporting under time domain measurement resource restriction with non-MBSFN ABS (eICIC) | Noc ±1.0 dB averaged over BWConfig Ês1 / Noc ±0.6 dB averaged over BWConfig Ês2 / Noc ±0.6 dB averaged over BWConfig | NOTE: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN Ês / Noc uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB |
| 8.1.8 E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) | Noc ±1.0 dB averaged over BWConfig Ês1 / Noc ±0.6 dB averaged over BWConfig Ês2 / Noc ±0.6 dB averaged over BWConfig Ês3 / Noc ±0.6 dB averaged over BWConfig | NOTE: Ês1 / Noc is the ratio of Cell 1 signal / AWGN Ês2 / Noc is the ratio of Cell 2 signal / AWGN Ês3 / Noc is the ratio of Cell 3 signal / AWGN Ês / Noc uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB |
| 8.1.9 E-UTRAN FDD-FDD intra frequency event triggered reporting under fading propagation conditions in asynchronous cells for 5MHz Bandwidth | Same as 8.1.1 | Same as 8.1.1 |
| 8.1.10 E-UTRAN FDD-FDD intra frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for 5MHz Bandwidth | Same as 8.1.3 | Same as 8.1.3 |
| 8.1.11 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0 | Noc ±1.0 dB averaged over BWConfig Ês1 / Noc ±0.6 dB averaged over BWConfig Ês2 / Noc ±0.6 dB averaged over BWConfig | NOTE: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN Ês / Noc uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB |
| 8.1.11_2 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 1bis | Same as 8.1.11 | Same as 8.1.11 |
| 8.1.12 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0 | Same as 8.1.11 | Same as 8.1.11 |
| 8.1.12_2 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 1bis | Same as 8.1.11 | Same as 8.1.11 |
| 8.1.13 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0 | Same as 8.1.11 | Same as 8.1.11 |
| 8.1.13_2 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 1bis | Same as 8.1.11 | Same as 8.1.11 |
| 8.1.17 E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0 | Same as 8.1.11 | Same as 8.1.11 |
| 8.1.17_2 E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 1bis | Same as 8.1.11 | Same as 8.1.11 |
| 8.1.18 E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0 | Same as 8.1.11 | Same as 8.1.11 |
| 8.1.18_2 E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 1bis | Same as 8.1.11 | Same as 8.1.11 |
| 8.1.19 E-UTRAN FD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0 | Noc ±1.0 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB averaged over BWConfig | NOTE: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 8.1.19_2 E-UTRAN FD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 1bis | Same as 8.1.19 | Same as 8.1.19 |
| 8.1.20 E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0 | Same as 8.1.19 | Same as 8.1.19 |
| 8.1.20_2 E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 1bis | Same as 8.1.19 | Same as 8.1.19 |
| 8.1.21 E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0 | Same as 8.1.19 | Same as 8.1.19 |
| 8.1.22 E-UTRAN HD- FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0 | Same as 8.1.19 | Same as 8.1.19 |
| 8.1.23 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA | Noc ±1.0 dB averaged over BWConfig Ês1 / Noc ±0.8 dB averaged over BWConfig Ês2 / Noc ±0.8 dB dB averaged over BWConfig | NOTE: Ês1 / Noc is the ratio of cell 1 signal / Fading Ês2 / Noc is the ratio of cell 2 signal / Fading Ês / Noc uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for 2 Tx antennas |
| 8.1.24 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA | Same as 8.1.23 | Same as 8.1.23 |
| 8.1.25 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX | Same as 8.1.23 | Same as 8.1.23 |
| 8.1.26 E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA | Same as 8.1.23 | Same as 8.1.23 |
| 8.1.27 E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA | Same as 8.1.23 | Same as 8.1.23 |
| 8.1.28 E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX | Same as 8.1.23 | Same as 8.1.23 |
| 8.1.29 E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA | Same as 8.1.23 | Same as 8.1.23 |
| 8.1.30 E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX | Same as 8.1.23 | Same as 8.1.23 |
| 8.1.31 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB | Same as 8.1.33 | Same as 8.1.33 |
| 8.1.32 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB | Same as 8.1.33 | Same as 8.1.33 |
| 8.1.33 E-UTRAN HD-FDD Intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB | Noc ±1.0 dB averaged over BWConfig Ês1 / Noc ±0.8 dB averaged over BWConfig Ês2 / Noc ±0.8 dB averaged over BWConfig | NOTE: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN Ês / Noc uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for 2 Tx |
| 8.1.34 E-UTRAN HD-FDD Intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB | Same as 8.1.33 | Same as 8.1.33 |
| 8.1.35 E-UTRAN TDD Intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB | Same as 8.1.33 | Same as 8.1.33 |
| 8.1.40 E-UTRAN FDD-FDD intra-frequency event triggered reporting for UE configured with highSpeedEnhancedMeasFlag in synchronous cells | AWGN absolute power, Noc ±1.0 dB Ratio of cell 1 signal /No Fading, Ês1 / Noc ±0.3 dB Ratio of cell 2 signal /No Fading, Ês2 / Noc ±0.3 dB | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of Cell 1 signal / AWGN Êsn1 / Noc1 is the ratio of nCell1 signal / AWGN Êsn2 / Noc1 is the ratio of nCell2 signal / AWGN |
| 8.1.41 E-UTRAN FDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for CE UE in CEModeA without gap | Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.6 dB averaged over BWConfig | NOTE: Ês / Noc is the ratio of cell 1 signal / AWGN Ês / Noc uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB |
| 8.1.42 E-UTRAN HD-FDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for CE UE in CEModeA without gap | Same as 8.1.41 | Same as 8.1.41 |
| 8.2.1 E-UTRAN TDD-TDD intra frequency event triggered reporting under fading propagation conditions in synchronous cells | Same as 8.1.1 | Same as 8.1.1 |
| 8.2.2 E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX | Same as 8.1.1 | Same as 8.1.1 |
| 8.2.3 E-UTRAN TDD-TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps | Noc ±1.0 dB averaged over BWConfig Ês1 / Noc ±0.3 dB averaged over BWConfig Ês2 / Noc ±0.3 dB averaged over BWConfig | NOTE: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 8.2.4 E-UTRAN TDD-TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX | Same as 8.2.3 | Same as 8.2.3 |
| 8.2.5 UE Measurement Procedures / E-UTRAN TDD-TDD Intra-frequency event-triggered reporting under time domain measurement resource restriction with non-MBSFN ABS (eICIC) | Same as 8.1.7 | Same as 8.1.7 |
| 8.2.6 E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) | Same as 8.1.8 | Same as 8.1.8 |
| 8.2.7 E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0 | Same as 8.1.19 | Same as 8.1.19 |
| 8.2.7_2 E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 1bis | Same as 8.1.19 | Same as 8.1.19 |
| 8.2.8 E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0 | Same as 8.1.19 | Same as 8.1.19 |
| 8.2.8_2 E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 1bis | Same as 8.1.19 | Same as 8.1.19 |
| 8.2.11 E-UTRAN TDD-TDD intra-frequency event triggered reporting for UE configured with highSpeedEnhancedMeasFlag in synchronous cells | AWGN absolute power, Noc ±1.0 dB Ratio of cell 1 signal /No Fading, Ês1 / Noc ±0.3 dB Ratio of cell 2 signal /No Fading, Ês2 / Noc ±0.3 dB | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of Cell 1 signal / AWGN Êsn1 / Noc1 is the ratio of nCell1 signal / AWGN Êsn2 / Noc1 is the ratio of nCell2 signal / AWGN |
| 8.2.14 E-UTRAN TDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for CE in CEModeA without gap | Same as 8.1.41 | Same as 8.1.41 |
| 8.3.1 E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.6 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.6 dB averaged over BWConfig | NOTE: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Each Ês / Noc uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB |
| 8.3.1_2 E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 1bis | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.6 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.6 dB averaged over BWConfig | NOTE: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Each Ês / Noc uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB |
| 8.3.2 E-UTRAN FDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells | Same as 8.3.1 | Same as 8.3.1 |
| 8.3.2_2 E-UTRAN FDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells for UE category 1bis | Same as 8.3.1_2 | Same as 8.3.1_2 |
| 8.3.3 E-UTRAN FDD-FDD Inter frequency event triggered reporting under AWGN propagation conditions in asynchronous cells with DRX when L3 filtering is used | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.3 dB averaged over BWConfig | NOTE: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |
| 8.3.3_2 E-UTRAN FDD-FDD inter-frequency event triggered reporting under AWGN propagation conditions in asynchronous cells with DRX when L3 filtering is used for UE category 1bis | Same as 8.3.3 | Same as 8.3.3 |
| 8.3.4 E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.3 dB averaged over BWConfig | NOTE: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |
| 8.3.5 E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX | Same as 8.3.4 | Same as 8.3.4 |
| 8.3.6 E-UTRAN FDD-FDD Inter-frequency event triggered reporting without measurement gaps under AWGN propagation conditions in asynchronous cells | Same as 8.3.4 | Same as 8.3.4 |
| 8.3.7 E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.6 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.6 dB averaged over BWConfig Noc3 ±1.0 dB averaged over BWConfig Ês3 / Noc3 ±0.6 dB averaged over BWConfig Noc4 ±1.0 dB averaged over BWConfig Ês4 / Noc4 ±0.6 dB averaged over BWConfig | NOTE: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Noc3 is the AWGN on cell 3 frequency Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Noc4 is the AWGN on cell 4 frequency Ês4 / Noc4 is the ratio of cell 4 signal / AWGN Each Ês / Noc uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB |
| 8.3.8 FDD-FDD Inter-frequency correct reporting of measurement events with reduced performance group configured, non DRX | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.3 dB averaged over BWConfig Noc3 ±1.0 dB averaged over BWConfig Ês3 / Noc3 ±0.3 dB averaged over BWConfig Noc4 ±1.0 dB averaged over BWConfig Ês4 / Noc4 ±0.3 dB averaged over BWConfig | Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Noc3 is the AWGN on cell 3 frequency Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Noc4 is the AWGN on cell 4 frequency Ês4 / Noc4 is the ratio of cell 4 signal / AWGN |
| 8.3.9 FDD-FDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX | Same as 8.3.8 | Same as 8.3.8 |
| 8.3.12 E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for CE UE with discontinuous MPDCCH monitoring in CEModeA | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.6 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.6 dB averaged over BWConfig | NOTE: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Each Ês / Noc uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB |
| 8.3.13 E-UTRAN HD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for CE UE with discontinuous MPDCCH monitoring in CEModeA | Same as 8.3.12 | Same as 8.3.12 |
| 8.3.14 E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for CE UE with discontinuous MPDCCH monitoring in CEModeB | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.6 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.6 dB averaged over BWConfig | NOTE: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Each Ês / Noc uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB |
| 8.3.15 E-UTRAN HD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for CE UE with discontinuous MPDCCH monitoring in CEModeB | Same as 8.3.14 | Same as 8.3.14 |
| 8.3.16 E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for CE UE in CEModeA when DRX is used | Same as 8.3.12 | Same as 8.3.12 |
| 8.3.17 E-UTRAN HD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for CE UE in CEModeA when DRX is used | Same as 8.3.12 | Same as 8.3.12 |
| 8.3.18 E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for CE UE in CEModeB when DRX is used | Same as 8.3.14 | Same as 8.3.14 |
| 8.3.19 E-UTRAN HD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for CE UE in CEModeB when DRX is used | Same as 8.3.14 | Same as 8.3.14 |
| 8.4.1 E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.6 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.6 dB averaged over BWConfig | NOTE: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Each Ês / Noc uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB |
| 8.4.1_2 E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 1bis | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.6 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.6 dB averaged over BWConfig | NOTE: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Each Ês / Noc uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB |
| 8.4.2 E-UTRAN TDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in synchronous cells | Same as 8.4.1 | Same as 8.4.1 |
| 8.4.2_2 E-UTRAN TDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in synchronous cells for UE category 1bis | Same as 8.4.1_2 | Same as 8.4.1_2 |
| 8.4.3 E-UTRAN TDD-TDD inter-frequency event triggered reporting under AWGN propagation conditions in synchronous cells with DRX when L3 filtering is used | Same as 8.3.3 | Same as 8.3.3 |
| 8.4.3_2 E-UTRAN TDD-TDD inter-frequency event triggered reporting under AWGN propagation conditions in synchronous cells with DRX when L3 filtering is used for UE category 1bis | Same as 8.3.3 | Same as 8.3.3 |
| 8.4.4 E-UTRAN TDD-TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.3 dB averaged over BWConfig | NOTE: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |
| 8.4.5 E-UTRAN TDD-TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX | Same as 8.4.4 | Same as 8.4.4 |
| 8.4.6 E-UTRAN TDD-TDD Inter-frequency event triggered reporting for TDD UL/DL configuration 0 | Same as 8.4.1 | Same as 8.4.1 |
| 8.4.6_2 E-UTRAN TDD-TDD Inter-frequency event triggered reporting for TDD UL/DL configuration 0 for UE category 1bis | Same as 8.4.1_2 | Same as 8.4.1_2 |
| 8.4.7 E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Increased Carrier Monitoring without Reduced Performance Group | Same as 8.3.7 | Same as 8.3.7 |
| 8.4.8 TDD-TDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX | Same as 8.3.8 | Same as 8.3.8 |
| 8.4.9 TDD-TDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX | Same as 8.3.9 | Same as 8.3.9 |
| 8.4.12 E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for CE UE with discontinuous MPDCCH monitoring in CEModeA | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.6 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.6 dB averaged over BWConfig | NOTE: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Each Ês / Noc uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB |
| 8.4.13 E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for CE UE with discontinuous MPDCCH monitoring in CEModeB | Same as 8.4.12 | Same as 8.4.12 |
| 8.4.14 TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for CE UE in CEModeA when DRX is used | Same as 8.4.12 | Same as 8.4.12 |
| 8.4.15 E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for CE UE in CEModeB when DRX is used | Same as 8.4.12 | Same as 8.4.12 |
| 8.5.1 E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions | E-UTRAN cell Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.6 dB averaged over BWConfig UTRA cell Ioc ±0.7 dB Ior/Ioc ±0.6 dB CPICH Ec/Ior ±0.1 dB | NOTE: Noc is the AWGN on Cell 1 frequency Ês / Noc is the ratio of Cell 1 signal / AWGN Ês / Noc uncertainty or Ior/Ioc uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB Ioc is the AWGN on Cell 2 (UTRA) frequency Ior/Ioc is the ratio of Cell 2 signal/AWGN CPICH Ec/Ior is the fraction of Cell 2 power assigned to the CPICH physical channel |
| 8.5.2 E-UTRAN FDD - UTRAN FDD SON ANR cell search reporting under AWGN propagation conditions | E-UTRA cell Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig UTRA cell Ioc ±0.7 dB Ior/Ioc ±0.3 dB CPICH Ec/Ior ±0.1 dB SCH Ec/Ior ±0.1 dB | NOTE: Noc is the AWGN on cell 1 frequency Ês / Noc is the ratio of cell 1 signal / AWGN Ioc is the AWGN on Cell 2 (UTRA) frequency Ior/Ioc is the ratio of Cell 2 signal/AWGN CPICH Ec/Ior is the fraction of Cell 2 power assigned to the CPICH physical channel SCH Ec/Ior is the fraction of Cell 2 power assigned to the SCH physical channel |
| 8.5.3 E-UTRAN FDD - UTRAN FDD event triggered reporting when DRX is used under fading propagation conditions | Same as 8.5.1 | Same as 8.5.1 |
| 8.5.4 E-UTRAN FDD - UTRAN FDD enhanced cell identification under AWGN propagation conditions | E-UTRA cell Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig UTRA cell Ioc ±0.7 dB Ior/Ioc ±0.3 dB CPICH Ec/Ior ±0.1 dB SCH Ec/Ior ±0.1 dB | NOTE: Noc is the AWGN on cell 1 frequency Ês / Noc is the ratio of cell 1 signal / AWGN Ioc is the AWGN on Cell 2 (UTRA) frequency Ior/Ioc is the ratio of Cell 2 signal/AWGN CPICH Ec/Ior is the fraction of Cell 2 power assigned to the CPICH physical channel SCH Ec/Ior is the fraction of Cell 2 power assigned to the SCH physical channel |
| 8.5.6 E-UTRAN FDD - UTRAN FDD event triggered reporting without measurement gaps under AWGN propagation conditions | Same as 8.5.4 | Same as 8.5.4 |
| 8.5.7 E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions for 5MHz Bandwidth | Same as 8.5.1 | Same as 8.5.1 |
| 8.5.8 E-UTRA FDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX | E-UTRA cell Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.6 dB averaged over BWConfig UTRA cells Ioc ±0.7 dB Ior/Ioc ±0.6 dB CPICH Ec/Ior ±0.1 dB SCH Ec/Ior ±0.1 dB | NOTE: Noc is the AWGN on cell 1 frequency Ês / Noc is the ratio of cell 1 signal / AWGN Ioc is the AWGN on Cells 2, 3, 4 (UTRA) frequency Ior/Ioc is the ratio of Cells 2, 3, 4 signal/AWGN CPICH Ec/Ior is the fraction of Cells 2, 3, 4 power assigned to the CPICH physical channel SCH Ec/Ior is the fraction of Cells 2, 3, 4 power assigned to the SCH physical channel |
| 8.6.1 E-UTRAN TDD - UTRAN FDD event triggered reporting under fading propagation conditions | Same as 8.5.1 | Same as 8.5.1 |
| 8.6.3 E-UTRA TDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX | Same as 8.5.8 | Same as 8.5.8 |
| 8.7.1 E-UTRAN TDD - UTRAN TDD event triggered reporting under fading propagation conditions | E-UTRA cell Noc ±0.7 dB averaged over BWConfig Ês / Noc ±0.6 dB averaged over BWConfig UTRA cell Ioc ±0.7 dB Ior/Ioc ±0.6 dB PCCPCH Ec/Ior ±0.1 dB DwPCH_Ec/Ior ±0.1 dB | Notes: Noc is the AWGN on cell 1 (E-UTRA )frequency Ês / Noc is the ratio of cell 1 signal / AWGN Ês / Noc uncertainty or Ior / Ioc uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty or Ior / Ioc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB Ioc is the AWGN on cell 2 (UTRA) frequency Ior / Ioc is the ratio of cell 2 signal / AWGN PCCPCH Ec / Ior is the fraction of cell 2 power assigned to the PCCPCH Physical channel DwPCH_Ec/Ior is the fraction of cell 2 power assigned to the DwPCH channel |
| 8.7.2 E-UTRAN TDD - UTRAN TDD cell search when DRX is used under fading propagation conditions | Same as 8.7.1 | Same as 8.7.1 |
| 8.7.3 E-UTRAN TDD - UTRAN TDD SON ANR cell search reporting under AWGN propagation conditions | E-UTRA cell Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig UTRA cell Ioc ±0.7 dB Ior/Ioc ±0.3 dB PCCPCH Ec/Ior ±0.1 dB DwPCH_Ec/Ior ±0.1 dB | NOTE: Noc is the AWGN on cell 1 frequency Ês / Noc is the ratio of cell 1 signal / AWGN Ioc is the AWGN on cell 2 (UTRA) frequency Ior / Ioc is the ratio of cell 2 signal / AWGN PCCPCH Ec / Ior is the fraction of cell 2 power assigned to the PCCPCH Physical channel DwPCH_Ec/Ior is the fraction of cell 2 power assigned to the DwPCH channel |
| 8.7.4 E-UTRAN TDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions | E-UTRA cell Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig UTRA cell Ioc ±0.7 dB Ior/Ioc ±0.3 dB PCCPCH Ec/Ior ±0.1 dB DwPCH_Ec/Ior ±0.1 dB | NOTE: Noc is the AWGN on cell 1 frequency Ês / Noc is the ratio of cell 1 signal / AWGN Ioc is the AWGN on cell 2 (UTRA) frequency Ior / Ioc is the ratio of cell 2 signal / AWGN PCCPCH Ec/Ior is the fraction of cell 2 power assigned to the PCCPCH Physical channel DwPCH_Ec/Ior is the fraction of cell 2 power assigned to the DwPCH channel |
| 8.7.5 E-UTRA TDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX | E-UTRA cell Noc ±1 dB averaged over BWConfig Ês / Noc ±0.6 dB averaged over BWConfig UTRA cell Ioc ±0.7 dB Ior/Ioc ±0.6 dB PCCPCH Ec/Ior ±0.1 dB DwPCH_Ec/Ior ±0.1 dB | Notes: Noc is the AWGN on cell 1 (E-UTRA) frequency Ês / Noc is the ratio of cell 1 signal / AWGN Ês / Noc uncertainty or Ior / Ioc uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty or Ior / Ioc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB Ioc is the AWGN on cells 2, 3 and 4 (UTRA) frequency Ior / Ioc is the ratio of cell 2 signal / AWGN PCCPCH Ec / Ior is the fraction of cell 2. 3 or 4 power assigned to the PCCPCH Physical channel DwPCH_Ec/Ior is the fraction of cell 2, 3 or 4 power assigned to the DwPCH channel |
| 8.7A.1 E-UTRA FDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX | Same as 8.7.5 | Same as 8.7.5 |
| 8.8.1 E-UTRAN FDD - GSM event triggered reporting in AWGN | E-UTRA Cell Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig GSM cell Signal level ±0.7 dB | NOTE: Noc is the AWGN on cell 1 frequency Ês / Noc is the ratio of cell 1 signal / AWGN Cell 2 (GSM) has only the wanted signal, without AWGN |
| 8.8.2 E-UTRAN FDD- GSM event triggered reporting when DRX is used in AWGN | Same as 8.8.1 | Same as 8.8.1 |
| 8.9.1 E-UTRAN FDD - UTRAN TDD event triggered reporting under fading propagation conditions | E-UTRA cell Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.6 dB averaged over BWConfig UTRA TDD cell Ioc ±0.7 dB Ior/Ioc ±0.6 dB PCCPCH_Ec /Ior ±0.1 dB DwPCH_Ec /Ior ±0.1 dB | Noc is the AWGN on cell 1 frequency Ês / Noc is the ratio of cell 1 signal / AWGN Ioc is the AWGN on Cell 2 (UTRA TDD) frequency Ior/Ioc is the ratio of Cell 2 signal/AWGN PCCPCH_Ec /Ior is the fraction on Cell 2 power assigned to the CPCCPCH physical channel DwPCH_Ec /Ior is the fraction on Cell 2 power assigned to the DwPCH physical channel Ês / Noc and Ior/Ioc uncertainty for fading condition comprise two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty |
| 8.9.2 E-UTRAN FDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions | Same as 8.7.4 | Same as 8.7.4 |
| 8.10.1 E-UTRAN TDD - GSM event triggered reporting in AWGN | Same as 8.8.1 | Same as 8.8.1 |
| 8.10.2 E-UTRAN TDD- GSM event triggered reporting when DRX is used in AWGN | Same as 8.8.2 | Same as 8.8.2 |
| 8.11.1 Multiple E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.6 dB averaged over BWConfig Noc3 ±1.0 dB averaged over BWConfig Ês3 / Noc3 ±0.6 dB averaged over BWConfig | NOTE: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Noc3 is the AWGN on cell 3 frequency Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Ês2 / Noc2 uncertainty or Ês3 / Noc3 uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB |
| 8.11.2 E-UTRAN TDD - E-UTRAN TDD and E-UTRAN TDD Inter-frequency event triggered reporting under fading propagation conditions | Same as 8.11.1 | Same as 8.11.1 |
| 8.11.3 E-UTRAN FDD-FDD Inter-frequency and UTRAN FDD event triggered reporting under fading propagation conditions | E-UTRA cells Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.6 dB averaged over BWConfig UTRA cell Ioc ±0.7 dB Ior/Ioc ±0.6 dB CPICH Ec/Ior ±0.1 dB | NOTE: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ioc is the AWGN on Cell 3 (UTRA) frequency Ior/Ioc is the ratio of Cell 3 signal/AWGN CPICH Ec/Ior is the fraction of Cell 3 power assigned to the CPICH physical channel Ês2 / Noc2 uncertainty or Ior/Ioc uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB |
| 8.11.4 InterRAT E-UTRA TDD to E-UTRA TDD and UTRA TDD cell search | E-UTRA cells Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.6 dB averaged over BWConfig UTRA cell Ioc ±0.7 dB Ior/Ioc ±0.6 dB PCCPCH Ec/Ior ±0.1 dB DwPCH_Ec/Ior ±0.1 dB | NOTE: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ioc is the AWGN on cell 3 (UTRA) frequency Ior / Ioc is the ratio of cell 3 signal / AWGN PCCPCH Ec / Ior is the fraction of cell 3 power assigned to the PCCPCH Physical channel DwPCH_Ec/Ior is the fraction of cell 3 power assigned to the DwPCH channel Ês2 / Noc2 uncertainty or Ior/Ioc uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc or Ior/Ioc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB |
| 8.11.5 Combined E-UTRAN FDD - E-UTRA FDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions | E-UTRA cells Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.6 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.6 dB averaged over BWConfig GSM cell Signal level ±0.7 dB | NOTE: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Cell 3 (GSM) has only the wanted signal, without AWGN Ês1 / Noc1 uncertainty or Ês2 / Noc2 uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB |
| 8.11.6 Combined E-UTRAN TDD - E-UTRA TDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions | Same as 8.11.5 | Same as 8.11.5 |
| 8.14.1 E-UTRAN TDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells | Same as 8.3.1 | Same as 8.3.1 |
| 8.14.2 E-UTRAN TDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells | Same as 8.3.1 | Same as 8.3.1 |
| 8.14.3 E-UTRAN TDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps | Same as 8.4.4 | Same as 8.4.4 |
| 8.15.1 E-UTRAN FDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells | Same as 8.4.1 | Same as 8.4.1 |
| 8.15.2 E-UTRAN FDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells | Same as 8.4.1 | Same as 8.4.1 |
| 8.15.3 E-UTRAN FDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps | Same as 8.4.4 | Same as 8.4.4 |
| 8.16.1 E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.6 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.6 dB averaged over BWConfig Ês3 / Noc2 ±0.6 dB averaged over BWConfig Time alignment error cell 2 rel. to cell 1: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc2 is the ratio of cell 3 signal / AWGN Ês1 / Noc1, Ês2 / Noc2 uncertainty and Ês3 / Noc2 uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 8.16.2 E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX | Same as 8.16.1 | Same as 8.16.1 |
| 8.16.3 E-UTRAN FDD-FDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.3 dB averaged over BWConfig Ês3 / Noc2 ±0.3 dB averaged over BWConfig Time alignment error cell 2 rel. to cell 1: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of Cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of Cell 2 signal / AWGN Ês3 / Noc2 is the ratio of Cell 3 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 8.16.4 E-UTRAN TDD-TDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX | Same as 8.16.3 | Same as 8.16.3 |
| 8.16.5 E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth | Same as 8.16.1 | Same as 8.16.1 |
| 8.16.6 E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth | Same as 8.16.1 | Same as 8.16.1 |
| 8.16.7 E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth | Same as 8.16.3 | Same as 8.16.3 |
| 8.16.8 E-UTRAN TDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth | Same as 8.16.3 | Same as 8.16.3 |
| 8.16.9 E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz | Same as 8.16.1 | Same as 8.16.1 |
| 8.16.10 E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz | Same as 8.16.1 | Same as 8.16.1 |
| 8.16.11 E-UTRAN FDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz | Same as 8.16.3 | Same as 8.16.3 |
| 8.16.12 E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz | Same as 8.16.3 | Same as 8.16.3 |
| 8.16.13 E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 5MHz+5MHz | Same as 8.16.1 | Same as 8.16.1 |
| 8.16.14 E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 5MHz+5MHz | Same as 8.16.1 | Same as 8.16.1 |
| 8.16.15 E-UTRAN FDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 5MHz+5MHz | Same as 8.16.3 | Same as 8.16.3 |
| 8.16.16 E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 5MHz+5MHz | Same as 8.16.3 | Same as 8.16.3 |
| 8.16.17 E-UTRAN FDD activation and deactivation of known SCell in non-DRX | Same as 8.16.37 | Same as 8.16.37 |
| 8.16.17A E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 20MHz | Same as 8.16.17 | Same as 8.16.17 |
| 8.16.18 E-UTRAN TDD activation and deactivation of known SCell in non-DRX | Same as 8.16.17 | Same as 8.16.17 |
| 8.16.18 E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 20MHz | Same as 8.16.17 | Same as 8.16.17 |
| 8.16.21 E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20MHz+10MHz bandwidth | Same as 8.16.1 | Same as 8.16.1 |
| 8.16.22 E-UTRAN TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20MHz+10MHz bandwidth | Same as 8.16.3 | Same as 8.16.3 |
| 8.16.23 E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in FDD | Same as 8.16.1 | Same as 8.16.1 |
| 8.16.24 E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in TDD | Same as 8.16.1 | Same as 8.16.1 |
| 8.16.25 E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in FDD | Same as 8.16.3 | Same as 8.16.3 |
| 8.16.26 E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in TDD | Same as 8.16.3 | Same as 8.16.3 |
| 8.16.27 E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with PCell in FDD | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.6 dB averaged over BWConfig Noc3 ±1.0 dB averaged over BWConfig Ês3 / Noc3 ±0.6 dB averaged over BWConfig Ês4 / Noc3 ±0.6 dB averaged over BWConfig Time alignment errors: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Noc3 is the AWGN on frequency 3 Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Ês4 / Noc3 is the ratio of cell 4 signal / AWGN Ês2 / Noc2, Ês3 / Noc3 uncertainty and Ês4 / Noc3 uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 8.16.28 E-UTRAN TDD-FDD 3DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with PCell in TDD | Same as 8.16.27 | Same as 8.16.27 |
| 8.16.29, 3DL FDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX | Same as 8.16.27 | Same as 8.16.27 |
| 8.16.30, 3DL TDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX | Same as 8.16.27 | Same as 8.16.27 |
| 8.16.31 E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in FDD | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.3 dB averaged over BWConfig Noc3 ±1.0 dB averaged over BWConfig Ês3 / Noc3 ±0.3 dB averaged over BWConfig Ês4 / Noc3 ±0.3 dB averaged over BWConfig Time alignment errors: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Noc3 is the AWGN on frequency 3 Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Ês4 / Noc3 is the ratio of cell 4 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 8.16.32 E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in TDD | Same as 8.16.31 | Same as 8.16.31 |
| 8.16.33 E-UTRAN FDD 3DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX | Same as 8.16.31 | Same as 8.16.31 |
| 8.16.34 E-UTRAN TDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX | Same as 8.16.31 | Same as 8.16.31 |
| 8.16.35 3 DL PCell in FDD CA Activation and Deactivation of Known SCell in Non-DRX | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.3 dB averaged over BWConfig Noc3 ±1.0 dB averaged over BWConfig Ês3 / Noc3 ±0.3 dB averaged over BWConfig Time alignment errors: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Noc3 is the AWGN on frequency 3 Ês3 / Noc3 is the ratio of cell 3 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 8.16.36 3 DL PCell in TDD CA Activation and Deactivation of Known SCell in Non-DRX | Same as 8.16.35 | Same as 8.16.35 |
| 8.16.37 3DL FDD CA Activation and Deactivation of known SCell in non-DRX | Same as 8.16.35 | Same as 8.16.35 |
| 8.16.38 3DL TDD CA Activation and Deactivation of known SCell in non-DRX | Same as 8.16.35 | Same as 8.16.35 |
| 8.16.39 E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in FDD | Same as 8.16.35 | Same as 8.16.35 |
| 8.16.40 E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in TDD | Same as 8.16.35 | Same as 8.16.35 |
| 8.16.41 3 DL FDD CA activation and deactivation of unknown SCell in non-DRX | Same as 8.16.35 | Same as 8.16.35 |
| 8.16.42 3 DL TDD CA activation and deactivation of unknown SCell in non-DRX | Same as 8.16.35 | Same as 8.16.35 |
| 8.16.47 2DL/2UL FDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX | Same as 8.16.17 | Same as 8.16.17 |
| 8.16.48 2DL/2UL TDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX | Same as 8.16.17 | Same as 8.16.17 |
| 8.16.49 2DL/2UL TDD-FDD CA (FDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX | Same as 8.16.17 | Same as 8.16.17 |
| 8.16.50 2DL/2UL TDD-FDD CA (TDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX | Same as 8.16.17 | Same as 8.16.17 |
| 8.16.51 E-UTRAN 4 DL FDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.6 dB averaged over BWConfig Noc3 ±1.0 dB averaged over BWConfig Ês3 / Noc3 ±0.6 dB averaged over BWConfig Noc4 ±1.0 dB averaged over BWConfig Ês4 / Noc4 ±0.6 dB averaged over BWConfig Ês5 / Noc4 ±0.6 dB averaged over BWConfig Time alignment errors: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Noc3 is the AWGN on frequency 3 Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Noc4 is the AWGN on frequency 4 Ês4 / Noc4 is the ratio of cell 4 signal / AWGN Ês5 / Noc4 is the ratio of cell 5 signal / AWGN Ês2 / Noc2, Ês3 / Noc3, Ês4 / Noc4 uncertainty and Ês5 / Noc4 uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 8.16.52 E-UTRAN 4 DL TDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX | Same as 8.16.51 | Same as 8.16.51 |
| 8.16.53 4 DL PCell in FDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX | Same as 8.16.51 | Same as 8.16.51 |
| 8.16.54 4 DL PCell in TDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX | Same as 8.16.51 | Same as 8.16.51 |
| 8.16.55 4DL FDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.3 dB averaged over BWConfig Noc3 ±1.0 dB averaged over BWConfig Ês3 / Noc3 ±0.3 dB averaged over BWConfig Noc4 ±1.0 dB averaged over BWConfig Ês4 / Noc4 ±0.3 dB averaged over BWConfig Ês5 / Noc4 ±0.3 dB averaged over BWConfig Time alignment errors: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Noc3 is the AWGN on frequency 3 Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Noc4 is the AWGN on frequency 4 Ês4 / Noc4 is the ratio of cell 4 signal / AWGN Ês5 / Noc4 is the ratio of cell 5 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 8.16.56 4DL TDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX | Same as 8.16.55 | Same as 8.16.55 |
| 8.16.57 4DL FDD CA Activation and Deactivation of Known SCell in Non-DRX | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.3 dB averaged over BWConfig Noc3 ±1.0 dB averaged over BWConfig Ês3 / Noc3 ±0.3 dB averaged over BWConfig Noc4 ±1.0 dB averaged over BWConfig Ês4 / Noc4 ±0.3 dB averaged over BWConfig Time alignment errors: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Noc3 is the AWGN on frequency 3 Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Noc4 is the AWGN on frequency 4 Ês4 / Noc4 is the ratio of cell 4 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 8.16.58 4DL TDD CA Activation and Deactivation of Known SCell in Non-DRX | Same as 8.16.57 | Same as 8.16.57 |
| 8.16.59 4DL PCell in FDD CA Activation and Deactivation of Known SCell in Non-DRX | Same as 8.16.57 | Same as 8.16.57 |
| 8.16.60 4DL PCell in TDD CA Activation and Deactivation of Known SCell in Non-DRX | Same as 8.16.57 | Same as 8.16.57 |
| 8.16.61 4DL FDD CA Activation and Deactivation of Unknown SCell in Non-DRX | Same as 8.16.57 | Same as 8.16.57 |
| 8.16.62 4DL TDD CA Activation and Deactivation of Unknown SCell in Non-DRX | Same as 8.16.57 | Same as 8.16.57 |
| 8.16.63 4DL PCell in FDD CA Activation and Deactivation of Unknown SCell in Non-DRX | Same as 8.16.57 | Same as 8.16.57 |
| 8.16.64 4DL PCell in TDD CA Activation and Deactivation of Unknown SCell in Non-DRX | Same as 8.16.57 | Same as 8.16.57 |
| 8.16.65 5 DL FDD-TDD with PCell in FDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX | Same as 8.16.71 | Same as 8.16.71 |
| 8.16.66 5 DL FDD-TDD with PCell in TDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX | Same as 8.16.71 | Same as 8.16.71 |
| 8.16.67 5DL Pcell in FDD CA activation and deactivation of unknown SCell in non-DRX | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.3 dB averaged over BWConfig Noc3 ±1.0 dB averaged over BWConfig Ês3 / Noc3 ±0.3 dB averaged over BWConfig Noc4 ±1.0 dB averaged over BWConfig Ês4 / Noc4 ±0.3 dB averaged over BWConfig Noc5 ±1.0 dB averaged over BWConfig Ês5 / Noc5 ±0.3 dB averaged over BWConfig Time alignment errors: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Noc3 is the AWGN on frequency 3 Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Noc4 is the AWGN on frequency 4 Ês4 / Noc4 is the ratio of cell 4 signal / AWGN Noc5 is the AWGN on frequency 5 Ês5 / Noc5 is the ratio of cell 5 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 8.16.68 5DL Pcell in TDD CA activation and deactivation of unknown SCell in non-DRX | Same as 8.16.67 | Same as 8.16.67 |
| 8.16.69 5DL FDD CA activation and deactivation of unknown SCell in non-DRX | Same as 8.16.67 | Same as 8.16.67 |
| 8.16.70 5DL TDD CA activation and deactivation of unknown SCell in non-DRX | Same as 8.16.67 | Same as 8.16.67 |
| 8.16.71 5 DL FDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.6 dB averaged over BWConfig Noc3 ±1.0 dB averaged over BWConfig Ês3 / Noc3 ±0.6 dB averaged over BWConfig Noc4 ±1.0 dB averaged over BWConfig Ês4 / Noc4 ±0.6 dB averaged over BWConfig Noc5 ±1.0 dB averaged over BWConfig Ês5 / Noc5 ±0.6 dB averaged over BWConfig Ês6 / Noc5 ±0.6 dB averaged over BWConfig Time alignment errors: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Noc3 is the AWGN on frequency 3 Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Noc4 is the AWGN on frequency 4 Ês4 / Noc4 is the ratio of cell 4 signal / AWGN Noc5 is the AWGN on frequency 5 Ês5 / Noc5 is the ratio of cell 5 signal / AWGN Ês6 / Noc5 is the ratio of cell 6 signal / AWGN Ês2 / Noc2, Ês3 / Noc3, Ês4 / Noc4, Ês5 / Noc5 uncertainty and Ês6 / Noc5 uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 8.16.72 5 DL TDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX | Same as 8.16.71 | Same as 8.16.71 |
| 8.16.73 5DL FDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX | Same as 8.16.71 | Same as 8.16.71 |
| 8.16.74 5DL TDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX | Same as 8.16.71 | Same as 8.16.71 |
| 8.16.75 5DL PCell in FDD CA activation and deactivation of known SCell in non-DRX | Same as 8.16.67 | Same as 8.16.67 |
| 8.16.76 5DL PCell in TDD CA activation and deactivation of known SCell in non-DRX | Same as 8.16.67 | Same as 8.16.67 |
| 8.16.77 5DL FDD CA activation and deactivation of known SCell in non-DRX | Same as 8.16.67 | Same as 8.16.67 |
| 8.16.78 E-UTRAN TDD 5DL CA activation and deactivation of known SCell in non-DRX | Same as 8.16.67 | Same as 8.16.67 |
| 8.16.83 3 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes | Same as 8.16.27 | Same as 8.16.27 |
| 8.16.84 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes | Same as 8.16.31 | Same as 8.16.31 |
| 8.16.85 3 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes | Same as 8.16.35 | Same as 8.16.35 |
| 8.16.86 3 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes | Same as 8.16.39 | Same as 8.16.39 |
| 8.16.87 4 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes | Same as 8.16.51 | Same as 8.16.51 |
| 8.16.88 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes | Same as 8.16.55 | Same as 8.16.55 |
| 8.16.91 5 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes | Same as 8.16.71 | Same as 8.16.71 |
| 8.16.92 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes | Same as 8.16.71 | Same as 8.16.71 |
| 8.16.89 4 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes | Same as 8.16.57 | Same as 8.16.57 |
| 8.16.90 4 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes | Same as 8.16.61 | Same as 8.16.61 |
| 8.16.93 5 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes | Same as 8.16.75 | Same as 8.16.75 |
| 8.16.94 5 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes | Same as 8.16.67 | Same as 8.16.67 |
| 8.16.95 6DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.6 dB averaged over BWConfig Noc3 ±1.0 dB averaged over BWConfig Ês3 / Noc3 ±0.6 dB averaged over BWConfig Noc4 ±1.0 dB averaged over BWConfig Ês4 / Noc4 ±0.6 dB averaged over BWConfig Noc5 ±1.0 dB averaged over BWConfig Ês5 / Noc5 ±0.6 dB averaged over BWConfig Noc6 ±1.0 dB averaged over BWConfig Ês6 / Noc6 ±0.6 dB averaged over BWConfig Ês7 / Noc6 ±0.6 dB averaged over BWConfig Time alignment errors: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Noc3 is the AWGN on frequency 3 Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Noc4 is the AWGN on frequency 4 Ês4 / Noc4 is the ratio of cell 4 signal / AWGN Noc5 is the AWGN on frequency 5 Ês5 / Noc5 is the ratio of cell 5 signal / AWGN Noc6 is the AWGN on frequency 6 Ês6 / Noc6 is the ratio of cell 6 signal / AWGN Ês7 / Noc6 is the ratio of cell 7 signal / AWGN Ês2 / Noc2, Ês3 / Noc3, Ês4 / Noc4, Ês5 / Noc5, Ês6 / Noc6 uncertainty and Ês7 / Noc6 uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 8.16.96 6 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes | Same as 8.16.95 | Same as 8.16.95 |
| 8.16.97 6 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.3 dB averaged over BWConfig Noc3 ±1.0 dB averaged over BWConfig Ês3 / Noc3 ±0.3 dB averaged over BWConfig Noc4 ±1.0 dB averaged over BWConfig Ês4 / Noc4 ±0.3 dB averaged over BWConfig Noc5 ±1.0 dB averaged over BWConfig Ês5 / Noc5 ±0.3 dB averaged over BWConfig¨ Noc6 ±1.0 dB averaged over BWConfig Ês6 / Noc6 ±0.3 dB averaged over BWConfig Time alignment errors: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Noc3 is the AWGN on frequency 3 Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Noc4 is the AWGN on frequency 4 Ês4 / Noc4 is the ratio of cell 4 signal / AWGN Noc5 is the AWGN on frequency 5 Ês5 / Noc5 is the ratio of cell 5 signal / AWGN Noc6 is the AWGN on frequency 6 Ês6 / Noc6 is the ratio of cell 6 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 8.16.98 6 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes | Same as 8.16.97 | Same as 8.16.97 |
| 8.16.99 7DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.6 dB averaged over BWConfig Noc3 ±1.0 dB averaged over BWConfig Ês3 / Noc3 ±0.6 dB averaged over BWConfig Noc4 ±1.0 dB averaged over BWConfig Ês4 / Noc4 ±0.6 dB averaged over BWConfig Noc5 ±1.0 dB averaged over BWConfig Ês5 / Noc5 ±0.6 dB averaged over BWConfig Noc6 ±1.0 dB averaged over BWConfig Ês6 / Noc6 ±0.6 dB averaged over BWConfig Noc7 ±1.0 dB averaged over BWConfig Ês7 / Noc7 ±0.6 dB averaged over BWConfig Ês8 / Noc7 ±0.6 dB averaged over BWConfig Time alignment errors: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Noc3 is the AWGN on frequency 3 Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Noc4 is the AWGN on frequency 4 Ês4 / Noc4 is the ratio of cell 4 signal / AWGN Noc5 is the AWGN on frequency 5 Ês5 / Noc5 is the ratio of cell 5 signal / AWGN Noc6 is the AWGN on frequency 6 Ês6 / Noc6 is the ratio of cell 6 signal / AWGN Noc7 is the AWGN on frequency 7 Ês7 / Noc7 is the ratio of cell 7 signal / AWGN Ês8 / Noc7 is the ratio of cell 8 signal / AWGN Ês2 / Noc2, Ês3 / Noc3, Ês4 / Noc4, Ês5 / Noc5, Ês6 / Noc6, Ês7 / Noc7 uncertainty and Ês8 / Noc7 uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 8.16.100 7 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes | Same as 8.16.99 | Same as 8.16.99 |
| 8.16.101 7 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.3 dB averaged over BWConfig Noc3 ±1.0 dB averaged over BWConfig Ês3 / Noc3 ±0.3 dB averaged over BWConfig Noc4 ±1.0 dB averaged over BWConfig Ês4 / Noc4 ±0.3 dB averaged over BWConfig Noc5 ±1.0 dB averaged over BWConfig Ês5 / Noc5 ±0.3 dB averaged over BWConfig¨ Noc6 ±1.0 dB averaged over BWConfig Ês6 / Noc6 ±0.3 dB averaged over BWConfig Noc7 ±1.0 dB averaged over BWConfig Ês7 / Noc7 ±0.3 dB averaged over BWConfig Time alignment errors: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Noc3 is the AWGN on frequency 3 Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Noc4 is the AWGN on frequency 4 Ês4 / Noc4 is the ratio of cell 4 signal / AWGN Noc5 is the AWGN on frequency 5 Ês5 / Noc5 is the ratio of cell 5 signal / AWGN Noc6 is the AWGN on frequency 6 Ês6 / Noc6 is the ratio of cell 6 signal / AWGN Noc7 is the AWGN on frequency 6 Ês7 / Noc7 is the ratio of cell 6 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 8.16.102 7 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes | Same as 8.16.101 | Same as 8.16.101 |
| 8.20.1 UE Measurement Procedures / RAT Measurements in CA mode / E-UTRAN FDD-FDD inter frequency event triggered reporting under fading propagation conditions in asynchronous cells | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.6 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.6 dB averaged over BWConfig Noc3 ±1.0 dB averaged over BWConfig Ês3 / Noc3 ±0.6 dB averaged over BWConfig | NOTE: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 3 frequency Ês2 / Noc2 is the ratio of cell 3 signal / AWGN Noc3 is the AWGN on cell 3 frequency Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Ês1 / Noc1 uncertainty, Ês2 / Noc2 uncertainty, Ês3 / Noc3 uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB |
| 8.20.2 UE Measurement Procedures / RAT Measurements in CA mode / E-UTRAN TDD-TDD inter frequency event triggered reporting under fading propagation conditions in asynchronous cells | Same as 8.20.1 | Same as 8.20.1 |
| 8.20.2A UE Measurement Procedures / RAT Measurements in CA mode / E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +20 MHz bandwidth | Same as 8.20.1 | Same as 8.20.1 |
| 8.20.2B UE Measurement Procedures / RAT Measurements in CA mode / E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +10 MHz bandwidth | Same as 8.20.1 | Same as 8.20.1 |
| 8.20.3 E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions | E-UTRA cells Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.6 dB averaged over BWConfig Noc3 ±1.0 dB averaged over BWConfig Ês3 / Noc3 ±0.6 dB averaged over BWConfig UTRA cell Ioc ±0.7 dB Ior/Ioc ±0.6 dB CPICH Ec/Ior ±0.1 dB | NOTE: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc3 is the AWGN on cell 3 frequency Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Ioc is the AWGN on Cell 2 (UTRA) frequency Ior/Ioc is the ratio of Cell 2 signal/AWGN CPICH Ec/Ior is the fraction of Cell 2 power assigned to the CPICH physical channel Ês1 / Noc1 uncertainty, Ês3 / Noc3 uncertainty or Ior/Ioc uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB |
| 8.20.4 E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions | E-UTRA cells Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.6 dB averaged over BWConfig Noc3 ±1.0 dB averaged over BWConfig Ês3 / Noc3 ±0.6 dB averaged over BWConfig UTRA cell Ioc ±0.7 dB Ior/Ioc ±0.6 dB PCCPCH Ec/Ior ±0.1 dB DwPCH_Ec/Ior ±0.1 dB | NOTE: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc3 is the AWGN on cell 3 frequency Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Ioc is the AWGN on cell 2 (UTRA) frequency Ior / Ioc is the ratio of cell 2 signal / AWGN PCCPCH Ec / Ior is the fraction of cell 2 power assigned to the PCCPCH Physical channel DwPCH_Ec/Ior is the fraction of cell 2 power assigned to the DwPCH channel Ês1 / Noc1 uncertainty, Ês3 / Noc3 uncertainty or Ior/Ioc uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc or Ior/Ioc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB |
| 8.20.4A E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions for 20 MHz + 20 MHz bandwidth | Same as 8.20.4 | Same as 8.20.4 |
| 8.20.4B E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions for 20 MHz + 10 MHz bandwidth | Same as 8.20.4 | Same as 8.20.4 |
| 8.22.1 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal | Same as 8.1.1 | Same as 8.1.1 |
| 8.22.2 E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal | Same as 8.1.1 | Same as 8.1.1 |
| 8.22.3 E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal | Same as 8.3.1 | Same as 8.3.1 |
| 8.22.4 E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal | Same as 8.3.1 | Same as 8.3.1 |
| 8.22.5 E-UTRAN FDD-FDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal | Noc ±1.0 dB averaged over BWConfig CRS Ês1 / Noc ±0.6 dB averaged over BWConfig CRS Ês2 / Noc ±0.6 dB averaged over BWConfig CSI-RS Ês1 / Noc ±0.6 dB averaged over BWConfig CSI-RS Ês2 / Noc ±0.6 dB averaged over BWConfig | NOTE: CRS Ês1 / Noc is the ratio of cell 1 CRS signal / AWGN CRS Ês2 / Noc is the ratio of cell 2 CRS signal / AWGN CSI-RS Ês1 / Noc is the ratio of cell 1 CSI-RS signal / AWGN CSI-RS Ês2 / Noc is the ratio of cell 2 CSI-RS signal / AWGN Ês / Noc uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB |
| 8.22.6 E-UTRAN TDD-TDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal | Same as 8.22.5 | Same as 8.22.5 |
| 8.22.7 E-UTRAN FDD-FDD inter-frequency event triggered reporting in DRX based on CSI-RS based discovery signal | Noc1 ±1.0 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.6 dB averaged over BWConfig Ês2 / Noc2 ±0.6 dB averaged over BWConfig CSI-RS Ês1 / Noc1 ±0.6 dB averaged over BWConfig CSI-RS Ês2 / Noc2 ±0.6 dB averaged over BWConfig | NOTE: Ês1 / Noc1 is the ratio of cell 1 CRS signal / AWGN Ês2 / Noc2 is the ratio of cell 2 CRS signal / AWGN CSI-RS Ês1 / Noc1 is the ratio of cell 1 CSI-RS signal / AWGN CSI-RS Ês2 / Noc2 is the ratio of cell 2 CSI-RS signal / AWGN Ês / Noc uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB |
| 8.22.8 E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation condition in DRX based on CSI-RS based discovery signal | Same as 8.22.7 | Same as 8.22.7 |
| 8.22.9 E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal | Same as 8.16.1 | Same as 8.16.1 |
| 8.22.10 E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal | Same as 8.16.1 | Same as 8.16.1 |
| 8.22.11 E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal | Noc1 ±1.0 dB averaged over BWConfig CRS Ês1 / Noc1 ±0.6 dB averaged over BWConfig CSI-RS Ês1 / Noc1 ±0.6 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig CRS Ês2 / Noc2 ±0.6 dB averaged over BWConfig CRS Ês3 / Noc2 ±0.6 dB averaged over BWConfig CSI-RS Ês2 / Noc2 ±0.6 dB averaged over BWConfig CSI-RS Ês3 / Noc2 ±0.6 dB averaged over BWConfig Time alignment error cell 2 rel. to cell 1: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | NOTE: Noc1 is the AWGN on frequency 1 CRS Ês1 / Noc1 is the ratio of cell 1 signal / AWGN CSI-RS Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 CRS Ês2 / Noc2 is the ratio of cell 2 CRS signal / AWGN CRS Ês3 / Noc2 is the ratio of cell 3 CRS signal / AWGN CSI-RS Ês2 / Noc2 is the ratio of cell 2 CSI-RS signal / AWGN CSI-RS Ês3 / Noc2 is the ratio of cell 3 CSI-RS signal / AWGN Ês1 / Noc1, Ês2 / Noc2 uncertainty and Ês3 / Noc2 uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 8.22.12 E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal | Same as 8.22.11 | Same as 8.22.12 |
| 8.23.1 E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.6 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.6 dB averaged over BWConfig | NOTE: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês1 / Noc1 uncertainty, Ês2 / Noc2 uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB |
| 8.23.2 E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in asynchronous DC | Same as 8.23.1 | Same as 8.23.1 |
| 8.23.3 E-UTRAN TDD-TDD DC intra-frequency event triggered reporting with DRX in synchronous DC | Same as 8.23.1 | Same as 8.23.1 |
| 8.23.4 E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.6 dB averaged over BWConfig Noc2 ±1.0 dB averaged over BWConfig Ês2 / Noc2 ±0.6 dB averaged over BWConfig Noc3 ±1.0 dB averaged over BWConfig Ês3 / Noc3 ±0.6 dB averaged over BWConfig | NOTE: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 3 frequency Ês2 / Noc2 is the ratio of cell 3 signal / AWGN Noc3 is the AWGN on cell 3 frequency Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Ês1 / Noc1 uncertainty, Ês2 / Noc2 uncertainty, Ês3 / Noc3 uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB |
| 8.23.5 E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in asynchronous DC | Same as 8.23.4 | Same as 8.23.4 |
| 8.23.6 E-UTRAN TDD-TDD DC inter-frequency event triggered reporting with DRX in synchronous DC | Same as 8.23.4 | Same as 8.23.4 |
| 8.26.1 E-UTRAN FDD-FS3 Activation and deactivation of known FS3 SCell with FDD PCell in non-DRX | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig Noc2 ±1.5dB averaged over BWConfig Ês2 / Noc2 ±0.3 dB averaged over BWConfig Time alignment error cell 2 rel. to cell 1: Inter-band CA: ±260 ns (±8Ts) | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 8.26.2 E-UTRAN TDD-FS3 Activation and deactivation of known FS3 SCell with TDD PCell in non-DRX | Same as 8.26.1 | Same as 8.26.1 |
| 8.26.3 E-UTRAN FDD-FS3 Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig Noc2 ±1.5dB averaged over BWConfig Ês2 / Noc2 ±0.3 dB averaged over BWConfig Ês3 / Noc2 ±0.3 dB averaged over BWConfig Time alignment error cell 2 rel. to cell 1: Inter-band CA: ±260 ns (±8Ts) | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc2 is the ratio of cell 3 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 8.26.3A E-UTRAN FDD-FS3 3-DL Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.3 dB averaged over BWConfig Noc2 ±1.5dB averaged over BWConfig Ês2 / Noc2 ±0.3 dB averaged over BWConfig Ês3 / Noc2 ±0.3 dB averaged over BWConfig Ês4 / Noc2 ±0.3 dB averaged over BWConfig Time alignment error cell 2 rel. to cell 1: Inter-band CA: ±260 ns (±8Ts) | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc2 is the ratio of cell 3 signal / AWGN Ês4 / Noc2 is the ratio of cell 4 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 8.26.4 E-UTRAN TDD-FS3 Event triggered reporting on deactivated FS3 SCell and TDD PCell interruption in non-DRX | Same as 8.26.3 | Same as 8.26.3 |
| 8.26.5 E-UTRAN FDD-TDD intra-frequency event triggered reporting in non-DRX for CRS based discovery signal under Operation with Frame Structure 3 | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.6 dB averaged over BWConfig Noc2 ±1.5dB averaged over BWConfig Ês2 / Noc2 ±0.6 dB averaged over BWConfig Ês3 / Noc2 ±0.6 dB averaged over BWConfig Time alignment error cell 2 rel. to cell 1: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc2 is the ratio of cell 3 signal / AWGN Ês1 / Noc1, Ês2 / Noc2 and Ês3 / Noc2 uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 8.26.6 E-UTRAN TDD-TDD intra-frequency event triggered reporting in non-DRX for CRS based discovery signal under Operation with Frame Structure 3 | Same as 8.26.5 | Same as 8.26.5 |
| 8.26.7 E-UTRAN FDD-FS3 intra-frequency event triggered reporting in DRX for CRS based discovery signal | Same as 8.26.5 | Same as 8.26.5 |
| 8.26.8 E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal | Same as 8.26.5 | Same as 8.26.5 |
| 8.26.9 E-UTRAN FDD-FS3 inter-frequency event triggered reporting under fading propagation conditions in synchronous cells | Noc1 ±1.0 dB averaged over BWConfig Ês1 / Noc1 ±0.6 dB averaged over BWConfig Noc2 ±1.5dB averaged over BWConfig Ês2 / Noc2 ±0.6 dB averaged over BWConfig Time alignment error cell 2 rel. to cell 1: Inter-band CA: ±260 ns (±8Ts) | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês1 / Noc1, Ês2 / Noc2 uncertainty for fading condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Ês / Noc uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 8.26.10 E-UTRAN TDD-FS3 inter-frequency event triggered reporting under fading propagation conditions in synchronous cells | Same as 8.26.9 | Same as 8.26.9 |
| 9.1.1.1 FDD Intra Frequency Absolute RSRP Accuracy | Noc ±1.0 dB averaged over BWConfig Noc ±1.3 dB for PRBs #22-27 Ês1 / Noc and Ês2 / Noc each ±0.3 dB averaged over BWConfig Ês1 / Noc and Ês2 / Noc each ±0.8 dB for PRBs #22-27 | NOTE: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 9.1.1.1_1 FDD Intra Frequency Absolute RSRP Accuracy (Rel 12 and forward) | Same as 9.1.1.1 | Same as 9.1.1.1 |
| 9.1.1.1_2 FDD Intra Frequency Absolute RSRP Accuracy for UE category 1bis | Same as 9.1.1.1 | Same as 9.1.1.1 |
| 9.1.1.1_3 FDD Intra Frequency Absolute RSRP Accuracy for CA Idle Mode Measurements | Same as 9.1.1.1 | Same as 9.1.1.1 |
| 9.1.1.2 FDD Intra Frequency Relative Accuracy of RSRP | Noc ±1.0 dB averaged over BWConfig Noc ±1.3 dB for PRBs #22-27 Ês1 / Noc and Ês2 / Noc each ±0.3 dB averaged over BWConfig Ês1 / Noc and Ês2 / Noc each ±0.8 dB for PRBs #22-27 | NOTE: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 9.1.1.2_2 FDD Intra Frequency Relative Accuracy of RSRP for UE category 1bis | Same as 9.1.1.2 | Same as 9.1.1.2 |
| 9.1.2.1 TDD Intra Frequency Absolute RSRP Accuracy | Same as 9.1.1.1 | Same as 9.1.1.1 |
| 9.1.2.1_1 TDD Intra Frequency Absolute RSRP Accuracy (Rel 12 and forward) | Same as 9.1.1.1 | Same as 9.1.1.1 |
| 9.1.2.1_2 TDD Intra Frequency Absolute RSRP Accuracy for UE category 1bis | Same as 9.1.1.1 | Same as 9.1.1.1 |
| 9.1.2.2 TDD Intra Frequency Relative RSRP Accuracy | Same as 9.1.1.2 | Same as 9.1.1.2 |
| 9.1.2.2_2 TDD Intra Frequency Relative Accuracy of RSRP for UE category 1bis | Same as 9.1.1.2 | Same as 9.1.1.2 |
| 9.1.3.1 FDD Inter Frequency Absolute RSRP Accuracy | Noc1 and Noc2 each ±1.0 dB averaged over BWConfig Noc1 and Noc2 each ±1.3 dB for PRBs #22-27 Ês1 / Noc1 and Ês2 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1 and Ês2 / Noc2 each ±0.8 dB for PRBs #22-27 | NOTE: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |
| 9.1.3.1_1 FDD Inter Frequency Absolute RSRP Accuracy (Rel 12 and forward) | Same as 9.1.3.1 | Same as 9.1.3.1 |
| 9.1.3.1_2 FDD - FDD Inter Frequency Absolute RSRP Accuracy for UE category 1bis | Same as 9.1.3.1 | Same as 9.1.3.1 |
| 9.1.3.1_3 FDD-FDD Inter Frequency Absolute RSRP Accuracy for CA Idle Mode Measurements for Overlapping Carrier | Same as 9.1.3.1 | Same as 9.1.3.1 |
| 9.1.3.1_4 FDD-FDD Inter Frequency Absolute RSRP Accuracy for CA Idle Mode Measurements for Non-Overlapping Carrier | Same as 9.1.3.1 | Same as 9.1.3.1 |
| 9.1.3.2 FDD Inter Frequency Relative RSRP Accuracy | Same as 9.1.3.1 | Same as 9.1.3.1 |
| 9.1.3.2_1 FDD Inter Frequency Relative RSRP Accuracy (Rel 12 and forward) | Same as 9.1.3.1 | Same as 9.1.3.1 |
| 9.1.3.2_2 FDD - FDD Inter Frequency Relative Accuracy of RSRP for UE category 1bis | Same as 9.1.3.1 | Same as 9.1.3.1 |
| 9.1.4.1 TDD Inter Frequency Absolute RSRP Accuracy | Same as 9.1.3.1 | Same as 9.1.3.1 |
| 9.1.4.1_1 TDD Inter Frequency Absolute RSRP Accuracy (Rel 12 and forward) | Same as 9.1.3.1 | Same as 9.1.3.1 |
| 9.1.4.1_2 TDD - TDD Inter Frequency Absolute RSRP Accuracy for UE category 1bis | Same as 9.1.3.1 | Same as 9.1.3.1 |
| 9.1.4.2 TDD Inter Frequency Relative RSRP Accuracy | Same as 9.1.3.1 | Same as 9.1.3.1 |
| 9.1.4.2_1 TDD Inter Frequency Relative RSRP Accuracy (Rel 12 and forward) | Same as 9.1.3.1 | Same as 9.1.3.1 |
| 9.1.4.2_2 TDD - TDD Inter Frequency Relative Accuracy of RSRP for UE category 1bis | Same as 9.1.3.1 | Same as 9.1.3.1 |
| 9.1.5.1 FDD - TDD Inter Frequency Absolute RSRP Accuracy | Noc1 and Noc2 each ±1.0 dB averaged over BWConfig Noc1 and Noc2 each ±1.3 dB for PRBs #22-27 Ês1 / Noc1 and Ês2 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1 and Ês2 / Noc2 each ±0.8 dB for PRBs #22-27 | NOTE: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |
| 9.1.5.1_1 FDD - TDD Inter Frequency Absolute RSRP Accuracy (Rel 12 and forward) | Same as 9.1.5.1 | Same as 9.1.5.1 |
| 9.1.5.1_2 FDD - TDD Inter Frequency Absolute RSRP Accuracy for UE category 1bis | Same as 9.1.5.1 | Same as 9.1.5.1 |
| 9.1.5.2 FDD - TDD Inter Frequency Relative Accuracy of RSRP | Noc1 and Noc2 each ±1.0 dB averaged over BWConfig Noc1 and Noc2 each ±1.3 dB for PRBs #22-27 Ês1 / Noc1 and Ês2 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1 and Ês2 / Noc2 each ±0.8 dB for PRBs #22-27 | NOTE: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |
| 9.1.5.2_1 FDD - TDD Inter Frequency Relative Accuracy of RSRP (Rel 12 and forward) | Same as 9.1.5.2 | Same as 9.1.5.2 |
| 9.1.5.2_2 FDD - TDD Inter Frequency Relative Accuracy of RSRP for UE category 1bis | Same as 9.1.5.2 | Same as 9.1.5.2 |
| 9.1.6.1 FDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB for PRBs #22-27 Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB for PRBs #22-27 Ês1 / Noc1, Ês2 / Noc2 and Ês3 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1, Ês2 / Noc2 and Ês3 / Noc2 each ±0.8 dB for PRBs #22-27 Time alignment error cell 2 rel. to cell 1: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc2 is the ratio of cell 3 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 9.1.6.1_1 Measurement Performance Requirements / FDD Absolute RSRP Accuracy E-UTRA for Carrier Aggregation (Rel 12 and forward) | Same as 9.1.6.1 | Same as 9.1.6.1 |
| 9.1.6.2 FDD Relative RSRP Accuracy for E-UTRA Carrier Aggregation | Same as 9.1.6.1 |  |
| 9.1.6.2_1 FDD Relative RSRP Accuracy E-UTRA for Carrier Aggregation (Rel 12 and forward) | Same as 9.1.6.1 | Same as 9.1.6.1 |
| 9.1.7.1 TDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation | Same as 9.1.6.1 |  |
| 9.1.7.1_1 TDD Absolute RSRP Accuracy E-UTRA for Carrier Aggregation (Rel 12 and forward) | Same as 9.1.6.1 |  |
| 9.1.7.2 TDD Relative RSRP Accuracy for E-UTRA Carrier Aggregation | Same as 9.1.6.1 |  |
| 9.1.7.2_1 TDD Relative RSRP Accuracy E-UTRA for Carrier Aggregation (Rel 12 and forward) | Same as 9.1.6.2_1 | Same as 9.1.6.2_1 |
| 9.1.8.1 FDD Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction with Non-MBSFN ABS | Noc ±1.0 dB averaged over BWConfig Noc ±1.3 dB for PRBs #22-27 Ês1 / Noc and Ês2 / Noc each ±0.3 dB averaged over BWConfig Ês1 / Noc and Ês2 / Noc each ±0.8 dB for PRBs #22-27 | NOTE: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 9.1.8.1_1 FDD Absolute RSRP Accuracy under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS (eICIC) (Rel-12 and forward) | Same as 9.1.8.1 | Same as 9.1.8.1 |
| 9.1.8.2 FDD Relative RSRP Accuracy under Time Domain Measurement Resource Restriction with Non-MBSFN ABS | Same as 9.1.8.1 |  |
| 9.1.9.1 TDD Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction with Non-MBSFN ABS | Same as 9.1.8.1 |  |
| 9.1.9.1_1 TDD Absolute RSRP Accuracy under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS (eICIC) (Rel 12 and forward) | Same as 9.1.8.1 | Same as 9.1.8.1 |
| 9.1.9.2 TDD Relative RSRP Accuracy under Time Domain Measurement Resource Restriction with Non-MBSFN ABS | Same as 9.1.8.1 |  |
| 9.1.10.1 FDD Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction with MBSFN ABS | Same as 9.1.8.1 |  |
| 9.1.10.1_1 FDD Absolute RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS (eICIC) (Rel-12 and forward) | Same as 9.1.8.1 |  |
| 9.1.10.2 FDD Relative RSRP Accuracy under Time Domain Measurement Resource Restriction with MBSFN ABS | Same as 9.1.8.1 |  |
| 9.1.11.1 TDD Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction with MBSFN ABS | Same as 9.1.8.1 |  |
| 9.1.11.1_1 TDD Absolute RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS (eICIC) (Rel 12 and forward) | Same as 9.1.8.1 |  |
| 9.1.11.2 TDD Relative RSRP Accuracy under Time Domain Measurement Resource Restriction with MBSFN ABS | Same as 9.1.8.1 |  |
| 9.1.12.1 FDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation for 20MHz | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB for PRBs #47-52 Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB for PRBs #47-52 Ês1 / Noc1, Ês2 / Noc2 and Ês3 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1, Ês2 / Noc2 and Ês3 / Noc2 each ±0.8 dB for PRBs #47-52 Time alignment error cell 2 rel. to cell 1: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc2 is the ratio of cell 3 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 9.1.12.1_1 Measurement Performance Requirements / FDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation for 20 MHz (Rel 12 and forward) | Same as 9.1.12.1 | Same as 9.1.12.1 |
| 9.1.12.2 FDD Relative RSRP Accuracy for E-UTRA Carrier Aggregation for 20MHz | Same as 9.1.12.1 |  |
| 9.1.12.2_1 FDD Relative RSRP Accuracy for E-UTRA Carrier Aggregation for 20 MHz (Rel 12 and forward) | Same as 9.1.12.1 | Same as 9.1.12.1 |
| 9.1.13.1 TDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation for 20MHz | Same as 9.1.12.1 |  |
| 9.1.13.1_1 TDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation for 20 MHz (Rel 12 and forward) | Same as 9.1.12.1 |  |
| 9.1.13.2 TDD Relative RSRP Accuracy for E-UTRA Carrier Aggregation for 20MHz | Same as 9.1.12.1 |  |
| 9.1.13.2_1 TDD Relative RSRP Accuracy for E-UTRA Carrier Aggregation for 20 MHz (Rel 12 and forward) | Same as 9.1.12.2_1 | Same as 9.1.12.2_1 |
| 9.1.14.1 FDD Intra Frequency Absolute RSRP Accuracy under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) | Noc ±1.0 dB averaged over BWConfig Noc ±1.3 dB for PRBs #22-27 Ês1 / Noc, Ês2 / Noc and Ês3 / Noc each ±0.3 dB averaged over BWConfig Ês1 / Noc, Ês2 / Noc and Ês3 / Noc each ±0.8 dB for PRBs #22-27 | NOTE: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN Ês3 / Noc is the ratio of cell 2 signal / AWGN |
| 9.1.14.1_1 FDD Intra Frequency Absolute RSRP Accuracy under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) (Rel 12 and forward) | Same as 9.1.14.1 | Same as 9.1.14.1 |
| 9.1.14.2 FDD Intra Frequency Relative RSRP Accuracy under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) | Noc ±1.0 dB averaged over BWConfig Noc ±1.3 dB for PRBs #22-27 Ês1 / Noc, Ês2 / Noc and Ês3 / Noc each ±0.3 dB averaged over BWConfigÊs1 / Noc, Ês2 / Noc and Ês3 / Noc each ±0.8 dB for PRBs #22-27 | NOTE: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN Ês3 / Noc is the ratio of cell 3 signal / AWGN |
| 9.1.15.1 TDD Intra Frequency Absolute RSRP Accuracy under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) | Same as 9.1.14.1 | Same as 9.1.14.1 |
| 9.1.15.2 TDD Intra Frequency Relative RSRP Accuracy under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) | Same as 9.1.14.2 |  |
| 9.1.16.1 FDD Intra Frequency Absolute RSRP Accuracy for 5MHz Bandwidth | Noc ±1.0 dB averaged over BWConfig Noc ±1.3 dB for PRBs #10-15 Ês1 / Noc and Ês2 / Noc each ±0.3 dB averaged over BWConfig Ês1 / Noc and Ês2 / Noc each ±0.8 dB for PRBs #10-15 | NOTE: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 9.1.16.1_1 FDD Intra Frequency Absolute RSRP Accuracy for 5MHz Bandwidth (Rel 12 and forward) | Same as 9.1.16.1 | Same as 9.1.16.1 |
| 9.1.16.2 FDD Intra Frequency Relative Accuracy of RSRP for 5MHz Bandwidth | Noc ±1.0 dB averaged over BWConfig Noc ±1.3 dB for PRBs #10-15 Ês1 / Noc and Ês2 / Noc each ±0.3 dB averaged over BWConfig Ês1 / Noc and Ês2 / Noc each ±0.8 dB for PRBs #10-15 | NOTE: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 9.1.17.1 FDD Inter Frequency Absolute RSRP Accuracy for 5MHz Bandwidth | Noc1 and Noc2 each ±1.0 dB averaged over BWConfig Noc1 and Noc2 each ±1.3 dB for PRBs #10-15 Ês1 / Noc1 and Ês2 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1 and Ês2 / Noc2 each ±0.8 dB for PRBs #10-15 | NOTE: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |
| 9.1.17.2 FDD - FDD Inter Frequency Relative Accuracy of RSRP for 5MHz Bandwidth | Noc1 and Noc2 each ±1.0 dB averaged over BWConfig Noc1 and Noc2 each ±1.3 dB for PRBs #10-15 Ês1 / Noc1 and Ês2 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1 and Ês2 / Noc2 each ±0.8 dB for PRBs #10-15 | NOTE: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |
| 9.1.18.1 FDD Absolute RSRP Accuracy for E-UTRA for Carrier Aggregation for 10MHz + 5MHz | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB for PRBs #22-27 Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB for PRBs #10-15 Ês1 / Noc1, Ês2 / Noc2 and Ês3 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1 ±0.8 dB for PRBs #22-27 Ês2 / Noc2 and Ês3 / Noc2 each ±0.8 dB for PRBs #10-15 Time alignment error cell 2 rel. to cell 1: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc2 is the ratio of cell 3 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 9.1.18.2 FDD Relative RSRP Accuracy E-UTRA for Carrier Aggregation for 10MHz + 5MHz | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB for PRBs #22-27 Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB for PRBs #10-15 Ês1 / Noc1, Ês2 / Noc2 and Ês3 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1 ±0.8 dB for PRBs #22-27 Ês2 / Noc2 and Ês3 / Noc2 each ±0.8 dB for PRBs #10-15 Time alignment error cell 2 rel. to cell 1: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc2 is the ratio of cell 3 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 9.1.18.2_1 FDD Relative RSRP Accuracy E-UTRA for Carrier Aggregation for 10MHz + 5MHz (Rel-12 and forward) | Same as 9.1.18.2 | Same as 9.1.18.2 |
| 9.1.19.1 TDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation for 10MHz + 5MHz | Same as 9.1.18.1 | Same as 9.1.18.1 |
| 9.1.19.2 TDD Relative RSRP Accuracy E-UTRA for Carrier Aggregation for 10MHz + 5MHz | Same as 9.1.18.2 | Same as 9.1.18.2 |
| 9.1.19.2_1 TDD Relative RSRP Accuracy for E-UTRA Carrier Aggregation for 10MHz + 5MHz (Rel-12 and forward) | Same as 9.1.18.2 | Same as 9.1.18.2 |
| 9.1.20.1 FDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation for 5MHz + 5MHz bandwidth | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB for PRBs #10-15 Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB for PRBs #10-15 Ês1 / Noc1, Ês2 / Noc2 and Ês3 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1, Ês2 / Noc2 and Ês3 / Noc2 each ±0.8 dB for PRBs #10-15 Time alignment error cell 2 rel. to cell 1: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc2 is the ratio of cell 3 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 9.1.20.2 FDD Relative RSRP Accuracy for E-UTRA Carrier Aggregation for 5MHz + 5MHz bandwidth | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB for PRBs #10-15 Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB for PRBs #10-15 Ês1 / Noc1, Ês2 / Noc2 and Ês3 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1, Ês2 / Noc2 and Ês3 / Noc2 each ±0.8 dB for PRBs #10-15 Time alignment error cell 2 rel. to cell 1: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc2 is the ratio of cell 3 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 9.1.21.1 TDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation for 5MHz + 5MHz bandwidth | Same as 9.1.20.1 | Same as 9.1.20.1 |
| 9.1.21.2 TDD Relative RSRP Accuracy for E-UTRA Carrier Aggregation for 5MHz + 5MHz bandwidth | Same as 9.1.20.2 | Same as 9.1.20.2 |
| 9.1.22 FDD-TDD RSRP Accuracy E-UTRA for Carrier Aggregation with PCell in FDD | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB over UE Measurement bandwidth Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB over UE Measurement bandwidth Ês1 / Noc1, Ês2 / Noc2, Ês3 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1, Ês2 / Noc2, Ês3 / Noc2 each ±0.8 dB over UE Measurement bandwidth Time alignment errors: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | Notes: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc2 is the ratio of cell 3 signal / AWGN UE Measurement bandwidth: 5MHz Ch BW: PRBs 10-15 10MHz Ch BW: PRBs 22-27 20MHz Ch BW: PRBs 47-52 TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 9.1.23 FDD-TDD RSRP Accuracy E-UTRA for Carrier Aggregation with PCell in TDD | Same as 9.1.22 | Same as 9.1.22 |
| 9.1.24.1 TDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation for 20MHz + 10MHz | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB for PRBs #47-52 Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB for PRBs #22-27 Ês1 / Noc1, Ês2 / Noc2 and Ês3 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1 ±0.8 dB for PRBs #47-52 Ês2 / Noc2 and Ês3 / Noc2 each ±0.8 dB for PRBs #22-27 Time alignment error cell 2 rel. to cell 1: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc2 is the ratio of cell 3 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 9.1.24.1_1 TDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation for 20MHz + 10MHz (Rel-12 and forward) | Same as 9.1.24.1 | Same as 9.1.24.1 |
| 9.1.24.2 TDD Relative RSRP Accuracy for E-UTRA Carrier Aggregation for 20MHz + 10MHz | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB for PRBs #47-52 Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB for PRBs #22-27 Ês1 / Noc1, Ês2 / Noc2 and Ês3 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1 ±0.8 dB for PRBs #47-52 Ês2 / Noc2 and Ês3 / Noc2 each ±0.8 dB for PRBs #22-27 Time alignment error cell 2 rel. to cell 1: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc2 is the ratio of cell 3 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 9.1.24.2_1 TDD Relative RSRP Accuracy for E-UTRA Carrier Aggregation for 20MHz + 10MHz (Rel-12 and forward) | Same as 9.1.24.2 | Same as 9.1.24.2 |
| 9.1.25 FDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal | Same as 9.1.1.1 | Same as 9.1.1.1 |
| 9.1.26 TDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal | Same as 9.1.1.1 | Same as 9.1.1.1 |
| 9.1.27 FDD—FDD inter-frequency absolute and relative RSRP accuracies in CRS based discovery signal | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB for PRBs #22-27 Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB for PRBs #22-27 Ês1 / Noc1, Ês2 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1, Ês2 / Noc2 each ±0.8 dB for PRBs #22-27 | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |
| 9.1.28 TDD—TDD inter-frequency absolute and relative RSRP accuracies in CRS based discovery signal | Same as 9.1.27 | Same as 9.1.27 |
| 9.1.29 FDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal | Noc ±1.0 dB averaged over BWConfig Noc ±1.3 dB for PRBs #22-27 CRS Ês1 / Noc and CRS Ês2 / Noc each ±0.3 dB averaged over BWConfig CSI-RS Ês1 / Noc and CSI-RS Ês2 / Noc each ±0.3 dB averaged over BWConfig CRS Ês1 / Noc and CRS Ês2 / Noc each ±0.8 dB for PRBs #22-27 CSI-RS Ês1 / Noc and CSI-RS Ês2 / Noc each ±0.8 dB for PRBs #22-27 | NOTE: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 9.1.30 TDD intra-frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal | Same as 9.1.29 | Same as 9.1.29 |
| 9.1.31 FDD-FDD inter-frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB for PRBs #22-27 Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB for PRBs #22-27 Ês1 / Noc1, Ês2 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1, Ês2 / Noc2 each ±0.8 dB for PRBs #22-27 | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |
| 9.1.32 TDD-TDD inter-frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal | Same as 9.1.32 | Same as 9.1.32 |
| 9.1.33 FDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB for PRBs #22-27 Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB for PRBs #22-27 Ês1 / Noc1, Ês2 / Noc2 and Ês3 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1, Ês2 / Noc2 and Ês3 / Noc2 each ±0.8 dB for PRBs #22-27 | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc2 is the ratio of cell 3 signal / AWGN |
| 9.1.34 TDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal | Same as 9.1.33 | Same as 9.1.33 |
| 9.1.35 FDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB for PRBs #22-27 Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB for PRBs #22-27 CSI-RS Ês1 / Noc1, CSI-RS Ês2 / Noc2, and CSI-RS Ês3 / Noc3 each ±0.3 dB averaged over BWConfig CSI-RS Ês1 / Noc1, CSI-RS Ês2 / Noc2, and CSI-RS Ês3 / Noc3 each ±0.8 dB for PRBs #22-27 | NOTE: Noc1 is the AWGN on frequency 1 CSI-RS Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 CSI-RS Ês2 / Noc2 is the ratio of cell 2 signal / AWGN CSI-RS Ês3 / Noc2 is the ratio of cell 3 signal / AWGN |
| 9.1.36 TDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal | Same as 9.1.35 | Same as 9.1.35 |
| 9.1.37 3DL PCell in FDD RSRP for E-UTRAN in Carrier Aggregation | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB over UE Measurement bandwidth Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB over UE Measurement bandwidth Noc3 ±1.0 dB averaged over BWConfig Noc3 ±1.3 dB over UE Measurement bandwidth Ês1 / Noc1, Ês2 / Noc2, Ês3 / Noc2, Ês4 / Noc3, Ês5 / Noc3 each ±0.3 dB averaged over BWConfig Ês1 / Noc1, Ês2 / Noc2, Ês3 / Noc2, Ês4 / Noc3, Ês5 / Noc3 each ±0.8 dB over UE Measurement bandwidth Time alignment errors: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | Notes: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc2 is the ratio of cell 3 signal / AWGN Noc3 is the AWGN on frequency 3 Ês4 / Noc3 is the ratio of cell 4 signal / AWGN Ês5 / Noc3 is the ratio of cell 5 signal / AWGN UE Measurement bandwidth: 5MHz Ch BW: PRBs 10-15 10MHz Ch BW: PRBs 22-27 20MHz Ch BW: PRBs 47-52 TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 9.1.38 3DL PCell in TDD RSRP for E-UTRAN in Carrier Aggregation | Same as 9.1.37 | Same as 9.1.37 |
| 9.1.39 3DL FDD RSRP for E-UTRAN in Carrier Aggregation | Same as 9.1.37 | Same as 9.1.37 |
| 9.1.39_1 3DL FDD RSRP for E-UTRAN in Carrier Aggregation (Rel-12 and forward) | Same as 9.1.37 | Same as 9.1.37 |
| 9.1.40 3DL TDD RSRP for E-UTRAN in Carrier Aggregation | Same as 9.1.37 | Same as 9.1.37 |
| 9.1.40_1 3DL TDD RSRP for E-UTRAN in Carrier Aggregation (Rel-12 and forward) | Same as 9.1.37 | Same as 9.1.37 |
| 9.1.41.1 FD-FDD Intra Frequency Absolute RSRP Accuracy for UE category 0 | Same as 9.1.1.1 | Same as 9.1.1.1 |
| 9.1.41.2 FD-FDD Intra Frequency Relative RSRP Accuracy for UE category 0 | Same as 9.1.1.2 | Same as 9.1.1.2 |
| 9.1.42.1 HD-FDD Intra Frequency Absolute RSRP Accuracy for UE category 0 | Same as 9.1.1.1 | Same as 9.1.1.1 |
| 9.1.42.2 HD-FDD Intra Frequency Relative RSRP Accuracy for UE category 0 | Same as 9.1.1.2 | Same as 9.1.1.2 |
| 9.1.43.1 TDD Intra Frequency Absolute RSRP Accuracy for UE category 0 | Same as 9.1.1.1 | Same as 9.1.1.1 |
| 9.1.43.2 TDD Intra Frequency Relative RSRP Accuracy for UE category 0 | Same as 9.1.1.2 | Same as 9.1.1.2 |
| 9.1.44 4 DL CA PCell in FDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB over UE Measurement bandwidth Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB over UE Measurement bandwidth Noc3 ±1.0 dB averaged over BWConfig Noc3 ±1.3 dB over UE Measurement bandwidth Noc4 ±1.0 dB averaged over BWConfig Noc4 ±1.3 dB over UE Measurement bandwidth Ês1 / Noc1, Ês2 / Noc2, Ês3 / Noc2, Ês4 / Noc3, Ês5 / Noc3, Ês6 / Noc4, Ês7 / Noc4 each ±0.3 dB averaged over BWConfig Ês1 / Noc1, Ês2 / Noc2, Ês3 / Noc2, Ês4 / Noc3, Ês5 / Noc3, Ês6 / Noc4, Ês7 / Noc4 each ±0.8 dB over UE Measurement bandwidth Time alignment errors: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | Notes: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc2 is the ratio of cell 3 signal / AWGN Noc3 is the AWGN on frequency 3 Ês4 / Noc3 is the ratio of cell 4 signal / AWGN Ês5 / Noc3 is the ratio of cell 5 signal / AWGN Noc4 is the AWGN on frequency 4 Ês6 / Noc4 is the ratio of cell 6 signal / AWGN Ês7 / Noc4 is the ratio of cell 7 signal / AWGN UE Measurement bandwidth: 5MHz Ch BW: PRBs 10-15 10MHz Ch BW: PRBs 22-27 20MHz Ch BW: PRBs 47-52 TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 9.1.45 4 DL CA PCell in TDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation | Same as 9.1.44 | Same as 9.1.44 |
| 9.1.46 4DL FDD RSRP for E-UTRAN in Carrier Aggregation | Same as 9.1.44 | Same as 9.1.44 |
| 9.1.47 4DL TDD RSRP for E-UTRAN in Carrier Aggregation | Same as 9.1.44 | Same as 9.1.44 |
| 9.1.48 5 DL FDD-TDD with PCell in FDD RSRP for E-UTRAN in Carrier Aggregation | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB over UE Measurement bandwidth Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB over UE Measurement bandwidth Noc3 ±1.0 dB averaged over BWConfig Noc3 ±1.3 dB over UE Measurement bandwidth Noc4 ±1.0 dB averaged over BWConfig Noc4 ±1.3 dB over UE Measurement bandwidth Noc5 ±1.0 dB averaged over BWConfig Noc5 ±1.3 dB over UE Measurement bandwidth Ês1 / Noc1, Ês2 / Noc2, Ês3 / Noc2, Ês4 / Noc3, Ês5 / Noc3, Ês6 / Noc4, Ês7 / Noc4 each ±0.3 dB averaged over BWConfig Ês1 / Noc1, Ês2 / Noc2, Ês3 / Noc2, Ês4 / Noc3, Ês5 / Noc3, Ês6 / Noc4, Ês7 / Noc4, Ês8 / Noc5, Ês8 / Noc5 each ±0.8 dB over UE Measurement bandwidth Time alignment errors: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | Notes: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc2 is the ratio of cell 3 signal / AWGN Noc3 is the AWGN on frequency 3 Ês4 / Noc3 is the ratio of cell 4 signal / AWGN Ês5 / Noc3 is the ratio of cell 5 signal / AWGN Noc4 is the AWGN on frequency 4 Ês6 / Noc4 is the ratio of cell 6 signal / AWGN Ês7 / Noc4 is the ratio of cell 7 signal / AWGN Noc5 is the AWGN on frequency 5 Ês8 / Noc5 is the ratio of cell 8 signal / AWGN Ês9 / Noc5 is the ratio of cell 9 signal / AWGN UE Measurement bandwidth: 5MHz Ch BW: PRBs 10-15 10MHz Ch BW: PRBs 22-27 20MHz Ch BW: PRBs 47-52 TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 9.1.49 5 DL FDD-TDD with PCell in TDD RSRP for E-UTRAN in Carrier Aggregation | Same as 9.1.48 | Same as 9.1.48 |
| 9.1.50 5 DL FDD RSRP for E-UTRAN in Carrier Aggregation | Same as 9.1.48 | Same as 9.1.48 |
| 9.1.51 5 DL TDD RSRP for E-UTRAN in Carrier Aggregation | Same as 9.1.48 | Same as 9.1.48 |
| 9.1.52 FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA | Noc ±1.0 dB averaged over BWConfig Noc ±1.3 dB for PRBs #22-27 Ês1 / Noc and Ês2 / Noc each ±0.3 dB averaged over BWConfig Ês1 / Noc and Ês2 / Noc each ±0.8 dB for PRBs #22-27 | NOTE: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 9.1.53 HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA | Same as 9.1.52 | Same as 9.1.52 |
| 9.1.54 TDD RSRP Intra frequency case for Cat-M1 UE in CEModeA | Same as 9.1.52 | Same as 9.1.52 |
| 9.1.55 FS3 Intra frequency absolute and relative RSRP accuracies with FDD PCell9.1.55 FS3 Intra frequency absolute and relative RSRP accuracies with FDD PCell | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB for PRBs #47-52 Noc2 ±1.5 dB averaged over BWConfig Noc2 ±1.8 dB for PRBs #47-52 Ês1 / Noc1, Ês2 / Noc2 and Ês3 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1, Ês2 / Noc2 and Ês3 / Noc2 each ±0.8 dB for PRBs #47-52 | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc2 is the ratio of cell 3 signal / AWGNNOTE: |
| 9.1.56 FS3 Intra frequency absolute and relative RSRP accuracies with TDD PCell9.1.56 FS3 Intra frequency absolute and relative RSRP accuracies with TDD PCell | Same as 9.1.55 | Same as 9.1.55 |
| 9.1.57 FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB | Noc ±1.0 dB averaged over BWConfig Noc ±1.3 dB for PRBs #22-27 Ês1 / Noc, Ês2 / Noc each ±0.3 dB averaged over BWConfig Ês1 / Noc, Ês2 / Noc each ±0.8 dB for PRBs #22-27 | NOTE: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 9.1.58 HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB | Same as 9.1.57 | Same as 9.1.57 |
| 9.1.59 TDD RSRP Intra frequency case for Cat-M1 UE in CEModeB | Same as 9.1.57 | Same as 9.1.57 |
| 9.1.60 FS3 absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal with FDD PCell | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB over UE Measurement bandwidth Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB over UE Measurement bandwidth CRS Ês1 / Noc1, CRS Ês2 / Noc2, CRS Ês3 / Noc2 each ±0.3 dB averaged over BWConfig CSI-RS Ês2 / Noc2, CSI-RS Ês3 / Noc2 each ±0.3 dB averaged over BWConfig CRS Ês2 / Noc2, CRS Ês3 / Noc2 each ±0.8 dB over UE Measurement bandwidth CSI-RS Ês1 / Noc1, CSI-RS Ês2 / Noc2, CSI-RS Ês3 / Noc2 each ±0.8 dB over UE Measurement bandwidth | Notes: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc2 is the ratio of cell 3 signal / AWGN |
| 9.1.61 FS3 absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal with TDD PCell | Same as 9.1.60 | Same as 9.1.60 |
| 9.1.68 3DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes | Same as 9.1.37 | Same as 9.1.37 |
| 9.1.69 4 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes | Same as 9.1.44 | Same as 9.1.44 |
| 9.1.70 5 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes | Same as 9.1.48 | Same as 9.1.48 |
| 9.1.71 6 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB over UE Measurement bandwidth Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB over UE Measurement bandwidth Noc3 ±1.0 dB averaged over BWConfig Noc3 ±1.3 dB over UE Measurement bandwidth Noc4 ±1.0 dB averaged over BWConfig Noc4 ±1.3 dB over UE Measurement bandwidth Noc5 ±1.0 dB averaged over BWConfig Noc5 ±1.3 dB over UE Measurement bandwidth Noc6 ±1.0 dB averaged over BWConfig Noc6 ±1.3 dB over UE Measurement bandwidth Ês1 / Noc1, Ês2 / Noc2, Ês3 / Noc2, Ês4 / Noc3, Ês5 / Noc3, Ês6 / Noc4, Ês7 / Noc4, Ês8 / Noc5, Ês9 / Noc5, Ês10 / Noc6, Ês11 / Noc6 each ±0.3 dB averaged over BWConfig Ês1 / Noc1, Ês2 / Noc2, Ês3 / Noc2, Ês4 / Noc3, Ês5 / Noc3, Ês6 / Noc4, Ês7 / Noc4, Ês8 / Noc5, Ês9 / Noc5, Ês10 / Noc6, Ês11 / Noc6 each ±0.8 dB over UE Measurement bandwidth Time alignment errors: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | Notes: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc2 is the ratio of cell 3 signal / AWGN Noc3 is the AWGN on frequency 3 Ês4 / Noc3 is the ratio of cell 4 signal / AWGN Ês5 / Noc3 is the ratio of cell 5 signal / AWGN Noc4 is the AWGN on frequency 4 Ês6 / Noc4 is the ratio of cell 6 signal / AWGN Ês7 / Noc4 is the ratio of cell 7 signal / AWGN Noc5 is the AWGN on frequency 5 Ês8 / Noc5 is the ratio of cell 8 signal / AWGN Ês9 / Noc5 is the ratio of cell 9 signal / AWGN Noc6 is the AWGN on frequency 6 Ês10 / Noc6 is the ratio of cell 10 signal / AWGN Ês11 / Noc6 is the ratio of cell 11 signal / AWGN UE Measurement bandwidth: 5MHz Ch BW: PRBs 10-15 10MHz Ch BW: PRBs 22-27 20MHz Ch BW: PRBs 47-52 TS = 1/(15000 x 2048) seconds, the basic timing unit defined in TS 36.211 [9] |
| 9.1.72 7 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB over UE Measurement bandwidth Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB over UE Measurement bandwidth Noc3 ±1.0 dB averaged over BWConfig Noc3 ±1.3 dB over UE Measurement bandwidth Noc4 ±1.0 dB averaged over BWConfig Noc4 ±1.3 dB over UE Measurement bandwidth Noc5 ±1.0 dB averaged over BWConfig Noc5 ±1.3 dB over UE Measurement bandwidth Noc6 ±1.3 dB over UE Measurement bandwidth Noc6 ±1.0 dB averaged over BWConfig Noc7 ±1.3 dB over UE Measurement bandwidth Noc7 ±1.0 dB averaged over BWConfig Ês1 / Noc1, Ês2 / Noc2, Ês3 / Noc2, Ês4 / Noc3, Ês5 / Noc3, Ês6 / Noc4, Ês7 / Noc4, Ês8 / Noc5, Ês9 / Noc5, Ês10 / Noc6, Ês11 / Noc6, Ês12 / Noc7, Ês13 / Noc7 each ±0.3 dB averaged over BWConfig Ês1 / Noc1, Ês2 / Noc2, Ês3 / Noc2, Ês4 / Noc3, Ês5 / Noc3, Ês6 / Noc4, Ês7 / Noc4, Ês8 / Noc5, Ês9 / Noc5, Ês10 / Noc6, Ês11 / Noc6, Ês12 / Noc7, Ês13 / Noc7 each ±0.8 dB over UE Measurement bandwidth Time alignment errors: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | Notes: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc2 is the ratio of cell 3 signal / AWGN Noc3 is the AWGN on frequency 3 Ês4 / Noc3 is the ratio of cell 4 signal / AWGN Ês5 / Noc3 is the ratio of cell 5 signal / AWGN Noc4 is the AWGN on frequency 4 Ês6 / Noc4 is the ratio of cell 6 signal / AWGN Ês7 / Noc4 is the ratio of cell 7 signal / AWGN Noc5 is the AWGN on frequency 5 Ês8 / Noc5 is the ratio of cell 8 signal / AWGN Ês9 / Noc5 is the ratio of cell 9 signal / AWGN Noc6 is the AWGN on frequency 6 Ês10 / Noc6 is the ratio of cell 10 signal / AWGN Ês11 / Noc6 is the ratio of cell 11 signal / AWGN Noc7 is the AWGN on frequency 7 Ês12 / Noc7 is the ratio of cell 12 signal / AWGN Ês13 / Noc7 is the ratio of cell 13 signal / AWGN UE Measurement bandwidth: 5MHz Ch BW: PRBs 10-15 10MHz Ch BW: PRBs 22-27 20MHz Ch BW: PRBs 47-52 TS = 1/(15000 x 2048) seconds, the basic timing unit defined in TS 36.211 [9] |
| 9.2.1.1 FDD Intra Frequency Absolute RSRQ Accuracy | Noc ±1.0 dB averaged over BWConfig Noc ±1.3 dB for PRBs #22-2 7 Ês1 / Noc and Ês2 / Noc each ±0.3 dB averaged over BWConfig Ês1 / Noc and Ês2 / Noc each ±0.8 dB for PRBs #22-27 | NOTE: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 9.2.1.1_1 FDD Intra Frequency Absolute RSRQ Accuracy for UE Category 1bis | Same as 9.2.1.1 | Same as 9.2.1.1 |
| 9.2.1.1_2 FDD Intra Frequency Absolute RSRQ Accuracy for UE category 1bis | Same as 9.2.1.1 | Same as 9.2.1.1 |
| 9.2.1.1_3 FDD Intra Frequency Absolute RSRQ Accuracy for CA Idle Mode Measurements | Same as 9.2.1.1 | Same as 9.2.1.1 |
| 9.2.2.1 TDD Intra Frequency Absolute RSRQ Accuracy | Same as 9.2.1.1 | Same as 9.2.1.1 |
| 9.2.2.1_1 TDD Intra Frequency Absolute RSRQ Accuracy for UE Category 1bis | Same as 9.2.1.1 | Same as 9.2.1.1 |
| 9.2.2.1_2 TDD Intra Frequency Absolute RSRQ Accuracy for UE category 1bis | Same as 9.2.1.1 | Same as 9.2.1.1 |
| 9.2.3.1 FDD - FDD Inter Frequency Absolute RSRQ Accuracy | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB for PRBs #22-27 Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB for PRBs #22-27 Ês1 / Noc1 and Ês2 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1 and Ês2 / Noc2 each ±0.8 dB for PRBs #22-27 | NOTE: Ês1 / Noc1 is the ratio of cell 1 signal / AWGN on frequency 1 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN on frequency 2 |
| 9.2.3.1_2 FDD - FDD Inter Frequency Absolute RSRQ Accuracy for UE Category 1bis | Same as 9.2.3.1 | Same as 9.2.3.1 |
| 9.2.3.1_3 FDD - FDD Inter Frequency Absolute RSRQ Accuracy for CA Idle Mode Measurements for Overlapping Carrier | Same as 9.2.3.1 | Same as 9.2.3.1 |
| 9.2.3.1_4 FDD - FDD Inter Frequency Absolute RSRQ Accuracy for CA Idle Mode Measurements for Non-Overlapping Carrier | Same as 9.2.3.1 | Same as 9.2.3.1 |
| 9.2.3.2 FDD - FDD Inter Frequency Relative RSRQ Accuracy | Same as 9.2.3.1 |  |
| 9.2.3.2_2 FDD - FDD Inter Frequency Relative RSRQ Accuracy for UE Category 1bis | Same as 9.2.3.1 | Same as 9.2.3.1 |
| 9.2.4.1 TDD - TDD Inter Frequency Absolute RSRQ Accuracy | Same as 9.2.3.1 |  |
| 9.2.4.1_1 TDD - TDD Inter Frequency Absolute RSRQ Accuracy (Rel-12 and forward) | Same as 9.2.3.1 |  |
| 9.2.4.1_2 TDD - TDD Inter Frequency Absolute RSRQ Accuracy for UE Category 1bis | Same as 9.2.3.1 | Same as 9.2.3.1 |
| 9.2.4.2 TDD - TDD Inter Frequency Relative RSRQ Accuracy | Same as 9.2.3.1 |  |
| 9.2.4.2_1 TDD - TDD Inter Frequency Relative RSRQ Accuracy (Rel-12 and forward) | Same as 9.2.3.2 |  |
| 9.2.4.2_2 TDD - TDD Inter Frequency Relative RSRQ Accuracy for UE Category 1bis | Same as 9.2.3.1 | Same as 9.2.3.1 |
| 9.2.4A.1 FDD - TDD Inter Frequency Absolute RSRQ Accuracy | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB for PRBs #22-27 Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB for PRBs #22-27 Ês1 / Noc1 and Ês2 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1 and Ês2 / Noc2 each ±0.8 dB for PRBs #22-27 | NOTE: Ês1 / Noc1 is the ratio of cell 1 signal / AWGN on frequency 1 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN on frequency 2 |
| 9.2.4A.1_2 FDD - TDD Inter Frequency Absolute RSRQ Accuracy for UE Category 1bis | Same as 9.2.4A.1 | Same as 9.2.4A.1 |
| 9.2.4A.2 FDD - TDD Inter Frequency Relative Accuracy of RSRQ | Same as 9.2.4A.1 |  |
| 9.2.4A.2_2 FDD - TDD Inter Frequency Relative Accuracy of RSRQ for UE Category 1bis | Same as 9.2.4A.1 | Same as 9.2.4A.1 |
| 9.2.5.1 FDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB for PRBs #22-27 Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB for PRBs #22-27 Ês1 / Noc1, Ês2 / Noc2 and Ês3 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1, Ês2 / Noc2 and Ês3 / Noc2 each ±0.8 dB for PRBs #22-27 Time alignment error cell 2 rel. to cell 1: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc2 is the ratio of cell 3 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 9.2.5.2 FDD Relative RSRQ Accuracy for E-UTRA Carrier Aggregation | Same as 9.2.5.1 |  |
| 9.2.6.1 TDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation | Same as 9.2.5.1 |  |
| 9.2.6.2 TDD Relative RSRQ Accuracy for E-UTRA Carrier Aggregation | Same as 9.2.5.1 |  |
| 9.2.7.1 FDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS | Noc ±1.0 dB averaged over BWConfig Noc ±1.3 dB for PRBs #22-27 Ês1 / Noc and Ês2 / Noc each ±0.3 dB averaged over BWConfig Ês1 / Noc and Ês2 / Noc each ±0.8 dB for PRBs #22-27 | NOTE: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 9.2.8.1 TDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS | Same as 9.2.7.1 | Same as 9.2.7.1 |
| 9.2.9.1 FDD Absolute RSRQ under Time Domain Measurement Resource Restriction with MBSFN ABS (eICIC) | Noc ±1.0 dB averaged over BWConfig Noc ±1.3 dB for PRBs #22-27 Ês1 / Noc and Ês2 / Noc each ±0.3 dB averaged over BWConfig Ês1 / Noc and Ês2 / Noc each ±0.8 dB for PRBs #22-27 | NOTE: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 9.2.10.1 TDD Absolute RSRQ under Time Domain Measurement Resource Restriction with MBSFN ABS (eICIC) | Same as 9.2.9.1 |  |
| 9.2.11.1 FDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation for 20MHz | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB for PRBs #47-52 Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB for PRBs #47-52 Ês1 / Noc1, Ês2 / Noc2 and Ês3 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1, Ês2 / Noc2 and Ês3 / Noc2 each ±0.8 dB for PRBs #47-52 Time alignment error cell 2 rel. to cell 1: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc2 is the ratio of cell 3 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 9.2.11.2 FDD Relative RSRQ Accuracy for E-UTRA Carrier Aggregation for 20MHz | Same as 9.2.11.1 |  |
| 9.2.12.1 TDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation for 20MHz | Same as 9.2.11.1 |  |
| 9.2.12.2 TDD Relative RSRQ Accuracy for E-UTRA Carrier Aggregation for 20MHz | Same as 9.2.11.1 |  |
| 9.2.15.1 FDD RSRQ Accuracy under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) | Noc ±1.0 dB averaged over BWConfig Noc ±1.3 dB for PRBs #22-27 Ês1 / Noc, Ês2 / Noc and Ês3 / Noc each ±0.3 dB averaged over BWConfig Ês1 / Noc, Ês2 / Noc and Ês3 / Noc each ±0.8 dB for PRBs #22-27 | NOTE: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN Ês3 / Noc is the ratio of cell 3 signal / AWGN |
| 9.2.16.1 TDD RSRQ Accuracy under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) | Same as 9.2.15.1 | Same as 9.2.15.1 |
| 9.2.17.1 FDD Intra Frequency Absolute RSRQ Accuracy for 5MHz Bandwidth | Noc ±1.0 dB averaged over BWConfig Noc ±1.3 dB for PRBs #10-15 Ês1 / Noc and Ês2 / Noc each ±0.3 dB averaged over BWConfig Ês1 / Noc and Ês2 / Noc each ±0.8 dB for PRBs #10-15 | NOTE: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 9.2.18.1 FDD - FDD Inter Frequency Absolute RSRQ Accuracy for 5MHz Bandwidth | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB for PRBs #10-15 Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB for PRBs #10-15 Ês1 / Noc1 and Ês2 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1 and Ês2 / Noc2 each ±0.8 dB for PRBs #10-15 | NOTE: Ês1 / Noc1 is the ratio of cell 1 signal / AWGN on frequency 1 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN on frequency 2 |
| 9.2.18.2 FDD - FDD Inter Frequency Relative RSRQ Accuracy for 5MHz Bandwidth | Same as 9.2.18.1 |  |
| 9.2.19.1 FDD-FDD Inter Frequency absolute WB-RSRQ accuracy | FFS | FFS |
| 9.2.20.1 TDD-TDD Inter Frequency absolute WB-RSRQ accuracy | FFS | FFS |
| 9.2.21.1 FDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation for 10MHz+5MHz | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB for PRBs #22-27 Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB for PRBs #10-15 Ês1 / Noc1, Ês2 / Noc2 and Ês3 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1 ±0.8 dB for PRBs #22-27 Ês2 / Noc2 and Ês3 / Noc2 each ±0.8 dB for PRBs #10-15 Time alignment error cell 2 rel. to cell 1: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc2 is the ratio of cell 3 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 9.2.21.2 FDD Relative RSRQ Accuracy for E-UTRA Carrier Aggregation for 10MHz+5MHz | Same as 9.2.21.1 |  |
| 9.2.22.1 TDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation for 10MHz+5MHz | Same as 9.2.21.1 |  |
| 9.2.22.2 TDD Relative RSRQ Accuracy for E-UTRA Carrier Aggregation for 10MHz+5MHz | Same as 9.2.21.1 |  |
| 9.2.23.1 FDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation for 5MHz+5MHz | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB for PRBs #10-15 Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB for PRBs #10-15 Ês1 / Noc1, Ês2 / Noc2 and Ês3 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1, Ês2 / Noc2 and Ês3 / Noc2 each ±0.8 dB for PRBs #10-15 Time alignment error cell 2 rel. to cell 1: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc2 is the ratio of cell 3 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 9.2.23.2 FDD Relative RSRQ Accuracy for E-UTRA Carrier Aggregation for 5MHz+5MHz | Same as 9.2.23.1 | Same as 9.2.23.1 |
| 9.2.24.1 TDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation for 5MHz+5MHz | Same as 9.2.23.1 | Same as 9.2.23.1 |
| 9.2.24.2 TDD Relative RSRQ Accuracy for E-UTRA Carrier Aggregation for 5MHz+5MHz | Same as 9.2.23.1 | Same as 9.2.23.1 |
| 9.2.25.1 Absolute RSRQ Accuracy for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB for PRBs #22-27 Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB for PRBs #22-27 Ês1 / Noc1 and Ês2 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1 and Ês2 / Noc2 each ±0.8 dB for PRBs #22-27 Time alignment error cell 2 rel. to cell 1: Inter-band CA: ±260 ns (±8Ts) | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 9.2.25.2 Relative RSRQ Accuracy for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD | Same as 9.2.25.1 |  |
| 9.2.26.1 Absolute RSRQ Accuracy for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD | Same as 9.2.25.1 |  |
| 9.2.26.2 Relative RSRQ Accuracy for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD | Same as 9.2.25.1 |  |
| 9.2.27.1 TDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation for 20MHz+10MHz | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB for PRBs #47-52 Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB for PRBs #22-27 Ês1 / Noc1, Ês2 / Noc2 and Ês3 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1 ±0.8 dB for PRBs #47-52 Ês2 / Noc2 and Ês3 / Noc2 each ±0.8 dB for PRBs #22-27 Time alignment error cell 2 rel. to cell 1: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc2 is the ratio of cell 3 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 9.2.27.2 TDD Relative RSRQ Accuracy for E-UTRA Carrier Aggregation for 20MHz+10MHz | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB for PRBs #47-52 Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB for PRBs #22-27 Ês1 / Noc1, Ês2 / Noc2 and Ês3 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1 ±0.8 dB for PRBs #47-52 Ês2 / Noc2 and Ês3 / Noc2 each ±0.8 dB for PRBs #22-27 Time alignment error cell 2 rel. to cell 1: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc2 is the ratio of cell 3 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 9.2.28 FDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal | Noc ±1.0 dB averaged over BWConfig Noc ±1.3 dB for PRBs #22-27 Ês1 / Noc and Ês2 / Noc each ±0.3 dB averaged over BWConfig Ês1 / Noc and Ês2 / Noc each ±0.8 dB for PRBs #22-27 | NOTE: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 9.2.29 TDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal | Same as 9.2.28 | Same as 9.2.28 |
| 9.2.30 FDD-FDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB for PRBs #22-27 Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB for PRBs #22-27 Ês1 / Noc1 and Ês2 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1 and Ês2 / Noc2 each ±0.8 dB for PRBs #22-27 | NOTE: Ês1 / Noc1 is the ratio of cell 1 signal / AWGN on frequency 1 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN on frequency 2 |
| 9.2.31 TDD-TDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal | Same as 9.2.30 | Same as 9.2.30 |
| 9.2.32 FDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB for PRBs #22-27 Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB for PRBs #22-27 Ês1 / Noc1, Ês2 / Noc2 and Ês3 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1, Ês2 / Noc2 and Ês3 / Noc2 each ±0.8 dB for PRBs #22-27 Time alignment error cell 2 rel. to cell 1: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc2 is the ratio of cell 3 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 9.2.33 TDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal | Same as 9.2.32 | Same as 9.2.32 |
| 9.2.38 3DL PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB over UE Measurement bandwidth Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB over UE Measurement bandwidth Noc3 ±1.0 dB averaged over BWConfig Noc3 ±1.3 dB over UE Measurement bandwidth Ês1 / Noc1, Ês2 / Noc2, Ês3 / Noc3 each ±0.3 dB averaged over BWConfig Ês1 / Noc1, Ês2 / Noc2, Ês3 / Noc3 each ±0.8 dB over UE Measurement bandwidth Time alignment errors: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | Notes: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Noc3 is the AWGN on frequency 3 Ês3 / Noc3 is the ratio of cell 3 signal / AWGN UE Measurement bandwidth: 5MHz Ch BW: PRBs 10-15 10MHz Ch BW: PRBs 22-27 20MHz Ch BW: PRBs 47-52 TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 9.2.39 3DL PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation | Same as 9.2.38 | Same as 9.2.38 |
| 9.2.40 3DL FDD RSRQ for E-UTRA Carrier Aggregation | Same as 9.2.38 | Same as 9.2.38 |
| 9.2.41 3DL TDD RSRQ for E-UTRA Carrier Aggregation | Same as 9.2.38 | Same as 9.2.38 |
| 9.2.42.1 FD-FDD Intra Frequency Absolute RSRQ Accuracy for UE category 0 | Noc ±1.0 dB averaged over BWConfig Noc ±1.3 dB for PRBs #22-27 Ês1 / Noc and Ês2 / Noc each ±0.3 dB averaged over BWConfig Ês1 / Noc and Ês2 / Noc each ±0.8 dB for PRBs #22-27 | NOTE: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 9.2.43.1 HD-FDD Intra Frequency Absolute RSRQ Accuracy for UE category 0 | Same as 9.2.42.1 | Same as 9.2.42.1 |
| 9.2.44.1 TDD Intra Frequency Absolute RSRQ Accuracy for UE category 0 | Same as 9.2.42.1 | Same as 9.2.42.1 |
| 9.2.45 4 DL CA PCell in FDD FDD-TDD RSRQ for E-UTRAN in Carrier Aggregation | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB over UE Measurement bandwidth Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB over UE Measurement bandwidth Noc3 ±1.0 dB averaged over BWConfig Noc3 ±1.3 dB over UE Measurement bandwidth Noc4 ±1.0 dB averaged over BWConfig Noc4 ±1.3 dB over UE Measurement bandwidth Ês1 / Noc1, Ês2 / Noc2, Ês3 / Noc3, Ês4 / Noc4 each ±0.3 dB averaged over BWConfig Ês1 / Noc1, Ês2 / Noc2, Ês3 / Noc3, Ês4 / Noc4 each ±0.8 dB over UE Measurement bandwidth Time alignment errors: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | Notes: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Noc3 is the AWGN on frequency 3 Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Noc4 is the AWGN on frequency 4 Ês4 / Noc4 is the ratio of cell 4 signal / AWGN UE Measurement bandwidth: 5MHz Ch BW: PRBs 10-15 10MHz Ch BW: PRBs 22-27 20MHz Ch BW: PRBs 47-52 Ts = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 9.2.46 4DL PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation | Same as 9.2.45 | Same as 9.2.45 |
| 9.2.47 5 DL PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB over UE Measurement bandwidth Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB over UE Measurement bandwidth Noc3 ±1.0 dB averaged over BWConfig Noc3 ±1.3 dB over UE Measurement bandwidth Noc4 ±1.0 dB averaged over BWConfig Noc4 ±1.3 dB over UE Measurement bandwidth Noc5 ±1.0 dB averaged over BWConfig Noc5 ±1.3 dB over UE Measurement bandwidth Ês1 / Noc1, Ês2 / Noc2, Ês3 / Noc3, Ês4 / Noc4, Ês5 / Noc5 each ±0.3 dB averaged over BWConfig Ês1 / Noc1, Ês2 / Noc2, Ês3 / Noc3, Ês4 / Noc4, Ês5 / Noc5 each ±0.8 dB over UE Measurement bandwidth Time alignment errors: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | Notes: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Noc3 is the AWGN on frequency 3 Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Noc4 is the AWGN on frequency 4 Ês4 / Noc4 is the ratio of cell 4 signal / AWGN Noc4 is the AWGN on frequency 5 Ês5 / Noc5 is the ratio of cell 5 signal / AWGN UE Measurement bandwidth: 5MHz Ch BW: PRBs 10-15 10MHz Ch BW: PRBs 22-27 20MHz Ch BW: PRBs 47-52 Ts = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 9.2.48 5 DL PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation | Same as 9.2.47 | Same as 9.2.47 |
| 9.2.51 FDD intra frequency absolute and relative RSRQ accuracies for SCell with frame structure 3 | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB averaged over UE Measurement bandwidth Noc2 ±1.5 dB averaged over BWConfig Noc2 ±1.8 dB for PRBs #47-52 Ês1 / Noc1, Ês2 / Noc2 and Ês3 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1, Ês2 / Noc2 and Ês3 / Noc2 each ±0.8 dB averaged over UE Measurement bandwidth Time alignment error cell 2 rel. to cell 1: Inter-band CA: ±260 ns (±8Ts) | NOTE: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc2 is the ratio of cell 3 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 9.2.52 TDD intra frequency absolute and relative RSRQ accuracies for SCell with frame structure 3 | Same as 9.2.51 | Same as 9.2.51 |
| 9.2.55 3DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes | Same as 9.2.38 | Same as 9.2.38 |
| 9.2.56 4 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes | Same as 9.2.45 | Same as 9.2.45 |
| 9.2.57 5 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes | Same as 9.2.47 | Same as 9.2.47 |
| 9.2.58 6 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB over UE Measurement bandwidth Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB over UE Measurement bandwidth Noc3 ±1.0 dB averaged over BWConfig Noc3 ±1.3 dB over UE Measurement bandwidth Noc4 ±1.0 dB averaged over BWConfig Noc4 ±1.3 dB over UE Measurement bandwidth Noc5 ±1.0 dB averaged over BWConfig Noc5 ±1.3 dB over UE Measurement bandwidth Noc6 ±1.0 dB averaged over BWConfig Noc6 ±1.3 dB over UE Measurement bandwidth Ês1 / Noc1, Ês2 / Noc2, Ês3 / Noc3, Ês4 / Noc4, Ês5 / Noc5, Ês6 / Noc6 each ±0.3 dB averaged over BWConfig Ês1 / Noc1, Ês2 / Noc2, Ês3 / Noc3, Ês4 / Noc4, Ês5 / Noc5ㄝÊs6 / Noc6 each ±0.8 dB over UE Measurement bandwidth Time alignment errors: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | Notes: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Noc3 is the AWGN on frequency 3 Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Noc4 is the AWGN on frequency 4 Ês4 / Noc4 is the ratio of cell 4 signal / AWGN Noc5 is the AWGN on frequency 5 Ês5 / Noc5 is the ratio of cell 5 signal / AWGN Noc6 is the AWGN on frequency 6 Ês6 / Noc6 is the ratio of cell 6 signal / AWGN UE Measurement bandwidth: 5MHz Ch BW: PRBs 10-15 10MHz Ch BW: PRBs 22-27 20MHz Ch BW: PRBs 47-52 Ts = 1/(15000 x 2048) seconds, the basic timing unit defined in TS 36.211 [9] |
| 9.2.59 7 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB over UE Measurement bandwidth Noc2 ±1.0 dB averaged over BWConfig Noc2 ±1.3 dB over UE Measurement bandwidth Noc3 ±1.0 dB averaged over BWConfig Noc3 ±1.3 dB over UE Measurement bandwidth Noc4 ±1.0 dB averaged over BWConfig Noc4 ±1.3 dB over UE Measurement bandwidth Noc5 ±1.0 dB averaged over BWConfig Noc5 ±1.3 dB over UE Measurement bandwidth Noc6 ±1.0 dB averaged over BWConfig Noc6 ±1.3 dB over UE Measurement bandwidth Noc7 ±1.0 dB averaged over BWConfig Noc7 ±1.3 dB over UE Measurement bandwidth Ês1 / Noc1, Ês2 / Noc2, Ês3 / Noc3, Ês4 / Noc4, Ês5 / Noc5, Ês6 / Noc6, Ês7 / Noc7 each ±0.3 dB averaged over BWConfig Ês1 / Noc1, Ês2 / Noc2, Ês3 / Noc3, Ês4 / Noc4, Ês5 / Noc5, Ês6 / Noc6, Ês7 / Noc7 each ±0.8 dB over UE Measurement bandwidth Time alignment errors: Intra-band contiguous CA: ±130 ns (±4Ts) Intra-band non-contiguous CA: ±260 ns (±8Ts) Inter-band CA: ±260 ns (±8Ts) | Notes: Noc1 is the AWGN on frequency 1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on frequency 2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Noc3 is the AWGN on frequency 3 Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Noc4 is the AWGN on frequency 4 Ês4 / Noc4 is the ratio of cell 4 signal / AWGN Noc5 is the AWGN on frequency 5 Ês5 / Noc5 is the ratio of cell 5 signal / AWGN Noc6 is the AWGN on frequency 6 Ês6 / Noc6 is the ratio of cell 6 signal / AWGN Noc7 is the AWGN on frequency 7 Ês7 / Noc7 is the ratio of cell 7 signal / AWGN UE Measurement bandwidth: 5MHz Ch BW: PRBs 10-15 10MHz Ch BW: PRBs 22-27 20MHz Ch BW: PRBs 47-52 Ts = 1/(15000 x 2048) seconds, the basic timing unit defined in TS 36.211 [9] |
| 9.3.1 E-UTRAN FDD - UTRA FDD CPICH RSCP absolute accuracy | E-UTRA cell Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig UTRA cell Ioc ±0.7 dB Ior / Ioc ±0.3 dB CPICH Ec / Ior ±0.1 dB | Notes: Noc is the AWGN on cell 1 (E-UTRA )frequency Ês / Noc is the ratio of cell 1 signal / AWGN Ioc is the AWGN on cell 2 (UTRA) frequency Ior / Ioc is the ratio of cell 2 signal / AWGN CPICH Ec / Ior is the fraction of cell 2 power assigned to the CPICH Physical channel |
| 9.3.2 E-UTRAN TDD - UTRA FDD CPICH RSCP absolute accuracy | Same as 9.3.1 |  |
| 9.3.3 E-UTRAN FDD - UTRA FDD CPICH RSCP absolute accuracy for 5MHz bandwidth | Same as 9.3.1 |  |
| 9.4.1 E-UTRAN FDD - UTRA FDD CPICH Ec/No absolute accuracy | E-UTRA cell Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig UTRA cell Ioc ±0.7 dB Ior / Ioc ±0.3 dB CPICH Ec / Ior ±0.1 dB | Notes: Noc is the AWGN on cell 1 (E-UTRA )frequency Ês / Noc is the ratio of cell 1 signal / AWGN Ioc is the AWGN on cell 2 (UTRA) frequency Ior / Ioc is the ratio of cell 2 signal / AWGN CPICH Ec / Ior is the fraction of cell 2 power assigned to the CPICH Physical channel |
| 9.4.2 E-UTRAN TDD - UTRA FDD CPICH Ec/No absolute accuracy | Same as 9.4.1 | Same as 9.4.1 |
| 9.4.3 E-UTRAN FDD - UTRA FDD CPICH Ec/No absolute accuracy for 5MHz bandwidth | Same as 9.4.1 | Same as 9.4.1 |
| 9.5.1 E-UTRAN FDD - UTRA TDD P-CCPCH RSCP absolute accuracy | E-UTRA cell Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig UTRA cell Ioc ±0.7 dB Ior / Ioc ±0.3 dB PCCPCH EC/Ior ±0.1 dB DwPCH_EC/Ior ±0.1 dB | Notes: Noc is the AWGN on cell 1 (E-UTRA )frequency Ês / Noc is the ratio of cell 1 signal / AWGN Ioc is the AWGN on cell 2 (UTRA) frequency Ior / Ioc is the ratio of cell 2 signal / AWGN PCCPCH Ec / Ior is the fraction of cell 2 power assigned to the PCCPCH Physical channel DwPCH_Ec / Ior is the fraction of cell 2 power assigned to the DwPCH channel |
| 9.5.2 E-UTRAN TDD - UTRA TDD P-CCPCH RSCP absolute accuracy | Same as 9.5.1 |  |
| 9.6.1 GSM RSSI accuracy for E-UTRAN FDD | E-UTRA Cell Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig GSM cell BCCH1 Signal level ±0.7 dB GSM cell BCCH 2 to 6 Signal level ±2.0 dB | NOTE: Noc is the AWGN on cell 1 frequency Ês / Noc is the ratio of cell 1 signal / AWGN GSM cells BCCH 1 to 6 have only the wanted signal, without AWGN |
| 9.6.2 GSM RSSI accuracy for E-UTRAN TDD | E-UTRA Cell Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig GSM cell BCCH1 Signal level ±0.7 dB GSM cell BCCH 2 to 6 Signal level ±2.0 dB | NOTE: Noc is the AWGN on cell 1 frequency Ês / Noc is the ratio of cell 1 signal / AWGN GSM cells BCCH 1 to 6 have only the wanted signal, without AWGN |
| 9.9.1.1 FDD Intra Frequency Serving Cell Absolute RSRP Accuracy | Noc ±1.0 dB averaged over BWConfig Noc ±1.3 dB for PRBs #22-27 Ês1 / Noc ±0.3 dB averaged over BWConfig Ês1 / Noc ±0.8 dB for PRBs #22-27 | NOTE: Ês1 / Noc is the ratio of cell 1 signal / AWGN |
| 9.9.1.1_1 FDD Intra Frequency Serving Cell Absolute RSRP Accuracy (Rel 12 and forward) | Same as 9.9.1.1 | Same as 9.9.1.1 |
| 9.9.1.2 FDD Intra Frequency Serving Cell Absolute RSRQ Accuracy | Noc ±1.0 dB averaged over BWConfig Noc ±1.3 dB for PRBs #22-27 Ês1 / Noc ±0.3 dB averaged over BWConfig Ês1 / Noc ±0.8 dB for PRBs #22-27 | NOTE: Ês1 / Noc is the ratio of cell 1 signal / AWGN |
| 9.9.2.1 TDD Intra Frequency Serving Cell Absolute RSRP Accuracy | Noc ±1.0 dB averaged over BWConfig Noc ±1.3 dB for PRBs #22-27 Ês1 / Noc ±0.3 dB averaged over BWConfig Ês1 / Noc ±0.8 dB for PRBs #22-27 | NOTE: Ês1 / Noc is the ratio of cell 1 signal / AWGN |
| 9.9.2.1_1 TDD Intra Frequency Serving Cell Absolute RSRP Accuracy (Rel 12 and forward) | Same as 9.9.2.1 | Same as 9.9.2.1 |
| 9.9.2.2 TDD Intra Frequency Serving Cell Absolute RSRQ Accuracy | Noc ±1.0 dB averaged over BWConfig Noc ±1.3 dB for PRBs #22-27 Ês1 / Noc ±0.3 dB averaged over BWConfig Ês1 / Noc ±0.8 dB for PRBs #22-27 | NOTE: Ês1 / Noc is the ratio of cell 1 signal / AWGN |
| 9.11.1 FS3 average RSSI accuracy case (PCell using FDD) | Noc1 ±1.0 dB averaged over BWConfig Noc1 ±1.3 dB for measurement PRBs Noc2 ±1.5 dB averaged over BWConfig Noc2 ±1.8 dB for measurement PRBs Ês1 / Noc1 and Ês2 / Noc2 each ±0.3 dB averaged over BWConfig Ês1 / Noc1 and Ês2 / Noc2 each ±0.8 dB for measurement PRBs | NOTE: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 9.11.2 FS3 average RSSI accuracy case (PCell using TDD) | Same as 9.11.1 | Same as 9.11.1 |
| 9.12.1 FS3 channel occupancy test (PCell using FDD) | Same as 9.11.1 | Same as 9.11.1 |
| 9.12.2 FS3 channel occupancy test (PCell using TDD) | Same as 9.11.1 | Same as 9.11.1 |
| 10.2 E-UTRAN FDD - Initiation/Cease of SLSS Transmission with ProSe Direct Communication | ± 0.6dB | Overall system uncertainty for AWGN condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |
| 10.4 E-UTRAN FDD - Initiation/Cease of SLSS Transmission with ProSe Direct Communication | ± 0.6dB | Overall system uncertainty for AWGN condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |
| 11.1 V2V UE Transmission Timing Accuracy Test | ±3Ts Uplink signal transmit timing relative to GNSS synchronization source Absolute GNSS signal level, ±1 dB | NOTE: TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] Same value as the A-GNSS Time Assistance tests in clause 5 of TS 37.571-1 |
| 11.2 Interruptions due to V2V sidelink communication | Noc ±3.0 dB averaged over BWConfig Ês / Noc ±0.3 dB Absolute GNSS signal level, ±1 dB | NOTE: Ês / Noc is the ratio of cell 1 signal / AWGN Same value as the A-GNSS Time Assistance tests in clause 5 of TS 37.571-1 |
| 12.1.1 V2X UE Transmission Timing Accuracy Test for eNB as Timing Reference | synchronization source Noc ±3.0 dB averaged over BWConfig Ês / Noc ±0.3 dB ±3Ts Uplink signal transmit timing relative to GNSS | NOTE: Ês / Noc is the ratio of cell 1 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 12.1.2 V2X UE Transmission Timing Accuracy Test for SyncRef UE as Timing Reference | Noc ±3.0 dB averaged over BWConfig Ês / Noc ±0.3 dB ±3Ts Uplink signal transmit timing relative to downlink | NOTE: Ês / Noc is the ratio of cell 1 signal / AWGN TS = 1/(15000 x 2048) seconds, the basic timing unit defined in 3GPP TS 36.211 [9] |
| 12.2.1 Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for eNB as Timing Reference | Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig | NOTE: Ês / Noc is the ratio of cell 1 signal / AWGN |
| 12.2.2 Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for SyncRef UE as Timing Reference | Noc ±1.5 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig | NOTE: Ês / Noc is the ratio of SyncRef UE signal / AWGN |
| 12.3.1 V2X Synchronization Reference Selection/Reselection Tests for GNSS configured as the highest priority | Noc2 ±1.0 dB averaged over BWConfig SCH Ês1 / Noc ±0.3 dB averaged over BWConfig SCH Ês2 / Noc ±0.3 dB averaged over BWConfig | NOTE: Ês1 / Noc is the ratio of SyncRef UE 1 signal / AWGN Ês2 / Noc is the ratio of SyncRef UE 2 signal / AWGN |
| 12.3.2 V2X Synchronization Reference Selection/Reselection Tests for eNB configured as the highest priority | Noc2 ±1.0 dB averaged over BWConfig SCH Ês1 / Noc ±0.3 dB averaged over BWConfig SCH Ês2 / Noc ±0.3 dB averaged over BWConfig | NOTE: Ês1 / Noc is the ratio of SyncRef UE 1 signal / AWGN Ês2 / Noc is the ratio of SyncRef UE 2 signal / AWGN |
| 12.4 Congestion Control Measurement Test for V2X UE | Noc ±1.5 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig | NOTE: Ês / Noc is the ratio of Active UE signal / AWGN |
| 12.5 Interruptions due to V2X Sidelink Communication | Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig | NOTE: Ês / Noc is the ratio of Active Sidelink UE signal / AWGN |
| 12.6.1 V2X UE Autonomous Resource Selection/Reselection Tests for PSSCH-RSRP measurements | Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig | NOTE: Ês / Noc is the ratio of Active Sidelink UE signal / AWGN |
| 12.6.2 V2X UE Autonomous Resource Selection/Reselection Tests for S-RSSI measurements | Noc ±1.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig | NOTE: Ês / Noc is the ratio of Active Sidelink UE signal / AWGN |
| 13.1.1.1 HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage | Noc1 ±1.0 dB averaged over BWConfig Êsn1 / Noc1 ±0.3 dB averaged over BWConfig Êsn2 / Noc1 ±0.3 dB averaged over BWConfig | NOTE: Noc1 is the AWGN on frequency 1 Êsn1 / Noc1 is the ratio of nCell1 signal / AWGN Êsn2 / Noc1 is the ratio of nCell2 signal / AWGN |
| 13.1.1.1_1 HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage for GSO and NGSO | Same as 13.1.1.1 | Same as 13.1.1.1 |
| 13.1.1.2 HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage with serving cell RRM measurement relaxation | Same as 13.1.1.1 | Same as 13.1.1.1 |
| 13.1.1.2_1 HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage with serving cell RRM measurement relaxation for GSO and NGSO | Same as 13.1.1.1 | Same as 13.1.1.1 |
| 13.1.1.3 HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage with UE specific DRX | Same as 13.1.1.1 | Same as 13.1.1.1 |
| 13.1.1.3_1 FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage with UE specific DRX for GSO and NGSO | Same as 13.1.1.1 | Same as 13.1.1.1 |
| 13.1.1.4 HD – FDD Inter frequency case for UE Category NB1 Standalone mode in normal coverage | Same as 13.1.1.1 | Same as 13.1.1.1 |
| 13.1.1.6 FDD Inter frequency case for UE Category NB1 Standalone mode in enhanced coverage, time-based cell reselection for NGSO | Same as 13.1.1.1 | Same as 13.1.1.1 |
| 13.3.1.1 HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in Standalone mode under normal coverage | Noc ±1.0 dB averaged over BWConfig Êsn1 / Noc ±0.3 dB averaged over BWConfig Êsn2 / Noc ±0.3 dB averaged over BWConfig | NOTE: Êsn1 / Noc is the ratio of nCell 1 signal / AWGN Êsn2 / Noc is the ratio of nCell 2 signal / AWGN |
| 13.3.1.1_1 HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in Standalone mode under normal coverage for GSO and NGSO | Same as 13.3.1.1 | Same as 13.3.1.1 |
| 13.3.1.2 HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in Standalone mode under enhanced coverage | Same as 13.3.1.1 | Same as 13.3.1.1 |
| 13.3.1.2_1 HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in Standalone mode under enhanced coverage for GSO and NGSO | Same as 13.3.1.1 | Same as 13.3.1.1 |
| 13.3.2.1 Contention Based Random Access Test for UE category NB1 UEs Standalone mode in normal coverage | Same as 6.2.16 | Same as 6.2.16 |
| 13.3.2.2 Contention Based Random Access Test for UE category NB1 UEs Standalone mode in Enhanced Coverage | Same as 6.2.16 | Same as 6.2.16 |
| 13.3.2.3 Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs Standalone mode in Enhanced Coverage | Same as 6.2.16 | Same as 6.2.16 |
| 13.4.1.1 E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under normal coverage for Satellite Access | Noc ±3.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig ±3Ts Uplink signal transmit timing relative to downlink ±0.5Ts Relative UE timing adjustment | NOTE: Ês / Noc is the ratio of active cell signal / AWGN The timing unit TS = 1/(15000 * 2048) seconds, as defined in 3GPP TS 36.211 [9] |
| 13.4.1.2 E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under enhanced coverage for Satellite Access | Noc ±3.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig ±3Ts Uplink signal transmit timing relative to downlink | Same as 13.4.1.1 |
| 13.4.1.3 E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under enhanced coverage with segment transmission in NGSO for Satellite Access | Same as 13.4.1.2 | Same as 13.4.1.1 |
| 13.4.2.1 HD-FDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Normal Coverage for Satellite Access | Noc ±3.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig Timing Advance Adjustment: ±0.5Ts | NOTE: Ês / Noc is the ratio of active cell signal / AWGN The timing unit TS = 1/(15000 * 2048) seconds, as defined in 3GPP TS 36.211 [9] |
| 13.4.2.2 HD-FDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Enhance Coverage for Satellite Access | Same as 13.4.2.1 | Same as 13.4.2.1 |
| 13.4.3.1 HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 Standalone mode in normal coverage | Noc ±3.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig | NOTE: Ês / Noc is the ratio of active cell signal / AWGN |
| 13.4.3.2 HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 Standalone mode in enhanced coverage | Same as 13.4.3.1 | Same as 13.4.3.1 |
| 13.4.3.3 HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 Standalone mode in Enhanced Coverage | Noc ±3.0 dB averaged over BWConfig Ês / Noc ±0.3 dB averaged over BWConfig | NOTE: Ês / Noc is the ratio of active cell signal / AWGN |
| 13.4.3.4 HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 Standalone mode in Normal Coverage | Same as 13.4.3.3 | Same as 13.4.3.3 |
| 13.4.3.5 HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage | Same as 13.4.3.3 | Same as 13.4.3.3 |
| 13.4.3.6 HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 Standalone mode in Enhanced Coverage | Same as 13.4.3.3 | Same as 13.4.3.3 |
| 13.4.3.7 HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage | Same as 13.4.3.1 | Same as 13.4.3.1 |
| 13.4.3.8 HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Enhanced Coverage | Same as 13.4.3.1 | Same as 13.4.3.1 |
| 13.6.2.1 HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under normal coverage | ± 0.6dB | Overall system uncertainty for AWGN condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |
| 13.6.2.2 HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under enhanced coverage | Same as 13.6.2.1 | Same as 13.6.2.1 |
| 13.6.2.3 HD-FDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under normal coverage | Same as 13.6.2.1 | Same as 13.6.2.1 |
| 13.6.2.4 HD-FDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under enhanced coverage | Same as 13.6.2.1 | Same as 13.6.2.1 |
| 13.6.2.5 HD-FDD Downlink channel quality reporting accuracy in RRC_CONNECTED for UE Category NB1 Standalone mode under normal coverage | Same as 13.6.2.1 | Same as 13.6.2.1 |
| 13.6.2.6 HD-FDD Downlink channel quality reporting accuracy in RRC_CONNECTED for UE Category NB1 Standalone mode under enhanced coverage | Same as 13.6.2.1 | Same as 13.6.2.1 |
| In addition, the following Test System uncertainties and related constraints apply. Any additional constraints are defined in the specific tests. |  |  |
| AWGN Bandwidth |  | ≥ 1.08MHz, 2.7MHz, 4.5MHz, 9MHz, 13.5MHz, 18MHz; NRB x 180kHz according to BWConfig |
| AWGN absolute power uncertainty |  | Test-specific |
| AWGN flatness and signal flatness, max deviation for any Resource Block, relative to average over BWConfig |  | ±2 dB |
| AWGN peak to average ratio |  | ≥10 dB @0.001% |
| Signal-to noise ratio uncertainty |  | Test-specific |
| Fading profile power uncertainty |  | ±0.5 dB |
| Fading profile delay uncertainty, relative to frame timing |  | ±5 ns (excludes absolute errors related to baseband timing) |

# F.2	Interpretation of measurement results (normative)

See 3GPP TS 36.521-1 [10] Annex F2.

# F.3	Test Tolerance and Derivation of Test Requirements (informative)

## F.3.0	General

See 3GPP TS 36.521-1 [10] Annex F3.

## F.3.1	Measurement of test environments

See 3GPP TS 36.521-1 [10] Annex F3.1.

## F.3.2	Measurement of RRM requirements

Because the relationships between the Test system uncertainties and the Test Tolerances are often complex, it is not always possible to give a simple derivation of the Test Requirement in this document. The analysis is recorded in 3GPP TR 36 903 [20].

Table F.3.2-1: Derivation of Test Requirements (RRM tests)

| Test |  |  | Minimum Requirement in 3GPP TS 36.133 |  |  | Test Tolerance (TT) |  |  |  |  |  |  | Test Requirement in 3GPP TS 36.521-3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4.2.1 E-UTRA FDD - FDD cell re-selection intra frequency |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +16.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +13.00dB Ês2 / Noc: +16.00dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: +16.00dB Ês2 / Noc: +13.00dB |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB +0.45dB During T3: 0dB +0.45dB 0dB |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +16.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +13.00dB Ês2 / Noc: +16.45dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: +16.45dB Ês2 / Noc: +13.00dB |  |
| 4.2.2 E-UTRA TDD - TDD cell re-selection intra frequency |  |  | Same as 4.2.1 |  |  | Same as 4.2.1 |  |  |  |  |  |  | Same as 4.2.1 |  |
| 4.2.3 E-UTRA FDD - FDD cell re-selection inter frequency |  |  | During T0: Noc1: -98dBm/15kHz Ês1 / Noc1: -4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +14.00dB During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +14.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: -4.00dB During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +14.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T3: Noc1: -98dBm /15kHz Ês1 / Noc1: +14.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +12.00dB |  |  | During T0: -1.35dB +0.3dB -1.35dB +0.4dB During T1: -1.35dB +2.4dB -1.35dB +0.3dB During T2: -1.35dB +2.4dB -1.35dB 0dB During T3: -1.35dB +2.4dB -1.35dB +2.4dB |  |  |  |  |  |  | During T0: Noc1: -99.35dBm/15kHz Ês1 / Noc1: -3.70dB Noc2: -99.35dBm/15kHz Ês2 / Noc2: +14.40dB During T1: Noc1: -99.35dBm/15kHz Ês1 / Noc1: +16.40dB Noc2: -99.35dBm/15kHz Ês2 / Noc2: -3.70dB During T2: Noc1: -99.35dBm/15kHz Ês1 / Noc1: +16.40dB Noc2: -99.35dBm/15kHz Ês2 / Noc2: -infinity During T3: Noc1: -99.35dBm /15kHz Ês1 / Noc1: +16.40dB Noc2: -99.35dBm/15kHz Ês2 / Noc2: +14.40dB |  |
| 4.2.4 E-UTRA FDD - TDD cell re-selection inter frequency |  |  | Same as 4.2.3 |  |  | Same as 4.2.3 |  |  |  |  |  |  | Same as 4.2.3 |  |
| 4.2.5 E-UTRA TDD - FDD cell re-selection inter frequency |  |  | Same as 4.2.3 |  |  | Same as 4.2.3 |  |  |  |  |  |  | Same as 4.2.3 |  |
| 4.2.6 E-UTRA TDD - TDD cell re-selection inter frequency |  |  | Same as 4.2.3 |  |  | Same as 4.2.3 |  |  |  |  |  |  | Same as 4.2.3 |  |
| 4.2.7 E-UTRA FDD Inter frequency re-selection in the existence of non-allowed CSG cell |  |  | During T0: Noc1: -98dBm/15kHz Ês1 / Noc1: +13.00dB Ês3 / Noc1: -infinity Noc2: -98dBm/15kHz Ês2 / Noc2: -3.00dB During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +8.00dB Ês3 / Noc1: +8.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +8.00dB Ês3 / Noc1: +13.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +13.00dB During T3: Noc1: -98dBm/15kHz Ês1 / Noc1: +13.00dB Ês3 / Noc1: +38.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +8.00dB |  |  | During T0: 0dB 0dB 0dB 0dB 0dB During T1: 0dB -0.2dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB 0dB |  |  |  |  |  |  | During T0: Noc1: -98dBm/15kHz Ês1 / Noc1: +13.00dB Ês3 / Noc1: -infinitydB Noc2: -98dBm/15kHz Ês2 / Noc2: -3.00dB During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +7.80dB Ês3 / Noc1: +8.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +8.00dB Ês3 / Noc1: +13.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +13.00dB During T3: Noc1: -98dBm/15kHz Ês1 / Noc1: +13.00dB Ês3 / Noc1: +38.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +8.00dB |  |
| 4.2.8 E-UTRA TDD Inter frequency re-selection in the existence of non-allowed CSG cell |  |  | Same as 4.2.7 |  |  | Same as 4.2.7 |  |  |  |  |  |  | Same as 4.2.7 |  |
| 4.2.9 E-UTRAN FDD - FDD intra frequency cell re-selection case for 5MHz bandwidth |  |  | Same as 4.2.1 |  |  | Same as 4.2.1 |  |  |  |  |  |  | Same as 4.2.1 |  |
| 4.2.10 E-UTRAN FDD - FDD reselection using an increased number of carriers |  |  | During T0: Noc1: -98dBm/15kHz Ês1 / Noc1: +14.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +8.00dB Noc3: -98dBm/15kHz Ês3 / Noc3: +8.00dB Noc4: -98dBm/15kHz Ês4 / Noc4: +8.00dB During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +8.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +14.00dB Noc3: -98dBm/15kHz Ês3 / Noc3: +8.00dB Noc4: -98dBm/15kHz Ês4 / Noc4: +8.00dB During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +8.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +8.00dB Noc3: -98dBm/15kHz Ês3 / Noc3: +14.00dB Noc4: -98dBm/15kHz Ês4 / Noc4: +8.00dB During T3: Noc1: -98dBm/15kHz Ês1 / Noc1: +8.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +8.00dB Noc3: -98dBm/15kHz Ês3 / Noc3: +8.00dB Noc4: -98dBm/15kHz Ês4 / Noc4: +14.00dB During T4: Noc1: -98dBm/15kHz Ês1 / Noc1: +14.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +8.00dB Noc3: -98dBm/15kHz Ês3 / Noc3: +8.00dB Noc4: -98dBm/15kHz Ês4 / Noc4: +8.00dB |  |  | During T0: -1.15dB +1.10dB -1.15dB 0dB -1.15dB 0dB -1.15dB 0dB During T1: -1.15dB 0dB -1.15dB +1.10dB -1.15dB 0dB -1.15dB 0dB During T2: -1.15dB 0dB -1.15dB 0dB -1.15dB +1.10dB -1.15dB 0dB During T3: -1.15dB 0dB -1.15dB 0dB -1.15dB 0dB -1.15dB +1.10dB During T4: -1.15dB +1.10dB -1.15dB 0dB -1.15dB 0dB -1.15dB 0dB |  |  |  |  |  |  | During T0: Noc1: -99.15dBm/15kHz Ês1 / Noc1: +15.10dB Noc2: -99.15dBm/15kHz Ês2 / Noc2: +8.00dB Noc3: -99.15dBm/15kHz Ês3 / Noc3: +8.00dB Noc4: -99.15dBm/15kHz Ês4 / Noc4: +8.00dB During T1: Noc1: -99.15dBm/15kHz Ês1 / Noc1: +8.00dB Noc2: -99.15dBm/15kHz Ês2 / Noc2: +15.10dB Noc3: -99.15dBm/15kHz Ês3 / Noc3: +8.00dB Noc4: -99.15dBm/15kHz Ês4 / Noc4: +8.00dB During T2: Noc1: -99.15dBm/15kHz Ês1 / Noc1: +8.00dB Noc2: -99.15dBm/15kHz Ês2 / Noc2: +8.00dB Noc3: -99.15dBm/15kHz Ês3 / Noc3: +15.10dB Noc4: -99.15dBm/15kHz Ês4 / Noc4: +8.00dB During T3: Noc1: -99.15dBm/15kHz Ês1 / Noc1: +8.00dB Noc2: -99.15dBm/15kHz Ês2 / Noc2: +8.00dB Noc3: -99.15dBm/15kHz Ês3 / Noc3: +8.00dB Noc4: -99.15dBm/15kHz Ês4 / Noc4: +15.10dB During T4: Noc1: -99.15dBm/15kHz Ês1 / Noc1: +15.10dB Noc2: -99.15dBm/15kHz Ês2 / Noc2: +8.00dB Noc3: -99.15dBm/15kHz Ês3 / Noc3: +8.00dB Noc4: -99.15dBm/15kHz Ês4 / Noc4: +8.00dB |  |
| 4.2.11 E-UTRAN TDD - TDD reselection using an increased number of carriers |  |  | Same as 4.2.10 |  |  | Same as 4.2.10 |  |  |  |  |  |  | Same as 4.2.10 |  |
| 4.2.12 E-UTRAN FDD - FDD Intra frequency case for Cat-M1 UE in normal coverage |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +16.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +12.00dB Ês2 / Noc: +16.00dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: +16.00dB Ês2 / Noc: +12.00dB |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB +0.45dB During T3: 0dB +0.45dB 0dB |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +16.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +12.00dB Ês2 / Noc: +16.45dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: +16.45dB Ês2 / Noc: +12.00dB |  |
| 4.2.13 E-UTRAN HD - FDD Intra frequency case for Cat-M1 UE in normal coverage |  |  | Same as 4.2.12 |  |  | Same as 4.2.12 |  |  |  |  |  |  | Same as 4.2.12 |  |
| 4.2.14 E-UTRA TDD - TDD intra frequency case for Cat-M1 UE in normal coverage |  |  | Same as 4.2.12 |  |  | Same as 4.2.12 |  |  |  |  |  |  | Same as 4.2.12 |  |
| 4.2.15 E-UTRAN FDD - FDD Intra frequency case for Cat-M1 UE in enhanced coverage |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: -7.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: -12.00dB Ês2 / Noc: -7.00dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: -7.00dB Ês2 / Noc: -12.00dB |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB +0.45dB During T3: 0dB +0.45dB 0dB |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: -7.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: -12.00dB Ês2 / Noc: -6.55dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: -6.55dB Ês2 / Noc: -12.00dB |  |
| 4.2.16 E-UTRAN HD - FDD Intra frequency case for Cat-M1 UE in enhanced coverage |  |  | Same as 4.2.15 |  |  | Same as 4.2.15 |  |  |  |  |  |  | Same as 4.2.15 |  |
| 4.2.17 E-UTRAN TDD - TDD Intra frequency case for Cat-M1 UE in enhanced coverage |  |  | Same as 4.2.15 |  |  | Same as 4.2.15 |  |  |  |  |  |  | Same as 4.2.15 |  |
| 4.2.18 HD-FDD Intra frequency cell reselection for Category NB1 UE in In-Band Mode under Normal Coverage |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: 3.00dB Êsn1 / Noc: 17.00dB Êsn2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: 3.00dB Êsn1 / Noc: 13.00dB Êsn2 / Noc: 17.00dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: 3.00dB Êsn1 / Noc: 17.00dB Êsn2 / Noc: 13.00dB |  |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB +0.45dB During T3: 0dB 0dB +0.45dB 0dB |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: 3.00dB Êsn1 / Noc: 17.00dB Êsn2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: 3.00dB Êsn1 / Noc: 13.00dB Êsn2 / Noc: 17.45dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: 3.00dB Êsn1 / Noc: 17.45dB Êsn2 / Noc: 13.00dB |  |
| 4.2.19 HD - FDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: 3.00dB Ês2 / Noc: 3.00dB Êsn1 / Noc: -9.00dB Êsn2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: 3.00dB Ês2 / Noc: 3.00dB Êsn1 / Noc: -9.00dB Êsn2 / Noc: -0.70dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: 3.00dB Ês2 / Noc: 3.00dB Êsn1 / Noc: -0.70dB Êsn2 / Noc: -9.00dB |  |  | During T1: 0dB 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB 0.45dB During T3: 0dB 0dB 0dB 0.45dB 0dB |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: 3.00dB Ês2 / Noc: 3.00dB Êsn1 / Noc: -9.00dB Êsn2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: 3.00dB Ês2 / Noc: 3.00dB Êsn1 / Noc: -9.00dB Êsn2 / Noc: -0.25dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: 3.00dB Ês2 / Noc: 3.00dB Êsn1 / Noc: -0.25dB Êsn2 / Noc: -9.00dB |  |
| 4.2.20 E-UTRAN FDD - FDD Intra frequency case for UE Category 1bis |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +17.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +13.00dB Ês2 / Noc: +17.00dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: +17.00dB Ês2 / Noc: +13.00dB |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB +0.45dB During T3: 0dB +0.45dB 0dB |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +17.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +13.00dB Ês2 / Noc: +17.45dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: +17.45dB Ês2 / Noc: +13.00dB |  |
| 4.2.21 E-UTRAN TDD - TDD Intra frequency case for UE Category 1bis |  |  | Same as 4.2.21 |  |  | Same as 4.2.21 |  |  |  |  |  |  | Same as 4.2.21 |  |
| 4.2.22 E-UTRAN FDD - FDD cell re-selection intra frequency case for UE configured with highSpeedEnhancedMeasFlag |  |  | Same as 4.2.1 |  |  | Same as 4.2.1 |  |  |  |  |  |  | Same as 4.2.1 |  |
| 4.2.23 E-UTRAN TDD - TDD cell re-selection intra frequency case for UE configured with highSpeedEnhancedMeasFlag |  |  | Same as 4.2.1 |  |  | Same as 4.2.1 |  |  |  |  |  |  | Same as 4.2.1 |  |
| 4.2.24 HD - FDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: 3.00dB Êsn1 / Noc: -9.00dB Êsn2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: 3.00dB Êsn1 / Noc: -9.00dB Êsn2 / Noc: -0.70dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: 3.00dB Êsn1 / Noc: -0.70dB Êsn2 / Noc: -9.00dB |  |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0.45dB During T3: 0dB 0dB 0.45dB 0dB |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: 3.00dB Êsn1 / Noc: -9.00dB Êsn2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: 3.00dB Êsn1 / Noc: -9.00dB Êsn2 / Noc: -0.25dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: 3.00dB Êsn1 / Noc: -0.25dB Êsn2 / Noc: -9.00dB |  |
| 4.2.28 E-UTRAN FDD - FDD Inter frequency case for CE UE in enhanced coverage |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: -5.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: -12.00dB Ês2 / Noc: -5.00dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: -5.00dB Ês2 / Noc: -12.00dB |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0.5dB During T3: 0dB 0.5dB 0dB |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: -5.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: -12.00dB Ês2 / Noc: -4.50dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: -4.50dB Ês2 / Noc: -12.00dB |  |
| 4.2.29 E-UTRAN HD - FDD Inter frequency case for CE UE in enhanced coverage |  |  | Same as 4.2.28 |  |  | Same as 4.2.28 |  |  |  |  |  |  | Same as 4.2.28 |  |
| 4.2.30 E-UTRAN TDD Inter frequency case for CE UE in enhanced coverage |  |  | Same as 4.2.28 |  |  | Same as 4.2.28 |  |  |  |  |  |  | Same as 4.2.28 |  |
| 4.2.35 E-UTRAN TDD - TDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage |  |  | Same as 4.2.18 |  |  | Same as 4.2.18 |  |  |  |  |  |  | Same as 4.2.18 |  |
| 4.2.36 E-UTRAN TDD - TDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage |  |  | Same as 4.2.19 |  |  | Same as 4.2.19 |  |  |  |  |  |  | Same as 4.2.19 |  |
| 4.2.37 E-UTRAN TDD - TDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage |  |  | Same as 4.2.24 |  |  | Same as 4.2.24 |  |  |  |  |  |  | Same as 4.2.24 |  |
| 4.2.38 HD - FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with serving cell RRM measurement relaxation |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: 3.00dB Êsn1 / Noc: 23.00dB Êsn2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: 3.00dB Êsn1 / Noc: 13.00dB Êsn2 / Noc: 17.00dB |  |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB +0.45dB |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: 3.00dB Êsn1 / Noc: 23.00dB Êsn2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: 3.00dB Êsn1 / Noc: 13.00dB Êsn2 / Noc: 17.45dB |  |
| 4.3.1.1 E-UTRA FDD - UTRAN FDD cell reselection: UTRA FDD is of higher priority |  |  | During T1: E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: +14.00dB UTRA Cell 2 Ioc: -70.00dBm/3.84MHz Ior / Ioc: -∞dB CPICH_Ec/Ior: -10.00dB During T2: E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: +14.00dB UTRA Cell 2 Ioc: -70.00dBm/3.84MHz Ior / Ioc: +11.00dB CPICH_Ec/Ior: -10.00dB During T3: E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: +14.00dB UTRA Cell 2 Ioc: -70.00dBm/3.84MHz Ior / Ioc: -5.00dB CPICH_Ec/Ior: -10.00dB |  |  | During T1: 0dB +1.05dB -0.1dB 0dB 0dB During T2: 0dB +1.05dB -0.1dB +0.9dB 0dB During T3: 0dB +1.05dB -0.1dB -0.7dB 0dB |  |  |  |  |  |  | During T1: E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: +1.05dB UTRA Cell 2 Ioc: -70.10dBm/3.84MHz Ior / Ioc: -∞dB CPICH_Ec/Ior: -10.00dB During T2: E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: +1.05dB UTRA Cell 2 Ioc: -70.10dBm/3.84MHz Ior / Ioc: +11.90dB CPICH_Ec/Ior: -10.00dB During T3: E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: +1.05dB UTRA Cell 2 Ioc: -70.10dBm/3.84MHz Ior / Ioc: -5.70dB CPICH_Ec/Ior: -10.00dB |  |
| 4.3.1.2 E-UTRAN FDD - UTRAN FDD cell re-selection: UTRA FDD is of lower priority |  |  | During T1: E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: +12.00dB UTRA Cell 2 Ioc: -70.00dBm/3.84MHz Ior / Ioc: +13.00dB CPICH_Ec/Ior: -10.00dB During T2: E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: -4.00dB UTRA Cell 2 Ioc: -70.00dBm/3.84MHz Ior / Ioc: +13.00dB CPICH_Ec/Ior: -10.00dB |  |  | During T1: -1.35dB +2.40dB 0dB +0.80dB 0dB During T2: -1.35dB +0.30dB 0dB +0.80dB 0dB |  |  |  |  |  |  | During T1: E-UTRA Cell 1 Noc: -99.35dBm/15kHz Ês / Noc: +14.40dB UTRA Cell 2 Ioc: -70.00dBm/3.84MHz Ior / Ioc: +13.80dB CPICH_Ec/Ior: -10.00dB During T2: E-UTRA Cell 1 Noc: -99.35dBm/15kHz Ês / Noc: -3.70dB UTRA Cell 2 Ioc: -70.00dBm/3.84MHz Ior / Ioc: +13.80dB CPICH_Ec/Ior: -10.00dB |  |
| 4.3.1.3 EUTRA FDD-UTRA FDD cell reselection in fading propagation conditions: UTRA FDD is of lower priority |  |  | During T1, T2: E-UTRA Cell 1 Noc: -104.00dBm/15kHz Ês / Noc: +22.00dB UTRA Cell 2 Ioc: -70.00dBm/3.84MHz Ior / Ioc: +13.00dB CPICH_Ec/Ior: -10.00dB During T3, T4: E-UTRA Cell 1 Noc: -104.00dBm/15kHz Ês / Noc: -3.00dB UTRA Cell 2 Ioc: -70.00dBm/3.84MHz Ior / Ioc: +13.00dB CPICH_Ec/Ior: -10.00dB |  |  | During T1,T2 0dB 0dB 0dB +0.80dB 0dB During T3,T4 0dB 0dB 0dB +0.80dB 0dB |  |  |  |  |  |  | During T1, T2: E-UTRA Cell 1 Noc: -104.00dBm/15kHz Ês / Noc: +22.00dB UTRA Cell 2 Ioc: -70.00dBm/3.84MHz Ior / Ioc: +13.80dB CPICH_Ec/Ior: -10.00dB During T3, T4: E-UTRA Cell 1 Noc: -104.00dBm/15kHz Ês / Noc: -3.00dB UTRA Cell 2 Ioc: -70.00dBm/3.84MHz Ior / Ioc: +13.80dB CPICH_Ec/Ior: -10.00dB |  |
| 4.3.1.4 E-UTRAN FDD - UTRAN FDD cell re-selection: UTRA FDD is of lower priority for 5MHz bandwidth |  |  | Same as 4.3.1.2 |  |  |  |  |  |  |  |  |  |  |  |
| 4.3.1.5 Idle mode FDD to UTRA FDD interRAT reselection |  |  | During T0: E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: +14.00dB UTRA Cell 2 Ioc: -70.00dBm/3.84MHz Ior / Ioc: +11.00dB CPICH_Ec/Ior: -10.00dB UTRA Cell 3 Ioc: -70.00dBm/3.84MHz Ior / Ioc: +11.00dB CPICH_Ec/Ior: -10.00dB During T1: E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: -4.00dB UTRA Cell 2 Ioc: -70.00dBm/3.84MHz Ior / Ioc: +11.00dB CPICH_Ec/Ior: -10.00dB UTRA Cell 3 Ioc: -70.00dBm/3.84MHz Ior / Ioc: -5.00dB CPICH_Ec/Ior: -10.00dB During T2 E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: -4.00dB UTRA Cell 2 Ioc: -70.00dBm/3.84MHz Ior / Ioc: +11.00dB CPICH_Ec/Ior: -10.00dB UTRA Cell 3 Ioc: -70.00dBm/3.84MHz Ior / Ioc: +11.00dB CPICH_Ec/Ior: -10.00dB During T3 E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: -4.00dB UTRA Cell 2 Ioc: -70.00dBm/3.84MHz Ior / Ioc: -5.00dB CPICH_Ec/Ior: -10.00dB UTRA Cell 3 Ioc: -70.00dBm/3.84MHz Ior / Ioc: +11.00dB CPICH_Ec/Ior: -10.00dB During T4 E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: -4.00dB UTRA Cell 2 Ioc: -70.00dBm/3.84MHz Ior / Ioc: +11.00dB CPICH_Ec/Ior: -10.00dB UTRA Cell 3 Ioc: -70.00dBm/3.84MHz Ior / Ioc: +11.00dB CPICH_Ec/Ior: -10.00dB |  |  | During T0: 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB During T1: 0dB 0.3dB 0dB 0.8dB 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0.8dB 0dB 0dB 0dB 0dB During T3: 0dB 0.3dB 0dB 0dB 0dB 0dB 0.8dB 0dB During T4: 0dB 0dB 0dB 0dB 0dB 0dB 0.8dB 0dB |  |  |  |  |  |  | During T0: E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: +14.00dB UTRA Cell 2 Ioc: -70.00dBm/3.84MHz Ior / Ioc: +11.00dB CPICH_Ec/Ior: -10.00dB UTRA Cell 3 Ioc: -70.00dBm/3.84MHz Ior / Ioc: +11.00dB CPICH_Ec/Ior: -10.00dB During T1: E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: -3.70dB UTRA Cell 2 Ioc: -70.00dBm/3.84MHz Ior / Ioc: +11.80dB CPICH_Ec/Ior: -10.00dB UTRA Cell 3 Ioc: -70.00dBm/3.84MHz Ior / Ioc: +11.00dB CPICH_Ec/Ior: -10.00dB During T2 E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: -4.00dB UTRA Cell 2 Ioc: -70.00dBm/3.84MHz Ior / Ioc: +11.80dB CPICH_Ec/Ior: -10.00dB UTRA Cell 3 Ioc: -70.00dBm/3.84MHz Ior / Ioc: +11.00dB CPICH_Ec/Ior: -10.00dB During T3 E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: -3.70dB UTRA Cell 2 Ioc: -70.00dBm/3.84MHz Ior / Ioc: -5.00dB CPICH_Ec/Ior: -10.00dB UTRA Cell 3 Ioc: -70.00dBm/3.84MHz Ior / Ioc: +11.80dB CPICH_Ec/Ior: -10.00dB During T4 E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: -4.00dB UTRA Cell 2 Ioc: -70.00dBm/3.84MHz Ior / Ioc: +11.00dB CPICH_Ec/Ior: -10.00dB UTRA Cell 3 Ioc: -70.00dBm/3.84MHz Ior / Ioc: +11.80dB CPICH_Ec/Ior: -10.00dB |  |
| 4.3.2 E-UTRA FDD - UTRAN TDD cell re-selection |  |  | During T1: E-UTRA Cell 1 Noc: -98.0dBm/15kHz Ês / Noc: +11.00dB UTRA Cell 2 Ioc: -80.0dBm/1.28MHz Îor / Ioc: +11.0dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB During T2: E-UTRA Cell 1 Noc: -98.0dBm/15kHz Ês / Noc: -3.0dB UTRA Cell 2 Ioc: -80.0dBm/1.28MHz Îor / Ioc: +11.0dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB |  |  | During T1: 0dB 0.1dB 0dB 0dB 0dB 0dB During T2: 0dB 0.1dB 0dB 0dB 0dB 0dB |  |  |  |  |  |  | During T1: E-UTRA Cell 1 Noc: -98.0dBm/15kHz Ês / Noc: +11.10dB UTRA Cell 2 Ioc: -80.0dBm/1.28MHz Îor / Ioc: +11.0dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB During T2: E-UTRA Cell 1 Noc: -98.0dBm/15kHz Ês / Noc: -3.1dB UTRA Cell 2 Ioc: -80.0dBm/1.28MHz Îor / Ioc: +11.0dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB |  |
| 4.3.2A E-UTRA FDD to UTRA TDD cell re-selection for IncMon |  |  | During T0: E-UTRA Cell 1 Noc: -98.0dBm/15kHz Ês / Noc: +11.00dB UTRA Cell 2 Ioc: -80.0dBm/1.28MHz Îor / Ioc: -3.0dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB UTRA Cell 3 Ioc: -80.0dBm/1.28MHz Îor / Ioc: -3.0dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB During T1: E-UTRA Cell 1 Noc: -98.0dBm/15kHz Ês / Noc: -3.00dB UTRA Cell 2 Ioc: -80.0dBm/1.28MHz Îor / Ioc: +11.0dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB UTRA Cell 3 Ioc: -80.0dBm/1.28MHz Îor / Ioc: -3.0dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB During T2: E-UTRA Cell 1 Noc: -98.0dBm/15kHz Ês / Noc: +11.00dB UTRA Cell 2 Ioc: -80.0dBm/1.28MHz Îor / Ioc: -3.0dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB UTRA Cell 3 Ioc: -80.0dBm/1.28MHz Îor / Ioc: -3.0dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB During T3: E-UTRA Cell 1 Noc: -98.0dBm/15kHz Ês / Noc: -3.00dB UTRA Cell 2 Ioc: -80.0dBm/1.28MHz Îor / Ioc: -3.0dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB UTRA Cell 3 Ioc: -80.0dBm/1.28MHz Îor / Ioc: +11.0dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB During T04 E-UTRA Cell 1 Noc: -98.0dBm/15kHz Ês / Noc: +11.00dB UTRA Cell 2 Ioc: -80.0dBm/1.28MHz Îor / Ioc: -3.0dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB UTRA Cell 3 Ioc: -80.0dBm/1.28MHz Îor / Ioc: -3.0dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB |  |  | During T0: 0dB +0.5dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB During T1: 0dB -0.5dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB During T2: 0dB +0.5dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB During T3: 0dB -0.5dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB During T4: 0dB +0.5dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB |  |  |  |  |  |  | During T0: E-UTRA Cell 1 Noc: -98.0dBm/15kHz Ês / Noc: +11.05dB UTRA Cell 2 Ioc: -80.0dBm/1.28MHz Îor / Ioc: -3.0dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB UTRA Cell 3 Ioc: -80.0dBm/1.28MHz Îor / Ioc: -3.0dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB During T1: E-UTRA Cell 1 Noc: -98.0dBm/15kHz Ês / Noc: -3.05dB UTRA Cell 2 Ioc: -80.0dBm/1.28MHz Îor / Ioc: +11.0dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB UTRA Cell 3 Ioc: -80.0dBm/1.28MHz Îor / Ioc: -3.0dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB During T2: E-UTRA Cell 1 Noc: -98.0dBm/15kHz Ês / Noc: +11.05dB UTRA Cell 2 Ioc: -80.0dBm/1.28MHz Îor / Ioc: -3.0dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB UTRA Cell 3 Ioc: -80.0dBm/1.28MHz Îor / Ioc: -3.0dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB During T3: E-UTRA Cell 1 Noc: -98.0dBm/15kHz Ês / Noc: -3.05dB UTRA Cell 2 Ioc: -80.0dBm/1.28MHz Îor / Ioc: +11.0dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB UTRA Cell 3 Ioc: -80.0dBm/1.28MHz Îor / Ioc: -3.0dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB During T4: E-UTRA Cell 1 Noc: -98.0dBm/15kHz Ês / Noc: +11.05dB UTRA Cell 2 Ioc: -80.0dBm/1.28MHz Îor / Ioc: -3.0dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB UTRA Cell 3 Ioc: -80.0dBm/1.28MHz Îor / Ioc: -3.0dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB |  |
| 4.3.3 E-UTRAN TDD - UTRAN FDD cell re-selection: UTRA FDD is of lower priority |  |  | Same as 4.3.1.2 |  |  |  |  | Same as 4.3.1.2 |  |  |  |  | Same as 4.3.1.2 |  |
| 4.3.3A Idle mode TDD to UTRA FDD interRAT reselection |  |  | Same as 4.3.1.5 |  |  |  |  | Same as 4.3.1.5 |  |  |  |  | Same as 4.3.1.5 |  |
| 4.3.4.1 E-UTRA TDD - UTRAN TDD cell re-selection : UTRA is of higher priority |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +11.00dB UTRA Cell 2 Ioc: -80dBm/1.28MHz Îor / Ioc: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm /15kHz Ês / Noc: +11.00dB UTRA Cell 2 Ioc: -80dBm/1.28MHz Îor / Ioc: +11.00dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB During T3: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +11.00dB UTRA Cell 2 Ioc: -80dBm/1.28MHz Îor / Ioc: -3.00dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB |  |  | During T1: 0dB +0.1dB 0dB 0dB During T2: 0dB +0.1dB 0dB 0dB 0dB 0dB During T3: 0dB +0.1dB 0dB 0dB 0dB 0dB |  |  |  |  |  |  | During T1: Noc: -98.0dBm/15kHz Ês / Noc: +11.1dB Ioc: -80.0dBm/1.28MHz Îor / Ioc: -infinity During T2: Noc: -98.0dBm /15kHz Ês / Noc: +11.1dB Ioc: -80.0dBm/1.28MHz Îor / Ioc: +11.0dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB During T3: Noc: -98.0dBm/15kHz Ês / Noc: +11.1dB Ioc: -80.0dBm/1.28MHz Îor / Ioc: -3.0dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB |  |
| 4.3.4.2 E-UTRA TDD - UTRAN TDD cell re-selection : UTRA is of lower priority |  |  | Same as 4.3.2 |  |  | Same as 4.3.2 |  |  |  |  |  |  | Same as 4.3.2 |  |
| 4.3.4.3 EUTRA TDD-UTRA TDD cell reselection in fading propagation conditions: UTRA TDD is of lower priority |  |  | During T1, T2: E-UTRA Cell 1 Noc: -104.0dBm/15kHz Ês / Noc: +22.00dB UTRA Cell 2 Ioc: -80.0dBm/1.28MHz Îor / Ioc: +13.0dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB During T3, T4: E-UTRA Cell 1 Noc: -104.0dBm/15kHz Ês / Noc: -3.0dB UTRA Cell 2 Ioc: -80.0dBm/1.28MHz Îor / Ioc: +13.0dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB |  |  | During T1, T2: 0dB 0dB 0dB 0dB 0dB 0dB During T3, T4: 0dB 0dB 0dB 0dB 0dB 0dB |  |  |  |  |  |  | During T1, T2: E-UTRA Cell 1 Noc: -104.0dBm/15kHz Ês / Noc: +22.00dB UTRA Cell 2 Ioc: -80.0dBm/1.28MHz Îor / Ioc: +13.0dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB During T3, T4: E-UTRA Cell 1 Noc: -104.0dBm/15kHz Ês / Noc: -3.0dB UTRA Cell 2 Ioc: -80.0dBm/1.28MHz Îor / Ioc: +13.0dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB |  |
| 4.3.4.4 E-UTRA TDD to UTRA TDD cell re-selection for IncMon |  |  | Same as 4.3.2A |  |  | Same as 4.3.2A |  |  |  |  |  |  | Same as 4.3.2A |  |
| 4.4.1 E-UTRAN FDD - GSM cell re-selection |  |  | During T1: E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: +9.00dB GSM Cell 2 Signal level: -90.00dBm During T2: E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: -4.00dB GSM Cell 2 Signal level: -75.0dBm |  |  | During T1: -1.4dB +1.5dB 0dB During T2: -1.4dB +0.3dB 0dB |  |  |  |  |  |  | During T1: E-UTRA Cell 1 Noc: -99.40dBm/15kHz Ês / Noc: +10.5dB GSM Cell 2 Signal level: -90.00dBm During T2: E-UTRA Cell 1 Noc: -99.40dBm/15kHz Ês / Noc: -3.70dB GSM Cell 2 Signal level: -75.00dBm |  |
| 4.4.2 E-UTRAN TDD - GSM cell re-selection |  |  | Same as 4.4.1 |  |  | Same as 4.4.1 |  |  |  |  |  |  | Same as 4.4.1 |  |
| 4.5.1.1 RRC IDLE / E-UTRAN to HRPD Cell re-selection / E-UTRAN FDD - HRPD cell re-selection: HRPD is of lower priority |  |  | During T1: E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: +9.00dB HRPD Cell 2 Ioc: -55.00dBm/1.2288MHz Ior / Ioc: 0 dB During T2: E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: -4.00dB HRPD Cell 2 Ioc: -55.00dBm/1.2288MHz Ior / Ioc: 0 dB |  |  | During T1 -1.4dB 1.5dB 0dB 0dB During T2 -1.4dB 0.3dB 0dB 0dB |  |  |  |  |  |  | During T1: E-UTRA Cell 1 Noc: -99.4dBm/15kHz Ês / Noc: +10.5dB HRPD Cell 2 Ioc: -55.00dBm/1.2288MHz Ior / Ioc: 0 dB During T2: E-UTRA Cell 1 Noc: -99.4dBm/15kHz Ês / Noc: -3.70dB HRPD Cell 2 Ioc: -55.00dBm/1.2288MHz Ior / Ioc: 0 dB |  |
| 4.5.2.1 E-UTRAN TDD - HRPD Cell Reselection: HRPD is of Lower Priority |  |  | Same as 4.5.1.1 |  |  | Same as 4.5.1.1 |  |  |  |  |  |  | Same as 4.5.1.1 |  |
| 5.1.1 E-UTRAN FDD-FDD Handover intra frequency case |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +11.00dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +11.00dB |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB +0.5dB During T3: 0dB 0dB +0.5dB |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +11.50dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: +8.00 Ês2 / Noc: +11.50dB |  |
| 5.1.2 E-UTRAN TDD-TDD Handover intra frequency case |  |  | Same as 5.1.1 |  |  | Same as 5.1.1 |  |  |  |  |  |  | Same as 5.1.1 |  |
| 5.1.3 E-UTRAN FDD-FDD Handover inter frequency case |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.0dB During T3: Noc1: -98dBm /15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.0dB |  |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB +0.5dB During T3: 0dB 0dB 0dB +0.5dB |  |  |  |  |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.50dB During T3: Noc1: -98dBm /15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.50dB |  |
| 5.1.3_2 E-UTRAN FDD - FDD Inter frequency handover for UE Category 1bis |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.0dB During T3: Noc1: -98dBm /15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.0dB |  |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB |  |  |  |  |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.0dB During T3: Noc1: -98dBm /15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.0dB |  |
| 5.1.4 E-UTRAN TDD-TDD Handover inter frequency case |  |  | Same as 5.1.3 |  |  | Same as 5.1.3 |  |  |  |  |  |  | Same as 5.1.3 |  |
| 5.1.4_2 E-UTRAN TDD - TDD Inter frequency handover for UE Category 1bis |  |  | Same as 5.1.3_2 |  |  | Same as 5.1.3_2 |  |  |  |  |  |  | Same as 5.1.3_2 |  |
| 5.1.5 E-UTRAN FDD-FDD inter-frequency Handover with unknown target cell |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.0dB |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB |  |  |  |  |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.0dB |  |
| 5.1.5_2 E-UTRAN FDD - FDD Inter frequency handover: unknown target cell for UE Category 1bis |  |  | Same as 5.1.5 |  |  | Same as 5.1.5 |  |  |  |  |  |  | Same as 5.1.5 |  |
| 5.1.6 E-UTRAN TDD-TDD inter-frequency Handover with unknown target cell |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: +5.0dB |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB |  |  |  |  |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: +5.0dB |  |
| 5.1.6_2 E-UTRAN TDD - TDD Inter frequency handover; unknown Target Cell for UE Category 1bis |  |  | Same as 5.1.6 |  |  | Same as 5.1.6 |  |  |  |  |  |  | Same as 5.1.6 |  |
| 5.1.7 E-UTRAN FDD-TDD Handover inter frequency case |  |  | Same as 5.1.3 |  |  | Same as 5.1.3 |  |  |  |  |  |  | Same as 5.1.3 |  |
| 5.1.7_2 E-UTRAN FDD - TDD Inter frequency handover for UE Category 1bis |  |  | Same as 5.1.3_2 |  |  | Same as 5.1.3_2 |  |  |  |  |  |  | Same as 5.1.3_2 |  |
| 5.1.8 E-UTRAN TDD-FDD Handover inter frequency case |  |  | Same as 5.1.3 |  |  | Same as 5.1.3 |  |  |  |  |  |  | Same as 5.1.3 |  |
| 5.1.8_2 E-UTRAN TDD - FDD Inter frequency handover for UE Category 1bis |  |  | Same as 5.1.3_2 |  |  | Same as 5.1.3_2 |  |  |  |  |  |  | Same as 5.1.3_2 |  |
| 5.1.9 E-UTRAN FDD-FDD Intra frequency handover for 5MHz bandwidth |  |  | Same as 5.1.1 |  |  | Same as 5.1.1 |  |  |  |  |  |  | Same as 5.1.1 |  |
| 5.1.10 E-UTRAN FDD-FDD Handover intra frequency handover for UE category 0 |  |  | Same as 5.1.1 |  |  | Same as 5.1.1 |  |  |  |  |  |  | Same as 5.1.1 |  |
| 5.1.11 E-UTRAN HD-FDD Handover intra frequency handover for UE category 0 |  |  | Same as 5.1.1 |  |  | Same as 5.1.1 |  |  |  |  |  |  | Same as 5.1.1 |  |
| 5.1.12 E-UTRAN TDD-TDD Handover intra frequency handover for UE category 0 |  |  | Same as 5.1.1 |  |  | Same as 5.1.1 |  |  |  |  |  |  | Same as 5.1.1 |  |
| 5.1.13 E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +12.00dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +12.00dB |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB +0.5dB During T3: 0dB 0dB +0.5dB |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +12.50dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +12.50dB |  |
| 5.1.14 E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA |  |  | Same as 5.1.13 |  |  | Same as 5.1.13 |  |  |  |  |  |  | Same as 5.1.13 |  |
| 5.1.15 E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeA |  |  | Same as 5.1.13 |  |  | Same as 5.1.13 |  |  |  |  |  |  | Same as 5.1.13 |  |
| 5.1.16 E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: -12.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: -12.00dB Ês2 / Noc: -7dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: -12.00dB Ês2 / Noc: -7dB |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB +0.5dB During T3: 0dB 0dB +0.5dB |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: -12.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: -12.00dB Ês2 / Noc: -6.5dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: -12.00 Ês2 / Noc: -6.5dB |  |
| 5.1.17 E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB |  |  | Same as 5.1.16 |  |  | Same as 5.1.16 |  |  |  |  |  |  | Same as 5.1.16 |  |
| 5.1.18 E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeB |  |  | Same as 5.1.16 |  |  | Same as 5.1.16 |  |  |  |  |  |  | Same as 5.1.16 |  |
| 5.1.19 E-UTRAN FDD - FDD Intra frequency handover for UE Category 1bis |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +12.00dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +12.00dB |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB +0.5dB During T3: 0dB 0dB +0.5dB |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +12.50dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: +8.00 Ês2 / Noc: +12.50dB |  |
| 5.1.20 E-UTRAN TDD - TDD Intra frequency handover for UE Category 1bis |  |  | Same as 5.1.19 |  |  | Same as 5.1.19 |  |  |  |  |  |  | Same as 5.1.19 |  |
| 5.1.28 E-UTRAN HD-FDD inter frequency handover for CE UEs in CEModeA |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: -3dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: -3dB Noc2: -98dBm/15kHz Ês2 / Noc2: +4dB During T3: Noc1: -98dBm /15kHz Ês1 / Noc1: -3dB Noc2: -98dBm/15kHz Ês2 / Noc2: +4dB |  |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB |  |  |  |  |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: -3dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: -3dB Noc2: -98dBm/15kHz Ês2 / Noc2: +4dB During T3: Noc1: -98dBm /15kHz Ês1 / Noc1: -3dB Noc2: -98dBm/15kHz Ês2 / Noc2: +4dB |  |
| 5.1.34 E-UTRAN HD-FDD intra frequency handover for CE UEs in CEModeA without SFN acquisition |  |  | Same as 5.1.14 The Test Tolerance values are taken from the analysis for similar HD-FDD Test case 5.1.14 and no separate analysis is needed. |  |  | Same as 5.1.14 The Test Tolerance values are taken from the analysis for similar HD-FDD Test case 5.1.14 and no separate analysis is needed. |  |  |  |  |  |  | Same as 5.1.14 The Test Tolerance values are taken from the analysis for similar HD-FDD Test case 5.1.14 and no separate analysis is needed. |  |
| 5.1.41 E-UTRAN FDD – FDD Intra-band Inter-frequency sync DAPS handover |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.0dB During T3: Noc1: -98dBm /15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.0dB During T4: Noc1: -98dBm /15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.0dB During T5: Noc1: -98dBm /15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.0dB |  |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB +0.5dB During T3: 0dB 0dB 0dB +0.5dB During T4: 0dB 0dB 0dB +0.5dB During T5: 0dB 0dB 0dB +0.5dB |  |  |  |  |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.50dB During T3: Noc1: -98dBm /15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.50dB During T4: Noc1: -98dBm/15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.50dB During T5: Noc1: -98dBm /15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.50dB |  |
| 5.1.42 E-UTRAN FDD – FDD Intra-band Inter-frequency async DAPS handover |  |  | Same as 5.1.41 |  |  | Same as 5.1.41 |  |  |  |  |  |  | Same as 5.1.41 |  |
| 5.1.43 E-UTRAN FDD – FDD Inter-band Inter-frequency sync DAPS handover |  |  | Same as 5.1.41 |  |  | Same as 5.1.41 |  |  |  |  |  |  | Same as 5.1.41 |  |
| 5.1.44 E-UTRAN FDD – FDD Inter-band Inter-frequency async DAPS handover |  |  | Same as 5.1.41 |  |  | Same as 5.1.41 |  |  |  |  |  |  | Same as 5.1.41 |  |
| 5.1.45 E-UTRAN FDD - FDD Intra frequency DAPS handover |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +11.00dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +11.00dB During T4: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +11.00dB During T5: Noc: -98dBm /15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +11.00dB |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB +0.5dB During T3: 0dB 0dB +0.5dB During T4: 0dB 0dB +0.5dB During T5: 0dB 0dB +0.5dB |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +11.50dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: +8.00 Ês2 / Noc: +11.50Db During T4: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +11.50dB During T5: Noc: -98dBm /15kHz Ês1 / Noc: +8.00 Ês2 / Noc: +11.50Db |  |
| 5.1.46 E-UTRAN TDD - TDD Intra frequency DAPS handover |  |  | Same as 5.1.45 |  |  | Same as 5.1.45 |  |  |  |  |  |  | Same as 5.1.45 |  |
| 5.1.47 E-UTRAN FDD - FDD Intra frequency conditional handover |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +11.00dB |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB +0.5dB |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +11.50dB |  |
| 5.1.48 E-UTRAN TDD - TDD Intra frequency conditional handover |  |  | Same as 5.1.47 |  |  | Same as 5.1.47 |  |  |  |  |  |  | Same as 5.1.47 |  |
| 5.1.49 E-UTRAN FDD - FDD Inter frequency conditional handover |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.0dB |  |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB |  |  |  |  |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7dB |  |
| 5.1.50 E-UTRAN TDD - TDD Inter frequency conditional handover |  |  | Same as 5.1.49 |  |  | Same as 5.1.49 |  |  |  |  |  |  | Same as 5.1.49 |  |
| 5.1.51 E-UTRAN FDD - TDD Inter frequency conditional handover |  |  | Same as 5.1.49 |  |  | Same as 5.1.49 |  |  |  |  |  |  | Same as 5.1.49 |  |
| 5.1.52 E-UTRAN TDD - FDD Inter frequency conditional handover |  |  | Same as 5.1.49 |  |  | Same as 5.1.49 |  |  |  |  |  |  | Same as 5.1.49 |  |
| 5.1.53 E-UTRAN TDD – TDD Intra-band Inter-frequency sync DAPS handover |  |  | Same as 5.1.41 |  |  | Same as 5.1.41 |  |  |  |  |  |  | Same as 5.1.41 |  |
| 5.1.54 E-UTRAN TDD – TDD Inter-band Inter-frequency sync DAPS handover |  |  | Same as 5.1.41 |  |  | Same as 5.1.41 |  |  |  |  |  |  | Same as 5.1.41 |  |
| 5.1.55 E-UTRAN FDD - TDD inter-band inter-frequency synchronous DAPS handover |  |  | Same as 5.1.41 |  |  | Same as 5.1.41 |  |  |  |  |  |  | Same as 5.1.41 |  |
| 5.1.56 E-UTRAN TDD - FDD inter-band inter-frequency synchronous DAPS handover |  |  | Same as 5.1.41 |  |  | Same as 5.1.41 |  |  |  |  |  |  | Same as 5.1.41 |  |
| 5.1.57 E-UTRAN FDD – TDD Inter-band Inter-frequency async DAPS handover |  |  | Same as 5.1.41 |  |  | Same as 5.1.41 |  |  |  |  |  |  | Same as 5.1.41 |  |
| 5.1.58 E-UTRAN TDD – FDD Inter-band Inter-frequency async DAPS handover |  |  | Same as 5.1.41 |  |  | Same as 5.1.41 |  |  |  |  |  |  | Same as 5.1.41 |  |
| 5.2.1 E-UTRAN FDD - UTRAN FDD handover |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 0.00dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 0.00dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc:-1.80dB During T3: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 0.00dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc:-1.80dB |  |  | During T1: 0dB -1.10dB 0dB - During T2: 0dB -1.10dB 0dB 0dB During T3: 0dB -1.10dB 0dB 0dB |  |  |  |  |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: -1.10dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: -1.10dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc:-1.80dB During T3: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: -1.10dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc:-1.80dB |  |
| 5.2.2 E-UTRAN TDD - UTRAN FDD handover |  |  | Same as 5.2.1 |  |  | Same as 5.2.1 |  |  |  |  |  |  | Same as 5.2.1 |  |
| 5.2.3 E-UTRAN FDD - GSM handover |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -85dBm During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -75dBm During T3: E-UTRA Cell 1 Noc: -98dBm /15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -75dBm |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB During T3: 0dB 0dB 0dB |  |  |  |  |  |  | During T1: E-UTRAN Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -85dBm During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -75dBm During T3: E-UTRA Cell 1 Noc: -98dBm /15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -75dBm |  |
| 5.2.4 E-UTRA TDD - UTRA TDD handover |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +13.00dB UTRA Cell 2 Ioc: -80dBm/1.28MHz Îor / Ioc: -3.00dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: -3.00dB UTRA Cell 2 Ioc: -80dBm/1.28MHz Îor / Ioc: 11.00dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB During T3: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: -3.00dB UTRA Cell 2 Ioc: -80dBm/1.28MHz Îor / Ioc: 11.00dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB |  |  | During T1: -1.05dB 2.1dB -0.8dB 0dB 0dB 0dB During T2: -1.05dB 0dB -0.8dB 1.6dB 0dB 0dB During T3: -1.05dB 0dB -0.8dB +1.6dB 0dB 0dB |  |  |  |  |  |  | During T1: Noc: -99.05dBm/15kHz Ês / Noc: +15.1dB Ioc: -80.8dBm/1.28MHz Îor / Ioc: -3.0dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB During T2: Noc: -99.05dBm/15kHz Ês / Noc: -3.0dB Ioc: -80.8dBm/1.28MHz Îor / Ioc: 12.6dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB During T3: Noc: -99.05dBm/15kHz Ês / Noc: -3.0dB Ioc: -80.8dBm/1.28MHz Îor / Ioc: 12.6dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB |  |
| 5.2.5 E-UTRA FDD - UTRA TDD handover |  |  | Same as 5.2.4 |  |  | Same as 5.2.4 |  |  |  |  |  |  | Same as 5.2.4 |  |
| 5.2.6 E-UTRA TDD - GSM handover |  |  | Same as 5.2.3 |  |  | Same as 5.2.3 |  |  |  |  |  |  | Same as 5.2.3 |  |
| 5.2.7 E-UTRAN FDD - UTRAN FDD handover: unknown target cell |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 0dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 0dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc: -1.8 dB |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB |  |  |  |  |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 0dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 0dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc: -1.8dB |  |
| 5.2.8 E-UTRAN FDD - GSM handover: unknown target cell |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -75 dBm |  |  | During T1: 0dB 0dB During T2: 0dB 0dB 0dB |  |  |  |  |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -75 dBm |  |
| 5.2.9 E-UTRAN TDD - GSM handover: unknown target cell |  |  | Same as 5.2.8 |  |  | Same as 5.2.8 |  |  |  |  |  |  | Same as 5.2.8 |  |
| 5.2.10 E-UTRAN TDD - UTRAN TDD HO test: unknown target cell |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 3dB UTRA Cell 2 Ioc: -80dBm/1.28MHz Ior / Ioc: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 3dB UTRA Cell 2 Ioc: -80dBm/1.28MHz Ior / Ioc: 13 dB PCCPCH_Ec/Ior: -3.00dB DwPCH_Ec/Ior: 0dB |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB 0dB 0dB |  |  |  |  |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 3dB UTRA Cell 2 Ioc: -80dBm/1.28MHz Ior / Ioc: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 3dB UTRA Cell 2 Ioc: -80dBm/1.28MHz Ior / Ioc: 13 dB PCCPCH_Ec/Ior: -3.00dB DwPCH_Ec/Ior: 0dB |  |
| 5.2.11 E-UTRAN FDD - UTRAN FDD Handover for 5MHz Bandwidth |  |  | Same as 5.2.1 |  |  | Same as 5.2.1 |  |  |  |  |  |  | Same as 5.2.1 |  |
| 5.3.1 RRC CONNECTED / Handover from E-UTRAN to non-3GPP RATs / E-UTRAN FDD - HRPD handover |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 0dB HRPD Cell 2 Ioc: -55.00dBm/1.2288MHz Ior / Ioc: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 0dB HRPD Cell 2 Ioc: -55.00dBm/1.2288MHz Ior / Ioc: 0 dB During T3: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 0dB HRPD Cell 2 Ioc: -55.00dBm/1.2288MHz Ior / Ioc: 0 dB |  |  | During T1: 0dB -0.8dB 0dB During T2: 0dB -0.8dB 0dB 0dB During T3: 0dB -0.8dB 0dB 0dB |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Ês / Noc: -0.8dB Ioc: -55.00dBm/1.2288MHz Ior / Ioc: -infinity During T2: Noc: -98dBm/15kHz Ês / Noc: -0.8dB Ioc: -55.00dBm/1.2288MHz Ior / Ioc: 0 dB During T3: Noc: -98dBm/15kHz Ês / Noc: -0.8dB Ioc: -55.00dBm/1.2288MHz Ior / Ioc: 0 dB |  |
| 5.3.5 E-UTRAN TDD - HRPD handover |  |  | Same as 5.3.1 |  |  | Same as 5.3.1 |  |  |  |  |  |  | Same as 5.3.1 |  |
| 6.1.1 E-UTRAN FDD Intra-frequency RRC Re-establishment |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +7.00dB Ês2 / Noc: +4.00dB During T2: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: +4.00dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: -infinity Ês2 / Noc: +4.00dB |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB During T3: 0dB 0dB 0dB |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +7.00dB Ês2 / Noc: +4.00dB During T2: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: +4.00dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: -infinity Ês2 / Noc: +4.00dB |  |
| 6.1.1_2 E-UTRAN FDD Intra-frequency RRC Re-establishment for UE Category 1bis |  |  | Same as 6.1.1 |  |  | Same as 6.1.1 |  |  |  |  |  |  | Same as 6.1.1 |  |
| 6.1.2 E-UTRAN FDD Inter-frequency RRC Re-establishment |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: -infinity Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T3: Noc1: -98dBm /15kHz Ês1 / Noc1: -infinity Noc2: -98dBm/15kHz Ês2 / Noc2: +7.00dB |  |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB |  |  |  |  |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: -infinity Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T3: Noc1: -98dBm /15kHz Ês1 / Noc1: -infinity Noc2: -98dBm/15kHz Ês2 / Noc2: +7.00dB |  |
| 6.1.2_2 E-UTRAN FDD Inter-frequency RRC Re-establishment for UE Category 1bis |  |  | Same as 6.1.2 |  |  | Same as 6.1.2 |  |  |  |  |  |  | Same as 6.1.2 |  |
| 6.1.3 E-UTRAN TDD Intra-frequency RRC Re-establishment |  |  | Same as 6.1.1 |  |  | Same as 6.1.1 |  |  |  |  |  |  | Same as 6.1.1 |  |
| 6.1.3_2 E-UTRAN TDD Intra-frequency RRC Re-establishment for UE Category 1bis |  |  | Same as 6.1.1 |  |  | Same as 6.1.1 |  |  |  |  |  |  | Same as 6.1.1 |  |
| 6.1.4 E-UTRAN TDD Inter-frequency RRC Re-establishment |  |  | Same as 6.1.3 |  |  | Same as 6.1.3 |  |  |  |  |  |  | Same as 6.1.3 |  |
| 6.1.4_2 E-UTRAN TDD Inter-frequency RRC Re-establishment for UE Category 1bis |  |  | Same as 6.1.2 |  |  | Same as 6.1.2 |  |  |  |  |  |  | Same as 6.1.2 |  |
| 6.1.5 E-UTRAN FDD Intra-frequency RRC Re-establishment for 5MHz Bandwidth |  |  | Same as 6.1.1 |  |  | Same as 6.1.1 |  |  |  |  |  |  | Same as 6.1.1 |  |
| 6.1.6 E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for UE category 0 |  |  | Same as 6.1.1 |  |  | Same as 6.1.1 |  |  |  |  |  |  | Same as 6.1.1 |  |
| 6.1.7 E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for UE category 0 |  |  | Same as 6.1.1 |  |  | Same as 6.1.1 |  |  |  |  |  |  | Same as 6.1.1 |  |
| 6.1.8 E-UTRAN TDD Intra-frequency RRC Re-establishment for UE category 0 |  |  | Same as 6.1.1 |  |  | Same as 6.1.1 |  |  |  |  |  |  | Same as 6.1.1 |  |
| 6.1.9 E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA |  |  | Same as 6.1.1 |  |  | Same as 6.1.1 |  |  |  |  |  |  | Same as 6.1.1 |  |
| 6.1.10 E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA |  |  | Same as 6.1.1 |  |  | Same as 6.1.1 |  |  |  |  |  |  | Same as 6.1.1 |  |
| 6.1.11 E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA |  |  | Same as 6.1.1 |  |  | Same as 6.1.1 |  |  |  |  |  |  | Same as 6.1.1 |  |
| 6.1.12 E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +7.00dB Ês2 / Noc: +4.00dB During T2: Noc: -98dBm/15kHz Ês1 / Noc: -15.00dB Ês2 / Noc: -12.00dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: -12.00dB During T4: Noc: -98dBm /15kHz Ês1 / Noc: -infinity Ês2 / Noc: -12.00dB |  |  | During T1: 0dB 0dB 0dB During T2: 0dB +0.6dB 0dB During T3: 0dB 0dB 0dB During T4: 0dB 0dB 0dB |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +7.00dB Ês2 / Noc: +4.00dB During T2: Noc: -98dBm/15kHz Ês1 / Noc: -14.40dB Ês2 / Noc: -12.00dB During T2: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: +4.00dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: -infinity Ês2 / Noc: -12.00dB |  |
| 6.1.13 E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB |  |  | Same as 6.1.12 |  |  | Same as 6.1.12 |  |  |  |  |  |  | Same as 6.1.12 |  |
| 6.1.14 E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB |  |  | Same as 6.1.12 |  |  | Same as 6.1.12 |  |  |  |  |  |  | Same as 6.1.12 |  |
| 6.1.15 HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in In-Band mode under enhanced coverage |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: -12.6dB Êsn1 / Noc: +7.0dB Êsn2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: -12.6dB Êsn1 / Noc: -infinity Êsn2 / Noc: -12.6dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: -12.6dB Êsn1 / Noc: -infinity Êsn2 / Noc: -12.6dB |  |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: -12.6dB Êsn1 / Noc: +7.0dB Êsn2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: -12.6dB Êsn1 / Noc: -infinity Êsn2 / Noc: -12.6dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: -12.6dB Êsn1 / Noc: -infinity Êsn2 / Noc: -12.6dB |  |
| 6.1.16 HD-FDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4.0dB Êsn1 / Noc: +7.0dB Êsn2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.0dB Êsn1 / Noc: -infinity Êsn2 / Noc: +4.0dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: +4.0dB Êsn1 / Noc: -infinity Êsn2 / Noc: +4.0dB |  |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4.0dB Êsn1 / Noc: +7.0dB Êsn2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.0dB Êsn1 / Noc: -infinity Êsn2 / Noc: +4.0dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: +4.0dB Êsn1 / Noc: -infinity Êsn2 / Noc: +4.0dB |  |
| 6.1.18 E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for CE UE in CEModeA |  |  | Same as 6.1.2 |  |  | Same as 6.1.2 |  |  |  |  |  |  | Same as 6.1.2s |  |
| 6.1.23 E-UTRAN TDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage |  |  | Same as 6.1.16 |  |  | Same as 6.1.16 |  |  |  |  |  |  | Same as 6.1.16 |  |
| 6.1.24 E-UTRAN TDD - TDD Intra-frequency RRC Re-establishment for UE category NB1 in In-Band mode under enhanced coverage |  |  | Same as 6.1.15 |  |  | Same as 6.1.15 |  |  |  |  |  |  | Same as 6.1.15 |  |
| 6.2.1 E-UTRAN FDD - Contention Based Random Access Test |  |  | Test 1 and Test 2 Absolute uplink power: Normal conditions ±9dB Extreme conditions ±12dB Relative uplink power step: Normal conditions ±3dB Extreme conditions ±5dB Uplink timing Te: ±12Ts |  |  | 1.5dB 1.5dB 0.7dB 0.7dB 3Ts |  |  |  |  |  |  | Test 1 and Test 2 Absolute uplink power: Normal conditions ±10.5dB Extreme conditions ±13.5dB Relative uplink power step: Normal conditions ±3.7dB Extreme conditions ±5.7dB Uplink timing Te: ±15Ts |  |
| 6.2.1_2 E-UTRAN FDD - Contention Based Random Access Test |  |  | Same as 6.2.1 |  |  | Same as 6.2.1 |  |  |  |  |  |  | Same as 6.2.1 |  |
| 6.2.2 E-UTRAN FDD - Non Contention Based Random Access Test |  |  | Same as 6.2.1 |  |  | Same as 6.2.1 |  |  |  |  |  |  | Same as 6.2.1 |  |
| 6.2.2_2 E-UTRAN FDD - Non Contention Based Random Access Test |  |  | Same as 6.2.1 |  |  | Same as 6.2.1 |  |  |  |  |  |  | Same as 6.2.1 |  |
| 6.2.3 E-UTRAN TDD - Contention Based Random Access Test |  |  | Same as 6.2.1 |  |  | Same as 6.2.1 |  |  |  |  |  |  | Same as 6.2.1 |  |
| 6.2.3_2 E-UTRAN TDD - Contention Based Random Access Test |  |  | Same as 6.2.1 |  |  | Same as 6.2.1 |  |  |  |  |  |  | Same as 6.2.1 |  |
| 6.2.4 E-UTRAN TDD - Non Contention Based Random Access Test |  |  | Same as 6.2.1 |  |  | Same as 6.2.1 |  |  |  |  |  |  | Same as 6.2.1 |  |
| 6.2.4_2 E-UTRAN TDD - Non Contention Based Random Access Test |  |  | Same as 6.2.1 |  |  | Same as 6.2.1 |  |  |  |  |  |  | Same as 6.2.1 |  |
| 6.2.5 E-UTRAN FDD - Contention Based Random Access Test for 5MHz bandwidth |  |  | Same as 6.2.1 |  |  | Same as 6.2.1 |  |  |  |  |  |  | Same as 6.2.1 |  |
| 6.2.6 E-UTRAN FDD - Non-contention Based Random Access Test for 5MHz bandwidth |  |  | Same as 6.2.1 |  |  | Same as 6.2.1 |  |  |  |  |  |  | Same as 6.2.1 |  |
| 6.2.7 E-UTRAN FDD - Non-Contention Based Random Access Test for SCell in sTAG |  |  | Test 1, Test 2 and Test 3 Absolute uplink power: Normal conditions ±9dB Extreme conditions ±12dB Relative uplink power step: Normal conditions ±3dB Extreme conditions ±5dB Uplink timing Te: ±12Ts |  |  | 1.5dB 1.5dB 0.7dB 0.7dB 3Ts |  |  |  |  |  |  | Test 1, Test 2 and Test 3 Absolute uplink power: Normal conditions ±10.5dB Extreme conditions ±13.5dB Relative uplink power step: Normal conditions ±3.7dB Extreme conditions ±5.7dB Uplink timing Te: ±15Ts |  |
| 6.2.8 E-UTRAN TDD - Non-Contention Based Random Access Test for SCell in sTAG |  |  | Same as 6.2.7 |  |  | Same as 6.2.7 |  |  |  |  |  |  | Same as 6.2.7 |  |
| 6.2.9 3DL/3UL TDD CA Non-Contention Based Random Access Test for 2 SCells |  |  | Same as 6.2.7 |  |  | Same as 6.2.7 |  |  |  |  |  |  | Same as 6.2.7 |  |
| 6.2.10 E-UTRAN FDD - Contention based random access test for Cat-M1 UEs in normal coverage |  |  | Test 1 and Test 2 Absolute uplink power: Normal conditions ±9dB Extreme conditions ±12dB Relative uplink power step: Normal conditions ±3dB Extreme conditions ±5dB Timing Te: ±24Ts rsrp-ThresholdsPrach {24, 27, 33} Noc: -103dBm/15kHz Ês / Noc: +3dB |  |  | 1.5dB 1.5dB 0.7dB 0.7dB 3Ts {0, 0, -2} for normal {-3, -3, -5} for extreme -0.4dB -0.3dB |  |  |  |  |  |  | Test 1 and Test 2 Absolute uplink power: Normal conditions ±10.5dB Extreme conditions ±13.5dB Relative uplink power step: Normal conditions ±3.7dB Extreme conditions ±5.7dB Uplink timing Te: ±27Ts rsrp-ThresholdsPrach {24, 27, 31} for normal conditions and {21, 24, 28} for extreme conditions Noc: -103.4dBm/15kHz Ês / Noc: +2.7dB |  |
| 6.2.11 E-UTRAN HD-FDD - Contention based random access test for Cat-M1 UEs in normal coverage |  |  | Same as 6.2.10 |  |  | Same as 6.2.10 |  |  |  |  |  |  | Same as 6.2.10 |  |
| 6.2.12 E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage |  |  | Same as 6.2.1 |  |  | Same as 6.2.10 |  |  |  |  |  |  | Same as 6.2.10 |  |
| 6.2.13 E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage |  |  | Test 1 and Test 2 Absolute uplink power: Normal conditions ±9dB Extreme conditions ±12dB Relative uplink power step: Normal conditions ±3dB Extreme conditions ±5dB Timing Te: ±48Ts rsrp-ThresholdsPrach {23, 37, 47} Noc: -98dBm/15kHz Ês / Noc: -12dB |  |  | 1.5dB 1.5dB 0.7dB 0.7dB 3Ts {-2, 1, 0} for normal {-5, 4, 3} for extreme -1.1dB 0.6dB |  |  |  |  |  |  | Test 1 and Test 2 Absolute uplink power: Normal conditions ±10.5dB Extreme conditions ±13.5dB Relative uplink power step: Normal conditions ±3.7dB Extreme conditions ±5.7dB Uplink timing Te: ±51Ts rsrp-ThresholdsPrach {21, 38, 47} for normal conditions and {18, 41, 50} for extreme conditions Noc: -99.1dBm/15kHz Ês / Noc: -11.4dB |  |
| 6.2.14 E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage |  |  | Same as 6.2.13 |  |  | Same as 6.2.13 |  |  |  |  |  |  | Same as 6.2.13 |  |
| 6.2.15 E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage |  |  | Same as 6.2.13 |  |  | Same as 6.2.13 |  |  |  |  |  |  | Same as 6.2.13 |  |
| 6.2.16 Contention Based Random Access Test for UE category NB1 UEs In-band mode in normal coverage |  |  | Absolute uplink power: Normal conditions ±9dB Extreme conditions ±12dB Relative uplink power step: Normal conditions ±2dB Extreme conditions ±4dB Uplink timing Te: ±80Ts rsrp-ThresholdsPrach {40, 55} Ês / Noc: 3dB |  |  | 1.4dB 1.4dB 0.7dB 0.7dB 3Ts {0, -1} for normal {0, -4} for extreme 0.1dB |  |  |  |  |  |  | Absolute uplink power: Normal conditions ±10.4dB Extreme conditions ±13.4dB Relative uplink power step: Normal conditions ±2.7dB Extreme conditions ±4.7dB Uplink timing Te: ±83Ts rsrp-ThresholdsPrach {40, 54} for normal conditions and {40, 51} for extreme conditions Ês / Noc: 3.1dB |  |
| 6.2.17 Contention Based Random Access Test for UE category NB1 UEs In-band mode in Enhanced Coverage |  |  | Absolute uplink power: Normal conditions ±13.3dB Extreme conditions ±16.3dB Relative uplink power step: Normal conditions ±2dB Extreme conditions ±4dB Uplink timing Te: ±80Ts rsrp-ThresholdsPrach {35, 56} |  |  | 1.0dB 1.0dB 0.7dB 0.7dB 3Ts {-1, 1} for normal {-4, 4} for extreme |  |  |  |  |  |  | Absolute uplink power: Normal conditions ±14.3dB Extreme conditions ±17.3dB Relative uplink power step: Normal conditions ±2.7dB Extreme conditions ±4.7dB Uplink timing Te: ±83Ts rsrp-ThresholdsPrach {34, 57} for normal conditions and {31, 60} for extreme conditions |  |
| 6.2.18 Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs In-band mode in Enhanced Coverage |  |  | Same as 6.2.17 |  |  | Same as 6.2.17 |  |  |  |  |  |  | Same as 6.2.17 |  |
| 6.2.19 TDD Contention Based Random Access Test for UE category NB1 UEs In-band mode in normal coverage |  |  | Same as 6.2.16 |  |  | Same as 6.2.16 |  |  |  |  |  |  | Same as 6.2.16 |  |
| 6.2.20 TDD Contention Based Random Access Test for UE category NB1 UEs In-band mode in enhanced coverage |  |  | Same as 6.2.17 |  |  | Same as 6.2.17 |  |  |  |  |  |  | Same as 6.2.17 |  |
| 6.2.21 TDD Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs In-band mode in Enhanced Coverage |  |  | Same as 6.2.17 |  |  | Same as 6.2.17 |  |  |  |  |  |  | Same as 6.2.17 |  |
| 6.3.1 Redirection from E-UTRAN FDD to UTRAN FDD |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 4dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc: 0.02 dB |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0.4dB |  |  |  |  |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 4dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc: 0.42dB |  |
| 6.3.1_2 Redirection from E-UTRAN FDD to UTRAN FDD |  |  | Same as 6.3.1 |  |  | Same as 6.3.1 |  |  |  |  |  |  | Same as 6.3.1 |  |
| 6.3.2 Redirection from E-UTRAN TDD to UTRAN FDD |  |  | Same as 6.3.1 |  |  | Same as 6.3.1 |  |  |  |  |  |  | Same as 6.3.1 |  |
| 6.3.2_2 Redirection from E-UTRAN TDD to UTRAN FDD |  |  | Same as 6.3.1 |  |  | Same as 6.3.1 |  |  |  |  |  |  | Same as 6.3.1 |  |
| 6.3.3 Redirection from E-UTRAN FDD to GERAN when System Information is provided |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -75 dBm |  |  | During T1: 0dB 0dB During T2: 0dB 0dB 0dB |  |  |  |  |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -75 dBm |  |
| 6.3.3_2 Redirection from E-UTRAN FDD to GERAN when System Information is provided |  |  | Same as 6.3.3 |  |  | Same as 6.3.3 |  |  |  |  |  |  | Same as 6.3.3 |  |
| 6.3.4 Redirection from E-UTRAN TDD to GERAN when System Information is provided |  |  | Same as 6.3.3 |  |  | Same as 6.3.3 |  |  |  |  |  |  | Same as 6.3.3 |  |
| 6.3.4_2 Redirection from E-UTRAN TDD to GERAN when System Information is provided |  |  | Same as 6.3.3 |  |  | Same as 6.3.3 |  |  |  |  |  |  | Same as 6.3.3 |  |
| 6.3.5 E-UTRA TDD RRC connection release redirection to UTRA TDD |  |  | During T1: E-UTRA Cell 1 Noc: -98.0dBm/15kHz Ês / Noc: +4.00dB UTRA Cell 2 Ioc: -80.0dBm/1.28MHz Îor / Ioc: -infinity PCCPCH_Ec/Ior: -4.77dB DwPCH_Ec/Ior: 0dB During T2: E-UTRA Cell 1 Noc: -98.0dBm/15kHz Ês / Noc: +4.0dB UTRA Cell 2 Ioc: -80.0dBm/1.28MHz Îor / Ioc: +8.0dB PCCPCH_Ec/Ior: -4.77dB DwPCH_Ec/Ior: 0dB |  |  | During T1: 0dB 0dB 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB 0dB 0dB |  |  |  |  |  |  | During T1: E-UTRA Cell 1 Noc: -98.0dBm/15kHz Ês / Noc: +4.00dB UTRA Cell 2 Ioc: -80.0dBm/1.28MHz Îor / Ioc: -infinity PCCPCH_Ec/Ior: -4.77dB DwPCH_Ec/Ior: 0dB During T2: E-UTRA Cell 1 Noc: -98.0dBm/15kHz Ês / Noc: +4.0dB UTRA Cell 2 Ioc: -80.0dBm/1.28MHz Îor / Ioc: +8.0dB PCCPCH_Ec/Ior: -4.77dB DwPCH_Ec/Ior: 0dB |  |
| 6.3.5_2 E-UTRA TDD RRC connection release redirection to UTRA TDD |  |  | Same as 6.3.5 |  |  | Same as 6.3.5 |  |  |  |  |  |  | Same as 6.3.5 |  |
| 6.3.6 E-UTRA FDD RRC connection release redirection to UTRA TDD |  |  | Same as 6.3.5 |  |  | Same as 6.3.5 |  |  |  |  |  |  | Same as 6.3.5 |  |
| 6.3.6_2 E-UTRA FDD RRC connection release redirection to UTRA TDD |  |  | Same as 6.3.5 |  |  | Same as 6.3.5 |  |  |  |  |  |  | Same as 6.3.5 |  |
| 6.3.7 E-UTRA TDD RRC connection release redirection to UTRA TDD without SI provided |  |  | Same as 6.3.5 |  |  | Same as 6.3.5 |  |  |  |  |  |  | Same as 6.3.5 |  |
| 6.3.7_2 E-UTRA TDD RRC connection release redirection to UTRA TDD without SI provided |  |  | Same as 6.3.5 |  |  | Same as 6.3.5 |  |  |  |  |  |  | Same as 6.3.5 |  |
| 6.3.8 E-UTRA FDD RRC connection release redirection to UTRA TDD without SI provided |  |  | Same as 6.3.5 |  |  | Same as 6.3.5 |  |  |  |  |  |  | Same as 6.3.5 |  |
| 6.3.8_2 E-UTRA FDD RRC connection release redirection to UTRA TDD without SI provided |  |  | Same as 6.3.5 |  |  | Same as 6.3.5 |  |  |  |  |  |  | Same as 6.3.5 |  |
| 6.3.9 Redirection from E-UTRAN FDD to UTRAN FDD without System Information |  |  | Same as 6.3.1 |  |  | Same as 6.3.1 |  |  |  |  |  |  | Same as 6.3.1 |  |
| 6.3.9_2 Redirection from E-UTRAN FDD to UTRAN FDD without System Information |  |  | Same as 6.3.1 |  |  | Same as 6.3.1 |  |  |  |  |  |  | Same as 6.3.1 |  |
| 6.3.10 Redirection from E-UTRAN FDD to GERAN when System Information is not provided |  |  | Same as 6.3.3 |  |  | Same as 6.3.3 |  |  |  |  |  |  | Same as 6.3.3 |  |
| 6.3.10_2 Redirection from E-UTRAN FDD to GERAN when System Information is not provided |  |  | Same as 6.3.3 |  |  | Same as 6.3.3 |  |  |  |  |  |  | Same as 6.3.3 |  |
| 6.3.11 Redirection from E-UTRAN TDD to GERAN when System Information is not provided |  |  | Same as 6.3.3 |  |  | Same as 6.3.3 |  |  |  |  |  |  | Same as 6.3.3 |  |
| 6.3.11_2 Redirection from E-UTRAN TDD to GERAN when System Information is not provided |  |  | Same as 6.3.3 |  |  | Same as 6.3.3 |  |  |  |  |  |  | Same as 6.3.3 |  |
| 6.3.12 E-UTRAN TDD RRC connection release redirection to UTRAN FDD without SI provided |  |  | Same as 6.3.1 |  |  | Same as 6.3.1 |  |  |  |  |  |  | Same as 6.3.1 |  |
| 6.3.12_2 E-UTRAN TDD RRC connection release redirection to UTRAN FDD without SI provided |  |  | Same as 6.3.1 |  |  | Same as 6.3.1 |  |  |  |  |  |  | Same as 6.3.1 |  |
| 7.1.1 E-UTRAN FDD - UE Transmit Timing Accuracy |  |  | Test 1 (10MHz Ch BW): Uplink timing: ±12Ts Max step size Tq: 3.5Ts Min adjust rate: 7Ts Max adjust rate: 3.5Ts Ês / Noc: +3.00dB Test 2 (10MHz Ch BW): Uplink timing: ±12Ts Ês / Noc: +3.00dB Test 3: (1.4MHz Ch BW) Uplink timing: ±24Ts Max step size Tq: 17.5Ts Max adjust rate: 17.5Ts Ês / Noc: +3.00dB |  |  | ±3Ts +0.5Ts -3.6Ts +1.1Ts +0.3dB ±3Ts +0.3dB ±3Ts +0.5Ts +1.1Ts +0.3dB |  |  |  |  |  |  | Test 1 (10MHz Ch BW): Uplink timing: ±15Ts Max step size Tq: 4.0T Min adjust rate: 3.4Ts Max adjust rate: 4.6Ts Ês / Noc: +3.30dB Test 2 (10MHz Ch BW): Uplink timing: ±15Ts Ês / Noc: +3.30dB Test 3: (1.4MHz Ch BW) Uplink timing: ±27Ts Max step size Tq: 18Ts Max adjust rate: 18.6Ts Ês / Noc: +3.30dB |  |
| 7.1.1_1 E-UTRAN FDD - UE Transmit Timing Accuracy (Non DRx UE) |  |  | Test 1 (10MHz Ch BW): Uplink timing: ±12Ts Max step size Tq: 3.5Ts Min adjust rate: 7Ts Max adjust rate: 3.5Ts Ês / Noc: +3.00dB Test 2 not applicable Test 3: (1.4MHz Ch BW) Uplink timing: ±24Ts Max step size Tq: 17.5Ts Max adjust rate: 17.5Ts Ês / Noc: +3.00dB |  |  | ±3Ts +0.5Ts -3.6Ts +1.1Ts +0.3dB ±3Ts +0.5Ts +1.1Ts +0.3dB |  |  |  |  |  |  | Test 1 (10MHz Ch BW): Uplink timing: ±15Ts Max step size Tq: 4Ts Min adjust rate: 3.4Ts Max adjust rate: 4.6Ts Ês / Noc: +3.30dB Test 2 not applicable Test 3: (1.4MHz Ch BW) Uplink timing: ±27Ts Max step size Tq: 18Ts Max adjust rate: 18.6Ts Ês / Noc: +3.30dB |  |
| 7.1.1_2 E-UTRAN FDD - UE Transmit Timing Accuracy for UE category 1bis |  |  | Same as 7.1.1 |  |  | Same as 7.1.1 |  |  |  |  |  |  | Same as 7.1.1 |  |
| 7.1.2 E-UTRAN TDD - UE Transmit Timing Accuracy |  |  | Test 1 (10MHz Ch BW): Uplink timing: (624 ±12) x Ts Max step size Tq: 3.5Ts Min adjust rate: 7Ts Max adjust rate: 3.5Ts Ês / Noc: +3.00dB Test 2 (10MHz Ch BW): Uplink timing: (624 ±12) x Ts Ês / Noc: +3.00dB Test 3: (1.4MHz Ch BW) Uplink timing: (624 ±24) x Ts Max step size Tq: 17.5Ts Max adjust rate: 17.5Ts Ês / Noc: +3.00dB |  |  | ±3Ts +0.5Ts -3.6Ts +1.1Ts +0.3dB ±3Ts +0.3dB ±3Ts +0.5Ts +1.1Ts +0.3dB |  |  |  |  |  |  | Test 1 (10MHz Ch BW): Uplink timing: (624 ±15) x Ts Max step size Tq: 4Ts Min adjust rate: 3.4Ts Max adjust rate: 4.6Ts Ês / Noc: +3.30dB Test 2 (10MHz Ch BW): Uplink timing: (624 ±15) x Ts Ês / Noc: +3.30dB Test 3: (1.4MHz Ch BW) Uplink timing: (624 ±27) x Ts Max step size Tq: 18Ts Max adjust rate: 18.6Ts Ês / Noc: +3.30dB |  |
| 7.1.2_1 E-UTRAN TDD - UE Transmit Timing Accuracy (Non DRx UE) |  |  | Test 1 (10MHz Ch BW): Uplink timing: (624 ±12) x Ts Max step size Tq: 3.5Ts Min adjust rate: 7Ts Max adjust rate: 3.5Ts Ês / Noc: +3.00dB Test 2 not applicable Test 3: (1.4MHz Ch BW) Uplink timing: (624 ±24) x Ts Max step size Tq: 17.5Ts Max adjust rate: 17.5Ts Ês / Noc: +3.00dB |  |  | ±3Ts +0.5Ts -3.6Ts +1.1Ts +0.3dB ±3Ts +0.5Ts +1.1Ts +0.3dB |  |  |  |  |  |  | Test 1 (10MHz Ch BW): Uplink timing: (624 ±15) x Ts Max step size Tq: 4Ts Min adjust rate: 3.4Ts Max adjust rate: 4.6Ts Ês / Noc: +3.30dB Test 2 not applicable Test 3: (1.4MHz Ch BW) Uplink timing: (624 ±27) x Ts Max step size Tq: 18Ts Max adjust rate: 18.6Ts Ês / Noc: +3.30dB |  |
| 7.1.2_2 E-UTRAN TDD - UE Transmit Timing Accuracy for UE category 1bis |  |  | Same as 7.1.2 |  |  | Same as 7.1.2 |  |  |  |  |  |  | Same as 7.1.2 |  |
| 7.1.3 E-UTRAN FDD - UE Transmit Timing Accuracy Tests for SCell |  |  | Test 1: Uplink timing: ±12Ts Max step size Tq: 3.5Ts Min adjust rate: 7Ts Max adjust rate: 3.5Ts Ês1 / Noc1: +3.00dB Ês2 / Noc2: +3.00dB Test 2: Uplink timing: ±12Ts Ês1 / Noc1: +3.00dB Ês2 / Noc2: +3.00dB |  |  | ±3Ts +0.5Ts -3.6Ts +1.1Ts +0.3dB +0.3dB ±3Ts +0.3dB +0.3dB |  |  |  |  |  |  | Test 1: Uplink timing: ±15Ts Max step size Tq: 4Ts Min adjust rate: 3.4Ts Max adjust rate: 4.6Ts Ês1 / Noc1: +3.30dB Ês2 / Noc2: +3.30dB Test 2: Uplink timing: ±15Ts Ês1 / Noc1: +3.30dB Ês2 / Noc2: +3.30dB |  |
| 7.1.3_1 E-UTRAN FDD - UE Transmit Timing Accuracy Tests for SCell (Release 12 and forward) |  |  | Test 1: Uplink timing: ±12Ts Max step size Tq: 3.5Ts Min adjust rate: 7Ts Max adjust rate: 3.5Ts Ês1 / Noc1: +3.00dB Ês2 / Noc2: +3.00dB Test 2 and Test 3: Uplink timing: ±12Ts Ês1 / Noc1: +3.00dB Ês2 / Noc2: +3.00dB |  |  | ±3Ts +0.5Ts -3.6Ts +1.1Ts +0.3dB +0.3dB ±3Ts +0.3dB +0.3dB |  |  |  |  |  |  | Test 1: Uplink timing: ±15Ts Max step size Tq: 4Ts Min adjust rate: 3.4Ts Max adjust rate: 4.6Ts Ês1 / Noc1: +3.30dB Ês2 / Noc2: +3.30dB Test 2 and Test 3: Uplink timing: ±15Ts Ês1 / Noc1: +3.30dB Ês2 / Noc2: +3.30dB |  |
| 7.1.4 E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell |  |  | Test 1: Uplink timing: (624 ±12) x Ts Max step size Tq: 3.5Ts Min adjust rate: 7Ts Max adjust rate: 3.5Ts Ês1 / Noc1: +3.00dB Ês2 / Noc2: +3.00dB Test 2: Uplink timing: (624 ±12) x Ts Ês1 / Noc1: +3.00dB Ês2 / Noc2: +3.00dB |  |  | ±3Ts +0.5Ts -3.6Ts +1.1Ts +0.3dB +0.3dB ±3Ts +0.3dB +0.3dB |  |  |  |  |  |  | Test 1: Uplink timing: (624 ±15) x Ts Max step size Tq: 4Ts Min adjust rate: 3.4Ts Max adjust rate: 4.6Ts Ês1 / Noc1: +3.30dB Ês2 / Noc2: +3.30dB Test 2: Uplink timing: (624 ±15) x Ts Ês1 / Noc1: +3.30dB Ês2 / Noc2: +3.30dB |  |
| 7.1.4_1 E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell (Release 12 and forward) |  |  | Test 1: Uplink timing: (624 ±12) x Ts Max step size Tq: 3.5Ts Min adjust rate: 7Ts Max adjust rate: 3.5Ts Ês1 / Noc1: +3.00dB Ês2 / Noc2: +3.00dB Test 2 and Test: Uplink timing: (624 ±12) x Ts Ês1 / Noc1: +3.00dB Ês2 / Noc2: +3.00dB |  |  | ±3Ts +0.5Ts -3.6Ts +1.1Ts +0.3dB +0.3dB ±3Ts +0.3dB +0.3dB |  |  |  |  |  |  | Test 1: Uplink timing: (624 ±15) x Ts Max step size Tq: 4Ts Min adjust rate: 3.4Ts Max adjust rate: 4.6Ts Ês1 / Noc1: +3.30dB Ês2 / Noc2: +3.00dB Test 2 and Test 3: Uplink timing: (624 ±15) x Ts Ês1 / Noc1: +3.30dB Ês2 / Noc2: +3.30dB |  |
| 7.1.4A E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell for 20 MHz +10 MHz bandwidth |  |  | Same as 7.1.4 |  |  | Same as 7.1.4 |  |  |  |  |  |  | Same as 7.1.4 |  |
| 7.1.5 E-UTRAN FDD - UE Transmit Timing Accuracy Tests for 5MHz Bandwidth |  |  | Test 1 (5MHz Ch BW): Uplink timing: ±12Ts Max step size Tq: 5.5Ts Min adjust rate: 11Ts in 1.57s Max adjust rate: 5.5Ts Ês / Noc: +3.00dB |  |  | ±3Ts +0.5Ts -5.3Ts +1.1Ts +0.3dB |  |  |  |  |  |  | Test 1 (5MHz Ch BW): Uplink timing: ±15Ts Max step size Tq: 6.0Ts Min adjust rate: 5.7Ts in 1.57s Max adjust rate: 6.6Ts Ês / Noc: +3.30dB |  |
| 7.1.6 E-UTRAN FDD - UE Transmit Timing Accuracy Tests for SCell in sTAG |  |  | Test 1: Uplink timing: ±12Ts Max step size Tq: 3.5Ts Min adjust rate: 7Ts Max adjust rate: 3.5Ts Ês1 / Noc1: +3.00dB Ês2 / Noc2: +3.00dB Test 2: Uplink timing: ±12Ts Ês1 / Noc1: +3.00dB Ês2 / Noc2: +3.00dB |  |  | ±3Ts +0.5Ts -3.6Ts +1.1Ts +0.3dB +0.3dB ±3Ts +0.3dB +0.3dB |  |  |  |  |  |  | Test 1: Uplink timing: ±15Ts Max step size Tq: 4.0Ts Min adjust rate: 3.4Ts Max adjust rate: 4.6Ts Ês1 / Noc1: +3.30dB Ês2 / Noc2: +3.30dB Test 2: Uplink timing: ±15Ts Ês1 / Noc1: +3.30dB Ês2 / Noc2: +3.30dB |  |
| 7.1.7 E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG |  |  | Test 1: Uplink timing: (624 ±12) x Ts Max step size Tq: 3.5Ts Min adjust rate: 7Ts Max adjust rate: 3.5Ts Ês1 / Noc1: +3.00dB Ês2 / Noc2: +3.00dB Test 2: Uplink timing: (624 ±12) x Ts Ês1 / Noc1: +3.00dB Ês2 / Noc2: +3.00dB |  |  | ±3Ts +0.5Ts -3.6Ts +1.1Ts +0.3dB +0.3dB ±3Ts +0.3dB +0.3dB |  |  |  |  |  |  | Test 1: Uplink timing: (624 ±15) x Ts Max step size Tq: 4.0Ts Min adjust rate: 3.4Ts Max adjust rate: 4.6Ts Ês1 / Noc1: +3.30dB Ês2 / Noc2: +3.30dB Test 2: Uplink timing: (624 ±15) x Ts Ês1 / Noc1: +3.30dB Ês2 / Noc2: +3.30dB |  |
| 7.1.7A E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +20MHz bandwidth |  |  | Sams as 7.1.7 |  |  | Sams as 7.1.7 |  |  |  |  |  |  | Sams as 7.1.7 |  |
| 7.1.7B E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +10MHz bandwidth |  |  | Sams as 7.1.7 |  |  | Sams as 7.1.7 |  |  |  |  |  |  | Sams as 7.1.7 |  |
| 7.1.10 E-UTRAN FDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA |  |  | Test 1: Same as 7.1.1 Test 1 Test 2 & 3: Same as 7.1.1 Test 2 |  |  | Test 1: Same as 7.1.1 Test 1 Test 2 & 3: Same as 7.1.1 Test 2 |  |  |  |  |  |  | Test 1: Same as 7.1.1 Test 1 Test 2 & 3: Same as 7.1.1 Test 2 |  |
| 7.1.11 E-UTRAN HD-FDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA |  |  | Test 1: Same as 7.1.1 Test 1 Test 2 & 3: Same as 7.1.1 Test 2 |  |  | Test 1: Same as 7.1.1 Test 1 Test 2 & 3: Same as 7.1.1 Test 2 |  |  |  |  |  |  | Test 1: Same as 7.1.1 Test 1 Test 2 & 3: Same as 7.1.1 Test 2 |  |
| 7.1.12 E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA |  |  | Test 1: Same as 7.1.1 Test 1 Test 2 & 3: Same as 7.1.1 Test 2 |  |  | Test 1: Same as 7.1.1 Test 1 Test 2 & 3: Same as 7.1.1 Test 2 |  |  |  |  |  |  | Test 1: Same as 7.1.1 Test 1 Test 2 & 3: Same as 7.1.1 Test 2 |  |
| 7.1.13 3DL/3UL TDD CA UE Transmit Timing Accuracy Tests for 2 SCells |  |  | Test 1: Uplink timing: (624 ±12) x Ts Max step size Tq: 5.5Ts for 5MHz BW 3.5Ts for 10 MHz BW Min adjust rate: 7Ts Max adjust rate: 5.5Ts for 5MHz BW 3.5Ts for 10 MHz BW Ês1 / Noc1: +3.00dB Ês2 / Noc2: +3.00dB Ês3 / Noc3: +3.00dB Test 2: Uplink timing: (624 ±12) x Ts Ês1 / Noc1: +3.00dB Ês2 / Noc2: +3.00dB Ês3 / Noc3: +3.00dB |  |  | ±3Ts +0.5Ts +0.5Ts -3.6Ts +1.1Ts +1.1Ts +0.3dB +0.3dB +0.3dB ±3Ts +0.3dB +0.3dB +0.3dB |  |  |  |  |  |  | Test 1: Uplink timing: (624 ±15) x Ts Max step size Tq: 6.0Ts for 5MHz BW 4.0Ts for 10 MHz BW Min adjust rate: 3.4Ts Max adjust rate: 6.6Ts for 5MHz BW 4.6Ts for 10 MHz BW Ês1 / Noc1: +3.30dB Ês2 / Noc2: +3.30dB Ês3 / Noc3: +3.30dB Test 2: Uplink timing: (624 ±15) x Ts Ês1 / Noc1: +3.30dB Ês2 / Noc2: +3.30dB Ês3 / Noc3: +3.30dB |  |
| 7.1.14 E-UTRAN FDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB |  |  | Uplink timing: ±48Ts Max step size Tq: 17.5Ts Max adjust rate: 17.5Ts Ês / Noc: -12.00dB |  |  | ±3Ts +0.5Ts +1.1Ts(f > 1GHz) +1.7Ts(f ≤ 1GHz) +0.3dB |  |  |  |  |  |  | Uplink timing: ±51Ts Max step size Tq: 18Ts Max adjust rate: 18.6Ts(f > 1GHz) Max adjust rate: 19.2Ts(f ≤ 1GHz) Ês / Noc: -11.70dB |  |
| 7.1.15 E-UTRAN HD-FDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB |  |  | Same as 7.1.14 |  |  | Same as 7.1.14 |  |  |  |  |  |  | Same as 7.1.14 |  |
| 7.1.16 E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB |  |  | Same as 7.1.14 |  |  | Same as 7.1.14 |  |  |  |  |  |  | Same as 7.1.14 |  |
| 7.1.17 E-UTRAN HD-FDD - UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage |  |  | Test 1 Uplink timing: ±80Ts Max step size: 58.33Ts Min adjust rate: 7Ts Max adjust rate: 58.33Ts Ês / Noc: +4.00dB |  |  | ±3Ts +0.5Ts -3.6Ts for carrier frequency under 1 GHz -6.6Ts for carrier frequency over 1 GHz +1.1Ts for carrier frequency under 1 GHz +1.7 Ts for carrier frequency over 1 GHz +0.3dB |  |  |  |  |  |  | Test 1 Uplink timing: ±83Ts Max step size: 58.83T Min adjust rate: 3.4Ts for carrier frequency under 1 GHz 0.4Ts for carrier frequency over 1 GHz Max adjust rate: 59.43Ts for carrier frequency under 1 GHz 60.03Ts for carrier frequency over 1 GHz Ês / Noc: +4.30dB |  |
| 7.1.18 E-UTRAN HD-FDD - UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under enhanced coverage |  |  | Test 1&Test 2 Uplink timing: ±80Ts Test 1 Uplink timing: 0Ts (during ongoing repetition) Ês / Noc: +4.00dB |  |  | ±3Ts ±3Ts +0.3dB |  |  |  |  |  |  | Test 1&Test 2 Uplink timing: ±83Ts Test 1 Uplink timing: ±3Ts (during ongoing repetition) Ês / Noc: +4.30dB |  |
| 7.1.21 E-UTRAN FDD - UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA |  |  | Test 1: Same as 7.1.1 Test 1 Test 2 & 3: Same as 7.1.1 Test 2 |  |  | Test 1: Same as 7.1.1 Test 1 Test 2 & 3: Same as 7.1.1 Test 2 |  |  |  |  |  |  | Test 1: Same as 7.1.1 Test 1 Test 2 & 3: Same as 7.1.1 Test 2 |  |
| 7.1.22 E-UTRAN HD-FDD - UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA |  |  | Test 1: Same as 7.1.1 Test 1 Test 2 & 3: Same as 7.1.1 Test 2 |  |  | Test 1: Same as 7.1.1 Test 1 Test 2 & 3: Same as 7.1.1 Test 2 |  |  |  |  |  |  | Test 1: Same as 7.1.1 Test 1 Test 2 & 3: Same as 7.1.1 Test 2 |  |
| 7.1.23 E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA |  |  | Test 1: Same as 7.1.1 Test 1 Test 2 & 3: Same as 7.1.1 Test 2 |  |  | Test 1: Same as 7.1.1 Test 1 Test 2 & 3: Same as 7.1.1 Test 2 |  |  |  |  |  |  | Test 1: Same as 7.1.1 Test 1 Test 2 & 3: Same as 7.1.1 Test 2 |  |
| 7.1.24 E-UTRAN FDD - UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB |  |  | Uplink timing: ±40Ts Max step size Tq: 17.5Ts Max adjust rate: 17.5Ts Ês / Noc: -12.00dB |  |  | ±3Ts +0.5Ts +1.1Ts(f > 1GHz) +1.7Ts(f ≤ 1GHz) +0.3dB |  |  |  |  |  |  | Uplink timing: ±43Ts Max step size Tq: 18Ts Max adjust rate: 18.6Ts(f > 1GHz) Max adjust rate: 19.2Ts(f ≤ 1GHz) Ês / Noc: -11.70dB |  |
| 7.1.27 E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage |  |  | Same as 7.1.17 |  |  | Same as 7.1.17 |  |  |  |  |  |  | Same as 7.1.17 |  |
| 7.1.28 E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Category NB1 UE In-band mode under enhanced coverage |  |  | Same as 7.1.18 |  |  | Same as 7.1.18 |  |  |  |  |  |  | Same as 7.1.18 |  |
| 7.2.1 E-UTRAN FDD - UE Timing Advanced Adjustment Accuracy |  |  | Timing Advance Adjustment: ±4Ts |  |  | 0.5TS |  |  |  |  |  |  | Timing Advance Adjustment: ±4.5Ts |  |
| 7.2.1_2 E-UTRAN FDD - UE Timing Advance Adjustment Accuracy for UE category 1bis |  |  | Same as 7.2.1 |  |  | Same as 7.2.1 |  |  |  |  |  |  | Same as 7.2.1 |  |
| 7.2.2 E-UTRAN TDD - UE Timing Advance Adjustment Accuracy |  |  | Same as 7.2.1 |  |  | Same as 7.2.1 |  |  |  |  |  |  | Same as 7.2.1 |  |
| 7.2.2_2 E-UTRAN TDD - UE Timing Advance Adjustment Accuracy for UE category 1bis |  |  | Same as 7.2.1 |  |  | Same as 7.2.1 |  |  |  |  |  |  | Same as 7.2.1 |  |
| 7.2.3 E-UTRAN FDD - UE Timing Advance Adjustment Accuracy Test for 5MHz Bandwidth |  |  | Same as 7.2.1 |  |  | Same as 7.2.1 |  |  |  |  |  |  | Same as 7.2.1 |  |
| 7.2.4 E-UTRAN FDD - UE Timing Advance Adjustment Accuracy Test for SCell in sTAG |  |  | Same as 7.2.1 |  |  | Same as 7.2.1 |  |  |  |  |  |  | Same as 7.2.1 |  |
| 7.2.5 E-UTRAN TDD - UE Timing Advance Adjustment Accuracy Test for SCell in sTAG |  |  | Same as 7.2.1 |  |  | Same as 7.2.1 |  |  |  |  |  |  | Same as 7.2.1 |  |
| 7.2.5A E-UTRAN TDD - UE Timing Advance Adjustment Accuracy Test for SCell in sTAG for 20MHz +20MHz bandwidth |  |  | Same as 7.2.1 |  |  | Same as 7.2.1 |  |  |  |  |  |  | Same as 7.2.1 |  |
| 7.2.5B E-UTRAN TDD - UE Timing Advance Adjustment Accuracy Test for SCell in sTAG for 20MHz +10MHz bandwidth |  |  | Same as 7.2.1 |  |  | Same as 7.2.1 |  |  |  |  |  |  | Same as 7.2.1 |  |
| 7.2.6 E-UTRAN FDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA |  |  | Same as 7.2.1 |  |  | Same as 7.2.1 |  |  |  |  |  |  | Same as 7.2.1 |  |
| 7.2.7 E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA |  |  | Same as 7.2.1 |  |  | Same as 7.2.1 |  |  |  |  |  |  | Same as 7.2.1 |  |
| 7.2.8 E-UTRAN TDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA |  |  | Same as 7.2.1 |  |  | Same as 7.2.1 |  |  |  |  |  |  | Same as 7.2.1 |  |
| 7.2.9 HD-FDD UE Timing Advance Adjustment Accuracy Test for Category NB1 UE in Standalone Mode under Enhance Coverage |  |  | Timing Advance Adjustment: ±13.33Ts |  |  | 0.5 TS |  |  |  |  |  |  | Timing Advance Adjustment: ±13.83Ts |  |
| 7.2.10 E-UTRAN FDD UE Timing Advance Adjustment Accuracy Test in CEModeB |  |  | Same as 7.2.1 |  |  | Same as 7.2.1 |  |  |  |  |  |  | Same as 7.2.1 |  |
| 7.2.11 E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test in CEModeB |  |  | Same as 7.2.1 |  |  | Same as 7.2.1 |  |  |  |  |  |  | Same as 7.2.1 |  |
| 7.2.12 E-UTRAN TDD UE Timing Advance Adjustment Accuracy Test in CEModeB |  |  | Same as 7.2.1 |  |  | Same as 7.2.1 |  |  |  |  |  |  | Same as 7.2.1 |  |
| 7.2.13 E-UTRAN FDD - UE Timing Advance Adjustment Accuracy Test for sTTI and Short Processing Time |  |  | Same as 7.2.1 |  |  | Same as 7.2.1 |  |  |  |  |  |  | Same as 7.2.1 |  |
| 7.2.14 E-UTRAN TDD - UE Timing Advance Adjustment Accuracy Test for sTTI and Short Processing Time |  |  | Same as 7.2.1 |  |  | Same as 7.2.1 |  |  |  |  |  |  | Same as 7.2.1 |  |
| 7.2.15 E-UTRAN TDD - TDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Enhanced Coverage |  |  | Same as 7.2.9 |  |  | Same as 7.2.9 |  |  |  |  |  |  | Same as 7.2.9 |  |
| 7.3.1 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync |  |  | SNRs as specified |  |  | 0.6dB (Subtests 1&2) 0.8dB (Subtest 3) 0.9dB (Subtest 4) |  |  |  |  |  |  | During T1: Formula: SNR + TT During T2: Formula: SNR + TT During T3: Formula: SNR - TT |  |
| 7.3.1_1 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync with 4 Rx antenna ports |  |  | SNRs as specified |  |  | Same as 7.3.1 |  |  |  |  |  |  | Same as 7.3.1 |  |
| 7.3.2 E-UTRAN FDD Radio Link Monitoring Test for In-sync |  |  | SNRs as specified |  |  | 0.8dB (Subtest 1) 0.9dB (Subtest 2) |  |  |  |  |  |  | During T1: Formula: SNR + TT During T2: Formula: SNR + TT During T3: Formula: SNR - TT During T4: Formula: SNR - TT S During T5: Formula: SNR + TT |  |
| 7.3.2_1 E-UTRAN FDD Radio Link Monitoring Test for In-sync with 4 Rx antenna ports |  |  | SNRs as specified |  |  | Same as 7.3.2 |  |  |  |  |  |  | Same as 7.3.2 |  |
| 7.3.3 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync |  |  | SNRs as specified |  |  | Same as 7.3.1 |  |  |  |  |  |  | Same as 7.3.1 |  |
| 7.3.3_1 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync with 4 Rx antenna ports |  |  | SNRs as specified |  |  | Same as 7.3.1 |  |  |  |  |  |  | Same as 7.3.1 |  |
| 7.3.4 E-UTRAN TDD Radio Link Monitoring Test for In-sync |  |  | SNRs as specified |  |  | Same as 7.3.2 |  |  |  |  |  |  | Same as 7.3.2 |  |
| 7.3.5 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync in DRX |  |  | SNRs as specified |  |  | 0.9dB (Subtest 1) 0.6dB (Subtest 2) |  |  |  |  |  |  | Same as 7.3.1 |  |
| 7.3.5_1 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync in DRX with 4 Rx antenna ports |  |  | SNRs as specified |  |  | Same as 7.3.5 |  |  |  |  |  |  | Same as 7.3.5 |  |
| 7.3.6 E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX |  |  | SNRs as specified |  |  | 0.6dB |  |  |  |  |  |  | Same as 7.3.2 |  |
| 7.3.6_1 E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX with 4 Rx antenna ports |  |  | SNRs as specified |  |  | Same as 7.3.6 |  |  |  |  |  |  | Same as 7.3.6 |  |
| 7.3.7 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX |  |  | SNRs as specified |  |  | 0.9dB (Subtest 1) 0.6dB (Subtest 2) |  |  |  |  |  |  | Same as 7.3.1 |  |
| 7.3.7_1 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX with 4 Rx antenna ports |  |  | SNRs as specified |  |  | Same as 7.3.7 |  |  |  |  |  |  | Same as 7.3.7 |  |
| 7.3.8 E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX |  |  | SNRs as specified |  |  | 0.6dB |  |  |  |  |  |  | Same as 7.3.2 |  |
| 7.3.8_! E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX with 4 Rx antenna ports |  |  | SNRs as specified |  |  | Same as 7.3.8 |  |  |  |  |  |  | Same as 7.3.8 |  |
| 7.3.9 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with Non MBSFN ABS (eICIC) |  |  | SNRs as specified |  |  | 0.9dB for cell 1 (TT1) 0.2dB for cell 2 (TT2) |  |  |  |  |  |  | During T1: Cell 1 Formula: SNR + TT1 Cell 2 Formula: SNR - TT2 During T2: Cell 1 Formula: SNR + TT1 Cell 2 Formula: SNR - TT2 During T3: Cell 1 Formula: SNR - TT1 Cell 2 Formula: SNR + TT2 Formuale for Test Tolerance values: TT1: Ês1/Noc uncertainty TT2: (Ês2/Ês1 uncertainty - Ês1/Noc uncertainty) |  |
| 7.3.10 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with Non MBSFN ABS (eICIC) |  |  | Same as 7.3.9 |  |  | Same as 7.3.9 |  |  |  |  |  |  | Same as 7.3.9 |  |
| 7.3.11 E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with Non MBSFN ABS (eICIC) |  |  | SNRs as specified |  |  | 0.9dB for cell 1 (TT1) 0.2dB for cell 2(TT2) |  |  |  |  |  |  | During T1: Cell 1 Formula: SNR + TT1 Cell 2 Formula: SNR - TT2 During T2: Cell 1 Formula: SNR + TT1 Cell 2 Formula: SNR - TT2 During T3: Cell 1 Formula: SNR - TT1 Cell 2 Formula: SNR + TT2 During T4: Cell 1 Formula: SNR - TT1 Cell 2 Formula: SNR + TT2 During T5: Cell 1 Formula: SNR + TT1 Cell 2 Formula: SNR - TT2 Formuale for Test Tolerance values: TT1: Ês1/Noc uncertainty TT2: (Ês2/Ês1 uncertainty - Ês1/Noc uncertainty) |  |
| 7.3.12 E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with Non MBSFN ABS (eICIC) |  |  | Same as 7.3.11 |  |  | Same as 7.3.11 |  |  |  |  |  |  | Same as 7.3.11 |  |
| 7.3.13 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS (eICIC) |  |  | Same as 7.3.9 |  |  | Same as 7.3.9 |  |  |  |  |  |  | Same as 7.3.9 |  |
| 7.3.14 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS (eICIC) |  |  | Same as 7.3.9 |  |  | Same as 7.3.9 |  |  |  |  |  |  | Same as 7.3.9 |  |
| 7.3.15 E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS (eICIC) |  |  | Same as 7.3.11 |  |  | Same as 7.3.11 |  |  |  |  |  |  | Same as 7.3.11 |  |
| 7.3.16 E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS (eICIC) |  |  | Same as 7.3.11 |  |  | Same as 7.3.11 |  |  |  |  |  |  | Same as 7.3.11 |  |
| 7.3.17 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS assistance information and Non MBSFN ABS (feICIC) |  |  | SNRs as specified |  |  |  |  | 0.9dB for cell 1 (TT1) 0.2dB for cell 2 (TT2) 0.2dB for cell 3 (TT3) |  |  |  |  | During T1: Cell 1 Formula: SNR + TT1 Cell 2 Formula: SNR - TT2 Cell 3 Formula: SNR - TT3 During T2: Cell 1 Formula: SNR + TT1 Cell 2 Formula: SNR - TT2 Cell 3 Formula: SNR - TT3 During T3: Cell 1 Formula: SNR - TT1 Cell 2 Formula: SNR + TT2 Cell 3 Formula: SNR + TT3 Formuale for Test Tolerance values: TT1: Ês1/Noc uncertainty TT2: (Ês2/Ês1 uncertainty - Ês1/Noc uncertainty) TT3: (Ês3/Ês1 uncertainty - Ês1/Noc uncertainty) |  |
| 7.3.18 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS assistance information and Non MBSFN ABS (feICIC) |  |  | Same as 7.3.17 |  |  |  |  | Same as 7.3.17 |  |  |  |  | Same as 7.3.17 |  |
| 7.3.19 E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with CRS assistance information and Non MBSFN ABS (feICIC) |  |  | SNRs as specified |  |  |  |  | 0.9dB for cell 1 (TT1) 0.2dB for cell 2(TT2) 0.2dB for cell 3 (TT3) |  |  |  |  | During T1: Cell 1 Formula: SNR + TT1 Cell 2 Formula: SNR - TT2 Cell 3 Formula: SNR - TT3 During T2: Cell 1 Formula: SNR + TT1 Cell 2 Formula: SNR - TT2 Cell 3 Formula: SNR - TT3 During T3: Cell 1 Formula: SNR - TT1 Cell 2 Formula: SNR + TT2 Cell 3 Formula: SNR + TT3 During T4: Cell 1 Formula: SNR - TT1 Cell 2 Formula: SNR + TT2 Cell 3 Formula: SNR + TT3 During T5: Cell 1 Formula: SNR + TT1 Cell 2 Formula: SNR - TT2 Cell 3 Formula: SNR - TT3 Formuale for Test Tolerance values: TT1: Ês1/Noc uncertainty TT2: (Ês2/Ês1 uncertainty - Ês1/Noc uncertainty) TT3: (Ês3/Ês1 uncertainty - Ês1/Noc uncertainty) |  |
| 7.3.20 E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with CRS assistance information and Non MBSFN ABS (feICIC) |  |  | Same as 7.3.19 |  |  |  |  | Same as 7.3.19 |  |  |  |  | Same as 7.3.19 |  |
| 7.3.21 E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with CRS assistance information and MBSFN ABS (feICIC) |  |  | Same as 7.3.19 |  |  |  |  | Same as 7.3.19 |  |  |  |  | Same as 7.3.19 |  |
| 7.3.22 E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with CRS assistance information and MBSFN ABS (feICIC) |  |  | Same as 7.3.19 |  |  |  |  | Same as 7.3.19 |  |  |  |  | Same as 7.3.19 |  |
| 7.3.23 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync for 5MHz Bandwidth |  |  | SNRs as specified |  |  | Same as 7.3.1 |  |  |  |  |  |  | Same as 7.3.1 |  |
| 7.3.23_1 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync for 5MHz Bandwidth with 4 Rx antenna ports |  |  | SNRs as specified |  |  | Same as 7.3.23 |  |  |  |  |  |  | Same as 7.3.23 |  |
| 7.3.24 E-UTRAN FDD Radio Link Monitoring Test for In-sync for 5MHz Bandwidth |  |  | SNRs as specified |  |  | 0.9dB |  |  |  |  |  |  | Same as 7.3.2 |  |
| 7.3.24_1 E-UTRAN FDD Radio Link Monitoring Test for In-sync for 5MHz Bandwidth with 4 Rx antenna ports |  |  | SNRs as specified |  |  | Same as 7.3.24 |  |  |  |  |  |  | Same as 7.3.24 |  |
| 7.3.25 E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX for 5MHz Bandwidth |  |  | SNRs as specified |  |  | 0.6dB |  |  |  |  |  |  | Same as 7.3.2 |  |
| 7.3.25_1 E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX for 5MHz Bandwidth with 4 Rx antenna ports |  |  | SNRs as specified |  |  | Same as 7.3.25 |  |  |  |  |  |  | Same as 7.3.25 |  |
| 7.3.26 E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for UE category 0 |  |  | SNRs as specified |  |  | 0.9dB |  |  |  |  |  |  | Same as 7.3.1 |  |
| 7.3.26_2 E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for UE Category 1bis |  |  | SNRs as specified |  |  | Same as 7.3.26 |  |  |  |  |  |  | Same as 7.3.1 |  |
| 7.3.27 E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync for UE category 0 |  |  | SNRs as specified |  |  | Same as 7.3.26 |  |  |  |  |  |  | Same as 7.3.2 |  |
| 7.3.27_2 E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync for UE Category 1bis |  |  | SNRs as specified |  |  | Same as 7.3.26 |  |  |  |  |  |  | Same as 7.3.1 |  |
| 7.3.28 E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0 |  |  | SNRs as specified |  |  | 0.6dB |  |  |  |  |  |  | Same as 7.3.1 |  |
| 7.3.28_2 E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE Category 1bis |  |  | SNRs as specified |  |  | Same as 7.3.28 |  |  |  |  |  |  | Same as 7.3.1 |  |
| 7.3.29 E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE category 0 |  |  | SNRs as specified |  |  | Same as 7.3.28 |  |  |  |  |  |  | Same as 7.3.2 |  |
| 7.3.29_2 E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category 1bis |  |  | SNRs as specified |  |  | Same as 7.3.28 |  |  |  |  |  |  | Same as 7.3.2 |  |
| 7.3.30 E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for UE category 0 |  |  | SNRs as specified |  |  | Same as 7.3.26 |  |  |  |  |  |  | Same as 7.3.1 |  |
| 7.3.31 E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync for UE category 0 |  |  | SNRs as specified |  |  | Same as 7.3.26 |  |  |  |  |  |  | Same as 7.3.2 |  |
| 7.3.32 E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0 |  |  | SNRs as specified |  |  | Same as 7.3.28 |  |  |  |  |  |  | Same as 7.3.1 |  |
| 7.3.33 E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE category 0 |  |  | SNRs as specified |  |  | Same as 7.3.28 |  |  |  |  |  |  | Same as 7.3.2 |  |
| 7.3.34 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for UE category 0 |  |  | SNRs as specified |  |  | Same as 7.3.26 |  |  |  |  |  |  | Same as 7.3.1 |  |
| 7.3.34_2 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for UE Category 1bis |  |  | SNRs as specified |  |  | Same as 7.3.26 |  |  |  |  |  |  | Same as 7.3.1 |  |
| 7.3.35 E-UTRAN TDD Radio Link Monitoring Test for In-sync for UE category 0 |  |  | SNRs as specified |  |  | Same as 7.3.26 |  |  |  |  |  |  | Same as 7.3.2 |  |
| 7.3.35_2 E-UTRAN TDD Radio Link Monitoring Test for In-sync for UE Category 1bis |  |  | SNRs as specified |  |  | Same as 7.3.26 |  |  |  |  |  |  | Same as 7.3.2 |  |
| 7.3.36 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0 |  |  | SNRs as specified |  |  | Same as 7.3.28 |  |  |  |  |  |  | Same as 7.3.1 |  |
| 7.3.36_2 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE Category 1bis |  |  | SNRs as specified |  |  | Same as 7.3.28 |  |  |  |  |  |  | Same as 7.3.1 |  |
| 7.3.37 E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE category 0 |  |  | SNRs as specified |  |  | Same as 7.3.28 |  |  |  |  |  |  | Same as 7.3.2 |  |
| 7.3.37_2 E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE Category 1bis |  |  | SNRs as specified |  |  | Same as 7.3.28 |  |  |  |  |  |  | Same as 7.3.2 |  |
| 7.3.38 E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC |  |  | SNRs as specified |  |  | 0.9dB |  |  |  |  |  |  | Same as 7.3.1 |  |
| 7.3.38_1 E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with 4 Rx antenna ports |  |  | SNRs as specified |  |  | Same as 7.3.38 |  |  |  |  |  |  | Same as 7.3.38 |  |
| 7.3.39 E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in asynchronous DC |  |  | SNRs as specified |  |  | 0.9dB |  |  |  |  |  |  | Same as 7.3.1 |  |
| 7.3.39 E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in asynchronous DC with 4 Rx antenna ports |  |  | SNRs as specified |  |  | Same as 7.3.39 |  |  |  |  |  |  | Same as 7.3.39 |  |
| 7.3.40 E-UTRAN TDD-TDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC |  |  | SNRs as specified |  |  | 0.9dB |  |  |  |  |  |  | Same as 7.3.1 |  |
| 7.3.41 E-UTRAN FDD-FDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity |  |  | SNRs as specified |  |  | 0.6dB |  |  |  |  |  |  | Same as 7.3.2 |  |
| 7.3.42 E-UTRAN FDD-FDD DC Radio Link Monitoring Test for In-sync in DRX in asynchronous DC |  |  | SNRs as specified |  |  | 0.6dB |  |  |  |  |  |  | Same as 7.3.2 |  |
| 7.3.43 E-UTRAN TDD-TDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity |  |  | SNRs as specified |  |  | 0.6dB |  |  |  |  |  |  | Same as 7.3.2 |  |
| 7.3.44 E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in FDD |  |  | SNRs as specified |  |  | 0.9dB |  |  |  |  |  |  | Same as 7.3.1 |  |
| 7.3.45 E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in TDD |  |  | SNRs as specified |  |  | 0.9dB |  |  |  |  |  |  | Same as 7.3.1 |  |
| 7.3.46 E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in FDD |  |  | SNRs as specified |  |  | 0.6dB |  |  |  |  |  |  | Same as 7.3.2 |  |
| 7.3.47 E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in TDD |  |  | SNRs as specified |  |  | 0.6dB |  |  |  |  |  |  | Same as 7.3.2 |  |
| 7.3.48 E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A |  |  | SNRs as specified |  |  | 0.9dB |  |  |  |  |  |  | Same as 7.3.1 |  |
| 7.3.49 E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A |  |  | SNRs as specified |  |  | 0.9dB |  |  |  |  |  |  | Same as 7.3.2 |  |
| 7.3.50: E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-Sync in DRX for UE Category M1 Configured in CEMode A |  |  | SNRs as specified |  |  | 0.6dB |  |  |  |  |  |  | Same as 7.3.1 |  |
| 7.3.51 E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE Category M1 Configured in CEMode A |  |  | SNRs as specified |  |  | 0.6dB |  |  |  |  |  |  | Same as 7.3.2 |  |
| 7.3.52 E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A |  |  | SNRs as specified |  |  | 0.9dB |  |  |  |  |  |  | Same as 7.3.1 |  |
| 7.3.53 E-UTRAN HD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A |  |  | SNRs as specified |  |  | 0.9dB |  |  |  |  |  |  | Same as 7.3.2 |  |
| 7.3.54: E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE Category M1 Configured in CEMode A |  |  | SNRs as specified |  |  | 0.6dB |  |  |  |  |  |  | Same as 7.3.1 |  |
| 7.3.55: E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A |  |  | SNRs as specified |  |  | 0.6dB |  |  |  |  |  |  | Same as 7.3.2 |  |
| 7.3.56:E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A |  |  | SNRs as specified |  |  | 0.9dB |  |  |  |  |  |  | Same as 7.3.1 |  |
| 7.3.57:E-UTRAN TDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A |  |  | SNRs as specified |  |  | 0.9dB |  |  |  |  |  |  | Same as 7.3.2 |  |
| 7.3.58:E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A |  |  | SNRs as specified |  |  | 0.6dB |  |  |  |  |  |  | Same as 7.3.1 |  |
| 7.3.59:E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A |  |  | SNRs as specified |  |  | 0.6dB |  |  |  |  |  |  | Same as 7.3.2 |  |
| 7.3.60 HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage |  |  | SNRs as specified |  |  | 0.6dB |  |  |  |  |  |  | During T1: Formula: SNR + TT During T2: Formula: SNR + TT During T3: Formula: SNR - TT During T4: Formula: SNR + TT |  |
| 7.3.61 HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage |  |  | SNRs as specified |  |  | 0.6dB |  |  |  |  |  |  | Same as 7.3.60 |  |
| 7.3.62 HD-FDD Radio Link Monitoring Test for In-sync with DRX for Category NB1 In-Band mode in Enhanced Coverage |  |  | SNRs as specified Time durations as specified |  |  | 0.6dB 0.28s, T2 only |  |  |  |  |  |  | During T1: Formula: SNR + TT T1 duration unchanged During T2: Formula: SNR - TT Formula: T2 duration -TT During T3: Formula: SNR + TT T3 duration unchanged |  |
| 7.3.63 HD-FDD Radio Link Monitoring Test for In-sync with DRX for Category NB1 In-Band mode in Normal Coverage |  |  | SNRs as specified Time durations as specified |  |  | 0.6dB 0.28s, T2 only |  |  |  |  |  |  | Same as 7.3.62 |  |
| 7.3.64 HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage |  |  | SNRs as specified Time durations as specified |  |  | 0.6dB 0.28s, T2 only |  |  |  |  |  |  | Same as 7.3.62 |  |
| 7.3.65 HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage |  |  | SNRs as specified Time durations as specified |  |  | 0.6dB 0.28s, T2 only |  |  |  |  |  |  | Same as 7.3.62 |  |
| 7.3.66 HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage |  |  | SNRs as specified |  |  | 0.6dB |  |  |  |  |  |  | Same as 7.3.60 |  |
| 7.3.67 HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage |  |  | SNRs as specified |  |  | 0.6dB |  |  |  |  |  |  | Same as 7.3.60 |  |
| 7.3.69 E-UTRAN HD-FDD Early Out-of-sync reporting Test for CE UE in CEMode A |  |  | SNRs as specified |  |  | 0.9dB |  |  |  |  |  |  | Same as 7.3.52 The Test Tolerance values are taken from the analysis for similar HD-FDD Test case 7.3.52 and no separate analysis is needed. |  |
| 7.3.88 TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage |  |  | Same as 7.3.60 |  |  | Same as 7.3.60 |  |  |  |  |  |  | Same as 7.3.60 |  |
| 7.3.89 TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage |  |  | Same as 7.3.61 |  |  | Same as 7.3.61 |  |  |  |  |  |  | Same as 7.3.61 |  |
| 7.3.90 TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Normal Coverage |  |  | Same as 7.3.28 |  |  | Same as 7.3.28 |  |  |  |  |  |  | Same as 7.3.28 |  |
| 7.3.91 TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage |  |  | Same as 7.3.28 |  |  | Same as 7.3.28 |  |  |  |  |  |  | Same as 7.3.28 |  |
| 7.3.92 TDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage |  |  | Same as 7.3.64 |  |  | Same as 7.3.64 |  |  |  |  |  |  | Same as 7.3.64 |  |
| 7.3.93 TDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage |  |  | Same as 7.3.65 |  |  | Same as 7.3.65 |  |  |  |  |  |  | Same as 7.3.65 |  |
| 7.3.94 TDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage |  |  | Same as 7.3.64 |  |  | Same as 7.3.64 |  |  |  |  |  |  | Same as 7.3.64 |  |
| 7.3.95 TDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage |  |  | Same as 7.3.64 |  |  | Same as 7.3.64 |  |  |  |  |  |  | Same as 7.3.64 |  |
| 7.4.1 E-UTRAN FDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC |  |  | SNRs as specified |  |  | 0.6dB |  |  |  |  |  |  | Formula: SNR + TT |  |
| 7.4.2 E-UTRAN TDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC |  |  | SNRs as specified |  |  | 0.6dB |  |  |  |  |  |  | Same as 7.4.1 |  |
| 7.4.3 E-UTRAN FDD-FDD Interruption at transitions between active and non-active during DRX in asynchronous dual connectivity |  |  | SNRs as specified |  |  | 0.6dB |  |  |  |  |  |  | Same as 7.4.1 |  |
| 7.5.1 E-UTRAN FDD - UE ProSe Direct Discovery Transmission Timing Accuracy Test |  |  | Uplink timing: ±12Ts Ês / Noc: +3.00dB |  |  | ±3Ts +0.3dB |  |  |  |  |  |  | Uplink timing: ±15Ts Ês / Noc: +3.30dB |  |
| 7.5.4 E-UTRAN FDD - UE ProSe Direct Communication Transmission Timing Accuracy Test |  |  | Uplink timing: ±12Ts Ês / Noc: +3.00dB |  |  | ±3Ts +0.3dB |  |  |  |  |  |  | Uplink timing: ±15Ts Ês / Noc: +3.30dB |  |
| 7.6.1 E-UTRAN FDD-TDD CA interruption at SRS carrier based switching |  |  | SNRs as specified |  |  | 0dB |  |  |  |  |  |  | Formula: SNR + TT |  |
| 7.6.2 E-UTRAN TDD-TDD CA interruption at SRS carrier based switching |  |  | SNRs as specified |  |  | 0dB |  |  |  |  |  |  | Same as 7.6.1 |  |
| 8.1.1 E-UTRAN FDD-FDD intra frequency event triggered reporting under fading propagation conditions in asynchronous cells |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB |  |  | During T1: 0dB 2.10dB 0dB During T2: 0dB 2.10dB 2.10dB |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +6.10dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +6.10dB Ês2 / Noc: +6.10dB |  |
| 8.1.2 E-UTRAN FDD-FDD intra frequency event triggered reporting under fading propagation conditions in synchronous cells |  |  | Same as 8.1.1 |  |  | Same as 8.1.1 |  |  |  |  |  |  | Same as 8.1.1 |  |
| 8.1.3 E-UTRAN FDD-FDD intra frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX |  |  | Same as 8.1.1 |  |  | Same as 8.1.1 |  |  |  |  |  |  | Same as 8.1.1 |  |
| 8.1.5 E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +11.00dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +11.00dB |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB During T3: 0dB 0dB 0dB |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +11.00dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +11.00dB |  |
| 8.1.6 E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX |  |  | Same as 8.1.5 |  |  | Same as 8.1.5 |  |  |  |  |  |  | Same as 8.1.5 |  |
| 8.1.7 UE Measurement Procedures / E-UTRAN FDD-FDD Intra-frequency event-triggered reporting under time domain measurement resource restriction with non-MBSFN ABS (eICIC) |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +1.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +1.00dB Ês2 / Noc: -4.00dB |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0.8dB |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +1.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +1.00dB Ês2 / Noc: -3.20dB |  |
| 8.1.8 E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +2.00dB Ês3 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +2.00dB Ês3 / Noc: -4.00dB |  |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0.8dB |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +2.00dB Ês3 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês1 / Noc: +2.00dB Ês2 / Noc: -3.20dB |  |
| 8.1.9 E-UTRAN FDD-FDD intra frequency event triggered reporting under fading propagation conditions in asynchronous cells for 5MHz Bandwidth |  |  | Same as 8.1.1 |  |  | Same as 8.1.1 |  |  |  |  |  |  | Same as 8.1.1 |  |
| 8.1.10 E-UTRAN FDD-FDD intra frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for 5MHz Bandwidth |  |  | Same as 8.1.3 |  |  | Same as 8.1.3 |  |  |  |  |  |  | Same as 8.1.3 |  |
| 8.1.11 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0 |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB |  |  | During T1: 0dB 2.10dB 0dB During T2: 0dB 2.10dB 2.10dB |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +6.10dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +6.10dB Ês2 / Noc: +6.10dB |  |
| 8.1.11_2 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 1bis |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +7.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +7.00dB Ês2 / Noc: +4.00dB |  |  | During T1: 0dB 0.9dB 0dB During T2: 0dB 0.9dB 2.40dB |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +7.90dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +7.90dB Ês2 / Noc: +6.40dB |  |
| 8.1.12 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0 |  |  | Same as 8.1.11 |  |  | Same as 8.1.11 |  |  |  |  |  |  | Same as 8.1.11 |  |
| 8.1.12_2 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 1bis |  |  | Same as 8.1.11_2 |  |  | Same as 8.1.11_2 |  |  |  |  |  |  | Same as 8.1.11_2 |  |
| 8.1.13 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0 |  |  | Same as 8.1.11 |  |  | Same as 8.1.11 |  |  |  |  |  |  | Same as 8.1.11 |  |
| 8.1.13_2 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 1bis |  |  | Same as 8.1.11_2 |  |  | Same as 8.1.11_2 |  |  |  |  |  |  | Same as 8.1.11_2 |  |
| 8.1.17 E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0 |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB |  |  | During T1: 0dB 2.60dB 0dB During T2: 0dB 2.60dB 2.60dB |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +6.60dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +6.60dB Ês2 / Noc: +6.60dB |  |
| 8.1.17_2 E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 1bis |  |  | Same as 8.1.11_2 |  |  | Same as 8.1.11_2 |  |  |  |  |  |  | Same as 8.1.11_2 |  |
| 8.1.18 E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0 |  |  | Same as 8.1.17 |  |  | Same as 8.1.17 |  |  |  |  |  |  | Same as 8.1.17 |  |
| 8.1.18_2 E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 1bis |  |  | Same as 8.1.11_2 |  |  | Same as 8.1.11_2 |  |  |  |  |  |  | Same as 8.1.11_2 |  |
| 8.1.19 E-UTRAN FD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0 |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +11.00dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +11.00dB |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB During T3: 0dB 0dB 0dB |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +11.00dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +11.00dB |  |
| 8.1.19_2 E-UTRAN FD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 1bis |  |  | Same as 8.1.19 |  |  | Same as 8.1.19 |  |  |  |  |  |  | Same as 8.1.19 |  |
| 8.1.20 E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0 |  |  | Same as 8.1.19 |  |  | Same as 8.1.19 |  |  |  |  |  |  | Same as 8.1.19 |  |
| 8.1.20_2 E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 1bis |  |  | Same as 8.1.20 |  |  | Same as 8.1.20 |  |  |  |  |  |  | Same as 8.1.20 |  |
| 8.1.21 E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0 |  |  | Same as 8.1.19 |  |  | Same as 8.1.19 |  |  |  |  |  |  | Same as 8.1.19 |  |
| 8.1.22 E-UTRAN HD- FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0 |  |  | Same as 8.1.19 |  |  | Same as 8.1.19 |  |  |  |  |  |  | Same as 8.1.19 |  |
| 8.1.23 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0.2dB |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.20dB |  |
| 8.1.24 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA |  |  | Same as 8.1.23 |  |  | Same as 8.1.23 |  |  |  |  |  |  | Same as 8.1.23 |  |
| 8.1.25 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX |  |  | Same as 8.1.23 |  |  | Same as 8.1.23 |  |  |  |  |  |  | Same as 8.1.23 |  |
| 8.1.26 E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA |  |  | Same as 8.1.23 |  |  | Same as 8.1.23 |  |  |  |  |  |  | Same as 8.1.23 |  |
| 8.1.27 E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA |  |  | Same as 8.1.23 |  |  | Same as 8.1.23 |  |  |  |  |  |  | Same as 8.1.23 |  |
| 8.1.28 E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX |  |  | Same as 8.1.23 |  |  | Same as 8.1.23 |  |  |  |  |  |  | Same as 8.1.23 |  |
| 8.1.29 E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA |  |  | Same as 8.1.23 |  |  | Same as 8.1.23 |  |  |  |  |  |  | Same as 8.1.23 |  |
| 8.1.30 E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX |  |  | Same as 8.1.23 |  |  | Same as 8.1.23 |  |  |  |  |  |  | Same as 8.1.23 |  |
| 8.1.31 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB |  |  | Same as 8.1.33 |  |  | Same as 8.1.33 |  |  |  |  |  |  | Same as 8.1.33 |  |
| 8.1.32 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB |  |  | Same as 8.1.33 |  |  | Same as 8.1.33 |  |  |  |  |  |  | Same as 8.1.33 |  |
| 8.1.33 E-UTRAN HD-FDD Intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: -12.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: -12.00dB Ês2 / Noc: -12.00dB |  |  | During T1: 0dB 0.8dB 0dB During T2: 0dB 1.2dB 1.2dB |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: -11.20dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: -10.80dB Ês2 / Noc: -10.80dB |  |
| 8.1.34 E-UTRAN HD-FDD Intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB |  |  | Same as 8.1.33 |  |  | Same as 8.1.33 |  |  |  |  |  |  | Same as 8.1.33 |  |
| 8.1.35 E-UTRAN TDD Intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB |  |  | Same as 8.1.33 |  |  | Same as 8.1.33 |  |  |  |  |  |  | Same as 8.1.33 |  |
| 8.1.40 E-UTRAN FDD-FDD intra-frequency event triggered reporting for UE configured with highSpeedEnhancedMeasFlag in synchronous cells |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: +4dB |  |  | During T1: 0dB 0.7dB 0dB During T2: 0dB 0.7dB 0.7dB |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4.7dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.7dB Ês2 / Noc: +4.7dB |  |
| 8.1.41 E-UTRAN FDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for CE UE in CEModeA without gap |  |  | During T1: Noc: -104dBm/15kHz Ês1 / Noc: +19dB During T2: Noc: -104dBm/15kHz Ês / Noc: -3dB |  |  | During T1: 0dB 0.6dB During T2: 0dB 0dB |  |  |  |  |  |  | During T1: Noc: -104dBm/15kHz Ês1 / Noc: +19.6dB During T2: Noc: -104dBm/15kHz Ês / Noc: -3dB |  |
| 8.1.42 E-UTRAN HD-FDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for CE UE in CEModeA without gap |  |  | Same as 8.1.41 |  |  | Same as 8.1.41 |  |  |  |  |  |  | Same as 8.1.41 |  |
| 8.2.1 E-UTRAN TDD-TDD intra frequency event triggered reporting under fading propagation conditions in synchronous cells |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB |  |  | During T1: 0dB 2.10dB 0dB During T2: 0dB 2.60dB 2.60dB |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +6.10dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +6.60dB Ês2 / Noc: +6.60dB |  |
| 8.2.2 E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX |  |  | Same as 8.2.1 |  |  | Same as 8.2.1 |  |  |  |  |  |  | Same as 8.2.1 |  |
| 8.2.3 E-UTRAN TDD-TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +11.00dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +11.00dB |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB During T3: 0dB 0dB 0dB |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +11.00dB During T3: Noc: -98dBm /15kHz Ês1 / Noc: +8.00dB Ês2 / Noc: +11.00dB |  |
| 8.2.4 E-UTRAN TDD-TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX |  |  | Same as 8.2.3 |  |  | Same as 8.2.3 |  |  |  |  |  |  | Same as 8.2.3 |  |
| 8.2.5 UE Measurement Procedures / E-UTRAN TDD-TDD Intra-frequency event-triggered reporting under time domain measurement resource restriction with non-MBSFN ABS (eICIC) |  |  | Same as 8.1.7 |  |  | Same as 8.1.7 |  |  |  |  |  |  | Same as 8.1.7 |  |
| 8.2.6 E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) |  |  | Same as 8.1.8 |  |  | Same as 8.1.8 |  |  |  |  |  |  | Same as 8.1.8 |  |
| 8.2.7 E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0 |  |  | Same as 8.1.19 |  |  | Same as 8.1.19 |  |  |  |  |  |  | Same as 8.1.19 |  |
| 8.2.7_2 E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 1bis |  |  | Same as 8.2.7 |  |  | Same as 8.2.7 |  |  |  |  |  |  | Same as 8.2.7 |  |
| 8.2.8 E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0 |  |  | Same as 8.1.19 |  |  | Same as 8.1.19 |  |  |  |  |  |  | Same as 8.1.19 |  |
| 8.2.8_2 E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 1bis |  |  | Same as 8.2.8 |  |  | Same as 8.2.8 |  |  |  |  |  |  | Same as 8.2.8 |  |
| 8.2.11 E-UTRAN TDD-TDD intra-frequency event triggered reporting for UE configured with highSpeedEnhancedMeasFlag in synchronous cells |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB |  |  | During T1: 0dB 0.9dB 0dB During T2: 0dB 0.9dB 0.9dB |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4.90dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.90dB Ês2 / Noc: +4.90dB |  |
| 8.2.14 E-UTRAN TDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for CE in CEModeA without gap |  |  | Same as 8.1.41 |  |  | Same as 8.1.41 |  |  |  |  |  |  | Same as 8.1.41 |  |
| 8.3.1 E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.00dB |  |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0.7dB |  |  |  |  |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.70dB |  |
| 8.3.1_2 E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 1bis |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.00dB |  |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 1.7dB |  |  |  |  |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +8.70dB |  |
| 8.3.2 E-UTRAN FDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells |  |  | Same as 8.3.1 |  |  | Same as 8.3.1 |  |  |  |  |  |  | Same as 8.3.1 |  |
| 8.3.2_2 E-UTRAN FDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells for UE category 1bis |  |  | Same as 8.3.1_2 |  |  | Same as 8.3.1_2 |  |  |  |  |  |  | Same as 8.3.1_2 |  |
| 8.3.3 E-UTRAN FDD-FDD Inter frequency event triggered reporting under AWGN propagation conditions in asynchronous cells with DRX when L3 filtering is used |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +4.00dB During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +24.00dB |  |  | During T1: +2.5dB 0dB 0dB 0dB During T2: +2.5dB -4.0dB 0dB 0dB |  |  |  |  |  |  | During T1: Noc1: -95.50dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +4.00dB During T2: Noc1: -95.50dBm/15kHz Ês1 / Noc1: +0.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +24.00dB |  |
| 8.3.3_2 E-UTRAN FDD-FDD inter-frequency event triggered reporting under AWGN propagation conditions in asynchronous cells with DRX when L3 filtering is used for UE category 1bis |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +4.00dB During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +24.00dB |  |  | During T1: +2.5dB 0dB 0dB 0dB During T2: +2.5dB -4.0dB 0dB +1dB |  |  |  |  |  |  | During T1: Noc1: -95.50dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +4.00dB During T2: Noc1: -95.50dBm/15kHz Ês1 / Noc1: +0.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +25.00dB |  |
| 8.3.4 E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.00dB During T3: Noc1: -98dBm /15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.00dB |  |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB |  |  |  |  |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.00dB During T3: Noc1: -98dBm /15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.00dB |  |
| 8.3.5 E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX |  |  | Same as 8.3.4 |  |  | Same as 8.3.4 |  |  |  |  |  |  | Same as 8.3.4 |  |
| 8.3.6 E-UTRAN FDD-FDD Inter-frequency event triggered reporting without measurement gaps under AWGN propagation conditions in asynchronous cells |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.00dB |  |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB |  |  |  |  |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.00dB |  |
| 8.3.7 E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Increased Carrier Monitoring without Reduced Performance Group |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity Noc3: -98dBm/15kHz Ês3 / Noc3: -infinity Noc4: -98dBm/15kHz Ês4 / Noc4: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.00dB Noc3: -98dBm/15kHz Ês3 / Noc3: +7.00dB Noc4: -98dBm/15kHz Ês4 / Noc4: +7.00dB |  |  | During T1: 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB |  |  |  |  |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity Noc3: -98dBm/15kHz Ês3 / Noc3: -infinity Noc4: -98dBm/15kHz Ês4 / Noc4: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.00dB Noc3: -98dBm/15kHz Ês3 / Noc3: +7.00dB Noc4: -98dBm/15kHz Ês4 / Noc4: +7.00dB |  |
| 8.3.8 FDD-FDD Inter-frequency correct reporting of measurement events with reduced performance group configured, non DRX |  |  | Same as 8.3.7 |  |  | Same as 8.3.7 |  |  |  |  |  |  | Same as 8.3.7 |  |
| 8.3.9 FDD-FDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity Noc3: -98dBm/15kHz Ês3 / Noc3: -infinity Noc4: -98dBm/15kHz Ês4 / Noc4: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +4.00dB Noc3: -98dBm/15kHz Ês3 / Noc3: +4.00dB Noc4: -98dBm/15kHz Ês4 / Noc4: +4.00dB |  |  | During T1: 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 1dB 0dB 1dB 0dB 1dB |  |  |  |  |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity Noc3: -98dBm/15kHz Ês3 / Noc3: -infinity Noc4: -98dBm/15kHz Ês4 / Noc4: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +5.00dB Noc3: -98dBm/15kHz Ês3 / Noc3: +5.00dB Noc4: -98dBm/15kHz Ês4 / Noc4: +5.00dB |  |
| 8.3.12 E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for CE UE with discontinuous MPDCCH monitoring in CEModeA |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +4.00dB |  |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 4.65dB |  |  |  |  |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +8.65dB |  |
| 8.3.13 E-UTRAN HD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for CE UE with discontinuous MPDCCH monitoring in CEModeA |  |  | Same as 8.3.12 |  |  | Same as 8.3.12 |  |  |  |  |  |  | Same as 8.3.12 |  |
| 8.3.14 E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for CE UE with discontinuous MPDCCH monitoring in CEModeB |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: -12.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: -12.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: -12.00dB |  |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 4.65dB |  |  |  |  |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: -12.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: -12.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: -8.35dB |  |
| 8.3.15 E-UTRAN HD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for CE UE with discontinuous MPDCCH monitoring in CEModeB |  |  | Same as 8.3.14 |  |  | Same as 8.3.14 |  |  |  |  |  |  | Same as 8.3.14 |  |
| 8.3.16 E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for CE UE in CEModeA when DRX is used |  |  | Same as 8.3.12 |  |  | Same as 8.3.12 |  |  |  |  |  |  | Same as 8.3.12 |  |
| 8.3.17 E-UTRAN HD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for CE UE in CEModeA when DRX is used |  |  | Same as 8.3.12 |  |  | Same as 8.3.12 |  |  |  |  |  |  | Same as 8.3.12 |  |
| 8.3.18 E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for CE UE in CEModeB when DRX is used |  |  | Same as 8.3.14 |  |  | Same as 8.3.14 |  |  |  |  |  |  | Same as 8.3.14 |  |
| 8.3.19 E-UTRAN HD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for CE UE in CEModeB when DRX is used |  |  | Same as 8.3.14 |  |  | Same as 8.3.14 |  |  |  |  |  |  | Same as 8.3.14 |  |
| 8.4.1 E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.00dB |  |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0.7dB |  |  |  |  |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.70dB |  |
| 8.4.1_2 E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 1bis |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.00dB |  |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 1.7dB |  |  |  |  |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +8.70dB |  |
| 8.4.2 E-UTRAN TDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in synchronous cells |  |  | Same as 8.4.1 |  |  | Same as 8.4.1 |  |  |  |  |  |  | Same as 8.4.1 |  |
| 8.4.2_2 E-UTRAN TDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in synchronous cells for UE category 1bis |  |  | Same as 8.4.1_2 |  |  | Same as 8.4.1_2 |  |  |  |  |  |  | Same as 8.4.1_2 |  |
| 8.4.3 E-UTRAN TDD-TDD inter-frequency event triggered reporting under AWGN propagation conditions in synchronous cells with DRX when L3 filtering is used |  |  | Same as 8.3.3 |  |  | Same as 8.3.3 |  |  |  |  |  |  | Same as 8.3.3 |  |
| 8.4.3_2 E-UTRAN TDD-TDD inter-frequency event triggered reporting under AWGN propagation conditions in synchronous cells with DRX when L3 filtering is used for UE category 1bis |  |  | Same as 8.3.3_2 |  |  | Same as 8.3.3_2 |  |  |  |  |  |  | Same as 8.3.3_2 |  |
| 8.4.4 E-UTRAN TDD-TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.00dB During T3: Noc1: -98dBm /15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.00dB |  |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB |  |  |  |  |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.00dB During T3: Noc1: -98dBm /15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.00dB |  |
| 8.4.5 E-UTRAN TDD-TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX |  |  | Same as 8.4.4 |  |  | Same as 8.4.4 |  |  |  |  |  |  | Same as 8.4.4 |  |
| 8.4.6 E-UTRAN TDD-TDD Inter-frequency event triggered reporting for TDD UL/DL configuration 0 |  |  | Same as 8.4.1 |  |  | Same as 8.4.1 |  |  |  |  |  |  | Same as 8.4.1 |  |
| 8.4.6_2 E-UTRAN TDD-TDD Inter-frequency event triggered reporting for TDD UL/DL configuration 0 for UE category 1bis |  |  | Same as 8.4.1_2 |  |  | Same as 8.4.1_2 |  |  |  |  |  |  | Same as 8.4.1_2 |  |
| 8.4.7 E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Increased Carrier Monitoring without Reduced Performance Group |  |  | Same as 8.3.7 |  |  | Same as 8.3.7 |  |  |  |  |  |  | Same as 8.3.7 |  |
| 8.4.8 TDD-TDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX |  |  | Same as 8.3.8 The Test Tolerance values are taken from the analysis for similar FDD-FDD Test case 8.3.8 and no separate analysis is needed. |  |  | Same as 8.3.9 The Test Tolerance values are taken from the analysis for similar FDD-FDD Test case 8.3.8 and no separate analysis is needed. |  |  |  |  |  |  | Same as 8.3.8 The Test Tolerance values are taken from the analysis for similar FDD-FDD Test case 8.3.8 and no separate analysis is needed. |  |
| 8.4.9 TDD-TDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX |  |  | Same as 8.3.9 The Test Tolerance values are taken from the analysis for similar FDD-FDD Test case 8.3.9 and no separate analysis is needed. |  |  | Same as 8.3.9 The Test Tolerance values are taken from the analysis for similar FDD-FDD Test case 8.3.9 and no separate analysis is needed. |  |  |  |  |  |  | Same as 8.3.9 The Test Tolerance values are taken from the analysis for similar FDD-FDD Test case 8.3.9 and no separate analysis is needed. |  |
| 8.4.12 E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for CE UE with discontinuous MPDCCH monitoring in CEModeA |  |  | Same as 8.3.12 The Test Tolerance values are taken from the analysis for similar FDD Test case 8.3.12 and no separate analysis is needed. |  |  | Same as 8.3.12 The Test Tolerance values are taken from the analysis for similar FDD Test case 8.3.12 and no separate analysis is needed. |  |  |  |  |  |  | Same as 8.3.12 The Test Tolerance values are taken from the analysis for similar FDD Test case 8.3.12 and no separate analysis is needed. |  |
| 8.4.13 E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for CE UE with discontinuous MPDCCH monitoring in CEModeB |  |  | Same as 8.3.13 The Test Tolerance values are taken from the analysis for similar FDD Test case 8.3.13 and no separate analysis is needed. |  |  | Same as 8.3.12 The Test Tolerance values are taken from the analysis for similar FDD Test case 8.3.13 and no separate analysis is needed. |  |  |  |  |  |  | Same as 8.3.12 The Test Tolerance values are taken from the analysis for similar FDD Test case 8.3.13 and no separate analysis is needed. |  |
| 8.4.14 TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for CE UE in CEModeA when DRX is used |  |  | Same as 8.4.12 |  |  | Same as 8.4.12 |  |  |  |  |  |  | Same as 8.4.12 |  |
| 8.4.15 E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for CE UE in CEModeB when DRX is used |  |  | Same as 8.4.13 |  |  | Same as 8.4.13 |  |  |  |  |  |  | Same as 8.4.13 |  |
| 8.5.1 E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4.00dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4.00dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc: -1.8 dB |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB |  |  |  |  |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4.00dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4.00dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc: -1.8dB |  |
| 8.5.2 E-UTRAN FDD - UTRAN FDD SON ANR cell search reporting under AWGN propagation conditions |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 4dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 4dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc: -3.35 dB |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0.4dB |  |  |  |  |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 4dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 4dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc: -2.95dB |  |
| 8.5.3 E-UTRAN FDD - UTRAN FDD event triggered reporting when DRX is used under fading propagation conditions |  |  | Same as 8.5.1 |  |  |  |  | Same as 8.5.1 |  |  |  |  | Same as 8.5.1 |  |
| 8.5.4 E-UTRAN FDD - UTRAN FDD enhanced cell identification under AWGN propagation conditions |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 4dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 4dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc: 0.02 dB |  |  |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB |  |  |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 4dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 4dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc: 0.02dB |  |
| 8.5.6 E-UTRAN FDD - UTRAN FDD event triggered reporting without measurement gaps under AWGN propagation conditions |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 4dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 4dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc: -1.8 dB |  |  |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB |  |  |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 4dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 4dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc: -1.8dB |  |
| 8.5.7 E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions for 5MHz Bandwidth |  |  | Same as 8.5.1 |  |  |  |  | Same as 8.5.1 |  |  |  |  | Same as 8.5.1 |  |
| 8.5.8 E-UTRA FDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 4dB UTRA Cells 2, 3 and 4 Ioc: -70dBm/3.84MHz Ior / Ioc: -infinity During T2: E-UTRA Cell 1: Noc: -98dBm/15kHz Ês / Noc: 4dB UTRA Cells 2, 3 and 4 Ioc: -70dBm/3.84MHz Ior / Ioc: -1.8 dB |  |  |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB |  |  |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 4dB UTRA Cells 2, 3 and 4 Ioc: -70dBm/3.84MHz Ior / Ioc: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 4dB UTRA Cells 2, 3 and 4 Ioc: -70dBm/3.84MHz Ior / Ioc: -1.8 dB |  |
| 8.6.1 E-UTRAN TDD -UTRAN FDD event triggered reporting under fading propagation conditions |  |  | Same as 8.5.1 |  |  | Same as 8.5.1 |  |  |  |  |  |  | Same as 8.5.1 |  |
| 8.6.3 E-UTRA TDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX |  |  | Same as 8.5.8 |  |  | Same as 8.5.8 |  |  |  |  |  |  | Same as 8.5.8 |  |
| 8.7.1 E-UTRAN TDD - UTRAN TDD event triggered reporting under fading propagation conditions |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +9dB UTRA Cell 2 Ioc: -80dBm/1.28MHz Ior / Ioc: -inf PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +9dB UTRA Cell 2 Ioc: -80dBm/1.28MHz Ior / Ioc: +5dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB |  |  | During T1: 0dB 0dB 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB 0dB 0dB |  |  |  |  |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +9dB UTRA Cell 2 Ioc: -80dBm/1.28MHz Ior / Ioc: -inf PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +9dB UTRA Cell 2 Ioc: -80dBm/1.28MHz Ior / Ioc: +5dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB |  |
| 8.7.2 E-UTRAN TDD - UTRAN TDD cell search when DRX is used under fading propagation conditions |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB UTRA Cell 2 Ioc: -80dBm/1.28MHz Ior / Ioc: -inf PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB UTRA Cell 2 Ioc: -80dBm/1.28MHz Ior / Ioc: +9dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB |  |  | During T1: 0dB 0dB -0.40dB 0dB 0dB 0dB During T2: 0dB 0dB -0.40dB 0dB 0dB 0dB |  |  |  |  |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB UTRA Cell 2 Ioc: -80.40dBm/1.28MHz Ior / Ioc: -inf PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB UTRA Cell 2 Ioc: -80.40dBm/1.28MHz Ior / Ioc: +9dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB |  |
| 8.7.3 E-UTRAN TDD - UTRAN TDD SON ANR cell search reporting under AWGN propagation conditions |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB UTRA Cell 2 Ioc: -75dBm/1.28MHz Ior / Ioc: -inf PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB UTRA Cell 2 Ioc: -75dBm/1.28MHz Ior / Ioc: +5dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB |  |  | During T1: 0dB 0dB 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB 0dB 0dB |  |  |  |  |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB UTRA Cell 2 Ioc: -75dBm/1.28MHz Ior / Ioc: -inf PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB UTRA Cell 2 Ioc: -75dBm/1.28MHz Ior / Ioc: +5dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB |  |
| 8.7.4 E-UTRAN TDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB UTRA Cell 2 Ioc: -80dBm/1.28MHz Ior / Ioc: - infinity PCCPCH_Ec/Ior: -4.77dB DwPCH_Ec/Ior: 0dB During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB UTRA Cell 2 Ioc: -80dBm/1.28MHz Ior / Ioc: +8dB PCCPCH_Ec/Ior: -4.77dB DwPCH_Ec/Ior: 0dB |  |  | During T1: 0dB 0dB +0.6dB 0dB 0dB 0dB During T2: 0dB 0dB +0.6dB 0dB 0dB 0dB |  |  |  |  |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB UTRA Cell 2 Ioc: -79.4dBm/1.28MHz Ior / Ioc: - infinity PCCPCH_Ec/Ior: -4.77dB DwPCH_Ec/Ior: 0dB During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB UTRA Cell 2 Ioc: -79.4dBm/1.28MHz Ior / Ioc: +8dB PCCPCH_Ec/Ior: -4.77dB DwPCH_Ec/Ior: 0dB |  |
| 8.7.5 E-UTRA TDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB UTRA Cells 2, 3 and 4 Ioc: -70dBm/1.28MHz Ior / Ioc: -inf PCCPCH_Ec/Ior: -inf DwPCH_Ec/Ior: -inf During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB UTRA Cells 2,3 and 4 Ioc: -70dBm/1.28MHz Ior / Ioc: +9dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB |  |  | During T1: 0dB 0dB 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB 0dB 0dB |  |  |  |  |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB UTRA Cells 2, 3 and 4 Ioc: -70dBm/1.28MHz Ior / Ioc: -inf PCCPCH_Ec/Ior: -inf DwPCH_Ec/Ior: -inf During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB UTRA Cells 2,3 and 4 Ioc: -70dBm/1.28MHz Ior / Ioc: +9dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB |  |
| 8.7A.1 E-UTRA FDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX |  |  | Same as 8.7.5 |  |  | Same as 8.7.5 |  |  |  |  |  |  | Same as 8.7.5 |  |
| 8.8.1 E-UTRAN FDD - GSM event triggered reporting in AWGN |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -75 dBm |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB |  |  |  |  |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -75 dBm |  |
| 8.8.2 E-UTRAN FDD- GSM event triggered reporting when DRX is used in AWGN |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4.00dB GSM Cell 2 Signal level: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4.00dB GSM Cell 2 Signal level: -75dBm |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB |  |  |  |  |  |  | During T1: E-UTRAN Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4.00dB GSM Cell 2 Signal level: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4.00dB GSM Cell 2 Signal level: -75dBm |  |
| 8.9.1 E-UTRAN FDD - UTRAN TDD event triggered reporting under fading propagation conditions |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Iot: 4dB UTRA Cell 2 Ioc: -70dBm/1.28MHz Ior / Ioc: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Iot: 4dB UTRA Cell 2 Ioc: -70dBm/1.28MHz Ior / Ioc: 9 dB |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0 dB |  |  |  |  |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Iot: 4dB UTRA Cell 2 Ioc: -70dBm/1.28MHz Ior / Ioc: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Iot: 4dB UTRA Cell 2 Ioc: -70dBm/1.28MHz Ior / Ioc: 9 dB |  |
| 8.9.2 E-UTRAN FDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions |  |  | Same as 8.7.4 |  |  | Same as 8.7.4 |  |  |  |  |  |  | Same as 8.7.4 |  |
| 8.10.1 E-UTRAN TDD - GSM event triggered reporting in AWGN |  |  | Same as 8.8.1 |  |  | Same as 8.8.1 |  |  |  |  |  |  | Same as 8.8.1 |  |
| 8.10.2 E-UTRAN TDD-GSM event triggered reporting when DRX is used in AWGN |  |  | Same as 8.8.2 |  |  | Same as 8.8.2 |  |  |  |  |  |  | Same as 8.8.2 |  |
| 8.11.1 Multiple E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 0dB E-UTRA Cell 2 Noc: -98dBm/15kHz Ês / Noc: -infinity E-UTRA Cell 3 Noc: -98dBm/15kHz Ês / Noc: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 0dB E-UTRA Cell 2 Noc: -98dBm/15kHz Ês / Noc: 3dB E-UTRA Cell 3 Noc: -98dBm/15kHz Ês / Noc: 3dB |  |  | During T1: 0dB 0dB 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB +0.6dB 0dB +0.6dB |  |  |  |  |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 0dB E-UTRA Cell 2 Noc: -98dBm/15kHz Ês / Noc: -infinity E-UTRA Cell 3 Noc: -98dBm/15kHz Ês / Noc: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 0dB E-UTRA Cell 2 Noc: -98dBm/15kHz Ês / Noc: 3.6dB E-UTRA Cell 3 Noc: -98dBm/15kHz Ês / Noc: 3.6dB |  |
| 8.11.2 E-UTRAN TDD - E-UTRAN TDD and E-UTRAN TDD Inter-frequency event triggered reporting under fading propagation conditions |  |  | Same as 8.11.1 |  |  | Same as 8.11.1 |  |  |  |  |  |  | Same as 8.11.1 |  |
| 8.11.3 E-UTRAN FDD-FDD Inter-frequency and UTRAN FDD event triggered reporting under fading propagation conditions |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4.00dB E-UTRA Cell 2 Noc: -98dBm/15kHz Ês / Noc: -infinity UTRA Cell 3 Ioc: -70dBm/3.84MHz Ior / Ioc: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4.00dB E-UTRA Cell 2 Noc: -98dBm/15kHz Ês / Noc: +7.00dB UTRA Cell 3 Ioc: -70dBm/3.84MHz Ior / Ioc: -1.8 dB |  |  | During T1: 0dB -1.05dB 0dB 0dB 0dB 0dB During T2: 0dB -1.05dB 0dB 0dB 0dB 0dB |  |  |  |  |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +2.95dB E-UTRA Cell 2 Noc: -98dBm/15kHz Ês / Noc: -infinity UTRA Cell 3 Ioc: -70dBm/3.84MHz Ior / Ioc: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +2.95dB E-UTRA Cell 2 Noc: -98dBm/15kHz Ês / Noc: +7.00dB UTRA Cell 3 Ioc: -70dBm/3.84MHz Ior / Ioc: -1.8 dB |  |
| 8.11.4 InterRAT E-UTRA TDD to E-UTRA TDD and UTRA TDD cell search |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4.00dB E-UTRA Cell 2 Noc: -98dBm/15kHz Ês / Noc: -infinity UTRA Cell 3 Ioc: -80dBm/1.28MHz Ior / Ioc: -infinity PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4.00dB E-UTRA Cell 2 Noc: -98dBm/15kHz Ês / Noc: +7.00dB UTRA Cell 3 Ioc: -80dBm/1.28MHz Ior / Ioc: 9.00dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB |  |  |  |  | During T1: 0dB -1.05dB 0dB 0dB -0.4dB 0dB 0dB 0dB During T2: 0dB -1.05dB 0dB 0dB -0.4dB 0dB 0dB 0dB |  |  |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +2.95dB E-UTRA Cell 2 Noc: -98dBm/15kHz Ês / Noc: -infinity UTRA Cell 3 Ioc: -80.4dBm/1.28MHz Ior / Ioc: -infinity PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +2.95dB E-UTRA Cell 2 Noc: -98dBm/15kHz Ês / Noc: +7.00dB UTRA Cell 3 Ioc: -80.4dBm/1.28MHz Ior / Ioc: 9.00dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB |  |
| 8.11.5 Combined E-UTRAN FDD - E-UTRA FDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4.00dB E-UTRA Cell 2 Noc: -98dBm/15kHz Ês / Noc: -infinity GSM Cell 3 Signal level: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4.00dB E-UTRA Cell 2 Noc: -98dBm/15kHz Ês / Noc: +7.00dB GSM Cell 3 Signal level: -75dBm |  |  |  |  | During T1: 0dB -0.2dB 0dB 0dB 0dB During T2: 0dB -0.2dB 0dB +0.5dB 0dB |  |  |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +3.80dB E-UTRA Cell 2 Noc: -98dBm/15kHz Ês / Noc: -infinity GSM Cell 3 Signal level: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +3.80dB E-UTRA Cell 2 Noc: -98dBm/15kHz Ês / Noc: +7.50dB GSM Cell 3 Signal level: -75dBm |  |
| 8.11.6 Combined E-UTRAN TDD - E-UTRA TDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions |  |  | Same as 8.11.5 |  |  |  |  | Same as 8.11.5 |  |  |  |  | Same as 8.11.5 |  |
| 8.14.1 E-UTRAN TDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells |  |  | Same as 8.3.1 |  |  |  |  | Same as 8.3.1 |  |  |  |  | Same as 8.3.1 |  |
| 8.14.2 E-UTRAN TDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells |  |  | Same as 8.3.1 |  |  |  |  | Same as 8.3.1 |  |  |  |  | Same as 8.3.1 |  |
| 8.14.3 E-UTRAN TDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps |  |  | Same as 8.4.4 |  |  |  |  | Same as 8.4.4 |  |  |  |  | Same as 8.4.4 |  |
| 8.15.1 E-UTRAN FDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells |  |  | Same as 8.4.1 |  |  |  |  | Same as 8.4.1 |  |  |  |  | Same as 8.4.1 |  |
| 8.15.2 E-UTRAN FDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells |  |  | Same as 8.4.1 |  |  |  |  | Same as 8.4.1 |  |  |  |  | Same as 8.4.1 |  |
| 8.15.3 E-UTRAN FDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps |  |  | Same as 8.4.4 |  |  |  |  | Same as 8.4.4 |  |  |  |  | Same as 8.4.4 |  |
| 8.16.1 E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX |  |  | During T1: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 19dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 19dB E-UTRA Cell 3 Ês3 / Noc2: -infinity During T2: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 19dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 19dB E-UTRA Cell 3 Ês3 / Noc2: 19dB During T3: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: -3dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: -3dB E-UTRA Cell 3 Ês3 / Noc2: -3dB |  |  |  |  | During T1: 0dB +0.2dB 0dB +0.2dB 0dB During T2: 0dB +0.2dB 0dB +0.2dB 0dB During T3: 0dB 0dB 0dB -0.2dB 0dB |  |  |  |  | During T1: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 19.2dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 19.2dB E-UTRA Cell 3 Ês3 / Noc2: -infinity During T2: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 19.2dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 19.2dB E-UTRA Cell 3 Ês3 / Noc2: 19.0dB During T3: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: -3.0dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: -3.2dB E-UTRA Cell 3 Ês3 / Noc2: -3.0dB |  |
| 8.16.2 E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX |  |  | Same as 8.16.1 |  |  |  |  | Same as 8.16.1 |  |  |  |  | Same as 8.16.1 |  |
| 8.16.3 E-UTRAN FDD-FDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX |  |  | During T1: E-UTRA Cell 1 Noc1: -98dBm/15kHz Ês1 / Noc1: 16dB E-UTRA Cell 2 Noc2: -98dBm/15kHz Ês2 / Noc2: 16dB E-UTRA Cell 3 Ês3 / Noc2: -infinity During T2: E-UTRA Cell 1 Noc1: -98dBm/15kHz Ês1 / Noc1: 16dB E-UTRA Cell 2 Noc2: -98dBm/15kHz Ês2 / Noc2: 16dB E-UTRA Cell 3 Ês3 / Noc2: 16dB |  |  |  |  | During T1: 0dB 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB 0dB |  |  |  |  | During T1: E-UTRA Cell 1 Noc1: -98dBm/15kHz Ês1 / Noc1: 16dB E-UTRA Cell 2 Noc2: -98dBm/15kHz Ês2 / Noc2: 16dB E-UTRA Cell 3 Ês3 / Noc2: -infinity During T2: E-UTRA Cell 1 Noc1: -98dBm/15kHz Ês1 / Noc1: 16dB E-UTRA Cell 2 Noc2: -98dBm/15kHz Ês2 / Noc2: 16dB E-UTRA Cell 3 Ês3 / Noc2: 16dB |  |
| 8.16.4 E-UTRAN TDD-TDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX |  |  | Same as 8.16.3 |  |  |  |  | Same as 8.16.3 |  |  |  |  | Same as 8.16.3 |  |
| 8.16.5 E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth |  |  | During T1: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 19dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 19dB E-UTRA Cell 3 Ês3 / Noc2: -infinity During T2: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 19dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 19dB E-UTRA Cell 3 Ês3 / Noc2: 19dB During T3: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: -3dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: -3dB E-UTRA Cell 3 Ês3 / Noc2: -3dB |  |  |  |  | During T1: 0dB +0.2dB -0.1dB +0.3dB 0dB During T2: 0dB +0.2dB -0.1dB +0.3dB 0dB During T3: 0dB 0dB -0.1dB -0.1dB 0dB |  |  |  |  | During T1: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 19.2dB E-UTRA Cell 2 Noc2: -104.1dBm/15kHz Ês2 / Noc2: 19.3dB E-UTRA Cell 3 Ês3 / Noc2: -infinity During T2: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 19.2dB E-UTRA Cell 2 Noc2: -104.1dBm/15kHz Ês2 / Noc2: 19.3dB E-UTRA Cell 3 Ês3 / Noc2: 19.0dB During T3: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: -3.0dB E-UTRA Cell 2 Noc2: -104.1dBm/15kHz Ês2 / Noc2: -3.1dB E-UTRA Cell 3 Ês3 / Noc2: -3.0dB |  |
| 8.16.6 E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth |  |  | Same as 8.16.5 |  |  |  |  | Same as 8.16.5 |  |  |  |  | Same as 8.16.5 |  |
| 8.16.7 E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth |  |  | During T1: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 16dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 16dB E-UTRA Cell 3 Ês3 / Noc2: -infinity During T2: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 16dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 16dB E-UTRA Cell 3 Ês3 / Noc2: 16dB |  |  |  |  | During T1: 0dB 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB 0dB |  |  |  |  | During T1: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 16dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 16dB E-UTRA Cell 3 Ês3 / Noc2: -infinity During T2: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 16dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 16dB E-UTRA Cell 3 Ês3 / Noc2: 16dB |  |
| 8.16.8 E-UTRAN TDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth |  |  | Same as 8.16.7 |  |  |  |  | Same as 8.16.7 |  |  |  |  | Same as 8.16.7 |  |
| 8.16.9 E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz |  |  | During T1: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 19dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 19dB E-UTRA Cell 3 Ês3 / Noc2: -infinity During T2: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 19dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 19dB E-UTRA Cell 3 Ês3 / Noc2: 19dB During T3: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: -3dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: -3dB E-UTRA Cell 3 Ês3 / Noc2: -3dB |  |  |  |  | During T1: 0dB +0.2dB 0dB +0.2dB 0dB During T2: 0dB +0.2dB 0dB +0.2dB 0dB During T3: 0dB 0dB 0dB 0dB 0dB |  |  |  |  | During T1: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 19.2dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 19.2dB E-UTRA Cell 3 Ês3 / Noc2: -infinity During T2: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 19.2dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 19.2dB E-UTRA Cell 3 Ês3 / Noc2: 19.0dB During T3: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: -3.0dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: -3.0dB E-UTRA Cell 3 Ês3 / Noc2: -3.0dB |  |
| 8.16.10 E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz |  |  | Same as 8.16.9 |  |  |  |  | Same as 8.16.9 |  |  |  |  | Same as 8.16.9 |  |
| 8.16.11 E-UTRAN FDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz |  |  | Same as 8.16.3 |  |  |  |  | Same as 8.16.3 |  |  |  |  | Same as 8.16.3 |  |
| 8.16.12 E-UTRAN FDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz |  |  | Same as 8.16.3 |  |  |  |  | Same as 8.16.3 |  |  |  |  | Same as 8.16.3 |  |
| 8.16.13 E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 5MHz+5MHz |  |  | During T1: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 19dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 19dB E-UTRA Cell 3 Ês3 / Noc2: -infinity During T2: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 19dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 19dB E-UTRA Cell 3 Ês3 / Noc2: 19dB During T3: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: -3dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: -3dB E-UTRA Cell 3 Ês3 / Noc2: -3dB |  |  |  |  | During T1: 0dB +0.2dB 0dB +0.2dB 0dB During T2: 0dB +0.2dB 0dB +0.2dB 0dB During T3: 0dB 0dB 0dB 0dB 0dB |  |  |  |  | During T1: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 19.2dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 19.2dB E-UTRA Cell 3 Ês3 / Noc2: -infinity During T2: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 19.2dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 19.2dB E-UTRA Cell 3 Ês3 / Noc2: 19.0dB During T3: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: -3.0dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: -3.0dB E-UTRA Cell 3 Ês3 / Noc2: -3.0dB |  |
| 8.16.14 E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 5MHz+5MHz |  |  | Same as 8.16.13 |  |  |  |  | Same as 8.16.13 |  |  |  |  | Same as 8.16.13 |  |
| 8.16.15 E-UTRAN FDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 5MHz+5MHz |  |  | Same as 8.16.3 |  |  |  |  | Same as 8.16.3 |  |  |  |  | Same as 8.16.3 |  |
| 8.16.16 E-UTRAN FDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 5MHz+5MHz |  |  | Same as 8.16.3 |  |  |  |  | Same as 8.16.3 |  |  |  |  | Same as 8.16.3 |  |
| 8.16.17 E-UTRAN FDD activation and deactivation of known SCell in non-DRX |  |  | Same as 8.16.37 |  |  |  |  | Same as 8.16.37 |  |  |  |  | Same as 8.16.37 |  |
| 8.16.17A E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 20MHz |  |  | Same as 8.16.17 |  |  |  |  | Same as 8.16.17 |  |  |  |  | Same as 8.16.17 |  |
| 8.16.18 E-UTRAN TDD activation and deactivation of known SCell in non-DRX |  |  | Same as 8.16.17 |  |  |  |  | Same as 8.16.17 |  |  |  |  | Same as 8.16.17 |  |
| 8.16.18A E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 20MHz |  |  | Same as 8.16.17 |  |  |  |  | Same as 8.16.17 |  |  |  |  | Same as 8.16.17 |  |
| 8.16.21 E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20MHz+10MHz bandwidth |  |  | During T1: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 19dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 19dB E-UTRA Cell 3 Ês3 / Noc2: -infinity During T2: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 19dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 19dB E-UTRA Cell 3 Ês3 / Noc2: 19dB During T3: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: -3dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: -3dB E-UTRA Cell 3 Ês3 / Noc2: -3dB |  |  |  |  | During T1: 0dB +0.2dB 0dB +0.2dB 0dB During T2: 0dB +0.2dB 0dB +0.2dB 0dB During T3: 0dB 0dB 0dB 0dB 0dB |  |  |  |  | During T1: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 19.2dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 19.2dB E-UTRA Cell 3 Ês3 / Noc2: -infinity During T2: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 19.2dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 19.2dB E-UTRA Cell 3 Ês3 / Noc2: 19.0dB During T3: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: -3.0dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: -3.0dB E-UTRA Cell 3 Ês3 / Noc2: -3.0dB |  |
| 8.16.22 E-UTRAN TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20MHz+10MHz bandwidth |  |  | Same as 8.16.3 |  |  |  |  | Same as 8.16.3 |  |  |  |  | Same as 8.16.3 |  |
| 8.16.23 E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in FDD |  |  | During T1: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 17dB E-UTRA Cell 3 Ês3 / Noc2: -infinity During T2: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 17dB E-UTRA Cell 3 Ês3 / Noc2: 17dB During T3: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: -3dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: -3dB E-UTRA Cell 3 Ês3 / Noc2: -3dB |  |  |  |  | During T1: 0dB +0.2dB -0.5dB +0.7dB 0dB During T2: 0dB +0.2dB -0.5dB +0.7dB 0dB During T3: 0dB 0dB 0dB 0dB 0dB |  |  |  |  | During T1: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17.2dB E-UTRA Cell 2 Noc2: -104.5dBm/15kHz Ês2 / Noc2: 17.7dB E-UTRA Cell 3 Ês3 / Noc2: -infinity During T2: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17.2dB E-UTRA Cell 2 Noc2: -104.5dBm/15kHz Ês2 / Noc2: 17.7dB E-UTRA Cell 3 Ês3 / Noc2: 17.0dB During T3: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: -3.0dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: -3.0dB E-UTRA Cell 3 Ês3 / Noc2: -3.0dB |  |
| 8.16.24 E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in TDD |  |  | Same as 8.16.23 |  |  |  |  | Same as 8.16.23 |  |  |  |  | Same as 8.16.23 |  |
| 8.16.25 E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in FDD |  |  | During T1: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 16dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 16dB E-UTRA Cell 3 Ês3 / Noc2: -infinity During T2: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 16dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 16dB E-UTRA Cell 3 Ês3 / Noc2: 16dB |  |  |  |  | During T1: 0dB 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB 0dB |  |  |  |  | During T1: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 16dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 16dB E-UTRA Cell 3 Ês3 / Noc2: -infinity During T2: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 16dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 16dB E-UTRA Cell 3 Ês3 / Noc2: 16dB |  |
| 8.16.26 E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in TDD |  |  | Same as 8.16.25 |  |  |  |  | Same as 8.16.25 |  |  |  |  | Same as 8.16.25 |  |
| 8.16.27 E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with PCell in FDD |  |  | During T1: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: -infinity E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: 17dB E-UTRA Cell 4 Ês4 / Noc3: -infinity During T2: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 17dB E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: 17dB E-UTRA Cell 4 Ês4 / Noc3: 17dB During T3: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: -3dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: -3dB E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: -3dB E-UTRA Cell 4 Ês4 / Noc3: -3dB |  |  |  |  | During T1: 0dB 0dB 0dB 0dB -0.5dB +0.7dB 0dB During T2: 0dB 0dB 0dB +0.2dB -0.5dB +0.7dB 0dB During T3: 0dB 0dB 0dB -0.2dB -0.5dB -0.7dB -0.7dB |  |  |  |  | During T1: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: -infinity E-UTRA Cell 3 Noc3: -104.5dBm/15kHz Ês3 / Noc3: 17.7dB E-UTRA Cell 4 Ês4 / Noc3: -infinity During T2: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 17.2dB E-UTRA Cell 3 Noc3: -104.5dBm/15kHz Ês3 / Noc3: 17.7dB E-UTRA Cell 4 Ês4 / Noc3: 17dB During T3: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: -3dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: -3.2dB E-UTRA Cell 3 Noc3: -104.5dBm/15kHz Ês3 / Noc3: -3.7dB E-UTRA Cell 4 Ês4 / Noc3: -3.7dB |  |
| 8.16.28 E-UTRAN TDD-FDD 3DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with PCell in TDD |  |  | Same as 8.16.27 |  |  |  |  | Same as 8.16.27 |  |  |  |  | Same as 8.16.27 |  |
| 8.16.29 3DL FDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX |  |  | Same as 8.16.27 |  |  |  |  | Same as 8.16.27 |  |  |  |  | Same as 8.16.27 |  |
| 8.16.30 3DL TDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX |  |  | Same as 8.16.27 |  |  |  |  | Same as 8.16.27 |  |  |  |  | Same as 8.16.27 |  |
| 8.16.31 E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in FDD |  |  | During T1: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 16dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 16dB E-UTRA Cell 3 Noc3: -101dBm/15kHz Ês3 / Noc3: 16dB E-UTRA Cell 4 Ês4 / Noc3: -infinity During T2: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 16dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 16dB E-UTRA Cell 3 Noc3: -101dBm/15kHz Ês3 / Noc3: 16dB E-UTRA Cell 4 Ês4 / Noc3: 16dB During T3: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 16dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 16dB E-UTRA Cell 3 Noc3: -101dBm/15kHz Ês3 / Noc3:16dB E-UTRA Cell 4 Ês4 / Noc3: -infinity During T4: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 16dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 16dB E-UTRA Cell 3 Noc3: -101dBm/15kHz Ês3 / Noc3: 16dB E-UTRA Cell 4 Ês4 / Noc3: 16dB |  |  |  |  | During T1: 0dB 0dB 0dB 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB |  |  |  |  | During T1: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 16dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 16 E-UTRA Cell 3 Noc3: -101dBm/15kHz Ês3 / Noc3: 16dB E-UTRA Cell 4 Ês4 / Noc3: -infinity During T2: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 16dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 16dB E-UTRA Cell 3 Noc3: -101dBm/15kHz Ês3 / Noc3: 16dB E-UTRA Cell 4 Ês4 / Noc3: 16dB During T3: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 16dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 16dB E-UTRA Cell 3 Noc3: -101dBm/15kHz Ês3 / Noc3: 16dB E-UTRA Cell 4 Ês4 / Noc3: -infinity E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 16dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 16dB E-UTRA Cell 3 Noc3: -101dBm/15kHz Ês3 / Noc3: 16dB E-UTRA Cell 4 Ês4 / Noc3: 16dB |  |
| 8.16.32 E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in TDD |  |  | Same as 8.16.31 |  |  |  |  | Same as 8.16.31 |  |  |  |  | Same as 8.16.31 |  |
| 8.16.33 E-UTRAN FDD 3DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX |  |  | Same as 8.16.31 |  |  |  |  | Same as 8.16.31 |  |  |  |  | Same as 8.16.31 |  |
| 8.16.34 E-UTRAN TDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX |  |  | Same as 8.16.31 |  |  |  |  | Same as 8.16.31 |  |  |  |  | Same as 8.16.31 |  |
| 8.16.35 3 DL PCell in FDD CA Activation and Deactivation of Known SCell in Non-DRX |  |  | During T1, T2, T3: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 17dB E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: 17dB |  |  | During T1,T2,T3: 0dB 0dB 0dB 0dB 0dB 0dB |  |  |  |  |  |  | During T1, T2, T3: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 17dB E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: 17dB |  |
| 8.16.36 3 DL PCell in TDD CA Activation and Deactivation of Known SCell in Non-DRX |  |  | Same as 8.16.35 |  |  | Same as 8.16.35 |  |  |  |  |  |  | Same as 8.16.35 |  |
| 8.16.37 3DL FDD CA Activation and Deactivation of known SCell in non-DRX |  |  | Same as 8.16.35 |  |  | Same as 8.16.35 |  |  |  |  |  |  | Same as 8.16.35 |  |
| 8.16.38 3DL TDD CA Activation and Deactivation of known SCell in non-DRX |  |  | Same as 8.16.35 |  |  | Same as 8.16.35 |  |  |  |  |  |  | Same as 8.16.35 |  |
| 8.16.39 E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in FDD |  |  | During T1: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: -infinity E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: 17dB During T2, T3: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 17dB E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: 17dB |  |  |  |  | During T1: 0dB 0dB 0dB 0dB 0dB 0dB During T2, T3: 0dB 0dB 0dB 0dB 0dB 0dB |  |  |  |  | During T1: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: -infinity E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: 17dB During T2, T3: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 17dB E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: 17dB |  |
| 8.16.40 E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in TDD |  |  | Same as 8.16.39 |  |  |  |  | Same as 8.16.39 |  |  |  |  | Same as 8.16.39 |  |
| 8.16.41 3 DL FDD CA activation and deactivation of unknown SCell in non-DRX |  |  | Same as 8.16.39 |  |  |  |  | Same as 8.16.39 |  |  |  |  | Same as 8.16.39 |  |
| 8.16.42 3 DL TDD CA activation and deactivation of unknown SCell in non-DRX |  |  | Same as 8.16.39 |  |  |  |  | Same as 8.16.39 |  |  |  |  | Same as 8.16.39 |  |
| 8.16.47 2DL/2UL FDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX |  |  | Same as 8.16.17 |  |  |  |  | Same as 8.16.17 |  |  |  |  | Same as 8.16.17 |  |
| 8.16.48 2DL/2UL TDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX |  |  | Same as 8.16.17 |  |  |  |  | Same as 8.16.17 |  |  |  |  | Same as 8.16.17 |  |
| 8.16.49 2DL/2UL TDD-FDD CA (FDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX |  |  | Same as 8.16.17 |  |  |  |  | Same as 8.16.17 |  |  |  |  | Same as 8.16.17 |  |
| 8.16.50 2DL/2UL TDD-FDD CA (TDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX |  |  | Same as 8.16.17 |  |  |  |  | Same as 8.16.17 |  |  |  |  | Same as 8.16.17 |  |
| 8.16.51 E-UTRAN 4 DL FDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX |  |  | During T1: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: -infinity E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: 17dB E-UTRA Cell 4 Noc4: -104dBm/15kHz Ês4 / Noc4: 17dB E-UTRA Cell 5 Ês5 / Noc4: -infinity During T2: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 17dB E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: 17dB E-UTRA Cell 4 Noc4: -104dBm/15kHz Ês4 / Noc4: 17dB E-UTRA Cell 5 Ês5 / Noc4: 17dB During T3: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: -3dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: -3dB E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: -3dB E-UTRA Cell 4 Noc4: -104dBm/15kHz Ês4 / Noc4: -3dB E-UTRA Cell 5 Ês5 / Noc4: -3dB |  |  |  |  | During T1: 0dB 0dB 0dB 0dB 0dB +0.2dB -0.5dB +0.7dB 0dB During T2: 0dB 0dB 0dB +0.2dB 0dB +0.2dB -0.5dB +0.7dB 0dB During T3: 0dB 0dB 0dB -0.2dB 0dB -0.2dB -0.5dB -0.7dB -0.7dB |  |  |  |  | During T1: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: -infinity E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: 17.2dB E-UTRA Cell 4 Noc4: -104.5dBm/15kHz Ês4 / Noc4: 17.7dB E-UTRA Cell 5 Ês5 / Noc4: -infinity During T2: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 17.2dB E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: 17.2dB E-UTRA Cell 4 Noc4: -104.5dBm/15kHz Ês4 / Noc4: 17.7dB E-UTRA Cell 5 Ês5 / Noc4: 17dB During T3: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: -3dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: -3.2dB E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: -3.2dB E-UTRA Cell 4 Noc4: -104.5dBm/15kHz Ês4 / Noc4: -3.7dB E-UTRA Cell 5 Ês5 / Noc4: -3.7dB |  |
| 8.16.52 E-UTRAN 4 DL TDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX |  |  | Same as 8.16.51 |  |  |  |  | Same as 8.16.51 |  |  |  |  |  |  |
| 8.16.53 4 DL PCell in FDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX |  |  | Same as 8.16.51 |  |  |  |  | Same as 8.16.51 |  |  |  |  |  |  |
| 8.16.54 4 DL PCell in TDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX |  |  | Same as 8.16.51 |  |  |  |  | Same as 8.16.51 |  |  |  |  |  |  |
| 8.16.55 4DL FDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX |  |  | During T1: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 16dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 16dB E-UTRA Cell 3 Noc3: -101dBm/15kHz Ês3 / Noc3: 16dB E-UTRA Cell 4 Noc4: -101dBm/15kHz Ês4 / Noc4: 16dB E-UTRA Cell 5 Ês5 / Noc4: -infinity During T2: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 16dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 16dB E-UTRA Cell 3 Noc3: -101dBm/15kHz Ês3 / Noc3: 16dB E-UTRA Cell 4 Noc4: -101dBm/15kHz Ês4 / Noc4: 16dB E-UTRA Cell 5 Ês5 / Noc4: 16dB During T3: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 16dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 16dB E-UTRA Cell 3 Noc3: -101dBm/15kHz Ês3 / Noc3:16dB E-UTRA Cell 4 Noc4: -101dBm/15kHz Ês4 / Noc4:16dB E-UTRA Cell 5 Ês5 / Noc4: -infinity During T4: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 16dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 16dB E-UTRA Cell 3 Noc3: -101dBm/15kHz Ês3 / Noc3: 16dB E-UTRA Cell 4 Noc4: -101dBm/15kHz Ês4 / Noc4: 16dB E-UTRA Cell 5 Ês5 / Noc4: 16dB |  |  |  |  | During T1: 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB |  |  |  |  | During T1: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 16dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 16 E-UTRA Cell 3 Noc3: -101dBm/15kHz Ês3 / Noc3: 16dB E-UTRA Cell 4 Noc4: -101dBm/15kHz Ês4 / Noc4: 16dB E-UTRA Cell 5 Ês5 / Noc4: -infinity During T2: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 16dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 16dB E-UTRA Cell 3 Noc3: -101dBm/15kHz Ês3 / Noc3: 16dB E-UTRA Cell 4 Noc4: -101dBm/15kHz Ês4 / Noc4: 16dB E-UTRA Cell 5 Ês5 / Noc4: 16dB During T3: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 16dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 16dB E-UTRA Cell 3 Noc3: -101dBm/15kHz Ês3 / Noc3: 16dB E-UTRA Cell 4 Noc4: -101dBm/15kHz Ês4 / Noc4: 16dB E-UTRA Cell 5 Ês5 / Noc4: -infinity E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 16dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 16dB E-UTRA Cell 3 Noc3: -101dBm/15kHz Ês3 / Noc3: 16dB E-UTRA Cell 4 Noc4: -101dBm/15kHz Ês4 / Noc4: 16dB E-UTRA Cell 5 Ês5 / Noc4: 16dB |  |
| 8.16.56 4DL TDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX |  |  | Same as 8.16.55 |  |  |  |  | Same as 8.16.55 |  |  |  |  | Same as 8.16.55 |  |
| 8.16.57 4DL FDD CA Activation and Deactivation of Known SCell in Non-DRX |  |  | During T1, T2, T3: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 17dB E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: 17dB E-UTRA Cell 4 Noc4: -104dBm/15kHz Ês4 / Noc4: 17dB |  |  |  |  | During T1,T2,T3: 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB |  |  |  |  | During T1, T2, T3: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 17dB E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: 17dB E-UTRA Cell 4 Noc4: -104dBm/15kHz Ês4 / Noc4: 17dB |  |
| 8.16.58 4DL TDD CA Activation and Deactivation of Known SCell in Non-DRX |  |  | Same as 8.16.57 |  |  |  |  | Same as 8.16.57 |  |  |  |  | Same as 8.16.57 |  |
| 8.16.59 4DL PCell in FDD CA Activation and Deactivation of Known SCell in Non-DRX |  |  | Same as 8.16.57 |  |  |  |  | Same as 8.16.57 |  |  |  |  | Same as 8.16.57 |  |
| 8.16.60 4DL PCell in TDD CA Activation and Deactivation of Known SCell in Non-DRX |  |  | Same as 8.16.57 |  |  |  |  | Same as 8.16.57 |  |  |  |  | Same as 8.16.57 |  |
| 8.16.61 4DL FDD CA Activation and Deactivation of Unknown SCell in Non-DRX |  |  | During T1: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: -infinity E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: 17dB E-UTRA Cell 4 Noc4: -104dBm/15kHz Ês4 / Noc4: 17dB During T2, T3: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 17dB E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: 17dB E-UTRA Cell 4 Noc4: -104dBm/15kHz Ês4 / Noc4: 17dB |  |  |  |  | During T1: 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB During T2, T3: 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB |  |  |  |  | During T1: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: -infinity E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: 17dB E-UTRA Cell 4 Noc4: -104dBm/15kHz Ês4 / Noc4: 17dB During T2, T3: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 17dB E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: 17dB E-UTRA Cell 4 Noc4: -104dBm/15kHz Ês4 / Noc4: 17dB |  |
| 8.16.62 4DL TDD CA Activation and Deactivation of Unknown SCell in Non-DRX |  |  | Same as 8.16.61 |  |  |  |  | Same as 8.16.61 |  |  |  |  | Same as 8.16.61 |  |
| 8.16.63 4DL PCell in FDD CA Activation and Deactivation of Unknown SCell in Non-DRX |  |  | Same as 8.16.61 |  |  |  |  | Same as 8.16.61 |  |  |  |  | Same as 8.16.61 |  |
| 8.16.64 4DL PCell in TDD CA Activation and Deactivation of Unknown SCell in Non-DRX |  |  | Same as 8.16.61 |  |  |  |  | Same as 8.16.61 |  |  |  |  | Same as 8.16.61 |  |
| 8.16.65 5 DL PCell in FDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX |  |  | During T1: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: -infinity E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: 17dB E-UTRA Cell 4 Noc4: -104dBm/15kHz Ês4 / Noc4: 17dB E-UTRA Cell 5 Noc5: -104dBm/15kHz Ês5 / Noc5: 17dB E-UTRA Cell 6 Ês6 / Noc5: -infinity During T2: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 17dB E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: 17dB E-UTRA Cell 4 Noc4: -104dBm/15kHz Ês4 / Noc4: 17dB E-UTRA Cell 5 Noc5: -104dBm/15kHz Ês5 / Noc5: 17dB E-UTRA Cell 6 Ês6 / Noc5: 17dB During T3: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: -3dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: -3dB E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: -3dB E-UTRA Cell 4 Noc4: -104dBm/15kHz Ês4 / Noc4: -3dB E-UTRA Cell 5 Noc5: -104dBm/15kHz Ês5 / Noc5: -3dB E-UTRA Cell 6 Ês6 / Noc5: -3dB |  |  |  |  | During T1: 0dB 0dB 0dB 0dB 0dB +0.2dB 0dB +0.2dB -0.5dB +0.7dB 0dB During T2: 0dB 0dB 0dB +0.2dB 0dB +0.2dB 0dB +0.2dB -0.5dB +0.7dB 0dB During T3: 0dB 0dB 0dB -0.2dB 0dB -0.2dB 0dB -0.2dB -0.5dB -0.7dB -0.7dB |  |  |  |  | During T1: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: -infinity E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: 17.2dB E-UTRA Cell 4 Noc4: -104.5dBm/15kHz Ês4 / Noc4: 17.7dB E-UTRA Cell 5 Noc5: -104.5dBm/15kHz Ês5 / Noc5: 17.7dB E-UTRA Cell 6 Ês6 / Noc5: -infinity During T2: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 17.2dB E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: 17.2dB E-UTRA Cell 4 Noc4: -104dBm/15kHz Ês4 / Noc4: 17.2dB E-UTRA Cell 5 Noc5: -104.5dBm/15kHz Ês5 / Noc5: 17.7dB E-UTRA Cell 6 Ês6 / Noc5: 17dB During T3: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: -3dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: -3.2dB E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: -3.2dB E-UTRA Cell 4 Noc4: -104dBm/15kHz Ês4 / Noc4: -3.2dB E-UTRA Cell 5 Noc5: -104.5dBm/15kHz Ês5 / Noc5: -3.7dB E-UTRA Cell 6 Ês6 / Noc5: -3.7dB |  |
| 8.16.66 5 DL PCell in TDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX |  |  | Same as 8.16.65 |  |  |  |  | Same as 8.16.65 |  |  |  |  | Same as 8.16.65 |  |
| 8.16.67 5DL Pcell in FDD CA activation and deactivation of unknown SCell in non-DRX |  |  | During T1: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: -infinity E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: 17dB E-UTRA Cell 4 Noc4: -104dBm/15kHz Ês4 / Noc4: 17dB E-UTRA Cell 5 Noc5: -104dBm/15kHz Ês5 / Noc5: 17dB During T2, T3: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 17dB E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: 17dB E-UTRA Cell 4 Noc4: -104dBm/15kHz Ês4 / Noc4: 17dB E-UTRA Cell 5 Noc5: -104dBm/15kHz Ês5 / Noc5: 17dB |  |  |  |  | During T1: 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB During T2, T3: 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB |  |  |  |  | During T1: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: -infinity E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: 17dB E-UTRA Cell 4 Noc4: -104dBm/15kHz Ês4 / Noc4: 17dB Noc5: -104dBm/15kHz Ês5 / Noc5: 17dB During T2, T3: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 17dB E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: 17dB E-UTRA Cell 4 Noc4: -104dBm/15kHz Ês4 / Noc4: 17dB E-UTRA Cell 5 Noc5: -104dBm/15kHz Ês5 / Noc5: 17dB |  |
| 8.16.68 5DL Pcell in TDD CA activation and deactivation of unknown SCell in non-DRX |  |  | Same as 8.16.67 |  |  |  |  | Same as 8.16.67 |  |  |  |  | Same as 8.16.67 |  |
| 8.16.69 5DL FDD CA activation and deactivation of unknown SCell in non-DRX |  |  | Same as 8.16.67 |  |  |  |  | Same as 8.16.67 |  |  |  |  | Same as 8.16.67 |  |
| 8.16.70 5DL TDD CA activation and deactivation of unknown SCell in non-DRX |  |  | Same as 8.16.67 |  |  |  |  | Same as 8.16.67 |  |  |  |  | Same as 8.16.67 |  |
| 8.16.71 5 DL FDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX |  |  | Same as 8.16.65 |  |  |  |  | Same as 8.16.65 |  |  |  |  | Same as 8.16.65 |  |
| 8.16.72 5 DL TDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX |  |  | Same as 8.16.65 |  |  |  |  | Same as 8.16.65 |  |  |  |  | Same as 8.16.65 |  |
| 8.16.73 5DL FDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX |  |  | During T1, T2, T3 and T4: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 16dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 16dB E-UTRA Cell 3 Noc3: -101dBm/15kHz Ês3 / Noc3: 16dB E-UTRA Cell 4 Noc4: -101dBm/15kHz Ês4 / Noc4: 16dB E-UTRA Cell 5 Noc5: -101dBm/15kHz Ês5 / Noc5: 16dB During T1 and T3: E-UTRA Cell 6 Ês6 / Noc5: -infinity During T2 and T4: E-UTRA Cell 6 Ês6 / Noc5: 16dB |  |  |  |  | During T1, T2, T3 and T4: 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB During T1 and T3: 0dB During T2 and T2: 0dB |  |  |  |  | During T1, T2, T3 and T4: E-UTRA Cell 1 Noc1: -101dBm/15kHz Ês1 / Noc1: 16dB E-UTRA Cell 2 Noc2: -101dBm/15kHz Ês2 / Noc2: 16dB E-UTRA Cell 3 Noc3: -101dBm/15kHz Ês3 / Noc3: 16dB E-UTRA Cell 4 Noc4: -101dBm/15kHz Ês4 / Noc4: 16dB E-UTRA Cell 5 Noc5: -101dBm/15kHz Ês5 / Noc5: 16dB During T1 and T3: E-UTRA Cell 6 Ês6 / Noc5: -infinity During T2 and T4: E-UTRA Cell 6 Ês6 / Noc5: 16dB |  |
| 8.16.74 5DL TDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX |  |  | Same as 8.16.73 |  |  |  |  | Same as 8.16.73 |  |  |  |  | Same as 8.16.73 |  |
| 8.16.75 5DL PCell in FDD CA activation and deactivation of known SCell in non-DRX |  |  | During T1, T2, T3: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 17dB E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: 17dB E-UTRA Cell 4 Noc4: -104dBm/15kHz Ês4 / Noc4: 17dB E-UTRA Cell 5 Noc5: -104dBm/15kHz Ês5 / Noc5: 17dB |  |  |  |  | During T1,T2,T3: 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB |  |  |  |  | During T1, T2, T3: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 17dB E-UTRA Cell 3 Noc3: -104dBm/15kHz Ês3 / Noc3: 17dB E-UTRA Cell 4 Noc4: -104dBm/15kHz Ês4 / Noc4: 17dB E-UTRA Cell 5 Noc5: -104dBm/15kHz Ês5 / Noc5: 17dB |  |
| 8.16.76 5DL PCell in TDD CA activation and deactivation of known SCell in non-DRX |  |  | Same as 8.16.75 |  |  |  |  | Same as 8.16.75 |  |  |  |  | Same as 8.16.75 |  |
| 8.16.77 5DL FDD CA activation and deactivation of known SCell in non-DRX |  |  | Same as 8.16.75 |  |  |  |  | Same as 8.16.75 |  |  |  |  | Same as 8.16.75 |  |
| 8.16.78 E-UTRAN TDD 5DL CA activation and deactivation of known SCell in non-DRX |  |  | Same as 8.16.75 |  |  |  |  | Same as 8.16.75 |  |  |  |  | Same as 8.16.75 |  |
| 8.16.83 3 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes |  |  | Same as 8.16.27 |  |  |  |  | Same as 8.16.27 |  |  |  |  | Same as 8.16.27 The Test Tolerance values are taken from the analysis for the related 3DL Test case 8.16.27, and no separate analysis is needed. |  |
| 8.16.84 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes |  |  | Same as 8.16.31 |  |  |  |  | Same as 8.16.31 |  |  |  |  | Same as 8.16.31 The Test Tolerance values are taken from the analysis for the related 3DL Test case 8.16.31, and no separate analysis is needed. |  |
| 8.16.85 3 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes |  |  | Same as 8.16.35 |  |  |  |  | Same as 8.16.35 |  |  |  |  | Same as 8.16.35 The Test Tolerance values are taken from the analysis for the related 3DL Test case 8.16.35, and no separate analysis is needed. |  |
| 8.16.86 3 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes |  |  | Same as 8.16.39 |  |  |  |  | Same as 8.16.39 |  |  |  |  | Same as 8.16.39 The Test Tolerance values are taken from the analysis for the related 3DL Test case 8.16.39, and no separate analysis is needed. |  |
| 8.16.87 4 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes |  |  | Same as 8.16.51 |  |  |  |  | Same as 8.16.51 |  |  |  |  | Same as 8.16.51 |  |
| 8.16.88 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes |  |  | Same as 8.16.55 |  |  |  |  | Same as 8.16.55 |  |  |  |  | Same as 8.16.55 |  |
| 8.16.91 5 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes |  |  | Same as 8.16.65 |  |  |  |  | Same as 8.16.65 |  |  |  |  | Same as 8.16.65 |  |
| 8.16.92 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes |  |  | Same as 8.16.73 |  |  |  |  | Same as 8.16.73 |  |  |  |  | Same as 8.16.73 |  |
| 8.16.89 4 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes |  |  | Same as 8.16.57 |  |  |  |  | Same as 8.16.57 |  |  |  |  | Same as 8.16.57 The Test Tolerance values are taken from the analysis for the related 3DL Test case 8.16.35, and no separate analysis is needed. |  |
| 8.16.90 4 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes |  |  | Same as 8.16.61 |  |  |  |  | Same as 8.16.61 |  |  |  |  | Same as 8.16.61 The Test Tolerance values are taken from the analysis for the related 3DL Test case 8.16.39, and no separate analysis is needed. |  |
| 8.16.93 5 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes |  |  | Same as 8.16.75 |  |  |  |  | Same as 8.16.75 |  |  |  |  | Same as 8.16.75 The Test Tolerance values are taken from the analysis for the related 3DL Test case 8.16.35, and no separate analysis is needed. |  |
| 8.16.94 5 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes |  |  | Same as 8.16.67 |  |  |  |  | Same as 8.16.67 |  |  |  |  | Same as 8.16.67 The Test Tolerance values are taken from the analysis for the related 3DL Test case 8.16.39, and no separate analysis is needed. |  |
| 8.16.95 6DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes |  |  | Same as 8.16.71 |  |  |  |  | Same as 8.16.71 |  |  |  |  | Same as 8.16.71 The Test Tolerance values are taken from the analysis for the related 5DL Test case 8.16.71, and no separate analysis is needed. |  |
| 8.16.96 6 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes |  |  | Same as 8.16.73 |  |  |  |  | Same as 8.16.73 |  |  |  |  | Same as 8.16.73 The Test Tolerance values are taken from the analysis for the related 5DL Test case 8.16.73, and no separate analysis is needed. |  |
| 8.16.97 6 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes |  |  | Same as 8.16.67 |  |  |  |  | Same as 8.16.67 |  |  |  |  | Same as 8.16.67 The Test Tolerance values are taken from the analysis for the related 3DL Test case 8.16.39, and no separate analysis is needed. |  |
| 8.16.98 6 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes |  |  | Same as 8.16.67 |  |  |  |  | Same as 8.16.67 |  |  |  |  | Same as 8.16.67 The Test Tolerance values are taken from the analysis for the related 3DL Test case 8.16.39, and no separate analysis is needed. |  |
| 8.16.99 7DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes |  |  | Same as 8.16.71 |  |  |  |  | Same as 8.16.71 |  |  |  |  | Same as 8.16.71 The Test Tolerance values are taken from the analysis for the related 5DL Test case 8.16.71, and no separate analysis is needed. |  |
| 8.16.100 7 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes |  |  | Same as 8.16.73 |  |  |  |  | Same as 8.16.73 |  |  |  |  | Same as 8.16.73 The Test Tolerance values are taken from the analysis for the related 5DL Test case 8.16.73, and no separate analysis is needed. |  |
| 8.16.101 7 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes |  |  | Same as 8.16.67 |  |  |  |  | Same as 8.16.67 |  |  |  |  | Same as 8.16.67 The Test Tolerance values are taken from the analysis for the related 3DL Test case 8.16.39, and no separate analysis is needed. |  |
| 8.16.102 7 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes |  |  | Same as 8.16.67 |  |  |  |  | Same as 8.16.67 |  |  |  |  | Same as 8.16.67 The Test Tolerance values are taken from the analysis for the related 3DL Test case 8.16.39, and no separate analysis is needed. |  |
| 8.20.1 UE Measurement Procedures / RAT Measurements in CA mode / E-UTRAN FDD-FDD inter frequency event triggered reporting under fading propagation conditions in asynchronous cells |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 4dB E-UTRA Cell 2 Noc: -98dBm/15kHz Ês / Noc: -infinity E-UTRA Cell 3 Noc: -98dBm/15kHz Ês / Noc: 4dB During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 4dB E-UTRA Cell 2 Noc: -98dBm/15kHz Ês / Noc: 7dB E-UTRA Cell 3 Noc: -98dBm/15kHz Ês / Noc: 4dB |  |  |  |  | During T1: 0dB 0dB 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0.7dB 0dB 0dB |  |  |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 4dB E-UTRA Cell 2 Noc: -98dBm/15kHz Ês / Noc: -infinity E-UTRA Cell 3 Noc: -98dBm/15kHz Ês / Noc: 4dB During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: 4dB E-UTRA Cell 2 Noc: -98dBm/15kHz Ês / Noc: 7.7dB E-UTRA Cell 3 Noc: -98dBm/15kHz Ês / Noc: 4dB |  |
| 8.20.2 UE Measurement Procedures / RAT Measurements in CA mode / E-UTRAN TDD-TDD inter frequency event triggered reporting under fading propagation conditions in asynchronous cells |  |  | Same as 8.20.1 |  |  |  |  | Same as 8.20.1 |  |  |  |  | Same as 8.20.1 |  |
| 8.20.2A UE Measurement Procedures / RAT Measurements in CA mode / E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +20 MHz bandwidth |  |  | Same as 8.20.1 |  |  |  |  | Same as 8.20.1 |  |  |  |  | Same as 8.20.1 |  |
| 8.20.2B UE Measurement Procedures / RAT Measurements in CA mode / E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +10 MHz bandwidth |  |  | Same as 8.20.1 |  |  |  |  | Same as 8.20.1 |  |  |  |  | Same as 8.20.1 |  |
| 8.20.3 E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4.00dB E-UTRA Cell 3 Noc: -98dBm/15kHz Ês / Noc: +4.00dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4.00dB E-UTRA Cell 3 Noc: -98dBm/15kHz Ês / Noc: +4.00dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc: -1.8 dB |  |  |  |  | During T1: 0dB 0dB 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB 0dB 0dB |  |  |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4.00dB E-UTRA Cell 3 Noc: -98dBm/15kHz Ês / Noc: +4.00dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc: -infinity During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4.00dB E-UTRA Cell 3 Noc: -98dBm/15kHz Ês / Noc: +4.00dB UTRA Cell 2 Ioc: -70dBm/3.84MHz Ior / Ioc: -1.8 dB |  |
| 8.20.4 E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +9.00dB E-UTRA Cell 3 Noc: -98dBm/15kHz Ês / Noc: +9.00dB UTRA Cell 2 Ioc: -80dBm/1.28MHz Ior / Ioc: -infinity PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +9.00dB E-UTRA Cell 3 Noc: -98dBm/15kHz Ês / Noc: +9.00dB UTRA Cell 2 Ioc: -80dBm/1.28MHz Ior / Ioc: 5.00dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB |  |  |  |  | During T1: 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB |  |  |  |  | During T1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +9.00dB E-UTRA Cell 3 Noc: -98dBm/15kHz Ês / Noc: +9.00dB UTRA Cell 2 Ioc: -80dBm/1.28MHz Ior / Ioc: -infinity PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB During T2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +9.00dB E-UTRA Cell 3 Noc: -98dBm/15kHz Ês / Noc: +9.00dB UTRA Cell 2 Ioc: -80dBm/1.28MHz Ior / Ioc: 5.00dB PCCPCH_Ec/Ior: -3dB DwPCH_Ec/Ior: 0dB |  |
| 8.20.4A E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions for 20 MHz + 20 MHz bandwidth |  |  | Same as 8.20.4 |  |  |  |  | Same as 8.20.4 |  |  |  |  | Same as 8.20.4 |  |
| 8.20.4B E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions for 20 MHz + 10 MHz bandwidth |  |  | Same as 8.20.4 |  |  |  |  | Same as 8.20.4 |  |  |  |  | Same as 8.20.4 |  |
| 8.22.1 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal |  |  | Same as 8.1.1 |  |  |  |  | Same as 8.1.1 |  |  |  |  | Same as 8.1.1 |  |
| 8.22.2 E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal |  |  | Same as 8.1.1 |  |  |  |  | Same as 8.1.1 |  |  |  |  | Same as 8.1.1 |  |
| 8.22.3 E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.00dB |  |  |  |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB |  |  |  |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +4.00dB Noc2: -98dBm/15kHz Ês2 / Noc2: +7.00dB |  |
| 8.22.4 E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal |  |  | Same as 8.22.3 |  |  |  |  | Same as 8.22.3 |  |  |  |  | Same as 8.22.3 |  |
| 8.22.5 E-UTRAN FDD-FDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal |  |  | During T1: Noc: -98dBm/15kHz CRS Ês1 / Noc: +4.00dB CRS Ês2 / Noc: -infinity CSI-RS Ês1 / Noc: +10.00dB CSI-RS Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB CSI-RS Ês1 / Noc: +10.00dB CSI-RS Ês2 / Noc: +10.00dB |  |  |  |  | During T1: 0dB 2.10dB 0dB 2.10dB 2.10dB During T2: 0dB 2.10dB 2.10dB 2.10dB 2.10dB |  |  |  |  | During T1: Noc: -98dBm/15kHz CRS Ês1 / Noc: +6.10dB CRS Ês2 / Noc: -infinity CSI-RS Ês1 / Noc: +12.10dB CSI-RS Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +6.10dB Ês2 / Noc: +6.10dB CSI-RS Ês1 / Noc: +12.10dB CSI-RS Ês2 / Noc: +12.10dB |  |
| 8.22.6 E-UTRAN TDD-TDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity CSI-RS Ês1 / Noc: +10.00dB CSI-RS Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB CSI-RS Ês1 / Noc: +10.00dB CSI-RS Ês2 / Noc: +10.00dB |  |  |  |  | During T1: 0dB 2.60dB 0dB 2.60dB 0dB During T2: 0dB 2.60dB 2.60dB 2.60dB 2.60dB |  |  |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +6.60dB Ês2 / Noc: -infinity CSI-RS Ês1 / Noc: +12.60dB CSI-RS Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +6.60dB Ês2 / Noc: +6.60dB CSI-RS Ês1 / Noc: +12.60dB CSI-RS Ês2 / Noc: +12.60dB |  |
| 8.22.7 E-UTRAN FDD-FDD inter-frequency event triggered reporting in DRX based on CSI-RS based discovery signal |  |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHzCRS Ês1 / Noc1: +4.00dB CRS Ês2 / Noc2: -infinity CSI-RS Ês1 / Noc1: +10.00dB CSI-RS Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4.00dB Ês2 / Noc2: +7.00dB CSI-RS Ês1 / Noc1: +10.00dB CSI-RS Ês2 / Noc2: +13.00dB |  |  |  |  | During T1: 0dB 0dB 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0.2dB 0dB 0.2dB |  |  |  |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHzCRS Ês1 / Noc1: +4dB CRS Ês2 / Noc2: -infinity CSI-RS Ês1 / Noc1: +10.00dB CSI-RS Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +7.2dB CSI-RS Ês1 / Noc1: +10.00dB CSI-RS Ês2 / Noc2: +13.20dB |  |
| 8.22.8 E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation condition in DRX based on CSI-RS based discovery signal |  |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHzCRS Ês1 / Noc1: +4.00dB CRS Ês2 / Noc2: -infinity CSI-RS Ês1 / Noc1: +10.00dB CSI-RS Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4.00dB Ês2 / Noc2: +7.00dB CSI-RS Ês1 / Noc1: +10.00dB CSI-RS Ês2 / Noc2: +13.00dB |  |  |  |  | During T1: 0dB 0dB 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB 0dB 0dB |  |  |  |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHzCRS Ês1 / Noc1: +4dB CRS Ês2 / Noc2: -infinity CSI-RS Ês1 / Noc1: +10.00dB CSI-RS Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +7.2dB CSI-RS Ês1 / Noc1: +10.00dB CSI-RS Ês2 / Noc2: +13.20dB |  |
| 8.22.9 E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal |  |  | Same as 8.16.1 |  |  |  |  | Same as 8.16.1 |  |  |  |  | Same as 8.16.1 |  |
| 8.22.10 E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal |  |  | Same as 8.16.1 |  |  |  |  | Same as 8.16.1 |  |  |  |  | Same as 8.16.1 |  |
| 8.22.11 E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal |  |  | During T1: E-UTRA Cell 1 Noc1: -101dBm/15kHz CRS Ês1 / Noc1: -3dB CSI-RS Ês1 / Noc1: 3dB E-UTRA Cell 2 Noc2: -101dBm/15kHz CRS Ês2 / Noc2: -3dB CSI-RS Ês2 / Noc2: 3dB E-UTRA Cell 3 CRS Ês3 / Noc2: -infinity CSI-RS Ês3 / Noc2: -infinity During T2: E-UTRA Cell 1 Noc1: -101dBm/15kHz CRS Ês1 / Noc1: 17dB CSI-RS Ês1 / Noc1: 23dB E-UTRA Cell 2 Noc2: -101dBm/15kHz CRS Ês2 / Noc2: 17dB CSI-RS Ês2 / Noc2: 23dB E-UTRA Cell 3 CRS Ês3 / Noc2: -infinity CSI-RS Ês3 / Noc2: -infinity During T3: E-UTRA Cell 1 Noc1: -101dBm/15kHz CRS Ês1 / Noc1: 17dB CSI-RS Ês1 / Noc1: 23dB E-UTRA Cell 2 Noc2: -101dBm/15kHz CRS Ês2 / Noc2: 17dB CSI-RS Ês2 / Noc2: 23dB E-UTRA Cell 3 CRS Ês3 / Noc2: 17dB CSI-RS Ês3 / Noc2: 23dB |  |  |  |  | During T1: 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB |  |  |  |  | During T1: E-UTRA Cell 1 Noc1: -101dBm/15kHz CRS Ês1 / Noc1: -3dB CSI-RS Ês1 / Noc1: 3dB E-UTRA Cell 2 Noc2: -101dBm/15kHz CRS Ês2 / Noc2: -3dB CSI-RS Ês2 / Noc2: 3dB E-UTRA Cell 3 CRS Ês3 / Noc2: -infinity CSI-RS Ês3 / Noc2: -infinity During T2: E-UTRA Cell 1 Noc1: -101dBm/15kHz CRS Ês1 / Noc1: 17dB CSI-RS Ês1 / Noc1: 23dB E-UTRA Cell 2 Noc2: -101dBm/15kHz CRS Ês2 / Noc2: 17dB CSI-RS Ês2 / Noc2: 23dB E-UTRA Cell 3 CRS Ês3 / Noc2: -infinity CSI-RS Ês3 / Noc2: -infinity During T3: E-UTRA Cell 1 Noc1: -101dBm/15kHz CRS Ês1 / Noc1: 17dB CSI-RS Ês1 / Noc1: 23dB E-UTRA Cell 2 Noc2: -101dBm/15kHz CRS Ês2 / Noc2: 17dB CSI-RS Ês2 / Noc2: 23dB E-UTRA Cell 3 CRS Ês3 / Noc2: 17dB CSI-RS Ês3 / Noc2: 23dB |  |
| 8.22.12 E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal |  |  | Same as 8.22.11 |  |  |  |  | Same as 8.22.11 |  |  |  |  | Same as 8.22.11 |  |
| 8.23.1 E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC |  |  | During T1: E-UTRA Cell 1 Noc: -104dBm/15kHz Ês / Noc: 16dB E-UTRA Cell 2 Noc: -104dBm/15kHz Ês / Noc: 16dB During T2: E-UTRA Cell 1 Noc: -104dBm/15kHz Ês / Noc: -2.5dB E-UTRA Cell 2 Noc: -104dBm/15kHz Ês / Noc: 2.5dB During T3: E-UTRA Cell 1 Noc: -104dBm/15kHz Ês / Noc: 20dB E-UTRA Cell 2 Noc: -104dBm/15kHz Ês / Noc: 20dB |  |  |  |  | During T1: 0dB 0.2dB 0dB 0.2dB During T2: 0dB -0.2dB 0dB -0.2dB During T3: 0dB 0.2dB 0dB 0.2dB |  |  |  |  | During T1: E-UTRA Cell 1 Noc: -104dBm/15kHz Ês / Noc: 16.2dB E-UTRA Cell 2 Noc: -104dBm/15kHz Ês / Noc: 16.2dB During T2: E-UTRA Cell 1 Noc: -104dBm/15kHz Ês / Noc: -2.7dB E-UTRA Cell 2 Noc: -104dBm/15kHz Ês / Noc: -2.7dB During T3: E-UTRA Cell 1 Noc: -104dBm/15kHz Ês / Noc: 20.2dB E-UTRA Cell 2 Noc: -104dBm/15kHz Ês / Noc: 20.2dB |  |
| 8.23.2 E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in asynchronous DC |  |  | Same as 8.23.1 |  |  |  |  | Same as 8.23.1 |  |  |  |  | Same as 8.23.1 |  |
| 8.23.3 E-UTRAN TDD-TDD DC intra-frequency event triggered reporting with DRX in synchronous DC |  |  | Same as 8.23.1 |  |  |  |  | Same as 8.23.1 |  |  |  |  | Same as 8.23.1 |  |
| 8.23.4 E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC |  |  | During T1: E-UTRA Cell 1 Noc: -101dBm/15kHz Ês / Noc: 4dB E-UTRA Cell 2 Noc: -101dBm/15kHz Ês / Noc: 4dB E-UTRA Cell 3 Noc: -101dBm/15kHz Ês / Noc: -infinity During T2: E-UTRA Cell 1 Noc: -101dBm/15kHz Ês / Noc: 4dB E-UTRA Cell 2 Noc: -101dBm/15kHz Ês / Noc: 4dB E-UTRA Cell 3 Noc: -101dBm/15kHz Ês / Noc: 7dB |  |  |  |  | During T1: 0dB 0dB 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB 0dB 0dB |  |  |  |  | During T1: E-UTRA Cell 1 Noc: -101dBm/15kHz Ês / Noc: 4dB E-UTRA Cell 2 Noc: -101dBm/15kHz Ês / Noc: 4dB E-UTRA Cell 3 Noc: -101dBm/15kHz Ês / Noc: -infinity During T2: E-UTRA Cell 1 Noc: -101dBm/15kHz Ês / Noc: 4dB E-UTRA Cell 2 Noc: -101dBm/15kHz Ês / Noc: 4dB E-UTRA Cell 3 Noc: -101dBm/15kHz Ês / Noc: 7dB |  |
| 8.23.5 E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in asynchronous DC |  |  | Same as 8.23.4 |  |  |  |  | Same as 8.23.4 |  |  |  |  | Same as 8.23.4 |  |
| 8.23.6 E-UTRAN TDD-TDD DC inter-frequency event triggered reporting with DRX in synchronous DC |  |  | Same as 8.23.4 |  |  |  |  | Same as 8.23.4 |  |  |  |  | Same as 8.23.4 |  |
| 8.26.1 E-UTRAN FDD-FS3 Activation and deactivation of known FS3 SCell with FDD PCell in non-DRX |  |  | During T1: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 17dB During T2: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 17dB |  |  |  |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB |  |  |  |  | During T1: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 17dB During T2: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 17dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 17dB |  |
| 8.26.2 E-UTRAN TDD-FS3 Activation and deactivation of known FS3 SCell with TDD PCell in non-DRX |  |  | Same as 8.26.1 |  |  |  |  | Same as 8.26.1 |  |  |  |  | Same as 8.26.1 |  |
| 8.26.3 E-UTRAN FDD-FS3 Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX |  |  | During T1: E-UTRA Cell 1 Noc1: -98dBm/15kHz Ês1 / Noc1: 13dB E-UTRA Cell 2 Noc2: -98dBm/15kHz Ês2 / Noc2: 13dB E-UTRA Cell 3 Ês3 / Noc2: -infinity During T2: E-UTRA Cell 1 Noc1: -98dBm/15kHz Ês1 / Noc1: 13dB E-UTRA Cell 2 Noc2: -98dBm/15kHz Ês2 / Noc2: 13dB E-UTRA Cell 3 Ês3 / Noc2: 13dB |  |  |  |  | During T1: 0dB 0dB 0dB -3.1dB 0dB During T2: 0dB 0dB 0dB -3.1dB -3.1dB |  |  |  |  | During T1: E-UTRA Cell 1 Noc1: -98dBm/15kHz Ês1 / Noc1: 13dB E-UTRA Cell 2 Noc2: -98dBm/15kHz Ês2 / Noc2: 9.9dB E-UTRA Cell 3 Ês3 / Noc2: -infinity During T2: E-UTRA Cell 1 Noc1: -98dBm/15kHz Ês1 / Noc1: 13dB E-UTRA Cell 2 Noc2: -98dBm/15kHz Ês2 / Noc2: 9.9dB E-UTRA Cell 3 Ês3 / Noc2: 9.9dB |  |
| 8.26.3A E-UTRAN FDD-FS3 3-DL Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX |  |  | During T1: E-UTRA Cell 1 Noc1: -98dBm/15kHz Ês1 / Noc1: 13dB E-UTRA Cell 2 Noc2: -98dBm/15kHz Ês2 / Noc2: 13dB E-UTRA Cell 3 Noc2: -98dBm/15kHz Ês3 / Noc2: 13dB E-UTRA Cell 4 Ês4 / Noc2: -infinity During T2: E-UTRA Cell 1 Noc1: -98dBm/15kHz Ês1 / Noc1: 13dB E-UTRA Cell 2 Noc2: -98dBm/15kHz Ês2 / Noc2: 13dB E-UTRA Cell 3 Noc2: -98dBm/15kHz Ês3 / Noc2: 13dB E-UTRA Cell 4 Ês4 / Noc2: 13dB |  |  |  |  | During T1: 0dB 0dB 0dB -3.1dB 0dB -3.1dB 0dB During T2: 0dB 0dB 0dB -3.1dB 0dB -3.1dB -3.1dB |  |  |  |  | During T1: E-UTRA Cell 1 Noc1: -98dBm/15kHz Ês1 / Noc1: 13dB E-UTRA Cell 2 Noc2: -98dBm/15kHz Ês2 / Noc2: 9.9dB E-UTRA Cell 3 Noc2: -98dBm/15kHz Ês3 / Noc2: 9.9dB E-UTRA Cell 4 Ês4 / Noc2: -infinity During T2: E-UTRA Cell 1 Noc1: -98dBm/15kHz Ês1 / Noc1: 13dB E-UTRA Cell 2 Noc2: -98dBm/15kHz Ês2 / Noc2: 9.9dB E-UTRA Cell 3 Noc2: -98dBm/15kHz Ês3 / Noc2: 9.9dB E-UTRA Cell 4 Ês4 / Noc2: 9.9dB |  |
| 8.26.4 E-UTRAN TDD-FS3 Event triggered reporting on deactivated FS3 SCell and TDD PCell interruption in non-DRX |  |  | Same as 8.26.3 |  |  |  |  | Same as 8.26.3 |  |  |  |  | Same as 8.26.3 |  |
| 8.26.5 E-UTRAN FDD-TDD intra-frequency event triggered reporting in non-DRX for CRS based discovery signal under Operation with Frame Structure 3 |  |  | During T1: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 4dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 4dB E-UTRA Cell 3 Ês3 / Noc2: -infinity During T2: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 4dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 4dB E-UTRA Cell 3 Ês3 / Noc2: 4dB |  |  |  |  | During T1: 0dB 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB 0dB |  |  |  |  | During T1: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 4dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 4dB E-UTRA Cell 3 Ês3 / Noc2: -infinity During T2: E-UTRA Cell 1 Noc1: -104dBm/15kHz Ês1 / Noc1: 4dB E-UTRA Cell 2 Noc2: -104dBm/15kHz Ês2 / Noc2: 4dB E-UTRA Cell 3 Ês3 / Noc2: 4dB |  |
| 8.26.6 E-UTRAN TDD-TDD intra-frequency event triggered reporting in non-DRX for CRS based discovery signal under Operation with Frame Structure 3 |  |  | Same as 8.26.5 |  |  |  |  | Same as 8.26.5 |  |  |  |  | Same as 8.26.5 |  |
| 8.26.7 E-UTRAN FDD-FS3 intra-frequency event triggered reporting in DRX for CRS based discovery signal |  |  | Same as 8.26.5 |  |  |  |  | Same as 8.26.5 |  |  |  |  | Same as 8.26.5 |  |
| 8.26.8 E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal |  |  | Same as 8.26.5 |  |  |  |  | Same as 8.26.5 |  |  |  |  | Same as 8.26.5 |  |
| 8.26.9 E-UTRAN FDD-FS3 inter-frequency event triggered reporting under fading propagation conditions in synchronous cells |  |  | During T1: E-UTRA Cell 1 Noc1: -98dBm/15kHz Ês1 / Noc1: 4dB E-UTRA Cell 2 Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity dB During T2: E-UTRA Cell 1 Noc1: -98dBm/15kHz Ês1 / Noc1: 4dB E-UTRA Cell 2 Noc2: -98dBm/15kHz Ês2 / Noc2: 7dB |  |  |  |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB |  |  |  |  | During T1: E-UTRA Cell 1 Noc1: -98dBm/15kHz Ês1 / Noc1: 4dB E-UTRA Cell 2 Noc2: -98dBm/15kHz Ês2 / Noc2: -infinity dB During T2: E-UTRA Cell 1 Noc1: -98dBm/15kHz Ês1 / Noc1: 4dB E-UTRA Cell 2 Noc2: -98dBm/15kHz Ês2 / Noc2: 7dB |  |
| 8.26.10 E-UTRAN TDD-FS3 inter-frequency event triggered reporting under fading propagation conditions in synchronous cells |  |  | Same as 8.26.9 |  |  |  |  | Same as 8.26.9 |  |  |  |  | Same as 8.26.9 |  |
| 9.1.1.1 FDD Intra Frequency Absolute RSRP Accuracy |  |  | Test 1: Noc: -106dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported RSRP values: ±6dB Test 2: Noc: -88dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported RSRP values: ±8dB Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +3.0dB Ês2 / Noc: -1.0dB Reported RSRP values: ±6dB |  |  |  |  | Test 1: -1.0dB 0dB +1.0dB Via mapping Test 2: 0dB 0dB +1.0dB Via mapping Test 3: 0dB 0dB +0.8dB Via mapping |  |  |  |  | Test 1: Noc: -107.0dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +2.0dB RSRP_29 to RSRP_43 Test 2: Noc: -88dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +2.0dB RSRP_45 to RSRP_64 Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +3.0dB Ês2 / Noc: -0.2dB RSRP_17 to RSRP_32 RSRP_18 to RSRP_33 RSRP_19 to RSRP_34 RSRP_20 to RSRP_35 depending on operating band |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 2 RSRP from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for normal conditions. In all cases the RSRP values are 3dB wider at each end for extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.1.1_1 FDD Intra Frequency Absolute RSRP Accuracy (Rel 12 and forward) |  |  | Test 1: Noc: -106dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported RSRP values: ±4.5dB Test 2: Noc: -88dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported RSRP values: ±8dB Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +3.0dB Ês2 / Noc: -1.0dB Reported RSRP values: ±4.5dB |  |  |  |  | Test 1: -1.0dB 0dB +1.0dB Via mapping Test 2: 0dB 0dB +1.0dB Via mapping Test 3: 0dB 0dB +0.8dB Via mapping |  |  |  |  | Test 1: Noc: -107.0dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +2.0dB RSRP_29 to RSRP_42 Test 2: Noc: -88dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +2.0dB RSRP_45 to RSRP_64 Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +3.0dB Ês2 / Noc: -0.2dB RSRP_18 to RSRP_30 RSRP_19 to RSRP_31 RSRP_20 to RSRP_32 RSRP_21 to RSRP_33 RSRP_22 to RSRP_34 depending on operating band |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 2 RSRP from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for normal conditions. The RSRP values are recalculated for extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.1.1_2 FDD Intra Frequency Absolute RSRP Accuracy for UE category 1bis |  |  | Test 1: Noc: -106dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported RSRP values: ±5.5dB Test 2: Noc: -88dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported RSRP values: ±9dB Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +3.0dB Ês2 / Noc: -1.0dB Reported RSRP values: ±5.5dB |  |  |  |  | Test 1: -1.0dB 0dB +1.0dB Via mapping Test 2: 0dB 0dB +1.0dB Via mapping Test 3: 0dB 0dB +0.8dB Via mapping |  |  |  |  | Test 1: Noc: -107.0dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +2.0dB RSRP_28 to RSRP_43 Test 2: Noc: -88dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +2.0dB RSRP_44 to RSRP_65 Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +3.0dB Ês2 / Noc: -0.2dB RSRP_17 to RSRP_31 RSRP_18 to RSRP_32 RSRP_19 to RSRP_33 RSRP_20 to RSRP_34 RSRP_21 to RSRP_35 depending on operating band |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 2 RSRP from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for normal conditions. The RSRP values are recalculated for extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.1.1_3 FDD Intra Frequency Absolute RSRP Accuracy for CA Idle Mode Measurements |  |  | Test 1: Noc: -96dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +3.0dB Reported RSRP values: ±6dB Test 2: Noc: -88dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +3.0dB Reported RSRP values: ±9.5dB Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +6.0dB Ês2 / Noc: +3.0dB Reported RSRP values: ±6dB |  |  |  |  | Test 1: -1.0dB 0dB +1.0dB Via mapping Test 2: 0dB 0dB +1.0dB Via mapping Test 3: 0dB 0dB +0.8dB Via mapping |  |  |  |  | Test 1: Noc: -97.0dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +4.0dB RSRP_42 to RSRP_54 Test 2: Noc: -88dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +4.0dB RSRP_48 to RSRP_67 Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +6.0dB Ês2 / Noc: +3.8dB RSRP_23 to RSRP_35 RSRP_24 to RSRP_36 RSRP_24 to RSRP_36 RSRP_25 to RSRP_37 RSRP_25 to RSRP_37 RSRP_26 to RSRP_38 RSRP_27 to RSRP_39 depending on operating band |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 2 RSRP from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for normal conditions. The RSRP values are recalculated for extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.1.2 FDD Intra Frequency Relative RSRP Accuracy |  |  | Test 1: Noc: -106dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported relative RSRP values: ±3dB Test 2: Noc: -88dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported relative RSRP values: ±3dB Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +3.0dB Ês2 / Noc: -1.0dB Reported relative RSRP values: ±3dB |  |  |  |  | Test 1: 0 dB 0 dB +1.0dB Via mapping Test 2: 0dB 0dB +1.0dB Via mapping Test 3: 0dB 0dB +1.0dB Via mapping |  |  |  |  | Test 1: Noc: -106 dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +2.0dB RSRP_x-9 to RSRP_x+1 Test 2: Noc: -88dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +2.0dB RSRP_x-9 to RSRP_x+1 Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +3.0dB Ês2 / Noc: 0dB RSRP_x-8 to RSRP_x+2 |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 1 and Cell 2 RSRP from Noc, Ês1 / Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for both normal and extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.1.2_2 FDD Intra Frequency Relative Accuracy of RSRP for UE category 1bis |  |  | Test 1: Noc: -106dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported relative RSRP values: ±3dB Test 2: Noc: -88dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported relative RSRP values: ±4dB Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +3.0dB Ês2 / Noc: -1.0dB Reported relative RSRP values: ±3dB |  |  | Test 1: 0 dB 0 dB +1.0dB Via mapping Test 2: 0dB 0dB +1.0dB Via mapping Test 3: 0dB 0dB +1.0dB Via mapping |  |  |  |  |  |  | Test 1: Noc: -106 dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +2.0dB RSRP_x-9 to RSRP_x+1 Test 2: Noc: -88dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +2.0dB RSRP_x-10 to RSRP_x+2 Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +3.0dB Ês2 / Noc: 0dB RSRP_x-8 to RSRP_x+2 |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 1 and Cell 2 RSRP from Noc, Ês1 / Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for both normal and extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.2.1 TDD Intra Frequency Absolute RSRP Accuracy |  |  | Same as 9.1.1.1 |  |  | Same as 9.1.1.1 |  |  |  |  |  |  | Same as 9.1.1.1 |  |
| 9.1.2.1_1 TDD Intra Frequency Absolute RSRP Accuracy (Rel 12 and forward) |  |  | Same as 9.1.1.1_1 |  |  | Same as 9.1.1.1_1 |  |  |  |  |  |  | Same as 9.1.1.1_1 |  |
| 9.1.2.1_2 TDD Intra Frequency Absolute RSRP Accuracy for UE category 1bis |  |  | Same as 9.1.1.1_2 |  |  | Same as 9.1.1.1_2 |  |  |  |  |  |  | Same as 9.1.1.1_2 |  |
| 9.1.2.2 TDD Intra Frequency Relative RSRP Accuracy |  |  | Same as 9.1.1.2 |  |  | Same as 9.1.1.2 |  |  |  |  |  |  | Same as 9.1.1.2 |  |
| 9.1.2.2_2 TDD Intra Frequency Relative Accuracy of RSRP for UE category 1bis |  |  | Same as 9.1.1.2_2 |  |  | Same as 9.1.1.2_2 |  |  |  |  |  |  | Same as 9.1.1.2_2 |  |
| 9.1.3.1 FDD Inter Frequency Absolute RSRP Accuracy |  |  | Test 1: Noc1: -88.65dBm/15kHz Ês1 / Noc1: +10.00dB Noc2: -88.65dBm/15kHz Ês2 / Noc2: +10.00dB Reported RSRP values: ±8dB Test 2: Noc1: (Noc2 +8dB) Ês1 / Noc1: +13.00dB Noc2: -117dBm to -113.5dBm /15kHz depending on operating band Ês2 / Noc2: -4.00dB Reported RSRP values: ±6dB |  |  | Test 1: -0.6dB 0dB -0.6dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0.8dB Via mapping |  |  |  |  |  |  | Test 1: Noc1: -89.25dBm/15kHz Ês1 / Noc1: +10.00dB Noc2: -89.25dBm/15kHz Ês2 / Noc2: +10.00dB RSRP_52 to RSRP_71 Test 2: Noc1: (Noc2 +8dB) Ês1 / Noc1: +13.00dB Noc2: -117dBm to -113.5dBm /15kHz depending on operating band Ês2 / Noc: -3.20dB RSRP_13 to RSRP_28 RSRP_14 to RSRP_29 RSRP_15 to RSRP_30 RSRP_16 to RSRP_31 depending on operating band |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 2 RSRP from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for normal conditions. In all cases the RSRP values are 3dB wider at each end for extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.3.1_1 FDD Inter Frequency Absolute RSRP Accuracy (Rel 12 and forward) |  |  | Test 1: Noc1: -88.65dBm/15kHz Ês1 / Noc1: +10.00dB Noc2: -88.65dBm/15kHz Ês2 / Noc2: +10.00dB Reported RSRP values: ±8dB Test 2: Noc1: (Noc2 +8dB) Ês1 / Noc1: +13.00dB Noc2: -117dBm to -113.5dBm /15kHz depending on operating band Ês2 / Noc2: -4.00dB Reported RSRP values: ±6dB |  |  | Test 1: -0.6dB 0dB -0.6dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0.8dB Via mapping |  |  |  |  |  |  | Test 1: Noc1: -89.25dBm/15kHz Ês1 / Noc1: +10.00dB Noc2: -89.25dBm/15kHz Ês2 / Noc2: +10.00dB RSRP_52 to RSRP_71 Test 2: Noc1: (Noc2 +8dB) Ês1 / Noc1: +13.00dB Noc2: -117dBm to -113.5dBm /15kHz depending on operating band Ês2 / Noc: -3.20dB Normal condition: RSRP_14 to RSRP_26 RSRP_15 to RSRP_27 RSRP_16 to RSRP_28 RSRP_17 to RSRP_29 RSRP_18 to RSRP_30 Extreme condition: RSRP_10 to RSRP_31 RSRP_11 to RSRP_32 RSRP_12 to RSRP_33 RSRP_13 to RSRP_34 depending on operating band |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 2 RSRP from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.3.1_2 FDD - FDD Inter Frequency Absolute RSRP Accuracy for UE category 1bis |  |  | Test 1: Noc1: -88.65dBm/15kHz Ês1 / Noc1: +10.00dB Noc2: -88.65dBm/15kHz Ês2 / Noc2: +10.00dB Reported RSRP values: ±9dB Test 2: Noc1: (Noc2 +8dB) Ês1 / Noc1: +13.00dB Noc2: -117dBm to -113.5dBm /15kHz depending on operating band Ês2 / Noc2: -4.00dB Reported RSRP values: ±5.5dB |  |  | Test 1: -0.6dB 0dB -0.6dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0.8dB Via mapping |  |  |  |  |  |  | Test 1: Noc1: -89.25dBm/15kHz Ês1 / Noc1: +10.00dB Noc2: -89.25dBm/15kHz Ês2 / Noc2: +10.00dB RSRP_51 to RSRP_72 Test 2: Noc1: (Noc2 +8dB) Ês1 / Noc1: +13.00dB Noc2: -117dBm to -113.5dBm /15kHz depending on operating band Ês2 / Noc: -3.20dB Normal condition: RSRP_13 to RSRP_27 RSRP_14 to RSRP_28 RSRP_15 to RSRP_29 RSRP_16 to RSRP_30 RSRP_17 to RSRP_31 Extreme condition: RSRP_9 to RSRP_32 RSRP_10 to RSRP_33 RSRP_11 to RSRP_34 RSRP_12 to RSRP_35 depending on operating band |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 2 RSRP from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.3.1_3 FDD-FDD Inter Frequency Absolute RSRP Accuracy for CA Idle Mode Measurements for Overlapping Carrier |  |  | Test 1: Noc1: -88.65dBm/15kHz Ês1 / Noc1: +10.00dB Noc2: -88.65dBm/15kHz Ês2 / Noc2: +10.00dB Reported RSRP values: ±9dB Test 2: Noc1: (Noc2 +8dB) Ês1 / Noc1: +13.00dB Noc2: -117dBm to -113.5dBm /15kHz depending on operating band Ês2 / Noc2: -4.00dB Reported RSRP values: ±5.5dB |  |  | Test 1: -0.6dB 0dB -0.6dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0.8dB Via mapping |  |  |  |  |  |  | Test 1: Noc1: -89.25dBm/15kHz Ês1 / Noc1: +10.00dB Noc2: -89.25dBm/15kHz Ês2 / Noc2: +10.00dB RSRP_51 to RSRP_72 Test 2: Noc1: (Noc2 +8dB) Ês1 / Noc1: +13.00dB Noc2: -117dBm to -113.5dBm /15kHz depending on operating band Ês2 / Noc: -3.20dB Normal condition: RSRP_13 to RSRP_27 RSRP_14 to RSRP_28 RSRP_15 to RSRP_29 RSRP_16 to RSRP_30 RSRP_17 to RSRP_31 Extreme condition: RSRP_9 to RSRP_32 RSRP_10 to RSRP_33 RSRP_11 to RSRP_34 RSRP_12 to RSRP_35 depending on operating band |  |
| 9.1.3.1_4 FDD-FDD Inter Frequency Absolute RSRP Accuracy for CA Idle Mode Measurements for Non-Overlapping Carrier |  |  | Test 1: Noc1: -95Bm/15kHz Ês1 / Noc1: +10.00dB Noc2: -95dBm/15kHz Ês2 / Noc2: +10.00dB Reported RSRP values: ±10.5dB Test 2: Noc1: (Noc2 +8dB) Ês1 / Noc1: +13.00dB Noc2: -117dBm to -113.5dBm /15kHz depending on operating band Ês2 / Noc2: -4.00dB Reported RSRP values: ±7dB |  |  | Test 1: -0.6dB 0dB -0.6dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0.8dB Via mapping |  |  |  |  |  |  | Test 1: Noc1: -95.6dBm/15kHz Ês1 / Noc1: +10.00dB Noc2: -95.6dBm/15kHz Ês2 / Noc2: +10.00dB RSRP_45 to RSRP_66 Test 2: Noc1: (Noc2 +8dB) Ês1 / Noc1: +13.00dB Noc2: -117dBm to -113.5dBm /15kHz depending on operating band Ês2 / Noc: -3.20dB Normal condition: RSRP_14 to RSRP_28 RSRP_15 to RSRP_29 RSRP_15 to RSRP_29 RSRP_16 to RSRP_30 RSRP_16 to RSRP_30 RSRP_17 to RSRP_31 RSRP_18 to RSRP_32 depending on operating band |  |
| 9.1.3.2 FDD Inter Frequency Relative RSRP Accuracy |  |  | Test 1: Noc1: -88.65dBm/15kHz Noc2: -88.65dBm/15kHz Ês1 / Noc1: +10dB Ês2 / Noc2: +10dB Reported relative RSRP values: ±6dB Test 2: Noc1: (Noc2 +8dB) Noc2: -117dBm to -113.5dBm /15kHz depending on operating band Ês1 / Noc1: +13dB Ês2 / Noc2: -4.0dB Reported relative RSRP values: ±6dB |  |  | Test 1: -0.6dB -0.6dB 0dB 0dB Via mapping Test 2: -1.0dB 0dB 0dB 0.8dB Via mapping |  |  |  |  |  |  | Test 1: Noc1: -89.25dBm/15kHz Noc2: -89.25dBm/15kHz Ês1 / Noc: +10dB Ês2 / Noc: +10dB RSRP_(x-9) to RSRP_(x+9) Test 2: Noc1: (Noc2 +7dB) Noc2: -117dBm to -113.5dBm /15kHz depending on operating band Ês1 / Noc1: +13dB Ês2 / Noc2: -3.2dB RSRP_(x-32) to RSRP_(x-16) |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 1 and Cell 2 RSRP from Noc1 and Ês1 / Noc1 and Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for both normal and extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.3.2_1 FDD Inter Frequency Relative RSRP Accuracy (Rel 12 and forward) |  |  | Test 1: Noc1: -88.65dBm/15kHz Noc2: -88.65dBm/15kHz Ês1 / Noc1: +10dB Ês2 / Noc2: +10dB Reported relative RSRP values normal: ±4.5dB Reported relative RSRP values extreme : ±6dB Test 2: Noc1: (Noc2 +8dB) Noc2: -117dBm to -113.5dBm /15kHz depending on operating band Ês1 / Noc1: +13dB Ês2 / Noc2: -4.0dB Reported relative RSRP values normal: ±4.5dB Reported relative RSRP values extreme : ±6dB |  |  | Test 1: -0.6dB -0.6dB 0dB 0dB Via mapping Test 2: -1.0dB 0dB 0dB 0.8dB Via mapping |  |  |  |  |  |  | Test 1: Noc1: -89.25dBm/15kHz Noc2: -89.25dBm/15kHz Ês1 / Noc: +10dB Ês2 / Noc: +10dB Normal condition: RSRP_(x-7) to RSRP_(x+7) Extreme condition: RSRP_(x-9) to RSRP_(x+9) Test 2: Noc1: (Noc2 +7dB) Noc2: -117dBm to -113.5dBm /15kHz depending on operating band Ês1 / Noc1: +13dB Ês2 / Noc2: -3.2dB Normal condition: RSRP_(x-30) to RSRP_(x-17) Extreme condition: RSRP_(x-32) to RSRP_(x-16) |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 1 and Cell 2 RSRP from Noc1 and Ês1 / Noc1 and Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.3.2_2 FDD - FDD Inter Frequency Relative Accuracy of RSRP for UE category 1bis |  |  | Test 1: Noc1: -88.65dBm/15kHz Noc2: -88.65dBm/15kHz Ês1 / Noc1: +10dB Ês2 / Noc2: +10dB Reported relative RSRP values normal: ±5.5dB Reported relative RSRP values extreme : ±7dB Test 2: Noc1: (Noc2 +8dB) Noc2: -117dBm to -113.5dBm /15kHz depending on operating band Ês1 / Noc1: +13dB Ês2 / Noc2: -4.0dB Reported relative RSRP values normal: ±5.5dB Reported relative RSRP values extreme : ±7dB |  |  | Test 1: -0.6dB -0.6dB 0dB 0dB Via mapping Test 2: -1.0dB 0dB 0dB 0.8dB Via mapping |  |  |  |  |  |  | Test 1: Noc1: -89.25dBm/15kHz Noc2: -89.25dBm/15kHz Ês1 / Noc: +10dB Ês2 / Noc: +10dB Normal condition: RSRP_(x-8) to RSRP_(x+8) Extreme condition: RSRP_(x-10) to RSRP_(x+10) Test 2: Noc1: (Noc2 +7dB) Noc2: -117dBm to -113.5dBm /15kHz depending on operating band Ês1 / Noc1: +13dB Ês2 / Noc2: -3.2dB Normal condition: RSRP_(x-31) to RSRP_(x-16) Extreme condition: RSRP_(x-33) to RSRP_(x-15) |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 1 and Cell 2 RSRP from Noc1 and Ês1 / Noc1 and Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.4.1 TDD Inter Frequency Absolute RSRP Accuracy |  |  | Same as 9.1.3.1 |  |  | Same as 9.1.3.1 |  |  |  |  |  |  | Same as 9.1.3.1 |  |
| 9.1.4.1_1 TDD Inter Frequency Absolute RSRP Accuracy (R12 and forward) |  |  | Same as 9.1.3.1 |  |  | Same as 9.1.3.1 |  |  |  |  |  |  | Same as 9.1.3.1 |  |
| 9.1.4.1_2 TDD - TDD Inter Frequency Absolute RSRP Accuracy for UE category 1bis |  |  | Same as 9.1.3.1_2 |  |  | Same as 9.1.3.1_2 |  |  |  |  |  |  | Same as 9.1.3.1_2 |  |
| 9.1.4.2 TDD Inter Frequency Relative RSRP Accuracy |  |  | Same as 9.1.3.2 |  |  | Same as 9.1.3.2 |  |  |  |  |  |  | Same as 9.1.3.2 |  |
| 9.1.4.2_1 TDD Inter Frequency Relative RSRP Accuracy (Rel 12 and forward) |  |  | Same as 9.1.3.2 |  |  | Same as 9.1.3.2 |  |  |  |  |  |  | Same as 9.1.3.2 |  |
| 9.1.4.2_2 TDD - TDD Inter Frequency Relative Accuracy of RSRP for UE category 1bis |  |  | Same as 9.1.3.2_2 |  |  | Same as 9.1.3.2_2 |  |  |  |  |  |  | Same as 9.1.3.2_2 |  |
| 9.1.5.1 FDD-TDD inter frequency absolute RSRP Accuracy |  |  | Test 1: Noc1: -88.65dBm/15kHz Noc2: -88.65dBm/15kHz Ês1 / Noc1: +10dB Ês2 / Noc2: +10dB Reported absolute RSRP values: ±8dB Test 2: Noc1: -104dBm /15kHz Noc2: -112dBm/15kHz Ês1 / Noc1: +13dB Ês2 / Noc2: -4.0dB Reported absolute RSRP values: ±6dB |  |  | Test 1: -0.6dB -0.6dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0.8dB Via mapping |  |  |  |  |  |  | Test 1: Noc1: -89.25dBm/15kHz Noc2: -89.25dBm/15kHz Ês1 / Noc: +10dB Ês2 / Noc: +10dB RSRP_52 to RSRP_71 Test 2: Noc1: -104Bm /15kHz Noc2: -112dBm /15kHz Ês1 / Noc1: +13dB Ês2 / Noc2: -3.2dB RSRP_18 to RSRP_33 |  |
| 9.1.5.1_1 FDD - TDD Inter Frequency Absolute RSRP Accuracy (Rel 12 and forward) |  |  | Test 1: Noc1: -88.65dBm/15kHz Noc2: -88.65dBm/15kHz Ês1 / Noc1: +10dB Ês2 / Noc2: +10dB Normal condition: Reported absolute RSRP values: ±8dB Extreme condition: Reported absolute RSRP values: ±11dB Test 2: Noc1: -104dBm /15kHz Noc2: -112dBm/15kHz Ês1 / Noc1: +13dB Ês2 / Noc2: -4.0dB Normal condition: Reported absolute RSRP values: ±4.5dB Extreme condition: Reported absolute RSRP values: ±8dB |  |  | Test 1: -0.6dB -0.6dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0.8dB Via mapping |  |  |  |  |  |  | Test 1: Noc1: -89.25dBm/15kHz Noc2: -89.25dBm/15kHz Ês1 / Noc: +10dB Ês2 / Noc: +10dB RSRP_52 to RSRP_71 for normal condition RSRP_49 to RSRP_74 for extreme condition Test 2: Noc1: -104Bm /15kHz Noc2: -112dBm /15kHz Ês1 / Noc1: +13dB Ês2 / Noc2: -3.2dB RSRP_19 to RSRP_31 for normal condition RSRP_15 to RSRP_36 for extreme condition |  |
| 9.1.5.1_2 FDD - TDD Inter Frequency Absolute RSRP Accuracy for UE category 1bis |  |  | Same as 9.1.5.1_1 |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.5.1_2 FDD - TDD Inter Frequency Absolute RSRP Accuracy for UE category 1bis |  |  | Test 1: Noc1: -88.65dBm/15kHz Noc2: -88.65dBm/15kHz Ês1 / Noc1: +10dB Ês2 / Noc2: +10dB Normal condition: Reported absolute RSRP values: ±9dB Extreme condition: Reported absolute RSRP values: ±12dB Test 2: Noc1: -104dBm /15kHz Noc2: -112dBm/15kHz Ês1 / Noc1: +13dB Ês2 / Noc2: -4.0dB Normal condition: Reported absolute RSRP values: ±5.5dB Extreme condition: Reported absolute RSRP values: ±10dB |  |  | Test 1: -0.6dB -0.6dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0.8dB Via mapping |  |  |  |  |  |  | Test 1: Noc1: -89.25dBm/15kHz Noc2: -89.25dBm/15kHz Ês1 / Noc: +10dB Ês2 / Noc: +10dB RSRP_51 to RSRP_72 for normal condition RSRP_48 to RSRP_75 for extreme condition Test 2: Noc1: -104Bm /15kHz Noc2: -112dBm /15kHz Ês1 / Noc1: +13dB Ês2 / Noc2: -3.2dB RSRP_18 to RSRP_32 for normal condition RSRP_14 to RSRP_37 for extreme condition |  |
| 9.1.5.2 FDD-TDD Inter Frequency Relative RSRP Accuracy |  |  | Test 1: Noc1: -88.65dBm/15kHz Noc2: -88.65dBm/15kHz Ês1 / Noc1: +10dB Ês2 / Noc2: +10dB Reported absolute RSRP values: ±6dB Test 2: Noc1: -104dBm /15kHz Noc2: -112dBm/15kHz Ês1 / Noc1: +13dB Ês2 / Noc2: -4.0dB Reported absolute RSRP values: ±6dB |  |  | Test 1: -0.6dB -0.6dB 0dB 0dB Via mapping Test 2: -1.0dB 0dB 0dB 0.8dB Via mapping |  |  |  |  |  |  | Test 1: Noc1: -89.25dBm/15kHz Noc2: -89.25dBm/15kHz Ês1 / Noc: +10dB Ês2 / Noc: +10dB RSRP_(x-9) to RSRP_(x+9) Test 2: Noc1: -105.0Bm /15kHz Noc2: -112dBm /15kHz Ês1 / Noc1: +13dB Ês2 / Noc2: -3.2dB RSRP_(x-32) to RSRP_(x-16) |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 1 and Cell 2 RSRP from Noc1 and Ês1 / Noc1 and Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for both normal and extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.5.2_1 FDD - TDD Inter Frequency Relative Accuracy of RSRP (Rel 12 and forward) |  |  | Test 1: Noc1: -88.65dBm/15kHz Noc2: -88.65dBm/15kHz Ês1 / Noc1: +10dB Ês2 / Noc2: +10dB Reported relative RSRP values normal: ±4.5dB Reported relative RSRP values extreme: ±6.0dB Test 2: Noc1: -104dBm /15kHz Noc2: -112dBm/15kHz Ês1 / Noc1: +13dB Ês2 / Noc2: -4.0dB Reported relative RSRP values normal: ±4.5dB Reported relative RSRP values extreme: ±6.0dB |  |  | Test 1: -0.6dB -0.6dB 0dB 0dB Via mapping Test 2: -1.0dB 0dB 0dB 0.8dB Via mapping |  |  |  |  |  |  | Test 1: Noc1: -89.25dBm/15kHz Noc2: -89.25dBm/15kHz Ês1 / Noc: +10dB Ês2 / Noc: +10dB Normal condition: RSRP_(x-7) to RSRP_(x+7) Extreme condition: RSRP_(x-9) to RSRP_(x+9) Test 2: Noc1: -105.0Bm /15kHz Noc2: -112dBm /15kHz Ês1 / Noc1: +13dB Ês2 / Noc2: -3.2dB Normal condition: RSRP_(x-30) to RSRP_(x-17) Extreme condition: RSRP_(x-32) to RSRP_(x-16) |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 1 and Cell 2 RSRP from Noc1 and Ês1 / Noc1 and Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.5.2_2 FDD - TDD Inter Frequency Relative Accuracy of RSRP for UE category 1bis |  |  | Test 1: Noc1: -88.65dBm/15kHz Noc2: -88.65dBm/15kHz Ês1 / Noc1: +10dB Ês2 / Noc2: +10dB Reported relative RSRP values normal: ±5.5dB Reported relative RSRP values extreme: ±7.0dB Test 2: Noc1: -104dBm /15kHz Noc2: -112dBm/15kHz Ês1 / Noc1: +13dB Ês2 / Noc2: -4.0dB Reported relative RSRP values normal: ±5.5dB Reported relative RSRP values extreme: ±7.0dB |  |  | Test 1: -0.6dB -0.6dB 0dB 0dB Via mapping Test 2: -1.0dB 0dB 0dB 0.8dB Via mapping |  |  |  |  |  |  | Test 1: Noc1: -89.25dBm/15kHz Noc2: -89.25dBm/15kHz Ês1 / Noc: +10dB Ês2 / Noc: +10dB Normal condition: RSRP_(x-8) to RSRP_(x+8) Extreme condition: RSRP_(x-10) to RSRP_(x+10) Test 2: Noc1: -105.0Bm /15kHz Noc2: -112dBm /15kHz Ês1 / Noc1: +13dB Ês2 / Noc2: -3.2dB Normal condition: RSRP_(x-31) to RSRP_(x-16) Extreme condition: RSRP_(x-33) to RSRP_(x-15) |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 1 and Cell 2 RSRP from Noc1 and Ês1 / Noc1 and Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.6.1 FDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation |  |  | Noc1: -117dBm or -116dBm or -115.5dBm or -115 dBm or -114dBm or -113.5dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Ês1 / Noc1: -4dB Ês2 / No2c: +3dB Ês3 / No2c: -1dB Reported RSRP values: ±6dB for normal conditions and ±9dB for extreme conditions |  |  | 0dB 0dB 0dB 0dB 0dB Via mapping Via mapping |  |  |  |  |  |  | Noc1: -117dBm or -116dBm or -115.5dBm or -115 dBm or -114dBm or -113.5dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Ês1 / Noc1: -4dB Ês2 / No2c: +3dB Ês3 / No2c: -1dB Cell 1: RSRP_12 to RSRP_27 RSRP_13 to RSRP_28 RSRP_13 to RSRP_29 RSRP_14 to RSRP_29 RSRP_15 to RSRP_30 RSRP_15 to RSRP_31 depending on operating band Cell 2: RSRP_20 to RSRP_35 RSRP_21 to RSRP_36 RSRP_21 to RSRP_37 RSRP_22 to RSRP_37 RSRP_23 to RSRP_38 RSRP_23 to RSRP_39 depending on Cell 1 band |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 1 RSRP from Noc1 and Ês1 / Noc1, the uncertainty in Cell 2 RSRP from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for normal conditions. In all cases the RSRP values are 3dB wider at each end for extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.6.1_1 Measurement Performance Requirements / FDD Absolute RSRP Accuracy E-UTRA for Carrier Aggregation (Rel 12 and forward) |  |  | Noc1: -117dBm or -116dBm or -115.5dBm or -115 dBm or -114dBm or -113.5dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Es1 / Noc1: -4dB Es2 / No2c: +3dB Es3 / No2c: -1dB Reported RSRP values: 4.5dB for normal conditions and 9dB for extreme conditions |  |  | 0dB 0dB 0dB 0dB 0dB Via mapping Via mapping |  |  |  |  |  |  | Noc1: -117dBm or -116dBm or -115.5dBm or -115 dBm or -114dBm or -113.5dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Es1 / Noc1: -4dB Es2 / No2c: +3dB Es3 / No2c: -1dB Normal condition Cell 1: RSRP_13 to RSRP_26 RSRP_14 to RSRP_26 RSRP_14 to RSRP_27 RSRP_15 to RSRP_27 RSRP_15 to RSRP_28 RSRP_16 to RSRP_29 RSRP_17 to RSRP_29 depending on operating band Cell 2: RSRP_21 to RSRP_34 RSRP_22 to RSRP_34 RSRP_22 to RSRP_35 RSRP_23 to RSRP_35 RSRP_23 to RSRP_36 RSRP_24 to RSRP_37 RSRP_25 to RSRP_37 depending on Cell 1 band Exreme condition Cell 1: RSRP_9 to RSRP_30 RSRP_9 to RSRP_31 RSRP_10 to RSRP_31 RSRP_10 to RSRP_31 RSRP_11 to RSRP_32 RSRP_12 to RSRP_33 RSRP_12 to RSRP_34 depending on operating band Cell 2: RSRP_17 to RSRP_38 RSRP_17 to RSRP_39 RSRP_18 to RSRP_39 RSRP_18 to RSRP_40 RSRP_19 to RSRP_40 RSRP_20 to RSRP_41 RSRP_20 to RSRP_42 depending on Cell 1 band |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 1 RSRP from Noc1 and Es1 / Noc1, the uncertainty in Cell 2 RSRP from Noc2 and Es2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.6.2 FDD Relative RSRP Accuracy for E-UTRA Carrier Aggregation |  |  | Noc1: -117dBm or -116dBm or -115.5dBm or -115 dBm or -114dBm or -113.5dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Ês1 / Noc1: -4dB Ês2 / No2c: +3dB Ês3 / No2c: -1dB Reported relative RSRP values: (Cell 2 - Cell 1): ±6dB (Cell 3 - Cell 2): ±3dB |  |  | 0dB 0dB 0dB 0dB +0.8dB Via mapping Via mapping |  |  |  |  |  |  | Noc1: -117dBm or -116dBm or -115.5dBm or -115 dBm or -114dBm or -113.5dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Ês1 / Noc1: -4dB Ês2 / No2c: +3dB Ês3 / No2c: -0.2dB (Cell 2 - Cell 1): RSRP_(x-1) to RSRP_(x+17) (Cell 3 - Cell 2): RSRP_(x-8) to RSRP_(x+1) |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 1 RSRP from Noc1 and Ês1 / Noc1, the uncertainty in Cell 2 RSRP from Noc2 and Ês2 / Noc2, the uncertainty in Cell 3 RSRP from Ês3 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for normal and extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.6.2_1 FDD Relative RSRP Accuracy E-UTRA for Carrier Aggregation (Rel 12 and forward) |  |  | Noc1: -117dBm or -116dBm or -115.5dBm or -115 dBm or -114dBm or -113.5dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Es1 / Noc1: -4dB Es2 / No2c: +3dB Es3 / No2c: -1dB Reported relative RSRP values: (Cell 2 - Cell 1): ±4.5dB for normal conditions and ±6dB for extreme conditions (Cell 3 - Cell 2): ±3dB |  |  | 0dB 0dB 0dB 0dB +0.8dB Via mapping Via mapping |  |  |  |  |  |  | Noc1: -117dBm or -116dBm or -115.5dBm or -115 dBm or -114dBm or -113.5dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Es1 / Noc1: -4dB Es2 / No2c: +3dB Es3 / No2c: -0.2dB Normal condition: (Cell 2 - Cell 1): RSRP_(x+2) to RSRP_(x+15) (Cell 3 - Cell 2): RSRP_(x-8) to RSRP_(x+1) Extreme condition: (Cell 2 - Cell 1): RSRP_(x-1) to RSRP_(x+17) (Cell 3 - Cell 2): RSRP_(x-8) to RSRP_(x+1) |  |
| 9.1.7.1 TDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation |  |  | Same as 9.1.6.1 |  |  | Same as 9.1.6.1 |  |  |  |  |  |  | Same as 9.1.6.1 |  |
| 9.1.7.1_1 TDD Absolute RSRP Accuracy E-UTRA for Carrier Aggregation (Rel 12 and forward) |  |  | Same as 9.1.6.1_1 |  |  | Same as 9.1.6.1_1 |  |  |  |  |  |  | Same as 9.1.6.1_1 |  |
| 9.1.7.2 TDD Relative RSRP Accuracy for E-UTRA Carrier Aggregation |  |  | Same as 9.1.6.2 |  |  | Same as 9.1.6.2 |  |  |  |  |  |  | Same as 9.1.6.2 |  |
| 9.1.7.2_1 TDD Relative RSRP Accuracy E-UTRA for Carrier Aggregation (Rel 12 and forward) |  |  | Same as 9.1.6.2_1 |  |  | Same as 9.1.6.2_1 |  |  |  |  |  |  | Same as 9.1.6.2_1 |  |
| 9.1.8.1 FDD Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction with Non-MBSFN ABS |  |  | Test 1: Noc: -106.0dBm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -2.0dB Reported RSRP values: ±6dB Test 2: Noc: -88.0dBm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -4.0dB Reported RSRP values: ±8dB Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +5.0dB Ês2 / Noc: -4.0dB Reported RSRP values: ±6dB |  |  | Test 1: 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB +0.95dB Via mapping Test 3: 0dB 0dB +0.95dB Via mapping |  |  |  |  |  |  | Test 1: Noc: -106.0Bm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -2.0dB RSRP_25 to RSRP_40 Test 2: Noc: -88.0dBm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -3.05dB RSRP_40 to RSRP_59 Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +5.0dB Ês2 / Noc: -3.05dB RSRP_14 to RSRP_29 RSRP_15 to RSRP_30 RSRP_16 to RSRP_31 RSRP_17 to RSRP_32 depending on operating band |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 2 RSRP from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for normal conditions. In all cases the RSRP values are 3dB wider at each end for extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.8.1_1 FDD Absolute RSRP Accuracy under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS (eICIC) (Rel-12 and forward) |  |  | Test 1: Noc: -106.0dBm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -2.0dB Reported RSRP values: ±6dB Test 2: Noc: -88.0dBm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -4.0dB Reported RSRP values: ±8dB Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +5.0dB Ês2 / Noc: -4.0dB Reported RSRP values: ±6dB |  |  | Test 1: 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB +0.95dB Via mapping Test 3: 0dB 0dB +0.95dB Via mapping |  |  |  |  |  |  | Test 1: Noc: -106.0Bm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -2.0dB RSRP_26 to RSRP_39 Test 2: Noc: -88.0dBm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -3.05dB RSRP_40 to RSRP_59 Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +5.0dB Ês2 / Noc: -3.05dB RSRP_15 to RSRP_27 RSRP_16 to RSRP_28 RSRP_17 to RSRP_29 RSRP_18 to RSRP_30 RSRP_19 to RSRP_31 depending on operating band |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 2 RSRP from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for normal conditions. The RSRP values are recalculated for extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.8.2 FDD Relative RSRP Accuracy under Time Domain Measurement Resource Restriction with Non-MBSFN ABS |  |  | Test 1: Noc: -106.0dBm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -2.0dB Reported relative RSRP values: ±2dB Test 2: Noc: -88.0dBm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -4.0dB Reported relative RSRP values: ±3dB Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +5.0dB Ês2 / Noc: -4.0dB Reported relative RSRP values: ±3dB |  |  | Test 1: 0dB 0dB +0.8dB Via mapping Test 2: 0dB 0dB +0.95dB Via mapping Test 3: 0dB 0dB +0.95dB Via mapping |  |  |  |  |  |  | Test 1: Noc: -106.0Bm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -1.2dB RSRP_x-10 to RSRP_x-4 Test 2: Noc: -88.0dBm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -3.05dB RSRP_x-13 to RSRP_x-4 Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +5.0dB Ês2 / Noc: -3.05dB RSRP_x-13 to RSRP_x-4 |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 1 and Cell 2 RSRP from Noc, Ês1 / Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for normal conditions. For Test 1 extreme conditions allow 1dB wider at each end. Test 2, Test 3 RSRP values for extreme conditions are the same for normal conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.9.1 TDD Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction with Non-MBSFN ABS |  |  | Same as 9.1.8.1 |  |  | Same as 9.1.8.1 |  |  |  |  |  |  | Same as 9.1.8.1 |  |
| 9.1.9.1_1 TDD Absolute RSRP Accuracy under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS (eICIC) (Rel 12 and forward) |  |  | Test 1: Noc: -106.0dBm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -2.0dB Reported RSRP values: ±6dB Test 2: Noc: -88.0dBm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -4.0dB Reported RSRP values: ±8dB Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +5.0dB Ês2 / Noc: -4.0dB Reported RSRP values: ±6dB |  |  | Test 1: 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB +0.95dB Via mapping Test 3: 0dB 0dB +0.95dB Via mapping |  |  |  |  |  |  | Test 1: Noc: -106.0Bm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -2.0dB RSRP_26 to RSRP_39 Test 2: Noc: -88.0dBm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -3.05dB RSRP_40 to RSRP_59 Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +5.0dB Ês2 / Noc: -3.05dB RSRP_15 to RSRP_27 RSRP_16 to RSRP_28 RSRP_17 to RSRP_29 depending on operating band |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 2 RSRP from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for normal conditions. The RSRP values are recalculated for extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.9.2 TDD Relative RSRP Accuracy under Time Domain Measurement Resource Restriction with Non-MBSFN ABS |  |  | Same as 9.1.8.2 |  |  | Same as 9.1.8.2 |  |  |  |  |  |  | Same as 9.1.8.2 |  |
| 9.1.10.1 FDD Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction with MBSFN ABS |  |  | Test 1: Noc: -106.0dBm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -2.0dB Reported RSRP values: ±6dB Test 2: Noc: -88.0dBm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -4.0dB Reported RSRP values: ±8dB Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +5.0dB Ês2 / Noc: -4.0dB Reported RSRP values: ±6dB |  |  | Test 1: 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB +0.95dB Via mapping Test 3: 0dB 0dB +0.95dB Via mapping |  |  |  |  |  |  | Test 1: Noc: -106.0Bm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -2.0dB RSRP_25 to RSRP_40 Test 2: Noc: -88.0dBm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -3.05dB RSRP_40 to RSRP_59 Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +5.0dB Ês2 / Noc: -3.05dB RSRP_14 to RSRP_29 RSRP_15 to RSRP_30 RSRP_16 to RSRP_31 RSRP_17 to RSRP_32 depending on operating band |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 2 RSRP from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for normal conditions. In all cases the RSRP values are 3dB wider at each end for extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.10.1_1 FDD Absolute RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS (eICIC) (Rel-12 and forward) |  |  | Test 1: Noc: -106.0dBm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -2.0dB Reported RSRP values: ±6dB Test 2: Noc: -88.0dBm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -4.0dB Reported RSRP values: ±8dB Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +5.0dB Ês2 / Noc: -4.0dB Reported RSRP values: ±6dB |  |  | Test 1: 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB +0.95dB Via mapping Test 3: 0dB 0dB +0.95dB Via mapping |  |  |  |  |  |  | Test 1: Noc: -106.0Bm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -2.0dB RSRP_26 to RSRP_40 Test 2: Noc: -88.0dBm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -3.05dB RSRP_39 to RSRP_59 Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +5.0dB Ês2 / Noc: -3.05dB RSRP_15 to RSRP_27 RSRP_16 to RSRP_28 RSRP_17 to RSRP_29 RSRP_18 to RSRP_30 RSRP_19 to RSRP_31 depending on operating band |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 2 RSRP from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for normal conditions. The RSRP values are recalculated for extreme conditions |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.10.2 FDD Relative RSRP Accuracy under Time Domain Measurement Resource Restriction with MBSFN ABS |  |  | Test 1: Noc: -106.0dBm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -2.0dB Reported relative RSRP values: ±2dB Test 2: Noc: -88.0dBm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -4.0dB Reported relative RSRP values: ±3dB Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +5.0dB Ês2 / Noc: -4.0dB Reported relative RSRP values: ±3dB |  |  |  |  | Test 1: 0dB 0dB +0.8dB Via mapping Test 2: 0dB 0dB +0.95dB Via mapping Test 3: 0dB 0dB +0.95dB Via mapping |  |  |  |  | Test 1: Noc: -106.0Bm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -1.2dB RSRP_x-10 to RSRP_x-4 Test 2: Noc: -88.0dBm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -3.05dB RSRP_x-13 to RSRP_x-4 Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +5.0dB Ês2 / Noc: -3.05dB RSRP_x-13 to RSRP_x-4 |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 1 and Cell 2 RSRP from Noc, Ês1 / Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for normal conditions. For Test 1 extreme conditions allow 1dB wider at each end. Test 2, Test 3 RSRP values for extreme conditions are the same for normal conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.11.1 TDD Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction with MBSFN ABS |  |  | Same as 9.1.10.1 |  |  |  |  | Same as 9.1.10.1 |  |  |  |  | Same as 9.1.10.1 |  |
| 9.1.11.1_1 TDD Absolute RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS (eICIC) (Rel 12 and forward) |  |  | Test 1: Noc: -106.0dBm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -2.0dB Reported RSRP values: ±6dB Test 2: Noc: -88.0dBm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -4.0dB Reported RSRP values: ±8dB Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +5.0dB Ês2 / Noc: -4.0dB Reported RSRP values: ±6dB |  |  |  |  | Test 1: 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB +0.95dB Via mapping Test 3: 0dB 0dB +0.95dB Via mapping |  |  |  |  | Test 1: Noc: -106.0Bm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -2.0dB RSRP_26 to RSRP_40 Test 2: Noc: -88.0dBm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -3.05dB RSRP_39 to RSRP_59 Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +5.0dB Ês2 / Noc: -3.05dB RSRP_15 to RSRP_27 RSRP_16 to RSRP_28 RSRP_17 to RSRP_29 depending on operating band |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 2 RSRP from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for normal conditions. The RSRP values are recalculated for extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.11.2 TDD Relative RSRP Accuracy under Time Domain Measurement Resource Restriction with MBSFN ABS |  |  | Same as 9.1.10.2 |  |  |  |  | Same as 9.1.10.2 |  |  |  |  | Same as 9.1.10.2 |  |
| 9.1.12.1 FDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation for 20MHz |  |  | Same as 9.1.6.1 |  |  |  |  | Same as 9.1.6.1 |  |  |  |  | Same as 9.1.6.1 |  |
| 9.1.12.1_1 Measurement Performance Requirements / FDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation for 20 MHz (Rel 12 and forward) |  |  | Same as 9.1.6.1_1 |  |  |  |  | Same as 9.1.6.1_1 |  |  |  |  | Same as 9.1.6.1_1 |  |
| 9.1.12.2 FDD Relative RSRP Accuracy for E-UTRA Carrier Aggregation for 20MHz |  |  | Same as 9.1.6.2 |  |  |  |  | Same as 9.1.6.2 |  |  |  |  | Same as 9.1.6.2 |  |
| 9.1.12.2_1 FDD Relative RSRP Accuracy for E-UTRA Carrier Aggregation for 20 MHz (Rel 12 and forward) |  |  | Same as 9.1.6.2_1 |  |  |  |  | Same as 9.1.6.2_1 |  |  |  |  | Same as 9.1.6.2_1 |  |
| 9.1.13.1 TDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation for 20MHz |  |  | Same as 9.1.6.1 |  |  |  |  | Same as 9.1.6.1 |  |  |  |  | Same as 9.1.6.1 |  |
| 9.1.13.1_1 TDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation for 20 MHz (Rel 12 and forward) |  |  | Same as 9.1.6.1_1 |  |  |  |  | Same as 9.1.6.1_1 |  |  |  |  | Same as 9.1.6.1_1 |  |
| 9.1.13.2 TDD Relative RSRP Accuracy for E-UTRA Carrier Aggregation for 20MHz |  |  | Same as 9.1.6.2 |  |  |  |  | Same as 9.1.6.2 |  |  |  |  | Same as 9.1.6.2 |  |
| 9.1.13.2_1 TDD Relative RSRP Accuracy for E-UTRA Carrier Aggregation for 20 MHz (Rel 12 and forward) |  |  | Same as 9.1.12.2_1 |  |  |  |  | Same as 9.1.12.2_1 |  |  |  |  | Same as 9.1.12.2_1 |  |
| 9.1.14.1 FDD Intra Frequency Absolute RSRP Accuracy under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) |  |  | Test 1: Noc: -106dBm/15kHz Ês1 / Noc: +4.0dB Ês2 / Noc: +2.0dB Ês3 / Noc: -1.5dB Reported RSRP values: ±6dB Test 2: Noc: -88dBm/15kHz Ês1 / Noc: +4.0dB Ês2 / Noc: +2.0dB Ês3 / Noc: -4.0dB Reported RSRP values: ±8dB Test 3: Noc: -116 to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +4.0dB Ês2 / Noc: +2.0dB Ês3 / Noc: -4.0dB Reported RSRP values: ±6dB |  |  |  |  | Test 1: 0dB 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB +1.0dB Via mapping Test 3: 0dB 0dB 0dB +1.0dB Via mapping |  |  |  |  | Test 1: Noc: -106.0dBm/15kHz Ês1 / Noc: +4.0dB Ês2 / Noc: +2.0dB Ês3 / Noc: -1.5dB RSRP_25 to RSRP_41 Test 2: Noc: -88dBm/15kHz Ês1 / Noc: +4.0dB Ês2 / Noc: +2.0dB Ês3 / Noc: -3.0dB RSRP_40 to RSRP_59 Test 3: Noc: -116 to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +4.0dB Ês2 / Noc: +2.0dB Ês3 / Noc: -3.0dB RSRP_14 to RSRP_29 RSRP_15 to RSRP_30 RSRP_15 to RSRP_31 RSRP_16 to RSRP_31 RSRP_17 to RSRP_32 RSRP_17 to RSRP_33 depending on operating band |  |
| 9.1.14.1_1 FDD Intra Frequency Absolute RSRP Accuracy under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) (Rel 12 and forward) |  |  | Test 1: Noc: -106dBm/15kHz Ês1 / Noc: +4.0dB Ês2 / Noc: +2.0dB Ês3 / Noc: -1.5dB Reported RSRP values: normal: ±4.5dB Reported RSRP values extreme: ±9dB Test 2: Noc: -88dBm/15kHz Ês1 / Noc: +4.0dB Ês2 / Noc: +2.0dB Ês3 / Noc: -4.0dB Reported RSRP values normal: ±8dB Reported RSRP values extreme: ±11dB Test 3: Noc: -116 to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +4.0dB Ês2 / Noc: +2.0dB Ês3 / Noc: -4.0dB Reported RSRP values normal: ±4.5dB Reported RSRP values extreme: ±9dB |  |  |  |  | Test 1: 0dB 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB +1.0dB Via mapping Test 3: 0dB 0dB 0dB +1.0dB Via mapping |  |  |  |  | Test 1: Noc: -106.0dBm/15kHz Ês1 / Noc: +4.0dB Ês2 / Noc: +2.0dB Ês3 / Noc: -1.5dB RSRP_27 to RSRP_39 RSRP_22 to RSRP_44 Test 2: Noc: -88dBm/15kHz Ês1 / Noc: +4.0dB Ês2 / Noc: +2.0dB Ês3 / Noc: -3.0dB RSRP_40 to RSRP_59 RSRP_37 to RSRP_62 Test 3: Noc: -116 to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +4.0dB Ês2 / Noc: +2.0dB Ês3 / Noc: -3.0dB RSRP_15 to RSRP_28 RSRP_16 to RSRP_28 RSRP_16 to RSRP_29 RSRP_17 to RSRP_29 RSRP_17 to RSRP_30 RSRP_18 to RSRP_31 RSRP_19 to RSRP_31 depending on operating band RSRP_11 to RSRP_32 RSRP_11 to RSRP_33 RSRP_12 to RSRP_33 RSRP_12 to RSRP_34 RSRP_13 to RSRP_34 RSRP_14 to RSRP_35 RSRP_14 to RSRP_36 depending on operating band |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 2 RSRP from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.14.2 FDD Intra Frequency Relative RSRP Accuracy under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) |  |  | Test 1: Noc: -106dBm/15kHz Ês1 / Noc: +4.0dB Ês2 / Noc: +2.0dB Ês3 / Noc: -1.5dB Reported relative RSRP values: ±2dB Test 2: Noc: -88dBm/15kHz Ês1 / Noc: +4.0dB Ês2 / Noc: +2.0dB Ês3 / Noc: -4.0dB Reported relative RSRP values: ±3dB Test 3: Noc: -116 to -112.5 dBm/15kHz depending on frequency band Ês1 / Noc: +4.0dB Ês2 / Noc: -3.0dB Ês3 / Noc: -4.0dB Reported relative RSRP values: ±3dB |  |  |  |  | Test 1: 0dB 0dB 0dB +1dB Via mapping Test 2: 0dB 0dB 0dB +1dB Via mapping Test 3: 0dB 0dB 0dB +1dB Via mapping |  |  |  |  | Test 1: Noc: -106.0dBm/15kHz Ês1 / Noc: +4.0dB Ês2 / Noc: +2.0dB Ês3 / Noc: -0.5dB RSRP_x-8 to RSRP_x-2 Test 2: Noc: -88dBm/15kHz Ês1 / Noc: +4.0dB Ês2 / Noc: +2.0dB Ês3 / Noc: -3.0dB RSRP_x-12 to RSRP_x-3 Test 3: Noc: -116 to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +4.0dB Ês2 / Noc: -3.0dB Ês3 / Noc: -3.0dB RSRP_x-12 to RSRP_x-3 |  |
| 9.1.15.1 TDD Intra Frequency Absolute RSRP Accuracy under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) |  |  | Same as 9.1.14.1 |  |  |  |  | Same as 9.1.14.1 |  |  |  |  | Same as 9.1.14.1 |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 2 RSRP from Noc and Ês2 / Noc, the uncertainty in Cell 3 RSRP from Noc and Ês3 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for normal conditions. For extreme conditions, the RSRP values are 3dB wider at each end in all cases. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.15.1_1 TDD Intra Frequency Absolute RSRP Accuracy under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) (Rel 12 and forward) |  |  | Same as 9.1.14.1_1 |  |  |  |  | Same as 9.1.14.1_1 |  |  |  |  | Same as 9.1.14.1_1 |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 2 RSRP from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for normal conditions. For extreme conditions,in test case 1 and 3 the RSRP values are 4.5 dB wider at each end , in test case 2 the RSRP values are 3 dB wider at each end |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.15.2 TDD Intra Frequency Relative RSRP Accuracy under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) |  |  | Same as 9.1.14.2 |  |  |  |  | Same as 9.1.14.2 |  |  |  |  | Same as 9.1.14.2 |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 3 RSRP from Noc and Ês3 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for normal conditions. For extreme conditions, the RSRP values are 1dB wider at each end for Test 1, the same than for normal conditions for Test 2, Test 3. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.16.1 FDD Intra Frequency Absolute RSRP Accuracy for 5MHz Bandwidth |  |  | Test 1: Noc: -103dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported relative RSRP values: ±3dB Test 2: Noc: -83dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported relative RSRP values: ±3dB Test 3: Noc: -109.5dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: -1.0dB Reported relative RSRP values: ±3dB |  |  |  |  | Test 1: -1.0dB 0 dB +1.0dB Via mapping Test 2: -1.0dB 0dB +1.0dB Via mapping Test 3: 0dB 0dB +0.8dB Via mapping |  |  |  |  | Test 1: Noc: -104.0dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +2.0dB RSRP_31 to RSRP_46 Test 2: Noc: -84dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +2.0dB RSRP_49 to RSRP_68 Test 3: Noc: -109.5dBm /15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: -0.2dB RSRP_23 to RSRP_38 |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 2 RSRP from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for normal conditions. In all cases the RSRP values are 3dB wider at each end for extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.16.1_1 FDD Intra Frequency Absolute RSRP Accuracy for 5MHz Bandwidth (Rel 12 and forward) |  |  | Test 1: Noc: -103dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported RSRP values: ±4.5dB Test 2: Noc: -83dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported RSRP values: ±8dB Test 3: Noc: -109.5dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: -1.0dB Reported RSRP values: ±4.5dB |  |  |  |  | Test 1: -1.0dB 0 dB +1.0dB Via mapping Test 2: -1.0dB 0dB +1.0dB Via mapping Test 3: 0dB 0dB +0.8dB Via mapping |  |  |  |  | Test 1: Noc: -104.0dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +2.0dB RSRP_32 to RSRP_45 Test 2: Noc: -84dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +2.0dB RSRP_49 to RSRP_68 Test 3: Noc: -109.5dBm /15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: -0.2dB RSRP_25 to RSRP_37 |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 2 RSRP from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for normal conditions. The RSRP values are recalculated for extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.16.2 FDD Intra Frequency Relative Accuracy of RSRP for 5MHz Bandwidth |  |  | Test 1: Noc: -103dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported relative RSRP values: ±3dB Test 2: Noc: -83dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported relative RSRP values: ±3dB Test 3: Noc: -109.5.5dBm /15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: -1.0dB Reported relative RSRP values: ±3dB |  |  |  |  | Test 1: 0 dB 0 dB +1.0dB Via mapping Test 2: -1.0dB 0dB +1.0dB Via mapping Test 3: 0dB 0dB +1.0dB Via mapping |  |  |  |  | Test 1: Noc: -103 dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +2.0dB RSRP_x-9 to RSRP_x+1 Test 2: Noc: -84dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +2.0dB RSRP_x-9 to RSRP_x+1 Test 3: Noc: -109.5dBm /15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: 0dB RSRP_x-8 to RSRP_x+2 |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 1 and Cell 2 RSRP from Noc, Ês1 / Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for both normal and extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.17.1 FDD Inter Frequency Absolute RSRP Accuracy for 5MHz Bandwidth |  |  | Test 1: Noc1: -85.65dBm/15kHz Ês1 / Noc1: +10.00dB Noc2: -85.65dBm/15kHz Ês2 / Noc2: +10.00dB Reported RSRP values: ±8dB Test 2: Noc1: (Noc2 +8dB) Ês1 / Noc1: +13.00dB Noc2: -110.5dBm /15kHz Ês2 / Noc2: -4.00dB Reported RSRP values: ±6dB |  |  |  |  | Test 1: -0.6dB 0dB -0.6dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0.8dB Via mapping |  |  |  |  | Test 1: Noc1: -86.25dBm/15kHz Ês1 / Noc1: +10.00dB Noc2: -86.25dBm/15kHz Ês2 / Noc2: +10.00dB RSRP_55 to RSRP_74 Test 2: Noc1: (Noc2 +8dB) Ês1 / Noc1: +13.00dB Noc2: -110.5dBm /15kHz Ês2 / Noc: -3.20dB RSRP_19 to RSRP_34 |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 2 RSRP from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for normal conditions. In all cases the RSRP values are 3dB wider at each end for extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.17.2 FDD - FDD Inter Frequency Relative Accuracy of RSRP for 5MHz Bandwidth |  |  | Test 1: Noc1: -85.65dBm/15kHz Noc2: -85.65dBm/15kHz Ês1 / Noc1: +10dB Ês2 / Noc2: +10dB Reported relative RSRP values: ±6dB Test 2: Noc1: -102.5dBm/15kHz Noc2: -110.5dBm/15kHz Ês1 / Noc1: +13dB Ês2 / Noc2: -4.0dB Reported relative RSRP values: ±6dB |  |  |  |  | Test 1: -0.6dB -0.6dB 0dB 0dB Via mapping Test 2: -1.0dB 0dB 0dB 0.8dB Via mapping |  |  |  |  | Test 1: Noc1: -86.25dBm/15kHz Noc2: -86.25dBm/15kHz Ês1 / Noc: +10dB Ês2 / Noc: +10dB RSRP_(x-9) to RSRP_(x+9) Test 2: Noc1: -103.5dBm/15kHz Noc2: -100.5dBm /15kHz Ês1 / Noc1: +13dB Ês2 / Noc2: -3.2dB RSRP_(x-32) to RSRP_(x-16) |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 1 and Cell 2 RSRP from Noc1 and Ês1 / Noc1 and Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for both normal and extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.18.1 FDD Absolute RSRP Accuracy for E-UTRA for Carrier Aggregation for 10MHz + 5MHz |  |  | Same as 9.1.6.1 |  |  |  |  | Same as 9.1.6.1 |  |  |  |  | Same as 9.1.6.1 |  |
| 9.1.18.2 FDD Relative RSRP Accuracy E-UTRA for Carrier Aggregation for 10MHz + 5MHz |  |  | Noc1: -117dBm or -116dBm or -115.5dBm or -115 dBm or -114dBm or -113.5dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Ês1 / Noc1: -4dB Ês2 / Noc2: +3dB Ês3 / Noc2: -1dB Reported relative RSRP values: (Cell 2 - Cell 1): ±6dB (Cell 3 - Cell 2): ±3dB |  |  |  |  | 0dB 0dB 0dB 0dB +0.8dB Via mapping Via mapping |  |  |  |  | Noc1: -117dBm or -116dBm or -115.5dBm or -115 dBm or -114dBm or -113.5dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Ês1 / Noc1: -4dB Ês2 / Noc2: +3dB Ês3 / Noc2: -0.2dB (Cell 2 - Cell 1): RSRP_(x-1) to RSRP_(x+17) (Cell 3 - Cell 2): RSRP_(x-8) to RSRP_(x+1) |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 1 RSRP from Noc1 and Ês1 / Noc1, the uncertainty in Cell 2 RSRP from Noc2 and Ês2 / Noc2, the uncertainty in Cell 3 RSRP from Ês3 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for normal and extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.18.2_1 FDD Relative RSRP Accuracy E-UTRA for Carrier Aggregation for 10MHz + 5MHz (Rel-12 and forward) |  |  | Noc1: -117dBm or -116dBm or -115.5dBm or -115 dBm or -114dBm or -113.5dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Ês1 / Noc1: -4dB Ês2 / Noc2: +3dB Ês3 / Noc2: -1dB Reported relative RSRP values: (Cell 2 - Cell 1): ±4.5dB normal condition ±6dB normal condition (Cell 3 - Cell 2): ±3dB normal and extreme condition |  |  |  |  | 0dB 0dB 0dB 0dB +0.8dB Via mapping Via mapping |  |  |  |  | Noc1: -117dBm or -116dBm or -115.5dBm or -115 dBm or -114dBm or -113.5dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Ês1 / Noc1: -4dB Ês2 / Noc2: +3dB Ês3 / Noc2: -0.2dB (Cell 2 - Cell 1): RSRP_(x+2) to RSRP_(x+15) RSRP_(x-1) to RSRP_(x+17) (Cell 3 - Cell 2): RSRP_(x-8) to RSRP_(x+1) |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 1 RSRP from Noc1 and Ês1 / Noc1, the uncertainty in Cell 2 RSRP from Noc2 and Ês2 / Noc2, the uncertainty in Cell 3 RSRP from Ês3 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.19.1 TDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation for 10MHz + 5MHz |  |  | Same as 9.1.6.1 |  |  |  |  | Same as 9.1.6.1 |  |  |  |  | Same as 9.1.6.1 |  |
| 9.1.19.2 TDD Relative RSRP Accuracy E-UTRA for Carrier Aggregation for 10MHz + 5MHz |  |  | Same as 9.1.18.2 |  |  |  |  | Same as 9.1.18.2 |  |  |  |  | Same as 9.1.18.2 |  |
| 9.1.19.2_1 TDD Relative RSRP Accuracy for E-UTRA Carrier Aggregation for 10MHz + 5MHz (Rel-12 and forward) |  |  | Same as 9.1.18.2_1 |  |  |  |  | Same as 9.1.18.2_1 |  |  |  |  | Same as 9.1.18.2_1 |  |
| 9.1.20.1 FDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation for 5MHz + 5MHz bandwidth |  |  | Noc1: -117dBm or -116dBm or -115.5dBm or -115 dBm or -114dBm or -113.5dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Ês1 / Noc1: -4dB Ês2 / Noc2: +3dB Ês3 / Noc2: -1dB Reported RSRP values: ±6dB for normal conditions and ±9dB for extreme conditions |  |  |  |  | 0dB 0dB 0dB 0dB 0dB Via mapping Via mapping |  |  |  |  | Noc1: -117dBm or -116dBm or -115.5dBm or -115 dBm or -114dBm or -113.5dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Ês1 / Noc1: -4dB Ês2 / Noc2: +3dB Ês3 / Noc2: -1dB Cell 1: RSRP_12 to RSRP_27 RSRP_13 to RSRP_28 RSRP_13 to RSRP_29 RSRP_14 to RSRP_29 RSRP_15 to RSRP_30 RSRP_15 to RSRP_31 depending on operating band Cell 2: RSRP_20 to RSRP_35 RSRP_21 to RSRP_36 RSRP_21 to RSRP_37 RSRP_22 to RSRP_37 RSRP_23 to RSRP_38 RSRP_23 to RSRP_39 depending on Cell 1 band |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 1 RSRP from Noc1 and Ês1 / Noc1, the uncertainty in Cell 2 RSRP from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for normal conditions. In all cases the RSRP values are 3dB wider at each end for extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.20.2 FDD Relative RSRP Accuracy E-UTRA for Carrier Aggregation for 5MHz + 5MHz |  |  | Noc1: -117dBm or -116dBm or -115.5dBm or -115 dBm or -114dBm or -113.5dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Ês1 / Noc1: -4dB Ês2 / Noc2: +3dB Ês3 / Noc2: -1dB Reported relative RSRP values: (Cell 2 - Cell 1): ±6dB (Cell 3 - Cell 2): ±3dB |  |  |  |  | 0dB 0dB 0dB 0dB +0.8dB Via mapping Via mapping |  |  |  |  | Noc1: -117dBm or -116dBm or -115.5dBm or -115 dBm or -114dBm or -113.5dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Ês1 / Noc1: -4dB Ês2 / Noc2: +3dB Ês3 / Noc2: -0.2dB (Cell 2 - Cell 1): RSRP_(x-1) to RSRP_(x+17) (Cell 3 - Cell 2): RSRP_(x-8) to RSRP_(x+1) |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 1 RSRP from Noc1 and Ês1 / Noc1, the uncertainty in Cell 2 RSRP from Noc2 and Ês2 / Noc2, the uncertainty in Cell 3 RSRP from Ês3 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for normal and extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.21.1 TDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation for 5MHz + 5MHz bandwidth |  |  | Same as 9.1.20.1 |  |  |  |  | Same as 9.1.20.1 |  |  |  |  | Same as 9.1.20.1 |  |
| 9.1.21.2 TDD Relative RSRP Accuracy E-UTRA for Carrier Aggregation for 5MHz + 5MHz |  |  | Same as 9.1.20.2 |  |  |  |  | Same as 9.1.20.2 |  |  |  |  | Same as 9.1.20.2 |  |
| 9.1.22 FDD-TDD RSRP Accuracy E-UTRA for Carrier Aggregation with PCell in FDD |  |  | Same as 9.1.37 Cell 1 and Cell 2 absolute RSRP; and Cell 2 -Cell 1 relative RSRP and Cell 3- Cell 2 relative RSRP |  |  |  |  | Same as 9.1.37 Cell 1, Cell 2 and Cell 3 |  |  |  |  | Same as 9.1.37 Cell 1 and Cell 2 absolute RSRP; and Cell 2 -Cell 1 relative RSRP and Cell 3- Cell 2 relative RSRP |  |
| 9.1.23 FDD-TDD RSRP Accuracy E-UTRA for Carrier Aggregation with PCell in TDD |  |  | Same as 9.1.37 Cell 1 and Cell 2 absolute RSRP; and Cell 2 -Cell 1 relative RSRP and Cell 3- Cell 2 relative RSRP |  |  |  |  | Same as 9.1.37 Cell 1, Cell 2 and Cell 3 |  |  |  |  | Same as 9.1.37 Cell 1 and Cell 2 absolute RSRP; and Cell 2 -Cell 1 relative RSRP and Cell 3- Cell 2 relative RSRP |  |
| 9.1.24.1 TDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation for 20MHz + 10MHz |  |  | Noc1: -117dBm or -116dBm or -115dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Ês1 / Noc1: -4dB Ês2 / No2c: +3dB Ês3 / No2c: -1dB Reported RSRP values: ±6dB for normal conditions and ±9dB for extreme conditions |  |  |  |  | 0dB 0dB 0dB 0dB 0dB Via mapping Via mapping |  |  |  |  | Noc1: -117dBm or -116dBm or -115dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Ês1 / Noc1: -4dB Ês2 / No2c: +3dB Ês3 / No2c: -1dB Cell 1: RSRP_12 to RSRP_27 RSRP_13 to RSRP_28 RSRP_14 to RSRP_29 depending on operating band Cell 2: RSRP_20 to RSRP_35 RSRP_21 to RSRP_36 RSRP_22 to RSRP_37 depending on Cell 1 band |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 1 RSRP from Noc1 and Ês1 / Noc1, the uncertainty in Cell 2 RSRP from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for normal conditions. In all cases the RSRP values are 3dB wider at each end for extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.24.2_1 TDD Relative RSRP Accuracy for E-UTRA Carrier Aggregation for 20MHz + 10MHz (Rel-12 and forward) |  |  | Noc1: -117dBm or -116dBm or -115dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Ês1 / Noc1: -4dB Ês2 / Noc2: +3dB Ês3 / Noc2: -1dB Reported relative RSRP values: (Cell 2 - Cell 1): ±4.5dB (Cell 3 - Cell 2): ±3dB |  |  |  |  | 0dB 0dB 0dB 0dB +0.8dB Via mapping Via mapping |  |  |  |  | Noc1: -117dBm or -116dBm or -115dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Ês1 / Noc1: -4dB Ês2 / Noc2: +3dB Ês3 / Noc2: -0.2dB (Cell 2 - Cell 1): RSRP_(x+2) to RSRP_(x+15) (Cell 3 - Cell 2): RSRP_(x-8) to RSRP_(x+1) |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 1 RSRP from Noc1 and Ês1 / Noc1, the uncertainty in Cell 2 RSRP from Noc2 and Ês2 / Noc2, the uncertainty in Cell 3 RSRP from Ês3 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for normal conditions. In all cases the RSRP values are 1.5 dB wider at each end for extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.24.1_1 TDD Absolute RSRP Accuracy for E-UTRA Carrier Aggregation for 20MHz + 10MHz (Rel-12 and forward) |  |  | Noc1: -117dBm or -116dBm or -115dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Ês1 / Noc1: -4dB Ês2 / No2c: +3dB Ês3 / No2c: -1dB Reported RSRP values: ±4.5dB for normal conditions and ±9dB for extreme conditions | 0dB 0dB 0dB 0dB 0dB Via mapping Via mapping |  |  |  |  |  | Noc1: -117dBm or -116dBm or -115dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Ês1 / Noc1: -4dB Ês2 / No2c: +3dB Ês3 / No2c: -1dB Cell 1: RSRP_13 to RSRP_26 RSRP_14 to RSRP_27 RSRP_15 to RSRP_28 depending on operating band Cell 2: RSRP_21 to RSRP_34 RSRP_22 to RSRP_35 RSRP_23 to RSRP_36 depending on Cell 1 band |  |  |  |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 1 RSRP from Noc1 and Ês1 / Noc1, the uncertainty in Cell 2 RSRP from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for normal conditions. By using separate calculation for normal and extreme condition, all cases the RSRP values are 4dB wider at each end for extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.24.2 TDD Relative RSRP Accuracy for E-UTRA Carrier Aggregation for 20MHz + 10MHz |  |  | Noc1: -117dBm or -116dBm or -115dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Ês1 / Noc1: -4dB Ês2 / Noc2: +3dB Ês3 / Noc2: -1dB Reported relative RSRP values: (Cell 2 - Cell 1): ±6dB (Cell 3 - Cell 2): ±3dB | 0dB 0dB 0dB 0dB +0.8dB Via mapping Via mapping |  |  |  |  |  | Noc1: -117dBm or -116dBm or -115dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Ês1 / Noc1: -4dB Ês2 / Noc2: +3dB Ês3 / Noc2: -0.2dB (Cell 2 - Cell 1): RSRP_(x-1) to RSRP_(x+17) (Cell 3 - Cell 2): RSRP_(x-8) to RSRP_(x+1) |  |  |  |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 1 RSRP from Noc1 and Ês1 / Noc1, the uncertainty in Cell 2 RSRP from Noc2 and Ês2 / Noc2, the uncertainty in Cell 3 RSRP from Ês3 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for normal and extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.25 FDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal |  |  | Test 1: Noc: -106dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported absolute RSRP values: Normal condtion: ±4.5dB Extreme condtion: ±9dB Reported relative RSRP values: ±3dB | Test 1: -1.0dB 0dB +1.0dB Via mapping |  |  |  |  |  | Test 1: Noc: -107.0dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +2.0dB RSRP_29 to RSRP_42 RSRP_25 to RSRP_46 RSRP_Cell 1 - 9 to RSRP_Cell 1 + 1 |  |  |  |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell1 and Cell 2 RSRP from Noc, Ês1 / Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.26 TDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal |  |  | Same as 9.1.25 | Same as 9.1.25 |  |  |  |  |  | Same as 9.1.25 |  |  |  |  |
|  |  |  | Same as 9.1.25 |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.27 FDD—FDD inter-frequency absolute and relative RSRP accuracies in CRS based discovery signal |  |  | Test 1: Noc1: = Noc2 +6dB Noc2: -115dBm to -111.5dBm /15kHz depending on operating band Ês1 / Noc1: 13dB Ês2 / Noc2: -6dB Reported absolute RSRP values: Normal condtion: ±4.5dB Extreme condtion: ±9dB Reported relative RSRP values: Normal condtion: ±4.5dB Extreme condtion: ±6dB | Test 1: 0dB 0dB 0dB +0.8dB Via mapping Via mapping Via mapping |  |  |  |  |  | Test 1: Noc1: = Noc2 +6dB Noc2: -115dBm to -111.5dBm /15kHz depending on operating band Ês1 / Noc1: 13dB Ês2 / Noc2: -5.2dB Cell 2, absolute: RSRP_14 to RSRP_26 RSRP_15 to RSRP_27 RSRP_15 to RSRP_27 RSRP_16 to RSRP_28 RSRP_16 to RSRP_28 RSRP_17 to RSRP_29 RSRP_18 to RSRP_30 depending on operating band Cell 2, absolute: RSRP_10 to RSRP_31 RSRP_10 to RSRP_31 RSRP_11 to RSRP_32 RSRP_11 to RSRP_32 RSRP_12 to RSRP_33 RSRP_13 to RSRP_34 RSRP_13 to RSRP_34 depending on operating band Relative: RSRP_(x-31) to RSRP_(x-18) RSRP_(x-33) to RSRP_(x-17) |  |  |  |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty from all applicable Noc and Ês / Noc values, the allowed UE reporting accuracy, and the UE mapping function. Where extreme condition UE accuracies differ from normal condition accuracies, reported values are recalculated. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.28 TDD—TDD inter-frequency absolute and relative RSRP accuracies in CRS based discovery signal |  |  | Same as 9.1.27 | Same as 9.1.27 |  |  |  |  |  | Same as 9.1.27 |  |  |  |  |
|  | Same as 9.1.27 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.29 FDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal |  |  | Test 1: Noc: -116dBm to -112.5dBm /15kHz depending on operating band CRS Ês1 / Noc: 3dB CSI-RS Ês1 / Noc: 9dB CRS Ês2 / Noc: -1dB CSI-RS Ês2 / Noc: 5dB Reported absolute CSI-RSRP values: Normal condtion: ±4.5dB Extreme condtion: ±9dB Reported relative CSI-RSRP values: Normal condition: ±2dB Extreme condition: ±3dB | Test 1: 0 dB 0 dB 0 dB 0 dB +0.8 dB Via mapping |  |  |  |  |  | Test 1: Noc: -116dBm to -112.5dBm /15kHz depending on operating band CRS Ês1 / Noc: 3dB CSI-RS Ês1 / Noc: 9dB CRS Ês2 / Noc: -1dB CSI-RS Ês2 / Noc: 5.8dB CSI-RSRP_24 to CSI-RSRP_43 CSI-RSRP_20 to CSI-RSRP_47 CSI-RSRP_Cell 1 - 7 to CSI-RSRP_Cell 1 - 1 CSI-RSRP_Cell 1 - 8 to CSI-RSRP_Cell 1 + 1 |  |  |  |  |
|  |  |  | The derivation of the CSI-RSRP values takes into account the uncertainty in Cell 1 and Cell 2 CSI-RSRP from Noc, CSI-RS Ês1 / Noc and CSI-RS Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.30 TDD intra-frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal |  |  | Same as 9.1.29 | Same as 9.1.29 |  |  |  |  |  | Same as 9.1.29 |  |  |  |  |
| 9.1.31 FDD-FDD inter-frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal |  |  | Noc1: = Noc2 +6dB Noc2: -115dBm to -111.5dBm /15kHz depending on operating band CSI-RS Ês1 / Noc1: 13dB CSI-RS Ês2 / Noc2: 0dB Reported absolute CSI-RSRP values: Normal condtion: ±4.5dB Extreme condtion: ±9dB Reported relative CSI-RSRP values: Normal condtion: ±4.5dB Extreme condtion: ±6dB | 0dB 0dB 0dB +0.8dB Via mapping Via mapping Via mapping Via mapping |  |  |  |  |  | Noc1: = Noc2 +6dB Noc2: -115dBm to -111.5dBm /15kHz depending on operating band CSI-RS Ês1 / Noc1: 13dB CSI-RS Ês2 / Noc2: 0.8dB Cell 2: CSI_RSRP_20 to CSI_RSRP_32 CSI_RSRP_21 to CSI_RSRP_33 CSI_RSRP_21 to CSI_RSRP_33 CSI_RSRP_22 to CSI_RSRP_34 CSI_RSRP_22 to CSI_RSRP_34 CSI_RSRP_23 to CSI_RSRP_35 CSI_RSRP_24 to CSI_RSRP_36 CSI_RSRP_16 to CSI_RSRP_37 CSI_RSRP_16 to CSI_RSRP_37 CSI_RSRP_17 to CSI_RSRP_38 CSI_RSRP_17 to CSI_RSRP_38 CSI_RSRP_18 to CSI_RSRP_39 CSI_RSRP_19 to CSI_RSRP_40 CSI_RSRP_19 to CSI_RSRP_40 Cell 2 - Cell 1 CSI_RSRP_Cell 1 - 25 to CSI_RSRP_Cell 1 - 12 CSI_RSRP_Cell 1 - 27 to CSI_RSRP_Cell 1 - 11 |  |  |  |  |
|  |  |  | The derivation of the CSI-RSRP values takes into account the uncertainty in Cell 1 and Cell 2 CSI-RSRP from Noc, CSI-RS Ês1 / Noc and CSI-RS Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.33 FDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal |  |  | Test 1: Noc1: -117dBm to -113.5dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Ês1 / Noc1: -4dB Ês2 / Noc2: 3dB Ês3 / Noc2: -1dB Reported absolute RSRP values: Normal condtion: ±4.5dB Extreme condtion: ±9dB Reported relative RSRP values: (Cell 3 - Cell 2): ±3dB (Cell 3 - Cell 1): Normal condtion: ±4.5dB Extreme condtion: ±6dB | Test 1: 0dB 0dB 0dB 0dB +0.8dB Via mapping Via mapping Via mapping |  |  |  |  |  | Test 1: Noc1: -117dBm to -113.5dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Ês1 / Noc1: -4dB Ês2 / Noc2: 3dB Ês3 / Noc2: -0.2dB Cell 3, absolute: RSRP_18 to RSRP_30 RSRP_19 to RSRP_31 RSRP_19 to RSRP_31 RSRP_20 to RSRP_32 RSRP_20 to RSRP_32 RSRP_21 to RSRP_33 RSRP_22 to RSRP_34 depending on operating band Cell 3, absolute: RSRP_14 to RSRP_35 RSRP_14 to RSRP_35 RSRP_15 to RSRP_36 RSRP_15 to RSRP_36 RSRP_16 to RSRP_37 RSRP_17 to RSRP_38 RSRP_17 to RSRP_38 depending on operating band Relative: RSRP_(x-8) to RSRP_(x+1) RSRP_(x-2) to RSRP_(x+12) RSRP_(x-4) to RSRP_(x+13) |  |  |  |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty from all applicable Noc and Ês / Noc values, the allowed UE reporting accuracy, and the UE mapping function. Where extreme condition UE accuracies differ from normal condition accuracies, reported values are recalculated. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.34 TDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal |  |  | Same as 9.1.33 | Same as 9.1.33 |  |  |  |  | Same as 9.1.33 |  |  |  |  |  |
|  |  |  | Same as 9.1.33 |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.35 FDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal |  |  | Noc1: -117dBm to -113.5dBm /15kHz depending on operating band Noc2: = Noc2 +1dB CSI-RS Ês1 / Noc1: 2dB CSI-RS Ês2 / Noc2: 9dB CSI-RS Ês3 / Noc3: 5dB Reported absolute CSI-RSRP values: Normal condtion: ±4.5dB Extreme condtion: ±9dB Reported intra-freq relative CSI-RSRP values: Normal condtion: ±2dB Extreme condtion: ±3dB Reported inter-freq relative CSI-RSRP values: Normal condtion: ±4.5dB Extreme condtion: ±6dB |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | The derivation of the CSI-RSRP values takes into account the uncertainty in Cell 1 and Cell 2 CSI-RSRP from Noc, CSI-RS Ês / Noc, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.36 TDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal |  |  | Same as 9.1.35 |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.37 3DL PCell in FDD RSRP for E-UTRAN in Carrier Aggregation |  |  | Noc1: -117dBm or -116.5dBm or -116dBm or -115.5dBm or -115 dBm or -114dBm or -113.5dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Noc3: = Noc1 +1dB Ês1 / Noc1: -4dB Ês2 / Noc2: +3dB Ês3 / Noc2: -1dB Ês4 / Noc3: +3dB Ês5 / Noc3: -1dB Reported absolute RSRP values: ±4.5dB for normal conditions and ±9dB for extreme conditions Reported relative RSRP values: (Cell 2 - Cell 1) and (Cell 4 - Cell 1): ±4.5dB for normal conditions ±6dB for extreme conditions (Cell 3 - Cell 2) and (Cell 5 - Cell 4): ±3dB |  |  | 0dB 0dB 0dB 0dB 0dB +0.8dB 0dB +0.8dB Via mapping Via mapping Via mapping |  |  |  |  |  | Noc1: -117dBm or -116.5dBm or -116dBm or -115.5dBm or -115 dBm or -114dBm or -113.5dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Noc3: = Noc1 +1dB Ês1 / Noc1: -4dB Ês2 / Noc2: +3dB Ês3 / Noc2: -0.2dB Ês4 / Noc3: +3dB Ês5 / Noc3: -0.2dB Cell 1, absolute: RSRP_13 to RSRP_26 RSRP_14 to RSRP_26 RSRP_14 to RSRP_27 RSRP_15 to RSRP_27 RSRP_15 to RSRP_28 RSRP_16 to RSRP_29 RSRP_17 to RSRP_29 depending on operating band Cell 2 and Cell 4, absolute: RSRP_21 to RSRP_34 RSRP_22 to RSRP_34 RSRP_22 to RSRP_35 RSRP_23 to RSRP_35 RSRP_23 to RSRP_36 RSRP_24 to RSRP_37 RSRP_25 to RSRP_37 depending on Cell 1 band Relative: (Cell 2 - Cell 1) and (Cell 4 - Cell 1): RSRP_(x+2) to RSRP_(x+15) (Cell 3 - Cell 2) and (Cell 5 - Cell 4): RSRP_(x-8) to RSRP_(x+1) |  |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty from all applicable Noc and Ês / Noc values, the allowed UE reporting accuracy, and the UE mapping function. Where extreme condition UE accuracies differ from normal condition accuracies, reported values are recalculated. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.38 3DL PCell in TDD RSRP for E-UTRAN in Carrier Aggregation |  |  | Same as 9.1.37 |  |  | Same as 9.1.37 |  |  |  |  |  | Same as 9.1.37 |  |  |
| 9.1.39 3DL FDD RSRP for E-UTRAN in Carrier Aggregation |  |  | Noc1: -117dBm or -116dBm or -115.5dBm or -115 dBm or -114dBm or -113.5dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Noc3: = Noc1 +1dB Ês1 / Noc1: -4dB Ês2 / Noc2: +3dB Ês3 / Noc2: -1dB Ês4 / Noc3: +3dB Ês5 / Noc3: -1dB Reported absolute RSRP values: ±6dB for normal conditions and ±9dB for extreme conditions Reported relative RSRP values: (Cell 2 - Cell 1) and (Cell 4 - Cell 1): ±6dB (Cell 3 - Cell 2) and (Cell 5 - Cell 4): ±3dB |  |  | 0dB 0dB 0dB 0dB 0dB +0.8dB 0dB +0.8dB Via mapping Via mapping Via mapping |  |  |  |  |  | Noc1: -117dBm or -116dBm or -115.5dBm or -115 dBm or -114dBm or -113.5dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Noc3: = Noc1 +1dB Ês1 / Noc1: -4dB Ês2 / Noc2: +3dB Ês3 / Noc2: -0.2dB Ês4 / Noc3: +3dB Ês5 / Noc3: -0.2dB Cell 1, absolute: RSRP_12 to RSRP_27 RSRP_13 to RSRP_28 RSRP_13 to RSRP_29 RSRP_14 to RSRP_29 RSRP_15 to RSRP_30 RSRP_15 to RSRP_31 depending on operating band Cell 2 and Cell 4, absolute: RSRP_20 to RSRP_35 RSRP_21 to RSRP_36 RSRP_21 to RSRP_37 RSRP_22 to RSRP_37 RSRP_23 to RSRP_38 RSRP_23 to RSRP_39 depending on Cell 1 band Relative: (Cell 2 - Cell 1) and (Cell 4 - Cell 1): RSRP_(x-1) to RSRP_(x+17) (Cell 3 - Cell 2) and (Cell 5 - Cell 4): RSRP_(x-8) to RSRP_(x+1) |  |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty from all applicable Noc and Ês / Noc values, the allowed UE reporting accuracy, and the UE mapping function. Where extreme condition UE accuracies differ from normal condition accuracies, reported values are recalculated. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.39_1 3DL FDD RSRP for E-UTRAN in Carrier Aggregation (Rel-12 and forward) |  |  | Same as 9.1.37 |  |  | Same as 9.1.37 |  |  |  |  |  | Same as 9.1.37 |  |  |
| 9.1.40 3DL TDD RSRP for E-UTRAN in Carrier Aggregation |  |  | Same as 9.1.39 |  |  | Same as 9.1.39 |  |  |  |  |  | Same as 9.1.39 |  |  |
| 9.1.40_1 3DL TDD RSRP for E-UTRAN in Carrier Aggregation (Rel-12 and forward) |  |  | Same as 9.1.37 |  |  | Same as 9.1.37 |  |  |  |  |  | Same as 9.1.37 |  |  |
| 9.1.41.1 FD-FDD Intra Frequency Absolute RSRP Accuracy for UE category 0 |  |  | Test 1: Noc: -106dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported RSRP values: ±7dB Test 2: Noc: -86dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported RSRP values: ±9dB Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +3.0dB Ês2 / Noc: -1.0dB Reported RSRP values: ±7dB |  |  | Test 1: -1.0dB 0dB +1.0dB Via mapping Test 2: -1.0dBdB 0dB +1.0dB Via mapping Test 3: 0dB 0dB +0.8dB Via mapping |  |  |  |  |  | Test 1: Noc: -107.0dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +2.0dB RSRP_27 to RSRP_44 Test 2: Noc: -87dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +2.0dB RSRP_45 to RSRP_66 Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +3.0dB Ês2 / Noc: -0.2dB RSRP_16 to RSRP_33 RSRP_17 to RSRP_34 RSRP_18 to RSRP_35 RSRP_19 to RSRP_36 depending on operating band |  |  |
| 9.1.41.2 FD-FDD Intra Frequency Relative RSRP Accuracy for UE category 0 |  |  | Test 1: Noc: -106dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported relative RSRP values: ±4dB Test 2: Noc: -86dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported relative RSRP values: ±4dB Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +3.0dB Ês2 / Noc: -1.0dB Reported relative RSRP values: ±4dB |  |  | Test 1: 0 dB 0 dB +1.0dB Via mapping Test 2: -1.0dB 0dB +1.0dB Via mapping Test 3: 0dB 0dB +1.0dB Via mapping |  |  |  |  |  | Test 1: Noc: -106 dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +2.0dB RSRP_x-10 to RSRP_x+2 Test 2: Noc: -87dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +2.0dB RSRP_x-10 to RSRP_x+2 Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +3.0dB Ês2 / Noc: 0dB RSRP_x-9 to RSRP_x+3 |  |  |
| 9.1.42.1 HD-FDD Intra Frequency Absolute RSRP Accuracy for UE category 0 |  |  | Same as 9.1.41.1 |  |  | Same as 9.1.41.1 |  |  |  |  |  | Same as 9.1.41.1 |  |  |
| 9.1.42.2 HD-FDD Intra Frequency Relative RSRP Accuracy for UE category 0 |  |  | Same as 9.1.41.2 |  |  | Same as 9.1.41.2 |  |  |  |  |  | Same as 9.1.41.2 |  |  |
| 9.1.43.1 TDD Intra Frequency Absolute RSRP Accuracy for UE category 0 |  |  | Same as 9.1.41.1 |  |  | Same as 9.1.41.1 |  |  |  |  |  | Same as 9.1.41.1 |  |  |
| 9.1.43.2 TDD Intra Frequency Relative RSRP Accuracy for UE category 0 |  |  | Same as 9.1.41.2 |  |  | Same as 9.1.41.2 |  |  |  |  |  | Same as 9.1.41.2 |  |  |
| 9.1.44 4 DL CA PCell in FDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation |  |  | Noc1: -117dBm or -116.5dBm or -116dBm or -115.5dBm or -115 dBm or -114dBm or -113.5dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Noc3: = Noc1 +1dB Noc4: = Noc1 +1dB Ês1 / Noc1: -4dB Ês2 / Noc2: +3dB Ês3 / Noc2: -1dB Ês4 / Noc3: +3dB Ês5 / Noc3: -1dB Ês6 / Noc4: +3dB Ês7 / Noc4: -1dB Reported absolute RSRP values: ±4.5dB for normal conditions and ±9dB for extreme conditions Reported relative RSRP values: (Cell 2 - Cell 1) and (Cell 4 - Cell 1), (Cell 6 - Cell 1): ±4.5dB for normal conditions ±6dB for extreme conditions (Cell 3 - Cell 2) and (Cell 5 - Cell 4), (Cell 7 - Cell 6): ±3dB |  |  | 0dB 0dB 0dB 0dB 0dB 0dB +0.8dB 0dB +0.8dB 0dB +0.8dB Via mapping Via mapping Via mapping |  |  |  |  |  | Noc1: -117dBm or -116.5dBm or -116dBm or -115.5dBm or -115 dBm or -114dBm or -113.5dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Noc3: = Noc1 +1dB Noc4: = Noc1 +1dB Ês1 / Noc1: -4dB Ês2 / Noc2: +3dB Ês3 / Noc2: -0.2dB Ês4 / Noc3: +3dB Ês5 / Noc3: -0.2dB Ês6 / Noc4: +3dB Ês7 / Noc4: -0.2dB Cell 1, absolute: RSRP_13 to RSRP_26 RSRP_14 to RSRP_26 RSRP_14 to RSRP_27 RSRP_15 to RSRP_27 RSRP_15 to RSRP_28 RSRP_16 to RSRP_29 RSRP_17 to RSRP_29 RSRP_20 to RSRP_32 depending on operating band Cell 2 and Cell 4, Cell 6, absolute: RSRP_21 to RSRP_34 RSRP_22 to RSRP_34 RSRP_22 to RSRP_35 RSRP_23 to RSRP_35 RSRP_23 to RSRP_36 RSRP_24 to RSRP_37 RSRP_25 to RSRP_37 RSRP_28 to RSRP_40 depending on Cell 1 band Relative: (Cell 2 - Cell 1) and (Cell 4 - Cell 1),(Cell 6 -Cell 1 ): RSRP_(x+2) to RSRP_(x+15) RSRP_(x-1) to RSRP_(x+17) (Cell 3 - Cell 2) and (Cell 5 - Cell 4), (Cell 7 - Cell 6): RSRP_(x-8) to RSRP_(x+1) The Test Tolerance values are taken from the analysis for the related 3DL Test case 9.1.37, and no separate analysis is needed. |  |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty from all applicable Noc and Ês / Noc values, the allowed UE reporting accuracy, and the UE mapping function. Where extreme condition UE accuracies differ from normal condition accuracies, reported values are recalculated. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.45 4 DL CA PCell in TDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation |  |  | Same as 9.1.44 |  |  | Same as 9.1.44 |  |  |  |  |  | Same as 9.1.44 |  |  |
| 9.1.46 4DL FDD RSRP for E-UTRAN in Carrier Aggregation |  |  | Same as 9.1.44 |  |  | Same as 9.1.44 |  |  |  |  |  | Same as 9.1.44 |  |  |
| 9.1.47 4DL TDD RSRP for E-UTRAN in Carrier Aggregation |  |  | Same as 9.1.44 |  |  | Same as 9.1.44 |  |  |  |  |  | Same as 9.1.44 |  |  |
| 9.1.48 5 DL FDD-TDD with PCell in FDD RSRP for E-UTRAN in Carrier Aggregation |  |  | Noc1: -117dBm or -116.5dBm or -116dBm or -115.5dBm or -115 dBm or -114dBm or -113.5dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Noc3: = Noc1 +1dB Noc4: = Noc1 +1dB Noc5: = Noc1 +1dB Ês1 / Noc1: -4dB Ês2 / Noc2: +3dB Ês3 / Noc2: -1dB Ês4 / Noc3: +3dB Ês5 / Noc3: -1dB Ês6 / Noc4: +3dB Ês7 / Noc4: -1dB Ês8 / Noc5: +3dB Ês9 / Noc5: -1dB Reported absolute RSRP values: ±4.5dB for normal conditions and ±9dB for extreme conditions Reported relative RSRP values: (Cell 2 - Cell 1) and (Cell 4 - Cell 1), (Cell 6 - Cell 1), (Cell 8 - Cell 1): ±4.5dB for normal conditions ±6dB for extreme conditions (Cell 3 - Cell 2) and (Cell 5 - Cell 4), (Cell 7 - Cell 6), (Cell 9 - Cell 8): ±3dB |  |  | 0dB 0dB 0dB 0dB 0dB 0dB 0dB +0.8dB 0dB +0.8dB 0dB +0.8dB 0dB +0.8dB Via mapping Via mapping Via mapping |  |  |  |  |  | Noc1: -117dBm or -116.5dBm or -116dBm or -115.5dBm or -115 dBm or -114dBm or -113.5dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Noc3: = Noc1 +1dB Noc4: = Noc1 +1dB Noc5: = Noc1 +1dB Ês1 / Noc1: -4dB Ês2 / Noc2: +3dB Ês3 / Noc2: -0.2dB Ês4 / Noc3: +3dB Ês5 / Noc3: -0.2dB Ês6 / Noc4: +3dB Ês7 / Noc4: -0.2dB Ês8 / Noc5: +3dB Ês9 / Noc5: -0.2dB Cell 1, absolute: RSRP_13 to RSRP_26 RSRP_14 to RSRP_26 RSRP_14 to RSRP_27 RSRP_15 to RSRP_27 RSRP_15 to RSRP_28 RSRP_16 to RSRP_29 RSRP_17 to RSRP_29 RSRP_20 to RSRP_32 depending on operating band Cell 2 and Cell 4, Cell 6, absolute: RSRP_21 to RSRP_34 RSRP_22 to RSRP_34 RSRP_22 to RSRP_35 RSRP_23 to RSRP_35 RSRP_23 to RSRP_36 RSRP_24 to RSRP_37 RSRP_25 to RSRP_37 RSRP_28 to RSRP_40 depending on Cell 1 band Relative: (Cell 2 - Cell 1) and (Cell 4 - Cell 1),(Cell 6 -Cell 1), (Cell 8 - Cell 1): RSRP_(x+2) to RSRP_(x+15) RSRP_(x-1) to RSRP_(x+17) (Cell 3 - Cell 2) and (Cell 5 - Cell 4), (Cell 7 - Cell 6), (Cell 9 - Cell 8): RSRP_(x-8) to RSRP_(x+1) The Test Tolerance values are taken from the analysis for the related 3DL Test case 9.1.37, and no separate analysis is needed. |  |  |
| 9.1.49 5 DL FDD-TDD with PCell in TDD RSRP for E-UTRAN in Carrier Aggregation |  |  | Same as 9.1.48 |  |  | Same as 9.1.48 |  |  |  |  |  | Same as 9.1.48 |  |  |
| 9.1.50 5 DL FDD RSRP for E-UTRAN in Carrier Aggregation |  |  | Same as 9.1.48 |  |  | Same as 9.1.48 |  |  |  |  |  | Same as 9.1.48 |  |  |
| 9.1.51 5 DL TDD RSRP for E-UTRAN in Carrier Aggregation |  |  | Same as 9.1.48 |  |  | Same as 9.1.48 |  |  |  |  |  | Same as 9.1.48 |  |  |
| 9.1.52 FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA |  |  | Test 1: Noc: -106dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported absolute RSRP values: Normal conditions: ±7dB Extreme conditions: ±10dB Reported relative RSRP values: Normal and extreme conditions: ±4dB Test 2: Noc: -86dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported absolute RSRP values: Normal conditions: ±9dB Extreme conditions: ±12dB Reported relative RSRP values: Normal and extreme conditions: ±4dB Test 3: Noc: -116dBm to -112.5 dBm/15kHz depending on operating band Ês1 / Noc: +3.0dB Ês2 / Noc: -1.0dB Reported absolute RSRP values: Normal conditions: ±7dB Extreme conditions: ±10dB Reported relative RSRP values: Normal and extreme conditions: ±4dB |  |  | Test 1: -1.0dB 0dB +1.0dB Via mapping Test 2: -1.0dB 0dB +1.0dB Via mapping Test 3: 0dB 0dB +0.8dB Via mapping |  |  |  |  |  | Test 1: Noc: -107dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +2.0dB RSRP_27 to RSRP_44 RSRP_24 to RSRP_47 RSRP_Cell 1 - 10 to RSRP_Cell 1 + 2 Test 2: Noc: -87dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +2.0dB RSRP_45 to RSRP_66 RSRP_52 to RSRP_69 RSRP_Cell 1 - 10 to RSRP_Cell 1 + 2 Test 3: Noc: -116dBm to -112.5 dBm/15kHz depending on operating band Ês1 / Noc: +3.0dB Ês2 / Noc: -0.2dB RSRP_16 to RSRP_33 RSRP_16 to RSRP_33 RSRP_17 to RSRP_34 RSRP_17 to RSRP_34 RSRP_18 to RSRP_35 RSRP_19 to RSRP_36 RSRP_19 to RSRP_36 RSRP_13 to RSRP_36 RSRP_13 to RSRP_36 RSRP_14 to RSRP_37 RSRP_14 to RSRP_37 RSRP_15 to RSRP_38 RSRP_16 to RSRP_39 RSRP_16 to RSRP_39 RSRP_Cell 1 - 9 to RSRP_Cell 1 + 2 |  |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty in Cell 1 and Cell 2 RSRP from Noc, Ês1 / Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The absolute RSRP values given above are different for normal and extreme conditions. The relative RSRP values given above are for both normal and extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.53 HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA |  |  | Same as 9.1.52 |  |  | Same as 9.1.52 |  |  |  |  |  | Same as 9.1.52 |  |  |
| 9.1.54 TDD RSRP Intra frequency case for Cat-M1 UE in CEModeA |  |  | Same as 9.1.52 |  |  | Same as 9.1.52 |  |  |  |  |  | Same as 9.1.52 |  |  |
| 9.1.55 FS3 Intra frequency absolute and relative RSRP accuracies with FDD PCell9.1.55 FS3 Intra frequency absolute and relative RSRP accuracies with FDD PCell |  |  | Test 1: Noc1: -117dBm to -113.5dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Ês1 / Noc1: -4dB Ês2 / Noc2: 3dB Ês3 / Noc2: -1dB Reported absolute RSRP values: Normal condtion: ±4.5dB Extreme condtion: ±9dB Reported relative RSRP values: (Cell 3 - Cell 2): ±3dB (Cell 3 - Cell 1): Normal condtion: ±4.5dB Extreme condtion: ±6dB |  |  | Test 1: 0dB 0dB 0dB 0dB +0.8dB Via mapping Via mapping Via mapping |  |  |  |  |  | Test 1: Noc1: -117dBm to -113.5dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Ês1 / Noc1: -4dB Ês2 / Noc2: 3dB Ês3 / Noc2: -0.2dB Cell 3, absolute: RSRP_18 to RSRP_31 RSRP_18 to RSRP_31 RSRP_19 to RSRP_32 RSRP_19 to RSRP_32 RSRP_20 to RSRP_33 RSRP_21 to RSRP_34 RSRP_22 to RSRP_34 depending on operating band Cell 3, absolute: RSRP_13 to RSRP_35 RSRP_14 to RSRP_36 RSRP_14 to RSRP_36 RSRP_15 to RSRP_37 RSRP_15 to RSRP_37 RSRP_16 to RSRP_38 RSRP_17 to RSRP_39 depending on operating band Relative: RSRP_(x-8) to RSRP_(x+1) RSRP_(x-3) to RSRP_(x+12) RSRP_(x-4) to RSRP_(x+14) |  |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty from all applicable Noc and Ês / Noc values, the allowed UE reporting accuracy, and the UE mapping function. Where extreme condition UE accuracies differ from normal condition accuracies, reported values are recalculated. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.56 FS3 Intra frequency absolute and relative RSRP accuracies with TDD PCell |  |  | Same as 9.1.55 |  |  | Same as 9.1.55 |  |  |  |  |  | Same as 9.1.55 |  |  |
| 9.1.57 FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB |  |  | Test 1: Noc: = -99dBm/15kHz Ês1 / Noc: -12dB Ês2 / Noc: -14dB Reported absolute RSRP values: Normal condtion: ±8dB Extreme condtion: ±11dB Reported relative RSRP values: Normal condtion: ±5dB Extreme condtion: ±5dB Test 2: Noc: = -79dBm/15kHz Ês1 / Noc: -12dB Ês2 / Noc: -14dB Reported absolute RSRP values: Normal condtion: ±10dB Extreme condtion: ±13dB Reported relative RSRP values: Normal condtion: ±5dB Extreme condtion: ±5dB Test 3: Noc: = -107dBm~-103.5dBm/15kHz depending on operating band Ês1 / Noc: -12dB Ês2 / Noc: -14dB Reported absolute RSRP values: Normal condtion: ±8dB Extreme condtion: ±11dB Reported relative RSRP values: Normal condtion: ±5dB Extreme condtion: ±5dB |  |  | Test 1: -0.3dB 0dB +0.1dB Via mapping Via mapping Test 2: -0.3dB 0dB +0.1dB Via mapping Via mapping Test 3: 0dB 0dB +0.1dB Via mapping Via mapping Via mapping |  |  |  |  |  | Test 1: Noc: = -99.3dBm/15kHz Ês1 / Noc: -12dB Ês2 / Noc: -13.9dB Cell 2, absolute: RSRP_18 to RSRP_37 RSRP_15 to RSRP_40 Relative: RSRP_(x-9) to RSRP_(x+5) RSRP_(x-9) to RSRP_(x+5) Test 2: Noc: = -79.3dBm/15kHz Ês1 / Noc: -12dB Ês2 / Noc: -13.9dB Cell 2, absolute: RSRP_36 to RSRP_59 RSRP_33 to RSRP_62 Relative: RSRP_(x-9) to RSRP_(x+5) RSRP_(x-9) to RSRP_(x+5) Test 3: Noc: = -107dBm~-103.5dBm/15kHz depending on operating band Ês1 / Noc: -12dB Ês2 / Noc: -13,9dB Cell 2, absolute: RSRP_10 to RSRP_29 RSRP_11 to RSRP_30 RSRP_11 to RSRP_30 RSRP_12 to RSRP_31 RSRP_12 to RSRP_31 RSRP_13 to RSRP_32 RSRP_14 to RSRP_33 depending on operating band Cell 2, absolute: RSRP_7 to RSRP_32 RSRP_8 to RSRP_33 RSRP_8 to RSRP_33 RSRP_9 to RSRP_34 RSRP_9 to RSRP_34 RSRP_10 to RSRP_35 RSRP_11 to RSRP_36 depending on operating band Relative: RSRP_(x-9) to RSRP_(x+5) RSRP_(x-9) to RSRP_(x+5) |  |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty from all applicable Noc and Ês / Noc values, the allowed UE reporting accuracy, and the UE mapping function. Where extreme condition UE accuracies differ from normal condition accuracies, reported values are recalculated. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.58 HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB |  |  | Same as 9.1.57 |  |  | Same as 9.1.57 |  |  |  |  |  | Same as 9.1.57 |  |  |
|  |  |  | Same as 9.1.57 |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.59 TDD RSRP Intra frequency case for Cat-M1 UE in CEModeB |  |  | Same as 9.1.57 |  |  | Same as 9.1.57 |  |  |  |  |  | Same as 9.1.57 |  |  |
|  |  |  | Same as 9.1.57 |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.60 FS3 absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal with FDD PCell |  |  | Noc1: -117dBm or -116.5dBm or -116dBm or -115.5dBm or -115 dBm or -114dBm or -113.5dBm /15kHz depending on operating band Noc2: = Noc1 +1dB CRS Ês1 / Noc1: -4dB CRS Ês2 / Noc2: +3dB CRS Ês3 / Noc2: -1dB CSI-RS Ês2 / Noc2: +9dB CSI-RS Ês3 / Noc2: +5dB Reported absolute RSRP values: ±4.5dB for normal conditions and ±9dB for extreme conditions Reported relative RSRP values: (Cell 3 - Cell 2): ±2dB for normal conditions and ±3dB for extreme conditions |  |  | 0dB 0dB 0dB 0dB 0dB 0dB +0.8dB Via mapping Via mapping |  |  |  |  |  | Noc1: -117dBm or -116.5dBm or -116dBm or -115.5dBm or -115 dBm or -114dBm or -113.5dBm /15kHz depending on operating band Noc2: = Noc1 +1dB CRS Ês1 / Noc1: -4dB CRS Ês2 / Noc2: +3dB CRS Ês3 / Noc2: -1dB CSI-RS Ês2 / Noc2: +9dB CSI-RS Ês3 / Noc2: +5.8dB Cell 2, absolute: RSRP_27 to RSRP_40 RSRP_28 to RSRP_40 RSRP_28 to RSRP_41 RSRP_29 to RSRP_41 RSRP_29 to RSRP_42 RSRP_30 to RSRP_43 RSRP_31 to RSRP_43 depending on Cell 1 operating band Cell 3, absolute: RSRP_24 to RSRP_36 RSRP_25 to RSRP_37 RSRP_25 to RSRP_37 RSRP_26 to RSRP_38 RSRP_26 to RSRP_38 RSRP_27 to RSRP_39 RSRP_28 to RSRP_40 depending on Cell 1 operating band Relative: RSRP_(x-7) to RSRP_(x-1) |  |  |
|  |  |  | The derivation of the RSRP values takes into account the uncertainty from all applicable Noc ,CRS Ês / Noc and CSI-RS Ês / Noc values, the allowed UE reporting accuracy, and the UE mapping function. Where extreme condition UE accuracies differ from normal condition accuracies, reported values are recalculated. |  |  |  |  |  |  |  |  |  |  |  |
| 9.1.61 FS3 absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal with TDD PCell |  |  | Same as 9.1.60 |  |  | Same as 9.1.60 |  |  |  |  |  | Same as 9.1.60 |  |  |
| 9.1.68 3DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes |  |  | Same as 9.1.37 |  |  | Same as 9.1.37 |  |  |  |  |  | Same as 9.1.37 |  |  |
| 9.1.69 4 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes |  |  | Same as 9.1.44 |  |  | Same as 9.1.44 |  |  |  |  |  | Same as 9.1.44 |  |  |
| 9.1.70 5 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes |  |  | Same as 9.1.48 |  |  | Same as 9.1.48 |  |  |  |  |  | Same as 9.1.48 |  |  |
| 9.1.71 6 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes |  |  | Noc1: -117dBm or -116.5dBm or -116dBm or -115.5dBm or -115 dBm or -114dBm or -113.5dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Noc3: = Noc1 +1dB Noc4: = Noc1 +1dB Noc5: = Noc1 +1dB Ês1 / Noc1: -4dB Ês2 / Noc2: +3dB Ês3 / Noc2: -1dB Ês4 / Noc3: +3dB Ês5 / Noc3: -1dB Ês6 / Noc4: +3dB Ês7 / Noc4: -1dB Ês8 / Noc5: +3dB Ês9 / Noc5: -1dB Ês10 / Noc6: +3dB Ês11 / Noc6: -1dB Reported absolute RSRP values: ±4.5dB for normal conditions and ±9dB for extreme conditions Reported relative RSRP values: (Cell 2 – Cell 1) and (Cell 4 – Cell 1), (Cell 6 – Cell 1), (Cell 8 – Cell 1), (Cell 10 – Cell 1): ±4.5dB for normal conditions ±6dB for extreme conditions (Cell 3 - Cell 2) and (Cell 5 - Cell 4), (Cell 7 - Cell 6), (Cell 9 – Cell 8), (Cell 11 – Cell 10): ±3dB |  |  | 0dB 0dB 0dB 0dB 0dB 0dB 0dB +0.8dB 0dB +0.8dB 0dB +0.8dB 0dB +0.8dB 0dB +0.8dB Via mapping Via mapping Via mapping |  |  |  |  |  | Noc1: -117dBm or -116.5dBm or -116dBm or -115.5dBm or -115 dBm or -114dBm or -113.5dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Noc3: = Noc1 +1dB Noc4: = Noc1 +1dB Noc5: = Noc1 +1dB Ês1 / Noc1: -4dB Ês2 / Noc2: +3dB Ês3 / Noc2: -0.2dB Ês4 / Noc3: +3dB Ês5 / Noc3: -0.2dB Ês6 / Noc4: +3dB Ês7 / Noc4: -0.2dB Ês8 / Noc5: +3dB Ês9 / Noc5: -0.2dB Ês10 / Noc6: +3dB Ês11 / Noc6: -0.2dB Cell 1, absolute: RSRP_13 to RSRP_26 RSRP_14 to RSRP_26 RSRP_14 to RSRP_27 RSRP_15 to RSRP_27 RSRP_15 to RSRP_28 RSRP_16 to RSRP_29 RSRP_17 to RSRP_29 depending on operating band Cell 2 and Cell 4, Cell 6, Cell 8, Cell 10 absolute: RSRP_21 to RSRP_34 RSRP_22 to RSRP_34 RSRP_22 to RSRP_35 RSRP_23 to RSRP_35 RSRP_23 to RSRP_36 RSRP_24 to RSRP_37 RSRP_25 to RSRP_37 depending on Cell 1 band Relative: (Cell 2 – Cell 1) and (Cell 4 – Cell 1),(Cell 6 –Cell 1), (Cell 8 – Cell 1), (Cell 10 – Cell 1): RSRP_(x+2) to RSRP_(x+15) RSRP_(x-1) to RSRP_(x+17) (Cell 3 - Cell 2) and (Cell 5 - Cell 4), (Cell 7 - Cell 6), (Cell 9 – Cell 8), (Cell 11 – Cell 10): RSRP_(x-8) to RSRP_(x+1) The Test Tolerance values are taken from the analysis for the related 3DL Test case 9.1.37, and no separate analysis is needed. |  |  |
| 9.1.72 7 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes |  |  | Noc1: -117dBm or -116.5dBm or -116dBm or -115.5dBm or -115 dBm or -114dBm or -113.5dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Noc3: = Noc1 +1dB Noc4: = Noc1 +1dB Noc5: = Noc1 +1dB Ês1 / Noc1: -4dB Ês2 / Noc2: +3dB Ês3 / Noc2: -1dB Ês4 / Noc3: +3dB Ês5 / Noc3: -1dB Ês6 / Noc4: +3dB Ês7 / Noc4: -1dB Ês8 / Noc5: +3dB Ês9 / Noc5: -1dB Ês10 / Noc6: +3dB Ês11 / Noc6: -1dB Ês12 / Noc7: +3dB Ês13 / Noc7: -1dB Reported absolute RSRP values: ±4.5dB for normal conditions and ±9dB for extreme conditions Reported relative RSRP values: (Cell 2 – Cell 1) and (Cell 4 – Cell 1), (Cell 6 – Cell 1), (Cell 8 – Cell 1), (Cell 10 – Cell 1), (Cell 12 – Cell 1): ±4.5dB for normal conditions ±6dB for extreme conditions (Cell 3 - Cell 2) and (Cell 5 - Cell 4), (Cell 7 - Cell 6), (Cell 9 – Cell 8), (Cell 11 – Cell 10), (Cell 13 – Cell 12): ±3dB |  |  | 0dB 0dB 0dB 0dB 0dB 0dB 0dB +0.8dB 0dB +0.8dB 0dB +0.8dB 0dB +0.8dB 0dB +0.8dB 0dB +0.8dB Via mapping Via mapping Via mapping |  |  |  |  |  | Noc1: -117dBm or -116.5dBm or -116dBm or -115.5dBm or -115 dBm or -114dBm or -113.5dBm /15kHz depending on operating band Noc2: = Noc1 +1dB Noc3: = Noc1 +1dB Noc4: = Noc1 +1dB Noc5: = Noc1 +1dB Ês1 / Noc1: -4dB Ês2 / Noc2: +3dB Ês3 / Noc2: -0.2dB Ês4 / Noc3: +3dB Ês5 / Noc3: -0.2dB Ês6 / Noc4: +3dB Ês7 / Noc4: -0.2dB Ês8 / Noc5: +3dB Ês9 / Noc5: -0.2dB Ês10 / Noc6: +3dB Ês11 / Noc6: -0.2dB Ês12 / Noc6: +3dB Ês13 / Noc6: -0.2dB Cell 1, absolute: RSRP_13 to RSRP_26 RSRP_14 to RSRP_26 RSRP_14 to RSRP_27 RSRP_15 to RSRP_27 RSRP_15 to RSRP_28 RSRP_16 to RSRP_29 RSRP_17 to RSRP_29 depending on operating band Cell 2 and Cell 4, Cell 6, Cell 8, Cell 10, Cell 12 absolute: RSRP_21 to RSRP_34 RSRP_22 to RSRP_34 RSRP_22 to RSRP_35 RSRP_23 to RSRP_35 RSRP_23 to RSRP_36 RSRP_24 to RSRP_37 RSRP_25 to RSRP_37 depending on Cell 1 band Relative: (Cell 2 – Cell 1) and (Cell 4 – Cell 1),(Cell 6 –Cell 1), (Cell 8 – Cell 1), (Cell 10 – Cell 1), (Cell 12 – Cell 1): RSRP_(x+2) to RSRP_(x+15) RSRP_(x-1) to RSRP_(x+17) (Cell 3 - Cell 2) and (Cell 5 - Cell 4), (Cell 7 - Cell 6), (Cell 9 – Cell 8), (Cell 11 – Cell 10), (Cell 13 – Cell 12): RSRP_(x-8) to RSRP_(x+1) The Test Tolerance values are taken from the analysis for the related 3DL Test case 9.1.37, and no separate analysis is needed. |  |  |
| 9.2.1.1 FDD Intra Frequency Absolute RSRQ Accuracy |  |  | Test 1: Noc: -84.76dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB Reported RSRQ values: ±2.5dB Test 2: Noc: -103.85dBm/15kHz Ês1 / Noc: -2.9dB Ês2 / Noc: -2.9dB Reported RSRQ values: ±3.5dB Test 3: Noc: -116dBm or -114dBm or -113dBm or -115dBm /15kHz depending on operating band Ês1 / Noc: -4.0dB Ês2 / Noc: -4.0dB Reported RSRQ values: ±3.5dB |  |  | Test 1: -0.75dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB Via mapping Test 3: 0dB +0.4dB +0.4dB Via mapping |  |  |  |  |  | Test 1: Noc: -85.51Bm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB RSRQ_04 to RSRQ_16 Test 2: Noc: -103.85dBm/15kHz Ês1 / Noc: -2.9dB Ês2 / Noc: -2.9dB RSRQ_00 to RSRP_14 Test 3: Noc: -116dBm or -114dBm or -113dBm or -115dBm /15kHz depending on operating band Ês1 / Noc: -3.6dB Ês2 / Noc: -3.6dB RSRQ_00 to RSRQ_14 |  |  |
|  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 2 RSRQ from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRQ values given above are for normal conditions. For test 1 the RSRQ values are 1.5dB wider at each end for extreme conditions, and for tests 2 and 3 the RSRQ values are 0.5dB wider at each end for extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.1.1_2 FDD Intra Frequency Absolute RSRQ Accuracy for UE Category 1bis |  |  | Test 1: Noc: -84.76dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB Reported RSRQ values: ±3.5dB Test 2: Noc: -103.85dBm/15kHz Ês1 / Noc: -2.9dB Ês2 / Noc: -2.9dB Reported RSRQ values: ±4.5dB Test 3: Noc: -116dBm or -114dBm or -113dBm or -115dBm /15kHz depending on operating band Ês1 / Noc: -4.0dB Ês2 / Noc: -4.0dB Reported RSRQ values: ±4.5dB |  |  | Test 1: -1.05dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB Via mapping Test 3: 0dB +0.4dB +0.4dB Via mapping |  |  |  |  |  | Test 1: Noc: -85.81Bm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB RSRQ_02 to RSRQ_18 Test 2: Noc: -103.85dBm/15kHz Ês1 / Noc: -2.9dB Ês2 / Noc: -2.9dB RSRQ_00 to RSRP_16 Test 3: Noc: -116dBm or -114dBm or -113dBm or -115dBm /15kHz depending on operating band Ês1 / Noc: -3.6dB Ês2 / Noc: -3.6dB RSRQ_00 to RSRQ_16 |  |  |
|  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 2 RSRQ from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRQ values given above are for normal conditions. For test 1 the RSRQ values are 1.5dB wider at each end for extreme conditions, and for tests 2 and 3 the RSRQ values are 0.5dB wider at each end for extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.1.1_3 FDD Intra Frequency Absolute RSRQ Accuracy for CA Idle Mode Measurements |  |  | Test 1: Noc: -84.76dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB Reported RSRQ values: ±3.5dB Test 2: Noc: -103.85dBm/15kHz Ês1 / Noc: -2.9dB Ês2 / Noc: -2.9dB Reported RSRQ values: ±4.5dB Test 3: Noc: -116dBm or -114dBm or -113dBm or -115dBm /15kHz depending on operating band Ês1 / Noc: -4.0dB Ês2 / Noc: -4.0dB Reported RSRQ values: ±4.5dB |  |  | Test 1: -1.05dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB Via mapping Test 3: 0dB +0.4dB +0.4dB Via mapping |  |  |  |  |  | Test 1: Noc: -85.81Bm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB RSRQ_02 to RSRQ_18 Test 2: Noc: -103.85dBm/15kHz Ês1 / Noc: -2.9dB Ês2 / Noc: -2.9dB RSRQ_00 to RSRP_16 Test 3: Noc: -116dBm or -114dBm or -113dBm or -115dBm /15kHz depending on operating band Ês1 / Noc: -3.6dB Ês2 / Noc: -3.6dB RSRQ_00 to RSRQ_16 |  |  |
|  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 2 RSRQ from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRQ values given above are for normal conditions. For test 1 the RSRQ values are 1.5dB wider at each end for extreme conditions, and for tests 2 and 3 the RSRQ values are 0.5dB wider at each end for extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.2.1 TDD Intra Frequency Absolute RSRQ Accuracy |  |  | Test 1: Noc: -84.76dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB Reported RSRQ values: ±2.5dB Test 2: Noc: -103.85dBm/15kHz Ês1 / Noc: -2.9dB Ês2 / Noc: -2.9dB Reported RSRQ values: ±3.5dB Test 3: Noc: -116dBm/15kHz Ês1 / Noc: -4.0dB Ês2 / Noc: -4.0dB Reported RSRQ values: ±3.5dB |  |  | Test 1: -0.75dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB Via mapping Test 3: 0dB +0.4dB +0.4dB Via mapping |  |  |  |  |  | Test 1: Noc: -85.51Bm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB RSRQ_04 to RSRQ_16 Test 2: Noc: -103.85dBm/15kHz Ês1 / Noc: -2.9dB Ês2 / Noc: -2.9dB RSRQ_00 to RSRQ_14 Test 3: Noc: -116dBm/15kHz Ês1 / Noc: -3.6dB Ês2 / Noc: -3.6dB RSRQ_00 to RSRQ_14 |  |  |
|  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 2 RSRQ from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRQ values given above are for normal conditions. For test 1 the RSRQ values are 1.5dB wider at each end for extreme conditions, and for test 2 the RSRQ values are 0.5dB wider at each end for extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.2.1_2 TDD Intra Frequency Absolute RSRQ Accuracy for UE Category 1bis |  |  | Same as 9.2.1.1_2 |  |  | Same as 9.2.1.1_2 |  |  |  |  |  | Same as 9.2.1.1_2 |  |  |
| 9.2.3.1 FDD - FDD Inter Frequency Absolute RSRQ Accuracy |  |  | Test 1: Noc1: -80dBm/15kHz Noc2: -80dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB for normal conditions and ±4dB for extreme conditions Test 2: Noc1: -104.7dBm/15kHz Noc2: -104.7dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -4dB Reported RSRQ values: ±3.5dB for normal conditions and ±4dB for extreme conditions Test 3: Noc1: -119.5dBm to -116dBm/15kHz depending on operating band Noc2: -119.5dBm to -116dBm/15kHz depending on operating band Ês1 / Noc1: -4dB Ês2 / No2c: -4dB Reported RSRQ values: ±3.5dB for normal conditions and ±4dB for extreme conditions |  |  | Test 1: 0dB -1.1dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0.8dB Via mapping Test 3: 0dB 0.3dB 0dB 0.8dB Via mapping |  |  |  |  |  | Test 1: Noc1: -80dBm/15kHz Noc2: -81.1dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_04 to RSRQ_16 (NTC) RSRQ_01 to RSRQ_19 (ETC) Test 2: Noc1: -104.7dBm/15kHz Noc2: -104.7dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -3.2dB RSRQ_00 to RSRQ_16 (NTC) RSRQ_00 to RSRQ_17 (ETC) Test 3: Noc1: -119.5dBm to-116 dBm /15kHz depending on operating band Noc2: -119.2dBm -115.7dBm /15kHz depending on operating band Ês1 / Noc1: -4dB Ês2 / No2c: -3.2dB RSRQ_00 to RSRQ_16 (NTC) RSRQ_00 to RSRQ_17 (ETC) |  |  |
|  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 2 RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.3.1_2 FDD - FDD Inter Frequency Absolute RSRQ Accuracy for UE Category 1bis |  |  | Test 1: Noc1: -80dBm/15kHz Noc2: -80dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±3.5dB for normal conditions and ±5dB for extreme conditions Test 2: Noc1: -104.7dBm/15kHz Noc2: -104.7dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -4dB Reported RSRQ values: ±4.5dB for normal conditions and ±5dB for extreme conditions Test 3: Noc1: -119.5dBm to -116dBm/15kHz depending on operating band Noc2: -119.5dBm to -116dBm/15kHz depending on operating band Ês1 / Noc1: -4dB Ês2 / No2c: -4dB Reported RSRQ values: ±4.5dB for normal conditions and ±5dB for extreme conditions |  |  | Test 1: 0dB -1.1dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0.8dB Via mapping Test 3: 0dB 0.3dB 0dB 0.8dB Via mapping |  |  |  |  |  |  |  | Test 1: Noc1: -80dBm/15kHz Noc2: -81.1dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_02 to RSRQ_18 (NTC) RSRQ_00 to RSRQ_21 (ETC) Test 2: Noc1: -104.7dBm/15kHz Noc2: -104.7dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -3.2dB RSRQ_00 to RSRQ_18 (NTC) RSRQ_00 to RSRQ_19 (ETC) Test 3: Noc1: -119.5dBm to-116 dBm /15kHz depending on operating band Noc2: -119.2dBm -115.7dBm /15kHz depending on operating band Ês1 / Noc1: -4dB Ês2 / No2c: -3.2dB RSRQ_00 to RSRQ_18 (NTC) RSRQ_00 to RSRQ_19 (ETC) |
|  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 2 RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.3.1_3 FDD - FDD Inter Frequency Absolute RSRQ Accuracy for CA Idle Mode Measurements for Overlapping Carrier |  |  | Test 1: Noc1: -80 dBm/15kHz Noc2: -80 dBm/15kHz Ês1 / Noc1: -1,75 dB Ês2 / Noc2: -1,75 dB Reported RSRQ values: ±4 dB for normal conditions and ±5.5 dB for extreme conditions Test 2: Noc1: -104.7 dBm/15kHz Noc2: -104.7 dBm/15kHz Ês1 / Noc1: -4 dB Ês2 / No2c: -4 dB Reported RSRQ values: ±5 dB for normal conditions and ±5.5 dB for extreme conditions Test 3: Noc1: -119.5 dBm to -116 dBm/15kHz depending on operating band Noc2: -119.5 dBm to -116 dBm/15kHz depending on operating band Ês1 / Noc1: -4 dB Ês2 / No2c: -4 dB Reported RSRQ values: ±5 dB for normal conditions and ±5.5 dB for extreme conditions |  |  | Test 1: 0dB -1.1dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0.8dB Via mapping Test 3: 0dB 0.3dB 0dB 0.8dB Via mapping |  |  |  |  |  |  |  | Test 1: Noc1: -80 dBm/15kHz Noc2: -81,1 dBm/15kHz Ês1 / Noc1: -1,75 dB Ês2 / Noc2: -1,75 dB RSRQ 1 to RSRQ 19 (NTC) RSRQ 0 to RSRQ 22 (ETC) Test 2: Noc1: -104.7 dBm/15kHz Noc2: -104.7 dBm/15kHz Ês1 / Noc1: -4 dB Ês2 / No2c: -3.2 dB RSRQ 0 to RSRQ 19 (NTC) RSRQ 0 to RSRQ 20 (ETC) Test 3: Noc1: -119.5 dBm to -116 dBm /15kHz depending on operating band Noc2: -119.2 dBm to -115.7 dBm /15kHz depending on operating band Ês1 / Noc1: -4 dB Ês2 / No2c: -3.2 dB RSRQ 0 to RSRQ 19 (NTC) RSRQ 0 to RSRQ 20 (ETC) |
|  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 2 RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.3.1_4 FDD - FDD Inter Frequency Absolute RSRQ Accuracy for CA Idle Mode Measurements for Overlapping Carrier |  |  | Test 1: Noc1: -80 dBm/15kHz Noc2: -80 dBm/15kHz Ês1 / Noc1: -1,75 dB Ês2 / Noc2: -1,75 dB Reported RSRQ values: ±5 dB for normal conditions and ±6.5 dB for extreme conditions Test 2: Noc1: -104.7 dBm/15kHz Noc2: -104.7 dBm/15kHz Ês1 / Noc1: -4 dB Ês2 / No2c: -4 dB Reported RSRQ values: ±6 dB for normal conditions and ±6.5 dB for extreme conditions Test 3: Noc1: -119.5 dBm to -116 dBm/15kHz depending on operating band Noc2: -119.5 dBm to -116 dBm/15kHz depending on operating band Ês1 / Noc1: -4 dB Ês2 / No2c: -4 dB Reported RSRQ values: ±6 dB for normal conditions and ±6.5 dB for extreme conditions |  |  | Test 1: 0dB -1.1dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0.8dB Via mapping Test 3: 0dB 0.3dB 0dB 0.8dB Via mapping |  |  |  |  |  |  |  | Test 1: Noc1: -80 dBm/15kHz Noc2: -81,1 dBm/15kHz Ês1 / Noc1: -1,75 dB Ês2 / Noc2: -1,75 dB RSRQ 0 to RSRQ 21 (NTC) RSRQ 0 to RSRQ 24 (ETC) Test 2: Noc1: -104.7 dBm/15kHz Noc2: -104.7 dBm/15kHz Ês1 / Noc1: -4 dB Ês2 / No2c: -3.2 dB RSRQ 0 to RSRQ 21 (NTC) RSRQ 0 to RSRQ 22 (ETC) Test 3: Noc1: -119.5 dBm to -116 dBm /15kHz depending on operating band Noc2: -119.2 dBm to -115.7 dBm /15kHz depending on operating band Ês1 / Noc1: -4 dB Ês2 / No2c: -3.2 dB RSRQ 0 to RSRQ 21 (NTC) RSRQ 0 to RSRQ 22 (ETC) |
|  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 2 RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.3.2 FDD - FDD Inter Frequency Relative RSRQ Accuracy |  |  | Test 1: Noc1: -80dBm/15kHz Noc2: -80dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported Relative RSRQ values: ±3dB for normal conditions and ±4dB for extreme conditions Test 2: Noc1: -104.7dBm/15kHz Noc2: -104.7dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -4dB Reported Relative RSRQ values: ±4dB Test 3: Noc1: -119.5dBm to -116dBm/15kHz depending on operating band Noc2: -119.5dBm to -116dBm /15kHz depending on operating band Ês1 / Noc1: -4dB Ês2 / No2c: -4dB Reported Relative RSRQ values: ±4dB |  |  | Test 1: -1.1dB -1.1dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0.8dB Via mapping Test 3: 0dB +0.3dB 0dB 0.8dB Via mapping |  |  |  |  |  | Test 1: Noc1: -81.1dBm/15kHz Noc2: -81.1dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_x - 8 to RSRQ_x + 8 (NTC) RSRQ_x - 10 to RSRQ_x + 10 (ETC) Test 2: Noc1: -104.7dBm/15kHz Noc2: -104.7dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -3.2dB RSRQ_x - 9 to RSRQ_x + 11 Test 3: Noc1: -119.5dBm to -116dBm/15kHz depending on operating band Noc2: -119.2dBm to -115.7dBm /15kHz depending on operating band Ês1 / Noc1: -4dB Ês2 / No2c: -3.2dB RSRQ_x - 9 to RSRQ_x + 11 |  |  |
|  |  |  | The derivation of the relative RSRQ values takes into account the uncertainty in Cell 1 RSRQ from Noc1 and Ês1 / Noc1 and Cell 2 RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.3.2_2 FDD - FDD Inter Frequency Relative RSRQ Accuracy for UE Category 1bis |  |  | Test 1: Noc1: -80dBm/15kHz Noc2: -80dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported Relative RSRQ values: ±4dB for normal conditions and ±4dB for extreme conditions Test 2: Noc1: -104.7dBm/15kHz Noc2: -104.7dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -4dB Reported Relative RSRQ values: ±5dB Test 3: Noc1: -119.5dBm to -116dBm/15kHz depending on operating band Noc2: -119.5dBm to -116dBm /15kHz depending on operating band Ês1 / Noc1: -4dB Ês2 / No2c: -4dB Reported Relative RSRQ values: ±5dB |  |  | Test 1: -1.1dB -1.1dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0.8dB Via mapping Test 3: 0dB +0.3dB 0dB 0.8dB Via mapping |  |  |  |  |  | Test 1: Noc1: -81.1dBm/15kHz Noc2: -81.1dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_x - 10 to RSRQ_x + 10 (NTC) RSRQ_x - 12 to RSRQ_x + 12 (ETC) Test 2: Noc1: -104.7dBm/15kHz Noc2: -104.7dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -3.2dB RSRQ_x - 11 to RSRQ_x + 13 Test 3: Noc1: -119.5dBm to -116dBm/15kHz depending on operating band Noc2: -119.2dBm to -115.7dBm /15kHz depending on operating band Ês1 / Noc1: -4dB Ês2 / No2c: -3.2dB RSRQ_x - 11 to RSRQ_x + 13 |  |  |
|  |  |  | The derivation of the relative RSRQ values takes into account the uncertainty in Cell 1 RSRQ from Noc1 and Ês1 / Noc1 and Cell 2 RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.4.1 TDD - TDD Inter Frequency Absolute RSRQ Accuracy |  |  | Same as 9.2.3.1 |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.4.1_1 TDD - TDD Inter Frequency Absolute RSRQ Accuracy (Rel-12 and forward) |  |  | Same as 9.2.3.1 |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.4.1_2 TDD - TDD Inter Frequency Absolute RSRQ Accuracy for UE Category 1bis |  |  | Same as 9.2.3.1_2 |  |  | Same as 9.2.3.1_2 |  |  |  |  |  | Same as 9.2.3.1_2 |  |  |
| 9.2.4.2 TDD - TDD Inter Frequency Relative RSRQ Accuracy |  |  | Same as 9.2.3.2 |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.4.2_1 TDD - TDD Inter Frequency Relative RSRQ Accuracy (Rel-12 and forward) |  |  | Same as 9.2.3.2 |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.4.2_2 TDD - TDD Inter Frequency Relative RSRQ Accuracy for UE Category 1bis |  |  | Same as 9.2.3.2_2 |  |  | Same as 9.2.3.2_2 |  |  |  |  |  | Same as 9.2.3.2_2 |  |  |
| 9.2.4A.1 FDD - TDD Inter Frequency Absolute RSRQ Accuracy |  |  | Test 1: Noc1: -80dBm/15kHz Noc2: -80dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB for normal conditions and ±4dB for extreme conditions Test 2: Noc1: -104.7dBm/15kHz Noc2: -104.7dBm/15kHz Ês1 / Noc1: -4.0dB Ês2 / No2c: -4.0dB Reported RSRQ values: ±3.5dB for normal conditions and ±4dB for extreme conditions Test 3: Noc1: -114.5dBm/15kHz Noc2: -114.5dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -4dB Reported RSRQ values: ±3.5dB for normal conditions and ±4dB for extreme conditions |  |  | Test 1: 0dB -1.10dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0.8dB Via mapping Test 3: 0dB 0dB 0dB 0.8dB Via mapping |  |  |  |  |  | Test 1: Noc1: -80dBm/15kHz Noc2: -81.10dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_04 to RSRQ_16 (NTC) RSRQ_01 to RSRQ_19 (ETC) Test 2: Noc1: -104.7dBm/15kHz Noc2: -104.7dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -3.2dB RSRQ_00 to RSRQ_16 (NTC) RSRQ_00 to RSRQ_17 (ETC) Test 3: Noc1: -114.5dBm/15kHz Noc2: -114.5dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -3.2dB RSRQ_00 to RSRQ_16 (NTC) RSRQ_00 to RSRQ_17 (ETC) |  |  |
|  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 2 RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.4A.1_2 FDD - TDD Inter Frequency Absolute RSRQ Accuracy for UE Category 1bis |  |  | Test 1: Noc1: -80dBm/15kHz Noc2: -80dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±3.5dB for normal conditions and ±5dB for extreme conditions Test 2: Noc1: -104.7dBm/15kHz Noc2: -104.7dBm/15kHz Ês1 / Noc1: -4.0dB Ês2 / No2c: -4.0dB Reported RSRQ values: ±4.5dB for normal conditions and ±5dB for extreme conditions Test 3: Noc1: -114.5dBm/15kHz Noc2: -114.5dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -4dB Reported RSRQ values: ±4.5dB for normal conditions and ±5dB for extreme conditions |  |  | Test 1: 0dB -1.10dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0.8dB Via mapping Test 3: 0dB 0dB 0dB 0.8dB Via mapping |  |  |  |  |  | Test 1: Noc1: -80dBm/15kHz Noc2: -81.10dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_02 to RSRQ_18 (NTC) RSRQ_00 to RSRQ_21 (ETC) Test 2: Noc1: -104.7dBm/15kHz Noc2: -104.7dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -3.2dB RSRQ_00 to RSRQ_18 (NTC) RSRQ_00 to RSRQ_19 (ETC) Test 3: Noc1: -114.5dBm/15kHz Noc2: -114.5dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -3.2dB RSRQ_00 to RSRQ_18 (NTC) RSRQ_00 to RSRQ_19 (ETC) |  |  |
|  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 2 RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.4A.2 FDD - TDD Inter Frequency Relative Accuracy of RSRQ |  |  | Test 1: Noc1: -80dBm/15kHz Noc2: -80dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported Relative RSRQ values: ±3dB for normal conditions and ±4dB for extreme conditions Test 2: Noc1: -104.7dBm/15kHz Noc2: -104.7dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -4dB Reported Relative RSRQ values: ±4dB Test 3: Noc1: -114.5dBm/15kHz Noc2: -114.5dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -4dB Reported Relative RSRQ values: ±4dB |  |  |  |  | Test 1: -1.10dB -1.10dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0.8dB Via mapping Test 3: 0dB 0dB 0dB 0.8dB Via mapping |  |  |  |  |  | Test 1: Noc1: -81.10dBm/15kHz Noc2: -81.10dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_x - 8 to RSRQ_x + 8 (NTC) RSRQ_x - 10 to RSRQ_x + 10 (ETC) Test 2: Noc1: -104.7dBm/15kHz Noc2: -104.7dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -3.2dB RSRQ_x - 10 to RSRQ_x + 10 Test 3: Noc1: -114.5dBm/15kHz Noc2: -114.5dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -3.2dB RSRQ_x - 10 to RSRQ_x + 10 |
|  |  |  | The derivation of the relative RSRQ values takes into account the uncertainty in Cell 1 RSRQ from Noc1 and Ês1 / Noc1 and Cell 2 RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.4A.2_2 FDD - TDD Inter Frequency Relative Accuracy of RSRQ for UE Category 1bis |  |  | Test 1: Noc1: -80dBm/15kHz Noc2: -80dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported Relative RSRQ values: ±4dB for normal conditions and ±5dB for extreme conditions Test 2: Noc1: -104.7dBm/15kHz Noc2: -104.7dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -4dB Reported Relative RSRQ values: ±5dB Test 3: Noc1: -114.5dBm/15kHz Noc2: -114.5dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -4dB Reported Relative RSRQ values: ±5dB |  |  |  |  | Test 1: -1.10dB -1.10dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0.8dB Via mapping Test 3: 0dB 0dB 0dB 0.8dB Via mapping |  |  |  |  |  | Test 1: Noc1: -81.10dBm/15kHz Noc2: -81.10dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_x - 10 to RSRQ_x + 10 (NTC) RSRQ_x - 12 to RSRQ_x + 12 (ETC) Test 2: Noc1: -104.7dBm/15kHz Noc2: -104.7dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -3.2dB RSRQ_x - 12 to RSRQ_x + 12 Test 3: Noc1: -114.5dBm/15kHz Noc2: -114.5dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -3.2dB RSRQ_x - 12 to RSRQ_x + 12 |
|  |  |  | The derivation of the relative RSRQ values takes into account the uncertainty in Cell 1 RSRQ from Noc1 and Ês1 / Noc1 and Cell 2 RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.5.1 FDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation |  |  | Noc1: -119.5dBm or -118.5dBm or -118dBm or -117.5 dBm or -116.5dBm or -116dBm /15kHz depending on operating band Noc2: -116dBm or -115dBm or -114.5dBm or -114 dBm or -113dBm or -112.5dBm /15kHz depending on operating band Ês1 / Noc1: -4dB Ês2 / No2c: -4dB Ês3 / No2c: -4dB Reported RSRQ values: ±3.5dB for normal conditions and ±4dB for extreme conditions |  |  |  |  | 0dB 0dB 0dB +0.3dB 0dB Via mapping Via mapping |  |  |  |  |  | Noc1: -119.5dBm or -118.5dBm or -118dBm or -117.5 dBm or -116.5dBm or -116dBm /15kHz depending on operating band Noc2: -116dBm or -115dBm or -114.5dBm or -114 dBm or -113dBm or -112.5dBm /15kHz depending on operating band Ês1 / Noc1: -4dB Ês2 / No2c: -3.7dB Ês3 / No2c: -4dB Cell 1: RSRQ_00 to RSRQ_15 (NTC) RSRQ_00 to RSRQ_16 (ETC) Cell 2: RSRQ_00 to RSRQ_14 (NTC) RSRQ_00 to RSRQ_15 (ETC) |
|  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 1 RSRQ from Noc1 and Ês1 / Noc1, the uncertainty in Cell 2 RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.5.2 FDD Relative RSRQ Accuracy E-UTRA for Carrier Aggregation |  |  | Noc1: -119.5dBm or -118.5dBm or -118dBm or -117.5 dBm or -116.5dBm or -116dBm /15kHz depending on operating band Noc2: -116dBm or -115dBm or -114.5dBm or -114 dBm or -113dBm or -112.5dBm /15kHz depending on operating band Ês1 / Noc1: -4dB Ês2 / No2c: -4dB Ês3 / No2c: -4dB Reported RSRQ values: ±4dB |  |  |  |  | 0dB 0dB 0dB +0.3dB 0dB Via mapping |  |  |  |  |  | Noc1: -119.5dBm or -118.5dBm or -118dBm or -117.5 dBm or -116.5dBm or -116dBm /15kHz depending on operating band Noc2: -116dBm or -115dBm or -114.5dBm or -114 dBm or -113dBm or -112.5dBm /15kHz depending on operating band Ês1 / Noc1: -4dB Ês2 / No2c: -3.7dB Ês3 / No2c: -4dB RSRQ_x - 12 to RSRQ_x + 9 (NTC and ETC) |
| 9.2.6.1 TDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation |  |  | Same as 9.2.5.1 |  |  |  |  | Same as 9.2.5.1 |  |  |  |  |  | Same as 9.2.5.1 |
| 9.2.6.2 TDD Relative RSRQ Accuracy for E-UTRA Carrier Aggregation |  |  | Same as 9.2.5.2 |  |  |  |  | Same as 9.2.5.2 |  |  |  |  |  | Same as 9.2.5.2 |
| 9.2.7.1 FDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS |  |  | Test 1: Noc: -84.76dBm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -2.0dB Reported RSRQ values: ±2.5dB Test 2: Noc: -103.85dBm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -2.0dB Reported RSRQ values: ±2.5dB Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +5.0dB Ês2 / Noc: -4.0dB Reported RSRQ values: ±3.5dB |  |  |  |  | Test 1: -1.0dB 0dB +0.8dB Via mapping Test 2: 0dB 0dB +0.8dB Via mapping Test 3: 0dB 0dB +0.95dB Via mapping |  |  |  |  |  | Test 1: Noc: -85.76Bm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -1.2dB RSRQ_04 to RSRQ_16 Test 2: Noc: -103.85dBm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -1.2dB RSRQ_04 to RSRQ_16 Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +5.0dB Ês2 / Noc: -3.05dB RSRQ_00 to RSRQ_16 |
|  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 2 RSRQ from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRQ values given above are for normal conditions. For tests 1 and 2 the RSRQ values are 1.5dB wider at each end for extreme conditions, and for test 3 the RSRQ values are 0.5dB wider at each end for extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.8.1 TDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS |  |  | Subset of 9.2.7.1 |  |  |  |  | Same as 9.2.7.1 |  |  |  |  |  | Subset of 9.2.7.1 |
| 9.2.9.1 FDD Absolute RSRQ under Time Domain Measurement Resource Restriction with MBSFN ABS (eICIC) |  |  | Test 1: Noc: -84.76dBm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -2.0dB Reported RSRQ values: ±2.5dB Test 2: Noc: -103.85dBm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -2.0dB Reported RSRQ values: ±2.5dB Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +5.0dB Ês2 / Noc: -4.0dB Reported RSRQ values: ±3.5dB |  |  |  |  | Test 1: -1.0dB 0dB +0.8dB Via mapping Test 2: 0dB 0dB +0.8dB Via mapping Test 3: 0dB 0dB +0.95dB Via mapping |  |  |  |  |  | Test 1: Noc: -85.76Bm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -1.2dB RSRQ_04 to RSRQ_16 Test 2: Noc: -103.85dBm/15kHz Ês1 / Noc: +5.0dB Ês2 / Noc: -1.2dB RSRQ_04 to RSRQ_16 Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +5.0dB Ês2 / Noc: -3.05dB RSRQ_00 to RSRQ_16 |
|  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 2 RSRQ from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRQ values given above are for normal conditions. For tests 1 and 2 the RSRQ values are 1.5dB wider at each end for extreme conditions, and for test 3 the RSRQ values are 0.5dB wider at each end for extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.10.1 TDD Absolute RSRQ under Time Domain Measurement Resource Restriction with MBSFN ABS (eICIC) |  |  | Subset of 9.2.9.1 |  |  |  |  | Same as 9.2.9.1 |  |  |  |  |  | Subset of 9.2.9.1 |
| 9.2.11.1 TDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation for 20MHz |  |  | Same as 9.2.5.1 |  |  |  |  | Same as 9.2.5.1 |  |  |  |  |  | Same as 9.2.5.1 |
| 9.2.11.2 TDD Relative RSRQ Accuracy for E-UTRA Carrier Aggregation for 20MHz |  |  | Same as 9.2.5.2 |  |  |  |  | Same as 9.2.5.2 |  |  |  |  |  | Same as 9.2.5.2 |
| 9.2.12.1 TDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation for 20MHz |  |  | Same as 9.2.5.1 |  |  |  |  | Same as 9.2.5.1 |  |  |  |  |  | Same as 9.2.5.1 |
| 9.2.12.2 TDD Relative RSRQ Accuracy for E-UTRA Carrier Aggregation for 20MHz |  |  | Same as 9.2.5.2 |  |  |  |  | Same as 9.2.5.2 |  |  |  |  |  | Same as 9.2.5.2 |
| 9.2.15.1 FDD RSRQ Accuracy under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) |  |  | Test 1: Noc: -84.76dBm/15kHz Ês1 / Noc: +4.0dB Ês2 / Noc: +2.0dB Ês3 / Noc: -1.5dB Reported RSRQ values: ±2.5dB Test 2: Noc: -103.85dBm/15kHz Ês1 / Noc: +4.0dB Ês2 / Noc: +2.0dB Ês3 / Noc: -1.5dB Reported RSRQ values: ±2.5dB Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +4.0dB Ês2 / Noc: +2.0dB Ês3 / Noc: -4.0dB Reported RSRQ values: ±3.5dB |  |  | Test 1: 0dB 0dB 0dB +1.0dB Via mapping Test 2: 0dB 0dB 0dB +1.0dB Via mapping Test 3: 0dB 0dB 0dB +1.0dB Via mapping |  |  |  |  |  | Test 1: Noc: -84.76Bm/15kHz Ês1 / Noc: +4.0dB Ês2 / Noc: +2.0dB Ês3 / Noc: -0.5dB RSRQ_05 to RSRQ_17 Test 2: Noc: -103.85dBm/15kHz Ês1 / Noc: +4.0dB Ês2 / Noc: +2.0dB Ês3 / Noc: -0.5dB RSRQ_05 to RSRQ_17 Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: +4.0dB Ês2 / Noc: +2.0dB Ês3 / Noc: -3.0dB RSRQ_00 to RSRQ_16 |  |  |
|  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 3 RSRQ from Ês1 / Noc, Ês2 / Noc, and Ês3 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRQ values given above are for normal conditions. For tests 1 and 2 the RSRQ values are 1.5dB wider at each end for extreme conditions, and for test 3 the RSRQ values are 0.5dB wider at each end for extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.16.1 TDD RSRQ Accuracy under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS (feICIC) |  |  | Subset of 9.2.15.1 |  |  | Same as 9.2.15.1 |  |  |  |  |  | Subset of 9.2.15.1 |  |  |
| 9.2.17.1 FDD Intra Frequency Absolute RSRQ Accuracy for 5MHz Bandwidth |  |  | Test 1: Noc: -81.76dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB Reported RSRQ values: ±2.5dB Test 2: Noc: -100.85dBm/15kHz Ês1 / Noc: -2.9dB Ês2 / Noc: -2.9dB Reported RSRQ values: ±3.5dB Test 3: Noc: -109.5dBm /15kHz Ês1 / Noc: -4.0dB Ês2 / Noc: -4.0dB Reported RSRQ values: ±3.5dB |  |  | Test 1: -1.05dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB Via mapping Test 3: 0dB +0.4dB +0.4dB Via mapping |  |  |  |  |  | Test 1: Noc: -82.81Bm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB RSRQ_04 to RSRQ_16 Test 2: Noc: -100.85dBm/15kHz Ês1 / Noc: -2.9dB Ês2 / Noc: -2.9dB RSRQ_00 to RSRP_14 Test 3: Noc: -109.5dBm /15kHz Ês1 / Noc: -3.6dB Ês2 / Noc: -3.6dB RSRQ_00 to RSRQ_14 |  |  |
|  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 2 RSRQ from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRQ values given above are for normal conditions. For test 1 the RSRQ values are 1.5dB wider at each end for extreme conditions, and for tests 2 and 3 the RSRQ values are 0.5dB wider at each end for extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.18.1 FDD - FDD Inter Frequency Absolute RSRQ Accuracy for 5MHz Bandwidth |  |  | Test 1: Noc1: -77dBm/15kHz Noc2: -77dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB for normal conditions and ±4dB for extreme conditions Test 2: Noc1: -101.7dBm/15kHz Noc2: -101.7dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -4dB Reported RSRQ values: ±3.5dB for normal conditions and ±4dB for extreme conditions Test 3: Noc1: -119.5dBm to -116dBm/15kHz depending on operating band Noc2: 113dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -4dB Reported RSRQ values: ±3.5dB for normal conditions and ±4dB for extreme conditions |  |  | Test 1: 0dB -1.1dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0.8dB Via mapping Test 3: 0dB 0.3dB 0dB 0.8dB Via mapping |  |  |  |  |  | Test 1: Noc1: -77dBm/15kHz Noc2: -78.1dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_04 to RSRQ_16 (NTC) RSRQ_01 to RSRQ_19 (ETC) Test 2: Noc1: -101.7dBm/15kHz Noc2: -101.7dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -3.2dB RSRQ_00 to RSRQ_16 (NTC) RSRQ_00 to RSRQ_17 (ETC) Test 3: Noc1: -119.5dBm to -116dBm /15kHz depending on operating band Noc2: -112.7dBm /15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -3.2dB RSRQ_00 to RSRQ_16 (NTC) RSRQ_00 to RSRQ_17 (ETC) |  |  |
|  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 2 RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.18.2 FDD - FDD Inter Frequency Relative RSRQ Accuracy for 5MHz Bandwidth |  |  | Test 1: Noc1: -77dBm/15kHz Noc2: -77dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported Relative RSRQ values: ±3dB for normal conditions and ±4dB for extreme conditions Test 2: Noc1: -101.7dBm/15kHz Noc2: -101.7dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -4dB Reported Relative RSRQ values: ±4dB Test 3: Noc1: -119.5dBm to -116dBm/15kHz depending on operating band Noc2: 113dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -4dB Reported Relative RSRQ values: ±4dB |  |  |  | Test 1: -1.1dB -1.1dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0.8dB Via mapping Test 3: 0dB +0.3dB 0dB 0.8dB Via mapping |  |  |  | Test 1: Noc1: -78.1dBm/15kHz Noc2: -78.1dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_x - 8 to RSRQ_x + 8 (NTC) RSRQ_x - 10 to RSRQ_x + 10 (ETC) Test 2: Noc1: -101.7dBm/15kHz Noc2: -101.7dBm/15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -3.2dB RSRQ_x - 9 to RSRQ_x + 11 Test 3: Noc1: -119.5dBm to -116dBm/15kHz depending on operating band Noc2: -112.7dBm /15kHz Ês1 / Noc1: -4dB Ês2 / No2c: -3.2dB RSRQ_x - 9 to RSRQ_x + 11 |  |  |  |
|  |  |  | The derivation of the relative RSRQ values takes into account the uncertainty in Cell 1 RSRQ from Noc1 and Ês1 / Noc1 and Cell 2 RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.19.1 FDD-FDD Inter Frequency absolute WB-RSRQ accuracy |  |  | FFS |  |  |  | FFS |  |  |  | FFS |  |  |  |
| 9.2.20.1 TDD-TDD Inter Frequency absolute WB-RSRQ accuracy |  |  | FFS |  |  |  | FFS |  |  |  | FFS |  |  |  |
| 9.2.21.1 FDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation for 10MHz+5MHz |  |  | Noc1: -119.5dBm or -118.5dBm or -118dBm or -117.5 dBm or -116.5dBm or -116dBm /15kHz depending on operating band Noc2: -116dBm or -115dBm or -114.5dBm or -114 dBm or -113dBm or -112.5dBm /15kHz depending on operating band Ês1 / Noc1: -4dB Ês2 / No2c: -4dB Ês3 / No2c: -4dB Reported RSRQ values: ±3.5dB for normal conditions and ±4dB for extreme conditions |  |  |  | 0dB 0dB 0dB +0.3dB 0dB Via mapping Via mapping |  |  |  |  | Noc1: -119.5dBm or -118.5dBm or -118dBm or -117.5 dBm or -116.5dBm or -116dBm /15kHz depending on operating band Noc2: -116dBm or -115dBm or -114.5dBm or -114 dBm or -113dBm or -112.5dBm /15kHz depending on operating band Ês1 / Noc1: -4dB Ês2 / No2c: -3.7dB Ês3 / No2c: -4dB Cell 1: RSRQ_00 to RSRQ_15 (NTC) RSRQ_00 to RSRQ_16 (ETC) Cell 2: RSRQ_00 to RSRQ_14 (NTC) RSRQ_00 to RSRQ_15 (ETC) |  |  |
|  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 1 RSRQ from Noc1 and Ês1 / Noc1, the uncertainty in Cell 2 RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.21.2 FDD Relative RSRQ Accuracy for E-UTRA Carrier Aggregation for 10MHz+5MHz |  |  | Noc1: -119.5dBm or -118.5dBm or -118dBm or -117.5 dBm or -116.5dBm or -116dBm /15kHz depending on operating band Noc2: -116dBm or -115dBm or -114.5dBm or -114 dBm or -113dBm or -112.5dBm /15kHz depending on operating band Ês1 / Noc1: -4dB Ês2 / No2c: -4dB Ês3 / No2c: -4dB Reported RSRQ values: ±4dB |  |  |  | 0dB 0dB 0dB +0.3dB 0dB Via mapping |  |  |  |  | Noc1: -119.5dBm or -118.5dBm or -118dBm or -117.5 dBm or -116.5dBm or -116dBm /15kHz depending on operating band Noc2: -116dBm or -115dBm or -114.5dBm or -114 dBm or -113dBm or -112.5dBm /15kHz depending on operating band Ês1 / Noc1: -4dB Ês2 / No2c: -3.7dB Ês3 / No2c: -4dB RSRQ_x - 12 to RSRQ_x + 9 (NTC and ETC) |  |  |
|  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 1 RSRQ from Noc1 and Ês1 / Noc1, the uncertainty in Cell 2 RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.22.1 TDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation for 10MHz+5MHz |  |  | Same as 9.2.21.1 |  |  |  | Same as 9.2.21.1 |  |  |  |  | Same as 9.2.21.1 |  |  |
| 9.2.22.2 TDD Relative RSRQ Accuracy for E-UTRA Carrier Aggregation for 10MHz+5MHz |  |  | Same as 9.2.21.2 |  |  |  | Same as 9.2.21.2 |  |  |  |  | Same as 9.2.21.2 |  |  |
| 9.2.23.1 FDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation for 5MHz+5MHz |  |  | Noc1: -119.5dBm or -118.5dBm or -118dBm or -117.5 dBm or -116.5dBm or -116dBm /15kHz depending on operating band Noc2: -116dBm or -115dBm or -114.5dBm or -114 dBm or -113dBm or -112.5dBm /15kHz depending on operating band Ês1 / Noc1: -4dB Ês2 / Noc2: -4dB Ês3 / Noc2: -4dB Reported RSRQ values: ±3.5dB for normal conditions and ±4dB for extreme conditions |  |  |  | 0dB 0dB 0dB +0.3dB 0dB Via mapping Via mapping |  |  |  |  | Noc1: -119.5dBm or -118.5dBm or -118dBm or -117.5 dBm or -116.5dBm or -116dBm /15kHz depending on operating band Noc2: -116dBm or -115dBm or -114.5dBm or -114 dBm or -113dBm or -112.5dBm /15kHz depending on operating band Ês1 / Noc1: -4dB Ês2 / Noc2: -3.7dB Ês3 / Noc2: -4dB Cell 1: RSRQ_00 to RSRQ_15 (NTC) RSRQ_00 to RSRQ_16 (ETC) Cell 2: RSRQ_00 to RSRQ_14 (NTC) RSRQ_00 to RSRQ_15 (ETC) |  |  |
|  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 1 RSRQ from Noc1 and Ês1 / Noc1, the uncertainty in Cell 2 RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.23.2 FDD Relative RSRQ Accuracy for E-UTRA Carrier Aggregation for 5MHz+5MHz |  |  | Noc1: -119.5dBm or -118.5dBm or -118dBm or -117.5 dBm or -116.5dBm or -116dBm /15kHz depending on operating band Noc2: -116dBm or -115dBm or -114.5dBm or -114 dBm or -113dBm or -112.5dBm /15kHz depending on operating band Ês1 / Noc1: -4dB Ês2 / Noc2: -4dB Ês3 / Noc2: -4dB Reported RSRQ values: ±4dB |  |  |  | 0dB 0dB 0dB +0.3dB 0dB Via mapping |  |  |  |  | Noc1: -119.5dBm or -118.5dBm or -118dBm or -117.5 dBm or -116.5dBm or -116dBm /15kHz depending on operating band Noc2: -116dBm or -115dBm or -114.5dBm or -114 dBm or -113dBm or -112.5dBm /15kHz depending on operating band Ês1 / Noc1: -4dB Ês2 / Noc2: -3.7dB Ês3 / Noc2: -4dB RSRQ_x - 12 to RSRQ_x + 9 (NTC and ETC) |  |  |
| 9.2.24.1 TDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation for 5MHz+5MHz |  |  | Same as 9.2.23.1 |  |  |  | Same as 9.2.23.1 |  |  |  |  | Same as 9.2.23.1 |  |  |
|  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 1 RSRQ from Noc1 and Ês1 / Noc1, the uncertainty in Cell 2 RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.24.2 TDD Relative RSRQ Accuracy for E-UTRA Carrier Aggregation for 5MHz+5MHz |  |  | Same as 9.2.23.2 |  |  |  | Same as 9.2.23.2 |  |  |  |  | Same as 9.2.23.2 |  |  |
| 9.2.25.1 Absolute RSRQ Accuracy for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD |  |  | Same as 9.2.38 Cell 1 and Cell 2, absolute RSRQ part |  |  |  | Same as 9.2.38 Cell 1 and Cell 2 |  |  |  |  | Same as 9.2.38 Cell 1 and Cell 2, absolute RSRQ part |  |  |
| 9.2.25.2 Relative RSRQ Accuracy for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD |  |  | Same as 9.2.38 Cell 1 and Cell 2, absolute RSRQ part |  |  |  | Same as 9.2.38 Cell 1 and Cell 2 |  |  |  |  | Same as 9.2.38 Cell 1 and Cell 2, absolute RSRQ part |  |  |
| 9.2.26.1 Absolute RSRQ Accuracy for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD |  |  | Same as 9.2.39 Cell 1 and Cell 2, absolute RSRQ part |  |  |  | Same as 9.2.39 Cell 1 and Cell 2 |  |  |  |  | Same as 9.2.39 Cell 1 and Cell 2, absolute RSRQ part |  |  |
| 9.2.26.2 Relative RSRQ Accuracy for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD |  |  | Same as 9.2.39 Cell 1 and Cell 2, absolute RSRQ part |  |  |  | Same as 9.2.39 Cell 1 and Cell 2 |  |  |  |  | Same as 9.2.39 Cell 1 and Cell 2, absolute RSRQ part |  |  |
| 9.2.27.1 TDD Absolute RSRQ Accuracy for E-UTRA Carrier Aggregation for 20MHz+10MHz |  |  | Noc1: -119.5dBm or -118.5dBm or -117.5 dBm /15kHz depending on operating band Noc2: -116dBm or -115dBm or -114 dBm /15kHz depending on operating band Ês1 / Noc1: -4dB Ês2 / No2c: -4dB Ês3 / No2c: -4dB Reported RSRQ values: ±3.5dB for normal conditions and ±4dB for extreme conditions |  |  |  | 0dB 0dB 0dB +0.3dB 0dB Via mapping Via mapping |  |  |  |  | Noc1: -119.5dBm or -118.5dBm or -117.5 dBm /15kHz depending on operating band Noc2: -116dBm or -115dBm or -114 dBm /15kHz depending on operating band Ês1 / Noc1: -4dB Ês2 / No2c: -3.7dB Ês3 / No2c: -4dB Cell 1: RSRQ_00 to RSRQ_15 (NTC) RSRQ_00 to RSRQ_16 (ETC) Cell 2: RSRQ_00 to RSRQ_14 (NTC) RSRQ_00 to RSRQ_15 (ETC) |  |  |
|  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 1 RSRQ from Noc1 and Ês1 / Noc1, the uncertainty in Cell 2 RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.27.2 TDD Relative RSRQ Accuracy for E-UTRA Carrier Aggregation for 20MHz+10MHz |  |  | Noc1: -119.5dBm or -118.5dBm or -117.5 dBm /15kHz depending on operating band Noc2: -116dBm or -115dBm or -114 dBm /15kHz depending on operating band Ês1 / Noc1: -4dB Ês2 / No2c: -4dB Ês3 / No2c: -4dB Reported RSRQ values: ±4dB |  |  |  | 0dB 0dB 0dB +0.3dB 0dB Via mapping |  |  |  |  | Noc1: -119.5dBm or -118.5dBm or -117.5 dBm /15kHz depending on operating band Noc2: -116dBm or -115dBm or -114 dBm /15kHz depending on operating band Ês1 / Noc1: -4dB Ês2 / No2c: -3.7dB Ês3 / No2c: -4dB RSRQ_x - 12 to RSRQ_x + 9 (NTC and ETC) |  |  |
|  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 1 RSRQ from Noc1 and Ês1 / Noc1, the uncertainty in Cell 2 RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.28 FDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal |  |  | Test 1: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: -4.0dB Ês2 / Noc: -4.0dB Reported RSRQ values: ±3.5dB |  |  |  | Test 1: 0dB +0.4dB +0.4dB Via mapping |  |  |  |  | Test 1: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: -3.6dB Ês2 / Noc: -3.6dB RSRQ_00 to RSRQ_14 |  |  |
|  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 2 RSRQ from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRQ values given above are for normal conditions. For test 1 the RSRQ values are 0.5dB wider at each end for extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.29 TDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal |  |  | Test 1: Noc: -116dBm to -114dBm /15kHz Ês1 / Noc: -4.0dB Ês2 / Noc: -4.0dB Reported RSRQ values: ±3.5dB |  |  |  | Test 1: 0dB +0.4dB +0.4dB Via mapping |  |  |  |  | Test 1: Noc: -116dBm to -114dBm /15kHz Ês1 / Noc: -3.6dB Ês2 / Noc: -3.6dB RSRQ_00 to RSRQ_14 |  |  |
|  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 2 RSRQ from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRQ values given above are for normal conditions. For test 1 the RSRQ values are 0.5dB wider at each end for extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.38 3DL PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation |  |  | Noc1: -119.5dBm to -116dBm /15kHz depending on operating band Noc2: -116dBm to -114dBm /15kHz depending on operating band Noc3: -116dBm to -114dBm /15kHz depending on operating band Ês1 / Noc1: -6dB Ês2 / Noc2: -6dB Ês3 / Noc3: -6dB Reported absolute RSRQ values: ±3.5dB for normal conditions and ±4dB for extreme conditions Reported relative RSRQ values: (Cell 2 - Cell 1) and (Cell 3 - Cell 1): ±4dB for normal conditions ±4dB for extreme conditions |  |  |  | 0.3dB 0dB 0dB +0.8dB +0.8dB +0.8dB Via mapping Via mapping |  |  |  |  | Noc1: -119.2dBm to -115.7dBm /15kHz depending on operating band Noc2: -116dBm to -114dBm /15kHz depending on operating band Noc3: -116dBm to -114dBm /15kHz depending on operating band Ês1 / Noc1: -5.20dB Ês2 / Noc2: -5.20dB Ês3 / Noc3: -5.20dB Absolute: Cell 1, Cell 2, and Cell 3 RSRQ_0 to RSRQ_14 Relative: (Cell 2 - Cell 1) and (Cell 3 - Cell 1): RSRQ_(x-10) to RSRQ_(x+10) |  |  |
|  |  |  | The derivation of the RSRQ values takes into account the uncertainty from all applicable Noc and Ês / Noc values, the allowed UE reporting accuracy, and the UE mapping function. Where extreme condition UE accuracies differ from normal condition accuracies, reported values are recalculated. |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.30 FDD-FDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal |  |  | Test 1: Noc1: -117.5dBm to -114dBm/15kHz depending on operating band Noc2: -117.5dBm to -114dBm/15kHz depending on operating band Ês1 / Noc1: -6dB Ês2 / No2c: -6dB Reported absolute RSRQ values: ±3.5dB for normal conditions and ±4dB for extreme conditions Reported absolute RSRQ values: ±4dB for both normal conditions and extreme conditions |  |  |  | Test 1: 0dB 0.3dB 0.8dB 0.8dB Via mapping |  |  |  |  | Test 1: Noc1: -117.5dBm to-114 dBm /15kHz depending on operating band Noc2: -117.2dBm -113.7dBm /15kHz depending on operating band Ês1 / Noc1: -5.2dB Ês2 / No2c: -5.2dB RSRQ_00 to RSRQ_14 (NTC) RSRQ_00 to RSRQ_15 (ETC) RSRQ_x - 10 to RSRQ_x + 10 |  |  |
|  |  |  | The derivation of the absolute RSRQ values takes into account the uncertainty in Cell 2 RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The derivation of the relative RSRQ values takes into account the uncertainty in Cell 1 RSRQ from Noc1 and Ês1 / Noc1 and Cell 2 RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.31 TDD-TDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal |  |  | Test 1: Noc1: -117.5dBm to -115.5dBm/15kHz depending on operating band Noc2: -117.5dBm to -115.5dBm/15kHz depending on operating band Ês1 / Noc1: -6dB Ês2 / No2c: -6dB Reported absolute RSRQ values: ±3.5dB for normal conditions and ±4dB for extreme conditions Reported absolute RSRQ values: ±4dB for both normal conditions and extreme conditions |  |  |  | Test 1: 0dB 0.3dB 0.8dB 0.8dB Via mapping |  |  |  |  | Test 1: Noc1: -117.5dBm to-115.5 dBm /15kHz depending on operating band Noc2: -117.2dBm -115.2dBm /15kHz depending on operating band Ês1 / Noc1: -5.2dB Ês2 / No2c: -5.2dB RSRQ_00 to RSRQ_14 (NTC) RSRQ_00 to RSRQ_15 (ETC) RSRQ_x - 10 to RSRQ_x + 10 |  |  |
|  |  |  | The derivation of the absolute RSRQ values takes into account the uncertainty in Cell 2 RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The derivation of the relative RSRQ values takes into account the uncertainty in Cell 1 RSRQ from Noc1 and Ês1 / Noc1 and Cell 2 RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.39 3DL PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation |  |  | Same as 9.2.38 |  |  |  | Same as 9.2.38 |  |  |  |  | Same as 9.2.38 |  |  |
| 9.2.40 3DL FDD RSRQ for E-UTRA Carrier Aggregation |  |  | Same as 9.2.38 |  |  |  | Same as 9.2.38 |  |  |  |  | Same as 9.2.38 |  |  |
| 9.2.41 3DL TDD RSRQ for E-UTRA Carrier Aggregation |  |  | Same as 9.2.38 |  |  |  | Same as 9.2.38 |  |  |  |  | Same as 9.2.38 |  |  |
| 9.2.42.1 FD-FDD Intra Frequency Absolute RSRQ Accuracy for UE category 0 |  |  | Test 1: Noc: -84.76dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB Reported RSRQ values: ±3.5dB Test 2: Noc: -103.85dBm/15kHz Ês1 / Noc: -2.9dB Ês2 / Noc: -2.9dB Reported RSRQ values: ±4.5dB Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: -4.0dB Ês2 / Noc: -4.0dB Reported RSRQ values: ±4.5dB |  |  |  | Test 1: -1.05dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB Via mapping Test 3: 0dB +0.4dB +0.4dB Via mapping |  |  |  |  | Test 1: Noc: -85.81Bm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB RSRQ_02 to RSRQ_18 Test 2: Noc: -103.85dBm/15kHz Ês1 / Noc: -2.9dB Ês2 / Noc: -2.9dB RSRQ_00 to RSRP_16 Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: -3.6dB Ês2 / Noc: -3.6dB RSRQ_00 to RSRQ_16 |  |  |
|  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 2 RSRQ from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRQ values given above are for normal conditions. For test 1 the RSRQ values are 1.5dB wider at each end for extreme conditions, and for tests 2 and 3 the RSRQ values are 0.5dB wider at each end for extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.43.1 HD-FDD Intra Frequency Absolute RSRQ Accuracy for UE category 0 |  |  | Same as 9.2.42.1 |  |  |  | Same as 9.2.42.1 |  |  |  |  | Same as 9.2.42.1 |  |  |
| 9.2.44.1 TDD Intra Frequency Absolute RSRQ Accuracy for UE category 0 |  |  | Test 1: Noc: -84.76dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB Reported RSRQ values: ±3.5dB Test 2: Noc: -103.85dBm/15kHz Ês1 / Noc: -2.9dB Ês2 / Noc: -2.9dB Reported RSRQ values: ±4.5dB Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: -4.0dB Ês2 / Noc: -4.0dB Reported RSRQ values: ±4.5dB |  |  |  | Test 1: -1.05dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB Via mapping Test 3: 0dB +0.4dB +0.4dB Via mapping |  |  |  |  | Test 1: Noc: -85.81Bm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB RSRQ_02 to RSRQ_18 Test 2: Noc: -103.85dBm/15kHz Ês1 / Noc: -2.9dB Ês2 / Noc: -2.9dB RSRQ_00 to RSRP_16 Test 3: Noc: -116dBm to -112.5dBm /15kHz depending on operating band Ês1 / Noc: -3.6dB Ês2 / Noc: -3.6dB RSRQ_00 to RSRQ_16 |  |  |
|  |  |  | The derivation of the RSRQ values takes into account the uncertainty in Cell 2 RSRQ from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRQ values given above are for normal conditions. For test 1 the RSRQ values are 1.5dB wider at each end for extreme conditions, and for test 2 the RSRQ values are 0.5dB wider at each end for extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.45 4 DL CA PCell in FDD FDD-TDD RSRQ for E-UTRAN in Carrier Aggregation |  |  | Noc1: -119.5dBm to -116dBm /15kHz depending on operating band Noc2: -116dBm to -114dBm /15kHz depending on operating band Noc3: -116dBm to -114dBm /15kHz depending on operating band Noc4: -116dBm to -114dBm /15kHz depending on operating band Ês1 / Noc1: -6dB Ês2 / Noc2: -6dB Ês3 / Noc3: -6dB Ês4 / Noc4: -6dB Reported absolute RSRQ values: ±3.5dB for normal conditions and ±4dB for extreme conditions Reported relative RSRQ values: (Cell 2 - Cell 1) , (Cell 3 - Cell 1) and (Cell 4 - Cell 1) : ±4dB for normal conditions ±4dB for extreme conditions |  |  |  | 0.3dB 0dB 0dB 0dB +0.8dB +0.8dB +0.8dB +0.8dB Via mapping Via mapping |  |  |  |  | Noc1: -119.2dBm to -115.7dBm /15kHz depending on operating band Noc2: -116dBm to -114dBm /15kHz depending on operating band Noc3: -116dBm to -114dBm /15kHz depending on operating band Noc4: -116dBm to -114dBm /15kHz depending on operating band Ês1 / Noc1: -5.20dB Ês2 / Noc2: -5.20dB Ês3 / Noc3: -5.20dB Ês3 / Noc3: -5.20dB Absolute: Cell 1, Cell 2, Cell 3 and Cell 4 RSRQ_00 to RSRQ_14 (NC) RSRQ_00 to RSRQ_15 (EC) Relative: (Cell 2 - Cell 1) , (Cell 3 - Cell 1) and (Cell 4 - Cell 1): RSRQ_(x-10) to RSRQ_(x+10) The Test Tolerance values are taken from the analysis for the related 3DL Test case 9.2.38, and no separate analysis is needed |  |  |
|  |  |  | The derivation of the RSRQ values takes into account the uncertainty from all applicable Noc and Ês / Noc values, the allowed UE reporting accuracy, and the UE mapping function. Where extreme condition UE accuracies differ from normal condition accuracies, reported values are recalculated. |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.46 4DL PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation |  |  | Same as 9.2.45 |  |  |  | Same as 9.2.45 |  |  |  |  | Same as 9.2.45 |  |  |
| 9.2.47 5 DL PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation |  |  | Noc1: -119.5dBm to -116dBm /15kHz depending on operating band Noc2: -116dBm to -114dBm /15kHz depending on operating band Noc3: -116dBm to -114dBm /15kHz depending on operating band Noc4: -116dBm to -114dBm /15kHz depending on operating band Noc5: -116dBm to -114dBm /15kHz depending on operating band Ês1 / Noc1: -6dB Ês2 / Noc2: -6dB Ês3 / Noc3: -6dB Ês4 / Noc4: -6dB Ês5 / Noc5: -6dB Reported absolute RSRQ values: ±3.5dB for normal conditions and ±4dB for extreme conditions Reported relative RSRQ values: (Cell 2 - Cell 1) , (Cell 3 - Cell 1) , (Cell 4 - Cell 1) and (Cell 5 - Cell 1) : ±4dB for normal conditions ±4dB for extreme conditions |  |  |  | 0.3dB 0dB 0dB 0dB 0dB +0.8dB +0.8dB +0.8dB +0.8dB +0.8dB Via mapping Via mapping |  |  |  |  | Noc1: -119.2dBm to -115.7dBm /15kHz depending on operating band Noc2: -116dBm to -114dBm /15kHz depending on operating band Noc3: -116dBm to -114dBm /15kHz depending on operating band Noc4: -116dBm to -114dBm /15kHz depending on operating band Noc5: -116dBm to -114dBm /15kHz depending on operating band Ês1 / Noc1: -5.20dB Ês2 / Noc2: -5.20dB Ês3 / Noc3: -5.20dB Ês4 / Noc4: -5.20dB Ês5 / Noc5: -5.20dB Absolute: Cell 1, Cell 2, Cell 3, Cell 4 and Cell 5 RSRQ_00 to RSRQ_14 (NC) RSRQ_00 to RSRQ_15 (EC) Relative: (Cell 2 - Cell 1) , (Cell 3 - Cell 1) , (Cell 4 - Cell 1) and (Cell 5 - Cell 1): RSRQ_(x-10) to RSRQ_(x+10) The Test Tolerance values are taken from the analysis for the related 4DL Test case 9.2.45, and no separate analysis is needed |  |  |
|  |  |  | The derivation of the RSRQ values takes into account the uncertainty from all applicable Noc and Ês / Noc values, the allowed UE reporting accuracy, and the UE mapping function. Where extreme condition UE accuracies differ from normal condition accuracies, reported values are recalculated. |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.48 5 DL PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation |  |  | Same as 9.2.47 |  |  |  | Same as 9.2.47 |  |  |  |  | Same as 9.2.47 |  |  |
| 9.2.51 FDD intra frequency absolute and relative RSRQ accuracies for SCell with frame structure 3 |  |  | Noc1: -119dBm to -116dBm/15kHz depending on operating band Noc2 and Noc3: Noc1 +3.5dB Ês1 / Noc1: -4dB Ês2 / Noc2: -4dB Ês3 / Noc2: -4dB Reported absolute RSRQ values: ±3.5dB for normal conditions and ±4dB for extreme conditions Reported absolute RSRQ values: ±4dB for both normal conditions and extreme conditions |  |  |  | 0dB 0dB 0dB 0dB 0.3dB Via mapping |  |  |  |  | Noc1: -119dBm to -116dBm/15kHz depending on operating band Noc2 and Noc3: Noc1 +3.5dB Ês1 / Noc1: -4dB Ês2 / Noc2: -4dB Ês3 / Noc2: -3.7dB RSRQ_00 to RSRQ_14 (NTC) RSRQ_00 to RSRQ_15 (ETC) RSRQ_x - 12 to RSRQ_x + 9 |  |  |
| 9.2.52 TDD intra frequency absolute and relative RSRQ accuracies for SCell with frame structure 3 |  |  | Same as 9.2.51 |  |  |  | Same as 9.2.51 |  |  |  |  | Same as 9.2.51 |  |  |
| 9.2.55 3DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes |  |  | Same as 9.2.38 |  |  |  | Same as 9.2.38 |  |  |  |  | Same as 9.2.38 |  |  |
| 9.2.56 4 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes |  |  | Same as 9.2.45 |  |  |  | Same as 9.2.45 |  |  |  |  | Same as 9.2.45 |  |  |
| 9.2.57 5 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes |  |  | Same as 9.2.47 |  |  |  | Same as 9.2.47 |  |  |  |  | Same as 9.2.47 |  |  |
| 9.2.58 6 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes |  |  | Noc1: -119.5dBm to -116dBm /15kHz depending on operating band Noc2: -116dBm to -114dBm /15kHz depending on operating band Noc3: -116dBm to -114dBm /15kHz depending on operating band Noc4: -116dBm to -114dBm /15kHz depending on operating band Noc5: -116dBm to -114dBm /15kHz depending on operating band Noc6: -116dBm to -114dBm /15kHz depending on operating band Ês1 / Noc1: -6dB Ês2 / Noc2: -6dB Ês3 / Noc3: -6dB Ês4 / Noc4: -6dB Ês5 / Noc5: -6dB Ês6 / Noc6: -6dB Reported absolute RSRQ values: ±3.5dB for normal conditions and ±4dB for extreme conditions Reported relative RSRQ values: (Cell 2 – Cell 1) , (Cell 3 – Cell 1) , (Cell 4 – Cell 1) and (Cell 5 – Cell 1), (Cell 6 – Cell 1): ±4dB for normal conditions ±4dB for extreme conditions |  |  |  | 0.3dB 0dB 0dB 0dB 0dB 0dB +0.8dB +0.8dB +0.8dB +0.8dB +0.8dB +0.8dB Via mapping Via mapping |  |  |  |  | Noc1: -119.2dBm to -115.7dBm /15kHz depending on operating band Noc2: -116dBm to -114dBm /15kHz depending on operating band Noc3: -116dBm to -114dBm /15kHz depending on operating band Noc4: -116dBm to -114dBm /15kHz depending on operating band Noc5: -116dBm to -114dBm /15kHz depending on operating band Noc6: -116dBm to -114dBm /15kHz depending on operating band Ês1 / Noc1: -5.20dB Ês2 / Noc2: -5.20dB Ês3 / Noc3: -5.20dB Ês4 / Noc4: -5.20dB Ês5 / Noc5: -5.20dB Ês6 / Noc6: -5.20dB Absolute: Cell 1, Cell 2, Cell 3, Cell 4 and Cell 5 RSRQ_00 to RSRQ_14 (NC) RSRQ_00 to RSRQ_15 (EC) Relative: (Cell 2 – Cell 1) , (Cell 3 – Cell 1) , (Cell 4 – Cell 1) and (Cell 5 – Cell 1), (Cell 6 – Cell 1): RSRQ_(x-10) to RSRQ_(x+10) The Test Tolerance values are taken from the analysis for the related 4DL Test case 9.2.45, and no separate analysis is needed |  |  |
|  |  |  | The derivation of the RSRQ values takes into account the uncertainty from all applicable Noc and Ês / Noc values, the allowed UE reporting accuracy, and the UE mapping function. Where extreme condition UE accuracies differ from normal condition accuracies, reported values are recalculated. |  |  |  |  |  |  |  |  |  |  |  |
| 9.2.59 7 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes |  |  | Noc1: -119.5dBm to -116dBm /15kHz depending on operating band Noc2: -116dBm to -114dBm /15kHz depending on operating band Noc3: -116dBm to -114dBm /15kHz depending on operating band Noc4: -116dBm to -114dBm /15kHz depending on operating band Noc5: -116dBm to -114dBm /15kHz depending on operating band Noc6: -116dBm to -114dBm /15kHz depending on operating band Noc7: -116dBm to -114dBm /15kHz depending on operating band Ês1 / Noc1: -6dB Ês2 / Noc2: -6dB Ês3 / Noc3: -6dB Ês4 / Noc4: -6dB Ês5 / Noc5: -6dB Ês6 / Noc6: -6dB Ês7 / Noc7: -6dB Reported absolute RSRQ values: ±3.5dB for normal conditions and ±4dB for extreme conditions Reported relative RSRQ values: (Cell 2 – Cell 1) , (Cell 3 – Cell 1) , (Cell 4 – Cell 1) and (Cell 5 – Cell 1), (Cell 6 – Cell 1), (Cell 7 – Cell 1): ±4dB for normal conditions ±4dB for extreme conditions |  |  |  | 0.3dB 0dB 0dB 0dB 0dB 0dB 0dB +0.8dB +0.8dB +0.8dB +0.8dB +0.8dB +0.8dB +0.8dB Via mapping Via mapping |  |  |  |  | Noc1: -119.2dBm to -115.7dBm /15kHz depending on operating band Noc2: -116dBm to -114dBm /15kHz depending on operating band Noc3: -116dBm to -114dBm /15kHz depending on operating band Noc4: -116dBm to -114dBm /15kHz depending on operating band Noc5: -116dBm to -114dBm /15kHz depending on operating band Noc6: -116dBm to -114dBm /15kHz depending on operating band Noc7: -116dBm to -114dBm /15kHz depending on operating band Ês1 / Noc1: -5.20dB Ês2 / Noc2: -5.20dB Ês3 / Noc3: -5.20dB Ês4 / Noc4: -5.20dB Ês5 / Noc5: -5.20dB Ês6 / Noc6: -5.20dB Ês7 / Noc7: -5.20dB Absolute: Cell 1, Cell 2, Cell 3, Cell 4 and Cell 5 RSRQ_00 to RSRQ_14 (NC) RSRQ_00 to RSRQ_15 (EC) Relative: (Cell 2 – Cell 1) , (Cell 3 – Cell 1) , (Cell 4 – Cell 1) and (Cell 5 – Cell 1), (Cell 6 – Cell 1), (Cell 7 – Cell 1): RSRQ_(x-10) to RSRQ_(x+10) The Test Tolerance values are taken from the analysis for the related 4DL Test case 9.2.45, and no separate analysis is needed |  |  |
|  |  |  | The derivation of the RSRQ values takes into account the uncertainty from all applicable Noc and Ês / Noc values, the allowed UE reporting accuracy, and the UE mapping function. Where extreme condition UE accuracies differ from normal condition accuracies, reported values are recalculated. |  |  |  |  |  |  |  |  |  |  |  |
| 9.3.1 E-UTRAN FDD - UTRA FDD CPICH RSCP absolute accuracy |  |  | Test 1: E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: +4.00dB UTRA Cell 2 Ioc: -60.00dBm/3.84MHz Ior / Ioc: +9.54dB CPICH_Ec/Ior: -10.00dB Reported CPICH_RSCP values: ±8dB Test 2: E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: +4.00dB UTRA Cell 2 Ioc: -94.46dBm or -92.46dBm or -90.96dBm or -91.46dBm or -93.46dBm /3.84MHz depending on operating band Ior / Ioc: -9.54dB CPICH_Ec/Ior: -10.00dB Reported CPICH_RSCP values: ±6dB |  |  |  | Test 1: 0dB 0dB -0.75dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0.7dB 0.35dB 0dB Via mapping |  |  |  |  | Test 1: E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: +4.00dB UTRA Cell 2 Ioc: -60.75dBm/3.84MHz Ior / Ioc: +9.54dB CPICH_Ec/Ior: -10.00dB CPICH_RSCP_46 to CPICH_RSCP_63 Test 2: E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: +4.00dB UTRA Cell 2 Ioc: -93.76dBm or -91.76dBm or -90.26dBm or -90.76dBm or -92.76dBm /3.84MHz depending on operating band Ior / Ioc: -9.19dB CPICH_Ec/Ior: -10.00dB CPICH_RSCP_-04 to CPICH_RSCP_9 CPICH_RSCP_-02 to CPICH_RSCP_11 CPICH_RSCP_-01 to CPICH_RSCP_12 CPICH_RSCP_-03 to CPICH_RSCP_10 depending on operating band |  |  |
|  |  |  | The derivation of the CPICH_RSCP values takes into account the uncertainty in Cell 2 CPICH_RSCP from Ioc, Ior / Ioc and CPICH_Ec/Ior, the allowed UE reporting accuracy, and the UE mapping function. The CPICH_RSCP values given above are for normal conditions. In all cases the CPICH_RSCP values are 3dB wider at each end for extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 9.3.2 E-UTRAN TDD - UTRA FDD CPICH RSCP absolute accuracy |  |  | Same as 9.3.1 |  |  |  | Same as 9.3.1 |  |  |  |  | Same as 9.3.1 |  |  |
| 9.3.3 E UTRAN FDD UTRA FDD CPICH RSCP absolute accuracy for 5MHz bandwidth |  |  | Same as 9.3.1 |  |  |  | Same as 9.3.1 |  |  |  |  | Same as 9.3.1 |  |  |
| 9.4.1 E-UTRAN FDD - UTRA FDD CPICH Ec/No absolute accuracy |  |  | Test 1: E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: +4.00dB UTRA Cell 2 Ioc: -52.22dBm/3.84MHz Ior / Ioc: -1.75dB CPICH_Ec/Ior: -10.00dB Reported CPICH_Ec/Io accuracy values: ±1.5dB for normal conditions and ±3dB for extreme conditions Test 2: E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: +4.00dB UTRA Cell 2 Ioc: -87.27dBm/3.84MHz Ior / Ioc: -4.7dB CPICH_Ec/Ior: -10.00dB Reported CPICH_Ec/Io accuracy values: ±2dB for normal conditions and ±3dB for extreme conditions Test 3: E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: +4.00dB UTRA Cell 2 Ioc: -94.46dBm or -92.46dBm or 90.96dBm or -91.46dBm or -93.46dBm/3.84MHz depending on operating band Ior / Ioc: -9.54dB CPICH_Ec/Ior: -10.00dB Reported CPICH_Ec/Io accuracy values: -4.2 to +3dB for normal conditions and extreme conditions. The additional 1.2 dB relaxation to the lower limit for both normal and extreme condition is to take into account the effect of thermal noise and noise generated in the receiver as specified in A.9.4.1.3 of TS 36.133 [4] |  |  |  | Test 1: 0dB -0dB -0.9dB 0.3dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0.3dB 0dB Via mapping Test 3: 0dB 0dB 0.7dB 0.4dB 0dB Via mapping |  |  |  |  | Test 1: E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: +4.00dB UTRA Cell 2 Ioc: -53.12dBm/3.84MHz Ior / Ioc: -1.45dB CPICH_Ec/Ior: -10.00dB CPICH_Ec/Io_17 to CPICH_Ec/Io_24 for normal conditions. CPICH_Ec/Io_14 to CPICH_Ec/Io_27 for extreme conditions Test 2: E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: +4.00dB UTRA Cell 2 Ioc: -87.27dBm/3.84MHz Ior / Ioc: -4.4dB CPICH_Ec/Ior: -10.00dB CPICH_Ec/Io_13 to CPICH_Ec/Io_22 for normal conditions. CPICH_Ec/Io_11 to CPICH_Ec/Io_24 for extreme conditions Test 3: E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: +4.00dB UTRA Cell 2 Ioc: -93.76dBm or -91.76dBm or 90.26dBm or -90.76dBm or -92.76dBm /3.84MHz depending on operating band Ior / Ioc: -9.14dB CPICH_Ec/Ior: -10.00dB CPICH_Ec/Io_0 to CPICH_Ec/Io_16 for normal and extreme conditions. |  |  |
| 9.4.2 E-UTRAN TDD - UTRA FDD CPICH Ec/No absolute accuracy |  |  | Same as 9.4.1 |  |  |  | Same as 9.4.1 |  |  |  |  | Same as 9.4.1 |  |  |
| 9.4.3 E-UTRAN FDD - UTRA FDD CPICH Ec/No absolute accuracy for 5MHz bandwidth |  |  | Same as 9.4.1 |  |  |  | Same as 9.4.1 |  |  |  |  | Same as 9.4.1 |  |  |
| 9.5.1 E-UTRAN FDD - UTRA TDD P-CCPCH RSCP absolute accuracy |  |  | Test 1: E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: +4.00dB UTRA Cell 2 Ioc: -54.1dBm/1.28MHz Ior / Ioc: 2.0dB PCCPCH_EC/Ior: -3dB DwPCH_EC/Ior: 0dB Reported PCCPCH RSCP accuracy values: ±8dB for normal conditions and ±11dB for extreme conditions Test 2: E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: +4.00dB UTRA Cell 2 Ioc: -75.2dBm/1.28MHz Ior / Ioc: 5.0dB PCCPCH_EC/Ior: -3dB DwPCH_EC/Ior: 0dB Reported PCCPCH RSCP accuracy values: ±8dB for normal conditions and ±11dB for extreme conditions Test 3: E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: +4.00dB UTRA Cell 2 Ioc: -97.0dBm/1.28MHz Ior / Ioc: 0dB PCCPCH_EC/Ior: -3dB DwPCH_EC/Ior: 0dB Reported PCCPCH RSCP accuracy values: ±6dB for normal conditions and ±9dB for extreme conditions |  |  |  | Test 1: 0dB 0dB -0.8dB 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0.8dB 0dB 0dB 0dB Via mapping |  |  |  |  | Test 1: E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: +4.00dB UTRA Cell 2 Ioc: -54.9dBm/1.28MHz Ior / Ioc: 2.0dB PCCPCH_EC/Ior: -3dB DwPCH_EC/Ior: 0dB PCCPCH RSCP 51 to PCCPCH RSCP 68 for normal conditions. PCCPCH RSCP 48 to PCCPCH RSCP 71 for extreme conditions Test 2: E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: +4.00dB UTRA Cell 2 Ioc: -75.2dBm/1.28MHz Ior / Ioc: 5.0dB PCCPCH_EC/Ior: -3dB DwPCH_EC/Ior: 0dB PCCPCH RSCP 34 to PCCPCH RSCP 51 for normal conditions. PCCPCH RSCP 31 to PCCPCH RSCP 54 for extreme conditions Test 3: E-UTRA Cell 1 Noc: -98.00dBm/15kHz Ês / Noc: +4.00dB UTRA Cell 2 Ioc: -96.2dBm/1.28MHz Ior / Ioc: 0dB PCCPCH_EC/Ior: -3dB DwPCH_EC/Ior: 0dB PCCPCH RSCP 10 to PCCPCH RSCP 23 for normal conditions. PCCPCH RSCP 07 to PCCPCH RSCP 26 for extreme conditions |  |  |
| 9.5.2 E-UTRAN TDD - UTRA TDD P-CCPCH RSCP absolute accuracy |  |  | Same as 9.5.1 |  |  |  | Same as 9.5.1 |  |  |  |  | Same as 9.5.1 |  |  |
| 9.6.1 GSM RSSI accuracy for E-UTRAN FDD |  |  | Subtest 1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -38.5dBm GSM Cell 3 Signal level: -38.5dBm Subtest 2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -48.5dBm GSM Cell 3 Signal level: -48.5dBm Subtest 3: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -70.5dBm GSM Cell 3 Signal level: -70.5dBm Subtest 4: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -109.5dBm GSM Cell 3 Signal level: -109.5dBm Subtest 5: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -57.5dBm GSM Cell 3 Signal level: -54.5dBm Subtest 6: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -64.5dBm GSM Cell 3 Signal level: -59.5dBm Subtest 7: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -71.5dBm GSM Cell 3 Signal level: -64.5dBm Subtest 8: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -78.5dBm GSM Cell 3 Signal level: -69.5dBm Subtest 9: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -85.5dBm GSM Cell 3 Signal level: -74.5dBm Subtest 10: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -92.5dBm GSM Cell 3 Signal level: -79.5dBm Subtest 11: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -99.5dBm GSM Cell 3 Signal level: -84.5dBm Subtest 12: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -106.5dBm GSM Cell 3 Signal level: -89.5dBm |  |  | Subtest 1: E-UTRA Cell 1 0 dB 0 dB GSM Cell 2 -0.2 dB GSM Cell 3 0 dB Subtest 2: E-UTRA Cell 1 0 dB 0 dB GSM Cell 2 -0.2 dB GSM Cell 3 -1.5 dB Subtest 3: E-UTRA Cell 1 0 dB 0 dB GSM Cell 2 -0.2 dB GSM Cell 3 0 dB Subtest 4: E-UTRA Cell 1 0 dB 0 dB GSM Cell 2 +0.2 dB GSM Cell 3 0 dB Subtest 5: E-UTRA Cell 1 0 dB 0 dB GSM Cell 2 0.dB GSM Cell 3 0 dB Subtest 6: E-UTRA Cell 1 0 dB 0 dB GSM Cell 2 0dB GSM Cell 3 0 dB Subtest 7: E-UTRA Cell 1 0 dB 0 dB GSM Cell 2 0dB GSM Cell 3 0 dB Subtest 8: E-UTRA Cell 1 0 dB 0 dB GSM Cell 2 0dB GSM Cell 3 0 dB Subtest 9: E-UTRA Cell 1 0 dB 0 dB GSM Cell 2 0dB GSM Cell 3 0 dB Subtest 10: E-UTRA Cell 1 0 dB 0 dB GSM Cell 2 0dB GSM Cell 3 0 dB Subtest 11: E-UTRA Cell 1 0 dB 0 dB GSM Cell 2 0dB GSM Cell 3 0 dB Subtest 12: E-UTRA Cell 1 0 dB 0 dB GSM Cell 2 0dB GSM Cell 3 0 dB |  |  |  |  |  |  |  | Subtest 1: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -38.7dBm GSM Cell 3 Signal level: -38.5dBm Subtest 2: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -48.7dBm GSM Cell 3 Signal level: -50.0dBm Subtest 3: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -70.7dBm GSM Cell 3 Signal level: -70.5dBm Subtest 4: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -109.3dBm GSM Cell 3 Signal level: -109.5dBm Subtest 5: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -57.5dBm GSM Cell 3 Signal level: -54.5dBm Subtest 6: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -64.5dBm GSM Cell 3 Signal level: -59.5dBm Subtest 7: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -71.5dBm GSM Cell 3 Signal level: -64.5dBm Subtest 8: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -78.5dBm GSM Cell 3 Signal level: -69.5dBm Subtest 9: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -85.5dBm GSM Cell 3 Signal level: -74.5dBm Subtest 10: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -92.5dBm GSM Cell 3 Signal level: -79.5dBm Subtest 11: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -99.5dBm GSM Cell 3 Signal level: -84.5dBm Subtest 12: E-UTRA Cell 1 Noc: -98dBm/15kHz Ês / Noc: +4dB GSM Cell 2 Signal level: -106.5dBm GSM Cell 3 Signal level: -89.5dBm |
| 9.6.2 GSM RSSI accuracy for E-UTRAN TDD |  |  | Same as 9.6.1 |  |  |  | Same as 9.6.1 |  |  |  |  | Same as 9.6.1 |  |  |
| 9.9.1.1 FDD Intra Frequency Serving Cell Absolute RSRP Accuracy |  |  | Noc: -122dBm to -118.5dBm /15kHz depending on operating band Ês1 / Noc: -4.0dB Reported RSRP values: ±6dB |  |  |  | 1.6dB 0dB Via mapping |  |  |  |  | Noc: -120.4dBm to -116.9dBm /15kHz depending on operating band Ês1 / Noc: -4.0dB RSRP_9 to RSRP_24 RSRP_10 to RSRP_25 RSRP_10 to RSRP_25 RSRP_11 to RSRP_26 RSRP_12 to RSRP_27 RSRP_12 to RSRP_27 depending on operating band |  |  |
| 9.9.1.1_1 FDD Intra Frequency Serving Cell Absolute RSRP Accuracy (Rel 12 and forward) |  |  | Noc: -122dBm to -118.5dBm /15kHz depending on operating band Ês1 / Noc: -4.0dB Reported RSRP values: ±4.5dB for normal condition ±9dB for extreme condition |  |  |  | 1.6dB 0dB Via mapping |  |  |  |  | Noc: -120.4dBm to -116.9dBm /15kHz depending on operating band Ês1 / Noc: -4.0dB Normal condition: RSRP_10 to RSRP_22 RSRP_11 to RSRP_23 RSRP_11 to RSRP_23 RSRP_12 to RSRP_24 RSRP_12 to RSRP_24 RSRP_13 to RSRP_25 RSRP_14 to RSRP_26 depending on operating band Extreme condition: RSRP_6 to RSRP_27 RSRP_6 to RSRP_27 RSRP_7 to RSRP_28 RSRP_7 to RSRP_28 RSRP_8 to RSRP_29 RSRP_9 to RSRP_30 RSRP_9 to RSRP_30 depending on operating band |  |  |
| 9.9.1.2 FDD Intra Frequency Serving Cell Absolute RSRQ Accuracy |  |  | Noc: -122dBm to -118.5dBm /15kHz depending on operating band Ês1 / Noc: -4.0dB Reported RSRQ values: ±3.5dB for normal conditions and ±4dB for extreme conditions |  |  | 1.6dB 0dB Via mapping |  |  |  |  |  |  |  | Noc: -120.4dBm to -116.9dBm /15kHz depending on operating band Ês1 / Noc: -4.0dB RSRQ_00 to RSRQ_15 (NTC) RSRQ_00 to RSRQ_16 (ETC) |
| 9.9.2.1 TDD Intra Frequency Serving Cell Absolute RSRP Accuracy |  |  | Noc: -122dBm to -120dBm /15kHz depending on operating band Ês1 / Noc: -4.0dB Reported RSRP values: ±6dB |  |  | 1.6dB 0dB Via mapping |  |  |  |  |  |  |  | Noc: -120.4dBm to -118.4dBm /15kHz depending on operating band Ês1 / Noc: -4.0dB RSRP_9 to RSRP_24 RSRP_10 to RSRP_25 RSRP_12 to RSRP_27 depending on operating band |
| 9.9.2.1_1 TDD Intra Frequency Serving Cell Absolute RSRP Accuracy (Rel 12 and forward) |  |  | Noc: -122dBm to -120dBm /15kHz depending on operating band Ês1 / Noc: -4.0dB Reported RSRP values: ±4.5dB for normal condition ±9dB for extreme condition |  |  | 1.6dB 0dB Via mapping |  |  |  |  |  |  |  | Noc: -120.4dBm to -118.4dBm /15kHz depending on operating band Ês1 / Noc: -4.0dB Normal condition: RSRP_10 to RSRP_22 RSRP_11 to RSRP_23 RSRP_12 to RSRP_24 depending on operating band Extreme condition: RSRP_6 to RSRP_27 RSRP_7 to RSRP_28 RSRP_8 to RSRP_29 depending on operating band |
| 9.9.2.2 TDD Intra Frequency Serving Cell Absolute RSRQ Accuracy |  |  | Noc: -122dBm to -120dBm /15kHz depending on operating band Ês1 / Noc: -4.0dB Reported RSRQ values: ±3.5dB for normal conditions and ±4dB for extreme conditions |  |  | 1.6dB 0dB Via mapping |  |  |  |  |  |  |  | Noc: -120.4dBm to -118.4dBm /15kHz depending on operating band Ês1 / Noc: -4.0dB RSRQ_00 to RSRQ_15 (NTC) RSRQ_00 to RSRQ_16 (ETC) |
| 9.11.1 FS3 average RSSI accuracy case (PCell using FDD) |  |  | Noc1: -106 dBm / 15kHz Noc2: -106 dBm / 15kHz outside RMTC Noc2: -83 dBm / 15kHz during RMTC Ês1 / Noc: 2.5dB Ês2 / Noc: 2.5dB outside RMTC Ês2 / Noc: -Infinity during RMTC Reported RSSI values: ±2.5dB for normal condition ±5.5dB for extreme condition |  |  | 0dB 0dB Via mapping |  |  |  |  |  |  |  | Noc1: -106 dBm / 15kHz Noc2: -106 dBm / 15kHz outside RMTC Noc2: -83 dBm / 15kHz during RMTC Ês1 / Noc: 2.5dB Ês2 / Noc: 2.5dB outside RMTC Ês2 / Noc: -Infinity during RMTC Reported RSSI values: RSSI_44 to RSSI_53 for normal condition RSSI_41 to RSSI_56 for extreme condition |
| 9.11.2 FS3 average RSSI accuracy case (PCell using TDD) |  |  | Same as 9.11.1 |  |  | Same as 9.11.1 |  |  |  |  |  |  |  | Same as 9.11.1 |
| 9.12.1 FS3 channel occupancy test (PCell using FDD) |  |  | Noc1: -106 dBm / 15kHz Noc2: -106 dBm / 15kHz outside RMTC Noc2: -96.28 dBm / 15kHz during RMTC and Channel Ocuppancy below threshold Noc2: -91.28 dBm / 15kHz during RMTC and Channel Ocuppancy above threshold Ês1 / Noc: 2.5dB Ês2 / Noc: 2.5dB outside RMTC Ês2 / Noc: -Infinity during RMTC |  |  | 0 dB 0 dB -1.89dB for normal conditions -4.89dB for extreme conditions 1.87dB for normal conditions 4.87dB for extreme conditions 0dB |  |  |  |  |  |  |  | Noc1: -106 dBm / 15kHz Noc2: -106 dBm / 15kHz outside RMTC Noc2: -98.17 dBm / 15kHz for NC and -101.17 dBm / 15 KHz for EC during RMTC and Channel Ocuppancy below threshold Noc2: -89.41 dBm / 15kHz for NC and -86.41 dBm / 15 KHz for EC during RMTC and Channel Ocuppancy above threshold Ês1 / Noc: 2.5dB Ês2 / Noc: 2.5dB outside RMTC Ês2 / Noc: -Infinity during RMTC |
| 9.12.2 FS3 channel occupancy test (PCell using TDD) |  |  | Same as 9.12.1 |  |  | Same as 9.12.1 |  |  |  |  |  |  |  | Same as 9.12.1 |
| 10.2 E-UTRAN FDD - Initiation/Cease of SLSS Transmission with ProSe Direct Communication |  |  | SNRs as specified |  |  | 0.6dB |  |  |  |  |  |  |  | During T1: Formula: SNR + TT During T2: Formula: SNR - TT During T3: Formula: SNR+ TT |
| 10.4 E-UTRAN FDD - Initiation/Cease of SLSS Transmission with ProSe Direct Communication |  |  | During T1: E-UTRA Cell Noc: -98dBm /15kHz Ês / Noc: -infinity SyncRef UE Noc: -98dBm /15kHz Ês / Noc: 13.0dB During T2: E-UTRA Cell Noc: -98dBm /15kHz Ês / Noc: -3.0dB SyncRef UE Noc: -98dBm /15kHz Ês / Noc: 13.0dB |  |  | During T1: EUTRA Cell: 0dB 0dB SyncRef UE: 0dB 0dB During T2: EUTRA Cell 0dB 0.6 dB SyncRef UE : 0dB 0dB |  |  |  |  |  |  |  | During T1: E-UTRA Cell Noc: -98dBm /15kHz Ês / Noc: -infinity SyncRef UE Noc: -98dBm /15kHz Ês / Noc: 13.0dB During T2: E-UTRA Cell Noc: -98dBm /15kHz Ês / Noc: -2.4dB SyncRef UE Noc: -98dBm /15kHz Ês / Noc: 13.0 dB |
| 11.1 V2V UE Transmission Timing Accuracy Test |  |  | Uplink timing: ±12Ts GNSS Signal Power: GPS: -128.5dBm Galileo: -127dBm GLONASS: -131dBm BDS: -133dBm |  |  | ±3Ts 0dB 0dB 0dB 0dB |  |  |  |  |  |  |  | Uplink timing: ±15Ts GNSS Signal Power: GPS: -128.5dBm Galileo: -127dBm GLONASS: -131dBm BDS: -133dBm |
| 11.2 Interruptions due to V2V sidelink communication |  |  | Noc: -98dBm /15kHz Ês / Noc: 16dB GNSS Signal Power: GPS: -128.5dBm Galileo: -127dBm GLONASS: -131dBm BDS: -133dBm |  |  | 0dB 0dB 0dB 0dB 0dB 0dB |  |  |  |  |  |  |  | Noc: -98dBm /15kHz Ês / Noc: 16dB GNSS Signal Power: GPS: -128.5dBm Galileo: -127dBm GLONASS: -131dBm BDS: -133dBm |
| 12.1.1 V2X UE Transmission Timing Accuracy Test for eNB as Timing Reference |  |  | Uplink timing: ±12Ts GNSS Signal Power: GPS: -128.5dBm Galileo: -127dBm GLONASS: -131dBm BDS: -133dBm |  |  | ±3Ts 0dB 0dB 0dB 0dB |  |  |  |  |  |  |  | Uplink timing: ±15Ts GNSS Signal Power: GPS: -128.5dBm Galileo: -127dBm GLONASS: -131dBm BDS: -133dBm |
| 12.1.2 V2X UE Transmission Timing Accuracy Test for SyncRef UE as Timing Reference |  |  | Uplink timing: ±12Ts |  |  | ±3Ts |  |  |  |  |  |  |  | Uplink timing: ±15Ts |
| 12.2.1 Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for eNB as Timing Reference |  |  | During T1: Noc: -110dBm/15kHz Ês / Noc: +4.5dB During T2: Noc: -110dBm/15kHz Ês / Noc: -4.5dB During T3: Noc: -110dBm/15kHz Ês / Noc: +4.5dB |  |  | During T1: 0dB 1.1dB During T2: 0dB -1.1dB During T3: 0dB 1.1dB |  |  |  |  |  |  |  | During T1: Noc: -110dBm/15kHz Ês / Noc: +5.6dB During T2: Noc: -110dBm/15kHz Ês / Noc: -5.6dB During T3: Noc: -110dBm/15kHz Ês / Noc: +5.6dB |
| 12.3.1 V2X Synchronization Reference Selection/Reselection Tests for GNSS configured as the highest priority |  |  | During T1: Noc: -95dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: -infinity During T2: Noc: -95dBm/15kHz Ês1 / Noc: 0dB Ês2 / Noc: -infinity During T3: Noc: -95dBm/15kHz Ês1 / Noc: 0dB Ês2 / Noc: 3dB |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0.3dB 0dB During T3: 0dB 0.3dB 0.5dB |  |  |  |  |  |  |  | During T1: Noc: -95dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: -infinity During T2: Noc: -95dBm/15kHz Ês1 / Noc: 0.3dB Ês2 / Noc: -infinity During T3: Noc: -95dBm/15kHz Ês1 / Noc: 0.3dB Ês2 / Noc: 3.5dB |
| 12.3.2 V2X Synchronization Reference Selection/Reselection Tests for eNB configured as the highest priority |  |  | During T1: Noc: -95dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: -infinity During T2: Noc: -95dBm/15kHz Ês1 / Noc: 0dB Ês2 / Noc: -infinity During T3: Noc: -95dBm/15kHz Ês1 / Noc: 0dB Ês2 / Noc: 3dB |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0.3dB 0dB During T3: 0dB 0.3dB 0.5dB |  |  |  |  |  |  |  | During T1: Noc: -95dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: -infinity During T2: Noc: -95dBm/15kHz Ês1 / Noc: 0.3dB Ês2 / Noc: -infinity During T3: Noc: -95dBm/15kHz Ês1 / Noc: 0.3dB Ês2 / Noc: 3.5dB |
| 12.2.2 Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for SyncRef UE as Timing Reference |  |  | During T1: Noc: -101dBm/15kHz Ês / Noc: +5.5dB During T2: Noc: -101dBm/15kHz Ês / Noc: -3.5dB During T3: Noc: -101dBm/15kHz Ês / Noc: +5.5dB |  |  | During T1: -1.6dB 3.2dB During T2: -1.6dB 0dB During T3: -1.6dB 3.2dB |  |  |  |  |  |  |  | During T1: Noc: -102.6dBm/15kHz Ês / Noc: +8.7dB During T2: Noc: -102.6dBm/15kHz Ês / Noc: -3.5dB During T3: Noc: -102.6dBm/15kHz Ês / Noc: +8.7dB |
| 12.4 Congestion Control Measurement Test for V2X UE |  |  | During T1: Noc: -106dBm/15kHz PUCCH Ês / Noc: +7.19dB During T2: Noc: -106dBm/15kHz PUCCH Ês / Noc: +13.19dB |  |  | During T1: 0dB -2.3dB During T2: 0dB 1.7dB |  |  |  |  |  |  |  | During T1: Noc: -106dBm/15kHz PUCCH Ês / Noc: +4.89dB During T2: Noc: -106dBm/15kHz PUCCH Ês / Noc: +14.89dB |
| 12.5 Interruptions due to V2X Sidelink Communication |  |  | During T1, T2 and T3: Noc: -98dBm/15kHz Ês / Noc: 16dB |  |  | During T1: 0dB 0dB |  |  |  |  |  |  |  | During T1, T2 and T3: Noc: -98dBm/15kHz Ês / Noc: 16dB |
| 12.6.1 V2X UE Autonomous Resource Selection/Reselection Tests for PSSCH-RSRP measurements |  |  | During T1: Noc: -103dBm/15kHz PSSCH Ês / Noc: 2dB During T2: Noc: -113dBm/15kHz PSSCH Ês / Noc: 2dB |  |  | During T1: 0dB 4.1dB During T2: 0dB -4.1dB |  |  |  |  |  |  |  | During T1: Noc: -103dBm/15kHz PSSCH Ês / Noc: 6.1dB During T2: Noc: -113dBm/15kHz PSSCH Ês / Noc: -2.1dB |
| 12.6.2 V2X UE Autonomous Resource Selection/Reselection Tests for S-RSSI measurements |  |  | During T1: Noc: -105dBm/15kHz PSSCH Ês / Noc: 2dB |  |  | During T1: 0dB 0dB |  |  |  |  |  |  |  | During T1: Noc: -105dBm/15kHz PSSCH Ês / Noc: 2dB |
| 13.1.1.1 HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage |  |  | During T1: Noc: -98dBm/15KHz Êsn1 / Noc: 17.00dB Êsn2 / Noc: -infinity During T2: Noc: -98dBm/15KHz Êsn1 / Noc: 13.00dB Êsn2 / Noc: 17.00dB During T3: Noc: -98dBm/15KHz Êsn1 / Noc: 17.00dB Êsn2 / Noc: 13.00dB |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB +0.45dB During T3: 0dB +0.45dB 0dB |  |  |  |  |  |  |  | During T1: Noc: -98dBm/15KHz Êsn1 / Noc: 17.00dB Êsn2 / Noc: -infinity During T2: Noc: -98dBm/15KHz Êsn1 / Noc 13.00dB Êsn2 / Noc: 17.45dB During T3: Noc: -98dBm/15KHz Êsn1 / Noc: 17.45dB Êsn2 / Noc: 13.00dB |
| 13.1.1.1_1 HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage for GSO and NGSO |  |  | Same as 13.1.1.1 |  |  | Same as 13.1.1.1 |  |  |  |  |  |  |  | Same as 13.1.1.1 |
| 13.1.1.2 HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage with serving cell RRM measurement relaxation |  |  | During T1: Noc: -98dBm/15kHz Êsn1 / Noc: 17dB Êsn2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Êsn1 / Noc: 7dB Êsn2 / Noc: 11dB |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB +0.45dB |  |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Êsn1 / Noc: 17dB Êsn2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Êsn1 / Noc: 7dB Êsn2 / Noc: 11.45dB |
| 13.1.1.2_1 HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage with serving cell RRM measurement relaxation for GSO and NGSO |  |  | Same as 13.1.1.2 |  |  | Same as 13.1.1.2 |  |  |  |  |  |  |  | Same as 13.1.1.2 |
| 13.1.1.3 HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage with UE specific DRX |  |  | Same as 13.1.1.1 |  |  | Same as 13.1.1.1 |  |  |  |  |  |  |  | Same as 13.1.1.1 |
| 13.1.1.3_1 FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage with UE specific DRX for GSO and NGSO |  |  | Same as 13.1.1.1 |  |  | Same as 13.1.1.1 |  |  |  |  |  |  |  | Same as 13.1.1.1 |
| 13.1.1.4 HD – FDD Inter frequency case for UE Category NB1 Standalone mode in normal coverage |  |  | During T1: Noc: -98dBm/15KHz Êsn1 / Noc: 17.00dB Êsn2 / Noc: -infinity During T2: Noc: -98dBm/15KHz Êsn1 / Noc: 13.00dB Êsn2 / Noc: 17.00dB During T3: Noc: -98dBm/15KHz Êsn1 / Noc: 17.00dB Êsn2 / Noc: 13.00dB |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB +1.5dB During T3: 0dB +1.5dB 0dB |  |  |  |  |  |  |  | During T1: Noc: -98dBm/15KHz Êsn1 / Noc: 17.00dB Êsn2 / Noc: -infinity During T2: Noc: -98dBm/15KHz Êsn1 / Noc 13.00dB Êsn2 / Noc: 18.5dB During T3: Noc: -98dBm/15KHz Êsn1 / Noc: 18.5dB Êsn2 / Noc: 13.00dB |
| 13.1.1.6 FDD Inter frequency case for UE Category NB1 Standalone mode in enhanced coverage, time-based cell reselection for NGSO |  |  | During T1: Noc: -98dBm/15KHz Êsn1 / Noc: 10.00dB Êsn2 / Noc: -infinity During T2: Noc: -98dBm/15KHz Êsn1 / Noc: -12.00dB Êsn2 / Noc: -infinity During T3: Noc: -98dBm/15KHz Êsn1 / Noc: -12.00dB Êsn2 / Noc: -2.7dB |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB During T3: 0dB 0dB +0.5dB |  |  |  |  |  |  |  | During T1: Noc: -98dBm/15KHz Êsn1 / Noc: 10.00dB Êsn2 / Noc: -infinity During T2: Noc: -98dBm/15KHz Êsn1 / Noc -12.00dB Êsn2 / Noc: -infinity During T3: Noc: -98dBm/15KHz Êsn1 / Noc: -12dB Êsn2 / Noc: -2.2dB |
| 13.3.1.1 HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in Standalone mode under normal coverage |  |  | During T1: Noc: -98dBm/15kHz Êsn1 / Noc: +7.0dB Êsn2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Êsn1 / Noc: -infinity Êsn2 / Noc: +4.0dB During T3: Noc: -98dBm /15kHz Êsn1 / Noc: -infinity Êsn2 / Noc: +4.0dB |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB During T3: 0dB 0dB 0dB |  |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Êsn1 / Noc: +7.0dB Êsn2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Êsn1 / Noc: -infinity Êsn2 / Noc: +4.0dB During T3: Noc: -98dBm /15kHz Êsn1 / Noc: -infinity Êsn2 / Noc: +4.0dB |
| 13.3.1.1_1 HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in Standalone mode under normal coverage for GSO and NGSO |  |  | Same as 13.3.1.1 |  |  | Same as 13.3.1.1 |  |  |  |  |  |  |  | Same as 13.3.1.1 |
| 13.3.1.2 HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in Standalone mode under enhanced coverage |  |  | During T1: Noc: -98dBm/15kHz Êsn1 / Noc: +7.0dB Êsn2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Êsn1 / Noc: -infinity Êsn2 / Noc: -12.6dB During T3: Noc: -98dBm /15kHz Êsn1 / Noc: -infinity Êsn2 / Noc: -12.6dB |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB During T3: 0dB 0dB 0dB |  |  |  |  |  |  |  | During T1: Noc: -98dBm/15kHz Êsn1 / Noc: +7.0dB Êsn2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Êsn1 / Noc: -infinity Êsn2 / Noc: -12.6dB During T3: Noc: -98dBm /15kHz Êsn1 / Noc: -infinity Êsn2 / Noc: -12.6dB |
| 13.3.1.2_1 HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in Standalone mode under enhanced coverage for GSO and NGSO |  |  | Same as 13.3.1.2 |  |  | Same as 13.3.1.2 |  |  |  |  |  |  |  | Same as 13.3.1.2 |
| 13.3.2.1 Contention Based Random Access Test for UE category NB1 UEs Standalone mode in normal coverage |  |  | Absolute uplink power: Normal conditions ±9dB Extreme conditions ±12dB Relative uplink power step: Normal conditions ±2dB Extreme conditions ±4dB Uplink timing Te: ±97Ts rsrp-ThresholdsPrach {40, 55} Ês / Noc: 3dB |  |  | 1.4dB 1.4dB 0.7dB 0.7dB 3Ts {0, -1} for normal {0, -4} for extreme 0.1dB |  |  |  |  |  |  |  | Absolute uplink power: Normal conditions ±10.4dB Extreme conditions ±13.4dB Relative uplink power step: Normal conditions ±2.7dB Extreme conditions ±4.7dB Uplink timing Te: ±100Ts rsrp-ThresholdsPrach {40, 54} for normal conditions and {40, 51} for extreme conditions Ês / Noc: 3.1dB |
| 13.3.2.2 Contention Based Random Access Test for UE category NB1 UEs Standalone mode in Enhanced Coverage |  |  | Absolute uplink power: Normal conditions ±13.3dB Extreme conditions ±16.3dB Relative uplink power step: Normal conditions ±2dB Extreme conditions ±4dB Uplink timing Te: ±97Ts rsrp-ThresholdsPrach {35, 56} |  |  | 1.0dB 1.0dB 0.7dB 0.7dB 3Ts {-1, 1} for normal {-4, 4} for extreme |  |  |  |  |  |  |  | Absolute uplink power: Normal conditions ±14.3dB Extreme conditions ±17.3dB Relative uplink power step: Normal conditions ±2.7dB Extreme conditions ±4.7dB Uplink timing Te: ±100Ts rsrp-ThresholdsPrach {34, 57} for normal conditions and {31, 60} for extreme conditions |
| 13.3.2.3 Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs Standalone mode in Enhanced Coverage |  |  | Same as 13.3.2.2 |  |  | Same as 13.3.2.2 |  |  |  |  |  |  |  | Same as 13.3.2.2 |
| 13.4.1.1 E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under normal coverage for Satellite Access |  |  | Uplink timing: ±( 97*Ts- TGNSS_margin) Max step size: 58.33*Ts Min adjustment rate: 7*Ts Max adjustment rate: 58.33*Ts Ês / Noc: +4.0dB TGNSS_margin is specified in TS 36.133 A.13.4.1.1.2 |  |  | ±3Ts +1.4Ts for GSO +0.7Ts for NGSO -3.6Ts +1.1Ts +0.3dB |  |  |  |  |  |  |  | Uplink timing: ±(100 *Ts- TGNSS_margin) Max step size: 59.73*Ts for GSO 59.03*Ts for NGSO Min adjustment rate: 3.4 *Ts Max adjustment rate: 59.43*Ts Ês / Noc: +4.3dB |
| 13.4.1.2 E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under enhanced coverage for Satellite Access |  |  | Uplink timing: ±(97*Ts- TGNSS_margin) Uplink timing: 0Ts (during ongoing repetition) Ês / Noc: -11.0dB TGNSS_margin is specified in TS 36.133 A.13.4.1.2.2 |  |  | ±3Ts ±3Ts +0.3dB |  |  |  |  |  |  |  | Uplink timing: ±(100 *Ts- TGNSS_margin) Uplink timing: ±3Ts (during ongoing repetition) Ês / Noc: -10.7dB |
| 13.4.1.3 E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under enhanced coverage with segment transmission in NGSO for Satellite Access |  |  | Same as 13.4.1.2 |  |  | Same as 13.4.1.2 |  |  |  |  |  |  |  | Same as 13.4.1.2 |
| 13.4.2.1 HD-FDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Normal Coverage for Satellite Access |  |  | Timing Advance Adjustment: ±13.33Ts |  |  | ±0.5Ts |  |  |  |  |  |  |  | Timing Advance Adjustment: ±13.83Ts |
| 13.4.2.2 HD-FDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Enhance Coverage for Satellite Access |  |  | Same as 13.4.2.1 |  |  | Same as 13.4.2.1 |  |  |  |  |  |  |  | Same as 13.4.2.1 |
| 13.4.3.1 HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 Standalone mode in normal coverage |  |  | During T1: SNR1: -3.1dB During T2: SNR2: -9.1dB During T3: SNR3: -14.1dB During T4: SNR1: -3.1dB |  |  | During T1: +0.3dB During T2: +0.3dB During T3: -0.3dB During T4: +0.3dB |  |  |  |  |  |  |  | During T1: SNR1: -2.8dB During T2: SNR2: -8.8dB During T2: SNR3: -14.4dB During T3: SNR1: -2.8dB |
| 13.4.3.2 HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 Standalone mode in enhanced coverage |  |  | During T1: SNR1: -6.3dB During T2: SNR2: -11.4dB During T3: SNR3: -17.4dB During T4: SNR1: -6.3dB |  |  | During T1: +0.4dB During T2: +0.3dB During T3: -0.3dB During T4: +0.4dB |  |  |  |  |  |  |  | During T1: SNR1: -5.9dB During T2: SNR2: -11.1dB During T2: SNR3: -17.7dB During T3: SNR1: -5.9dB |
| 13.4.3.3 HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 Standalone mode in Enhanced Coverage |  |  | During T1: SNR1: -6.3dB T2 duration: 2.12s During T2: SNR2: -17.4dB During T3: SNR1: -6.3dB |  |  | During T1: +0.4dB T2 duration: -0.16s During T2: -0.3dB During T3: +0.4dB |  |  |  |  |  |  |  | During T1: SNR1: -5.9dB T2 duration: 1.96s During T2: SNR2: -17.7dB During T3: SNR1: -5.9dB |
| 13.4.3.4 HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 Standalone mode in Normal Coverage |  |  | During T1: SNR1: -3.1dB T2 duration: 2.12s During T2: SNR2: -14.1dB During T3: SNR1: -3.1dB |  |  | During T1: +0.3dB T2 duration: -0.16s During T2: -0.3dB During T3: +0.3dB |  |  |  |  |  |  |  | During T1: SNR1: -2.8dB T2 duration: 1.96s During T2: SNR2: -14.4dB During T3: SNR1: -2.8dB |
| 13.4.3.5 HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage |  |  | Same as 13.4.3.4 |  |  | Same as 13.4.3.4 |  |  |  |  |  |  |  | Same as 13.4.3.4 |
| 13.4.3.6 HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 Standalone mode in Enhanced Coverage |  |  | Same as 13.4.3.3 |  |  | Same as 13.4.3.3 |  |  |  |  |  |  |  | Same as 13.4.3.3 |
| 13.4.3.7 HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage |  |  | Same as 13.4.3.1 |  |  | Same as 13.4.3.1 |  |  |  |  |  |  |  | Same as 13.4.3.1 |
| 13.4.3.8 HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Enhanced Coverage |  |  | Same as 13.4.3.2 |  |  | Same as 13.4.3.2 |  |  |  |  |  |  |  | Same as 13.4.3.2 |
| 13.6.2.1 HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under normal coverage |  |  | SNRs as specified |  |  | 0.6dB |  |  |  |  |  |  |  | Formula: SNR + TT |
| 13.6.2.2 HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under enhanced coverage |  |  | SNRs as specified |  |  | 0dB |  |  |  |  |  |  |  | SNRs as specified |
| 13.6.2.3 HD-FDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under normal coverage |  |  | SNRs as specified |  |  | Same as 13.6.2.1 |  |  |  |  |  |  |  | Same as 13.6.2.1 |
| 13.6.2.4 HD-FDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under enhanced coverage |  |  | SNRs as specified |  |  | Same as 13.6.2.2 |  |  |  |  |  |  |  | Same as 13.6.2.2 |
| 13.6.2.5 HD-FDD Downlink channel quality reporting accuracy in RRC_CONNECTED for UE Category NB1 Standalone mode under normal coverage |  |  | SNRs as specified |  |  | 0.6dB |  |  |  |  |  |  |  | Formula: SNR + TT |
| 13.6.2.6 HD-FDD Downlink channel quality reporting accuracy in RRC_CONNECTED for UE Category NB1 Standalone mode under enhanced coverage |  |  | SNRs as specified |  |  | Same as 13.6.2.5 |  |  |  |  |  |  |  | Same as 13.6.2.5 |

<<< END OF CHANGES 1>>>