3GPP TSG-RAN5 Meeting #107	R5-253169

# Malta, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4964 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to MAC SDT Test Case 7.1.1.13.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MT_SDT-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Specific message contents for test case 2-Step RACH/Success needs to be added |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specific message contents for test case 2-Step RACH/Success is added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case is incompleted. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.13.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R5-252202r1 |  |  |  |  |  |  |  |  |

7.1.1.13.6	RA Based MT-SDT / 2-step RACH/ Success

7.1.1.13.6.1	Test Purpose (TP)

(1)

with { UE in NR RRC_INACTIVE state and SDT-CG-Config is not configured and Random Access resources for 2-step RA-SDT are configured }

ensure that {

when { UE receives paging message with mt-SDT indication included and RSRP is above the configured mt-SDT-RSRP-Threshold }

then { UE shall initiate 2-step RA based mt-SDT procedure using legacy Random Access resources with resumeCause set to mt-SDT in RRC connection resumption procedure }

}

(2)

with { UE in NR RRC_INACTIVE state and SDT-CG-Config is not configured and Random Access resources for 2-step RA-SDT are configured }

ensure that {

when { UE initiates 2-step RA based mt-SDT procedure }

then { UE is successfully able to receive subsequent SDT data }

}

(3)

with { UE in NR RRC_INACTIVE state and SDT-CG-Config is not configured and Random Access resources for 2-step RA-SDT are configured }
ensure that {
  when { UE receives paging message with mt-SDT indication included and RSRP is below the configured mt-SDT-RSRP-Threshold }
    then { UE shall not initiate RA based mt-SDT procedure and starts normal RRC Resume procedure resumeCause set to mt-Access in RRC connection resumption procedure }
            }

7.1.1.13.6.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 38.321, clause 5.1.1b, 5.1.1c & 5.27, 3GPP TS 38.331, clause 5.3.2.3 & 5.3.13.1b. Unless otherwise stated these are Rel-18 requirements.

[TS 38.321, clause 5.1.1b]

The MAC entity shall:

1>	if the BWP selected for Random Access procedure is configured with both set(s) of Random Access resources with msg3-Repetitions set to true and set(s) of Random Access resources without msg3-Repetitions set to true and the RSRP of the downlink pathloss reference is less than rsrp-ThresholdMsg3; or

1>	if the BWP selected for Random Access procedure is only configured with the set(s) of Random Access resources with msg3-Repetitions set to true:

2>	assume Msg3 repetition is applicable for the current Random Access procedure.

1>	else:

2>	assume Msg3 repetition is not applicable for the current Random Access procedure.

...

1>	if neither contention-free Random Access Resources nor Random Access Resources for SI request have been provided for this Random Access procedure and one or more of the features including (e)RedCap and/or Slicing and/or SDT and/or MSG3 repetition and/or MSG1 repetition is applicable for this Random Access procedure:

NOTE 2:	The applicability of SDT is determined by MAC entity according to clause 5.27. The applicability of NSAG-ID is determined by upper layers when the Random Access procedure is initiated. The applicability of (e)RedCap is also determined by upper layers when Random Access procedure is initiated and it is applicable to the Random Access procedures initiated by PDCCH orders and any Random Access procedure initiated by the MAC entity.

NOTE 3:	SDT is not applicable for the Random Access procedure initiated by upper layers for MT-SDT.

2>	if none of the sets of Random Access resources are available for any feature applicable to the current Random Access procedure (as specified in clause 5.1.1c):

3>	select the set(s) of Random Access resources that are not associated with any feature indication (as specified in clause 5.1.1c) for this Random Access procedure.

2>	else if there is one set of Random Access resources available which can be used for indicating all features triggering this Random Access procedure:

3>	select this set of Random Access resources for this Random Access procedure.

2>	else if there are more than one set of Random Access resources available which can be used for indicating all features triggering this Random Access procedure and Msg1 repetition is applicable for this Random Access procedure:

3>	select the set of Random Access resources that associated with highest repetition number among the sets of Random Access resources.

2>	else (i.e. there are one or more sets of Random Access resources available that are configured with indication(s) for a subset of all features triggering this Random Access procedure):

3>	select a set of Random Access resources from the available set(s) of Random Access resources based on the priority order indicated by upper layers as specified in clause 5.1.1d for this Random Access Procedure.

...

1>	else:

2>	select the set of Random Access resources that is not associated with any feature indication (as specified in clause 5.1.1c) for the current Random Access procedure.

[TS 38.321, clause 5.1.1c]

The MAC entity shall for each set of configured Random Access resources:

1>	if eRedCap is set to true for a set of Random Access resources:

2>	consider the set of Random Access resources as not available for a Random Access procedure for which eRedCap is not applicable.

1>	if redCap is set to true for a set of Random Access resources:

2>	consider the set of Random Access resources as not available for a Random Access procedure for which RedCap is not applicable.

1>	if smallData is set to true for a set of Random Access resources:

2>	consider the set of Random Access resources as not available for the Random Access procedure which is not triggered for RA-SDT by MO-SDT as specified in TS 38.331 [5].

1>	if NSAG-List is configured for a set of Random Access resources:

2>	consider the set of Random Access resources as not available for the Random Access procedure unless it is triggered for any one of the NSAG-ID(s) in the NSAG-List.

1>	if msg3-Repetitions is set to true for a set of Random Access resources:

2>	consider the set of Random Access resources as not available for the Random Access procedure if Msg3 repetition is not applicable.

1>	if msg1-Repetitions is set to true for a set of Random Access resources:

2>	if Msg1 repetition is not applicable to the current Random Access procedure; or

2>	if the set of Random Access resources is not associated with any of the Msg1 repetition number that is applicable to the current Random Access procedure:

3>	consider the set of Random Access resources as not available for the Random Access procedure.

1>	if a set of Random Access resources is not configured with FeatureCombination:

2>	consider the set of Random Access resources to not associated with any feature.

[TS 38.321, clause 5.27]

The MAC entity may be configured by RRC with SDT and the SDT procedure may be initiated by RRC layer for MO-SDT or MT-SDT. The SDT procedure initiated for MO-SDT can be performed either by Random Access procedure with 2-step RA type or 4-step RA type (i.e., RA-SDT) or by configured grant Type 1 (i.e., CG-SDT). The SDT procedure initiated for MT-SDT cannot be performed by RA-SDT (i.e., RA-SDT is not applicable as specified in clause 5.1.1b), but can be performed either by Random Access procedure (i.e., with 2-step RA type or 4-step RA type) or by configured grant Type 1 (i.e., CG-SDT).

RRC configures the following parameters for SDT procedure:

-	sdt-DataVolumeThreshold: data volume threshold for the UE to determine whether to perform SDT procedure initiated for MO-SDT;

-	sdt-RSRP-Threshold: RSRP threshold for UE to determine whether to perform SDT procedure initiated for MO-SDT;

-	mt-SDT-RSRP-Threshold: RSRP threshold for UE to determine whether to perform SDT procedure initiated for MT-SDT;

-	cg-SDT-RSRP-ThresholdSSB: an RSRP threshold configured for SSB selection for CG-SDT;

-	cg-MT-SDT-MaxDurationToNextCG-Occasion: time threshold which is used by the UE to determine whether to perform CG-SDT for MT-SDT;

-	cg-SDT-MaxDurationToNextCG-Occasion: time threshold configured per logical channel which is used by the UE to determine whether to perform CG-SDT for MO-SDT;

-	sdt-BeamFailureRecoveryProhibitTimer: prohibit timer to avoid frequent triggering of Random Access procedure due to beam failure recovery during RA-SDT procedure or during MT-SDT procedure initiated by Random Access procedure.

The following UE variable is used for the SDT procedure:

-	MAX_DURATION_TO_NEXT_CG_OCCASION;

-	RSRP_THRESHOLD.

The MAC entity shall, if initiated by the upper layers for SDT procedure:

1>	if SDT procedure is initiated for MO-SDT as specified in TS 38.331 [5]:

2>	set the MAX_DURATION_TO_NEXT_CG_OCCASION to the shortest value of cg-SDT-MaxDurationToNextCG-Occasion, if configured, among all the logical channels configured with this parameter by upper layer and having data for transmission;

2>	set the RSRP_THRESHOLD to the value of sdt-RSRP-Threshold, if configured.

1>	else if SDT procedure is initiated for MT-SDT as specified in TS 38.331 [5]:

2>	set the MAX_DURATION_TO_NEXT_CG_OCCASION to the value of cg-MT-SDT-MaxDurationToNextCG-Occasion, if configured;

2>	if mt-SDT-RSRP-Threshold is configured:

3>	set the RSRP_THRESHOLD to the value of mt-SDT-RSRP-Threshold.

2>	else if sdt-RSRP-Threshold is configured:

3>	set the RSRP_THRESHOLD to the value of sdt-RSRP-Threshold.

1>	if the SDT procedure is initiated for MO-SDT as specified in TS 38.331 [5], and the data volume of the pending UL data across all RBs configured for SDT is less than or equal to sdt-DataVolumeThreshold, or if the SDT procedure is initiated for MT-SDT as specified in TS 38.331 [5]; and

NOTE 1:	For SDT procedure, the MAC entity also considers the suspended RBs configured with SDT for data volume calculation. It is up to the UE's implementation how the UE calculates the data volume for the suspended RBs. Size of the CCCH message is not considered for data volume calculation

1>	if the RSRP of the downlink pathloss reference is higher than RSRP_THRESHOLD or if RSRP_THRESHOLD is not set:

2>	if the Serving Cell is configured with supplementary uplink as specified in TS 38.331 [5]; and

2>	if the RSRP of the downlink pathloss reference is less than rsrp-ThresholdSSB-SUL:

3>	select the SUL carrier.

2>	else:

3>	select the NUL carrier.

2>	if CG-SDT is configured on the selected UL carrier, and TA for CG-SDT is valid according to clause 5.27.2 in the first available CG occasion for initial CG-SDT transmission with CCCH message according to clause 5.8.2; and

2>	if the SDT procedure is initiated for MO-SDT as specified in TS 38.331 [5], and, for each RB having data available for transmission, configuredGrantType1Allowed, if configured for CG-SDT, is configured with value true for the corresponding logical channel, or if the SDT procedure is initiated for MT-SDT as specified in TS 38.331 [5]; and

2>	if at least one SSB configured for CG-SDT with SS-RSRP above cg-SDT-RSRP-ThresholdSSB is available, and if either the time gap between the initiation of the SDT procedure and first available CG occasion for initial CG-SDT transmission with CCCH message according to clause 5.8.2 is less than MAX_DURATION_TO_NEXT_CG_OCCASION, or if the MAX_DURATION_TO_NEXT_CG_OCCASION is not set:

3>	indicate to the upper layers that the conditions for initiating SDT procedure are fulfilled;

3>	perform CG-SDT procedure on the selected UL carrier according to clause 5.8.2.

2>	else if a set of Random Access resources for RA-SDT is configured and can be selected according to clause 5.1.1b on the selected UL carrier on the BWP configured by initialUplinkBWP-RedCap, if configured for an (e)RedCap UE; otherwise, on the BWP configured by initialUplinkBWP; or

2>	if the SDT procedure is initiated for MT-SDT as specified in TS 38.331 [5]:

3>	if cg-SDT-TimeAlignmentTimer is running, consider cg-SDT-TimeAlignmentTimer as expired and perform the corresponding actions in clause 5.2;

3>	indicate to the upper layers that the conditions for initiating SDT procedure are fulfilled.

2>	else:

3>	indicate to the upper layers that the conditions for initiating SDT procedure are not fulfilled.

1>	else:

2>	indicate to the upper layers that the conditions for initiating SDT procedure are not fulfilled.

If Random Access procedure is selected above for SDT procedure initiated for MO-SDT or MT-SDT and after the Random Access procedure is successfully completed (see clause 5.1.6), the UE monitors PDCCH addressed to C-RNTI received in random access response until the SDT procedure is terminated. If CG-SDT is selected above and after the initial transmission for CG-SDT is performed, the UE monitors PDCCH addressed to C-RNTI as stored in UE Inactive AS context as specified in TS 38.331 [5] and CS-RNTI until the SDT procedure is terminated.

The MAC entity shall:

1>	if sdt-BeamFailureRecoveryProhibitTimer is configured and not running; and

1>	if RA-SDT procedure for MO-SDT or MT-SDT procedure initiated by Random Access procedure is ongoing:

2>	if SS-RSRP of the SSB selected in the last successfully completed Random Access procedure during ongoing RA-SDT procedure or ongoing MT-SDT procedure initiated by Random Access procedure is less than rsrp-ThresholdSSB:

3>	start the sdt-BeamFailureRecoveryProhibitTimer;

3>	initiate a Random Access procedure (see clause 5.1).

NOTE 1a:	It is up to UE implementation when to measure SSBs during ongoing RA-SDT procedure or ongoing MT-SDT procedure initiated by Random Access procedure. Based on implementation the UE need not initiate the Random Access procedure if there is no SSB with SS-RSRP above rsrp-ThresholdSSB. The UE uses rsrp-ThresholdSSB in Random Access configuration selected by the UE when RA-SDT or MT-SDT procedure was initiated.

NOTE 2:	When the UE determines if there is an SSB with SS-RSRP above cg-SDT-RSRP-ThresholdSSB or less than rsrp-ThresholdSSB, the UE uses the latest unfiltered L1-RSRP measurement.

[TS 38.331, clause 5.3.2.3]

1>	if in RRC_INACTIVE, for each of the PagingRecord, if any, included in the Paging message, or

1>	if in RRC_INACTIVE, for the PagingRecord, if any, included in the UuMessageTransferSidelink message received from the connected L2 U2N Relay UE:

2>	if the ue-Identity included in the PagingRecord matches the UE's stored fullI-RNTI:

...

3>	else if mt-SDT indication was included in the Paging message and if the conditions for initiating SDT for a resume procedure initiated in response to RAN paging according to 5.3.13.1b are fulfilled:

4>	if pagingGroupList was not included in the Paging message; or

4>	if pagingGroupList was included in the Paging message but the UE has not joined any MBS session(s) indicated by the TMGI(s) included in the pagingGroupList; or

4>	if pagingGroupList was included in the Paging message, all the MBS session(s) indicated by the TMGI(s) included in the pagingGroupList that the UE has joined are configured to be received in RRC_INACTIVE, and inactiveReceptionAllowed was included for all these MBS session(s):

5>	initiate the RRC connection resumption procedure according to 5.3.13 with resumeCause set to mt-SDT;

NOTE 1a:	If a UE receives a Paging message including mt-SDT indication and inactiveReceptionAllowed indications for all the multicast session(s) the UE has joined and the UE initiates RRC connection resume, the UE starts monitoring the corresponding G-RNTI(s), if configured, and if multicast MCCH is present, the UE starts monitoring the Multicast MCCH-RNTI and acquires the MBSMulticastConfiguration message on multicast MCCH.

4>	else:

5>	initiate the RRC connection resumption procedure according to 5.3.13 with resumeCause set to mt-Access;

NOTE 2:	If both conditions for initiating MT-SDT and MO-SDT according to 5.3.13.1b are fulfilled, UE may initiate RRC connection resumption procedure for MT-SDT or MO-SDT based on implementation.

[TS 38.331, clause 5.3.13.1b]

When requesting lower layers to check the conditions for initiating SDT, RRC indicates to lower layers whether the resume procedure is initiated for mobile originated or mobile terminated case.

A UE in RRC_INACTIVE initiates the resume procedure for SDT when all of the following conditions are fulfilled:

1>	for the resume procedure initiated by the upper layers (i.e. mobile originated case):

2>	SIB1 includes sdt-ConfigCommon; and

2>	sdt-Config is configured; and

2>	all the pending data in UL is mapped to the radio bearers configured for SDT; and

2>	for an (e)RedCap UE when RedCap-specific initial downlink BWP includes no CD-SSB, ncd-SSB-RedCapInitialBWP-SDT is configured; and

2>	lower layers indicate that conditions for initiating MO-SDT as specified in TS 38.321 [3] are fulfilled.

1>	for the resume procedure initiated in response to RAN paging (i.e. mobile terminated case):

2>	lower layers indicate that conditions for initiating MT-SDT as specified in TS 38.321 [3] are fulfilled.

NOTE:	How the UE determines that all pending data in UL is mapped to radio bearers configured for SDT is left to UE implementation.

7.1.1.13.6.3	Test description

7.1.1.13.6.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1.

UE:

-	None.

Preamble:

-	The UE is in state 3N-A and Test Mode Activated according to TS 38.508-1 [4] Table 4.4A.2-3 with UE test loop mode A is established except that set to return no data in uplink.

7.1.1.13.6.3.2	Test procedure sequence

Table 7.1.1.13.6.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a RRCRelease message including sdt-Config-r17 in suspendConfig. | <-- | NR RRC: RRCRelease | - | - |
| 2 | The SS transmits a Paging message including mt-SDT indication. | <-- | NR RRC: Paging | - | - |
| 3 | Check: Does the UE transmit MSGA using a preamble outside of the SmallData FeatureCombinationPreambles, with the associated PUSCH resource containing a RRCResumeRequest message with resumeCause set to mt-SDT? | --> | MAC PDU ( NR RRC: RRCResumeRequest) | 1 | P |
| 4 | The SS transmits a MSGB including a successRAR MAC subPDU containing matching Contention Resolution Identity, C-RNTI and Timing Advance Command. | <-- | MAC PDU(successRAR) | - | - |
| 5 | SS transmits a RLC PDU in a MAC PDU on the DRB configured with SDT. | <-- | MAC PDU | - | - |
| 6 | Check: Does the UE transmit an HARQ ACK on PUCCH? | --> | HARQ ACK | 2 | P |
| 7 | The SS transmits an RRCResume message to bring UE to RRC_CONNECTED State. | <-- | NR RRC: RRCResume | - | - |
| 8 | The UE transmits an RRCResumeComplete message. | --> | NR RRC: RRCResumeComplete | - | - |
| 9 | The SS changes the parameter ‘mt-SDT-RSRP-Threshold-r18’ in SIB1 of NR Cell 1 to 76 and starts broadcasting updated SIB1. Note: This value should result in meeting condition ‘RSRP is below the configured mt-SDT-RSRP-Threshold’ | - | - | - | - |
| 10 | The SS transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification. | <-- | PDCCH (DCI 1_0): Short Message | - | - |
| 11 | Wait for 2.1* modification period second for the UE to receive new system information. (Note 1) | - | - | - | - |
| 12 | The SS transmits a RRCRelease message including sdt-Config-r17 in suspendConfig. | <-- | NR RRC: RRCRelease | - | - |
| 13 | The SS transmits a Paging message including mt-SDT indication. | <-- | NR RRC: Paging | - | - |
| 14 | Check: Does the UE transmit MSGA using a preamble outside of the SmallData FeatureCombinationPreambles, with the associated PUSCH resource containing a RRCResumeRequest message with resumeCause set to mt-Access? | --> | MAC PDU ( NR RRC: RRCResumeRequest) | 3 | P |
| 15 | The SS transmits a MSGB including a successRAR MAC subPDU containing matching Contention Resolution Identity, C-RNTI and Timing Advance Command. | <-- | MAC PDU(successRAR) | - | - |
| 16 | The SS transmits a RRCResume message. | <-- | NR RRC: RRCResume | - | - |
| 17 | The UE transmits a RRCResumeComplete message. | --> | NR RRC: RRCResumeComplete | - | - |
| 18 | The SS transmits a RRCRelease message | <-- | NR RRC: RRCRelease | - | - |
| Note 1: The modification period, expressed in number of radio frames = modificationPeriodCoeff * defaultPagingCycle. |  |  |  |  |  |

7.1.1.13.6.3.3	Specific message contents

Table 7.1.1.13.6.3.3-1: SIB1 (preamble and step 9, Table 7.1.1.13.6.3.2-1)

| Derivation path: TS 38.508-1 [4] Table 4.6.1-28 with condition MT-SDT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| servingCellConfigCommon | ServingCellConfigCommon | Table 7.1.1.13.6.3.3-2 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| mt-SDT-ConfigCommonSIB-r18 SEQUENCE { |  |  |  |
| mt-SDT-RSRP-Threshold-r18 | 66 (-90dBm) |  | Preamble |
|  | 76 (-80dBm) |  | Step 9 |
| sdt-LogicalChannelSR-DelayTimer-r18 | sf512 |  |  |
| t319a-r18 | Not Present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.13.6.3.3-2: ServingCellConfigCommon (7.1.1.13.6.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-168 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommon ::= SEQUENCE { |  |  |  |
| downlinkConfigCommon | DownlinkConfigCommon | Table 7.1.1.13.6.3.3-3 |  |
| uplinkConfigCommon | UplinkConfigCommon | Table 7.1.1.13.6.3.3-5 |  |
| } |  |  |  |

Table 7.1.1.13.6.3.3-3: DownlinkConfigCommon (Table 7.1.1.13.6.3.3-2)

| Derivation Path: TS 38.508-1 [4] table 4.6.3-52 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DownlinkConfigCommon ::= SEQUENCE { |  |  |  |
| initialDownlinkBWP | BWP-DownlinkCommon | Table 7.1.1.13.6.3.3-4 |  |
| } |  |  |  |

Table 7.1.1.13.6.3.3-4: BWP-DownlinkCommon(Table 7.1.1.13.6.3.3-3)

| Derivation Path: TS 38.508-1 [4] table 4.6.3-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-DownlinkCommon ::= SEQUENCE { |  |  |  |
| pdcch-ConfigCommon CHOICE { |  |  |  |
| setup | PDCCH-ConfigCommon with condition SDT |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.13.6.3.3-5: UplinkConfigCommon (7.1.1.13.6.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-201 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UplinkConfigCommon ::= SEQUENCE { |  |  |  |
| initialUplinkBWP | BWP-UplinkCommon | Table 7.1.1.13.6.3.3-6 |  |
| } |  |  |  |

Table 7.1.1.13.6.3.3-6: BWP-UplinkCommon (Table 7.1.1.13.6.3.3-5)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkCommon ::= SEQUENCE { |  |  |  |
| msgA-ConfigCommon-r16 CHOICE { |  |  |  |
| setup | MsgA-ConfigCommon-r16 | TS 38.508-1 [4] Table 4.6.3-81A |  |
| } |  |  |  |
| AdditionalRACH-ConfigList-r17 SEQUENCE { |  |  |  |
| AdditionalRACH-Config-r17 SEQUENCE { |  |  |  |
| rach-ConfigCommon-r17 | Not present |  |  |
| msgA-ConfigCommon-r17 | MsgA-ConfigCommon-r16 | Table 7.1.1.13.6.3.3-7 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.13.6.3.3-7: MsgA-ConfigCommon-r16 (Table 7.1.1.13.6.3.3-6)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-81A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MsgA-ConfigCommon-r16 :: = SEQUENCE { |  |  |  |
| rach-ConfigCommonTwoStepRA-r16 | RACH-ConfigCommonTwoStepRA-r16 | Table 7.1.1.13.6.3.3-8 |  |
| } |  |  |  |

Table 7.1.1.13.6.3.3-8: RACH-ConfigCommonTwoStepRA-r16 (Table 7.1.1.13.6.3.3-7)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-128A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommonTwoStepRA-r16 ::= SEQUENCE { |  |  |  |
| featureCombinationPreamblesList-r17 SEQUENCE { |  |  |  |
| FeatureCombinationPreambles-r17 | FeatureCombinationPreambles | Table 7.1.1.13.6.3.3-9 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.13.6.3.3-9: FeatureCombinationPreambles (Table 7.1.1.13.6.3.3-8)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-56E |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FeatureCombinationPreambles-r17 ::= SEQUENCE { |  |  |  |
| featureCombination-r17 ::= SEQUENCE { |  |  |  |
| smallData-r17 | true |  |  |
| } |  |  |  |
| startPreambleForThisPartition-r17 | 8 | Randomly selected |  |
| numberOfPreamblesPerSSB-ForThisPartition-r17 | 12 |  |  |
| ssb-SharedRO-MaskIndex-r17 | Not present |  |  |
| groupBconfigured-r17 | Not present |  |  |
| separateMsgA-PUSCH-Config-r17 | MsgA-PUSCH-Config | Table 7.1.1.13.6.3.3-10 |  |
| msgA-RSRP-Threshold-r17 | 57 | -100 dBm |  |
| rsrp-ThresholdSSB-r17 | RSRP-Range | TS 38.508-1 [4] table 4.6.3-152 |  |
| deltaPreamble-r17 | Not present |  |  |
| } |  |  |  |

Table 7.1.1.13.6.3.3-10: MsgA-PUSCH-Config (Table 7.1.1.13.6.3.3-9)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-81B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MsgA-PUSCH-Config-r16 ::= SEQUENCE { |  |  |  |
| msgA-PUSCH-ResourceGroupA-r16 SEQUENCE { |  |  |  |
| msgA-MCS-r16 | 1 |  |  |
| nrofPRBs-PerMsgA-PO-r16 | 15 |  |  |
| } |  |  |  |

Table 7.1.1.13.6.3.3-11: RRCRelease (Steps 1, 12 Table 7.1.1.13.6.3.2-1)

| Derivation Path: TS 38.508-1 [4] table 4.6.1-16 with condition NR_RRC_INACTIVE and MT_SDT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCRelease ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcRelease SEQUENCE { |  |  |  |
| suspendConfig SEQUENCE { |  |  |  |
| sdt-Config-r17 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| sdt-DRB-List-r17 SEQUENCE (SIZE (0..maxDRB)) OF DRB-Identity { | 1 entry |  |  |
| DRB-Identity[1] | DRB-Identity using condition DRBj | Entry 1 j is the ID of the DRB established during the preamble which is allocated according to internal TTCN mapping |  |
| } |  |  |  |
| sdt-SRB2-Indication-r17 | Not present |  |  |
| sdt-MAC-PHY-CG-Config-r17 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.13.6.3.3-12: Paging (Steps 2, 13 Table 7.1.1.13.6.3.2-1)

| Derivation Path: TS 38.508-1 [4] table 4.6.1-9 with condition NR_RRC_RESUME and MT_SDT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Paging ::= SEQUENCE { |  |  |  |
| pagingRecordList SEQUENCE (SIZE(1..maxNrofPageRec)) OF PagingRecord { |  | 1 entry |  |  |
| PagingRecord[1] SEQUENCE { |  |  | entry 1 |  |
| ue-Identity CHOICE { |  |  |  |  |
| fullI-RNTI |  | I-RNTI-Value |  |  |
| } |  |  |  |  |
| accessType |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  | 1 entry |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| PagingRecordList-v1800 SEQUENCE (SIZE(1..maxNrofPageRec)) OF PagingRecord-v1800 { | 1 entry |  |  |
| PagingRecord-v1800[1] SEQUENCE { |  | entry 1 |  |
| mt-SDT | true |  |  |
| } |  |  |  |
| } |  |  |  |
| pagingGroupList-v1800 | Not present |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.13.6.3.3-13: RRCResumeRequest (Steps 3, 14 Table 7.1.1.13.6.3.2-1)

| Derivation Path: TS 38.508-1 [4] table 4.6.1-19 with condition MT_SDT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCResumeRequest ::= SEQUENCE { |  |  |  |
| rrcResumeRequest SEQUENCE { |  |  |  |  |  |
| resumeCause | ResumeCause | Table 7.1.1.13.6.3.3-14 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.13.6.3.3-14: ResumeCause (Step 3, Step 14, Table 7.1.1.13.6.3.3-13)

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ResumeCause | mt-SDT-v1810 |  | Step 3 |
|  | mt-Access |  | Step 14 |