3GPP TSG-RAN WG5 Meeting #107	R5-253166

St Julian’s, Malta, 19th-23nd May, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-2 | CR | 0194 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new SLPP acknowledgment procedure test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Support of sidelink positioning in NR systems is one of the objectibve of the work item Expanded and improved NR positioning. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added new test case 9.5.2.4 SLPP Acknowledgment between LMF and UE - Added new test case 9.5.2.5 SLPP Acknowledgment between UEs (Target UE is in coverage) - Added new test case 9.5.2.6 SLPP Acknowledgment between UEs (Target UE is out of coverage) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The release 18 positioning work item will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.5.2.4(new), 9.5.2.5(new), 9.5.2.6(new), |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 37.571-3 CR 0175 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

9.5.2.4	SLPP Acknowledgment between LMF and UE

9.5.2.4.1	Test Purpose (TP)

(1)

with { a NAS signalling connection existing }

ensure that {
  when {	UE receives an SLPP message carrying an acknowledgement request indicator }

then { UE returns an SLPP acknowledgement response }

}

9.5.2.4.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.355 clauses 4.3.3. Unless otherwise stated these are Rel-18 requirements.

[TS 38.355, clause 4.3.3]

Each SLPP message may carry an acknowledgement request and/or an acknowledgement indicator. An SLPP message including an acknowledgement request (i.e., that include the field ackRequested set to TRUE) shall also include a sequence number. Upon reception of an SLPP message which includes the field ackRequested set to TRUE, a receiver returns an SLPP message with an acknowledgement response (i.e., that includes the field ackIndicator set to the same sequence number of the message being acknowledged). An acknowledgement response may contain no SLPP message body (in which case only the sequence number being acknowledged is significant); alternatively, the acknowledgement may be sent in an SLPP message along with an SLPP message body. An acknowledgement is returned for each received SLPP message that requested an acknowledgement including any duplicate(s). Once a sender receives an acknowledgement for an SLPP message, and provided any included sequence number is matching, it is permitted to send the next SLPP message. No message reordering is needed at the receiver since this stop-and-wait method of sending ensures that messages normally arrive in the correct order.

When an SLPP message is transported via a NAS SL-MO-LR request, the message does not request an acknowledgement.

9.5.2.4.3	Test description

9.5.2.4.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1.

-	System information combination NR-1 as defined in TS 38.508-1 [30] clause 4.4.3.1.3 is used in NR cell.

UE:

- None.

Preamble:

-	The UE is in state 3N-A as defined in TS 38.508-1 [30], subclause 4.4A on NR Cell 1.

Related PICS/PIXIT Statements:

-

9.5.2.4.3.2	Test procedure sequence

Table 9.5.2.4.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS sends an SLPP message of type Request Capabilities including a request for acknowledgement. | <-- | DLInformationTransfer (SLPP REQUEST CAPABILITIES) | - | - |
| 2 Option 1 | Option 1: The UE sends an acknowledgement along with an SLPP message of type Provide Capabilities. | --> | ULInformationTransfer (SLPP PROVIDE CAPABILITIES, incl. acknowledgement response) | 1 | P |
| 2 Option 2 | Option 2: The UE sends an SLPP Acknowledgement response, followed by an SLPP message of type Provide Capabilities. | --> --> | ULInformationTransfer (SLPP ACKNOWLEDGEMENT) ULInformationTransfer (SLPP PROVIDE CAPABILITIES) | 1 | P |
| 3 | IF the UE SLPP message at step 2 includes an acknowledgment request THEN SS sends an SLPP Acknowledgement response. | <-- | DLInformationTransfer (SLPP ACKNOWLEDGEMENT) | - | - |

9.5.2.4.3.3	Specific message contents

Table 9.5.2.4.3.3-1: DLInformationTransfer (steps 1 and 3, Table 9.5.2.4.3.2-1)

| Derivation Path: 38.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| DLInformationTransfer ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| dlInformationTransfer SEQUENCE { |  |  |  |  |
| dedicatedNAS-Message OCTET STRING |  | Set according to Table 9.5.2.4.3.3-2 | DL NAS TRANSPORT |  |
| nonCriticalExtension SEQUENCE {} |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 9.5.2.4.3.3-2: DL NAS TRANSPORT (steps 1 and 3, Table 9.5.2.4.3.2-1)

| Derivation Path: 24.501 Table 8.2.11.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended Protocol discriminator |  | 01111110 | 5GS mobility management messages |  |
| Security header type |  | 0000 | Plain 5GS NAS message |  |
| Spare half octet |  | 0000 | Downlink generic NAS transport |  |
| DL NAS TRANSPORT message identity |  | 01101000 | DL NAS transport |  |
| Payload container type |  | 1100 | SLPP message container |  |
| Spare half octet |  | 0000 |  |  |
| Payload container |  | Step 1: Set according to Table 9.5.2.4.3.3-3 | SLPP Request Capabilities |  |
|  |  | Step 3: Set according to Table 9.5.2.4.3.3-4 | SLPP Acknowledgement |  |
| Additional information |  | Present | Routing Identifier/Correlation ID |  |

Table 9.5.2.4.3.3-3: SLPP REQUEST CAPABILITIES (step 1, Table 9.5.2.4.3.2-1)

| Derivation Path: Table 8.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| As defined in Table 8.5-1 with the following exception: |  |  |  |
| SLPP-Message ::= SEQUENCE { |  |  |  |
| sequenceNumber | 0 |  |  |
| sessionID | Not present |  |  |
| acknowledgement SEQUENCE { |  |  |  |
| ackRequested | True |  |  |
| ackIndicator | (0..255) |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.5.2.4.3.3-4: SLPP Acknowledgement (step 3, Table 9.5.2.4.3.2-1)

| Derivation Path: 38.355 clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SLPP-Message ::= SEQUENCE { |  |  |  |
| applicationLayerID | Not present |  |  |
| transactionID | Not present |  |  |
| endTransaction | TRUE |  |  |
| sequenceNumber | Not present |  |  |
| sessionID | Not present |  |  |
| acknowledgement SEQUENCE { |  |  |  |
| ackRequested | FALSE |  |  |
| ackIndicator | (0..255) | Contains the same value of the sequenceNumber field in step 2, Table 9.5.2.4.3.2-1. |  |
| } |  |  |  |
| slpp-MessageBody | Not present |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |

Table 9.5.2.4.3.3-5: ULInformationTransfer (step 2, Table 9.5.2.4.3.2-1)

| Derivation Path: 38.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| ULInformationTransfer ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| ulInformationTransfer SEQUENCE { |  |  |  |  |
| dedicatedNAS-Message OCTET STRING |  | Set according to Table 9.5.2.4.3.3-6 | UL NAS TRANSPORT |  |
| nonCriticalExtension SEQUENCE {} |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 9.5.2.4.3.3-6: UL NAS TRANSPORT (step 2, Table 9.5.2.4.3.2-1)

| Derivation Path: 24.501 Table 8.2.10.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended Protocol discriminator |  | 01111110 | 5GS mobility management messages |  |
| Security header type |  | 0000 | Plain 5GS NAS message |  |
| Spare half octet |  | 0000 |  |  |
| UL NAS TRANSPORT message identity |  | 01100111 | UL NAS TRANSPORT |  |
| Payload container type |  | 1100 | SLPP message container |  |
| Spare half octet |  | 0000 |  |  |
| Payload container |  | Step 2: Set according to Table Table 9.5.2.4.3.3-7 | SLPP Provide Capabilities |  |
|  |  | Step 2 (Option 2)： Set according to Table Table 9.5.2.4.3.3-8 | SLPP Acknowledgement |  |
| Additional information |  | Present | The UE includes the Routing Identifier received in the Additional Information IE of the DOWNLINK GENERIC NAS TRANSPORT message (step 1 Table 9.5.2.4.3.2-1) |  |

Table 9.5.2.4.3.3-7: SLPP Provide Capabilities (step 2, Table 9.5.2.4.3.2-1)

| Derivation Path: Table 9.5.1.1.3.3-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| As defined in Table 9.5.1.1.3.3-6 with the following exception: |  |  |  |
| SLPP-Message ::= SEQUENCE { |  |  |  |
| applicationLayerID | Application Layer ID in target UE side |  |  |
| transactionID | (0..255) | Contains the same value of the transactionID field in step 1, Table 9.5.2.4.3.2-1. |  |
| endTransaction | True |  |  |
| sequenceNumber | (0..255) |  |  |
| sessionID | Not present |  |  |
| acknowledgement SEQUENCE { | Present, or not present. Present for Option 1. |  |  |
| ackRequested | TRUE or FALSE |  |  |
| ackIndicator | 0 (Option 1) Not present (Option 2) |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.5.2.4.3.3-8: SLPP Acknowledgement (step 2 – Option 2, Table 9.5.2.4.3.2-1)

| Derivation Path: 38.355 clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SLPP-Message ::= SEQUENCE { |  |  |  |
| applicationLayerID | Application Layer ID in target UE side |  |  |
| transactionID | Not present |  |  |
| endTransaction | False |  |  |
| sequenceNumber | Not present |  |  |
| sessionID | Not present |  |  |
| acknowledgement SEQUENCE { |  |  |  |
| ackRequested | FALSE |  |  |
| ackIndicator | 0 |  |  |
| } |  |  |  |
| slpp-MessageBody | Not present |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |

9.5.2.5	SLPP Acknowledgment between UEs (Target UE is in coverage)

9.5.2.5.1	Test Purpose (TP)

(1)

with { Target UE is in coverage and having established PC5 RRC connection with peer UE on unicast sidelink }

ensure that {
  when {	UE receives an SLPP message carrying an acknowledgement request indicator }

then { UE returns an SLPP acknowledgement response }

}

9.5.2.5.2	Conformance requirements

As defined in clause 9.5.2.4.2.

9.5.2.5.3	Test description

9.5.2.5.3.1	Pre-test conditions

System Simulator:

SS-NW

-	NR Cell 1.

-	System information combination NR-14 as defined in TS 38.508-1 [30] clause 4.4.3.1.3 is used in NR cell.

NR-SS-UE

-	NR-SS-UE1 operating as NR sidelink communication device on the resources (i.e. the frequency included in pre-configuration) that UE is expected to use for transmission and reception via PC5 interface.

-	NR-SS-UE1 uses GNSS as the synchronization reference source.

GNSS simulator

-	The GNSS simulator is started and configured for Scenario #1.

UE:

-	UE is authorised to perform NR sidelink communication.

-	For V2X capable UE, the UE is equipped with a USIM containing default values as per TS 38.508-1 [30] clause 4.8.3.3.3.

-	For 5G ProSe capable UE, the UE is equipped with a USIM containing default values as per TS 38.508-1 [30] clause 4.8.3.3.4.

-	UE is synchronised on GNSS.

Preamble:

-	The UE is in state 3N-B RRC_CONNECTED_with_SL and Test Mode (On) with UE test loop mode E as defined in TS 38.508-1 [30], subclause 4.4A using generic procedure parameter Sidelink (On), test mode (On) and GNSS Sync (On) as defined in TS 38.508-1 [30], subclause 4.5.1, using the following procedures:

-	for V2X capable UE, using UE initiated unicast mode NR sidelink communication procedure in subclause 4.9.22;

-	for 5G ProSe capable UE, using UE initiated unicast mode ProSe Direct communication procedure in subclause 4.9.39.

Related PICS/PIXIT Statements:

-

9.5.2.5.3.2	Test procedure sequence

Table 9.5.2.5.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The NR-SS-UE 1 sends an SLPP message of type Request Capabilities including a request for acknowledgement. | <-- | SLPP REQUEST CAPABILITIES | - | - |
| 2 Option 1 | Option 1: The UE sends an acknowledgement along with an SLPP message of type Provide Capabilities. | --> | SLPP PROVIDE CAPABILITIES, incl. acknowledgement response | 1 | P |
| 2 Option 2 | Option 2: The UE sends an SLPP Acknowledgement response, followed by an SLPP message of type Provide Capabilities. | --> --> | SLPP ACKNOWLEDGEMENT SLPP PROVIDE CAPABILITIES | 1 | P |
| 3 | IF the UE SLPP message at step 2 includes an acknowledgment request THEN NR-SS-UE 1 sends an SLPP Acknowledgement response. | <-- | SLPP ACKNOWLEDGEMENT | - | - |

9.5.2.5.3.3	Specific message contents

Table 9.5.2.5.3.3-1: SLPP REQUEST CAPABILITIES (step 1, Table 9.5.2.5.3.2-1)

| Derivation Path: Table 8.5-1 with UE-only mode condition. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| As defined in Table 8.5-1 with the following exception: |  |  |  |
| SLPP-Message ::= SEQUENCE { |  |  |  |
| sequenceNumber | 0 |  |  |
| sessionID | Present. Any value acceptable. | OCTET STRING (SIZE (6)) |  |
| acknowledgement SEQUENCE { |  |  |  |
| ackRequested | True |  |  |
| ackIndicator | (0..255) |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.5.2.5.3.3-2: SLPP Provide Capabilities (step 2, Table 9.5.2.5.3.2-1)

| Derivation Path: Table 9.5.1.1.3.3-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| As defined in Table 9.5.1.1.3.3-6 with the following exception: |  |  |  |
| SLPP-Message ::= SEQUENCE { |  |  |  |
| applicationLayerID | Application Layer ID in target UE side |  |  |
| transactionID | (0..255) | Contains the same value of the transactionID field in step 1, Table 9.5.2.5.3.2-1. |  |
| endTransaction | True |  |  |
| sequenceNumber | (0..255) |  |  |
| sessionID | Present. | Contains the same value of the sessionID field in step 1, Table 9.5.2.5.3.2-1. |  |
| acknowledgement SEQUENCE { | Present, or not present. Present for Option 1. |  |  |
| ackRequested | TRUE or FALSE |  |  |
| ackIndicator | 0 (Option 1) Not present (Option 2) |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.5.2.5.3.3-3: SLPP Acknowledgement (step 2 – Option 2, Table 9.5.2.5.3.2-1)

| Derivation Path: 38.355 clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SLPP-Message ::= SEQUENCE { |  |  |  |
| applicationLayerID | Application Layer ID in target UE side |  |  |
| transactionID | Not present |  |  |
| endTransaction | False |  |  |
| sequenceNumber | Not present |  |  |
| sessionID | Present. | Contains the same value of the sessionID field in step 1, Table 9.5.2.5.3.2-1. |  |
| acknowledgement SEQUENCE { |  |  |  |
| ackRequested | FALSE |  |  |
| ackIndicator | 0 |  |  |
| } |  |  |  |
| slpp-MessageBody | Not present |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |

Table 9.5.2.5.3.3-4: SLPP Acknowledgement (step 3, Table 9.5.2.5.3.2-1)

| Derivation Path: 38.355 clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SLPP-Message ::= SEQUENCE { |  |  |  |
| applicationLayerID | '00 00 01 00'H | Application Layer ID in initiating UE side |  |
| transactionID | Not present |  |  |
| endTransaction | TRUE |  |  |
| sequenceNumber | Present. Any value acceptable. | OCTET STRING (SIZE (6)) |  |
| sessionID | Present. | Contains the same value of the sessionID field in step 2, Table 9.5.2.5.3.2-1. |  |
| acknowledgement SEQUENCE { |  |  |  |
| ackRequested | FALSE |  |  |
| ackIndicator | (0..255) | Contains the same value of the sequenceNumber field in step 2, Table 9.5.2.5.3.2-1. |  |
| } |  |  |  |
| slpp-MessageBody | Not present |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |

9.5.2.6	SLPP Acknowledgment between UEs (Target UE is out of coverage)

9.5.2.6.1	Test Purpose (TP)

(1)

with { Target UE is out of coverage and having established PC5 RRC connection with peer UE on unicast sidelink }

ensure that {
  when {	UE receives an SLPP message carrying an acknowledgement request indicator }

then { UE returns an SLPP acknowledgement response }

}

9.5.2.6.2	Conformance requirements

As defined in clause 9.5.2.4.2.

9.5.2.6.3	Test description

9.5.2.6.3.1	Pre-test conditions

System Simulator:

NR-SS-UE

-	NR-SS-UE1 operating as NR sidelink communication device on the resources (i.e. the frequency included in pre-configuration) that UE is expected to use for transmission and reception via PC5 interface.

-	NR-SS-UE1 uses GNSS as the synchronization reference source.

GNSS simulator

-	The GNSS simulator is started and configured for Scenario #1.

UE:

-	UE is authorised to perform NR sidelink communication.

-	For V2X capable UE, the UE is equipped with a USIM containing default values as per TS 38.508-1 [30] clause 4.8.3.3.3.

-	For 5G ProSe capable UE, the UE is equipped with a USIM containing default values as per TS 38.508-1 [30] clause 4.8.3.3.4.

-	UE is synchronised on GNSS.

Preamble:

-	The UE is in state 4-A and Test Mode (On), Test Loop Function (Off) as defined in TS 38.508-1 [30], Table 4.5.7.2-1 using generic procedure parameter Sidelink (On),Cast Type (Unicast), UE initiating unicast mode NR sidelink communication, GNSS Sync (On), using the following procedures:

-	for V2X capable UE, using UE initiated unicast mode NR sidelink communication procedure in subclause 4.9.22;

-	for 5G ProSe capable UE, using UE initiated unicast mode ProSe Direct communication procedure in subclause 4.9.39.

Related PICS/PIXIT Statements:

-

9.5.2.6.3.2	Test procedure sequence

As defined in clause 9.5.2.5.3.2.

9.5.2.6.3.3	Specific message contents

As defined in clause 9.5.2.5.3.3.