3GPP TSG-RAN5 Meeting #107	R5-253607

# Malta, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.561 | CR | 0025 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to Talk Mode TRP and TRS test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple Inc |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FR1_TRP_TRS_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Update test procedures and test tolerances for Rel-18 Talk Mode and also implementation of an agreed CR R5-251722 from RAN5#106 was missed |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update test case and Annex D with TT for Talk Mode and also implement agreed CR which was missed to be implemented |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing test case details, MU and TT values for Rel-18 Talk Mode |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1.1.1, 6.2.1.2.1, 7.2.1.2.1,Annex D |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1 makes the following updates - - Neccessary editorial correction to Table 6.2.1.1.1.3.2-1 - 3GPP Style correction to sub-clause 6.2.1.2.1.2 - Update structure of minimum conformance requirements subclause and correct Table number - Correct table numbers and titles - Updated impacted clauses on the cover page |  |  |  |  |  |  |  |  |

<Unchanged sections Skipped>

Table 6.2.1.1.1.3.2-1: Handheld PC3 UE TRP minimum performance requirement for NR FR1 bands in the hand phantom browsing position and the primary mechanical mode

| NR Band | Bandwidth (MHz) | Usage Scenario | Power Class 3 |  |
| --- | --- | --- | --- | --- |
|  |  |  | Average TRP (dBm) |  |
|  |  |  | UE width ≤ 72mm | UE width > 72mm |
| n1 | 15 | HL and HR |  | 11.6 |
| n28 | 20 | HL and HR |  | 101 |
| n41 | 100 | HL and HR |  | 9.5 |
| n78 | 100 | HL and HR |  | 10 |
| n79 | 100 | HL and HR |  |  |
| NOTE 1: 0.5dB higher value will be adopted in March 2026 (RAN#111) in the core specifications. |  |  |  |  |

<Unchanged sections Skipped>

##### 6.2.1.2.1	Total Radiated Power (TRP) for FR1 NR Standalone (SA) in Talk Mode with Head and Hand Phantom

6.2.1.2.1.1	Test purpose

To verify that the total radiated power (TRP) of a 5G NR FR1 UE in talk mode with head and hand phantom does not exceed the range prescribed by the specified nominal maximum output power and tolerance.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

6.2.1.2.1.2	Test applicability

This test case applies to Release 18 and forward UEs that support.NR Power Class 2 and Power Class 3

6.2.1.2.1.3	Minimum conformance requirements

The average measured Total Radiated Power (TRP) of low, mid and high channel for handheld UE shall be higher than the average TRP requirement specified in subclauses 6.2.1.1 and 6.2.1.2. The averaging shall be done in linear scale for the TRP results of both right and left side of the phantom head in case of beside the head and hand phantom positions. For the hand phantom browsing mode position the averaging shall be done in linear scale for the TRP results of both right and left hand phantom measurements.

TRPaverage=10log10Pleft_low/10+10Pleft_mid/10+10Pleft_high/10+10Pright_low/10+10Pright_mid/10+10Pright_high/106

For UEs which do not support NR FR1 in stand-alone mode, EN-DC mode requirements apply. SA and EN-DC test case applicability is defined in Clause 4.3, and test case applicability based on power class capability is defined in Clause 4.4. The relevant test methodology is defined in Annexes A and B of the present document.

Handheld UE TRP minimum performance requirement for NR FR1 bands in the beside head and hand phantom talk position and the primary mechanical mode are defined in Tables 6.2.1.2.1-1 and 6.2.1.2.1-2.

Table 6.2.1.2.1-1: VOID

Table 6.2.1.2.1-2: VOID

6.2.1.2.1.3.1	Head and Hand phantom Talk mode

Head and Hand phantom Talk mode positions are defined in Clause B.3.2.

6.2.1.2.1.3.2	Minimum conformance requirements for NR FR1 in head and hand phantom talk mode position

Handheld UE TRP minimum performance requirement for NR FR1 bands in the beside head and hand phantom talk position and the primary mechanical mode are defined in Tables 6.2.1.2.1-1 and 6.2.1.2.1-2.

Table 6.2.1.2.1.3.2-1: Handheld PC3 UE TRP minimum performance requirement for NR FR1 bands in the beside head and hand phantom talk position and the primary mechanical mode

| NR Band | Bandwidth (MHz) | Usage Scenario | Power Class 3 |  |
| --- | --- | --- | --- | --- |
|  |  |  | Average TRP (dBm) |  |
|  |  |  | UE width ≤ 72mm | UE width > 72mm |
| n1 | 15 | BHHL and BHHR |  | 10.6 |
| n28 | 20 | BHHL and BHHR |  | 7.21 |
| n41 | 100 | BHHL and BHHR |  | 7 |
| n78 | 100 | BHHL and BHHR |  | 8.52 |
| n79 | 100 | BHHL and BHHR |  |  |
| NOTE 1: 0.4 dB higher value will be adopted in March 2026 (RAN#111) in the core specifications. NOTE 2: 0.5 dB higher value will be adopted in March 2026 (RAN#111) in the core specifications. |  |  |  |  |

Table 6.2.1.2.1.3.2-2: Handheld PC2 UE TRP minimum performance requirement for NR FR1 bands in the beside head and hand phantom talk position and the primary mechanical mode

| NR Band | Bandwidth (MHz) | Usage Scenario | Power Class 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | Average TRP (dBm) |  |
|  |  |  | UE width ≤ 72mm | UE width > 72mm |
| n1 | 15 | BHHL and BHHR |  |  |
| n28 | 20 | BHHL and BHHR |  |  |
| n41 | 100 | BHHL and BHHR |  | 10 |
| n78 | 100 | BHHL and BHHR |  | 11.51 |
| n79 | 100 | BHHL and BHHR |  |  |
| NOTE 1: 0.5dB higher value will be adopted in March 2026 (RAN#111) in the core specifications. |  |  |  |  |

6.2.1.2.1.4	Test description

6.2.1.2.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.2.1-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing and are shown in table 5.3-1. The details of the uplink reference measurement channels (RMCs) mainly comprising the uplink modulation and uplink RB allocation are also specified in Table 5.3-1. Configurations of PDSCH and PDCCH before measurement are specified in Annex A.0.

The following steps are recommended as per Annex A.2.2.

-	P-MPRc shall be 0 dB.

-	FR1 TRP radiated conformance testing shall be performed with the UE consistently operating at maximum power level, e.g., Time-Averaged Algorithm (TAA) and other power back-off functions should be disabled. The above functions being OFF should be based on manufacturer declaration; if declared, then the manufacturer is required to provide a mechanism for the test lab to enable/disable the function.

-	The NR SS should send continuous uplink power control “up” commands to the DUT to ensure the DUT’s transmitter is at maximum output power during the SA TRP and TRS test.

-	It is recommended to disable Transmit Antenna Switching (TAS) as per the procedure defined in Annex A.3.6

1.	For DUT containing multiple Tx antennas, it shall be ensured that the Tx Antenna Switching (TAS) function should be OFF, and the TRP should be measured for each Tx antenna individually.

NOTE: The primary antenna should be selected based on manufacturer declaration. In the absence of manufacturer declaration, the antenna with better TRP is identified as the primary antenna, and the corresponding TRP result will be used to determine the pass/fail compliance.

2.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [2] Annex A, Figure A.3.1.1.1 for TE diagram and section A.3.2 for UE diagram.

3.	The parameter settings for the cell are set up according to TS 38.508-1 [2] subclause 4.4.3.

4.	Downlink signals are initially set up according to Annex C in TS 38.521-1 [11] and uplink signals according to Annex G in TS. 38.521-1 [11].

5.	The UL and DL parameters are set according to Table 5.3-1.

6.	Propagation conditions are set to Static.

7.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [2] clause 4.5. Message contents are defined in clause 6.2.1.4.3.

6.2.1.2.1.4.2	Test procedure

For TRP measurement, the evaluations shall be performed at maximum transmit power. The applicability rules for testing of UE power class in clause 4.4 shall be followed. The measurement procedure includes the following steps:

1.	Place the DUT inside the QZ following the UE positioning guidelines defined in Annex B.3.1.

2.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 5.3-1. For any additional RMC parameters beyond Table 5.3-2, refer to Table 6.2.1.4.1-1 of TS 38.521-1 [11].. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

3.	Ensure the DUT transmits with its maximum power by continuously sending uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

4.	The TRP of the DUT is measured by sampling the radiated transmit power of the DUT with three-dimensional scan at various locations surrounding the device. Measure the power at each measurement point, and calculate EIRPθ,ϕ by adding the composite loss of the entire transmission path.

5.	The measurement is performed with a constant sampling step in both theta () and phi () axes using any of the measurement grids and quadrature options outlined and applicable to TRP in Table A.4.2.12-1. Any of the measurement grids in Table A.4.2.12-1 could be used for testing. Measurements at theta = 0 and 180 degrees only require one measurement each. For test systems that cannot measure 180º EIRP, an extrapolation approach shall be adopted when generating the 3D antenna pattern and calculating TRP.

6.	All the measured power values will be integrated to TRP, as defined in Annex A.3.5.1 with the procedure further detailed in A.3.3.2.

6.2.1.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 and 5.4 with the following exceptions.

Table 6.2.1.1.1.4.3-1: PUSCH-Config

Derivation Path: TS 38.508-1 [5], Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED

6.2.1.2.1.5	Test requirement

The TRP across low, mid and high channels tested with head and hand phantom in Talk mode position shall be higher than test performance requirements shown in Table 6.2.1.2.1.5-1 for Power Class 3 and 6.2.1.2.1.5-2 for Power Class 2.

Table 6.2.1.2.5-1: Handheld PC3 UE TRP test requirement for NR FR1 bands in the beside head and hand phantom talk position and the primary mechanical mode

| NR Band | Bandwidth (MHz) | Usage Scenario | Power Class 3 |  |
| --- | --- | --- | --- | --- |
|  |  |  | Average TRP (dBm) 3 |  |
|  |  |  | UE width ≤ 72mm | UE width > 72mm |
| n1 | 15 | BHHL and BHHR |  | 10.6-TT |
| n28 | 20 | BHHL and BHHR |  | 7.21-TT |
| n41 | 100 | BHHL and BHHR |  | 7-TT |
| n78 | 100 | BHHL and BHHR |  | 8.52-TT |
| n79 | 100 | BHHL and BHHR |  |  |
| NOTE 1: 0.4dB higher value will be adopted in March 2026 (RAN#111) in the core specifications. NOTE 2: 0.5 dB higher value will be adopted in March 2026 (RAN#111) in the core specifications. NOTE 3: The test requirement shall be calculated by utilizing the test tolerances defined in Table 6.2.1.2.1.5-3 |  |  |  |  |

Table 6.2.1.2.5-2: Handheld PC2 UE TRP test requirement for NR FR1 bands in the beside head and hand phantom talk position and the primary mechanical mode

| NR Band | Bandwidth (MHz) | Usage Scenario | Power Class 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | Average TRP (dBm) 2 |  |
|  |  |  | UE width ≤ 72mm | UE width > 72mm |
| n1 | 15 | BHHL and BHHR |  |  |
| n28 | 20 | BHHL and BHHR |  |  |
| n41 | 100 | BHHL and BHHR |  | 10-TT |
| n78 | 100 | BHHL and BHHR |  | 11.51-TT |
| n79 | 100 | BHHL and BHHR |  |  |
| NOTE 1: 0.5dB higher value will be adopted in March 2026 (RAN#111) in the core specifications. NOTE 2: The test requirement shall be calculated by utilizing the test tolerances defined in Table 6.2.1.2.1.5-3 |  |  |  |  |

Table 6.2.1.2.1.5-3: Test Tolerance (NR FR1 TRP)

| Operating band | Test Tolerance (dB) |
| --- | --- |
| n1 | 1.17 |
| n28 | 1.17 |
| n41 | 1.17 |
| n78 | 1.22 |

<Unchanged sections skipped>

##### 7.2.1.2.1	Total Radiated Sensitivity (TRS) for FR1 NR Standalone (SA) in Talk Mode with Head and Hand Phantom

7.2.1.2.1.1	Test purpose

To verify that the total radiated sensitivity (TRS) of a 5G NR FR1 UE in talk mode with head and hand phantom does not exceed the range prescribed by the specified nominal maximum output power and tolerance.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

7.2.1.2.1.2	Test applicability

This test case applies to all types of NR Power Class 2 and Power Class 3 UEs Release 18 and forward.

7.2.1.2.1.3	Minimum conformance requirements

7.2.1.2.1.3	Minimum conformance requirements

The average measured total radiated sensitivity (TRS) of low, mid and high channel for handheld UE shall be lower than the average TRS requirement specified in subclauses 7.2.1.1 and 7.2.1.2. The averaging shall be done in linear scale for the TRS results of both right and left side of the phantom head in case of beside the head and hand phantom positions. For the hand phantom browsing mode position the averaging shall be done in linear scale for the TRS results of both right and left hand phantom measurements.

TRSaverage=10log6/(110Pleft_low10+110Pleft_mid10+110Pleft_high10+110Pright_low10+110Pright_mid10+110Pright_high10)

For UEs which do not support NR FR1 in stand-alone mode, EN-DC mode requirements apply. SA and EN-DC test case applicability is defined in Clause 4.3, and test case applicability based on power class capability is defined in Clause 4.4. The relevant test methodology is defined in Annexes A and B of the present document.

7.2.1.2.1.3.1	Head and Hand phantom browsing mode

Hand phantom browsing mode positions are defined in Clause B.3.1.

7.2.1.2.1.3.2	Minimum conformance requirements for NR FR1 in head and hand phantom talk mode position

Handheld UE TRP minimum performance requirement for NR FR1 bands in the beside head and hand phantom position and the primary mechanical mode are defined in Tables 7.2.1.2.1-1 and 7.2.1.2.1-2.

Table 7.2.1.2.1-1: Handheld PC3 and PC2 UE TRS minimum performance requirement for NR FR1 bands in the beside head and hand phantom position and the primary mechanical mode

| NR Band | Bandwidth (MHz) | Usage Scenario | Power Class 3 and Power Class 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | Average TRS (dBm) |  |
|  |  |  | UE width ≤ 72mm | UE width > 72mm |
| n1 | 15 | BHHL and BHHR |  | -87 |
| n28 | 20 | BHHL and BHHR |  | -80 |
| n41 | 100 | BHHL and BHHR |  | -79 |
| n78 | 100 | BHHL and BHHR |  | -80 |
| n79 | 100 | BHHL and BHHR |  |  |

7.2.1.2.1.4	Test description

7.2.1.1.1.4.1	Initial conditions

Initial conditions are a set of test configurations that the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.2.1-1. All these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing and are shown in table 5.3-2. The details of the uplink and downlink test parameters mainly comprising the uplink modulation and uplink RB allocation are also specified in Table 5.3-2. Configurations of PDSCH and PDCCH before measurement are specified in Annex A.0

The following steps are recommended as per Annex A.2.2.

-	P-MPRc shall be 0 dB.

-	FR1 TRP radiated conformance testing shall be performed with the UE consistently operating at maximum power level, e.g., Time-Averaged Algorithm (TAA) and other power back-off functions should be disabled. The above functions being OFF should be based on manufacturer declaration; if declared, then the manufacturer is required to provide a mechanism for the test lab to enable/disable the function.

-	The NR SS should send continuous uplink power control “up” commands to the DUT to ensure the DUT’s transmitter is at maximum output power during the SA TRS test.

-	For TRS measurement, no specific setting is needed for Rx antennas. By default, the maximum number of Rx antennas supported at each band should be enabled during the TRS test.

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [2] Annex A, Figure A.3.1.1.1 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [2] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C in TS 38.521-1 [11] and uplink signals according to Annex G in TS 38.521-1 [11].

4.	The UL and DL parameters are set according to Table 5.3-2.

5.	Propagation conditions are set to Static.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [2] clause 4.5. Message contents are defined in clause 7.2.1.4.3.

7.2.1.2.1.4.2	Test procedure

For TRS measurement, the evaluations shall be performed at maximum transmit power. The measurement procedure includes the following steps:

1.	Place the DUT inside the QZ following the UE positioning guidelines defined in Annex B.3.1.

2.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 5.3-2. For any additional RMC parameters beyond Table 5.3-2 refer to Table 7.3.2.4.1-1 of TS 38.521-1 [11]. The SS sends downlink MAC padding bits on the DL RMC.

3.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL parameters defined in Table 5.3-2 and any additional parameters from Table 7.3.2.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

4.	Ensure the DUT transmits with its maximum power by continuously sending uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

5.	The TRS of the DUT is measured by sampling effective isotropic sensitivity (EIS) of the DUT with three-dimensional scan at various locations surrounding the device. Determine each EIS by adjusting the downlink signal level until the minimum power level at which the throughput exceeds or equal to 95% of the maximum throughput of the specified RMC, at each sampling point. The downlink power step size shall be no more than 0.5 dB when the RF power level is near the NR sensitivity level.

6.	The measurement is performed with a constant sampling step of in both theta () and phi () axes using any of the measurement grids and quadratures' options outlined and applicable to TRS in Table A.4.2.12-1. Any of the measurement grids in Table A.4.2.12-1 could be used for testing. Measurements at theta = 0 and 180 degrees only require one measurement each. For test systems that cannot measure 180º EIS, an extrapolation approach shall be adopted when generating the 3D antenna pattern and calculating TRS.

7.	All of the measured power values at each position or measurement angle will be integrated to TRS, as defined in Annex A.3.5.2. with the procedure further detailed in Annex A.3.3.3

8.	The reported TRS value shall be corrected by -0.5·output level step resolution of the final power step search and note the correction in the test report. If the alternate EIS search procedure is applied as described in A.3.3.3.1 to assess the EIS at each grid point, no correction of TRS shall be applied.

7.2.1.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 and 5.4 with the following exceptions.

Table 7.2.1.1.1.4.3-1: PUSCH-Config

Derivation Path: TS 38.508-1 [5], Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED

7.2.1.2.1.5	Test requirement

The TRS across low, mid and high channels tested with hand phantom browsing mode position shall be lower than test performance requirements shown in Table 7.2.1.1.1.5-1.

Table 7.2.1.2.1.5-1: Handheld FR1 TRS test requirement for NR FR1 bands in the beside head and hand phantom position and the primary mechanical mode

| NR Band | Bandwidth (MHz) | Usage Scenario | Power Class 3 and Power Class 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | Average TRS (dBm)1 |  |
|  |  |  | UE width ≤ 72mm | UE width > 72mm |
| n1 | 15 | BHHL and BHHR |  | -87+TT |
| n28 | 20 | BHHL and BHHR |  | -80+TT |
| n41 | 100 | BHHL and BHHR |  | -79+TT |
| n78 | 100 | BHHL and BHHR |  | -80+TT |
| NOTE 1: The test requirement shall be calculated by utilizing the test tolerances defined in Table 6.2.1.2.1.5-3 |  |  |  |  |

Table 7.2.1.2.1.5-2: Test Tolerance (NR FR1 TRS)

| Operating band | Test Tolerance (dB) |
| --- | --- |
| n1 | 1.40 |
| n28 | 1.40 |
| n41 | 1.40 |
| n78 | 1.44 |

<Unchanged sections skipped>

Annex D (normative): Maximum uncertainty of test system and test tolerance

# D.1	Maximum uncertainty of test system

- The maximum acceptable uncertainty of the Test System is specified in Table D.1-1 (for Browsing Mode) and Table D.1-2 (for Talk Mode) for each test, where appropriate. The Test System shall enable each test to be measured with an uncertainty not exceeding the specified values. All ranges and uncertainties are absolute values and are valid for a confidence level of 95 %.

- In Annex A.4, the estimation of measurement uncertainty is defined.

- Table D.1-1: Maximum Test System uncertainties for TRP and TRS hand only (browsing mode) tests using anechoic chamber method for NR FR1 bands

| - Clause | - Maximum Test System Uncertainty (MTSU) | - Derivation of Test System Uncertainty |
| --- | --- | --- |
| - 6.2.1.1.1 Total Radiated Power (TRP) for FR1 NR Standalone (SA) in Browsing Mode with Hand Phantom | - 1.64 dB : < 3 GHz - 1.73 dB >= 3 GHz | - Refer to Annex A.4 |
| - 7.2.1.1.1 Total Radiated Sensitivity (TRS) for FR1 NR Standalone (SA) in Browsing Mode with Hand Phantom | - 2.06 dB : < 3 GHz - 2.13 dB >= 3 GHz | - Refer to Annex A.4 |

- Table D.1-2: Maximum Test System uncertainties for TRP and TRS Beside Head and Hand (Talk mode) tests using anechoic chamber method for NR FR1 bands

| - Clause | - Maximum Test System Uncertainty (MTSU) | - Derivation of Test System Uncertainty |
| --- | --- | --- |
| - 6.2.1.2.1 Total Radiated Power (TRP) for FR1 NR Standalone (SA) in Talk Mode with Head and Hand Phantom | 1.88 dB : < 3 GHz 1.96 dB >= 3 GHz | - Refer to Annex A.4 |
| - 7.2.1.2.1 Total Radiated Sensitivity (TRS) for FR1 NR Standalone (SA) in Talk Mode with Head and Hand Phantom | 2.25 dB : < 3 GHz 2.31 dB >= 3 GHz | - Refer to Annex A.4 |

### D.2	Test Tolerances

- Test tolerances in Table D.2-1 (for browsing mode) and Table D.2-2 (for talk mode) shall be used to relax the Minimum Requirements in the present document and to derive the Test Requirements.

- Table D.2-1: Test Tolerances for TRP and TRS hand only (browsing mode) tests using anechoic chamber method for NR FR1 bands

| - Clause | - Test Tolerance (TT) |
| --- | --- |
| - 6.2.1.1.1 Total Radiated Power (TRP) for FR1 NR Standalone (SA) in Browsing Mode with Hand Phantom | TT = 0.62 x MTSU 1.02 dB : < 3 GHz 1.07 dB >= 3 GHz |
| - 7.2.1.1.1 Total Radiated Sensitivity (TRS) for FR1 NR Standalone (SA) in Browsing Mode with Hand Phantom | TT = 0.62 x MTSU 1.28 dB : < 3 GHz - 1.32 dB >= 3 GHz |

- Table D.2-2: Test Tolerances for TRP and TRS head and hand (Talk mode) tests using anechoic chamber method for NR FR1 bands

| - Clause | - Test Tolerance (TT) |
| --- | --- |
| - 6.2.1.2.1 Total Radiated Power (TRP) for FR1 NR Standalone (SA) in Browsing Mode with Hand Phantom | TT = 0.62 x MTSU 1.17 dB : < 3 GHz 1.22 dB >= 3 GHz |
| - 7.2.1.2.1 Total Radiated Sensitivity (TRS) for FR1 NR Standalone (SA) in Browsing Mode with Hand Phantom | TT = 0.62 x MTSU 1.40 dB : < 3 GHz - 1.44 dB >= 3 GHz |

<End of changes>