3GPP TSG-RAN5 Meeting #107	R5-252372

# Malta, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4988 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR NTN testcase 7.1.1.1.31 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_enh_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Please refer to Prose CR R5-252347 |  |  |  |  |  |  |  |  |
| Summary of change: |  | NR-28 sys info combination is replaced with NR-29 sys info combination. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Prose will refer to a voided sys info combination |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.1.31 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.508-1 (R5-252347) |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 7.1.1.1.31	NR NTN Enhancements / Random access procedure / Coverage Enhancement

7.1.1.1.31.1	Test Purpose (TP)

(1)

with { UE supporting NTN Enh and in NR RRC_IDLE state }

ensure that {

when { UE receives SIB19 configured with numberOfMsg4HARQ-ACK-Repetitions-r18 }

then { UE reports the capability using LCID Code point for Msg4 HARQ-ACK repeatition in Msg3 PUSCH and UE repeats Msg4 HARQ-ACK with the repetition number as configured earlier in SIB19  numberOfMsg4HARQ-ACK-Repetitions-r18 }

(2)

with { UE supporting NTN Enh and in NR RRC_IDLE state}

ensure that {

when { UE receives SIB19 configured with numberOfMsg4HARQ-ACK-Repetitions-r18 and  rsrp-ThresholdMsg4HARQ }

then { UEs reports the capability of PUCCH repetition for Msg4 HARQ-ACK in Msg3 PUSCH and repeats Msg4 HARQ-ACK with the repetition number configured for PUCCH when RSRP condition is met }

7.1.1.1.31.2	Conformance requirements

References: The conformance requirements covered in the present test case are specified in: TS38.300 clause 16.14.9. TS 38.321 clause 6.1.2. 6.2.1 , TS38.213 clause 9.2.6 and 9.2.1. Unless otherwise stated these are Rel-18 requirements.

[TS 38.300, clause 16.14.9]

To improve NR uplink coverage in NTN, the following enhancements are supported:

-	PUCCH repetition for Msg4 HARQ-ACK configured in system information or dynamically in DCI for Msg4 when multiple repetition factors are configured in the system information:

-	UEs reports the capability of PUCCH repetition for Msg4 HARQ-ACK in Msg3 PUSCH;

-	When Msg4 HARQ-ACK is repeated, PUCCH repetition is applied for all PUCCH transmission before dedicated PUCCH resource is provided.

-	Improved channel estimation by NTN-specific PUSCH DMRS bundling enhancement that enables DMRS bundling in presence of timing drift, where the UE maintains phase continuity by considering effects of transmission delay variation between the UE and the uplink time synchronization reference point.

[TS 38.321, clause 6.1.2]

A MAC subheader except for fixed sized MAC CE, padding, and a MAC SDU containing UL CCCH consists of the header fields R/F/LCID/(eLCID)/L. A MAC subheader for fixed sized MAC CE and padding consists of the header fields R/LCID/(eLCID). A MAC subheader for a MAC SDU containing UL CCCH consists of the header fields (LX)/R/LCID.

...

Figure 6.1.2-3: (LX)/R/LCID/(eLCID) MAC subheader

MAC CEs are placed together. DL MAC subPDU(s) with MAC CE(s) is placed before any MAC subPDU with MAC SDU and MAC subPDU with padding as depicted in Figure 6.1.2-4. UL MAC subPDU(s) with MAC CE(s) is placed after all the MAC subPDU(s) with MAC SDU and before the MAC subPDU with padding in the MAC PDU as depicted in Figure 6.1.2-5. The size of padding can be zero.

Figure 6.1.2-4: Example of a DL MAC PDU

Figure 6.1.2-5: Example of a UL MAC PDU

A maximum of one MAC PDU can be transmitted per TB per MAC entity.

[TS 38.321, clause 6.2.1]

The MAC subheader consists of the following fields:

-	LCID: The Logical Channel ID field identifies the logical channel instance of the corresponding MAC SDU or the type of the corresponding MAC CE or padding as described in Tables 6.2.1-1 and 6.2.1-1c for the DL-SCH and Tables 6.2.1-2 and 6.2.1-2c for the UL-SCH. There is one LCID field per MAC subheader. The size of the LCID field is 6 bits. If the LCID field is set to 34 as in Table 6.2.1-1 or 6.2.1-2, one additional octet is present in the MAC subheader containing the eLCID field and follow the octet containing LCID field. If the LCID field is set to 33 as in Table 6.2.1-1 or 6.2.1-2, two additional octets are present in the MAC subheader containing the eLCID field and these two additional octets follow the octet containing LCID field;

NOTE 1:	For MBS broadcast, a logical channel is identified based on G-RNTI and LCID if the same LCID is allocated for logical channels corresponding to different G-RNTIs.

-	eLCID: The extended Logical Channel ID field identifies the logical channel instance of the corresponding MAC SDU or the type of the corresponding MAC CE as described in tables 6.2.1-1a, 6.2.1-1b, 6.2.1-2a and 6.2.1-2b for the DL-SCH and UL-SCH respectively. The size of the eLCID field is either 8 bits or 16 bits.

NOTE 2:	The extended Logical Channel ID space using two-octet eLCID and the relevant MAC subheader format is used, only when configured, on the NR backhaul links between IAB nodes or between IAB node and IAB Donor, or for multicast MTCHs.

-	L: The Length field indicates the length of the corresponding MAC SDU or variable-sized MAC CE in bytes. There is one L field per MAC subheader except for subheaders corresponding to fixed-sized MAC CEs, padding, and MAC SDUs containing UL CCCH. The size of the L field is indicated by the F field;

-	F: The Format field indicates the size of the Length field. There is one F field per MAC subheader except for subheaders corresponding to fixed-sized MAC CEs, padding, and MAC SDUs containing UL CCCH. The size of the F field is 1 bit. The value 0 indicates 8 bits of the Length field. The value 1 indicates 16 bits of the Length field;

-	LX: The LCID extension field indicates the use of extended LCID space. The size of the LX field is 1 bit. The LX field set to 1 indicates the use of Table 6.2.1-2c, otherwise R bit is present instead (i.e. set to 0);

-	R: Reserved bit, set to 0.

...

Table 6.2.1-2c: Values of LCID for UL-SCH when the LX field is set to 1

| Codepoint | Index | LCID values |
| --- | --- | --- |
| 0 | (216 + 320) | CCCH of size 48 bits for an eRedCap UE |
| 1 | (216 + 321) | CCCH of size 64 bits for an eRedCap UE |
| 2 | (216 + 322) | CCCH of size 48 bits for PUCCH repetition of Msg4 HARQ-ACK, except for an (e)RedCap UE |
| 3 | (216 + 323) | CCCH of size 64 bits for PUCCH repetition of Msg4 HARQ-ACK, except for an (e)RedCap UE |
| 4 | (216 + 324) | CCCH of size 48 bits for PUCCH repetition of Msg4 HARQ-ACK of a RedCap UE |
| 5 | (216 + 325) | CCCH of size 64 bits for PUCCH repetition of Msg4 HARQ-ACK of a RedCap UE |
| 6 | (216 + 326) | CCCH of size 48 bits for PUCCH repetition of Msg4 HARQ-ACK of an eRedCap UE |
| 7 | (216 + 327) | CCCH of size 64 bits for PUCCH repetition of Msg4 HARQ-ACK of an eRedCap UE |
| 8 to 63 | (216 + 328) to (216 + 383) | Reserved |
| NOTE 1: The MAC entity may use the code point corresponding to a given feature or feature combination in Table 6.2.1-2c only if network indicates support for the corresponding feature or feature combination. NOTE 2: CCCH of size 48 bits and CCCH of size 64 bits are referred to as CCCH and CCCH1, respectively, in TS 38.331 [5]. NOTE 3: For UE capable of PUCCH repetition of Msg4 HARQ-ACK, the MAC entity uses the code points corresponding to PUCCH repetition of Msg4 HARQ-ACK if numberOfMsg4HARQ-ACK-Repetitions is configured and rsrp-ThresholdMsg4HARQ-ACK is not configured, or if both are configured and the RSRP of the downlink pathloss reference is less than rsrp-ThresholdMsg4HARQ-ACK. |  |  |

[TS 38.213, clause 9.2.6]

A UE that does not have dedicated PUCCH resource configuration and indicates a capability to transmit with repetitions a PUCCH with HARQ-ACK information [11, TS 38.321], determines a number of NPUCCHrepeat slots for repetitions of a PUCCH transmission with HARQ-ACK information based on an indication by numberOfPUCCHforMsg4HARQACK-RepetitionsList. If numberOfPUCCHforMsg4HARQACK-RepetitionsList provides more than one values, the DAI field in a DCI format 1_0 with CRC scrambled by a TC-RNTI scheduling a PDSCH reception that includes a UE contention resolution identity indicates NPUCCHrepeat from the more than one values. The UE transmits any PUCCH with repetitions over NPUCCHrepeat slots before dedicated PUCCH resource configuration is provided. The UE transmits each repetition of the PUCCH using frequency hopping as described in Clause 9.2.1.

[TS 38.213, clause 9.2.1]

If a UE does not have dedicated PUCCH resource configuration, provided by PUCCH-ResourceSet in PUCCH-Config, a PUCCH resource set is provided by pucch-ResourceCommon through an index to a row of Table 9.2.1-1 for transmission of HARQ-ACK information on PUCCH in an initial UL BWP of NBWPsize PRBs. For operation in FR2-2, nrofPRBs provided in PUCCH-ConfigCommon can also provide a number of NRB RBs for the PUCCH resource set; otherwise NRB=1.

The PUCCH resource set includes sixteen resources, each corresponding to a PUCCH format, a first symbol, a duration, a PRB offset RBBWPoffset, and a cyclic shift index set for a PUCCH transmission.

The UE transmits a PUCCH using frequency hopping if not provided useInterlacePUCCH-PUSCH in BWP-UplinkCommon; otherwise, the UE transmits a PUCCH without frequency hopping.

7.1.1.1.31.3	Test description

7.1.1.1.31.3.1	Pre-test conditions

System Simulator:

- NR Cell 1 as specified in TS 38.508-1 [4] Table 4.4.2.1.

- System information combination NR-29 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used.

UE:

-	The pre-configured UE location is defined in TS 38.508-1 [4] Clause 4.5C.

Preamble:

-	The UE is in 1N-A state on NR Cell 1 using generic procedure parameter Connectivity (NR) and Test loop function(Off) according to TS 38.508-1 [4].

7.1.1.1.31.3.2	Test procedure sequence

Table 7.1.1.1.31.3.2-1 illustrate the downlink power levels and other changing parameters to be applied for the cell at

various time instants of the test execution. The exact instants on which these values shall be applied are described in the

texts in this clause. Configurations marked "T0" is applied for Preamble. Configurations marked "T1" and "T2" are

applied at the points indicated in the Main behaviour description in Table 7.1.1.1.31.3.2-2.

Table 7.1.1.1.31.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | NR Cell 1 |
| --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -85 |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -95 |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -85 |

Table 7.1.1.1.31.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes the SIB19 of NR Cell 1 to configure the specific Msg4 PUCCH configuration | - | - | - | - |
| 2 | The SS transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification. | <-- | PDCCH (DCI 1_0): Short Message | - | - |
| 3 | Wait for 2.1* modification period second for the UE to receive new system information. (Note 1) | - | - | - | - |
| 4 | The SS transmits a Paging message including a matched identity. | <-- | NR RRC: Paging | - | - |
| 5 | UE transmits Preamble on PRACH on NR Cell 1? | --> | PRACH Preamble | - | - |
| 6 | The SS transmits Random Access Response | <-- | Random Access Response | - |  |
| 7 | Check: Does the UE transmit a MAC PDU containing an RRCSetupRequest message by setting LCID to 0 and setting LX to 1 and uses code point corresponding to PUCCH repetition of Msg4 HARQ-ACK? | --> | MAC PDU (RRCSetupRequest) | 1 | P |
| 8 | The SS transmits an RRCSetup message with the number of repetitions for Msg4 HARK-ACK in Msg4 PDCCH | <-- | NR RRC: RRCSetup | - | - |
| 9 | Check: Does the UE transmit an HARQ ACK on PUCCH with the repetition number configured with numberOfMsg4HARQ-ACK-Repetitions-r18? | --> | HARQ ACK (n) | 1 | P |
| 10 | The UE transmits RRCSetupComplete message | --> | NRRRC : RRCSetupComplete | - | - |
| 11-14 | The test steps 5 to 8 of generic test procedure in TS 38.508-1 [4] Table 4.5.4.2-3 are performed on NR Cell 1 | - | - | - | - |
| 15 | The SS transmits RRCRelease. | <-- | NR RRC: RRC Release | - | - |
| 16 | The SS changes the SIB19 of NR Cell 1 to configure the specific Msg4 PUCCH configuration | - | - | - | - |
| 17 | The SS adjusts the SS/PBCH EPRE according to row "T1" in table 7.1.1.1.31.3.2-1 | - | - | - | - |
| 18 | The SS transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification. | <-- | PDCCH (DCI 1_0): Short Message | - | - |
| 19 | Wait for 2.1* modification period second for the UE to receive new system information. (Note 1) | - | - | - | - |
| 20 | The SS transmits a Paging message including a matched identity. | <-- | NR RRC: Paging | - | - |
| 21 | UE transmits Preamble on PRACH on NR Cell 1? | --> | PRACH Preamble | - | - |
| 22 | The SS transmits Random Access Response | <-- | Random Access Response | - |  |
| 23 | Check: Does the UE transmit a MAC PDU containing an RRCSetupRequest message by setting LCID to 0 and setting LX to 1 and uses code point corresponding to PUCCH repetition of Msg4 HARQ-ACK? | --> | MAC PDU (RRCSetupRequest) | 2 | P |
| 24 | The SS transmits an RRCSetup message with the number of repetitions for Msg4 HARK-ACK in Msg4 PDCCH | <-- | NR RRC: RRCSetup | - | - |
| 25 | Check: Does the UE transmit an HARQ ACK on PUCCH with the repetition number configured with numberOfMsg4HARQ-ACK-Repetitions-r18? | --> | HARQ ACK (n) | 2 | P |
| 26 | The UE transmits RRCSetupComplete message | --> | NRRRC : RRCSetupComplete | - | - |
| 27-30 | The test steps 5 to 8 of generic test procedure in TS 38.508-1 [4] Table 4.5.4.2-3 are performed on NR Cell 1 | - | - | - | - |
| 31 | The SS transmits RRCRelease. | <-- | NR RRC: RRC Release | - | - |
| 33 | The SS adjusts the SS/PBCH EPRE according to row "T2" in table 7.1.1.1.31.3.2-1 | - | - | - | - |
| 33 | The SS transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification. | <-- | PDCCH (DCI 1_0): Short Message | - | - |
| 35 | The SS transmits a Paging message including a matched identity. | <-- | NR RRC: Paging | - | - |
| 36 | UE transmits Preamble on PRACH on NR Cell 1? | --> | PRACH Preamble | - | - |
| 37 | The SS transmits Random Access Response | <-- | Random Access Response | - | - |
| 38 | UE transmit a MAC PDU containing an RRCSetupRequest message by setting LCID to 0 and setting LX to 1 and uses code point corresponding to PUCCH repetition of Msg4 HARQ-ACK | --> | MAC PDU (RRCSetupRequest) | - | - |
| 39 | The SS transmits an RRCSetup message with the number of repetitions for Msg4 HARK-ACK in Msg4 PDCCH | <-- | NR RRC: RRCSetup | - | - |
| 40 | Check: Does the UE transmit an HARQ ACK on PUCCH with the repetition number configured with numberOfMsg4HARQ-ACK-Repetitions-r18? | --> | HARQ ACK (n) | 2 | F |
| 41 | The UE transmits RRCSetupComplete message | --> | NRRRC : RRCSetupComplete | - | - |
| 42-45 | The test steps 5 to 8 of generic test procedure in TS 38.508-1 [4] Table 4.5.4.2-3 are performed on NR Cell 1 | - | - | - | - |
| 46 | The SS transmits RRCRelease. | <-- | NR RRC: RRC Release | - | - |
| Note 1: The modification period, expressed in number of radio frames = modificationPeriodCoeff * defaultPagingCycle. |  |  |  |  |  |

7.1.1.1.31.3.3	Specific message contents

Table 7.1.1.1.31.3.3-1: SIB19 (Step1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.2-18C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| ntnCovEnh-r18 SEQUENCE { |  |  |  |
| numberOfMsg4HARQ-ACK-Repetitions-r18 | ‘0010 | Corresponds to repetition factor 2 |  |
| rsrp-ThresholdMsg4HARQ-ACK-r18 } | Not Present |  |  |
| } |  |  |  |

Table 7.1.1.1.31.3.3-2: SIB19 (Step16)

| Derivation Path: TS 38.508-1 [4], Table 4.6.2-18C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| ntnCovEnh-r18 SEQUENCE { |  |  |  |
| numberOfMsg4HARQ-ACK-Repetitions-r18 | ‘0010 | Corresponds to repetition factor 2 |  |
| rsrp-ThresholdMsg4HARQ-ACK-r18 } | 61 | RSRP of 95dBm |  |
| } |  |  |  |