3GPP TSG-RAN5 Meeting #107	R5-252809

# St Julian, Malta, 19th – 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-2 | CR | 0374 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add applicability for new test cases 17.19 and 17.20 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To add applicability for test cases 17.19 and 17.20. MO Speech / add RTT and remove RTT / EVS / Default configuration and MT Speech / add RTT and remove RTT / EVS / Default configuration. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The applicability has been added for test cases 17.19 and 17.20. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test coverage for RTT according to EU directive 2019/882 remains incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 34.229-1 CR 1608, CR 1609 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of first modified section>

## 4.1	Applicabilities for test cases specified in TS 34.229-1

Table 4.1: Applicability of TS 34.229-1 tests

<<<Skipped part >>>

| 17 | Media use cases |  |  |  |
| --- | --- | --- | --- | --- |
| 17.1 | MO Speech, add video remove video | Rel-8 | C71 | UE supports MTSI and initiating a session and MTSI speech and MTSI video and MTSI video H.264 CBP Level 1.2 and preconditions and E-UTRA and not UE category M1 |
| 17.2 | MT Speech, add video remove video | Rel-8 | C70 | UE supports MTSI and MTSI speech and MTSI video and MTSI video H.264 CBP Level 1.2 and preconditions and E-UTRA and not UE category M1 |
| 17.4 | Void |  |  |  |
| 17.5 | Void |  |  |  |
| 17.6 | Void |  |  |  |
| 17.8 | Void |  |  |  |
| 17.10 | Void |  |  |  |
| 17.12 | Void |  |  |  |
| 17.14 | Void |  |  |  |
| 17.16 | Void |  |  |  |
| 17.17 | Void |  |  |  |
| 17.18 | Void |  |  |  |
| 17.19 | MO Speech / add RTT and remove RTT / EVS / Default configuration | Rel-15 | Cnnn | UE supports MTSI and MTSI speech and preconditions and Real-time text (NOTE n) |
| 17.20 | MT Speech / add RTT and remove RTT / EVS / Default configuration | Rel-15 | Cnnn | UE supports MTSI and MTSI speech and preconditions and Real-time text (NOTE n) |

<<<Skipped part >>>

NOTE 1:	Applicability of test cases 13.2 and 13.3 are currently marked as FFS. The reason to this is that the contents of the specific messages sent by the SS (as currently specified within those Call Control test cases) do not match the contents of those messages as expected by any specific IMS application known. Further on the test specification apparently lacks support for certain application specific message exchanges which are however mandatory for a few specific IMS applications specified outside of TS 24.229. It is necessary to fully resolve the problem (by e.g. defining the applications for which the Call Control test cases would be applicable, possibly specifying the extensions to the test cases like required by those applications and creating the corresponding application profiles) before the applicability statements of Call Control test cases can be unambiguously defined.

NOTE 2:	Either one of the two adjacent test cases, i.e. (19.1.3 or 19.1.3a), (19.3.2 or 19.3.2a), shall be executed.

NOTE 3:	This TC can optionally be executed against Rel-8 and onwards UEs.

NOTE 4:	In case the UE supports both GBA and HTTP Digest, GBA shall be used as XCAP authentication.

NOTE 5:	In case the UE supports both GBA and HTTP Digest, HTTP Digest shall be used as XCAP authentication.

NOTE 6:	The TC contains multi-RAT branches not all mandatory in the scope of the TC. The E-UTRA/EPC branch will be executed always; the TC will go through any other RAT branch depending on the UE capability. Execution only of the E-UTRA/EPC branch regardless of the UE capabilities can also be imposed by setting the IXIT px_RATComb_Tested = EUTRA_only. For UEs supporting both UTRA and GERAN the TC should be executed once only for the E-UTRA/EPC and UTRA combination by setting the px_RATComb_Tested = EUTRA_UTRA.

NOTE 7:	This TC can optionally be executed against Rel-9 and onwards UEs.

NOTE 8:	If TC 12.23a is executed TC 12.23 is optional. If TC 12.24a is executed TC 12.24 is optional.

NOTE n:	This TC can optionally be executed against Rel-12 and onwards UEs.

<<<Skipped part >>>

Table 4.1a: Applicability Conditions for TS 34.229-1 tests

<<<Skipped part >>>

| C176 | IF A.12/54 AND [8] A.10/16 AND [8] A.10/19 AND A.4.5/67 THEN R ELSE N/A | IMS eCall type of emergency services over EPS and Manual type of eCall initiation and capable of triggering a Test eCall and subjected to national regulations in CEN EN 17184:2024 [100] for IMS eCall over EPS |
| --- | --- | --- |
| C177 | IF A.12/55 AND [8] A.10/16 AND [8] A.10/19 AND A.4.5/67 THEN R ELSE N/A | IMS eCall Only type of emergency services over EPS and Manual type of eCall initiation and capable of triggering a Test eCall and subjected to national regulations in CEN EN 17184:2024 [100] for IMS eCall over EPS |
| Cnnn | IF A.3A/50 AND A.15/1 AND A.4/16 AND A.15/15 THEN R ELSE N/A | MTSI and MTSI speech and preconditions and Real-time text |

<End of modified section>