3GPP TSG-RAN5 Meeting #107	R5-252019

# St. Julian, Malta, 19th – 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-4 | CR | 0963 | rev | - | Current version: | 18.6.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update for TDD RMCs for sustained data rate testing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-04-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For TDD 70 MHz bandwidth the sustained data rate RMC is missing and needs to be added to test all devices properly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add missing TDD RMC for 70 MHz channel bandwidth. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Required RMCs remain missing in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex A.3.2_1.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### < Unchanged sections omitted >

#### A.3.2_1.2.1	Reference measurement channels for SCS 30 kHz FR1

Table A.3.2_1.2.1-1: Sustained Downlink Data Rate Reference Channel for TDD 30kHz SCS FR1 (64QAM)

| Parameter | Channel bandwidth | Subcarrier spacing | Allocated resource blocks | Number of consecutive PDSCH symbols for allocated full DL slots (Note 1) | MCS Index (Note 2) | Modulation | Target Coding Rate | Number of MIMO layers | LDPC Base Graph | Information Bit Payload per Slot for allocated full DL slots (Note 1) | Transport block CRC per Slot for allocated full DL slots (Note 1) | Number of Code Blocks per Slot for allocated full DL slots (Note 1, 6) | Binary Channel Bits per Slot for allocated full DL slots (Note 1) | Max. Throughput averaged over 2 frames |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MHz | kHz | PRBs | Symbols |  |  |  |  |  | Bits | Bits | CBs | Bits | Mbps |
|  | 5 | 30 | 11 | 13 | 18 | 64QAM | 0.46 | 1 | 1 | 4352 | 24 | 1 | 9504 | 5.44 |
|  | 10 | 30 | 24 | 13 | 18 | 64QAM | 0.46 | 1 | 1 | 9480 | 24 | 2 | 20736 | 11.85 |
|  | 15 | 30 | 38 | 13 | 18 | 64QAM | 0.46 | 1 | 1 | 14856 | 24 | 2 | 32832 | 18.57 |
|  | 20 | 30 | 51 | 13 | 18 | 64QAM | 0.46 | 1 | 1 | 19968 | 24 | 3 | 44064 | 24.96 |
|  | 25 | 30 | 65 | 13 | 18 | 64QAM | 0.46 | 1 | 1 | 25608 | 24 | 4 | 56160 | 32.01 |
|  | 30 | 30 | 78 | 13 | 18 | 64QAM | 0.46 | 1 | 1 | 30728 | 24 | 4 | 67392 | 38.41 |
|  | 35 | 30 | 92 | 13 | 18 | 64QAM | 0.46 | 1 | 1 | 35856 | 24 | 5 | 79488 | 44.82 |
|  | 40 | 30 | 106 | 13 | 18 | 64QAM | 0.46 | 1 | 1 | 42016 | 24 | 5 | 91584 | 52.52 |
|  | 45 | 30 | 119 | 13 | 18 | 64QAM | 0.46 | 1 | 1 | 47112 | 24 | 6 | 102816 | 58.89 |
|  | 50 | 30 | 133 | 13 | 18 | 64QAM | 0.46 | 1 | 1 | 52224 | 24 | 7 | 114912 | 65.28 |
|  | 60 | 30 | 162 | 13 | 18 | 64QAM | 0.46 | 1 | 1 | 63528 | 24 | 8 | 139968 | 79.41 |
|  | 70 | 30 | 189 | 13 | 18 | 64QAM | 0.46 | 1 | 1 | 73776 | 24 | 9 | 163296 | 92.22 |
|  | 80 | 30 | 217 | 13 | 18 | 64QAM | 0.46 | 1 | 1 | 86040 | 24 | 11 | 187488 | 107.55 |
|  | 90 | 30 | 245 | 13 | 18 | 64QAM | 0.46 | 1 | 1 | 96264 | 24 | 12 | 211680 | 120.33 |
|  | 100 | 30 | 273 | 13 | 18 | 64QAM | 0.46 | 1 | 1 | 106576 | 24 | 13 | 235872 | 133.22 |
|  | 5 | 30 | 11 | 13 | 22 | 64QAM | 0.65 | 1 | 1 | 6144 | 24 | 1 | 9504 | 7.68 |
|  | 10 | 30 | 24 | 13 | 22 | 64QAM | 0.65 | 1 | 1 | 13576 | 24 | 2 | 20736 | 16.97 |
|  | 15 | 30 | 38 | 13 | 22 | 64QAM | 0.65 | 1 | 1 | 21504 | 24 | 3 | 32832 | 26.88 |
|  | 20 | 30 | 51 | 13 | 22 | 64QAM | 0.65 | 1 | 1 | 28680 | 24 | 4 | 44064 | 35.85 |
|  | 25 | 30 | 65 | 13 | 22 | 64QAM | 0.65 | 1 | 1 | 36896 | 24 | 5 | 56160 | 46.12 |
|  | 30 | 30 | 78 | 13 | 22 | 64QAM | 0.65 | 1 | 1 | 44040 | 24 | 6 | 67392 | 55.05 |
|  | 35 | 30 | 92 | 13 | 22 | 64QAM | 0.65 | 1 | 1 | 51216 | 24 | 7 | 79488 | 64.02 |
|  | 40 | 30 | 106 | 13 | 22 | 64QAM | 0.65 | 1 | 1 | 59432 | 24 | 8 | 91584 | 74.29 |
|  | 45 | 30 | 119 | 13 | 22 | 64QAM | 0.65 | 1 | 1 | 67584 | 24 | 9 | 102816 | 84.48 |
|  | 50 | 30 | 133 | 13 | 22 | 64QAM | 0.65 | 1 | 1 | 73776 | 24 | 9 | 114912 | 92.22 |
|  | 60 | 30 | 162 | 13 | 22 | 64QAM | 0.65 | 1 | 1 | 90176 | 24 | 11 | 139968 | 112.72 |
|  | 70 | 30 | 189 | 13 | 22 | 64QAM | 0.65 | 1 | 1 | 106576 | 24 | 13 | 163296 | 133.22 |
|  | 80 | 30 | 217 | 13 | 22 | 64QAM | 0.65 | 1 | 1 | 122976 | 24 | 15 | 187488 | 153.72 |
|  | 90 | 30 | 245 | 13 | 22 | 64QAM | 0.65 | 1 | 1 | 139376 | 24 | 17 | 211680 | 174.22 |
|  | 100 | 30 | 273 | 13 | 22 | 64QAM | 0.65 | 1 | 1 | 151608 | 24 | 18 | 235872 | 189.51 |
|  | 5 | 30 | 11 | 13 | 23 | 64QAM | 0.70 | 1 | 1 | 6656 | 24 | 1 | 9504 | 8.32 |
|  | 10 | 30 | 24 | 13 | 23 | 64QAM | 0.70 | 1 | 1 | 14600 | 24 | 2 | 20736 | 18.25 |
|  | 15 | 30 | 38 | 13 | 23 | 64QAM | 0.70 | 1 | 1 | 23040 | 24 | 3 | 32832 | 28.8 |
|  | 20 | 30 | 51 | 13 | 23 | 64QAM | 0.70 | 1 | 1 | 30728 | 24 | 4 | 44064 | 38.41 |
|  | 25 | 30 | 65 | 13 | 23 | 64QAM | 0.70 | 1 | 1 | 38936 | 24 | 5 | 56160 | 48.67 |
|  | 30 | 30 | 78 | 13 | 23 | 64QAM | 0.70 | 1 | 1 | 47112 | 24 | 6 | 67392 | 58.89 |
|  | 35 | 30 | 92 | 13 | 23 | 64QAM | 0.70 | 1 | 1 | 55304 | 24 | 7 | 79488 | 69.13 |
|  | 40 | 30 | 106 | 13 | 23 | 64QAM | 0.70 | 1 | 1 | 64552 | 24 | 8 | 91584 | 80.69 |
|  | 45 | 30 | 119 | 13 | 23 | 64QAM | 0.70 | 1 | 1 | 71688 | 24 | 9 | 102816 | 89.61 |
|  | 50 | 30 | 133 | 13 | 23 | 64QAM | 0.70 | 1 | 1 | 79896 | 24 | 10 | 114912 | 99.87 |
|  | 60 | 30 | 162 | 13 | 23 | 64QAM | 0.70 | 1 | 1 | 98376 | 24 | 12 | 139968 | 122.97 |
|  | 70 | 30 | 189 | 13 | 23 | 64QAM | 0.70 | 1 | 1 | 114776 | 24 | 14 | 163296 | 143.47 |
|  | 80 | 30 | 217 | 13 | 23 | 64QAM | 0.70 | 1 | 1 | 131176 | 24 | 16 | 187488 | 163.97 |
|  | 90 | 30 | 245 | 13 | 23 | 64QAM | 0.70 | 1 | 1 | 147576 | 24 | 18 | 211680 | 184.47 |
|  | 100 | 30 | 273 | 13 | 23 | 64QAM | 0.70 | 1 | 1 | 163976 | 24 | 20 | 235872 | 204.97 |
|  | 5 | 30 | 11 | 13 | 27 | 64QAM | 0.89 | 1 | 1 | 8456 | 24 | 2 | 9504 | 10.57 |
|  | 10 | 30 | 24 | 13 | 27 | 64QAM | 0.89 | 1 | 1 | 18432 | 24 | 3 | 20736 | 23.04 |
|  | 15 | 30 | 38 | 13 | 27 | 64QAM | 0.89 | 1 | 1 | 29192 | 24 | 4 | 32832 | 36.49 |
|  | 20 | 30 | 51 | 13 | 27 | 64QAM | 0.89 | 1 | 1 | 38936 | 24 | 5 | 44064 | 48.67 |
|  | 25 | 30 | 65 | 13 | 27 | 64QAM | 0.89 | 1 | 1 | 50184 | 24 | 6 | 56160 | 62.73 |
|  | 30 | 30 | 78 | 13 | 27 | 64QAM | 0.89 | 1 | 1 | 59432 | 24 | 8 | 67392 | 74.29 |
|  | 35 | 30 | 92 | 13 | 27 | 64QAM | 0.89 | 1 | 1 | 69672 | 24 | 9 | 79488 | 87.09 |
|  | 40 | 30 | 106 | 13 | 27 | 64QAM | 0.89 | 1 | 1 | 81976 | 24 | 10 | 91584 | 102.47 |
|  | 45 | 30 | 119 | 13 | 27 | 64QAM | 0.89 | 1 | 1 | 92200 | 24 | 11 | 102816 | 115.25 |
|  | 50 | 30 | 133 | 13 | 27 | 64QAM | 0.89 | 1 | 1 | 102416 | 24 | 13 | 114912 | 128.02 |
|  | 60 | 30 | 162 | 13 | 27 | 64QAM | 0.89 | 1 | 1 | 125016 | 24 | 15 | 139968 | 156.27 |
|  | 70 | 30 | 189 | 13 | 27 | 64QAM | 0.89 | 1 | 1 | 143400 | 24 | 18 | 163296 | 179.25 |
|  | 80 | 30 | 217 | 13 | 27 | 64QAM | 0.89 | 1 | 1 | 167976 | 24 | 20 | 187488 | 209.97 |
|  | 90 | 30 | 245 | 13 | 27 | 64QAM | 0.89 | 1 | 1 | 188576 | 24 | 23 | 211680 | 235.72 |
|  | 100 | 30 | 273 | 13 | 27 | 64QAM | 0.89 | 1 | 1 | 208976 | 24 | 25 | 235872 | 261.22 |
|  | 5 | 30 | 11 | 13 | 18 | 64QAM | 0.46 | 2 | 1 | 8712 | 24 | 2 | 19008 | 10.89 |
|  | 10 | 30 | 24 | 13 | 18 | 64QAM | 0.46 | 2 | 1 | 18960 | 24 | 3 | 41472 | 23.7 |
|  | 15 | 30 | 38 | 13 | 18 | 64QAM | 0.46 | 2 | 1 | 29704 | 24 | 4 | 65664 | 37.13 |
|  | 20 | 30 | 51 | 13 | 18 | 64QAM | 0.46 | 2 | 1 | 39936 | 24 | 5 | 88128 | 49.92 |
|  | 25 | 30 | 65 | 13 | 18 | 64QAM | 0.46 | 2 | 1 | 51216 | 24 | 7 | 112320 | 64.02 |
|  | 30 | 30 | 78 | 13 | 18 | 64QAM | 0.46 | 2 | 1 | 61480 | 24 | 8 | 134784 | 76.85 |
|  | 35 | 30 | 92 | 13 | 18 | 64QAM | 0.46 | 2 | 1 | 71688 | 24 | 9 | 158976 | 89.61 |
|  | 40 | 30 | 106 | 13 | 18 | 64QAM | 0.46 | 2 | 1 | 83976 | 24 | 10 | 183168 | 104.97 |
|  | 45 | 30 | 119 | 13 | 18 | 64QAM | 0.46 | 2 | 1 | 94248 | 24 | 12 | 205632 | 117.81 |
|  | 50 | 30 | 133 | 13 | 18 | 64QAM | 0.46 | 2 | 1 | 104496 | 24 | 13 | 229824 | 130.62 |
|  | 60 | 30 | 162 | 13 | 18 | 64QAM | 0.46 | 2 | 1 | 127080 | 24 | 16 | 279936 | 158.85 |
|  | 70 | 30 | 189 | 13 | 18 | 64QAM | 0.46 | 2 | 1 | 147576 | 24 | 18 | 326592 | 184.47 |
|  | 80 | 30 | 217 | 13 | 18 | 64QAM | 0.46 | 2 | 1 | 172176 | 24 | 21 | 374976 | 215.22 |
|  | 90 | 30 | 245 | 13 | 18 | 64QAM | 0.46 | 2 | 1 | 192624 | 24 | 23 | 423360 | 240.78 |
|  | 100 | 30 | 273 | 13 | 18 | 64QAM | 0.46 | 2 | 1 | 213176 | 24 | 26 | 471744 | 266.47 |
|  | 5 | 30 | 11 | 13 | 22 | 64QAM | 0.65 | 2 | 1 | 12296 | 24 | 2 | 19008 | 15.37 |
|  | 10 | 30 | 24 | 13 | 22 | 64QAM | 0.65 | 2 | 1 | 27144 | 24 | 4 | 41472 | 33.93 |
|  | 15 | 30 | 38 | 13 | 22 | 64QAM | 0.65 | 2 | 1 | 43032 | 24 | 6 | 65664 | 53.79 |
|  | 20 | 30 | 51 | 13 | 22 | 64QAM | 0.65 | 2 | 1 | 57376 | 24 | 7 | 88128 | 71.72 |
|  | 25 | 30 | 65 | 13 | 22 | 64QAM | 0.65 | 2 | 1 | 73776 | 24 | 9 | 112320 | 92.22 |
|  | 30 | 30 | 78 | 13 | 22 | 64QAM | 0.65 | 2 | 1 | 88064 | 24 | 11 | 134784 | 110.08 |
|  | 35 | 30 | 92 | 13 | 22 | 64QAM | 0.65 | 2 | 1 | 102416 | 24 | 13 | 158976 | 128.02 |
|  | 40 | 30 | 106 | 13 | 22 | 64QAM | 0.65 | 2 | 1 | 118896 | 24 | 15 | 183168 | 148.62 |
|  | 45 | 30 | 119 | 13 | 22 | 64QAM | 0.65 | 2 | 1 | 135296 | 24 | 17 | 205632 | 169.12 |
|  | 50 | 30 | 133 | 13 | 22 | 64QAM | 0.65 | 2 | 1 | 147576 | 24 | 18 | 229824 | 184.47 |
|  | 60 | 30 | 162 | 13 | 22 | 64QAM | 0.65 | 2 | 1 | 180376 | 24 | 22 | 279936 | 225.47 |
|  | 70 | 30 | 189 | 13 | 22 | 64QAM | 0.65 | 2 | 1 | 213176 | 24 | 26 | 326592 | 266.47 |
|  | 80 | 30 | 217 | 13 | 22 | 64QAM | 0.65 | 2 | 1 | 245976 | 24 | 30 | 374976 | 307.47 |
|  | 90 | 30 | 245 | 13 | 22 | 64QAM | 0.65 | 2 | 1 | 278776 | 24 | 34 | 423360 | 348.47 |
|  | 100 | 30 | 273 | 13 | 22 | 64QAM | 0.65 | 2 | 1 | 303240 | 24 | 36 | 471744 | 379.05 |
|  | 5 | 30 | 11 | 13 | 23 | 64QAM | 0.70 | 2 | 1 | 13320 | 24 | 2 | 19008 | 16.65 |
|  | 10 | 30 | 24 | 13 | 23 | 64QAM | 0.70 | 2 | 1 | 29192 | 24 | 4 | 41472 | 36.49 |
|  | 15 | 30 | 38 | 13 | 23 | 64QAM | 0.70 | 2 | 1 | 46104 | 24 | 6 | 65664 | 57.63 |
|  | 20 | 30 | 51 | 13 | 23 | 64QAM | 0.70 | 2 | 1 | 61480 | 24 | 8 | 88128 | 76.85 |
|  | 25 | 30 | 65 | 13 | 23 | 64QAM | 0.70 | 2 | 1 | 77896 | 24 | 10 | 112320 | 97.37 |
|  | 30 | 30 | 78 | 13 | 23 | 64QAM | 0.70 | 2 | 1 | 94248 | 24 | 12 | 134784 | 117.81 |
|  | 35 | 30 | 92 | 13 | 23 | 64QAM | 0.70 | 2 | 1 | 110632 | 24 | 14 | 158976 | 138.29 |
|  | 40 | 30 | 106 | 13 | 23 | 64QAM | 0.70 | 2 | 1 | 129128 | 24 | 16 | 183168 | 161.41 |
|  | 45 | 30 | 119 | 13 | 23 | 64QAM | 0.70 | 2 | 1 | 143400 | 24 | 18 | 205632 | 179.25 |
|  | 50 | 30 | 133 | 13 | 23 | 64QAM | 0.70 | 2 | 1 | 159880 | 24 | 19 | 229824 | 199.85 |
|  | 60 | 30 | 162 | 13 | 23 | 64QAM | 0.70 | 2 | 1 | 196776 | 24 | 24 | 279936 | 245.97 |
|  | 70 | 30 | 189 | 13 | 23 | 64QAM | 0.70 | 2 | 1 | 229576 | 24 | 28 | 326592 | 286.97 |
|  | 80 | 30 | 217 | 13 | 23 | 64QAM | 0.70 | 2 | 1 | 262376 | 24 | 32 | 374976 | 327.97 |
|  | 90 | 30 | 245 | 13 | 23 | 64QAM | 0.70 | 2 | 1 | 295176 | 24 | 36 | 423360 | 368.97 |
|  | 100 | 30 | 273 | 13 | 23 | 64QAM | 0.70 | 2 | 1 | 327888 | 24 | 39 | 471744 | 409.86 |
|  | 5 | 30 | 11 | 13 | 27 | 64QAM | 0.89 | 2 | 1 | 16896 | 24 | 3 | 19008 | 21.12 |
|  | 10 | 30 | 24 | 13 | 27 | 64QAM | 0.89 | 2 | 1 | 36896 | 24 | 5 | 41472 | 46.12 |
|  | 15 | 30 | 38 | 13 | 27 | 64QAM | 0.89 | 2 | 1 | 58384 | 24 | 7 | 65664 | 72.98 |
|  | 20 | 30 | 51 | 13 | 27 | 64QAM | 0.89 | 2 | 1 | 77896 | 24 | 10 | 88128 | 97.37 |
|  | 25 | 30 | 65 | 13 | 27 | 64QAM | 0.89 | 2 | 1 | 100392 | 24 | 12 | 112320 | 125.49 |
|  | 30 | 30 | 78 | 13 | 27 | 64QAM | 0.89 | 2 | 1 | 118896 | 24 | 15 | 134784 | 148.62 |
|  | 35 | 30 | 92 | 13 | 27 | 64QAM | 0.89 | 2 | 1 | 139376 | 24 | 17 | 158976 | 174.22 |
|  | 40 | 30 | 106 | 13 | 27 | 64QAM | 0.89 | 2 | 1 | 163976 | 24 | 20 | 183168 | 204.97 |
|  | 45 | 30 | 119 | 13 | 27 | 64QAM | 0.89 | 2 | 1 | 184424 | 24 | 22 | 205632 | 230.53 |
|  | 50 | 30 | 133 | 13 | 27 | 64QAM | 0.89 | 2 | 1 | 204976 | 24 | 25 | 229824 | 256.22 |
|  | 60 | 30 | 162 | 13 | 27 | 64QAM | 0.89 | 2 | 1 | 250056 | 24 | 30 | 279936 | 312.57 |
|  | 70 | 30 | 189 | 13 | 27 | 64QAM | 0.89 | 2 | 1 | 286976 | 24 | 35 | 326592 | 358.72 |
|  | 80 | 30 | 217 | 13 | 27 | 64QAM | 0.89 | 2 | 1 | 335976 | 24 | 40 | 374976 | 419.97 |
|  | 90 | 30 | 245 | 13 | 27 | 64QAM | 0.89 | 2 | 1 | 376896 | 24 | 45 | 423360 | 471.12 |
|  | 100 | 30 | 273 | 13 | 27 | 64QAM | 0.89 | 2 | 1 | 417976 | 24 | 50 | 471744 | 522.47 |
|  | 5 | 30 | 11 | 13 | 19 | 64QAM | 0.50 | 4 | 1 | 17424 | 24 | 3 | 34848 | 21.78 |
|  | 10 | 30 | 24 | 13 | 19 | 64QAM | 0.50 | 4 | 1 | 37896 | 24 | 5 | 76032 | 47.37 |
|  | 15 | 30 | 38 | 13 | 19 | 64QAM | 0.50 | 4 | 1 | 60456 | 24 | 8 | 120384 | 75.57 |
|  | 20 | 30 | 51 | 13 | 19 | 64QAM | 0.50 | 4 | 1 | 81976 | 24 | 10 | 161568 | 102.47 |
|  | 25 | 30 | 65 | 13 | 19 | 64QAM | 0.50 | 4 | 1 | 104496 | 24 | 13 | 205920 | 130.62 |
|  | 30 | 30 | 78 | 13 | 19 | 64QAM | 0.50 | 4 | 1 | 125016 | 24 | 15 | 247104 | 156.27 |
|  | 35 | 30 | 92 | 13 | 19 | 64QAM | 0.50 | 4 | 1 | 147576 | 24 | 18 | 291456 | 184.47 |
|  | 40 | 30 | 106 | 13 | 19 | 64QAM | 0.50 | 4 | 1 | 167976 | 24 | 20 | 335808 | 209.97 |
|  | 45 | 30 | 119 | 13 | 19 | 64QAM | 0.50 | 4 | 1 | 188576 | 24 | 23 | 376992 | 235.72 |
|  | 50 | 30 | 133 | 13 | 19 | 64QAM | 0.50 | 4 | 1 | 213176 | 24 | 26 | 421344 | 266.47 |
|  | 60 | 30 | 162 | 13 | 19 | 64QAM | 0.50 | 4 | 1 | 258144 | 24 | 31 | 513216 | 322.68 |
|  | 70 | 30 | 189 | 13 | 19 | 64QAM | 0.50 | 4 | 1 | 303240 | 24 | 36 | 598752 | 379.05 |
|  | 80 | 30 | 217 | 13 | 19 | 64QAM | 0.50 | 4 | 1 | 344376 | 24 | 41 | 687456 | 430.47 |
|  | 90 | 30 | 245 | 13 | 19 | 64QAM | 0.50 | 4 | 1 | 393272 | 24 | 47 | 776160 | 491.59 |
|  | 100 | 30 | 273 | 13 | 19 | 64QAM | 0.50 | 4 | 1 | 434280 | 24 | 52 | 864864 | 542.85 |
|  | 5 | 30 | 11 | 13 | 23 | 64QAM | 0.70 | 4 | 1 | 24576 | 24 | 3 | 34848 | 30.72 |
|  | 10 | 30 | 24 | 13 | 23 | 64QAM | 0.70 | 4 | 1 | 53288 | 24 | 7 | 76032 | 66.61 |
|  | 15 | 30 | 38 | 13 | 23 | 64QAM | 0.70 | 4 | 1 | 83976 | 24 | 10 | 120384 | 104.97 |
|  | 20 | 30 | 51 | 13 | 23 | 64QAM | 0.70 | 4 | 1 | 112648 | 24 | 14 | 161568 | 140.81 |
|  | 25 | 30 | 65 | 13 | 23 | 64QAM | 0.70 | 4 | 1 | 143400 | 24 | 18 | 205920 | 179.25 |
|  | 30 | 30 | 78 | 13 | 23 | 64QAM | 0.70 | 4 | 1 | 172176 | 24 | 21 | 247104 | 215.22 |
|  | 35 | 30 | 92 | 13 | 23 | 64QAM | 0.70 | 4 | 1 | 204976 | 24 | 25 | 291456 | 256.22 |
|  | 40 | 30 | 106 | 13 | 23 | 64QAM | 0.70 | 4 | 1 | 237776 | 24 | 29 | 335808 | 297.22 |
|  | 45 | 30 | 119 | 13 | 23 | 64QAM | 0.70 | 4 | 1 | 262376 | 24 | 32 | 376992 | 327.97 |
|  | 50 | 30 | 133 | 13 | 23 | 64QAM | 0.70 | 4 | 1 | 295176 | 24 | 36 | 421344 | 368.97 |
|  | 60 | 30 | 162 | 13 | 23 | 64QAM | 0.70 | 4 | 1 | 360488 | 24 | 43 | 513216 | 450.61 |
|  | 70 | 30 | 189 | 13 | 23 | 64QAM | 0.70 | 4 | 1 | 417976 | 24 | 50 | 598752 | 522.47 |
|  | 80 | 30 | 217 | 13 | 23 | 64QAM | 0.70 | 4 | 1 | 483464 | 24 | 58 | 687456 | 604.33 |
|  | 90 | 30 | 245 | 13 | 23 | 64QAM | 0.70 | 4 | 1 | 540776 | 24 | 65 | 776160 | 675.97 |
|  | 100 | 30 | 273 | 13 | 23 | 64QAM | 0.70 | 4 | 1 | 606504 | 24 | 72 | 864864 | 758.13 |
|  | 5 | 30 | 11 | 13 | 24 | 64QAM | 0.75 | 4 | 1 | 26120 | 24 | 4 | 34848 | 32.65 |
|  | 10 | 30 | 24 | 13 | 24 | 64QAM | 0.75 | 4 | 1 | 57376 | 24 | 7 | 76032 | 71.72 |
|  | 15 | 30 | 38 | 13 | 24 | 64QAM | 0.75 | 4 | 1 | 90176 | 24 | 11 | 120384 | 112.72 |
|  | 20 | 30 | 51 | 13 | 24 | 64QAM | 0.75 | 4 | 1 | 120936 | 24 | 15 | 161568 | 151.17 |
|  | 25 | 30 | 65 | 13 | 24 | 64QAM | 0.75 | 4 | 1 | 155776 | 24 | 19 | 205920 | 194.72 |
|  | 30 | 30 | 78 | 13 | 24 | 64QAM | 0.75 | 4 | 1 | 184424 | 24 | 22 | 247104 | 230.53 |
|  | 35 | 30 | 92 | 13 | 24 | 64QAM | 0.75 | 4 | 1 | 221376 | 24 | 27 | 291456 | 276.72 |
|  | 40 | 30 | 106 | 13 | 24 | 64QAM | 0.75 | 4 | 1 | 254176 | 24 | 31 | 335808 | 317.72 |
|  | 45 | 30 | 119 | 13 | 24 | 64QAM | 0.75 | 4 | 1 | 286976 | 24 | 35 | 376992 | 358.72 |
|  | 50 | 30 | 133 | 13 | 24 | 64QAM | 0.75 | 4 | 1 | 319784 | 24 | 38 | 421344 | 399.73 |
|  | 60 | 30 | 162 | 13 | 24 | 64QAM | 0.75 | 4 | 1 | 385272 | 24 | 46 | 513216 | 481.59 |
|  | 70 | 30 | 189 | 13 | 24 | 64QAM | 0.75 | 4 | 1 | 450984 | 24 | 54 | 598752 | 563.73 |
|  | 80 | 30 | 217 | 13 | 24 | 64QAM | 0.75 | 4 | 1 | 516312 | 24 | 62 | 687456 | 645.39 |
|  | 90 | 30 | 245 | 13 | 24 | 64QAM | 0.75 | 4 | 1 | 590128 | 24 | 71 | 776160 | 737.66 |
|  | 100 | 30 | 273 | 13 | 24 | 64QAM | 0.75 | 4 | 1 | 655800 | 24 | 78 | 864864 | 819.75 |
|  | 5 | 30 | 11 | 13 | 27 | 64QAM | 0.89 | 4 | 1 | 30728 | 24 | 4 | 34848 | 38.41 |
|  | 10 | 30 | 24 | 13 | 27 | 64QAM | 0.89 | 4 | 1 | 67584 | 24 | 9 | 76032 | 84.48 |
|  | 15 | 30 | 38 | 13 | 27 | 64QAM | 0.89 | 4 | 1 | 106576 | 24 | 13 | 120384 | 133.22 |
|  | 20 | 30 | 51 | 13 | 27 | 64QAM | 0.89 | 4 | 1 | 143400 | 24 | 18 | 161568 | 179.25 |
|  | 25 | 30 | 65 | 13 | 27 | 64QAM | 0.89 | 4 | 1 | 184424 | 24 | 22 | 205920 | 230.53 |
|  | 30 | 30 | 78 | 13 | 27 | 64QAM | 0.89 | 4 | 1 | 221376 | 24 | 27 | 247104 | 276.72 |
|  | 35 | 30 | 92 | 13 | 27 | 64QAM | 0.89 | 4 | 1 | 258144 | 24 | 31 | 291456 | 322.68 |
|  | 40 | 30 | 106 | 13 | 27 | 64QAM | 0.89 | 4 | 1 | 295176 | 24 | 36 | 335808 | 368.97 |
|  | 45 | 30 | 119 | 13 | 27 | 64QAM | 0.89 | 4 | 1 | 335976 | 24 | 40 | 376992 | 419.97 |
|  | 50 | 30 | 133 | 13 | 27 | 64QAM | 0.89 | 4 | 1 | 376896 | 24 | 45 | 421344 | 471.12 |
|  | 60 | 30 | 162 | 13 | 27 | 64QAM | 0.89 | 4 | 1 | 458896 | 24 | 55 | 513216 | 573.62 |
|  | 70 | 30 | 189 | 13 | 27 | 64QAM | 0.89 | 4 | 1 | 524640 | 24 | 63 | 598752 | 655.8 |
|  | 80 | 30 | 217 | 13 | 27 | 64QAM | 0.89 | 4 | 1 | 606504 | 24 | 72 | 687456 | 758.13 |
|  | 90 | 30 | 245 | 13 | 27 | 64QAM | 0.89 | 4 | 1 | 688776 | 24 | 82 | 776160 | 860.97 |
|  | 100 | 30 | 273 | 13 | 27 | 64QAM | 0.89 | 4 | 1 | 770568 | 24 | 92 | 864864 | 963.21 |
| Note 1: Allocated full DL slots are with slot index i, if mod(i,10) = 0,1,2,3,4,5,6 and i is not in {0,20,21} for i = 0,1,...,39. So total number of allocated slots per 2 frames is 25. Note 2: MCS Index is based on MCS Table defined in TS38.214 when 256QAM is not enabled. MCS 18 and 19 are equivalent to MCS 11 and 12 in 256QAM table, respectively. Note 3: Number of DMRS REs per RB = 12,12,24,24 for number of MIMO layers = 1,2,3,4, respectively Note 4: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms. Note 5: Overhead parameter for TBS determination is 0. Note 6: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.3.2_1.2.1-2: Sustained Downlink Data Rate Reference Channel for TDD 30kHz SCS FR1(256QAM)

| Parameter | Channel bandwidth | Subcarrier spacing | Allocated resource blocks | Number of consecutive PDSCH symbols for allocated full DL slots (Note 1) | MCS Index (Note 2) | Modulation | Target Coding Rate | Number of MIMO layers | LDPC Base Graph | Information Bit Payload per Slot for allocated full DL slots (Note 1) | Transport block CRC per Slot for allocated full DL slots (Note 1) | Number of Code Blocks per Slot for allocated full DL slots (Note 1, 6) | Binary Channel Bits per Slot for allocated full DL slots (Note 1) | Max. Throughput averaged over 2 frames |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MHz | kHz | PRBs | Symbols |  |  |  |  |  | Bits | Bits | CBs | Bits | Mbps |
|  | 5 | 30 | 11 | 13 | 20 | 256QAM | 0.67 | 1 | 1 | 8456 | 24 | 2 | 12672 | 10.57 |
|  | 10 | 30 | 24 | 13 | 20 | 256QAM | 0.67 | 1 | 1 | 18432 | 24 | 3 | 27648 | 23.04 |
|  | 15 | 30 | 38 | 13 | 20 | 256QAM | 0.67 | 1 | 1 | 29192 | 24 | 4 | 43776 | 36.49 |
|  | 20 | 30 | 51 | 13 | 20 | 256QAM | 0.67 | 1 | 1 | 38936 | 24 | 5 | 58752 | 48.67 |
|  | 25 | 30 | 65 | 13 | 20 | 256QAM | 0.67 | 1 | 1 | 50184 | 24 | 6 | 74880 | 62.73 |
|  | 30 | 30 | 78 | 13 | 20 | 256QAM | 0.67 | 1 | 1 | 59432 | 24 | 8 | 89856 | 74.29 |
|  | 35 | 30 | 92 | 13 | 20 | 256QAM | 0.67 | 1 | 1 | 69672 | 24 | 9 | 105984 | 87.09 |
|  | 40 | 30 | 106 | 13 | 20 | 256QAM | 0.67 | 1 | 1 | 81976 | 24 | 10 | 122112 | 102.47 |
|  | 45 | 30 | 119 | 13 | 20 | 256QAM | 0.67 | 1 | 1 | 92200 | 24 | 11 | 137088 | 115.25 |
|  | 50 | 30 | 133 | 13 | 20 | 256QAM | 0.67 | 1 | 1 | 102416 | 24 | 13 | 153216 | 128.02 |
|  | 60 | 30 | 162 | 13 | 20 | 256QAM | 0.67 | 1 | 1 | 125016 | 24 | 15 | 186624 | 156.27 |
|  | 70 | 30 | 189 | 13 | 20 | 256QAM | 0.67 | 1 | 1 | 143400 | 24 | 18 | 217728 | 179.25 |
|  | 80 | 30 | 217 | 13 | 20 | 256QAM | 0.67 | 1 | 1 | 167976 | 24 | 20 | 249984 | 209.97 |
|  | 90 | 30 | 245 | 13 | 20 | 256QAM | 0.67 | 1 | 1 | 188576 | 24 | 23 | 282240 | 235.72 |
|  | 100 | 30 | 273 | 13 | 20 | 256QAM | 0.67 | 1 | 1 | 208976 | 24 | 25 | 314496 | 261.22 |
|  | 5 | 30 | 11 | 13 | 21 | 256QAM | 0.69 | 1 | 1 | 8712 | 24 | 2 | 12672 | 10.89 |
|  | 10 | 30 | 24 | 13 | 21 | 256QAM | 0.69 | 1 | 1 | 18960 | 24 | 3 | 27648 | 23.7 |
|  | 15 | 30 | 38 | 13 | 21 | 256QAM | 0.69 | 1 | 1 | 30216 | 24 | 4 | 43776 | 37.77 |
|  | 20 | 30 | 51 | 13 | 21 | 256QAM | 0.69 | 1 | 1 | 40976 | 24 | 5 | 58752 | 51.22 |
|  | 25 | 30 | 65 | 13 | 21 | 256QAM | 0.69 | 1 | 1 | 52224 | 24 | 7 | 74880 | 65.28 |
|  | 30 | 30 | 78 | 13 | 21 | 256QAM | 0.69 | 1 | 1 | 62504 | 24 | 8 | 89856 | 78.13 |
|  | 35 | 30 | 92 | 13 | 21 | 256QAM | 0.69 | 1 | 1 | 73776 | 24 | 9 | 105984 | 92.22 |
|  | 40 | 30 | 106 | 13 | 21 | 256QAM | 0.69 | 1 | 1 | 83976 | 24 | 10 | 122112 | 104.97 |
|  | 45 | 30 | 119 | 13 | 21 | 256QAM | 0.69 | 1 | 1 | 94248 | 24 | 12 | 137088 | 117.81 |
|  | 50 | 30 | 133 | 13 | 21 | 256QAM | 0.69 | 1 | 1 | 106576 | 24 | 13 | 153216 | 133.22 |
|  | 60 | 30 | 162 | 13 | 21 | 256QAM | 0.69 | 1 | 1 | 129128 | 24 | 16 | 186624 | 161.41 |
|  | 70 | 30 | 189 | 13 | 21 | 256QAM | 0.69 | 1 | 1 | 151608 | 24 | 18 | 217728 | 189.51 |
|  | 80 | 30 | 217 | 13 | 21 | 256QAM | 0.69 | 1 | 1 | 172176 | 24 | 21 | 249984 | 215.22 |
|  | 90 | 30 | 245 | 13 | 21 | 256QAM | 0.69 | 1 | 1 | 196776 | 24 | 24 | 282240 | 245.97 |
|  | 100 | 30 | 273 | 13 | 21 | 256QAM | 0.69 | 1 | 1 | 217128 | 24 | 26 | 314496 | 271.41 |
|  | 5 | 30 | 11 | 13 | 26 | 256QAM | 0.90 | 1 | 1 | 11272 | 24 | 2 | 12672 | 14.09 |
|  | 10 | 30 | 24 | 13 | 26 | 256QAM | 0.90 | 1 | 1 | 24576 | 24 | 3 | 27648 | 30.72 |
|  | 15 | 30 | 38 | 13 | 26 | 256QAM | 0.90 | 1 | 1 | 38936 | 24 | 5 | 43776 | 48.67 |
|  | 20 | 30 | 51 | 13 | 26 | 256QAM | 0.90 | 1 | 1 | 52224 | 24 | 7 | 58752 | 65.28 |
|  | 25 | 30 | 65 | 13 | 26 | 256QAM | 0.90 | 1 | 1 | 67584 | 24 | 9 | 74880 | 84.48 |
|  | 30 | 30 | 78 | 13 | 26 | 256QAM | 0.90 | 1 | 1 | 79896 | 24 | 10 | 89856 | 99.87 |
|  | 35 | 30 | 92 | 13 | 26 | 256QAM | 0.90 | 1 | 1 | 94248 | 24 | 12 | 105984 | 117.81 |
|  | 40 | 30 | 106 | 13 | 26 | 256QAM | 0.90 | 1 | 1 | 108552 | 24 | 13 | 122112 | 135.69 |
|  | 45 | 30 | 119 | 13 | 26 | 256QAM | 0.90 | 1 | 1 | 122976 | 24 | 15 | 137088 | 153.72 |
|  | 50 | 30 | 133 | 13 | 26 | 256QAM | 0.90 | 1 | 1 | 135296 | 24 | 17 | 153216 | 169.12 |
|  | 60 | 30 | 162 | 13 | 26 | 256QAM | 0.90 | 1 | 1 | 167976 | 24 | 20 | 186624 | 209.97 |
|  | 70 | 30 | 189 | 13 | 26 | 256QAM | 0.90 | 1 | 1 | 196776 | 24 | 24 | 217728 | 245.97 |
|  | 80 | 30 | 217 | 13 | 26 | 256QAM | 0.90 | 1 | 1 | 225480 | 24 | 27 | 249984 | 281.85 |
|  | 90 | 30 | 245 | 13 | 26 | 256QAM | 0.90 | 1 | 1 | 254176 | 24 | 31 | 282240 | 317.72 |
|  | 100 | 30 | 273 | 13 | 26 | 256QAM | 0.90 | 1 | 1 | 278776 | 24 | 34 | 314496 | 348.47 |
|  | 5 | 30 | 11 | 13 | 20 | 256QAM | 0.67 | 2 | 1 | 16896 | 24 | 3 | 25344 | 21.12 |
|  | 10 | 30 | 24 | 13 | 20 | 256QAM | 0.67 | 2 | 1 | 36896 | 24 | 5 | 55296 | 46.12 |
|  | 15 | 30 | 38 | 13 | 20 | 256QAM | 0.67 | 2 | 1 | 58384 | 24 | 7 | 87552 | 72.98 |
|  | 20 | 30 | 51 | 13 | 20 | 256QAM | 0.67 | 2 | 1 | 77896 | 24 | 10 | 117504 | 97.37 |
|  | 25 | 30 | 65 | 13 | 20 | 256QAM | 0.67 | 2 | 1 | 100392 | 24 | 12 | 149760 | 125.49 |
|  | 30 | 30 | 78 | 13 | 20 | 256QAM | 0.67 | 2 | 1 | 118896 | 24 | 15 | 179712 | 148.62 |
|  | 35 | 30 | 92 | 13 | 20 | 256QAM | 0.67 | 2 | 1 | 139376 | 24 | 17 | 211968 | 174.22 |
|  | 40 | 30 | 106 | 13 | 20 | 256QAM | 0.67 | 2 | 1 | 163976 | 24 | 20 | 244224 | 204.97 |
|  | 45 | 30 | 119 | 13 | 20 | 256QAM | 0.67 | 2 | 1 | 184424 | 24 | 22 | 274176 | 230.53 |
|  | 50 | 30 | 133 | 13 | 20 | 256QAM | 0.67 | 2 | 1 | 204976 | 24 | 25 | 306432 | 256.22 |
|  | 60 | 30 | 162 | 13 | 20 | 256QAM | 0.67 | 2 | 1 | 250056 | 24 | 30 | 373248 | 312.57 |
|  | 70 | 30 | 189 | 13 | 20 | 256QAM | 0.67 | 2 | 1 | 286976 | 24 | 35 | 435456 | 358.72 |
|  | 80 | 30 | 217 | 13 | 20 | 256QAM | 0.67 | 2 | 1 | 335976 | 24 | 40 | 499968 | 419.97 |
|  | 90 | 30 | 245 | 13 | 20 | 256QAM | 0.67 | 2 | 1 | 376896 | 24 | 45 | 564480 | 471.12 |
|  | 100 | 30 | 273 | 13 | 20 | 256QAM | 0.67 | 2 | 1 | 417976 | 24 | 50 | 628992 | 522.47 |
|  | 5 | 30 | 11 | 13 | 21 | 256QAM | 0.69 | 2 | 1 | 17424 | 24 | 3 | 25344 | 21.78 |
|  | 10 | 30 | 24 | 13 | 21 | 256QAM | 0.69 | 2 | 1 | 37896 | 24 | 5 | 55296 | 47.37 |
|  | 15 | 30 | 38 | 13 | 21 | 256QAM | 0.69 | 2 | 1 | 60456 | 24 | 8 | 87552 | 75.57 |
|  | 20 | 30 | 51 | 13 | 21 | 256QAM | 0.69 | 2 | 1 | 81976 | 24 | 10 | 117504 | 102.47 |
|  | 25 | 30 | 65 | 13 | 21 | 256QAM | 0.69 | 2 | 1 | 104496 | 24 | 13 | 149760 | 130.62 |
|  | 30 | 30 | 78 | 13 | 21 | 256QAM | 0.69 | 2 | 1 | 125016 | 24 | 15 | 179712 | 156.27 |
|  | 35 | 30 | 92 | 13 | 21 | 256QAM | 0.69 | 2 | 1 | 147576 | 24 | 18 | 211968 | 184.47 |
|  | 40 | 30 | 106 | 13 | 21 | 256QAM | 0.69 | 2 | 1 | 167976 | 24 | 20 | 244224 | 209.97 |
|  | 45 | 30 | 119 | 13 | 21 | 256QAM | 0.69 | 2 | 1 | 188576 | 24 | 23 | 274176 | 235.72 |
|  | 50 | 30 | 133 | 13 | 21 | 256QAM | 0.69 | 2 | 1 | 213176 | 24 | 26 | 306432 | 266.47 |
|  | 60 | 30 | 162 | 13 | 21 | 256QAM | 0.69 | 2 | 1 | 258144 | 24 | 31 | 373248 | 322.68 |
|  | 70 | 30 | 189 | 13 | 21 | 256QAM | 0.69 | 2 | 1 | 303240 | 24 | 36 | 435456 | 379.05 |
|  | 80 | 30 | 217 | 13 | 21 | 256QAM | 0.69 | 2 | 1 | 344376 | 24 | 41 | 499968 | 430.47 |
|  | 90 | 30 | 245 | 13 | 21 | 256QAM | 0.69 | 2 | 1 | 393272 | 24 | 47 | 564480 | 491.59 |
|  | 100 | 30 | 273 | 13 | 21 | 256QAM | 0.69 | 2 | 1 | 434280 | 24 | 52 | 628992 | 542.85 |
|  | 5 | 30 | 11 | 13 | 26 | 256QAM | 0.90 | 2 | 1 | 22536 | 24 | 3 | 25344 | 28.17 |
|  | 10 | 30 | 24 | 13 | 26 | 256QAM | 0.90 | 2 | 1 | 49176 | 24 | 6 | 55296 | 61.47 |
|  | 15 | 30 | 38 | 13 | 26 | 256QAM | 0.90 | 2 | 1 | 77896 | 24 | 10 | 87552 | 97.37 |
|  | 20 | 30 | 51 | 13 | 26 | 256QAM | 0.90 | 2 | 1 | 104496 | 24 | 13 | 117504 | 130.62 |
|  | 25 | 30 | 65 | 13 | 26 | 256QAM | 0.90 | 2 | 1 | 135296 | 24 | 17 | 149760 | 169.12 |
|  | 30 | 30 | 78 | 13 | 26 | 256QAM | 0.90 | 2 | 1 | 159880 | 24 | 19 | 179712 | 199.85 |
|  | 35 | 30 | 92 | 13 | 26 | 256QAM | 0.90 | 2 | 1 | 188576 | 24 | 23 | 211968 | 235.72 |
|  | 40 | 30 | 106 | 13 | 26 | 256QAM | 0.90 | 2 | 1 | 217128 | 24 | 26 | 244224 | 271.41 |
|  | 45 | 30 | 119 | 13 | 26 | 256QAM | 0.90 | 2 | 1 | 245976 | 24 | 30 | 274176 | 307.47 |
|  | 50 | 30 | 133 | 13 | 26 | 256QAM | 0.90 | 2 | 1 | 270576 | 24 | 33 | 306432 | 338.22 |
|  | 60 | 30 | 162 | 13 | 26 | 256QAM | 0.90 | 2 | 1 | 335976 | 24 | 40 | 373248 | 419.97 |
|  | 70 | 30 | 189 | 13 | 26 | 256QAM | 0.90 | 2 | 1 | 393272 | 24 | 47 | 435456 | 491.59 |
|  | 80 | 30 | 217 | 13 | 26 | 256QAM | 0.90 | 2 | 1 | 450984 | 24 | 54 | 499968 | 563.73 |
|  | 90 | 30 | 245 | 13 | 26 | 256QAM | 0.90 | 2 | 1 | 507984 | 24 | 61 | 564480 | 634.98 |
|  | 100 | 30 | 273 | 13 | 26 | 256QAM | 0.90 | 2 | 1 | 557416 | 24 | 67 | 628992 | 696.77 |
|  | 5 | 30 | 11 | 13 | 22 | 256QAM | 0.74 | 4 | 1 | 33816 | 24 | 5 | 46464 | 42.27 |
|  | 10 | 30 | 24 | 13 | 22 | 256QAM | 0.74 | 4 | 1 | 73776 | 24 | 9 | 101376 | 92.22 |
|  | 15 | 30 | 38 | 13 | 22 | 256QAM | 0.74 | 4 | 1 | 118896 | 24 | 15 | 160512 | 148.62 |
|  | 20 | 30 | 51 | 13 | 22 | 256QAM | 0.74 | 4 | 1 | 159880 | 24 | 19 | 215424 | 199.85 |
|  | 25 | 30 | 65 | 13 | 22 | 256QAM | 0.74 | 4 | 1 | 200808 | 24 | 24 | 274560 | 251.01 |
|  | 30 | 30 | 78 | 13 | 22 | 256QAM | 0.74 | 4 | 1 | 241720 | 24 | 29 | 329472 | 302.15 |
|  | 35 | 30 | 92 | 13 | 22 | 256QAM | 0.74 | 4 | 1 | 286976 | 24 | 35 | 388608 | 358.72 |
|  | 40 | 30 | 106 | 13 | 22 | 256QAM | 0.74 | 4 | 1 | 327888 | 24 | 39 | 447744 | 409.86 |
|  | 45 | 30 | 119 | 13 | 22 | 256QAM | 0.74 | 4 | 1 | 368872 | 24 | 44 | 502656 | 461.09 |
|  | 50 | 30 | 133 | 13 | 22 | 256QAM | 0.74 | 4 | 1 | 409616 | 24 | 49 | 561792 | 512.02 |
|  | 60 | 30 | 162 | 13 | 22 | 256QAM | 0.74 | 4 | 1 | 507984 | 24 | 61 | 684288 | 634.98 |
|  | 70 | 30 | 189 | 13 | 22 | 256QAM | 0.74 | 4 | 1 | 590128 | 24 | 71 | 798336 | 737.66 |
|  | 80 | 30 | 217 | 13 | 22 | 256QAM | 0.74 | 4 | 1 | 671976 | 24 | 80 | 916608 | 839.97 |
|  | 90 | 30 | 245 | 13 | 22 | 256QAM | 0.74 | 4 | 1 | 770568 | 24 | 92 | 1034880 | 963.21 |
|  | 100 | 30 | 273 | 13 | 22 | 256QAM | 0.74 | 4 | 1 | 852696 | 24 | 102 | 1153152 | 1065.87 |
|  | 5 | 30 | 11 | 13 | 23 | 256QAM | 0.78 | 4 | 1 | 35856 | 24 | 5 | 46464 | 44.82 |
|  | 10 | 30 | 24 | 13 | 23 | 256QAM | 0.78 | 4 | 1 | 79896 | 24 | 10 | 101376 | 99.87 |
|  | 15 | 30 | 38 | 13 | 23 | 256QAM | 0.78 | 4 | 1 | 125016 | 24 | 15 | 160512 | 156.27 |
|  | 20 | 30 | 51 | 13 | 23 | 256QAM | 0.78 | 4 | 1 | 167976 | 24 | 20 | 215424 | 209.97 |
|  | 25 | 30 | 65 | 13 | 23 | 256QAM | 0.78 | 4 | 1 | 213176 | 24 | 26 | 274560 | 266.47 |
|  | 30 | 30 | 78 | 13 | 23 | 256QAM | 0.78 | 4 | 1 | 258144 | 24 | 31 | 329472 | 322.68 |
|  | 35 | 30 | 92 | 13 | 23 | 256QAM | 0.78 | 4 | 1 | 303240 | 24 | 36 | 388608 | 379.05 |
|  | 40 | 30 | 106 | 13 | 23 | 256QAM | 0.78 | 4 | 1 | 352440 | 24 | 42 | 447744 | 440.55 |
|  | 45 | 30 | 119 | 13 | 23 | 256QAM | 0.78 | 4 | 1 | 393272 | 24 | 47 | 502656 | 491.59 |
|  | 50 | 30 | 133 | 13 | 23 | 256QAM | 0.78 | 4 | 1 | 434280 | 24 | 52 | 561792 | 542.85 |
|  | 60 | 30 | 162 | 13 | 23 | 256QAM | 0.78 | 4 | 1 | 540776 | 24 | 65 | 684288 | 675.97 |
|  | 70 | 30 | 189 | 13 | 23 | 256QAM | 0.78 | 4 | 1 | 622760 | 24 | 74 | 798336 | 778.45 |
|  | 80 | 30 | 217 | 13 | 23 | 256QAM | 0.78 | 4 | 1 | 721000 | 24 | 86 | 916608 | 901.25 |
|  | 90 | 30 | 245 | 13 | 23 | 256QAM | 0.78 | 4 | 1 | 803304 | 24 | 96 | 1034880 | 1004.13 |
|  | 100 | 30 | 273 | 13 | 23 | 256QAM | 0.78 | 4 | 1 | 901344 | 24 | 107 | 1153152 | 1126.68 |
|  | 5 | 30 | 11 | 13 | 26 | 256QAM | 0.90 | 4 | 1 | 42016 | 24 | 5 | 46464 | 52.52 |
|  | 10 | 30 | 24 | 13 | 26 | 256QAM | 0.90 | 4 | 1 | 90176 | 24 | 11 | 101376 | 112.72 |
|  | 15 | 30 | 38 | 13 | 26 | 256QAM | 0.90 | 4 | 1 | 143400 | 24 | 18 | 160512 | 179.25 |
|  | 20 | 30 | 51 | 13 | 26 | 256QAM | 0.90 | 4 | 1 | 192624 | 24 | 23 | 215424 | 240.78 |
|  | 25 | 30 | 65 | 13 | 26 | 256QAM | 0.90 | 4 | 1 | 245976 | 24 | 30 | 274560 | 307.47 |
|  | 30 | 30 | 78 | 13 | 26 | 256QAM | 0.90 | 4 | 1 | 295176 | 24 | 36 | 329472 | 368.97 |
|  | 35 | 30 | 92 | 13 | 26 | 256QAM | 0.90 | 4 | 1 | 344376 | 24 | 41 | 388608 | 430.47 |
|  | 40 | 30 | 106 | 13 | 26 | 256QAM | 0.90 | 4 | 1 | 401640 | 24 | 48 | 447744 | 502.05 |
|  | 45 | 30 | 119 | 13 | 26 | 256QAM | 0.90 | 4 | 1 | 450984 | 24 | 54 | 502656 | 563.73 |
|  | 50 | 30 | 133 | 13 | 26 | 256QAM | 0.90 | 4 | 1 | 500136 | 24 | 60 | 561792 | 625.17 |
|  | 60 | 30 | 162 | 13 | 26 | 256QAM | 0.90 | 4 | 1 | 606504 | 24 | 72 | 684288 | 758.13 |
|  | 70 | 30 | 189 | 13 | 26 | 256QAM | 0.90 | 4 | 1 | 721000 | 24 | 86 | 798336 | 901.25 |
|  | 80 | 30 | 217 | 13 | 26 | 256QAM | 0.90 | 4 | 1 | 819256 | 24 | 97 | 916608 | 1024.07 |
|  | 90 | 30 | 245 | 13 | 26 | 256QAM | 0.90 | 4 | 1 | 934152 | 24 | 111 | 1034880 | 1167.69 |
|  | 100 | 30 | 273 | 13 | 26 | 256QAM | 0.90 | 4 | 1 | 1032192 | 24 | 123 | 1153152 | 1290.24 |
| Note 1: Allocated full DL slots are with slot index i, if mod(i,10) = 0,1,2,3,4,5,6 and i is not in {0,20,21} for i = 0,1,...,39. So total number of allocated slots per 2 frames is 25. Note 2: MCS Index is based on MCS Table defined in TS38.214 when 256QAM is enabled. Note 3: Number of DMRS REs per RB = 12,12,24,24 for number of MIMO layers = 1,2,3,4, respectively Note 4: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms. Note 5: Overhead parameter for TBS determination is 0. Note 6: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.3.2_1.2.1-3: Sustained Downlink Data Rate Reference Channel for TDD 30kHz SCS FR1(1024QAM)

| Parameter | Channel bandwidth | Subcarrier spacing | Allocated resource blocks | Number of consecutive PDSCH symbols for allocated full DL slots (Note 1) | MCS Index (Note 2) | Modulation | Target Coding Rate | Number of MIMO layers | LDPC Base Graph | Information Bit Payload per Slot for allocated full DL slots (Note 1) | Transport block CRC per Slot for allocated full DL slots (Note 1) | Number of Code Blocks per Slot for allocated full DL slots (Note 1, 6) | Binary Channel Bits per Slot for allocated full DL slots (Note 1) | Max. Throughput averaged over 2 frames |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MHz | kHz | PRBs | Symbols |  |  |  |  |  | Bits | Bits | CBs | Bits | Mbps |
|  | 5 | 30 | 11 | 13 | 23 | 1024QAM | 0.79 | 1 | 1 | 12552 | 24 | 2 | 15840 | 15.69 |
|  | 10 | 30 | 24 | 13 | 23 | 1024QAM | 0.79 | 1 | 1 | 27144 | 24 | 4 | 34560 | 33.93 |
|  | 15 | 30 | 38 | 13 | 23 | 1024QAM | 0.79 | 1 | 1 | 43032 | 24 | 6 | 54720 | 53.79 |
|  | 20 | 30 | 51 | 13 | 23 | 1024QAM | 0.79 | 1 | 1 | 57376 | 24 | 7 | 73440 | 71.72 |
|  | 25 | 30 | 65 | 13 | 23 | 1024QAM | 0.79 | 1 | 1 | 73776 | 24 | 9 | 93600 | 92.22 |
|  | 30 | 30 | 78 | 13 | 23 | 1024QAM | 0.79 | 1 | 1 | 88064 | 24 | 11 | 112320 | 110.08 |
|  | 35 | 30 | 92 | 13 | 23 | 1024QAM | 0.79 | 1 | 1 | 104496 | 24 | 13 | 132480 | 130.62 |
|  | 40 | 30 | 106 | 13 | 23 | 1024QAM | 0.79 | 1 | 1 | 120936 | 24 | 15 | 152640 | 151.17 |
|  | 45 | 30 | 119 | 13 | 23 | 1024QAM | 0.79 | 1 | 1 | 135296 | 24 | 17 | 171360 | 169.12 |
|  | 50 | 30 | 133 | 13 | 23 | 1024QAM | 0.79 | 1 | 1 | 151608 | 24 | 18 | 191520 | 189.51 |
|  | 60 | 30 | 162 | 13 | 23 | 1024QAM | 0.79 | 1 | 1 | 184424 | 24 | 22 | 233280 | 230.53 |
|  | 70 | 30 | 189 | 13 | 23 | 1024QAM | 0,79 | 1 | 1 | 213176 | 24 | 26 | 272160 | 266,47 |
|  | 80 | 30 | 217 | 13 | 23 | 1024QAM | 0.79 | 1 | 1 | 245976 | 24 | 30 | 312480 | 307.47 |
|  | 90 | 30 | 245 | 13 | 23 | 1024QAM | 0.79 | 1 | 1 | 278776 | 24 | 33 | 352800 | 348.47 |
|  | 100 | 30 | 273 | 13 | 23 | 1024QAM | 0.79 | 1 | 1 | 311368 | 24 | 37 | 393120 | 389.21 |
|  | 5 | 30 | 11 | 13 | 24 | 1024QAM | 0.83 | 1 | 1 | 13064 | 24 | 2 | 15840 | 16.33 |
|  | 10 | 30 | 24 | 13 | 24 | 1024QAM | 0.83 | 1 | 1 | 28680 | 24 | 4 | 34560 | 35.85 |
|  | 15 | 30 | 38 | 13 | 24 | 1024QAM | 0.83 | 1 | 1 | 45096 | 24 | 6 | 54720 | 56.37 |
|  | 20 | 30 | 51 | 13 | 24 | 1024QAM | 0.83 | 1 | 1 | 61480 | 24 | 8 | 73440 | 76.85 |
|  | 25 | 30 | 65 | 13 | 24 | 1024QAM | 0.83 | 1 | 1 | 77896 | 24 | 10 | 93600 | 97.37 |
|  | 30 | 30 | 78 | 13 | 24 | 1024QAM | 0.83 | 1 | 1 | 94248 | 24 | 12 | 112320 | 117.81 |
|  | 35 | 30 | 92 | 13 | 24 | 1024QAM | 0.83 | 1 | 1 | 110632 | 24 | 14 | 132480 | 138.29 |
|  | 40 | 30 | 106 | 13 | 24 | 1024QAM | 0.83 | 1 | 1 | 127080 | 24 | 16 | 152640 | 158.85 |
|  | 45 | 30 | 119 | 13 | 24 | 1024QAM | 0.83 | 1 | 1 | 143400 | 24 | 18 | 171360 | 179.25 |
|  | 50 | 30 | 133 | 13 | 24 | 1024QAM | 0.83 | 1 | 1 | 159880 | 24 | 19 | 191520 | 199.85 |
|  | 60 | 30 | 162 | 13 | 24 | 1024QAM | 0.83 | 1 | 1 | 192624 | 24 | 23 | 233280 | 240.78 |
|  | 70 | 30 | 189 | 13 | 24 | 1024QAM | 0,83 | 1 | 1 | 225480 | 24 | 27 | 272160 | 281,85 |
|  | 80 | 30 | 217 | 13 | 24 | 1024QAM | 0.83 | 1 | 1 | 262376 | 24 | 32 | 312480 | 327.97 |
|  | 90 | 30 | 245 | 13 | 24 | 1024QAM | 0.83 | 1 | 1 | 295176 | 24 | 35 | 352800 | 368.97 |
|  | 100 | 30 | 273 | 13 | 24 | 1024QAM | 0.83 | 1 | 1 | 327888 | 24 | 39 | 393120 | 409.86 |
|  | 5 | 30 | 11 | 13 | 23 | 1024QAM | 0.79 | 2 | 1 | 25104 | 24 | 3 | 31680 | 31.38 |
|  | 10 | 30 | 24 | 13 | 23 | 1024QAM | 0.79 | 2 | 1 | 54296 | 24 | 7 | 69120 | 67.87 |
|  | 15 | 30 | 38 | 13 | 23 | 1024QAM | 0.79 | 2 | 1 | 86040 | 24 | 11 | 109440 | 107.55 |
|  | 20 | 30 | 51 | 13 | 23 | 1024QAM | 0.79 | 2 | 1 | 114776 | 24 | 14 | 146880 | 143.47 |
|  | 25 | 30 | 65 | 13 | 23 | 1024QAM | 0.79 | 2 | 1 | 147576 | 24 | 18 | 187200 | 184.47 |
|  | 30 | 30 | 78 | 13 | 23 | 1024QAM | 0.79 | 2 | 1 | 176208 | 24 | 21 | 224640 | 220.26 |
|  | 35 | 30 | 92 | 13 | 23 | 1024QAM | 0.79 | 2 | 1 | 208976 | 24 | 25 | 264960 | 261.22 |
|  | 40 | 30 | 106 | 13 | 23 | 1024QAM | 0.79 | 2 | 1 | 241720 | 24 | 29 | 305280 | 302.15 |
|  | 45 | 30 | 119 | 13 | 23 | 1024QAM | 0.79 | 2 | 1 | 270576 | 24 | 33 | 342720 | 338.22 |
|  | 50 | 30 | 133 | 13 | 23 | 1024QAM | 0.79 | 2 | 1 | 303240 | 24 | 36 | 383040 | 379.05 |
|  | 60 | 30 | 162 | 13 | 23 | 1024QAM | 0.79 | 2 | 1 | 368872 | 24 | 44 | 466560 | 461.09 |
|  | 70 | 30 | 189 | 13 | 23 | 1024QAM | 0,79 | 2 | 1 | 426336 | 24 | 51 | 544320 | 532,92 |
|  | 80 | 30 | 217 | 13 | 23 | 1024QAM | 0.79 | 2 | 1 | 491800 | 24 | 59 | 624960 | 614.75 |
|  | 90 | 30 | 245 | 13 | 23 | 1024QAM | 0.79 | 2 | 1 | 557416 | 24 | 66 | 705600 | 696.77 |
|  | 100 | 30 | 273 | 13 | 23 | 1024QAM | 0.79 | 2 | 1 | 622760 | 24 | 74 | 786240 | 778.45 |
| Note 1: Allocated full DL slots are with slot index i, if mod(i,10) = 0,1,2,3,4,5,6 and i is not in {0,20,21} for i = 0,1,...,39. So total number of allocated slots per 2 frames is 25. Note 2: MCS Index is based on MCS Table defined in TS38.214 when 1024QAM is enabled. Note 3: Number of DMRS REs per RB = 12,12,24,24 for number of MIMO layers = 1,2,3,4, respectively Note 4: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms. Note 5: Overhead parameter for TBS determination is 0. Note 6: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### A.3.2_1.2.2	Reference measurement channels for SCS 120 kHz FR2

Table A.3.2_1.2.2-1: Sustained Downlink Data Rate Reference Channel for TDD 120kHz SCS FR2 (QPSK)

| Parameter | Channel bandwidth | Subcarrier spacing | Allocated resource blocks | Number of consecutive PDSCH symbols for allocated full DL slots (Note 1) | MCS Index (Note 2) | Modulation | Target Coding Rate | Number of MIMO layers | LDPC Base Graph | Information Bit Payload per Slot for allocated full DL slots (Note 1) | Transport block CRC per Slot for allocated full DL slots (Note 1) | Number of Code Blocks per Slot for allocated full DL slots (Note 1, 6) | Binary Channel Bits per Slot for allocated full DL slots (Note 1) | Max. Throughput averaged over 2 frames |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MHz | kHz | PRBs | Symbols |  |  |  |  |  | Bits | Bits | CBs | Bits | Mbps |
|  | 50 | 120 | 32 | 13 | 9 | QPSK | 0.66 | 1 | 1 | 5888 | 24 | 1 | 8800 | 27.379 |
|  | 100 | 120 | 66 | 13 | 9 | QPSK | 0.66 | 1 | 1 | 12040 | 24 | 2 | 18150 | 55.986 |
|  | 200 | 120 | 132 | 13 | 9 | QPSK | 0.66 | 1 | 1 | 24072 | 24 | 3 | 36300 | 111.935 |
|  | 50 | 120 | 32 | 13 | 9 | QPSK | 0.66 | 2 | 1 | 11784 | 24 | 2 | 17600 | 54.796 |
|  | 100 | 120 | 66 | 13 | 9 | QPSK | 0.66 | 2 | 1 | 24072 | 24 | 3 | 36300 | 111.935 |
|  | 200 | 120 | 132 | 13 | 9 | QPSK | 0.66 | 2 | 1 | 48168 | 24 | 6 | 72600 | 223.981 |
| Note 1: Allocated full DL slots are with slot index i, if mod(i,5) = 0,1,2 and i is not in {0,80,81} for i = 0,1,...,159. So total number of allocated slots per 2 frames is 93. Note 2: MCS Index is based on MCS Table defined in TS38.214 when 256QAM is not enabled. Note 3: Number of DMRS REs per RB is 12. Note 4: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms. Note 5: Overhead parameter for TBS determination is 6. Note 6: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.3.2_1.2.2-2: Sustained Downlink Data Rate Reference Channel for TDD 120kHz SCS FR2 (16QAM)

| Parameter | Channel bandwidth | Subcarrier spacing | Allocated resource blocks | Number of consecutive PDSCH symbols for allocated full DL slots (Note 1) | MCS Index (Note 2) | Modulation | Target Coding Rate | Number of MIMO layers | LDPC Base Graph | Information Bit Payload per Slot for allocated full DL slots (Note 1) | Transport block CRC per Slot for allocated full DL slots (Note 1) | Number of Code Blocks per Slot for allocated full DL slots (Note 1, 6) | Binary Channel Bits per Slot for allocated full DL slots (Note 1) | Max. Throughput averaged over 2 frames |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MHz | kHz | PRBs | Symbols |  |  |  |  |  | Bits | Bits | CBs | Bits | Mbps |
|  | 50 | 120 | 32 | 13 | 16 | 16QAM | 0.64 | 1 | 1 | 11272 | 24 | 2 | 17600 | 52.415 |
|  | 100 | 120 | 66 | 13 | 16 | 16QAM | 0.64 | 1 | 1 | 23568 | 24 | 3 | 36300 | 109.591 |
|  | 200 | 120 | 132 | 13 | 16 | 16QAM | 0.64 | 1 | 1 | 47112 | 24 | 6 | 72600 | 219.071 |
|  | 50 | 120 | 32 | 13 | 16 | 16QAM | 0.64 | 2 | 1 | 22536 | 24 | 3 | 35200 | 104.792 |
|  | 100 | 120 | 66 | 13 | 16 | 16QAM | 0.64 | 2 | 1 | 47112 | 24 | 6 | 72600 | 219.071 |
|  | 200 | 120 | 132 | 13 | 16 | 16QAM | 0.64 | 2 | 1 | 94248 | 24 | 12 | 145200 | 438.253 |
| Note 1: Allocated full DL slots are with slot index i, if mod(i,5) = 0,1,2 and i is not in {0,80,81} for i = 0,1,...,159. So total number of allocated slots per 2 frames is 93. Note 2: MCS Index is based on MCS Table defined in TS38.214 when 256QAM is not enabled. Note 3: Number of DMRS REs per RB is 12. Note 4: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms. Note 5: Overhead parameter for TBS determination is 6. Note 6: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.3.2_1.2.2-3: Sustained Downlink Data Rate Reference Channel for TDD 120kHz SCS FR2 (64QAM)

| Parameter | Channel bandwidth | Subcarrier spacing | Allocated resource blocks | Number of consecutive PDSCH symbols for allocated full DL slots (Note 1) | MCS Index (Note 2) | Modulation | Target Coding Rate | Number of MIMO layers | LDPC Base Graph | Information Bit Payload per Slot for allocated full DL slots (Note 1) | Transport block CRC per Slot for allocated full DL slots (Note 1) | Number of Code Blocks per Slot for allocated full DL slots (Note 1, 6) | Binary Channel Bits per Slot for allocated full DL slots (Note 1) | Max. Throughput averaged over 2 frames |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MHz | KHz | PRBs | Symbols |  |  |  |  |  | Bits | Bits | CBs | Bits | Mbps |
|  | 50 | 120 | 32 | 13 | 27 | 64QAM | 0.89 | 1 | 1 | 23568 | 24 | 3 | 26400 | 109.591 |
|  | 100 | 120 | 66 | 13 | 27 | 64QAM | 0.89 | 1 | 1 | 48168 | 24 | 6 | 54450 | 223.981 |
|  | 200 | 120 | 132 | 13 | 27 | 64QAM | 0.89 | 1 | 1 | 96264 | 24 | 12 | 108900 | 447.628 |
|  | 50 | 120 | 32 | 13 | 27 | 64QAM | 0.89 | 2 | 1 | 47112 | 24 | 6 | 52800 | 219.071 |
|  | 100 | 120 | 66 | 13 | 27 | 64QAM | 0.89 | 2 | 1 | 96264 | 24 | 12 | 108900 | 447.628 |
|  | 200 | 120 | 132 | 13 | 27 | 64QAM | 0.89 | 2 | 1 | 192624 | 24 | 23 | 217800 | 895.702 |
| Note 1: Allocated full DL slots are with slot index i, if mod(i,5) = 0,1,2 and i is not in {0,80,81} for i = 0,1,...,159. So total number of allocated slots per 2 frames is 93. Note 2: MCS Index is based on MCS Table defined in TS38.214 when 256QAM is not enabled. Note 3: Number of DMRS REs per RB is 12. Note 4: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms. Note 5: Overhead parameter for TBS determination is 6. Note 6: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### < End of changes >